*** XMODEM Protocol ***

XMODEM  refers  to  the  file transfer protocol introduced by Ward

Christensen in 1977.  XMODEM  is  widely  supported  on  most  all

bulletin board systems.

The following ASCII character definitions are used in XMODEM.

<SOH> = 01H  Is always the first byte in each block.

<EOT> = 04H  Sent instead of SOH to mark the end of transmission.

<ACK> = 06H  Positive acknowledgment.

<NAK> = 15H  Negative acknowledgment.

<CAN> = 18H  Cancel transfer.

XMODEM  is  a  receiver driven, asynchronous, 8 data bit protocol.

Each packet looks like the following:

<SOH> <packet #> <compliment #> <data> <checksum>

where:

<SOH>         = 01H

<packet #>    = Packet number, starting at 01, incrementing by 1,

and wraps from 0FFH to 00H (not to 01H).

<compliment #>= 255 minus the packet number.

<data>        = 128 bytes of binary data.

<checksum>    = The sum of the data bytes. Starting with a zero

add each data byte to the checksum. Use only the

rightmost 8 bits.

When the receiver is ready, it sends a NAK every 10 seconds  (  up

to one minute ) until the NAK is ackowledged by the transmitter by

sending  the  first  packet.  The transmitter continues by sending

each  packet  in  turn,  always  waiting  for  the  packet  to  be

ackowleged  before sending the next. Finally, when the transmitter

has no more data, it sends an EOT instead of a SOH to complete the

transfer.

Each packet is sent by the transmitter as follows:

(1) Each packet always starts with a SOH.

(2) The  packet  number  is  sent  next,  starting  with 01H, and

incrementing by 1. The packet number wraps from 0FFH to 00H.

(3) The packet number compliment  is  sent  next.  It  is  always

calculated a 255 minus the packet number.

(4) Exactly 128 bytes of data is sent.  If the transmitter doesn't

have  128  bytes  of  data to send, then control-Z's (1AH) are

padded to the data block.

(5) The checksum is calculated by  added  together  all  128  data

bytes and sending the least significant 8 bits.

(6) The transmitter waits ( up to 10 seconds ) for a response. If

the response is an ACK, then the transmitter goes on  to  the

next  packet.  If the response is a NAK, then the transmitter

re-sends the entire packet. If the response is  a  CAN,  then

all further transmission is cancelled.

Each packet is handled by the receiver as follows:

(1) If the first character is an EOT  then  the  transfer  is  now

complete, and the receiver should ACK the EOT before exitting.

If the  first  character is a CAN,  then  all  further  action

is cancelled. If the first character is  a  SOH,  then this is

the  first character of the next packet.

(2) If the packet number is not correct ( the first packet is 1 ),

then  this  is  a  fatal error, and the receiver should send a

CAN to the transmitter in order to cancel further transmission.

(3) If the packet number compliment is not correct, this is also a

fatal error and the receiver sends a CAN to the transmitter.

(4) Exactly 128 bytes of data are received.

(5) The  checksum  is  received.  It  is  compared with the value

obtained by computing the  checksum  from  the  just  received

data.  If the  checksum values are the same, an ACK to sent to

the transmitter. Otherwise, a NAK is sent.

Here is a XMODEM example of the data flow.  It  includes  the  two

most  common  line  hits:  a  garbaged  block,  and an <ACK> reply

getting  garbaged.  <data>  represent  128  bytes  of  data.  <CS>

represents the checksum byte.

SENDER                                  RECEIVER

======                                  ========

<---              <NAK>

<SOH> 01 FE <data> <CS>       --->

<---              <ACK>

<SOH> 02 FD <data> <CS>       --->       (data gets line hit)

<---              <NAK>

<SOH> 02 FD <data> <CS>       --->

<---              <ACK>

<SOH> 03 FC <data> <CS>       --->

(ACK gets garbaged)           <---              <ACK>

<SOH> 03 FC <data> <CS>       --->              <ACK>

<EOT>                         --->

<---       <anything except ACK>

<EOT>                         --->

<---              <ACK>

*** XMODEM/CRC Protocol ***

The XMODEM/CRC protocol is similar to the XMODEM protocol,  except

that  the  receiver specifies CRC-16 by sending C (Hex 43) instead

of NAK when requesting the FIRST block.  A two byte CRC is sent in

place of the one byte arithmetic checksum.

*** XMODEM/1K Protocol ***

The XMODEM/1K protocol is identical  to  XMODEM/CRC,  except  that

1024  byte  data blocks in addition to 128 byte data blocks can be

sent.  An STX (02H) replaces the SOH (01H) at the beginning of the

transmitted block to notify  the  receiver  of  the  longer  block

length.   The  transmitted block contains 1024 bytes of data.  The

receiver should be able to accept any mixture of 128 and 1024 byte

blocks.  The block number (in the second and third  bytes  of  the

block)  is  incremented  by  one  for each block regardless of the

block length.

The  sender  must  not  change  between  128  and  1024 byte block

lengths if it has not received a valid ACK for the current  block.

Here is an example of XMODEM/1K with 1024 blocks:

SENDER                                  RECEIVER

======                                  ========

C

STX 01 FE Data[1024] CRC CRC

ACK

STX 02 FD Data[1024] CRC CRC

ACK

STX 03 FC Data[1000] ^Z[24] CRC CRC

ACK

EOT

ACK

Mixed 1024 and 128 byte Blocks

SENDER                                  RECEIVER

======                                  ========

C

STX 01 FE Data[1024] CRC CRC

ACK

STX 02 FD Data[1024] CRC CRC

ACK

SOH 03 FC Data[128] CRC CRC

ACK

SOH 04 FB Data[100] ^Z[28] CRC CRC

ACK

EOT

ACK

*** YMODEM Protocol ***

The  YMODEM  Batch  protocol  is  an  extension  to  the XMODEM/1K

protocol that allows 0 or more files to be transmitted in a single

session. YMODEM always sends an information packet (  number  0  )

with each file containing the filename and the file length.

The filename is sent as a null terminated ASCII string.   This  is

the    filename    format   used  by  the  handle  oriented  MSDOS

functions.

The  length  field  is sent as a decimal ascii string counting the

number of data bytes in  the  file.   The  file  length  does  not

include any ^Z or other characters used to pad the last block. All

unused bytes in packet 0 must be set to 0.

After  the  file  has  been transmitted, the receiver asks for the

next file by sending a 'C'.

Transmission of a null pathname terminates the YMODEM protocol.

Here is an example of YMODEM:

SENDER                                  RECEIVER

======                                  ========

ACK

C

STX 02 FD Data[1024] CRC CRC

ACK

SOH 03 FC Data[128] CRC CRC

ACK

SOH 04 FB Data[100] ^Z[28] CRC CRC

ACK

EOT

NAK

EOT

ACK

C

SOH 00 FF NUL[128] CRC CRC

ACK