--------------------------------- Copyright ----------------------------------

Copyright (c) 1993..1996 by Michael Lee Finney.  All rights reserved.

--------------------------------- Trademarks ---------------------------------

Many names used in this document are trademarked.  I apologize for the

omission of any trademark which I have used and failed to mention as

trademarked.  If notified, I will gladly place the appropriate notice in

the next revision of this file.  The trademarks of which I am aware are...

...Trademark...   ...................Owner...................

AceCat II         AceCad

Altra             Altra

Bit Pad Plus      Summagraphics

Felix             Altra

IBM               International Business Machines Corporation

Inport            Microsoft Corporation

Kurta             Kurta Corporation

Logitech          Logitech

Microsoft         Microsoft Corporation

MicroTouch        MicroTouch Systems, Inc.

Mouseman          Logitech

OS/2              International Business Machines Corporation

PC Mouse          MSC Technologies

PS/2              International Business Machines Corporation

SummaSketch       Summagraphics

SummaSketch FX    Summagraphics

SummaSketch II    Summagraphics

SummaSketch III   Summagraphics

ThinkPad          International Business Machines Corporation

Trackman          Logitech

UnMouse           MicroTouch Systems, Inc.

WIN-OS/2          International Business Machines Corporation

Windows           Microsoft Corporation

--------------------------------- Shareware ----------------------------------

This program is shareware.  If you find this program useful you must

register your copy.  To register a private copy, send $15.00 (US funds)

for each copy to:

Michael Lee Finney

114 Old Wiggington Road

Lynchburg, Va. 24502-4669

Please make remittance payable to: Michael Lee Finney, and include your

company name (for a corporate license), name, address and phone number.

I would also appreciate the options used for your particular mouse.  This

information will allow me to better tune the options in the next release

and possibly provide an installation program.  The registration cost for

corporate site licenses is...

1..4   copies -- $25 each

5..10  copies -- $20 each

11..20  copies -- $15 each

21..40  copies -- $10 each

41...   copies --  $5 each

These prices are accumulative, so the first four copies cost $25 each,

the next six cost $20 each, and so on.  Thus, for example, a corporate

license for 23 copies would cost...

4 @ $25 ... $100

6 @ $20 ... $120

10 @ $15 ... $150

3 @ $10 ...  $30

------

$400

To obtain the latest version of this program when you register, send

$10 for shipping and handling to the above address.  This $10 does NOT

apply to the registration cost and only one copy will be sent regardless

of the number of copies registered (the author is NOT in the business of

diskette duplication).  However, the $10 is a flat fee and not a per

registered copy charge.  If you send me $25 without indicating that a

private license with requested diskette or a commercial license without

a requested diskette is desired, I will take my cue from the method of

payment and request -- namely a check that appears to be a company check

or the use of a company letterhead or the mention of a company name will

be assumed to indicate a commercial registration whereas a private check

and a simple note without the mention of any company will be assumed to

be a private registration with a request for a diskette.

Note: Registration is not required for public beta releases of this program.

Private beta releases are licensed only to the person receiving them

directly from the author.  Private beta releases are not and may not

be publically distributed.  The license to use any beta release expires

when the corresponding general availability release is issued and with

this release the license to use all prior beta releases has expired.

However, licensed users that are using a private beta release may

continue doing so since such use is licensed by registering the

associated general availability release.

---------------------------------- License -----------------------------------

Unregistered users (personal or corporate) are permitted to use this

program for a trial period of one month.  By the end of this period you

must either register the program or remove it from your system(s).

Registered users are permitted to use this program on one system at a

time and may make as many archived copies for backup purposes as needed.

It may be used on multiple systems (for a single license) only under the

condition that only one of the systems is in use at a time.  Since this

program is an operating system device driver, the assumption is that it is

in continual use when the system in use.  Therefore a registered copy

is required for each system on which this program is used simultaneously.

Of course, if you have two machines are never used at the same time (such

as a primary system and a laptop for offsite use) the same registered copy

may be used on both machines.

Registered users are entitled to support.  If after registration a

program error is found, the author will make every attempt possible to

repair the program.  I may require a registered user to order a diskette

containing the latest internal beta release before investing significant

time in analyzing a problem so that I can be certain that the problem has

not already been fixed (many problems have obscure symptoms).

If the failure cannot be repaired and makes the product unusable the

author will refund the registration fee (but the shipping and handling cost

for ordering a diskette is not considered the part of the registration fee).

Otherwise, registered users are eligible for the "bug" reward (see details

below) assuming that the problem is caused by a bug in RODENT.  Additionally,

registered users will also be sent notices of new versions of this program,

and of other software releases by the author.  In particular a registered

beta user will be notified of the general availability release.  Your name

will NOT be sold to a mailing list.

This program is not intentionally crippled in any manner, therefore a

new copy of the program will NOT be sent when the program is registered

unless explicitly requested.

Note: The license to use previous beta releases has expired with this

release of the general availability version of this software.  This

internal beta release is licenced for use by the license of the

most recent general availability release.

---------------------------------- Contents ----------------------------------

The distribution file RODNT100.ZIP contains the following files...

LICENSE.DOC  - Licensing information

ORDER.DOC    - Ordering information

RELEASE.DOC  - Release information

README.1ST   - Quick installation hints

RODENT.DOC   - This file

RODENT.SYS   - Mouse driver

RODENT.EXE   - Presentation Manager test program

BUSS.DDP     - Device driver installation file

CALCMP2X.DDP - Device driver installation file

CALCMP3X.DDP - Device driver installation file

CALCMP9X.DDP - Device driver installation file

CALCMPWZ.DDP - Device driver installation file

FELIX.DDP    - Device driver installation file

INPORT.DDP   - Device driver installation file

LOGITECH.DDP - Device driver installation file

MSOFT.DDP    - Device driver installation file

MSYSTEMS.DDP - Device driver installation file

PS2.DDP      - Device driver installation file

SGRAPH.DDP   - Device driver installation file

If this device driver is placed on a bulletin board, all files must

be distributed -- preferably in the original RODNT100.ZIP file.

---------------------------------- Summary -----------------------------------

RODENT.SYS is a mouse driver which supports most mice.  It provides

three button support.  The advantages of RODENT.SYS over the IBM mouse

drivers (assuming that they even work for your mouse) are...

1. Up to 7 buttons are supported (but only three can be reported

to OS/2 at this time due to the device driver interface provided

by IBM).

2. The mouse buttons can be arbitrarily reassigned (reordered).

3. The middle button on most 3 button mouse can be programmed as a

chord of the left and right buttons.

4. The interrupt handlers have been carefully tuned resulting in a

lower system load (compared to the IBM mouse drivers) when the

mouse is active.

5. The FIFO buffer of an 16550AFN or 16552 can be enabled, further

lowering the system load under the appropriate conditions.

6. More mice are supported using more protocols than the IBM mouse

drivers support.  Some digitizer tablets are supported.  All mice

supported by the IBM mouse drivers are supported.

7. Auto-detects the type of mouse, where possible.

8. Auto-detects the number of mouse buttons, where possible.

9. Auto-detects the type of uart (for serial mice).

10. Supports additional baud rates over the basic 1200 provided by the

IBM mouse drivers.  Baud rates of 150, 300, 600, 1200, 2400, 4800,

9600 and 19200 are supported for those mice which both allow the

baud rate to be set and which have a reset function available that

returns the mouse to a known state.

11. Supports additional reporting rates over the default provided by

the IBM mouse drivers.  Reporting rates of 10, 20, 30, 40, 50,

60, 80, 100 and 200 are supported for those mice which allow the

reporting rate to be set.

12. Any 8250 compatible uart can be used at any port address and any irq.

The IBM drivers only support COM1..COM2.

13. Should not "lose" the mouse during a reboot via Ctrl-Alt-Del.

14. Mouse deinstalls correctly on ABIOS systems (a bug in some of the IBM

mouse drivers).

15. Low battery detection for serial Logitech radio mice is provided

during system boot.

16. The actual mouse resolution can be specified.

17. The mouse sensitivity can be increased or reduced by a factor of

1, 2, 4 or 8.  This is useful for very low or high resolution mice

and for handicapped individuals who have problems with fine motor

control.

18. The direction of movement in either or both of the X and Y directions

can be reversed.  The X and Y cooridinates can themselves be exchanged.

19. Irq sharing can be controlled.

20. The Felix mouse supports a virtual "window" for absolute positioning

which may be smaller than the actual display.

21. The device name RODENT uses may be specified by the user to avoid

conflicts with other device drivers.

22. RODENT can be used to provide support for a second mouse when the

first mouse is supported by MOUSE.SYS (for those versions of MOUSE.SYS

which support the undocumented STYPE option).  This feature is

considered "experimental" since it relies upon an undocumented option

for MOUSE.SYS (which, in fact, does not always work).

---------------------------------- Support -----------------------------------

This is a shareware product.  Support is only available to registered

users.  The registration cost is only $15 so PLEASE register if you find

this product useful.  Problems can be reported via email (in order of

preference at:

Internet:   mfinney@inmind.com

Compuserve: 71573,1075

Internet:   71573.1075@COMPUSERVE.COM

IBM NTS:    Mike Finney

Note: If you send a message and fail to get a response please try

again, possibly with another address.  I respond to EVERY

message, but have had a few returned and without a repost

have no way to deliver my response.  And while I will usually

respond to any message within 48 hours of receipt, if on

vacation or otherwise unavailable I will respond as soon as

I do receive the message.  On some systems, e-mail not read

by a certain date will be discarded and if this happens please

repost (this did happen to one user when I was in the hospital

for a minor injury for a time).

Note: The first e-mail address (mfinney@inmind.com) gets checked

most frequently, but since it is a local provider it will

become invalid if I find it necessary to change my internet

provider.

and by U.S. Mail at:

Michael Lee Finney

114 Old Wiggington Road

Lynchburg, Va. 24502-4669

Please include all pertinent information regarding the problem, including

hardware configuration, operating system release and corrective service

level.  Include your name, address, email address, phone number, etc. so that

I can contact you.  Please include time zone, and the best hours to reach you

by phone and the hours you would rather NOT be reached by phone (shareware

authors aren't permitted to sleep).  However, a binary copy of another mouse

driver is useless (several people have actually sent me one).

I will attempt to resolve any reported problems.  However, if the problem

is caused by a hardware incompatibility then resolution may be difficult if

I am unable to locate equivalent hardware.  I will work with you as far as

possible.  I am willing to support incompatible mice, but I must be able to

get the necessary hardware.  If you have such a mouse send me a note with a

return address or phone number and I will work with you.

----------------------------------- Reward -----------------------------------

In order to promote software quality, and as an assurance of the existing

software quality of this product, a reward of 25% of the registration fee

with a minimum of $5.00 and maximum of $25.00 is offered to any registered

user for each report of a previously unknown program error.

RULES:

1. The reward is only offered to registered users who have not requested

a refund (however, I will accept bug reports from any user).

2. The maximum reward paid to a single user is limited to twice the

registration cost paid by the user (if you register 10 copies then

your maximum possible reward is greater than if you register 1 copy).

3. I am the final arbiter with regards to precedence between users

reporting the same program error.

4. I am the final arbiter in determining that a reported program error

does, in fact, exist.

5. Failure to support incompatible hardware is NOT considered an error.

I cannot be responsible for every device some twisted hardware

designer creates (even IBM cannot do that).  However, I will work

with users to resolve any problem to the best of my ability.  To

date, the vast majority of all reported problems have fallen into

this category.

6. This reward is not offered for beta releases and experimental

features.  If the bug is found in a beta release and is also in the

most recent non-beta release then the reward does apply.

7. The reward is only offered to end users.  A distributor that purchases

copies for their users is NOT considered an end user.  A company that

purchases copies for their internal use IS considered an end user.

8. Shipping and handling fees for ordering a diskette are not considered

part of the registration cost.

-------------------------------- Installation --------------------------------

This mouse driver is an OS/2 2.x device driver ONLY, it does NOT

function under OS/2 1.x or under any version of DOS, Windows or Unix.

It is not possible to install this driver under OS/2 1.x, DOS, Windows

or Unix.

Installing RODENT.SYS during OS/2 installation:

1. Unzip RODNT100.ZIP and copy the files to the root directory on

a diskette prior to installing OS/2 (unless using a diskette

received from the author, in which case this step has been done

for you).

2. Proceed with installing OS/2.

3. At the "Select Mouse" screen, select "No pointing device support".

This prevents OS/2 from placing incorrect statements in CONFIG.SYS

during installation.  Any other choice will result in incorrect

installation of the mouse, in particular "other" is not acceptable.

4. At the "Advanced Options" screen, select "Install Device Support

Diskette".

5. At the "OS/2 2.1 Device Driver Installation" screen, place the

diskette from step 1 into the diskette drive and select "Install".

6. At the "Select Device Drivers" screen, select the appropriate mouse

driver (down arrow to change current highlighted entry and space bar

to select the current highlighted entry).  Press <tab> to move to

the "Ok" button and then press <Enter>.  OS/2 will copy RODENT.SYS

into the \OS2 directory and update your CONFIG.SYS file.  It will

then present the "OS/2 2.1 Device Driver Installation" screen again

because you might have other device drivers.  If so, install them

now.  After all of your device drivers have been installed use the

right arrow to select "Exit" and press <Enter>.  At the "Exit the

Program" dialog, select "Yes".  OS/2 will then present you with a

message that your system configuration has been changed.  Press

<Enter> to continue with the installation of OS/2.

7. At the "Advanced Options" screen, press <Enter> to continue.  You

may now complete the installation of OS/2.

Warning: OS/2 will add the statements to CONFIG.SYS in the incorrect

order.  If you have installed a serial mouse you may need

to edit your CONFIG.SYS file before your mouse can be used.

The mouse statements in your CONFIG.SYS file must occur in

the following order...

DEVICE=...POINTDD.SYS...

DEVICE=...RODENT.SYS...

DEVICE=...MOUSE.SYS...

DEVICE=...VMOUSE.SYS...

DEVICE=...COM.SYS...

DEVICE=...VCOM.SYS...

If this is the case, then when OS/2 is rebooted it may tell

you that it failed to install some or all of RODENT.SYS,

MOUSE.SYS and VMOUSE.SYS.  Press <Enter> to continue booting

OS/2, start an editor (the system editor is always available)

and change the CONFIG.SYS file so that the mouse statements

are in the correct order.  This should be done when installing

a serial mouse even if OS/2 boots without error messages to

avoid conflicts with the mouse driver and the serial port

driver.

Installing RODENT.SYS after OS/2 installation:

The file RODENT.SYS should be copied to a directory of your

choice -- the C:\OS2 directory is the usual choice, but any directory

can be used.  If OS/2 does not support your mouse at all then remember

that the mouse will not function correctly until RODENT.SYS has been

installed.  It is necessary to modify the CONFIG.SYS file contained in

the root directory of your boot drive to contain the following lines...

DEVICE=x:\OS2\POINTDD.SYS

DEVICE=x:\path\RODENT.SYS [options...]

DEVICE=x:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=x:\OS2\MDOS\VMOUSE.SYS

and any other existing lines related to the mouse driver should be

removed.  These lines should be placed in the order given and for

serial mice must precede the "DEVICE=...COM.SYS..." line.  For

example, assuming that OS/2 is located on the C: drive and that

RODENT.SYS has been placed in the C:\OS2 directory:

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

or:

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS BAUD=19200 BUFFERED DPI=400

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

or:

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS PORT=73D8 IRQ=11 BAUD=2400 BUFFERED MOUSE=C PROTOCOL=MM BUTTONS=2 DPI=300

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

The first DEVICE line in the last example is longer than 80 characters.

In most cases an installation similar to the first two examples would

be used.  If you have multiple mice attached to your system, then you

may need to specify the mouse type or the serial port for the desired

mouse.  Only one mouse can be used at a time.  Lines in your CONFIG.SYS

which are mouse related are...

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\VISION.SYS...

DEVICE=C:\OS2\PCLOGIC.SYS...

DEVICE=C:\OS2\MOUSE.SYS...

DEVICE=C:\OS2\MDOS\VMOUSE.SYS...

all such lines should be removed and replaced as shown above.  There may

be other mouse related lines for different releases of OS/2 and where

mouse vendors have provided an OS/2 mouse driver.  Further, the

statements in CONFIG.SYS should occur in the following order...

DEVICE=...POINTDD.SYS...

DEVICE=...RODENT.SYS...

DEVICE=...MOUSE.SYS...

DEVICE=...VMOUSE.SYS...

DEVICE=...COM.SYS...

DEVICE=...VCOM.SYS...

failure to observe this ordering may result in failure of the mouse to

load properly or of the serial ports to operate correctly.  This mouse

driver has been tested under OS/2 2.0 GA, 2.0 GA+SP, 2.1 March beta and

2.1 GA.  It has been tested on a 50Mhz 486DX AT-clone and on an IBM

PS/2 model 70 (25Mhz 386).  The following mice have been tested...

AceCat II

Bit Pad Plus

CalComp Wiz digitizer

CalComp DrawingPad digitizer

Felix mouse (328 point version, 2 buttons)

Felix mouse (640 point version, 2 buttons)

Felix mouse (640 point version, 3 buttons)

Felix mouse (1280 point version, 3 buttons)

Honeywell Mouse

Logitech C9 Mouse

Logitech First Mouse (serial)

Logitech MouseMan Cordless Mouse (serial)

Logitech MouseMan Mouse (buss)

Logitech TrackMan Mouse (serial -- Mouse Systems compatible)

Microsoft Mouse (Inport buss)

Microsoft Mouse (serial)

Mouse System's 2D/3D mouse

SummaSketch II Plus

SummaSketch II Professional Plus

SummaSketch III

UnMouse

Only these mice have been tested by the author with this mouse

driver.  However, the driver is MUCH more capable because special

provisions have been made to support "partially" compatible mice

and most common mice use one of the available support protocols.

Additional mice have been tested by users who worked directly with

the author (such as the Kurta digitizer), but the author has not

personally tested any mouse not listed above.

In some cases a mouse might function with the driver, but require

some of the more exotic options (see the option list below).  For

example, you could have a mouse which recognized most of the protocols

associated with a Logitech type C mouse, but could not be automagically

recognized.  In this case you would use the CONFIG.SYS entries:

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=1 MOUSE=C

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

(assuming the mouse is attached to COM1) which forces the mouse

driver to accept the mouse as a Logitech type C mouse, but avoids

the recognition check used for the type C mice.  However, all

programming codes are still sent to the mouse.  Or as a more severe

example, you could have a mouse which supports the Mouse System's 5B

communication protocol (default with Mouse System's mice) but nothing

else works.  In that case you would use the CONFIG.SYS entries:

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=1 MOUSE=* PROTOCOL=5B

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

which forces the mouse driver to use the correct communication protocol,

but with NO attempt made at sending programming codes to the mouse.

The mouse must default to the appropriate state to use the specified

protocol.  Many mice can be setup this way, however other mice either

cannot be automagically recognized or intrinsically REQUIRE programming.

------------------------- Known Setup Configurations -------------------------

Felix mouse (no PS/2 connector, 640 point version, either 2 or 3 buttons)

Honeywell Mouse (as shipped)

Keen Mouse (switch in MS mode, provides two buttons)

Logitech C7 Mouse (serial)

Logitech C9 Mouse (serial or buss)

Logitech First Mouse (serial -- model: M-MD15L-9F)

Logitech MouseMan Cordless Mouse (serial -- model: M-RA12)

Logitech MouseMan Mouse (buss -- model: M-PD13-9MD)

Logitech TrackMan Mouse (serial -- models: T-CA1- and T-CA1-9F)

Microsoft Mouse (serial or Inport buss)

Mouse System's 2D/3D mouse (and other newer mice)

PS/2 port mice

IBM ThinkPad (red button)

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT\RODENT.SYS

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

Felix mouse (328 point version, 2 buttons)

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT\RODENT.SYS X=328 Y=328

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

Felix mouse (640 point version, 3 buttons)

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT\RODENT.SYS MOUSE=FX

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

Felix mouse (1280 point version, 3 buttons)

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT\RODENT.SYS X=1280 Y=1280 MOUSE=FX

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

Honeywell Mouse (with jumper J3 cut)

Keen Mouse (switch in PC mode, provides three buttons)

Mouse Systems PC Mouse III

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=# MOUSE=MS

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

CalComp Wiz and 2x00 digitizers

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=# MOUSE=DW

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

CalComp 3300 (and newer) digitizers

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=# MOUSE=D3

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

CalComp 9x00 digitizers

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=# MOUSE=D9

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

Summagraphics FX digitizers

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=# MOUSE=SX

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

Summagraphics digitizers which recognize the master reset

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=# MOUSE=SG

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

Summagraphics digitizers which fail to recognize the master reset and

which have a 4-button puck or a stylus attached

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=# MOUSE=SM

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

Summagraphics digitizers which fail to recognize the master reset and

which have a 16-button puck attached

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=# MOUSE=SU

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

Summagraphics Bit Pad Plus digitizers

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=# MOUSE=BP

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

Summagraphics Bit Pad Plus digitizers in CR mode

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=# MOUSE=CR

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

Kurta digitizers

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=# MOUSE=KR

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

AceCat II digitizers

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=# MOUSE=AC

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

UnMouse

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=# MOUSE=UN

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

"No mouse"

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS MOUSE=NM [IRQ=#]

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

----------------------------------- Hints ------------------------------------

8514:

The 8514 screen driver in OS/2 2.1 has a problem in full screen WIN-OS2

sessions.  When using absolute pointing devices.  The mouse cursor is

restricted to the top half of the screen.  DOS full screen sessions and

Presentation Manager sessions do not have this problem.  Changing screen

drivers eliminates the problem.  This problem has been reported at

1024x768 resolution and may or may not exist in other resolutions.  This

is NOT a bug in RODENT.

Absolute pointing device:

When using the Felix mouse, or any other absolute positioning device

whose maximum resolution is less than the screen dimensions the mouse

pointer will not traverse every screen point.  For example, on a 640x480

screen and an absolute device that has 320 points, only every other

horizontal screen point can be pointed at using the mouse cursor.

This does not cause a problem for most applications.  However, when

attempting to size a window it may not be possible to "grab" the

window's border.  There are two ways to alleviate this problem: first,

change the default border width using the Scheme Palette and second

use the window menu to select the "resize" option.  After the window

has been resized once using the mouse there will be no future

difficulties unless the screen resolution or the mouse resolution

is subsequently changed.

AceCat digitizers

The author has tested with an AceCat II digitizer, but has not had an

older tablet to test.  To use an older tablet, it may be necessary to

use one of the Summagraphic's MOUSE options, or to use MOUSE=* and to

specify the hardware protocol to be used.

Advanced Power Management

RODENT does not currently support power saving features.  If the mouse

becomes disabled due to being powered down (such as for PS/2 mouse)

then RODENT should not be used and the IBM mouse driver used instead.

AMD-386

While these are undoubtably fine processors, I have had one report from

a user who obtained a machine using one of these processors and was

unable to obtain reliable operation without disabling the cache.  This

could have been an early release of the AMD-386 or it could have been a

motherboard or BIOS flaw.  Nevertheless, he was unable to use RODENT (or

anything else) for more than a few minutes without a system crash.

AMI BIOS and BUFFERED

Some versions of this BIOS have problems recognizing the serial port

after a warm reboot and when the serial port has the FIFO buffer

enabled.  The symptoms of this is that the BUFFERED option is in use

and on a warm reboot the mouse is not found, but a power reset fixes

the problem (until the next warm reboot).  The fix for this is to

upgrade the BIOS.  However, there is a work-around using RODENT.

Instead of using the COM option, or allowing RODENT to find the mouse,

use the PORT and IRQ options with the appropriate value for the serial

port connected to the mouse.  This tells RODENT where the mouse is

located whether or not the BIOS recognized the serial port.  The COM

option and the automatic scan of the serial ports require the BIOS

to recognize the serial port.

Bit Pad digitizers in CR mode:

It appears that when using the digitizer in CR mode with the relative

protocol (MOUSE=CR PROTOCOL=CR) that pressing/releasing a button causes

the mouse to move.  The magnitude of this motion can be substantial and

can make it difficult to click or double click.  Use of the RESPONSE

option with a negative value can reduce the severity of this problem

(e.g. RESPONSE=-2) but does not eliminate the problem.  The problem is

further compounded by the fact that the buttons are stiff and tend to

cause the puck to "jitter".  The cause of this problem seems to be with

the electronics of the tablet and tends to be worse when the buttons are

rapidly pressed and released.  When using the digitizer in CR mode with

the absolute protocol this problem does not seem to occur.  This is

probably due to the lack of acceleration in absolute mode.  However, the

same tablet used in BitPad mode does not have the problem.  The suggested

solution is therefore to simply set the tablet for BitPad mode (which is

the factory default).

Button ordering:

When using a digitizer or mouse whose buttons aren't arranged in a good

default order, use the ORDER option to rearrange the buttons.

If the button order (i.e. right hand vs left hand) option has been

changed using the Presentation Manager mouse setup options the button

ordering is not maintained correctly for all sessions, and may even

reorder the wrong buttons.  Do not use the PM mouse setup option for

button ordering.  Use the "ORDER=..." option for RODENT.SYS instead.

This allows button reordering to apply to EVERY session and is

independent of PM.

CalComp DrawingPad

I have seen one occasion that the DrawingPad reversed the vertical

direction (down became up and vice-versa).  Admittedly, this WAS after

plugging in the wrong power supply <g> (but operating with the correct

supply).  Powering down the system and rebooting corrected the problem.

Clone mice:

Many of these mice (the Honeywell and Keen mice, for example) emulate

Microsoft mice and Mouse System's mice.  However, frequently these mice

have three buttons and when emulating Microsoft mice only allow the

outermost two buttons to be used.  Sometimes the middle button may

produce a chord click (the Honeywell mouse, for example) or may simply

ignore the middle button (the Keen mouse, for example).  RODENT.SYS

cannot do anything about this situation because that is a mouse hardware

limitation.  However, when emulating Mouse System's mice all three

buttons work properly.  This is the recommended mode for such mice.

This mode can frequently be selected by a slide switch.  The Honeywell

mouse requires disassembly of the mouse and cutting jumper J3 (see the

note below about Honeywell mice).

COM4 Installation:

There's a problem in VVGA.SYS and VSVGA.SYS that can kill COM4 when you

enter (or exit) a DOS full-screen session, even if that session isn't

using that port.  The drivers which handle that screen switching,

mistakenly think that there's an 8514/A video card also in the system.

and write to two registers on that card.  Of course, if you really DO

have an 8514/A or S3-based video card then you CAN'T use COM4 at 0x02E8

for this very reason.

You can patch VVGA.SYS and VSVGA.SYS to eliminate the problem; look for

the

MOV DX,4AE8

instruction and NOP out the next two OUT DX,AL instructions which write

to 0x4AE8 and 0x4AE9 (which map to 0x02E8 and 0x02E9 on most com boards).

This appears to be fixed in the March beta of OS/2 2.1 and in the GA

release of OS/2 2.1.

Thanks to Jens Schlatter for relaying this information from Bill Hinkle.

DOS session support:

All DOS support is provided by the IBM virtual mouse driver VMOUSE.SYS.

If you do not have DOS support, i.e. your CONFIG.SYS contains the line:

PROTECTONLY=YES

then none of the lines:

DEVICE=...VMOUSE.SYS...

DEVICE=...VCOM.SYS...

should be included in your CONFIG.SYS file.

DOS Virtual Machine support:

When booting a version of DOS in a virtual machine (VMB) then a native

mouse driver should NOT be used, but instead use the "stub" driver that

OS/2 provides.  This driver is called MOUSE.COM and is located in the

OS2\MDOS directory.  It can be loaded in your AUTOEXEC.BAT if you so

desire.

Dual boot or boot manager:

The Logitech type C mice frequently use the MM protocol when in DOS and

so the option "PROTOCOL=MM" may be necessary.  This allows OS/2 to use the

same protocol as used under DOS so the mouse won't be lost when switching

between DOS and OS/2.  In general, if using dual boot or boot manager and

the mouse uses a particular protocol under DOS the appropriate option may

need to be used under OS/2 to allow switching without losing the mouse.

However, under normal circumstances programmable mice such as the Logitech

type C mice are reset by RODENT.SYS to avoid such problems.  But (see the

note below for Logitech mice) there are some older type C mice which

appear to have initialization problems.

Felix mouse:

When this mouse is first powered on, it is necessary to move the mouse

stick to each edge of the mouse for the hardware to correctly recognize

the range of motion for the mouse stick.  If the mouse stick is moved

to one side and the cursor stops partway across the screen then this

resetting action needs to be performed.  The Felix mouse is an absolute

pointing device which has lower resolution than the screen.  Therefore

it cannot point to every pixel on the screen.  Normally this is not a

problem.  See the hints section for more information on how to handle

this situation.  Also see the RESPONSE statement for new uses in Felix

operation.  Further, if you have one of the newer Felix mice, your

available resolution has been increased (depending on hardware release),

so if the mouse pointer will only move to a portion of the screen and

an older Felix mouse is in use, make sure that the "X=328 Y=328" options

are in use.  If the entire screen is covered using only a portion of

the available mouse stick travel range, then mkae sure that the

"X=1280 Y=1280" options are in use.  If you have a three button Felix

mouse, then all three buttons are supported.  If you have a two button

Felix mouse, the driver reports three buttons present (since it can't

tell the difference) but only two buttons ever report events.

Newer Felix mice support multiple protocols.  Autorecognition on those

versions of the Felix defaults to MOUSE=M instead of MOUSE=FX.  If

desirable to run the Felix in absolute mode then MOUSE=FX must be

specified in the parameter list.  When using the default protocol, the

newer Felix mice the middle button generates a "chord" click, so only

two buttons are available.  When using MOUSE=FX, all three buttons are

supported (and if desired, of course, BUTTONS=2 will cause the middle

button to generate a "chord" click).  If both relative mode and three

button support is desired, then the options "MOUSE=FX PROTOCOL=FR" must

be specified in the parameter list.

Also, note that the default button ordering that RODENT uses for the

Felix was changed with the introduction of the three button Felix.

The ORDER option can be used to change the button ordering if the

default ordering is not satisfactory.  This change was made to match

the ordered expected by most users (it was less clear on the older

Felix mice as to what the user's expectations were in this regard).

The newer Felix mice provide a PS/2 connector and an adapter from that

connecter to a 9-pin serial connector.  This release of RODENT does not

support the Felix as a PS/2 device in absolute mode.  The Felix may

work in relative mode as a PS/2 device, but that configuration has not

been tested.

Honeywell mice:

These mice emulate Microsoft mice, but the middle button generates a

chord click.  However, they can also emulate Mouse System's mice if

jumper J3 is cut.  This requires disassembly of the mouse.  Please

call Honeywell at 800/445-6939 to obtain instructions for making this

modification.  I take no responsibility for any such hardware

modification.  Please note that any hardware alteration may affect

your warranty.  Honeywell is releasing a mouse with a slide switch to

overcome this problem 4Q '93.  Call Honeywell for availability and

pricing.

Hot plugging:

Unplugging the mouse and then plugging it back in while the system

is active is not supported in this release.  This is normally not

a problem unless you are using a mouse switcher between multiple

systems.  However, many protocols have no problem with this because

they automatically resynchronize.  Usually the mouse must be present

for the mouse driver to load, however using the "MOUSE=* PROTOCOL=xx"

options (where xx is the desired protocol may allow OS/2 to be booted

without the mouse being present.  The mouse must default to the

specified protocol or must be attached to a computer which sets the

protocol properly when booted.  PS/2 port mice disable themselves

when unplugged and replugged so they may not be used in this type of

application.

IBM 8516 Touch Screen

This is not supported.  IBM has a special driver for this discontinued

device, see the OS/2 documentation for installation instructions.

IBM PS/2 mice:

Some IBM PS/2 mice with a black roller ball have hardware problems.

IBM's recommendation for these mice is to replace them with IBM PS/2

mice with a gray roller ball (or any OTHER mouse for that matter).

Inport mice:

These are buss mice, but are NOT the same as AT buss mice.  There are

some Inport buss mice (Microsoft sells one) and AT buss mice (Logitech

sells one).  RODENT supports both kinds of buss mice and can tell them

apart.  Version 1.00 had a bug in the Inport buss mouse interrupt

handler that potentially could cause it to ignore mouse movement (but

the button presses were handled correctly).  This has been corrected.

Keyboard operation of OS/2:

When installing RODENT.SYS, especially if experimenting with different

features make sure that you have an editor easily accessible from the

desktop and make sure that you know the various keyboard commands

needed to navigate the desktop during times that the system is booted

without mouse support.  Some keyboard commands that are especially

useful are...

<Alt> + <Tab>

Move to the next window

<Up arrow> or <down arrow>

Use to highlight an object in a window or option in a menu

<Enter>

Use to select the highlighted object

<Alt> + F O

Use to open a file once the system editor (E.EXE) has been started

<Alt> + F S

Use to save a file once altered when using the system editor

<Alt> + <F4>

Use to close a window (for example the system editor)

<Alt> + <Shift> + <Tab>

Use to select the desktop

<Ctrl> + "\"

Use to deselect all objects on the desktop once the desktop has

been selected

<Shift> + <F10>

Use to pop-up the desktop menu that contains the shutdown option

Note: these last three commands are usually used in order to shutdown

OS/2 when the mouse is not available.

Kurta digitizers:

These digitizers are clones of the Summagraphics digitizers, but fail

to fully emulate the Summagraphic's digitizers.  In particular they

fail to support the master reset and all of the "z" commands.  The

MOUSE=KR option should be used for these tablets.

Logitech mice:

There are some older Logitech mice which seem to have initialization

problems.  These mice typically hang up or completely refuse to

install.  Using the "UART=..." option and possibly the "MOUSE=C" or the

"MOUSE=*" and "PROTOCOL=5B" options may help.  If the mouse installs but

hangs up and unplugging the mouse and then plugging it back in makes the

mouse come "alive" then try the above options.  Later versions of RODENT

have introduced an additional delay following some commands issued to the

mouse.  This appears to have eliminated this problem for at least some

users.  The requirement for this delay is undocumented by Logitech.

Microsoft PS/2 mice:

Microsoft PS/2 mice are not fully compatible with IBM PS/2 mice and

Logitech PS/2 mice.  This causes some difficulties in the interrupt

handler because the protocol is dependent upon the number of physical

buttons.  If using any three button PS/2 mouse with the options:

PORT=0060 IRQ=12 MOUSE=* PROTOCOL=PS BUTTONS=2

then the middle button will not be recognized.  However, this problem

does not exist when using any of:

...\RODENT.SYS

...\RODENT.SYS BUTTONS=2

...\RODENT.SYS BUTTONS=3

...\RODENT.SYS MOUSE=PS

...\RODENT.SYS MOUSE=PS BUTTONS=2

...\RODENT.SYS MOUSE=PS BUTTONS=3

because these allow the actual number of physical buttons to be

determined.  Also, the option:

...\RODENT.SYS PORT=0060 IRQ=12 MOUSE=* PROTOCOL=PS BUTTONS=3

can be used as well because it correctly specified the number of

physical buttons.  There are no problems when using two button

PS/2 mice.

Mouse sensitivity:

When the mouse is too sensitive or too slow, use the RESPONSE option

to change the mouse responsiveness.  Remember that the BAUD and RATE

options interact with the RESPONSE option.  If the reporting frequency

is increased then the average distance moved by the mouse between

reports is smaller.  Since there is a gradual acceleration threshold

to allow pixel by pixel control at slow speeds this means that the

mouse must be moved faster before acceleration becomes effective.

Conversely reducing the reporting frequency will effectively make the

mouse more "active" because the acceleration threshold is more likely

to be reached.

If the mouse sensitivity has been changed using the Presentation

Manager mouse setup options it may be found that the sensitivity in

other sessions is not appropriate.  Use the "RESPONSE=..." option

for RODENT.SYS instead.  This allows the mouse sensitivity to apply

to EVERY session and is independent of PM.  Also the Presentation

Manager acceleration occurs in addition to the acceleration provided

by RODENT.SYS.

Mouse Systems Buss card:

Mouse Systems has a buss mouse.  This is not really a buss mouse, but

is a special serial interface card.  It is supported by RODENT using

the PORT and IRQ options.

Multiple mice:

While multiple mice may be present in your computer system, only one

mouse can normally be used at a time (but see below).  The mouse to be

used is selected when OS/2 is booted and cannot be changed without

rebooting OS/2.  When searching for a mouse, RODENT.SYS uses the search

order:

Inport mice

Buss mice

PS/2 mice

"No mouse"

Serial mice

Logitech type C mice

Microsoft compatible mice (also Logitech type M, MW, V or VW mice)

Older Felix mice

Newer Felix mice                   (MOUSE=FX required)

Mouse System mice                  (MOUSE=MS required)

CalComp 2x00 and WIZ digitizers    (MOUSE=DW required)

CalComp 3300 and newer digitizers  (MOUSE=D3 required)

CalComp 9x00 digitizers            (MOUSE=D9 required)

SummaGraphics FX digitizers        (MOUSE=SX required)

SummaGraphics digitizers           (MOUSE=SG required)

SummaGraphics digitizers           (MOUSE=SM required)

SummaGraphics digitizers           (MOUSE=SU required)

Kurta digitizers                   (MOUSE=KR required)

AceCat II digitizers               (MOUSE=AC required)

UnMouse touch pad                  (MOUSE=UN required)

Bit Pad digitizers in Bit Pad mode (MOUSE=BP required)

Bit Pad digitizers in CR mode      (MOUSE=CR required)

Therefore Buss mice, Inport mice and PS/2 mice are recognized before

serial mice.  If you have one of these mice in your system but also

have a serial mouse attached you can force only serial mice to be

recognized by using the "MOUSE=S" option.  You can also force a

particular mouse to be recognized by using the appropriate "MOUSE=xx"

option.  You can prevent recognition of any mouse by using the "MOUSE=*"

option with the appropriate "PROTOCOL=xx" option.

However, MOUSE.SYS has an undocumeted option.  If you have two mice, one

of which is supported by MOUSE.SYS then RODENT can be used to provide

support for a second mouse.  MOUSE.SYS is not especially intelligent

at this, and takes the device characteristics from the mouse that it

supports (as of OS/2 2.1).  Thus, for example, assume that you have a

three button Logitech buss mouse and a three button Logitech serial

mouse.  MOUSE.SYS only supports two buttons on buss mice, but (as of

OS/2 2.1) supports three buttons on Logitech serial mouse (but the OS/2

2.1 version has bugs in this support).  So, to use RODENT to provide

support for a second mouse this can be done in two ways for this mouse

combination.  Now, the normal RODENT installation for the two mice in

this setup would be...

DEVICE=...\RODENT.SYS

DEVICE=...\MOUSE.SYS STYPE=RODENT$

DEVICE=...\RODENT.SYS MOUSE=S COM=1

DEVICE=...\MOUSE.SYS STYPE=RODENT$

where the first pair of lines provide support for the buss mouse and the

second pair of lines provide support for the serial mouse (but not at the

same time).  If support for both mice at the same time were desired, then

the setup would be...

DEVICE=...\RODENT.SYS

DEVICE=...\MOUSE.SYS STYPE=RODENT$ SERIAL=COM1

DEVICE=...\RODENT.SYS MOUSE=S COM=1

DEVICE=...\MOUSE.SYS STYPE=RODENT$

where the first pair of lines uses RODENT to support the buss mouse and

MOUSE.SYS to support the serial mouse.  This configuration will recognize

all three mouse buttons because MOUSE.SYS supports three buttons on the

serial mouse (even if incorrectly).

The second pair of lines uses RODENT to support the serial mouse and

MOUSE.SYS to support the buss mouse.  This configuration only recognizes

two mouse buttons because MOUSE.SYS only supports two buttons on the

buss mouse.

MOUSE.SYS will not allow three mice to be used at the same time (to the

best of my knowledge, and up to OS/2 2.1).

Please note that the STYPE option of MOUSE.SYS is undocumented, and as

such has not been used by many users.  It is therefore much more likely

to have bugs and the version of MOUSE.SYS in use may not allow the STYPE

option at all or may allow several occurrences.  MOUSE.SYS may allow the

use of STYPE with some mice and not with others even though there is no

apparent difference (to MOUSE.SYS).  In the examples above, if the serial

mouse is replaced by a Summmagraphics FX digitizer, then MOUSE.SYS will

not honor the STYPE option.  In the case that MOUSE.SYS allows multiple

occurrences of the STYPE option (perhaps a later release of OS/2), the

NAME option can be used to load multiple occurrences of RODENT to provide

support for as many mice as MOUSE.SYS will allow.  As of OS/2 2.1, the

TYPE and STYPE options for MOUSE.SYS are mutually exclusive, so RODENT

cannot be used to support both mice.

Also note that while it is possible to explicitly control the mouse that

RODENT is using, the same is not true of MOUSE.SYS.  If MOUSE.SYS supports

(however poorly) both mice then it may not be possible to get MOUSE.SYS

to provide secondary mouse support for a particular combination without

allowing MOUSE.SYS to support its "default" mouse and RODENT to support

the remaining mouse.

Use of multiple mice must be considered an "experimental" feature until

MOUSE.SYS provides more consistent support and so the "bug reward" does

not apply to problems with the simultaneous use of multiple mice.  Please

notice that RODENT does nothing special to enable this usage, this is an

undocumented feature of MOUSE.SYS.

Caution: RODENT.SYS will suppress mouse events which duplicate the last

event reported.  However, MOUSE.SYS does not do so.  So unless the

hardware does so, then when using the STYPE option, MOUSE.SYS merges

the two event streams.  Thus if a button is pressed on the device

managed by RODENT but not pressed by the device managed by MOUSE.SYS

then the effects can be that drags on RODENT's device are problematic

because RODENT's button down is followed immediately by MOUSE's button

up so there is no interventing mouse motion and that clicking a button

on RODENT's device results in many mouse clicks and double clicks

becuase many of the reports by RODENT with a button down are followed

by one or more reports by MOUSE with the same button up.  RODENT cannot

solve this problem.  The only solution is either for MOUSE.SYS to be

fixed to not report duplicate events or to use a mouse that MOUSE.SYS

controls which only sends reports when mouse motion is present or a

button is present (having RODENT reporting duplicate events does NOT

solve the problem).

PS/2 Installation:

Some PS/2 systems will require you to tell the hardware that you have "no

mouse" when installing a serial mouse instead of a PS/2 mouse.

PS/2 mice:

Some motherboards have incompatible keyboard/mouse controllers.  Normally

an 8042 chip is used, but some clones have used the 8242.  PS/2 mice will

not function on such systems.

RODENT.SYS not found

Check the drive and path used in your CONFIG.SYS file.

RODENT.SYS not installed

Check the options used for RODENT.SYS in your CONFIG.SYS file.

Serial port support:

If you are using a serial mouse and only have a single serial port then

none of the lines:

DEVICE=...COM.SYS...

DEVICE=...VCOM.SYS...

should be placed in your CONFIG.SYS file.

Summagraphics digitizers

This release does not support changing the puck while OS/2 is running.

Shutdown OS/2, power off the digitizer, change the puck (and CONFIG.SYS

if necessary) and then reboot the system.  Some older Summagraphics

tablets do not support the master reset that RODENT uses.  MOUSE=SM or

MOUSE=SU should be used (as appropriate) for these tablets.  This is also

true of some "Summagraphics compatible" digitizers.  Use of MOUSE=KR may

also be useful for some "Summagraphics compatible" digitizers.  A "soft"

reset following the master reset was added so that these older tablets

would be better supported.  While that may be adequate for some tablets,

it was found that other tablets would lock up when presented with the

master reset so the MOUSE=SM and MOUSE=SU options were added.  This is

necessary because Summagraphics tablets set the default hardware protocol

based on the attached puck, and the master reset is the only way to reset

the protocol.  Therefore, the 16-button puck cannot be used on these

tablets except with the MOUSE=SU option.  Further, these tablets must

default in hardware to 9600 baud because the baud rate is also not reset

by the "soft" reset.

SummaSketch II

These tablets have been observed to have some delayed motion after moving

the mouse.  This is a problem with the tablets and not with RODENT, and

is related to the "jitter" filter.  Replacing a SummaSketch II with a

SummaSketch III eliminates the problem.  This is most noticable when using

PROTOCOL=UR.

SummaSketch III

If the SummaSketch III is attached to a PS/2 model 70 and is powered on

before the model 70 then the model 70 may signal a hardware failure

during the power on hardware check.  This is a hardware issue and cannot

be alleviated by RODENT.SYS because the problem occurs before RODENT.SYS

is loaded into memory.  To alleviate this problem simply power up the

SummaSketch III after the model 70 has completed its hardware checks.

ThinkPad 720C

This machine has been tested with RODENT, but apparently the "dual" mouse

feature only works with some two button mice.  No three button mice could

be used with the ThinkPad 720C tested, nor could most versions of the

Microsoft serial mouse (it is possible in this case than an incorrect

adapter cable was used).  However, the mice that worked were identical

when either RODENT.SYS or the IBM mouse driver was used (but, of course,

the features of RODENT were available when RODENT was in use).

UnMouse

The UnMouse can, unfortunately, store its serial port configuration and

other parameters in non-volatile memory and there is no hardware reset

capability provided.  This is also the recommended mode of operation for

the UnMouse by MicroTouch.  This can make it impossible for RODENT to

initiate communications with the UnMouse if the UnMouse is not left in

the factory default setting (9600 baud, 7 data bits, no parity and 2 stop

bits).  The solution is to use the DOS or Windows program for the UnMouse

that was used to alter the setting and to reset the options back to the

factory default.  While other options are also stored, RODENT will reset

those if it is able to communicate with the UnMouse.  RODENT will not

permanently alter the settings on the UnMouse so that a power down will

reset the UnMouse back to the factory default settings.  If the UnMouse

is used in DOS or Windows by dual booting or using the boot manager then

care should be used to avoid altering the factory defaults.  RODENT will

be able to initiate communications if the default settings are left at

9600 baud, 8 data bits, no parity and 1 stop bit which are the settings

that RODENT uses.

VMOUSE.SYS not installed, but RODENT.SYS and MOUSE.SYS installed

This is frequently due to the failure to specify either the COM option

or the PORT and IRQ options when using MOUSE=* PROTOCOL=xx.  This

generally happens when RODENT.SYS installed but the mouse is not

actually present.  This can happen for mice which cannot be checked

and when checking is disabled.  This has also occurred on systems where

the FIFO buffering was enabled by the BIOS during the cold boot and

MOUSE.SYS was used (RODENT has no problem with this situation).

WIN-OS2 Jerky mouse movement

This is not a bug in RODENT, but a problem with Windows in part and in

WIN-OS2 in part.  I have seen suggestions that the following settings

seem to help with the problem...

IDLE_SECONDS=2 or 3

IDLE_SENSITIVITY=100

If you find a better solution to this problem, please let me know.

---------------------------------- Testing -----------------------------------

After booting OS/2 preliminary testing can be accomplished simply by

moving the mouse and making sure that the mouse cursor follows the movement

of the mouse.  Execute RODENT.EXE to further test this device driver.  This

test program will display a Presentation Manager window.  It will display

the number of buttons as perceived by OS/2 and the mouse resolution as

perceived by OS/2.  Further, if you click the mouse buttons, the associated

OS/2 mouse event is displayed.  The most recent mouse event is flagged by

an asterisk.  If the mouse pointer follows mouse movement correctly and

all buttons operate correctly then you will probably not have any trouble

with the mouse driver.  Other signs of trouble might be consistently missing

mouse events (such as clicks) or events occurring without any associated

physical mouse action.

When evaluating missing or extra events reasonable care should be taken.

It has been observed that occasionally OS/2 will "lose" a mouse click during

rapid mouse activity -- this is related to the fact that OS/2 only maintains

a small mouse event queue to avoid "piling" up an extremely large number of

events.  Sometimes losing a button release has the appearance of additional

clicks if an application auto-repeats while a mouse button is pressed.  The

loss, by OS/2, of mouse events is VERY rare except under very heavy mouse

activity.  However, it appears to be slightly more common when using higher

baud rates because the internal OS/2 queue is more likely to overflow.  This

has been seen using the Logitech radio mouse at 9600 baud.  Also this loss

might not actually be the fault of OS/2, but is occurring in the applications

instead (actually more probable).  Nevertheless, this behavior has sometimes

been seen, both with these mouse drivers AND with the OS/2 mouse drivers.

It has been seen most often using Logitech's radio mouse and Microsoft's PWB

in a full screen session.  Sometimes if this happens when scrolling releasing

the mouse button will not stop the scrolling.  Simply click the mouse button

again to stop scrolling.

This is a low-level mouse driver.  OS/2 handles all of the higher level

functions, including routing mouse events to DOS sessions, OS/2 sessions,

PM sessions, WIN-OS2 sessions and VMB sessions.  OS/2 reports all mouse

events (in the appropriate form) to the currently active session.  Therefore

DOS sessions CAN take advantage of all three buttons.  Since this mouse

driver is not involved in these actions, failure of OS/2 in that manner

is probably not caused by this device driver.

In particular, if when RODENT is installed, it works in Presentation

Manager but there are problems with DOS or WIN-OS2 sessions or if after

some time period or other event the mouse stops working in Presentation

Manager, these types of problems are almost certainly not related to RODENT.

There have been of these odd cases where MOUSE.SYS would work and RODENT

would not, but RODENT was functioning correctly.  Since RODENT optimizes

the interface, it does not send duplicate events to OS/2, MOUSE.SYS may

do so.  Where the problem is related to losing" mouse events, RODENT may

appear to fail while MOUSE.SYS appears to function correctly.  In every

case I have examined, both drivers were working, but some other operating

system component or application was causing the problem and was more

sensitive to RODENT's optimization of the event stream.  In many if not

most cases, the problems have actually been caused by the video device

driver which seem to be notorious for this type of behaviour.

Note: The resolution reported by OS/2 is in "mickeys per centimeter".

Therefore, the dots per inch provided to the device driver is divided by

2.54 and rounded when reporting to OS/2.  This device driver reports the

closest value possible.  For example, 400 dpi reports as 157 mickeys.

Since the value reported to OS/2 is a fixed point number, the result if

multiplied by 2.54 is only 399 (after rounding) instead of the original 400.

---------------------------------- Options -----------------------------------

BAUD=#

This option allows the baud rate to be specified.  It may be one of

150, 300, 600, 1200, 2400, 4800, 9600 or 19200

All mice do not support all baud rates.  To use the lowest baud rate

supported by the mouse specify:

BAUD=150

and to use the highest baud rate supported by the mouse specify:

BAUD=19200.

The lowest baud rate which gives good performance should be used to

reduce system load.  If this option is not specified then it defaults

to a value determined by the protocol in use.  Each protocol has a

default baud rate based on the default mouse baud rate.  If a baud

rate is specified which is not supported by the protocol the highest

baud rate which is less than the specified baud rate will be used, if

any, and otherwise the lowest baud rate which is higher than the

specified baud rate will be used.

BUFFERED

This option enables the FIFO buffer if a 16550 uart is detected.  The

FIFO buffer is NOT enabled by default because some 16550 uarts do not

function correctly.  All 16550AFN uarts should function correctly, if

you have a 16550, 16550A or 16550AF uart then it should be replaced by

a 16550AFN uart.  RODENT.SYS will function correctly if the uart is

not replaced, but this option can only be used with a 16550AFN or

equivalent uart (such as 16c552 or 16c554).

BUTTONS=#

This option indicates the number of buttons to be supported.  Many

protocols support both 2 button and 3 button mice.  This option may

be specified as:

BUTTONS=2

BUTTONS=3

If BUTTONS=3 is specified for a 2 button mouse the mouse driver will

function normally, but will report to OS/2 that there are two buttons

if RODENT.SYS can obtain the number of buttons from the mouse and

will otherwise report to OS/2 that there are three buttons.  However,

events for the third button will simply never occur.  If BUTTONS=3 is

specified for a 3 button mouse (the default) then all three buttons

will be returned to OS/2.  OS/2 treats clicking any two or three)

mouse buttons together as a "chord" click.  It also handles single or

double clicks and single click and drag from all three buttons.

If BUTTONS=2 is specified for a 3 button mouse then the mouse driver

will convert a press of the middle button into a press of both left

and right buttons.  This means that clicking the middle button is

equivalent to a "chord" click.  However, if PROTOCOL=MI is specified

for a three button mouse (the default is PROTOCOL=MP) this cannot be

done because the Microsoft-compatible 2 button protocol is not aware

of events for the middle button.  In this case the middle button will

simply be ignored.  The included PM test program will report the number

of mouse buttons as seen by OS/2.

COM=#

This option allows the serial port to explicitly specified.  This

option may be specified as:

COM=1

COM=2

COM=3

COM=4

where 1..4 is one of the standard com ports.  So,

COM=1

is normally equivalent to:

PORT=03f8 IRQ=4

If both this option and the PORT option are omitted then the mouse

driver will scan COM1..COM4 (in order) to find the mouse.  This

procedure works and is reasonably reliable, however it is impossible

to completely restore the uart state so automatic scanning could

potentially cause problems with other devices attached to serial

ports.  This mouse driver will restore more of the serial port state

than the IBM mouse drivers.  Specifying either this option or the

PORT option also reduces the time required to initialize the mouse

driver -- by as much as a couple of seconds.  This option can ONLY

be specified for serial mice.  It is recommended that either this

option or the PORT and IRQ options be used for all serial mice.

When the MOUSE option is used either this option or the PORT and IRQ

options may be necessary.

DPI=#

This option allows the mouse resolution to be specified.  This must

be in the range 100..640 (inclusive).  The default is approximately

DPI=200 (actually the default is 80 mickeys per centimeter).  Most

newer mice are higher resolution (and even some of the older mice).

The C9 Logitech Buss Mouse has a resolution of 320 dpi and the newer

Logitech mice have a resolution of 400 dpi.  The value used should be

the HARDWARE resolution, many DOS mouse drivers allow the resolution

to be "set" in the software -- this does not affect the actual mouse

events but only their reporting by the DOS mouse driver.  This value

is reported to OS/2, but at this time, OS/2 does not appear to make

a significant use of the value due to a bug in MOUSE.SYS.  The

included test program will report the value as set by RODENT.SYS

and as seen by OS/2 (but which is ignored by MOUSE.SYS).

FILTER=#

This option is only effective for devices which can configure the

device to discard marginal data points.  The acceptable values are

device dependent.

MOUSE=UN:

The filter parameter determines the number of values which are to

be discarded after a touch.  This must in the range of 0 to 99,

inclusive.  The default and recommended value is 20.  A value of

0 is not recommended.

FLIPXY

Reverses the X and Y coordinates.  Effectively rotates the mouse by

90 degrees.  Usually at least one of the REVERSEX and REVERSEY options

will be desirable.

Note: The REVERSEX and REVERSEY options are applied to the physical

coordinates and therefore BEFORE the FLIPXY option, so when

the FLIPXY option is in effect use REVERSEX instead of REVERSEY

and vice-versa.

To rotate a digitizer tablet 90 degrees to the right, the options

FLIPXY REVERSEY should be used.

To rotate a digitizer tablet 90 degrees to the left, the options

FLIPXY REVERSEX should be used.

To rotate a digitizer tablet 180 degrees then options

REVERSEX REVERSEY should be used.

IRQ=#

This option allows the interrupt number to explicitly specified.  Any

interrupt from 2..15 may be used, unlike the IBM drivers which are

restricted to interrupts 2..7.  If omitted, the interrupt level to

be used will be determined from the port address.  This procedure is

only reliable for the standard COM1 and COM2 port addresses 3f8 and

2f8.  It may work for COM3 and COM4 (depending on the actual port

addresses).  The IBM mouse driver has the same (undocumented) problem,

but does not provide a mechanism to specify the interrupt number.

MARGIN=#

This option allows the margin on an absolute pointing device to be

specified.  The default is MARGIN=0 since some digitizer tablets have

a hardware margin.  Units are in terms of the device resolution.  Most

digitizers are programmed for 1000lpi so MARGIN=500 gives a half inch

margin on each side of the tablet.  When the cursor is in the margin

area button presses will be recognized and the mouse cursor will move

in two dimensions but will be "pegged" to the appropriate side and so

will not move in that dimension.  Use of a margin prevents a "cliff"

effect.  The hardware margins provided by some devices will ONLY

recognize button presses.

MOUSE=xx

This option allows the mouse type to be specified.  This is useful to

reduce the time required to load the mouse driver, or if the mouse

used responds correctly but the mouse type cannot be auto-determined.

If an mouse type is specified that fails to match the hardware in use

then the mouse driver may or may not load, and if loads may fail to

operate properly.  This option may be specified as:

MOUSE=AC

AceCat digitizers -- this is an "absolute" pointing device.

Each point on the digitizer tablet corresponds exactly to a

point on the screen.  This is similar to "touch" screens in

operation.  This feature can be awkward to those not used to

it, but may be preferred by those who use a digitizer tablet

on a regular basis.  This option is required for these

digitizers, they cannot be automagically recognized.

Protocols: MA, MM, MR, UA, UR

default: MA

Baud Rate: 9600

default: 9600

Note: This type of mouse cannot be automagically recognized,

so this parameter is required for such mice.  Unlike the

Summagraphic's digitizers, it is not possible to determine

the puck type so it is always assumed that three buttons

are present.  The ORDER option will usually be required.

The recommended values depend upon the puck and protocol

which is in use...

MA/MM/MR      UA/UR

16-button puck                      ORDER=213   ORDER=1243

4-button puck                      ORDER=123   ORDER=2143

stylus (1 button and tip button)    ORDER=132   ORDER=312

stylus (2 buttons and tip button)   ORDER=132   ORDER=3142

MOUSE=B

Buss mouse, can be two button or three button.  Note that an

Inport mouse is NOT the same as a buss mouse.

Protocols: BS

default: BS

Rate: 30, 60

note: set by hardware jumper

MOUSE=BP

Summagraphics Bit Pad digitizer -- this is an "absolute" pointing

device.  This option is required for these digitizers, they

cannot be automagically recognized.

Protocols: BA, BR

default: BA

Baud Rate: 9600

Note: When using the 4-button puck, ORDER=132 or ORDER=123 are

both reasonably acceptable.  When using the stylus, only

a single button is available so this is normally button 1.

Note: These digitizers have a maximum range of 4096 points.

RODENT sets the maximum possible resolution.  However,

the tablet reduces this value based on the actual active

area which can vary between tablets.  Further the tablet

requires this value to be an integral multiple of the

tablet's dimension.  The nominal active area is 12 inches

with a resulting maximum of 4092 (since 4092 is the

smallest multiple of twelve which is less than or equal

to 4096).  However, the resolution is also a factor

of the active area.  Thus, the maximum returned by a

particular test tablet was X=4092 and Y=3900.  But, when

the hardware default resolution of 2400 was set (instead

of 4096) this same tablet set the maximum to X=2400 and

Y=2336.  This shows that the actual active area is partly

a function of the resolution used.

Therefore RODENT cannot control precisely the resolution

used by the tablet.  Except for explictly setting the

margin or a range this is not usually a problem.  A fixed

margin and variable range can be set using MARGIN=#.  A

fixed range and variable margin can be set by using

X=#..# and Y=#..# where the maximum is less than the any

probable active area variation.  Without knowing the

exact resolution for a particular tablet setting both a

fixed margin and a fixed range cannot be done.  It is

likely that the exact resolution could vary over time for

a given tablet.  The amount of this variation is not

mentioned in the hardware specifications but is probably

less than 5%.

MOUSE=C

Logitech Mouse System's compatible mouse.  DOS mouse drivers

frequently operate this mouse using the MM protocol.  When

using dual boot or Boot Manager it may be desirable to use

the MM protocol.

Protocols: 5B, MM, RE

default: 5B

better:  MM

Baud Rate: 1200, 2400, 4800, 9600

default: 1200

MOUSE=CR

Summagraphics Bit Pad digitizer -- this is an "absolute" pointing

device.  This option is required for these digitizers, they

cannot be automagically recognized.

Protocols: CA, CR

default: CA

Baud Rate: 9600

Note: When using the 4-button puck, ORDER=132 or ORDER=123 are

both reasonably acceptable.  When using the stylus, only

a single button is available so this is normally button 1.

See the note under MOUSE=BP, since the only difference is

the mode the tablet is jumpered to use the resolution

problem is likely to exist when using MOUSE=CR as well.

The digitizer must have physical jumpers changed to use

this mode.  Support for the factory default is provided

by MOUSE=BP.

MOUSE=D3

CalComp 3300 and newer digitizers -- this is an "absolute"

pointing device.  Each point on the digitizer table corresponds

exactly to a point on the screen.  This is similar to "touch"

screens in operation.  This feature can be awkward to those not

used to it, but may be preferred by those who use a digitizer

tablet on a regular basis.  This option is required for these

digitizers, they cannot be automagically recognized.

Protocols: HA, HR

default: HA

Baud Rate: 150, 300, 600, 1200, 2400, 4800, 9600, 19200

default: 9600

Note: The PRESSURE option may be used with this device.  The

value must be in the (inclusive) range of 1 to 255.

Note: This type mouse cannot be automagically recognized, so

this parameter is required for such mice.  CalComp has

many different styli and pucks available so the use of

the ORDER option is almost essential.  For the 4-button

puck arranged in a diamond, ORDER=2143 is recommended.

For the styli with a tip and double-action button, the

default of ORDER=123 has the tip has the "left" button.

This is much more subject to personal preference so no

recommendation will be made.  There are many other pucks

available from CalComp and trial and error is the only

guide to the best ordering for those.

MOUSE=D9

CalComp 9x00 digitizers -- this is an "absolute" pointing device.

Each point on the digitizer table corresponds exactly to a point

on the screen.  This is similar to "touch" screens in operation.

This feature can be awkward to those not used to it, but may be

preferred by those who use a digitizer tablet on a regular basis.

The 9x00 digitizers are older products which do not support full

software initialization.  These tablets MUST be set for 9600

baud.  The 9100 model tablets must have the hardware switches

configured for format 5.  If reconfigured by other applications

the tablet must be reset before this driver will function.  This

option is required for these digitizers, they cannot be

automagically recognized.

Protocols: HA, HR

default: HA

Baud Rate: 9600

default: 9600

Note: This type mouse cannot be automagically recognized, so

this parameter is required for such mice.  CalComp has

many different styli and pucks available so the use of

the ORDER option is almost essential.

MOUSE=DW

CalComp WIZ and 2x00 digitizers -- these are "absolute" pointing

devices.  Each point on the digitizer table corresponds exactly

to a point on the screen.  This is similar to "touch" screens in

operation.  This feature can be awkward to those not used to it,

but may be preferred by those who use a digitizer tablet on a

regular basis.  The WIZ and 2x00 digitizers are older products

which do not support full software initialization.  These tablets

MUST be set for 9600 baud.  If reconfigured by other applications

the tablet must be reset to 9600 baud before this driver will

function.  This option is required for these digitizers, they

cannot be automagically recognized.

Protocols: HA, HR

default: HA

Baud Rate: 9600

default: 9600

Note: This type mouse cannot be automagically recognized, so

this parameter is required for such mice.  CalComp has

many different styli and pucks available so the use of

the ORDER option is almost essential.  For the 4-button

puck arranged in a row, ORDER=1234 is recommended.

MOUSE=FX

Altra's Felix mouse.  This is an "absolute" pointing device.

Each position of the mouse "handle" corresponds exactly to a

point on the screen.

Protocols: FA, FR

default: FA

Baud Rate: 9600

default: 9600

Note: This type of mouse allows different baud rates to be set

but the commands used are inconsistent between different

mice with the same internal version number depending on

the cystal used during manufacture of the mouse.  Since

this mouse can function in "delta" mode only the hardware

default baud rate is allowed because this can be set

consistently.

MOUSE=I

Inport buss mouse, can be two button or three button.  Note

that an Inport mouse is NOT the same as a buss mouse.

Protocols: IN

default: IN

Rate: 30, 50, 100, 200

default: 100

MOUSE=KR

Kurta digitizers, these are clones of the Summagraphic's

digitizers but fail to support the master reset and all of

the "z" commands.  This is an "absolute" pointing device.

Each point on the digitizer tablet corresponds exactly to a

point on the screen.  This is similar to "touch" screens in

operation.  This feature can be awkward to those not used

to it, but may be preferred by those who use a digitizer

tablet on a regular basis.  This option is required for

these digitizers, they cannot be automagically recognized.

Protocols: MA, MM, MR

default: MA

Baud Rate: 9600

default: 9600

Note: This type mouse cannot be automagically recognized, so

this parameter is required for such mice.  Further, if

the stylus is used then "ORDER=312" is recommended as

well.  The default "ORDER=123" is suggested for the

4-button puck.

MOUSE=M

Microsoft compatible serial 2 button mouse, non-programmable

Protocols: MI

default: MI

Baud Rate: 1200

default: 1200

MOUSE=MS

Mouse System's compatible serial 3 button mouse

Protocols: 5B

default: 5B

Baud Rate: 1200

default: 1200

Note: This type mouse cannot be automagically recognized, so

this parameter is required for such mice.

MOUSE=MW

Logitech's Microsoft/Mouse System compatible serial 3 button

mouse

Protocols: MI, MP, 5B

default: MI

Baud Rate: 1200, 9600

default: 1200

MOUSE=NM

Provides "no mouse" support.  This can be used to tell OS/2

that a mouse is present when one is not actually present but

it is necessary for OS/2 to provide high-level mouse support

that would be omitted if MOUSE.SYS and VMOUSE.SYS were simply

omitted from CONFIG.SYS.

Protocols: NM

default: NM

Note: In order to provide DOS mouse support when using this

option, the IRQ option must be used as well because

VMOUSE.SYS will not load due to the fact that it assumes

that the mouse actually owns an interrupt.  If the IRQ

option is used, it may specify any free interrupt.  If

the IRQ option is not used then VMOUSE.SYS will not load

and should therefore be removed from the CONFIG.SYS file.

MOUSE=PS

PS/2 port compatible mouse

Protocols: PS

Rate: 10, 20, 40, 60, 80, 100, 200

default: 100

MOUSE=S

This causes RODENT.SYS to only check for serial mice.  Any

other mice are ignored.  This could be useful if a buss mouse

interface is present, but the mouse itself is not present.

This option is supplied because some motherboards and video

adapters have a buss or Inport mouse interface built-in which

should be ignored.

MOUSE=SG

Summagraphics digitizers -- this is an "absolute" pointing

device.  Each point on the digitizer tablet corresponds exactly

to a point on the screen.  This is similar to "touch" screens

in operation.  This feature can be awkward to those not used

to it, but may be preferred by those who use a digitizer

tablet on a regular basis.  This option is required for these

digitizers, they cannot be automagically recognized.

Protocols: MA, MM, MR, UA, UR

default: MA

Baud Rate: 9600

default: 9600

Note: This type mouse cannot be automagically recognized, so

this parameter is required for such mice.  Further, if

the stylus is used then "ORDER=132" is recommended as

well.  If the 16-button puck is used with the MA, MM or

MR protocols then "ORDER=213" is recommended.  The default

"ORDER=123" is suggested for the 4-button puck when using

the MA, MM or MR protocols and for the 16-button puck used

with the UA and UR protocols.  For the 4-button puck used

with the UA or UR protocol, ORDER=2143 is recommended.

Note: Some Summagraphics compatible tablets do not handle the

master reset correctly.  These tablets should use MOUSE=SM

if a 4-button puck or a stylus is attached to the tablet

and should use MOUSE=SU if a 16-button puck is attached to

the tablet.  If used with MOUSE=SG the tablet may not

work correctly and in some cases will lock up.

MOUSE=SM

Summagraphics digitizers which have the 4-button puck or stylus

attached and which do not recognize the master reset.  This is

an "absolute" pointing device.  Each point on the digitizer

tablet corresponds exactly to a point on the screen.  This is

similar to "touch" screens in operation.  This feature can be

awkward to those not used to it, but may be preferred by those

who use a digitizer tablet on a regular basis.  This option is

required for these digitizers, they cannot be automagically

recognized.  This option cannot be used for tablets which have

the 16-button puck attached.

Protocols: MA, MM, MR

default: MA

Baud Rate: 9600

default: 9600

Note: This type mouse cannot be automagically recognized, so

this parameter is required for such mice.  Further, if

the stylus is used then "ORDER=132" is recommended as

well.  The default "ORDER=123" is suggested for the

4-button puck.

MOUSE=SU

Summagraphics digitizers which have the 16-button puck attached

and which do not recognize the master reset.  This is an

"absolute" pointing device.  Each point on the digitizer tablet

corresponds exactly to a point on the screen.  This is similar

to "touch" screens in operation.  This feature can be awkward to

those not used to it, but may be preferred by those who use a

digitizer tablet on a regular basis.  This option is required

for these digitizers, they cannot be automagically recognized.

This option cannot be used for tablets which have either the

stylus or the 4-button puck attached.

Protocols: UA, UR

default: UA

Baud Rate: 9600

default: 9600

Note: This type mouse cannot be automagically recognized, so

this parameter is required for such mice.  The default

"ORDER=123" is suggested.

MOUSE=SX

Summagraphics FX digitizers -- this is an "absolute" pointing

device.  Each point on the digitizer tablet corresponds exactly

to a point on the screen.  This is similar to "touch" screens

in operation.  This feature can be awkward to those not used

to it, but may be preferred by those who use a digitizer

tablet on a regular basis.  This option is required for these

digitizers, they cannot be automagically recognized.

Protocols: MA, MM, MR, UA, UR

default: MA

Baud Rate: 9600

default: 9600

Note: The PRESSURE option may be used with this device.  The

value must be in the (inclusive) range of 0 to 99.

Note: This type mouse cannot be automagically recognized, so

this parameter is required for such mice.  It is always

assumed that three buttons are present because the pucks

are (optionally) cordless and can be changed without the

knowledge of RODENT.

In fact, the stylus has a tip and two buttons...but one

of those buttons may generate a chord click in hardware

(depending on the protocol) and so in that case only two

buttons are available.

If the stylus is used then "ORDER=132" is recommended.

If the 16-button puck is used with the MA, MM or MR

protocols then "ORDER=213" is recommended.  The default

"ORDER=123" is suggested for the 4-button puck when using

the MA, MM or MR protocols and for the 16-button puck used

with the UA and UR protocols.  For the 4-button puck used

with the UA or UR protocol, ORDER=2143 is recommended.

Note: These Summagraphics tablets use different pucks than the

older tablets.  All stylus and pucks have three or more

physical buttons.  If these tablets are used with

MOUSE=SG, MOUSE=SM or MOUSE=SU then the correct puck

may not be recognized.

MOUSE=V

Logitech compatible serial 3 button mouse, non-programmable.

If the MI protocol is used then the middle mouse button cannot

be programmed as a chord.

Protocols: MI, MP

default: MP

Baud Rate: 1200

default: 1200

MOUSE=VW

Logitech's Microsoft/Mouse System compatible serial 3 button

mouse

Protocols: MI, MP, 5B

default: MP

Baud Rate: 1200, 9600

default: 1200

MOUSE=UN

MicroTouch's UnMouse protocol

Protocols: TA, TR

default: TA

Baud Rate: 9600

default: 9600

Note: The PRESSURE option may be used with this device.  The

value must be in the (inclusive) range of 0 to 99.

Note: The FILTER and NOISE options may be used with this device.

Some older UnMouse devices do not handle the FILTER command

properly, in which case the FILTER command will be accepted

but will have no affect.

Note: RODENT requires the UnMouse to be set to 9600 baud and

either 7 data bits, no parity and 2 stop bits or 8 data

bits, no parity and 1 stop bit.  When using a large blunt

object (such as a finger) to touch the UnMouse using the

MARGIN command may be needed to allow the cursor to move

all points on the screen.  Values of MARGIN=64 or

MARGIN=80 are reasonable.

MOUSE=*

Unknown mouse.  This option would be used when the mouse is

known to communicate with a given protocol, but the mouse type

cannot be recognized and the mouse cannot be programmed.  In

all cases, the PROTOCOL=xx and either COM=# or PORT=# IRQ=#

must also be present.  This is NOT checked for and failure

to provide these options will prevent the mouse from being

properly initialized.  In some cases it can also cause the

system to lockup requiring a reboot.  If an absolute protocol

is in use then the X=# and Y=# options must also be used.

NAME=xx..xx

This option specifies the name of the device driver.  It defaults to

"RODENT", but may be set to any desired name.  The specified name must

be 1 to 7 characters in length and must be a valid filename component.

This latter constraint is not checked since the names considered valid

by OS/2 is subject to change.  However, if RODENT fails to install and

an unusual name is being specified then try changing the name or

omitting the option entirely and letting RODENT use its default name.

NOISE=#

This option is only effective for devices which can configure the

device to adjust sensitivity to noise.  The acceptable values are

device dependent.

MOUSE=UN:

The noise parameter adjusts the frequency at which the device

operates (thereby affecting its noise senstivity in a particular

environment).  This must in the range of 15 to 45, inclusive.

ORDER=#

This option specifies the button ordering.  The default button ordering

is usually adequate for all mice.  However, digitizer pucks tend to be

inconsistent between logical button numbering and physical button

placement and this option is frequently required for a digitizer.

Further, while OS/2 allows the left and right buttons to be swapped

for left hand users, it doesn't do so correctly for three button mice

and the swapping does not function at all for DOS and OS/2 full screen

sessions.  Trailing digits may be omitted if they are in order.  At

least one digit must be specified.  This mouse driver supports from 2

to 7 physical buttons. Any buttons above button 7 will be ignored.

This option may be specified as:

ORDER=1234567

where each digit is the button to be used for the corresponding button

position.  Omitted digit positions which be assigned their position

index.  It is an error to specify the same button twice, or to fail to

specify a button.  Thus:

ORDER=1

ORDER=12

ORDER=123

ORDER=1234

ORDER=12345

ORDER=123456

ORDER=1234567

all result in the default button ordering.  But:

ORDER=122

ORDER=124

are both errors.  And:

ORDER=7654321

reverses the button ordering.  So button 1 will be treated the same as

button 7, button 2 the same as button 6, and so on.  The most common

button orderings for mice are:

ORDER=321

which reverses the left and right buttons, leaving the middle button

unaffected.  Also:

ORDER=132

which reverses the middle and right buttons.  Thus OS/2 button 1

is physical button 1, OS/2 button 2 is physical button 2 and OS/2

button 3 is physical button 3.

One useful ordering for some digitizer pucks arranged in a "diamond"

ordering is:

ORDER=2143

For mice, the left mouse button is always physical button 1, the right

mouse button always physical button 3 and the middle mouse button (if

present) is always physical button 2.

The general button assignment strategy for two button mice is...

Button 1 -- OS/2 button 1 (left button down)

Button 2 -- OS/2 button 1+2 (all buttons down)

Button 3 -- OS/2 button 2 (right button down)

Button 4 -- OS/2 button 1+2 (all buttons down)

Button 5 -- OS/2 button 1+2 (all buttons down)

Button 6 -- OS/2 button 1+2 (all buttons down)

Button 7 -- OS/2 button 1+2 (all buttons down)

Notice that for two button mice no more than 3 buttons can be

profitably used for OS/2.  The general button assignment strategy for

three button mice is...

Button 1 -- OS/2 button 1 (left button down)

Button 2 -- OS/2 button 3 (middle button down)

Button 3 -- OS/2 button 2 (right button down)

Button 4 -- OS/2 button 1+2 chord (left + right buttons down)

Button 5 -- OS/2 button 1+3 chord (left + middle buttons down)

Button 6 -- OS/2 button 2+3 chord (middle + right buttons down)

Button 7 -- OS/2 button 1+2+3 chord (all buttons down)

While different chord clicks are generated, OS/2 recognizes all

multiple button presses as a chord click so any program that wishes

to distinguish between chord clicks must examine the primitive button

event messages.

Notice that for three button mice no more than 7 buttons can be

profitably used for OS/2.  Also, many digitizer pucks have the

limitation that only one button can be pressed at a time and there

are both mice and digitizer pucks which have buttons hard-wired to

generate chord clicks of other buttons and where multiple presses

of buttons may result in the appearance of some other button being

pressed.  These problems cannot be resolved by this driver, but the

ORDER option can allow the best setup for a particular mouse or puck.

To obtain the best ordering some experimentation may be necessary.

Note: Even if a mouse or puck only has two buttons, it may be necessary

to use an option such as ORDER=132 because sometimes the second

button may be assigned to a higher button number.  Usually

nothing higher than ORDER=1234 is needed unless there are more

than four buttons.

PRESSURE=#

This option is only effective for devices with pressure sensitive

buttons such as digitizer pens and touch screens.  This option sets

the pressure threshold at which such a device recognizes a button

press.  The acceptable values are device dependent.

MOUSE=D3:

The pressure must be in the range of 1 to 255, inclusive.  The

default, and value recommended by CalComp for their pressure

pens is 4.  The larger the number the more pressure is required.

MOUSE=SX:

The pressure must be in the range of 0 to 99, inclusive.  The

default is 5.  The larger the number the more pressure is required.

MOUSE=UN:

The pressure must in the range of 0 to 3, inclusive.  The default

is 0.  The larger the number the more pressure is required.

PORT=#

This option allows the serial port address to explicitly specified,

and may be specified as:

PORT=xxxx

where "xxxx" is the hex address of the port to be used.  If the COM

option is also specified, this option takes precedence.

Any serial port which is compatible with the 8250 uart may be used.

This mouse driver is not restricted to COM1..COM2, unlike the IBM

mouse drivers.  If both this option and the COM option are omitted

then the mouse driver will scan COM1..COM4 (in order) to find the

mouse.  This procedure works and is reasonably reliable, however it

is impossible to completely restore the uart state so automatic

scanning could potentially cause problems with other devices attached

to serial ports.  This mouse driver will restore more of the serial

port state than the IBM mouse drivers.  Specifying this option also

reduces the time required to initialize the mouse driver -- by as

much as a couple of seconds.

PROTOCOL=xx

This option allows the communications protocol to be specified.  This

is useful to select a non-default protocol and when using the MOUSE=*

option.  If the mouse type is known and an unsupported protocol is

selected the default protocol for that mouse type will be used.  This

option may be specified as:

PROTOCOL=3B

Three byte binary protocol -- this is an experimental protocol

which MAY work with some older mice.  If used then MOUSE=* must

also be used.  This protocol will work instead of the 5B protocol

but the mouse will "slip".  It also supports either a 2 button

or 3 button mouse.  Since this is an experimental protocol, the

reward does NOT apply to its use.

PROTOCOL=5B

Five byte binary protocol.  It is the default for Mouse System's

compatible mice.  This protocol supports either a 2 button or 3

button mouse.

PROTOCOL=BA

Summagraphics Bit Pad One/Two protocol.  This is the default

protocol for the Summagraphics Bit Pad series.  This is NOT

the same as the RE protocol (which is also called a Bit Pad

protocol even though it has no relationship with the Bit Pad

digitizers).  This is an "absolute" protocol.

PROTOCOL=BR

Summagraphics Bit Pad One/Two protocol.  This is NOT the same

as the RE protocol (which is also called a Bit Pad protocol even

though it has no relationship with the Bit Pad digitizers).  This

is a "relative" protocol.

PROTOCOL=BS

Buss mouse protocol, supports either 2 or 3 buttons.  This

protocol is used for buss mice.  Normally using MOUSE=B is

preferred to this option, but MOUSE=* PROTOCOL=BS is allowed.

PROTOCOL=CA

Summagraphics Bit Pad CR protocol.  This is an "absolute"

protocol.

PROTOCOL=CR

Summagraphics Bit CR protocol.  This is a "relative" protocol.

PROTOCOL=FA

Felix mouse protocol.  This is an "absolute" protocol, i.e. each

position of the mouse corresponds exactly to a point on the

screen.

PROTOCOL=FR

Felix mouse protocol.

PROTOCOL=HA

CalComp high resolution binary digitizer protocol, supports

either the 4 button puck (which may actually have 6 buttons, but

two of the buttons are hardware-wired as chords of the remaining

buttons) or the 16 button puck.  Pens are also supported, but

obtaining a "chord" click can be difficult if the middle button

is not programmed to generate a chord.  Pressure pens may also

have difficultly generating a double click for the tip.  OS/2

only supports three buttons so extra buttons are mapped to chord

clicks.  This protocol is normally used with digitizers.  This

is an "absolute" protocol, i.e. each position on the digitizer

tablet corresponds exactly to a point on the screen.

PROTOCOL=HR

CalComp high resolution binary digitizer protocol, supports

either the 4 button puck (which may actually have 6 buttons, but

two of the buttons are hardware-wired as chords of the remaining

buttons) or the 16 button puck.  Pens are also supported, but

obtaining a "chord" click can be difficult if the middle button

is not programmed to generate a chord.  Pressure pens may also

have difficultly generating a double click for the tip.  OS/2

only supports three buttons so extra buttons are mapped to chord

clicks.  This protocol is normally used with digitizers.

PROTOCOL=IN

Inport buss mouse protocol, supports either 2 or 3 buttons.

This protocol is used for Inport buss mice.  Normally using

MOUSE=I is preferred to this option, but MOUSE=* PROTOCOL=IN

is allowed.

PROTOCOL=MA

Summagraphics digitizer protocol.  OS/2 only supports three

buttons so extra buttons are mapped to chord clicks.  This

protocol is normally used with digitizers.  This is an

"absolute" protocol, i.e. each position on the digitizer

tablet corresponds exactly to a point on the screen.

PROTOCOL=MI

Microsoft compatible 2 button protocol.  Can be used with a 3

button mouse, but completely ignores the middle button.

PROTOCOL=MM

Mouse System's compatible protocol, supports either 2 or 3

buttons.  Also available on Summagraphics digitizer tablets.

PROTOCOL=MP

Extension to MI protocol which supports either 2 or 3 buttons.

If only two buttons are present the MI protocol is more

efficient.

PROTOCOL=MR

Summagraphics digitizer protocol.  OS/2 only supports three

buttons so extra buttons are mapped to chord clicks.  This

protocol is normally used with digitizers.

PROTOCOL=NM

Provides "no mouse" support.  This option may not be specified

by the user, this protocol is automatically provided when the

MOUSE=NM option is specified.

PROTOCOL=RE

Relative Bit Pad One protocol, supports either 2 or 3 buttons.

In spite of the name, this is NOT a protocol used by Bit Pad

One tablets.  It is used by Logitech Type C mice.

PROTOCOL=PS

PS/2 port protocol, supports either 2 or 3 buttons.

PROTOCOL=TA

MicroTouch's UnMouse tablet protocol.  This is an

"absolute" protocol, i.e. each position on the tablet

tablet corresponds exactly to a point on the screen.

PROTOCOL=TR

MicroTouch's UnMouse tablet protocol.

PROTOCOL=UA

Summagraphics UIOF digitizer protocol.  OS/2 only supports

three buttons so extra buttons are mapped to chord clicks.

This protocol is normally used with digitizers.  This is an

"absolute" protocol, i.e. each position on the digitizer

tablet corresponds exactly to a point on the screen.

PROTOCOL=UR

Summagraphics UIOF digitizer protocol.  OS/2 only supports

three buttons so extra buttons are mapped to chord clicks.

This protocol is normally used with digitizers.

RATE=#

This option allows the reporting rate to be specified for those mice

which accept this option (currently only Inport mice and PS/2 mice).

It may be one of:

10, 20, 30, 40, 50, 60, 80, 100 or 200

All mice do not support all reporting rates.  To use the lowest

reporting rate supported by the mouse specify:

RATE=10

and to use the highest reporting rate supported by the mouse specify:

RATE=200.

The lowest reporting rate which gives good performance should be used

to reduce system load.  If this option is not specified then it

defaults to a value determined by the protocol in use.  Each protocol

has a default reporting rate based on the default mouse reporting rate.

If a reporting rate is specified which is not supported by the protocol

the highest baud rate which is less than the specified rate will be

used, if any, and otherwise the lowest rate which is higher than the

specified rate will be used.  For some mice the reporting rate is set

via hardware jumper.

RESPONSE=#

This option specifies a divisor to increase or reduce the mouse

responsiveness.  If omitted it defaults to 0.  It may be specified as:

RESPONSE=-8

RESPONSE=-4

RESPONSE=-2

RESPONSE=-1

RESPONSE=0

RESPONSE=1

RESPONSE=2

RESPONSE=4

RESPONSE=8

and only these values may be used.  If the value is negative then the

responsiveness will be reduced by the given factor, and if positive

increased by the given factor.  If zero then there will be no change.

Since increasing or decreasing the responsiveness by a factor of one

has no effect these values are equivalent to zero.

This can be useful for very low or very high resolution mice (some

digitizers are 1000 dpi which is FAR too sensitive for most people).

Further, if a user has difficultly holding the mouse steady (such as

Parkinson's victims) this allows the mouse to ignore small movements.

Caution: with this option it is possible to slowly move the mouse

without moving the mouse cursor at all.  This effect does NOT happen

for RESPONSE=-1 or any non-negative value and would be most noticeable

for RESPONSE=-8.  Thus, RESPONSE=-8 would effectively make a 200 dpi

mouse into a 25 dpi mouse.  As for the ORDER option, OS/2 DOES provide

a mouse sensitivity control, but that control is not effective for DOS

or OS/2 full screen sessions and frequently the available range is

insufficient.

This option does not have an effect on absolute protocols, with the

exception of the Felix mouse, since those protocols map mouse positions

directly to screen positions.

For the Felix mouse a response value of 2, 4 or 8 may be used.  This

has the effect of multiplying the hardware resolution by that factor.

The mouse will report a position in a "window" where the size of the

window is 1/2, 1/4 or 1/8 of the full mouse motion.  This means that

the resolution inside this window is 2, 4 or 8 times higher than

otherwise.  The "position" of the window may be altered by moving the

mouse to the edge of the tablet into the margin.  The speed of motion

is then controlled by how far the mouse is moved into the margin.  The

default margin in this case is 4 instead of 0.

The operation of this feature is analogous to the operation of a

virtual desktop which is larger than the physical screen.

Since the "natural" action is to move to the extreme edge of the

tablet, to control how fast the window moves change the effective

margin.  Larger values will move the window faster.

The Felix has the characteristic that the last bit or so of maximum

travel is "iffy".  The default tablet dimensions are: X=640 Y=640, but

if the window moves faster in one direction or another then increase

or decrease the approriate value for that dimension.  Increasing the

value will cause the motion to faster in one direction and decreasing

the value will slow the motion in that direction.  The other direction

is not affected because zero is not "iffy".

This option is not as useful for the newer Felix mice due to their

increased resolution unless very high screen resolutions are in use.

REVERSEX

Reverses movement in the X direction.

Note: The REVERSEX and REVERSEY options are applied to the physical

coordinates and therefore BEFORE the FLIPXY option, so when

the FLIPXY option is in effect use REVERSEX instead of REVERSEY

and vice-versa.

REVERSEY

Reverses movement in the Y direction.

Note: The REVERSEX and REVERSEY options are applied to the physical

coordinates and therefore BEFORE the FLIPXY option, so when

the FLIPXY option is in effect use REVERSEX instead of REVERSEY

and vice-versa.

SHARE

If the SHARE option is present then RODENT will attempt to share the

irq if permitted by OS/2.  Otherwise, RODENT requests exclusive

ownership of the irq.  This option does not allow another program to

control the mouse (or digitizer) at the same time as RODENT, but

instead allows a different port to share the irq.  This is normally

only possible on PS/2 and EISA systems.

UART=#

This option allows the uart type to be specified.  It may be specified

as...

UART=8250

UART=16450

UART=16550

UART=16550A

UART=16552

If this option is omitted, the uart type will be determined by testing

the uart.  All uart testing will be skipped if this option is present.

This option is provided because some mice are sensitive to testing for

the uart type.  This sensitivity is very rare and probably also

depends on the design of the serial port.  This option should not be

specified unless RODENT.SYS fails to work properly.  The symptoms of

the only known occurrence of this problem was that RODENT.SYS would

load but the mouse pointer would not move.  If this option is specified

then either the COM option or the PORT and IRQ options must also be

specified.

X=[#..]#

This option specifies the active horizontal range for a digitizer

tablet.  It may be specified as:

X=#

X=#..#

In the first case, the active range is assumed to be 0..#-1 and in the

second case the active range is #1..#2-1.  Thus, X=7500 would result

in an active range of 0 to 7499, inclusive.  And X=500..7000 would

result in an active range of 500 to 6999, inclusive.  If "..#" is

present, spaces may NOT surround "..".  However, spaces MAY surround

"=" as in all other options.

This option is only necessary when it is not possible to determine

the digitizer size automatically or when it is not desired to use the

entire digitizer surface as the active mouse area.  This option can be

used to create nonsymmetrical margins.  Normally the MARGIN parameter

is used to specify symmetrical margins.  The MARGIN value is added to

the minimum and subtracted from the maximum horizontal range.  However,

if the digitizer tablet's physically active area was X=0..7500 then

specifying X=1000..4000 would give a left (or right) margin of 1000

and a right (or left) margin of 3500 (some tablets may physically

reverse left and right).

Y=[#..]#

This option specifies the active vertical range for a digitizer

tablet.  It may be specified as:

Y=#

Y=#..#

In the first case, the active range is assumed to be 0..#-1 and in the

second case the active range is #1..#2-1.  Thus, Y=7500 would result

in an active range of 0 to 7499, inclusive.  And Y=500..7000 would

result in an active range of 500 to 6999, inclusive.  If "..#" is

present, spaces may NOT surround "..".  However, spaces MAY surround

"=" as in all other options.

This option is only necessary when it is not possible to determine

the digitizer size automatically or when it is not desired to use the

entire digitizer surface as the active mouse area.  This option can be

used to create nonsymmetrical margins.  Normally the MARGIN parameter

is used to specify symmetrical margins.  The MARGIN value is added to

the minimum and subtracted from the maximum horizontal range.  However,

if the digitizer tablet's physically active area was Y=0..7500 then

specifying Y=1000..4000 would give a bottom (or top) margin of 1000

and a top (or bottom) margin of 3500 (some tablets may physically

reverse up and down).

-------------------------------- Testimonials --------------------------------

This section is not present to "sell" you on RODENT, but this quote has

been selected from quite a few to illustrate that RODENT is more efficient

than the standard IBM mouse drivers, the difference is not necessarily

noticable on a fast system, but RODENT really shows its colors when OS/2 is

pressed.

RODENT.SYS (with a Logitech at 400 DPI) is a real pleasure to use.  Due

to a bad SIMM for the last few days I have gone from 20Megs RAM to 4Megs.

2.1 is limping, no, crawling along, but is very visibly more responsive

than with the IBM driver.  You may quote me. -- Allen Koppelman

--------------------------------- What's New ---------------------------------

Release  ..Date..  ..........................Changes..........................

1.0.13  96/10/13  Felix mouse recognition sequence timing was altered for

absolute mode because of hardware changes in new devices.

1.0.12  95/12/06  Felix mouse now has three buttons.  Support added for

third button.  Default button order also changed.  Older

Felix mice still work correctly, but will report three

buttons instead of two buttons because the driver can't

tell the difference.  Felix mice now require MOUSE=FX to

operate in absolute mode.

1.0.11  95/05/14  Beta version 1.01 introduced a bug into the InPort mouse

handler which prevented motion which was exactly and

only in the X dimension from being reported properly.

1.0.10  94/08/14  Auto-recognition code completely reorganized and

rewritten.  Recognition support added for Mouse System's

2D/3D mice and other newer mice and additional logic

added to help recognize Logitech type C mice.

1.0.9   94/07/31  Support for AceCat digitizers added.  Also numbering

scheme added a decimal point so that more than 9 minor

revisions.  Thus x.y.z means release x, major revision

y and minor revision z.

1.08   94/06/21  Support for the Summagraphics BitPad digitizer added.

UnMouse support was "tweaked" because of compatability

problems with older devices.  Support for "no mouse"

added.  Support for the AceCat II added.

1.07   94/04/27  Support for the Summagraphics FX digitizer and for the

UnMouse added.  The FILTER and NOISE options added.

1.06   94/03/11  The NAME option was added to allow RODENT to alter its

device name to avoid conflicts with other device drivers.

1.05   94/01/23  The default dimensions of the Felix mouse has been

changed from 328 to 640 to accomodate the newer Felix

mice.  The older mice will, after a short transition

period, no longer be sold.  However, the older Felix

mice can still be used by simply using the options

"X=328 Y=328" which will restore the defaults needed

for the older mice.  Serious consideration was given to

creating an entirely separate mouse type, but this only

adds space and time complexity to the code with very few

benefits.  The same considerations caused the rejection

of the option of autodetecting the actual dimensions of

the Felix mouse becuase that would have increased the

space and time complexity of the Felix interrupt handler.

1.04   93/11/19  The RESPONSE option now allows increased resolution

for the Felix mouse in absolute mode.

1.03   93/11/15  The SHARE option added to control irq sharing.  Also

support for Kurta digitizers added.  These are clones

of the Summagraphic's digitizers but fail to support

the full command set.

1.02   93/10/10  New options added to support the UIOF protocol for

Summagraphics digitizer tablets.  Support added for

older Summagraphics tablets which lock up when sent

the master reset.

1.01   93/09/03  The REVERSEX, REVERSEY and FLIPXY options were added.

An undocumented timing dependency was uncovered which

affected initialization of some older Logitech mice.

Some older Summagraphics tablets fail to handle the

master reset properly.  An additional soft reset was

added to allow those tablets to be used.  This also

affected some Summagraphics compatible tablets.

An error in the RESPONSE logic was corrected.  This bug

could cause the system to crash when loading or cause

Lan Server to fail to install correctly.  This bug

was only encountered when RESPONSE=2, RESPONSE=4 or

RESPONSE=8 was used.

An error in the Inport interrupt routine was corrected.

This bug would allow the mouse buttons to be handled

properly, but potentially ignored mouse movement.

1.00   93/07/12  First general availability release.  Support added

for PS/2 mice, Inport mice, Felix mice, Summagraphics

digitizers and CalComp digitizers.  Many new PROTOCOL

options added.  The PORT, IRQ, RESPONSE, ORDER, UART,

RATE, X, Y, MARGIN and PRESSURE options were added.

The COM option was split into the COM and PORT/IRQ

options to allow a consistent format for each option.

The MOUSE=X option was replaced by the MOUSE=* option

to avoid a conflict with the X option.

Initialization error fixed which caused the ordering of

statements in CONFIG.SYS to be important for systems

using Lan Server or NetWare.

Uart testing affected some sensitive mice.  This was

not a bug but caused some sensitive (older) mice to

lock up.  The uart testing was changed to reduce the

chances of this problem and the UART option was added

to assist with these mice by completely avoiding uart

testing.

0.90   93/04/07  Beta of initial release with digitizer support disabled.

-------------------------------- Coming Soon ---------------------------------

No explicit plans have been made at this time.  However, some new

features under consideration are...

Support for 3D/6D mice and the tilt/pressure features of digitizers.  This

requires the cooperation of IBM and is currently under consideration by

IBM, but until they arrive at (or accept) an extended mouse interface for

both RODENT and Presentation Manager these devices can only be supported

in the more limited 2D mode.

IBM has not made the protocol extensions available which are

needed for 3D/6D support.

A "drag" feature where clicking the middle button starts a right button

drag and clicking it again would "drop" the object being dragged.  This

feature, if implemented, would be an alternate use of the middle button

for the "chord" feature currently implemented.

I have gotten no requests for this feature.

An installation program (sending in the options you use will facilitate

this feature by providing the necessary information).

This will probably require a major rewrite.  It will be postponed

until the OS/2 32-bit device driver interface is released.

A Presentation Manager program to allow changing options "on the fly".

This will probably require a major rewrite.  It will be postponed

until the OS/2 32-bit device driver interface is released.

Possible support for more than three mouse buttons to "kick off" a program

or for a button to automatically generate a single or double click (these

features might require a Presentation Manager program to be active at all

times).

Using the middle button to generate a left button double click

turns out to be more difficult in the OS/2 device driver arena

than expected.  The appropriate location for such a feature is

in a Presentation Manager program that "hooks" the system queue.

Consequently, this feature is not likely to be implemented in

the near future.

Support for additional mice.

As they become available.

Better "hot plug" support.

This will probably require a major rewrite.  It will be postponed

until the OS/2 32-bit device driver interface is released.

However, none of these new features will be possible unless RODENT.SYS is

used by a sufficient number of people who register their copies.  So PLEASE

register!  If you have features you would like to see please send in your

suggestions when you register (or at any time, for that matter).

------------------------------------------------------------------------------