VEC2VIF

Space Shuttle state vector conversion utility

Conversion of Space Shuttle state vectors

(from sts##.vec file on GSFC OIG RBBS) to

format readable by VEC2TLE / V2KEP programs.

Copyright 1994 by Curtis Haase.

Use and Distribute Freely; No Modification.

PURPOSE

The accompanying software (VEC2VIF.EXE) was created to automate the

conversion of Space Shuttle state vectors from the NASA Goddard Space

Flight Center Orbital Information Group (GSFC OIG) bulletin board into

a format that can be read directly by Ken Ernandes' VEC2TLE or V2KEP

programs.  See the VEC2TLE documentation for information on accessing

the GSFC bulletin board.

The Space Shuttle state vectors posted on the GSFC bulletin board

are updated hourly during a mission, so they are often the most current

source of Shuttle tracking information.  These state vectors are found

in a file named sts##.vec (where ## is the mission designation number,

e.g., sts66.vec for vectors during the STS-66 mission).  The vectors

must be converted into standard two-line Keplerian elements before they

can be used by most satellite tracking software.  The easiest way to do

this is by using Ken Ernandes' VEC2TLE (or V2KEP) program.

Unfortunately, the state vectors in the sts##.vec file are not in a

format directly readable by VEC2TLE or V2KEP.  This meant that a text

editor had to be used to re-format the file each time, or the vector

data manually entered into VEC2TLE.  The VEC2VIF program was created

to perform this tedious conversion task automatically.

SETUP

Make sure that the default sts.vec file is in the same directory

as VEC2VIF.EXE.

INSTRUCTIONS

VEC2VIF can be operated in either of two modes: an interactive mode,

or a batch file mode.

(A)   INTERACTIVE MODE

Interactive mode is the usual mode of operation, where instructions

for program operation are entered via the keyboard.  The program

is started by entering "VEC2VIF" at the operating system prompt.  The

user is then prompted for four inputs:

(1) "Name of state vector file to read (default = sts.vec):"

Respond with the name of the GSFC OIG Shuttle vector

file that needs to be converted.  If you simply press ENTER,

VEC2VIF will look for a file named "sts.vec" and read it.

Note: the vector input file is not altered by VEC2VIF.

(2) "Convert all vectors in file, or latest one only?"

The two allowed values are:

1 = Convert latest vector only (the default)

2 = Convert all vectors in file.

The "latest vector only" option has been chosen as the

default so that VEC2TLE can have immediate access to the

newest state vector.  Because of the order of the vectors

in the GSFC sts##.vec file, selecting "all vectors" results

in the latest one being at the end of the VIF (requiring

repeated VEC2TLE conversion cycles to get to it, and making

it unavailable to V2KEP).  When "latest vector" is selected,

VEC2VIF will save the latest one, even if the sts##.vec file

vectors happen to be out of order.

(3) "Select format for VIF output:"

The two allowed values are:

1 = Single data item per line (the default)

2 = Two-line space delimited.

See the documentation with VEC2TLE for a discussion

of VIF (vector information file) formats.  The "single

data item per line" format has been chosen as the default

because it supplies more information to VEC2TLE, and is

the format required by V2KEP.

Note:  The GSFC sts##.vec file does not contain all the

information required for a complete "single data item per

line" VIF file.  In particular, the drag parameters (ndot/2,

nddt/6, and Bstar), elset #, and Rev @ Epoch are not given.

VEC2VIF places "-1" in these fields, which signals VEC2TLE

to estimate the value of the parameter (except for the

elset#, which is set equal to the count in the vector file).

Since a launch time/date is required in order to estimate

the orbit count, VEC2TLE will prompt the user for these

values if a launch time/date (LTD) file entry is not

present.  This needs to be done only once for any given

shuttle mission.

(4) "Name for VIF output file (default = sts.vif):"

Specify the name of the file you want to contain the

re-formated state vectors.  If you simply press ENTER,

the output is written to the file "sts.vif".

WARNING: VEC2VIF will overwrite an existing file

with the new VIF data.  Any existing file contents

are erased (rather than appended to) by the new data.

This is done so that outdated vectors do not clutter

up the VIF files.  VEC2VIF will issue a warning before

it does this (allowing the user to select a different

VIF output filename if desired).  In addition, the filename

extensions .exe, .com, .bat, .sys, .ini, and .doc are

not allowed.  This restriction on the VIF output filename

helps ensure that critical files will not be accidentally

overwritten.

Finally, it is suggested that you always give the VIF output

filename a ".vif" extension since this makes it easier for

VEC2TLE to locate the file.

When all the inputs have been supplied, this message follows:

"Press any key to begin processing vector file... (or ESC to quit)"

Press a key, and VEC2VIF will convert the vectors (displaying

the epoch of each) and write the new VIF file.  The program

will then automatically end.

****************************************************************************

*      Suggestion for easiest operation:  Make a copy of the current       *

*      sts##.vec file and name it "sts.vec".  Make sure it is in the       *

*      same directory as VEC2VIF.  Start up VEC2VIF and accept default     *

*      values for all the inputs, i.e., just keep pressing ENTER.  The     *

*      latest Shuttle state vector will then automatically be written      *

*      to the file "sts.vif" (replacing any previous sts.vif entries).     *

****************************************************************************

(B)   BATCH MODE

Batch mode is used when it is desired to completely automate the

operation of VEC2VIF by running it from a batch file.  In this mode

the necessary inputs are obtained from the command line rather than

the keyboard.  There are four command line parameters, corresponding

to the four input prompt responses in interactive mode.   Note that

VEC2VIF attempts to start up in batch mode any time the command line

is not blank.

Usage:      VEC2VIF inputfile [allvectors?] [format?] [outputfile]

inputfile = Name of file with GSFC OIG sts##.vec format requiring

conversion.  This is the first item on the command line, and as

such cannot be omitted.  The default vector input file (sts.vec)

can be specified by an asterisk (*) for this parameter.  if an

invalid filename is supplied, VEC2VIF will revert to interactive

mode and prompt the user for instructions.

[allvectors?] = same as second input prompt in interactive mode.

The two allowed values are:

1 = Convert latest vector only (the default)

2 = Convert all vectors in file.

An asterisk (*) (or any non-allowed value) signals default.

[format?] = same as third input prompt in interactive mode.

The two allowed values are:

1 = Single data item per line (the default)

2 = Two-line space delimited.

An asterisk (*) (or any non-allowed value) signals default.

[outputfile] = Name for VIF output file.

If this parameter is omitted, or if an asterisk (*)

or invalid filename is used, VEC2VIF will default to

using "sts.vif" as the output file.

Remember that VEC2VIF will overwrite an existing file.

VIF output file names with the extensions .exe, .com,

.bat, .sys, .ini, and .doc are not allowed.

If any command line parameter (other than the first one) is missing,

the default values will be used.  For example,

VEC2VIF *

would command VEC2VIF to start up in batch mode using all defaults

(read sts.vec and write latest vector in single-line format to sts.vif).

Another example:

VEC2VIF sts66.vec 2 1 sts66.vif

commands VEC2VIF to start up in batch mode and convert ALL the

vectors in the sts66.vec file into single-line format and write them

to the file sts66.vif.

(C) USING CONVERT.BAT TO RUN VEC2VIF AND V2KEP

Those users who have V2KEP (a command-line version of VEC2TLE obtained

by registering as a user of Ken Ernandes' VEC2TLE software) can

completely automate the sts##.vec --> sts.vif --> two-line elements

conversion process.  CONVERT.BAT is a batch file included with VEC2VIF

that runs both VEC2VIF and V2KEP, producing two-line Keplerian elements

from the sts##.vec file.  CONVERT.BAT accepts command line parameters

as follows:

CONVERT [vecfile] [kepfile] [ltdfile]

where       vecfile = name of the GSFC OIG sts##.vec file

to be converted.  The default is "sts.vec".

kepfile = name of the file that V2KEP will write the

two-line Keplerian elements to.  The default is

the V2KEP default, i.e., "NNNNN.TLE", where NNNNN

is the Shuttle's satellite catalog number.

ltdfile = launch time/date file containing an entry for

the sts## mission.  The default is STSPLUS.LTD,

which is the same as the V2KEP default.

All of the command line parameters are optional.  If the command line

is blank ("CONVERT") then the default values indicated above are used.

(In this case it is assumed that you have already copied the sts##.vec

file to "sts.vec").  Note that the third command line parameter (the

ltdfile) defaults to STSPLUS.LTD, and so will rarely (if ever) be used.

An example of the usage of CONVERT.BAT follows:

CONVERT sts66.vec

instructs VEC2VIF to convert the sts66.vec file to VIF format, and then

instructs V2KEP to convert the VIF into two-line elements that will be

found in the default output file (in this case 23340.TLE for STS-66).

Another example:

CONVERT * shuttle.tle

instructs VEC2VIF to convert the default shuttle vector file (sts.vec)

and have V2KEP put the two-line elements in the file "shuttle.tle".

AUTHOR INFORMATION

Curtis Haase

2601 Hudnall #105

Dallas, Texas  75235

CompuServe address: 73007,2024