DEM data sources.doc

----------------

TerrainBase/ETOPO5, from the NOAA National Data Center/NGDC (18 MB uncompressed)

http://www.ngdc.noaa.gov/seg/fliers/se-1104.shtml

TerrainBase Global 5-min DTM Ver 1.0 (NGDC).  little_endian. 2160 rows x 4320 columns

The file may be thought of as having a logical record size of 8640 bytes. The data start at the North Pole (90 deg N, 0 deg 0' E) and are arranged in bands of 360 degrees x 12 points/degree = 4320 values (8640 bytes) ranging eastward from 0 deg 0' East longitude to 359 deg 55' East longitude (since it represents the North Pole, all possible longitudes still refer to a single point, thus the first band has 4320 identical values of -4290 m). The 8641st starts the latitude band for 89 deg 55' N, and so on. There is NO record for the South Pole (elevation 2810 m.)

ETOPO2, Smith, W. H. F. and D. T. Sandwell, NOAA/NGDC (133 MB uncompressed)

http://topex.ucsd.edu/marine_topo/mar_topo.html

http://www.ngdc.noaa.gov/mgg/fliers/01mgg04.html

2-minute (latitude-longitude) resolution

1 minute of latitude = 1 nautical mile, or 1.852 km).

(8)1350 x (4)1350 =10800 x 5400

A 6336 by 10800 grid of 2-byte integers = 136,857,600 bytes(16-bit signed integer ).

Byte order is big_endian. The Mercator projected image spans longitudes from 0 E to 360 E and latitudes from 72.006 N to -72.006 N.

"ETOPO2.dos" is in little-endian format for Intel (PC) and DEC (Vax) computers. The data are cell-centered, with an array size of 10800 columns x 5400 rows; coverage is from 180 West to 179 58' East and 90 North to 89 58' South. A data record for the South Pole is not in these files -- assume 2810m as the elevation.

GTOPO30 are regularly spaced at 30-arc seconds (approximately 1 kilometer). 33 smaller pieces, or tiles. Total size = 21,600 rows and 43,200 columns. Motorola byte order (most significant byte first) The global 16-bit DEM (21,600 rows by 43,200 columns) has a size of 1.74 gigabytes.

GTOPO30 is a global data set covering the full extent of latitude from 90 degrees south to 90 degrees north, and the full extent of longitude from 180 degrees west to 180 degrees east.  The horizontal grid spacing is 30-arc seconds (0.008333333333333 degrees), resulting in a DEM having dimensions of 21,600 rows and 43,200 columns.  The horizontal coordinate system is decimal degrees of latitude and longitude referenced to WGS84. The vertical units represent elevation in meters above mean sea level. The elevation values range from -407 to 8,752 meters.  In the DEM, ocean areas have been masked as "no data" and have been assigned a value of -9999.

GLOBE, "Global Land One-km Base Elevation" project (287 MB, expands to 1.7GB). http://www.ngdc.noaa.gov/seg/topo/globe.shtml

Bathymetry is not included in GLOBE Version 1.0. (See NGDC's TerrainBase or ETOPO5 for lower-resolution bathymetric data. GLOBE Version 2.0 may include bathymetric data.) Every tile contains values of -500 for oceans

21600 rows x 43200 columns  little-endian (Intel) byte order. Datum WGS84

NIMA Level 0 elevation post spacing is 30 arc second (nominally one kilometer).

Maui_1k, BT file from the "Virtual Terrain Project" (4MB uncompressed)

http://www.vterrain.org

http://www.vterrain.org/BT/index.html

USGS 1/250,000 DEMs: Map size not dem Rez

7.5-minute, : 10 or 30 meter

15-minute,

2-arc-second (also known as 30-minute),

and 1-degree units. The 7.5- and 15-minute DEMs are included in the large scale category while 2-arc-second DEMs fall within the intermediate scale category and 1-degree DEMs fall within the small scale category - (Source: USGS)

7.5-minute (1:24,000-scale) DEMs; 7.5-minute tiles; resolution: 30 meters; some at 10 meters.

15-minute (1:63,360-scale) DEMs; 15-minute tiles; resolution: 2 by 3 arc seconds (approximately 60 meters); coverage: Alaska

1-degree (1:250,000-scale) DEMs; 1-degree tiles; resolution: 3 arc seconds (approximately 90 meters)

250k: http://edc.usgs.gov/glis/hyper/guide/1_dgr_demfig/states.html

24k: http://gisdatadepot.com/dem

sdts2dem to convert 24k sdts files to a dem file for import by DTM.

250k dems imported by DTM

SRTM demo dems:The NIMA-MOU stipulates that full resolution (1-arc-second) terrain and radar image data over the United States will be releasable without restrictions to all users.  For non -U.S. areas, reduced resolution (3-arc-second) terrain data will be fully releasable without restrictions. Motorola 16-bit signed integer

SRTM-3 files contain 1201 lines and 1201 samples (1 point overlap){90 Meters}

SRTM-1 files contain 3601 lines and 3601 samples ("")){30 Meters}

National Elevation Dataset (NED)

NED: NED has a resolution of one arc-second (approximately 30 meters). The NED will be delivered in the Spatial Data Transfer Standard (SDTS) raster profile.

demo dems: BIL format in Motorola bytes.16-bit signed integer

1201x1201 (3-arc-second)

ArcGrid or Surfer .grd

XYZ ascii

TIN : import as mesh

Shape file fan or strip.. as mesh

Mars / planeteary dems

Fractal dems

... input X,Y size and type of data: Shortint, Integer, Float, Double

10 Meter  (360x180x64= 4147200 cells x ~10meg = ?41 terra ; x~3meg = 12.4T)

3456000 x 1728000 = 5,971,968,000,000 points  [24k 10M dems]

30 Meter: 1 Arc Second (= 4147200x ~1meg = ?4 terra)     [24k 30M dems]

1AS SRTM 1 degree cells 3601x3601= ~25 meg each

90 Meter: 3 AS (360x180x ~10meg = 648 gig  x~3meg= 194G) [250k dems]

{120(1) x 120(1) per degree = 432000 x 216000}

{The above are wrong as land only covers ~30 percent

and the poles make cells smaller;

conversion from Ascii to binary = ~10 to ~3 meg files}

1000 M: 30 AS [43,200 columns x 21,600 rows] (1.7G)      [Globe]

1852 M: 1 minute of latitude = 1 nautical mile, or 1.852 km [NA]

3704 M: 2-minute (latitude-longitude) resolution [10800 columns x 5400 rows]

~10000 M: 5 Minute [4320 columns x 2160 rows] (18meg)    [Etopo]

Maps >68 degrees 15x36'

62..68 15x30'

59..62 15x22.5'

>59 15x20'

1 degree Dems > 70 degrees every 9" = 401 points  (all are 1201 vertically)

50..70 6" = 601

-50..50 3" = 1201x1201 points

Tree of DEM 5', 30AS, 3AS, 10M

Datum Differences : WGS84...

Cell Centroid: Offset to Zero

Given location: (360 x 180 degrees = 64800 1* cells (x64 24k cells= 4147200 cells))

If 24k else If 1* else If Global else Etopo: then display

TYPES of DISPLAY

Elevation Tint

Slope tint

Hydro Basins: Stream - flow - Valleys

Contours

LOS: Point A to B, A Circled, Profile A to B

GL TIN- Roam

Rotating Globe <-> Projected global map displays (Mercator, Robinson, etc.)

Overlay Point, Line, Area .shp files (2D & 3D)

Use Patterns for Areal fill (multiple objects or texture bitmap)

Objects for Points and Lines (Trees for fill...)

3D Geology (depth)

Movement <> Temporal history

Processes (Flood, Route, Areal selection, Boolean - Solid +,-,both

Dem to tpcfg loader

Dem LR coord id <> Gtopo UL coords id map location

LR Coords: Map ID- Map name - 4 corners determine bounds

Need ID to determine map - Coords are double: hard to check 'actual' location...round to integers

Need Box of data required to make grid array

Resize selected area (Add maps together)(Crop off boundary)

Rescale Trim % of Rows-Columns

Image of area-> Full cropped area, Rescaled full area

Display types

Overlay Adjustment of Datum Fit / Center Cell

Processing dems to HTF

Get dems

Unpack from .gz

Rename Extensions to .dem

Zip original dems (for posterity)

Delete .gzs

Convert dems to bins

convert bins to HTF

Zip conversions (bins)

Delete dems

Image TEXTURES  <> World is ~ Meters around

?Weather images            10K pixels

AVHRR  Global Imagery     ~ 1K pixels     ~200 Meg?

Landsat 1..3  Path / Row   90M pixels

Landsat 4..7  Path / Row   30M pixels

NHAP photos              ~ 10M Pixels

HiRes satellites         ~ 1 Meter Pixels

A 3D Map comprised of layers of information.

Tiles of data enable moving from 1 area to another while reducing the amount of data required to be held in memory.

Layers:

Base layer of elevation data in

5', 30AS, 3AS, (30)10 meter file formats. (cell size)

Prepare by doing a Search of disks for Dems.. process into Htf files.

The texture defaults to a color-range from blue to orange based on Terrain elevations.

Imagery of the area. If not at the same scale, then scaled to fit the elevation layer. (Registered/rectified)

Shape files may contain polygons representing "ground cover", transportation (roads-railways), hydrology, etc. Shape files may be translated into "flat" texture or be represented by 3D objects: Vegetation, roads with embankments-guard rails, flowing rivers w/waves, canal-locks, river banks, telephone poles with wires, ...

Transform 2D into 3D .shp files.

.	Edit files as a 2D overlay on a DEM.

.	Get elevation for shape point locations

.	Save file as a 3D file

.	Convert Dem into .shp file fans?

.	Dem into a TIN w/shape file fan import

.	(added resolution to the grid data)

Entrance to a tile would require flags that data was available and from the data available which "texture" was applied, and from there the representation of the layers of shape files. A shape file may contain 2D or 3D polygons, lines, points; 3d fans-or-strips; however, only 1 type of shape pertaining to 1 type of object it represents per file. (1 layer each for vegetation, roads, railways, structures,... )

An area tiles : 1 Elevation, 0..1 Image, 0..1..n layers of shape files.

gee to further complicate things the shape files might cover many tiles of elevation-image. There might be various "scales" of elevation-images covering the same area. Selection of the Scale would determine which of the available layers was actually accessed... Zooming in would change the elevation-image tile selection while the shape coverage might remain the same.  Actually to get real complicated the selection could determine nothing and the availability of the data would determine what was used, even mixing elevation-image-shape coverage from different "suggested" scale ranges. (a representation of availability could be a 3x3 of "led's". the center is always where you are, Green is >1 scale range, yellow is 1, red is no data available.)

Selection of objects representing the shape file data could range from basic geometric shapes (default-minimum) to high polygon models.

Legend selection of a file per object type-layer to actual files per each object in that layer. The Roam-distance-movement-speed could be used to select from a shape to a shape/w/texture (decals..imposters) to a low-rez model to a high-rez model to a fully detailed high resolution model.

Besides placing 3D objects upon the terrain the shape file objects would also have to be linked to a database so their attributes could be displayed (pop up window or a status bar), used in analysis of the ?scenario? physical geographical relationships, or determine the actual representation of the object in a changing time aware scenario. Annotation of objects would not be a default capability, although it could be accomplished... with another layer.

A 360x180 array (64800) x4 'layers' =259200  Layers: Topo, Photo, Shape, Data Type

Each array 'degree' has 4 bytes coded: Each byte has 8 bits On/Off : Available

REZ:   [1]              [2]      [3]    [4]

300AS  Topo  10,000M   Weather   Global Data

30AS  Globe  1,000M   AVHRR     Scale

3AS  Dems      90M   MSS       ?  {Latitude changes density... data points per}

?    1AS  ....      30M   TM        ?  {because it is there and Landsat fits it?}

0.3AS  ....      10M   TM+       ?

0.03AS  ....       1M   ?         ?

0.003AS  ....      .1M   ?         ?

0.0003AS  ....    0.01M   ?         ?

Data Type:

Geology,

Complete coverage

Interior data,

Interior Photo,

Interior Panoramic Photo,

Exterior Photo,

Exterior Panoramic Photo,

Metadata.. Sundry information ..Computed-Interpolated/Measured Accuracy

==============================================================

Ideas for to do Stuff

Load Files  md2, HL, obj, 3DS,...

MSE Export to POV, VRML 1&2, VDO, X

Romka Tutor: DXF

Terragen Terrain file specification, DEM,Fractals... a Glscene format: ascii header and a .bin file of data

Elevation/Image/Object placement

DEM base; Utilities Tiler, Various 'Roam types'

Texture/Image Overlay

Building/Object Placement

Actor Movement	Actor -bone/flesh/skin editing Morphing Animal Generation

Route-Behavior-physics:

Object movement Car-Plane-Train.. floods, volcanic flow, tornado

Portal - Maze Generator.. Black box- press 'button' and a door opens.. use as a passage through a 'wall'... regen maze when out of this tile

Keyboard - Mouse - Joystick control (Object selection for editing/Morph/ Link to attributes) (warping <> Morph sticky-stay there, linear pull)

bmpfont <> Fonts (Nehe outline & 3D fonts; Delphi3D text3d.zip)

Fireworks -type selection/Placement/Duration-Direction / Timeline-Script / Sound

fireworks, particles, metaballs...

Planet Rotation/Generation(fractal)