���������������������������������������������������������������

LxPro V2.2

(c)1999 Stefan Peichl, Heidelberg, Germany

http://home.t-online.de/home/stefan.peichl

eMail: stefan.peichl@t-online.de

���������������������������������������������������������������

LxPro is a TSR (Terminate and Stay Resident) utility program

designed for the HP Palmtops 100LX/200LX/1000CX. It has the

following windows:

- HELP

- BATTERY

- MEMORY

- ASCII

- CALCULATOR

- KEYCODE

- PORTS

As a TSR it is only one keystroke away, under SysMgr and DOS.

It works in all 16 HP-Palmtop Video Modes.

LxPro (m)onitors/(c)hanges:

m   - battery voltages

m/c - battery low warning

m   - card status

m/c - serial power status for COM1

m/c - serial channel

m/c - serial port configuration (baud, frame, parity, stop)

for COM1 and COM2

m/c - battery type

m/c - battery charging status

m/c - display time out

m/c - beeper volume

m   - free main memory

m   - video mode

m   - power source

m/c - all I/O Ports

m/c - all Hornet Ports/Registers

m/c - 1MB RAM (00000-FFFFF) with search function

m   - all ASCII codes (BIN/OCT/HEX/DEC)

m   - all key codes (SCAN/ASCII)

m   - HEX/DEC/OCT/BIN conversions up to 16 bits

Additionaly, LxPro has one window with a Programmers Calculator

(HEX/DEC/OCT/BIN) in RPN (Reversed Polish Notation) with

arithmetic and logical operators.

The real strength of LxPro lies in it's TSR capability. You

don't have to leave your application to see the battery voltage

or to switch the serial power on/off. The memory window enables

you to watch your program while it is running. The ascii table

let's you find any ascii code, while you are in an editor. With

the calculator you can do a DEC-HEX conversion, while you are

programming.

���������������������������������������������������������������

REVISION HISTORY

���������������������������������������������������������������

Version 2.2   -Use of EMS if available

(05-MAR-99)   -Automatic exit at power off (=deep sleep)

Version 2.1   -COM1/COM2 toggle switch {/} added

(13-OCT-98)

Version 2.0   -PORT/HORNET-INDEX window added

(14-MAY-98)   -Cycle through screen modes with {Fn=}

-Readable colors with {.} in battery window

-Printer Init with {0} in battery window

-SH{L}, SH{R} added in Calculator window

-Bitmap bug in PAL applications fixed

Version 1.4   -Version 1.3 bug fixed (edit in memory window)

(22-OCT-96)

Version 1.3   -Version 1.2 bug fixed

(04-NOV-95)

Version 1.2   -Installation parameter /C to save 8KB RAM

(12-OCT-95)   -Baud rate 0, if COM1 power is off

-Free main memory in BATTERY window

-Symbol difference between Code Page 850 and 437

indicated as '!!' in ASCII window's Mnm field.

Version 1.1   -Monitor/Change serial port configuration.

(18-AUG-95)   -LxProMin zoomable in graphic modes.

-Quicker ASCII table screen output.

Version 1.0   -Beta Test Version

(01-AUG-95)

New versions of LxPro will be available on the Internet under:

http://home.t-online.de/home/stefan.peichl/lxpro.zip

���������������������������������������������������������������

WHAT'S IN YOUR ZIP FILE

���������������������������������������������������������������

LXPRO.DOC     This file

LXPRO.COM     Full version

a -MUST- for LxPro_grammers

LXPROMIN.COM  Mini version (only battery window)

useful to everybody

LXPROHOT.BAT  Changes hotkey from CTRL-TAB to CC_MAIL/QUICKEN

LXPROHOT.DAT  Input file for LXPROHOT.BAT

LxPro is by default activated with the key CTRL-TAB. This is a

key combination. If you prefer a single keystroke, you may

change the activation key to any key you want (see TECHNICAL).

LXPROHOT.BAT is a batch file, that changes the activation key

to the CC_MAIL key (100LX) respectivly the QUICKEN key (200LX).

I suggest this key, because these applications are normally the

less used applications on the palmtop.

NOTE: CC_MAIL/QUICKEN is not lost, if you use their key for

LxPro. You may still start these application from the '&...'

key (=Other Applications).

LXPROHOT.BAT assumes, that you run it on a HP Palmtop, which

means, that DEBUG.COM is found under D:\DOS. LXPROHOT.BAT

changes the hotkey for LXPRO.COM and LXPROMIN.COM. LXPROHOT.BAT

works only, if all files are in the same directory.

���������������������������������������������������������������

INSTALLATION

���������������������������������������������������������������

Put the line

{path\}LXPRO

into your AUTOEXEC.BAT file, or put the line

INSTALL {path\}LXPRO.COM

into your CONFIG.SYS file and reboot (CTRL-ALT-DEL)

If you find LxPro useful, and you want to use it every day,

I recommend to install EMS on your palmtop using the TREMM

driver by Mack Baggette. Alternatively you may use the /C

parameter to install LxPro. Both methods free up 8 KB of RAM

(4 KB for LxProMin). If your computer is equipped with EMS

the /C switch is ignored. See 'TECHNICAL' for more information

on how to install using the /C parameter.

You can use three command line parameters with LxPro, once it

is loaded resident:

/P passivates the program. It stays in memory, but cannot be

started with the hotkey.

/A activates the program after it was passivated. The hotkey

is hot again.

/R removes the program from memory.

If you enter LxPro without parameters, you get the actual state

of LxPro (that is: active or passive). Unknown parameters or /?

result in a syntax output.

���������������������������������������������������������������

HOW TO USE LXPRO

���������������������������������������������������������������

LxPro works like good old SIDEKICK: Pressing the activation key

the first time opens the battery window. Pressing this key

again, closes the window. That's all for LxProMin users,

because LxProMin has only the battery window.

Once, the battery window is open, you may open all other windos

with the keys F1-F7. The window on top is called active window.

Every input is passed to this window. All other windows sleep

in the background, but can be brought to the foreground and

activated with their corresponding F-key. You may switch

between all windows by pressing their F-keys.

The ESC key closes the active window (makes it disapear).

Pressing ESC repeatedly, closes all windows and exits finally

LxPro. The next activation then starts with the battery window

open again.

If windows are open, and you press the activation key to exit,

(instead of the ESC key) you will find the same windows open,

when you reenter LxPro. This feature is useful if you only need

one window for a time. Let's say, you only need the calculator.

Then do the following; Start LxPro with the activation key. The

battery window will open. Press F4 for the calculator. Press F7

to make the battery window active again and press ESC to close

the battery window. Now, you have only the calculator window on

your screen. Press the activation key to stop/start LxPro with

the single calculator window.

While windows are open, you may use the zoom facility of your

palmtop for better visibility. In text mode, normally three

zooms are available. The size and position of the windows fits

to the biggest text zoom, which is also compatible with the

HP95LX mode. In graphic modes, two zooms are implemented.

Installation with the /C parameter adds a third graphic zoom.

���������������������������������������������������������������

F7�BATTERY WINDOW

���������������������������������������������������������������

After pressing the activation key the first time, you will see

the battery window:

musical note symbol: system low warning and beep enabled

�       battery status (ok|low|min)

�       �    LOW warning occurs at indicated voltage

�       �    �    measured voltage

�       �    �    �

��������������������ͻ ����������������������BATTERY MONITOR�

����Voltage��low��now� (Card status -if there-)

�  Main  ok�2.20�2.58� 1.57-3.30|????|nois

�  BkUp  ok�2.80�3.02� 1.57-3.30|????|nois

��������������������ĺ ���������������������SYSTEM VARIABLES�

�1�  9600  N,8,1�COM1� COM1|COM2|OFF (xxxxxx baud�N81|E71)

�2�COM1 Channel �WIRE� WIRE|  IR

�3�Battery Type �ALKA� ALKA|NICD

�4�Charging     � OFF�  OFF| xxh (Fast|Slow)

�5�Time Out     �  3m�   1m|2m|3m|4m|5m|10m|30m|60m|OFF

�6�Volume       � ��ܺ    �|  ��| ���| OFF

��������������������ĺ ������������������������MISCELLANEOUS�

�565136�640x2BwG�BATT� BATT|ADAP

��������������������ͼ

�       �      �

�       �      Power Source

�       Video Mode

Free main memory

�BATTERY MONITOR�

In the 'now' column you see the actual measured voltages for

the main and backup batteries. A '????' instead of a value

means, that the voltage dropped below 1.57v (minimal measurable

voltage), or that no battery at all is inserted.

The 'low' column shows the voltages, when the respective

battery low warning will occur. These are fixed values, to give

you an idea, on how long your batteries will last. The low

value for the main battery changes with the battery type (see

below). The very low warning for the main battery will occur at

2.00v independant of the battery type.

The left column displays the battery status as reported by the

system. "ok" means, no low or very low warning was displayed.

"low" means, the respective 'battery low' warning was displayed

and a significantly higher voltage was not measured in the

meantime. "min" (only for main battery) means, the 'main

battery very low' warning has occured.

On the very left you see musical note symbols. They indicate,

that the system will announce the respective battery warnings.

You may supress the 'main battery low' warning by pressing Fn+

(Paste). The 'main battery very low' warning is not affected.

Press Fn+ again to enable the warning. Use this feature only,

if your data are always safe, because the time between the

'very low' warning and system shut down may be very short.

�SYSTEM VARIABLES�

In this part of the window you see 6 items, numbered 1 to 6.

You can press key 1 to 6 to change the respective items. Key 1

to 4 are so called toggle switches, that means, the item has

two values, and pressing the key several times toggles between

these two values. Key 5 and 6 are similar, except that the

items have more than two values. Pressing key 5 and 6

repeatedly leads you through the range.

The keys +,-,* are new since Version 1.1 and enable you to set

the baud rate, frame size, parity and stop bits of the selected

COM port.

KEY  FUNCTION

/    Toggle between COM1 and COM2

*    Toggles COMx configuration between:

8N1 = 8 bit frame, None parity, 1 stop bit    and

7E1 = 7 bit frame, Even parity, 1 stop bit

(these are the two standard configurations for data

communication.)

NOTE: You may only switch between 8N1 and 7E1, but LxPro

monitors all possible configurations:

Frame size: 5, 6, 7, 8

Parity    : (N)one, (E)ven, (O)dd, (M)ark, (S)pace

Stop bits : 1, 2

+    Increase baud rate (up to 115200)

-    Decrease baud rate (down to 0)

Don't worry about strange baud rates like "10472". You may

set every valid baud rate between 9600 and 115200. Below

9600, only the famous baud rates are implemented. (It was

simply easier to program it this way.)

1    COM1 Power and configuration

In this line, the actual baud rate, frame size, parity and

stop bits are monitored. You may switch COM1 power ON and

OFF. In OFF poition, the baud rate is set to 0. This

disables the UART clock and saves power. If no communi-

cation is active, serial power should always be OFF to

save battery power. If power is indicated as OFF, it is

always off for WIRE and IR. If power is ON it is only on

for the selected channel.

NOTE: You cannot switch COM2 Power OFF

2    COM1 Channel

Choose between WIRE and IR. Changing serial channel always

switchs serial power OFF for WIRE and IR. You have to turn

it ON with key 1.

3    Battery Type

Select between ALKA and NICD (rechargable). This has

effect on the indicated battery low voltage. ALKA are

supposed to be low at 2.2v, NICD are low at 2.3v.

4    Charging

Charging is only enabled, if battery type is NICD.

Charging starts, when selected and your machine is

connected to the adapter. The first 6 hours are charged

in Fast mode, the rest in Slow mode, as indicated. The

elapsed hours of charging are also indicated.

5    Time Out

Controls, after how many minutes your Palmtop will go into

deep sleep. You can select between 1,2,3,4,5,10,30,60

minutes and OFF. In OFF position, your Palmtop will never

go into deep sleep, draining your battery very quickly.

Set time out to OFF only if really needed.

Works only if disconnected from the AC adapter.

6    Volume

Three different volumes and OFF are available. A beep for

every volume shows you the intensity.

Fn+  Toggles "Main Battery Low" warning on and off

Fn=  Cycles through all available screen modes of the palmtop

.    tries to make unreadable text more readable

0    Configures COM1 for a (my) serial printer:WIRE,9600,N,8,1

�MISCELLANEOUS�

The last row indicates:

- Free main memory in bytes. Under plain DOS, this is the

same value as reported by CHKDSK. Use this feature, to find

out the memory requirements of a DOS program and place the

value behind the '|' in the path field of the application

manager (&...). Under SysMgr, the LxPro free main memory

value differs from what SysMgr reports. SysMgr has it's

own memory management.

NOTE: Lotus 123 takes all free memory from the top down,

while open, but you may still open new applications, because

SysMgr asks 123 to release a portion of memory.

- one of the following 16 video modes:

MODE

0h = 40x25BwT       ���ABBREVIATIONS���

1h = 40x25CoT       Bw = Black & White

2h = 80x25BwT       Co = Color

3h = 80x25CoT       T  = Text

4h = 320x2CoG       G  = Graphic

5h = 320x2BwG       Md = MDA

6h = 640x2BwG       Z  = Zoom

7h = 40x16MdT

20h = HP95_LXG

21h = 80x25MdT

80h = 64x18BwZ

81h = 64x18CoZ

82h = 40x25BwZ

83h = 40x25CoZ

84h = 40x16BwZ

85h = 40x16CoZ

- Power Source (BATT|ADAP)

The items in the battery window are updated with any keystroke

(if valid or not). For example, when you load batteries and

leave the window open, the actual voltage and the indicated

hours of charge will only be updated, if you press a key.

THIS IS ALL YOU NEED TO KNOW, IF YOU ONLY USE LXPROMIN.COM

���������������������������������������������������������������

F1�HELP WINDOW

���������������������������������������������������������������

Pressing F1 always gives you context sensitiv help about the

active window. Once, the help window is open, you may read all

7 help texts, using the CuUp or CuDn key. Press Esc to exit

help.

���������������������������������������������������������������

F2�MEMORY WINDOW

���������������������������������������������������������������

Address Field Cursorposition

�                         Edit Cursorposition

�                         �

�v�������������������������v������ͻ

�FFFB0�CCCC CCCC CCCC CCCC���������

�FFFB8�CCCC CCCC CCCC CCCC���������

�FFFC0�CCCC CCCC CCCC CCCC���������

�FFFC8�CCCC CCCC CCCC CCCC���������

�FFFD0�CCCC CCCC CCCC CCCC���������

�FFFD8�CCCC CCCC CCCC CCCC���������

�FFFE0�CCCC CCCC CCCC CCCC���������

�FFFE8�CCCC CCCC CCCC CCCC���������

�FFFF0�EA5B E000 F030 382F��[���08/�

�FFFF8�3230 2F39 33CC FE00�20/93���

����������������������������������ĺ

�              Search[+/-]�        �

���������������������������������ͼ

�

Search Cursorposition

The memory window uses absolut address representation for the

1MB address space from 00000h - FFFFFh. More is not addressable

by the processor. Don't be sad if you own a 2MB model, The

second MB of RAM is paged in fractions somewhere between BC000h

and F0000h when needed.

Look for a technical DOS and/or HP reference, to see, how the

1MB address space is organized.

�ADDRESS FIELD�

KEY    FUNCTION

CuDn   down one line   (+ 8 bytes)

CuUp   up one line     (- 8 bytes)

PgDn   down one page   (+80 bytes)

PgUp   up one page     (-80 bytes)

Home   goto 00000h

End    goto FFFFFh

CuRi   move cursor right in address field

CuLe   move cursor left  in address field

�SEARCH�

Press S to move the cursor to the search input field. Enter the

string you want to search in the RAM. You may only use

backspace to edit the string. Press enter to start searching.

The whole address space of 1MB is searched, starting from the

actual displayed address. You will hear a click for every

segment (64KB) searched. If the string is found, you hear a

higher click, and the address, where the string was found, is

displayed.

Press + to find the next occurance, - for the previous. As long

as you see the search string in it's field, you may always type

+ or - to search for it again.

NOTE: You will always find the search string at least once:

This is the memory location within LxPro, where the

search string is stored!

�EDIT MODE�

Press the Del key to enter/exit edit mode. The cursor is

positioned to the ascii representation of the address, which is

displayed in the first line. You may only edit this byte. If

you want to edit more than one byte, you always have to change

the address first.

You may enter any value from 0-255. I suggest to use the

Alt+Menu+nnn (nnn=001-255) input form. The key combination for

0 is ^@.

NOTE: You may not write to any address. If the address is read

only, nothing happens.

Use edit mode only, if you exactly know what you do, otherwise

you sure have to reboot your system. This is a razorblade in

the hand of a skilled programmer! You may use this feature to

change data or code of your own program while running. You may

also change BIOS data, but be careful!

���������������������������������������������������������������

F3�ASCII TABLE WINDOW

���������������������������������������������������������������

The displayed ascii table corresponds to the active code page.

DOS uses code page 437, system manager uses code page 850.

��������������������������������Ŀ

�    (1.row appears only live)   �

� !"#$%&'()*+,-.�Ŀ23456789:;<=>?�

�@ABCDEFGHIJKLMN�P�RSTUVWXYZ[\]^_�

�`abcdefghijklmn���rstuvwxyz{|}~�

����������������������������������

�����������������������R����������

��������������������������������߳

������������������������������� �

��������������������������������ĳ

�Mnm�Key�Sym� Binary �Oct�Hex�Dec�

��������������������������������ĳ

�   � P � P �01010000�120� 50�080�

����������������������������������

The little frame surrounds the character, which is presented in

the base row. Use CuUp, CuDn, CuLe, CuRi to move the frame to

the character you want to see.

You may also press any key, key combination or key sequence,

which has an ascii representation. The frame is immediately

positioned around the corresponding character and its values

are displayed in the base row.

The 'Mnm' field tells the mnemonics of the ascii codes 00h-20h

as used in data communication protocols. A !! in the Mnm field

means, that the symbol differs between code page 437 and 850.

The 'Key' field tells the keys or Ctrl key combinations to

produce the respective character.

The 'Sym' field tells the screen representation of the

corresponding character.

The Bin, Oct, Hex and Dec Value of the character are displayed

in the rest of the base row.

���������������������������������������������������������������

F4�CALCULATOR WINDOW

���������������������������������������������������������������

Number base for X and Y (HEX|DEC|OCT|BIN)

�   only RPN is implemented

�   �    result flags of the XY operation

�   �    �

��������������������ͻ

�HEX�RPN�FLAGS=      �                possible FLAGS

��������������������ĺ

�Hex             FFFF� X in Hex       O = overflow

�Dec            65535� X in Dec       S = sign

�Oct           177777� X in Oct       Z = zero

�Bin 1111111111111111� X in Bin       A = auxiliary

��������������������ĺ                P = parity

�Y                  0� Y register     C = carry

�X               FFFF� X register

���������������������

�

Cursorposition for input

KEY        FUNCTION

CuUp,CuDn  Changes number base for input and X,Y representation

The actual number base is displayed in the upper

left. Dependent on the base, you may only enter

numbers, that fit to this base. Otherwise you hear a

beep. You may change the number base at any time,

even in the middle of an input.

CuLe       Exchanges X and Y register

Backspace  Removes last number of your input

Del        Sets register X to 0

+ - * /    Arithmetic operators

N O X      AND, OR, XOR operator.

NOTE: The AND operator is represented by a N,

because A is a valid hex number.

L R        Shift X:  L=SHL X,1  R=SHR X,1

If you hear a beep during input, you tried to key in a number

that does not fit to the selected number base, or your input

exceeds the 16 bit boundary.

The calculator uses only RPN (Reverse Polish Notation).

To do a calculation proceed as follows:

1.operand     (is stored in X)

enter         (X is copied to Y)

2.operand     (is stored in X)

operation     (X operation Y is done, result in X, Y set to 0)

The calculator uses only 16 bits. X, Y and the result may not

exceed FFFFh. If the result exeeds FFFFh, only the low 16 bit

part of the result is displayed. Think of it as an operation

with two 16 bit registers, where the result is also stored in

a 16 bit register. The flags are set as reported by the

processor for this operation. (Actually nothing else is done

in the code)

The advantage of the calculator is it's simultaneous display of

the X register in all 4 number bases and the possibility to

change the number base used for input and representation of

the X and Y register at any time.

���������������������������������������������������������������

F5�KEYCODE WINDOW

���������������������������������������������������������������

���������������ͻ

� Press any key �

� �����Ŀ   �Ŀ �

� �19 70� = �p� �

� �������   ��� �

� Scn Asc   Sym �

���������������ͼ

The keycode window reports the scan and ascii code of the

pressed key in the same way as the extended functions of INT16

do. A '00' in the ascii field means, that the key has a special

scancode with no correspondence to an ascii character.

Try Shift/Alt/Ctrl key combinations and Fn key sequences to

find out, which symbols are represented by which keys.

You may use this window to find your new activation key for

LxPro.

���������������������������������������������������������������

F6�PORT WINDOW

���������������������������������������������������������������

Input of Index or Port number

|

�������������v�

�Index      1E� currently selected Port/Index (hex)

�������6�4�2�0� bit position

�In   11111101� byte red from actual Port/Index (bin)

�Out  00000000� byte to output to actual Port/Index (bin)

��������������

�

Cursorposition to edit a bit of the Out byte

The Port window works in two modes, selected using the {P}ort

or {I}ndex key. In Port mode, the In byte is red directly from

the selected port, and the Out byte will be send to this port.

The index mode allows you to access all HORNET registers in a

simple way. Normally you have to write an index to port 22h and

then you can write/read a byte to/from port 23h. LxPro does all

this work for you in Index mode. Simply enter the index and

LxPro accesses the register via port 22h/23h.

Use {+} and {-} to increment or decrement the Index or Port.

This allows you to easily step through a whole range.

The In byte is always red immediately. The Out byte is only

sent, if you press {O}. You may copy the In byte to Out byte

with {CR}. You may edit every single bit of the Out byte by

using the {CuLe} and {CuRi} keys and then pressing 0 or 1.

Be very careful, before you output a byte. Don't do it without

having red in the SDK about the meaning of the bits in the

register. It is very likely, that you have to reboot your

system after a false output. It is always nice to refer to an

actual backup in those situations.

KEY        FUNCTION

P          Select Port mode

I          Select Index mode

CuUp,CuDn  Toggles the input area between Port/Index and

Out byte. Port/Index has to be entered as Hex value,

the Out byte has to be entered as Bin value

CuLe,CuRi  Move the cursorposition in the Out byte range

CR         Copy the In byte to the Out byte

O          Output the Out byte to the selected Port/Index

Backspace  Removes last number of your Port/Index input

Del        Set Port/Index or Out byte to 0

+ -        INC/DEC the Port/Index number

���������������������������������������������������������������

ROUGH EDGES

���������������������������������������������������������������

I have to mention one inconveniences, that might occur in

conjunction with alarms.

If an alarm occurs under system manager, while windows are

open, do the following:

1 - press Esc until all LxPro windows are closed. About half

of the alarm message will stay on the screen.

2 - press Esc once more to quit the alarm message. Your screen

will be restored.

If you don't press Esc once more, after all LxPro windows are

closed, your keyboard will be frozen until you press Esc.

Don't activate LxPro during a transition from one built in

application to another. Your machine may lock up and you have

to reboot. Thanks to Vanildo Maldi for this advice.

���������������������������������������������������������������

TECHNICAL

���������������������������������������������������������������

If you want to use another activation key, you have to patch

the two bytes at offset 0Dh,0Eh in LXPRO.COM (or LXPROMIN.COM).

Don't forget to add 100h if you do it under DEBUG. Put the

ascii code of your new hot key at offset 0Dh and the scancode

at offset 0Eh. See also LXPROHOT.BAT, which does this job

automatically.

If you want to save memory, install EMS on your palmtop. This

is the only way for 32/64 MB memory upgraded machines. Smaller

or not upgraded palmtops allow the use of the somehow awkward

/C parameter.

You may install LxPro with the /C parameter, which tells LxPro,

to use a part of your C: RAM disk instead of main memory RAM as

a screen save buffer. This frees about 8 KB (4 KB for LxProMin)

of main memory for other use, but takes away permanently 16 KB

of your C: disk. If you use a RAM Card in drive A: and/or you

always have enough space left on your C: drive, this way of

installing LxPro will be interesting for you. Proceed as

follows:

- make a backup of your C: drive, all files will be deleted!

- append the /C parameter in your AUTOEXEC.BAT or CONFIG.SYS

- boot using Ctrl+Alt+Del

- boot again using Ctrl+Shift+On

- Yes, initialize your C: RAM disk

- rebuild your C: drive

- boot using Ctrl+Alt+Del

This is the only way, to shrink the C: drive, and it is done

permanently. During the first boot, LxPro changes the disk

resource table. The second (hardware) boot informs the

operating system of the new disk resource table. The

initialisation of the C: RAM disk builds a new disk description

table according to the new resource table. From now on, the C:

drive has 16 KB less resources. The third boot has only to be

done, if your configuration files were not on your A: drive.

In order to put C: back to it's factory configuration, you have

to remove the LxPro/C command line from your AUTOEXEC.BAT or

CONFIG.SYS file and disconnect your palmtop from any power

source! Remove all batteries and the AC Adapter for a moment.

Again, all your files on C: are deleted! On rebooting, C: will

have it's old size. As you see, booting with Ctrl-Shift-On does

not completely rebuild the factory configuration of your

palmtop!

If you simply install LxPro/C without the procedure discribed

above, part of your data on the C: RAM disk will sooner or

later be distroyed without any warning.

LxPro takes the last 16KB of the second 512KB RAM chip NRAS[1]

away as a resource for drive C:. Once, LxPro has cut away the

16KB, it uses bank switching to access the memory. It is bank

switched into page 8(E0000-E4000) when needed, in order to save

or restore the screen buffer.

Thanks to Gilles Kohl, who made the suggestion to use a portion

of the RAM disk as screen save buffer instead of main memory.

LxPro chains into INT6 and INT9. LxPro takes INT15 while

windows are open.

LxPro uses BitBlt functions to write to the screen in graphic

modes.

In text modes, LxPro writes directly into the CGA/MDA frame

buffer.

���������������������������������������������������������������

DISCLAIMER AGREEMENT

���������������������������������������������������������������

LxPro is copyrighted freeware. Users of LxPro must accept this

disclaimer of warranty:

"LxPro is supplied as is. The author disclaims all warranties,

expressed or implied, including, without limitation, the

warranties of merchantability and of fitness for any purpose.

The author assumes no liability for damages, direct or

consequential, which may result from the use of LxPro."

���������������������������������������������������������������