S V A S M 0 2

A two-pass cross-referencing assembler

for

6502 and 65C02 microproccessors

Written by

John Michne, KM2H

10 Evergreen Ave.

Clifton Park, New York  12065

Version 1.0

May, 1988

S V A S M 0 2

USER'S MANUAL - Version 1.0 - May, 1988

TABLE OF CONTENTS

INTRODUCTION . . . . . . . . . . . . . . . . . . . . . . 1

RUNNING THE SVASM02 ASSEMBLER. . . . . . . . . . . . . . 1

Source filename. . . . . . . . . . . . . . . . . . . . 1

Object filename. . . . . . . . . . . . . . . . . . . . 1

Print filename . . . . . . . . . . . . . . . . . . . . 2

INPUT FILE LINE STRUCTURE. . . . . . . . . . . . . . . . 2

Label field. . . . . . . . . . . . . . . . . . . . . . 2

Instruction field. . . . . . . . . . . . . . . . . . . 2

Argument field . . . . . . . . . . . . . . . . . . . . 3

Comment field. . . . . . . . . . . . . . . . . . . . . 3

ARITHMETIC . . . . . . . . . . . . . . . . . . . . . . . 3

Symbols. . . . . . . . . . . . . . . . . . . . . . . . 3

Program counter. . . . . . . . . . . . . . . . . . . . 3

Constants. . . . . . . . . . . . . . . . . . . . . . . 3

Expressions. . . . . . . . . . . . . . . . . . . . . . 4

Arithmetic operators . . . . . . . . . . . . . . . . . 4

Logical operators. . . . . . . . . . . . . . . . . . . 5

ADDRESSING MODE SYNTAX . . . . . . . . . . . . . . . . . 5

6502 Addressing modes. . . . . . . . . . . . . . . . . 5

65C02 Additional addressing modes. . . . . . . . . . . 7

ASSEMBLER DIRECTIVES (Psuedo-ops). . . . . . . . . . . . 8

.BYTE. . . . . . . . . . . . . . . . . . . . . . . . . 8

.WORD. . . . . . . . . . . . . . . . . . . . . . . . . 8

.CMOS. . . . . . . . . . . . . . . . . . . . . . . . . 9

.END . . . . . . . . . . . . . . . . . . . . . . . . . 9

.LIST. . . . . . . . . . . . . . . . . . . . . . . . . 9

.NOLIST. . . . . . . . . . . . . . . . . . . . . . . . 9

.INCLUDE . . . . . . . . . . . . . . . . . . . . . . . 9

.PAGE. . . . . . . . . . . . . . . . . . . . . . . . . 9

.WIDTH . . . . . . . . . . . . . . . . . . . . . . . .10

.PRINTC. . . . . . . . . . . . . . . . . . . . . . . .10

.IF, .ELSE, .ENDIF, .LISTF . . . . . . . . . . . . . .10

ERRORS . . . . . . . . . . . . . . . . . . . . . . . . .11

Fatal errors . . . . . . . . . . . . . . . . . . . . .11

Invocation errors. . . . . . . . . . . . . . . . . . .11

Assembler errors . . . . . . . . . . . . . . . . . . .12

APPENDIX A

SVASM02 LST Output File Description. . . . . . . . . .14

APPENDIX B

SVASM02 HEX Object File Description. . . . . . . . . .16

COPYRIGHT NOTICE

The  SVASM02  program and User's Manual are Copyright

1988 by John Michne.   Both may be freely copied  and

distributed for PRIVATE,  NON-PROFIT,  NON-COMMERCIAL

USE ONLY.   The SVASM02 program and User's Manual may

not  be copied and distributed commercially except as

arranged with the author.

The   author  makes  no  warranty  or  guarantee   of

suitability of SVASM02 for any purpose,  and will not

be   held   liable  for  any   damages,   direct   or

consequential,  that may arise due to the use of this

software.

INTRODUCTION

SVASM02  is  a  two-pass  cross-referencing  assembler  for  6502  and   65C02

microprocessors.   It  runs in IBM and compatible machines,  and has been  run

under both PC-DOS (Version 3.1) and MSDOS (Version 2.11).  Since it is written

in  8086  assembly language,  memory requirerments are minimal  and  execution

speed is optimum.   It is approximately 12K long, and will run in as little as

64K.   The  original  Version  1.0 distribution was via GEnie  to  the  public

domain.   This was primarily to get the program out to where it may be useful,

and get it tested under conditions of actual use by programmers other than the

author.   Future  versions will be distributed to registered users only.   See

the file INFORMAL.DOC for details.

This document is not intended to teach assembly language programming.  It will

fully  explain  the  operation of the SVASM02  assembler,  and  with  numerous

examples, will guide the user to efficient and successful assembly of 6502 and

65C02 code.

RUNNING THE SVASM02 ASSEMBLER

SVASM02  is invoked in the usual manner - merely type the program  name.   The

screen will clear and a sign-on line will appear, followed by a prompt for the

input file name.  It looks like this:

SVENSON ELECTRONICS 6502/65C02 ASSEMBLER - V.1.0 - MAY, 1988

Source filename [.ASM]:

At  this  point,  enter the requested filename.   A valid disk designator  may

preceed  the  filename,  but a path designation may  not.   SVASM02  does  not

support  directory paths.   If no drive specifier is used,  the default logged

drive  is  assumed.   Note  in the example above the  "[.ASM]"  following  the

filename prompt.   This is the default filename extension that will be used in

the  absence  of any input filename  extension.   For  example,  the  filename

SOURCE.ASM may be input as SOURCE or as SOURCE.ASM.  If the filename extension

is  anything other than .ASM,  it must be specified.   Any error situation  is

immediately  displayed,  with  a  repeat of the prompt.   The source  file  is

expected  to be standard ASCII text,  which may be prepared with any  suitable

editor or word processor.

The next prompt is for the object filename:

Object filename [.HEX]:

Again,  disk  designations  are supported,  paths are  not,  and  the  default

filename extension is .HEX.  If no drive specifier is used,  the default drive

is  assumed.   A  filename must be entered to obtain an object  file.   If  no

object  file is desired,  merely strike the ENTER key.   Output is normally to

disk  as ASCII characters in the standard Intel format (see Appendix B  for  a

description  of the HEX file format).   Reserved Device Names may be used  for

directing HEX output to the COM ports,  console, or printers.  This is done by

DOS,  and  is not supported by SVASM02.   Obviously,  you do this at your  own

peril.  The results may not be as expected.

The final prompt is for the name of the listing or print filename:

Print filename [.LST]:

The same rules apply here as for the .HEX file, except the listing file may be

directed  to  the screen by entering CON or to the printer  by  entering  LPT.

Entering  a valid optional drive and filename will cause the print file to  be

written  to disk.  If no drive specifier is used,  the default logged drive is

assumed.   Use  of  the system Reserved Device Names  may  produce  unexpected

results,  except  for CON.   Drivers have been included to produce the correct

results using CON, and using LPT instead of any of the system names produces a

correct  printer output.   See Appendix A for a description of the LST  output

file format.

Assembly now begins.  If any assembly errors are found, the offending numbered

line is output to the screen,  followed by a line containing the error  number

and  a  description of the error.   Error output to the screen  is  automatic,

regardless of the presence or absence of a .LST or .HEX file.   If a .LST file

is being written, the same error sequence will appear in it.  Errors are fully

discussed  anon.   Completion of assembly is signaled by the appearance of  an

ending  statement containing  the number of bytes assembled and the number  of

errors, followed by the DOS prompt.

INPUT FILE LINE STRUCTURE

SVASM02 uses the standard format for assembler input.   Each instruction is on

a single line,  and each line is composed of four fields.  A line may start in

any column.  The fields are:

Label     Instruction      Argument     ;Comment

All  fields  are  generally optional.   The argument field may or may  not  be

required depending on the nature of the instruction.   Fields are separated by

one or more space or tab characters.  The argument field may be separated from

the comment field by the comment delimiting semicolon character ";".

Label Field

Labels are used as references within the code.   Their use is optional, but if

a  label is used it is the first non-blank field on the line.   They may be up

to  six  characters long and must begin with a letter.   Only the  upper  case

alphabetic  characters  from A to Z and the numerals 0 to 9 may be used  in  a

label.   A label does NOT require a colon. In fact, use of a colon following a

label  will generate an error.   A label may be the only field on a line.

Instruction Field

The  instruction  field  is  composed  of  microprocessor  instructions.   All

instructions  are three upper case characters long.   The instruction mnemonic

is as described by the manufacturer.   The instruction field is the first non-

blank field on the line if a label is not used,  or if the statement is not an

equate or directive.   If a label is used, one or more blank or tab characters

must  separate  the label field from the instruction field.   For  a  complete

listing of all of the 6502 and 65C02 instructions, see the file TEST02.ASM.

Argument Field

The argument field contains any argument required by the instruction.   It may

be a constant,  a symbol,  register name, or an expression.  Numeric constants

may be expressed in decimal,  octal,  hexadecimal,  or binary.   Quoted string

constants  are  also  allowed.   See the Arithmetic  section  for  a  complete

description of constants, symbols, and expressions.

Comment Field

The comment field is the last field on a line, and must begin with a semicolon

(;).   Anything,  regardless of case,  that follows a semicolon is  considered

commentary  information  and is ignored by SVASM02.   Comments appear  in  the

source  listing  exactly as they appear in the source line.   A blank line  is

considered a comment, and need not be preceeded by a semicolon.

ARITHMETIC

Symbols

A symbol is a name assigned to represent a value.  A symbol, once defined, may

not  be redefined.    Labels are symbols,  since the value of the label is the

value  of the program counter when the label is defined.   A label is  defined

when  it  appears  as  the first field on a  line.

Program Counter

The  program counter is represented symbolically by the asterisk (*).  If  the

program  counter  is not defined,  a starting value of zero is  assumed.   The

value assigned to the program counter may not be a forward reference.

Consider the following sequence:

* = $100       ; The program counter is defined a value of 100 hex.

FLAG * = * + 1      ; The value of FLAG is 100 hex, and the program counter

; is advanced by 1.

START     LDA #1    ; The value of START is 101 hex

In  the  above sequence,  FLAG is uninitialized storage at location  100  hex.

START is a label having a value of the program counter, or 101 hex.

Symbols follow the same rules of syntax as labels - up to six characters (A-Z,

0-9), beginning with a letter.

Constants

A constant is an unsigned numerical value between 0 and 65,535 (decimal).   It

may be expressed in any of four radices  - decimal,  hexadecimal,  octal,  and

binary - or  may  be a quoted string one or two characters  long.

A  decimal  constant may contain the numerals 0 through 9,  and must  have  no

leading or trailing radix specifier.

A hexadecimal constant may contain the numerals 0 through 9 and the upper case

letters  A  through  F.   It must be either preceeded by the dollar  sign  ($)

character  or contain the letter H as the last character.   If the trailing  H

specifier is used, the first character must be a numeral between 0 and 9.

An  octal constant may contain the numerals 0 through 7,  and must  be  either

preceeded by the @ character or trailed by the letter Q.

A  binary constant consists of up to sixteen 0 and 1 characters,  preceeded by

the % character or trailed by the letter B.

A quoted string constant is any one or two characters enclosed between  single

quote characters.   The value of a single character quoted string is the 8-bit

ASCII code value of the character.   A two character quoted string has similar

characteristics,  being the 16-bit value of the characters, in ascending order

(from  left  to right).   Note that the double quote is invalid  as  a  string

delimiter,  and is considered to be a character just like K,  a,  &, etc. (See

the  discussion  of  the  .BYTE directive for additional  use  of  the  quoted

string.)

Some examples:

1233      Decimal, no radix specifier

$4ACF     Hexadecimal, leading radix specifier

4ACFH     Hexadecimal, trailing specifier

@377      Octal, leading specifier

377Q      Octal, trailing specifier

%10110    Binary

10100110B Binary

'A'       Single character quoted string

'CQ'      Two character quoted string

Expressions

An expression is a symbol,  constant,  or combination of symbols and constants

connected  by  arithmetic and/or logical operators such that the value of  the

expression does not exceed 16 bits, or 65,535.  The arithmetic operators are:

+         Addition

-         Subtraction

*         Multiplication

/         Division

<         Extraction of low 8 bits

>         Extraction of high 8 bits

Arithmetic Operators

The four arithmetic operators (+, -, *, and /) apply to addition, subtraction,

multiplication,   and   division,   respectively,   and  warrant  no   further

explanation.   The byte extraction operators extract either the low 8 bits  or

high 8 bits from a 16-bit argument.  This is best illustrated by examples:

LOW8 = <$1234       ; The value of LOW8 is $34

HIGH8 = >$1234      ; The value of HIGH8 is $12

The logical operators are:

AND       Logical AND of the arguments

OR        Logical OR

NOT       Complement of the argument

XOR       Exclusive OR of the arguments

MOD       Remainder

SHL n     Shift left n bits

SHR n     Shift right n bits

Logical Operators

All logical operations assume 16-bit arguments.  Leading bits not contributing

to the value of the operation are either 0 or 1, as appropriate.  For example,

NOT 0 has a value of $FFFF, while $1234 AND $34 has a value of $0034.  Results

of a shift operation (SHL n or SHR n) do not wrap around,  ie.,  a bit shifted

right out of bit 0 or shifted left out of bit 15 is lost.

Evaluation of expressions is from left to right only.   There is no hierarchy,

and  parenthesis are not allowed.   Examples of all of the above may be had by

assembling the TEST02.ASM file.

ADDRESSING MODE SYNTAX

The  generation  of the appropriate opcode by SVASM02 is totally dependent  on

the addressing mode used.   The syntax of the instruction and its argument (if

any)  determines  the  addressing  mode,  and therefore what  opcode  will  be

generated.  The 6502 uses thirteen addressing modes, and the 65C02 uses all of

these with two additional.  The following descriptions of each addressing mode

give an example illustrating the syntax of each:

6502 Addressing Modes:

Zero Page - LDA $20

Only the low eight bits are specified and refer to an address of $00FF or

less.  The high eight bits of the address are assumed to be 0.  This is a

very efficient addressing mode for the 6502/65C02.

Absolute - LDA $125F

The  second and third bytes of the instruction contain a 16-bit  address.

Any address within the 6502 64K address space may be specified.

Immediate - LDA #$FF

The  second byte of the instruction contains immediate data requiring  no

further addressing.  The # symbol signifies immediate data.  The value of

the data is limited to 8 bits.

Indexed Zero Page - LDA $20,X or LDA $20,Y

The  second byte of the instruction contains the value of a base  address

on page zero to which is added the contents of the X or Y register.   The

result  is  the  effective  page zero address and  must  not  exceed  255

(decimal).

Indexed Absolute - LDA $453D,X or LDA $453D,Y

The second and third bytes of the instruction contain the value of a base

address  to  which  is added the contents of the X or  Y  register.   The

result is a 16-bit value which is the effective address.   The value must

not exceed 65,535 (decimal).

Zero Page Indexed Indirect - LDA ($20,X)

The  second  byte of the instruction is added to the contents  of  the  X

register,  with any carry discarded.   The result points to a location on

page  zero  which contains the low eight bits of the  effective  address.

The  next  higher page zero address contains the high eight bits  of  the

effective  address.   CAUTION  - Both  memory  locations  containing  the

effective address must be on page zero.   SVASM02 has no way of  checking

this  at  assembly  time,  so the programmer is cautioned  to  heed  this

warning.

Indirect Indexed - LDA ($20),Y

The second byte of the instruction points to a page zero  location.   The

contents  of  this location is added to the contents of the  Y  register,

producing a result which is the low 8 bits of the effective address.  Any

carry  resulting  from the addition is added to the contents of the  next

page  zero  address,  the result being the high 8 bits of  the  effective

address.  See the CAUTION in the preceeding paragraph.

Absolute Indirect - JMP ($122F)

The  second and third bytes of the instruction contain a 16-bit  address.

The  contents  of that address contain the low 8 bits  of  the  effective

address,  and the next higher memory location contains the high 8 bits of

the  effective  address.   This addressing mode applies only to  the  JMP

instruction.

Implied - TSX

The addressing is implied by the instruction,  and no further address  is

required.

Acumulator -  LSR A

This  addressing mode is but a single byte,  which specifies an operation

to  be  performed on the accumulator.   The argument must be  the  single

letter A.

Relative - BEQ *+5

The  second  byte  of the instruction contains an 8-bit offset  which  is

added to the program counter value of the next instruction.   The  result

is the new value of the program counter.   The assembler uses the present

value  of the program counter to make the calculation.   When writing the

code,  use  the  value  of the program counter at the  beginning  of  the

present line.   The assembler takes this into account when computing  the

offset,  and  the proper offset to the target is produced.   This  avoids

undetected errors in the offset. For example,

.

.

12      1000   F003               BEQ *+5

13      1002   200041             JSR FIX3     ; A subroutine at $4100

14      1005   4C0010     TARGET  JMP $1000

.

.

Line 12 (program counter = address 1000) specifies a branch to the target

at  5  bytes  beyond the present program counter to  address  1005.   The

assembled  offset  is 03,  which is the target relative  to  the  program

counter  at line 13.   This 2 byte difference is due to the way in  which

the 6502/65C02 handles the program counter and relative addressing.

The permissible range of the offset is -128 to +127 bytes relative to the

next instruction, or -126 to +129 relative to the present instruction.

65C02 Additional Addressing Modes:

Zero Page Indirect - LDA ($20)

The  second  byte of the instruction contains a page zero  address  whose

contents is the low 8 bits of the effective address.   The high 8 bits of

the effective address is in the next highest page zero location.  See the

CAUTION under Zero Page Indexed Indirect above.

Indexed Absolute Indirect - JMP ($203B,X)

The second and third bytes of the insruction contain a base address.  The

contents  of  the X register is added to it,  and the result points to  a

memory location containing the low 8 bits of the effective address.   The

high 8 bits are loaded from the next highest memory location.

In  the above examples numeric constants were used as arguments to  illustrate

the magnitude of the argument (8 or 16 bits).  In actual use, the argument may

be expressed symbolically as well.

Symbolic  zero page (8 bit) arguments must be defined prior to their use in an

expression.   While it would be correct to use a 16 bit value with the high  8

bits set to zero, to do so is not good programming practice and is not allowed

by SVASM02.  For example:

STA TEMP2

;

TEMP2 = $12

This will generate a "Forward Reference to Page Zero" error.  If the order was

reversed,

TEMP2 = $12

;

STA TEMP2

no  error  would be generated.   There are instructions where  this  does  not

apply,  however.   Those  instructions that have no zero page mode may use  an

absolute argument with a value anywhere in the range of 0 to 65,535.

ASSEMBLER DIRECTIVES (Pseudo-ops)

SVASM02 utilizes several directives,  or pseudo-operators,  for directing  the

assembly.  These include directives for page length and width control, printer

control,   listing  control,  conditional  assembly,  CMOS  (65C02)  assembly,

inclusion  of source code from an alternate file,  and definition of byte  and

word storage.  All pseudo-ops must begin with a period.  The pseudo-ops are as

follows:

.BYTE          ; Defines initialized byte storage

.WORD          ; Defines initialized word (two bytes in low/high format)

.CMOS          : Allows assembly of 65C02 instructions

.END           ; Defines the physical end of the source code

.LIST          ; Turns on listing if it had been off

.NOLIST        ; Turns off assembler listing

.INCLUDE       ; Reads source code from an alternate disk file

.PAGE          ; Controls page length

.WIDTH         ; Controls page width

.PRINTC        ; Outputs printer control characters

.IF            ; Assembles following statements if argument is non-zero

.ELSE          ; Alternate assembly if .IF argument is zero

.ENDIF         ; Terminates conditional assembly

.LISTF         ; List but do not assemble false conditionals

A detailed description of each pseudo-op follows.

.BYTE

The  .BYTE pseudo-op is perhaps the most used.   It initializes storage of its

arguments  in the code at the address of the value of the program  counter  at

the  beginning  of  the  line.   The  arguments  may  be  constants,  symbols,

expressions,  or  quoted strings.   Each individual argument must be separated

from the next by a comma.   The magnitude of each evaluated argument must  not

exceed 8 bits.   In the case of quoted string arguments, the only limit is the

length  of the line.   The delimiter for the quoted string is the single quote

character.   A  double  quote  has  no meaning in  SVASM02  other  than  as  a

character.   To include a single quote character as part of the quoted string,

use two single quotes together.  Examples:

.BYTE $45,$77,$FF,0,3              ; Constants

.BYTE SHELL + 4                    ; Expression (value of 8 bits or less)

.BYTE 'This Can''t be'             ; Quoted string with embedded

; single quote

.WORD

Similar to .BYTE,  the .WORD pseudo-op assembles 16-bit values at the  address

of  the value of the program counter at the beginning of the line.   Since the

6502/65C02  handles  16-bit values (addresses) in low  byte/high  byte  order,

the  .WORD directive assembles the 16-bit values of its arguments in  low/high

order.   The arguments to .WORD follow the same rules as for .BYTE, except the

value of each argument may be within the range of 0 to 65,535.  Examples:

.WORD START         ; The 16-bit value of START is placed in the code

.WORD ASSEM+9       ; 9 is added to the value of ASSEM, and the result

; is placed in the code

.CMOS

Directs  SVASM02  to  assemble 65C02 instructions in addition  to  6502.   The

default is off (do not assemble 65C02).   If assembly of 65C02 instructions or

address  modes  is attempted without first invoking .CMOS,  an error  will  be

generated.  The .CMOS directive has no arguments.

.END

This directive causes assembly to stop, and must be used as the last statement

in the source code.   No further assembly occurs following the .END directive.

The .END directive has no arguments,  and its omission causes an error.   .END

must be used in .INCLUDE files as well.   The .END in an .INCLUDE file  causes

termination of assembly of the .INCLUDED source statements and a return to the

primary source file.

.LIST

This  directive,  together  with its complementary .NOLIST,  may  be  used  to

selectively list the assembled source code in the output file.  If no LST file

or  device is selected at program invocation,  the .LIST directive is ignored.

The default is on (list assembler output).  There are no arguments.

.NOLIST

Turns off assembler output listing.   Source code lines following the  .NOLIST

are  counted  and assembled,  but generate no LST output.   If no LST file  or

device  is selected at program invocation,  the .NOLIST directive is  ignored.

The default is .LIST, and there are no arguments.

.INCLUDE

This directive causes SVASM02 to read source code input from the file named as

the  argument.   The  filename must be valid,  and may include  a  disk  drive

specifier.  Omission of a drive specifier causes the default drive to be used.

A pathname may not be used.  The source code in the INCLUDEd file must contain

the  .END directive.   Its omission will cause an error.  INCLUDEs may not  be

nested (only a single open INCLUDE at a time is allowed).  Example:

.INCLUDE DISTRIB.TAB     ; Source code is taken from the file named

; DISTRIB.TAB on the default logged disk.

.PAGE

This directive controls the length of the LST output page and causes a  header

to be printed on the top of each new page.   The header includes the assembler

name and version number,  the name of the file being assembled,  the date  and

time  at which assembly was started,  and the page number.   Four blank  lines

separate  the header from the output.   The default is no paging.

The  .PAGE  directive  may or may not have an argument.   If  no  argument  is

specified,  paging  is initiated at the default 58 output lines per page or at

the previous number of lines per page if a previous .PAGE n had been used.  If

a .PAGE 0 is specified,  paging is turned off if it had been on.  .PAGE n will

initiate paging at n lines per page.   The maximum lines per page is  255.   A

PAGE  may be used to cause a page eject.   The .PAGE directive is not printed,

and has no line number assigned.  Examples:

.PAGE          ; Initiate paging at 58 lines/page or the previous value

.PAGE 60       ; Use 60 lines/page

.PAGE 0        ; Turn off paging

.WIDTH

This directive controls the number of character spaces on the LST output line.

The default is 76, and includes the neccessary fill spaces for tab characters.

A tab character is assumed to be eight character spaces long,  and occur every

eight  character positions.   Beginning at position 1,  a tab can position the

print head at column 9,  17,  25,  33, etc.  If a tab character is encountered

between the tab settings,  blanks are added to position the print head at  the

next tab stop.  The .WIDTH directive must have an argument, and its value must

be  in  the range of 64 to 255.   Any input source code characters beyond  the

number  specified  in the argument are assembled,  but are ignored in the  LST

output.

.PRINTC

Many printers use "escape codes" or other control characters to control pitch,

form  length,  page width,  etc.   The .PRINTC directive is intended  to  send

initializing  or control characters to the printer.   The argument contains  a

string  that is evaluated as binary bytes by SVASM02 and is written to the LST

output  device prior to any other output.   The bytes are not counted  against

line width,  the line is not counted and it does not appear in the LST output.

There  is no default.   When used,  the .PRINTC directive should be  the  very

first statement in the source code.  Example:

.PRINTC '[x2,'      ; Causes the four byte string [x2, to be sent

; to the LST output file or device.

.IF, .ELSE, .ENDIF, .LISTF

These  pseudo-ops  will be discussed together,  since they are  the  so-called

conditional  assembly directives.

A  conditional  assembly sequence consists of the openning .IF statement  with

its  argument,  source statements,  and the closing .ENDIF statement.   If the

argument of the .IF evaluates non-zero (true),  the source statements  between

the .IF and .ENDIF are assembled.  If the argument evaluates zero (false), the

statements between the .IF and .ENDIF are not assembled.   Statements within a

false  conditional are not listed on the LST output unless .LISTF (list  false

conditionals) has been invoked.

The  .ELSE  directive is used between the .IF and .ENDIF to  permit  alternate

assembly or non-assembly of the code following,  depending on the value of the

IF argument.  This is best shown by illustration:

.IF arg

.                 If arg is non-zero, the statements up to the

.                 .ELSE are assembled and those between the .ELSE

.                 and the .ENDIF are not.

.

.ELSE

.                 If arg is zero, the statements between the .ELSE

.                 and the .ENDIF are assembled and those between the

.                 .IF and .ELSE are not.

.

.ENDIF

The  argument to the .IF is required,  and may be  any  constant,  symbol,  or

expression.  .ELSE, .ENDIF, and .LISTF do not require arguments.  There are no

defaults to conditional assembly.  Conditionals may be nested up to 16 levels.

It  should  be  noted that this version of SVASM02 does not allow the  use  of

relational operators such as EQ, LT, GT, etc. in the argument to the .IF.

ERRORS

Fatal Errors

There are three fatal errors possible in SVASM02.   Each causes the program to

stop and an error message to be displayed on the screen.   Control is returned

to DOS.

1. Directory  full - An attempt was made to create a file entry in  a

disk  directory  that contained no available  slots.   Reduce  the

number of files on the disk or use another disk.

2. Input  file read error - The source file is corrupted or otherwise

not readable.  Try another copy of the file.

3. Disk full - An attempt was made to write data on a full disk.  Use

another disk or delete files to make room.

Invocation errors

These errors are non-fatal, merely causing a reissuing of the prompt.

1. Filename  error  - An  incorrect  syntax was  used  to  specify  a

filename for the source, object, or list file.

2. Invalid drive - The disk drive specified does not exist or is  not

avaialable.

3. File  not found - The source file could not be found on the  drive

specified.

Assembler Errors

There  are  25  possible assembly time errors.   Each is  numbered,  and  when

encountered, is output to the screen along with the line containing the error.

For example,

381    2079                     .PSGE 60          ; Use 60 lines/page

*** 10 ***  Unidentified directive

The  first line is the assembler output of the line containing the error.   In

the  above  example,  381 is the line number and 2079 is the  program  counter

value.   The  remainder  of the line is as input.   The error  message  number

appears  before  the  description of the error and is  the  reference  to  the

detailed discussion of the error in this document.  If an output file is being

generated or there is output to a list device, the same error sequence appears

there.   Regardless of the output medium,  all errors are output to the screen

as shown above.   For errors occurring in lines containing instructions,  one,

two,  or  three  NOP's ($EA) are assembled depending on the  number  of  bytes

expected to be required for the instruction.

1  Undefined  symbol  - A reference was made to a symbol that has  never  been

defined.   This could be caused by a misspelling of a valid symbol or label

name,  or  omission  of a line of code.   An improper numeric  argument  or

directive name could also be considered a "symbol" and as such, could cause

this error.

2  Label  or  symbol previously defined - An attempt was made  to  redefine  a

symbol or use a label more than once.  Once defined, SVASM02 does not allow

a symbol to be redefined.

3  Syntax  error  - The stucture of the line is  not  recognized.   Check  the

statement for proper syntax, missing arguments, etc.

4  Illegal  first character in label or symbol - Labels and symbols must begin

with a letter.   Am improperly structured or used numeric argument may also

cause this error.

5  Forward  reference to page zero - All references to addresses on page  zero

must  be  known before they are used.   See the discussion on page 6  under

Addressing Modes.

6  Label  or  symbol more than 6 characters - The present version  of  SVASM02

limits the number of characters in a label or symbol to six.

7  No  .END  directive  in INCLUDE file - The .END directive  is  required  to

terminate  assembly.   An end of file was reached without finding the  .END

directive.

8  Forward  reference  in equate or origin - An expression contained a  symbol

which was not defined prior to its use to define the program counter or  an

equate.

9  (Reserved)

10 Unidentified directive - A directive or pseudo-op was found that is not  in

the  list  of valid directives.   Check for a stray leading period  or  bad

spelling.

11 Invalid quoted string - An attempt was made to use a quoted string constant

greater than two characters long, or there was a missing quote character.

12 Branch  target  out of range - The target address of a  branch  instruction

was  beyond the maximum allowed displacement from the  source  instruction.

See the discussion on relative addressing.

13 Invalid  addressing  mode - An attempt was made to use an  addressing  mode

that  is  invalid  for  the instruction.   See the file  TEST02.ASM  for  a

complete listing of all valid addressing modes for all instructions.

14 Invalid numeric argument - A numeric argument contained a character invalid

for the radix used.

15 Arithmetic overflow - The result of an arithmetic operation exceeded 65,535

(decimal).

16 No  .END directive - The last statement in the source file was not an  .END

directive.  .END must be encountered before the physical end of the file.

17 Expression  value  >  8 bits - An arithmetic operation  produced  a  result

greater  than 255 (decimal) for an instruction or addressing mode requiring

a single byte value.

18 Division  by zero - The divisor in a division operation had a value of zero

when the operation was attempted.

19 CMOS assembly not selected - An attempt was made to use a 65C02 instruction

or addressing mode before invoking the .CMOS directive.

20 Open  .IF  limit exceeded - The number of open  conditional  assembly  (.IF

directive) exceeds the maximum of 16.

21 Forward  reference  in  conditional - The value of the argument in  an  .IF

directive was unknown at the time the .IF was encountered during the  first

pass.  The argument value must be known before the .IF can be openned.

22 No  open  .IF - An .ELSE or .ENDIF was encountered for which there  was  no

corresponding .IF directive previously specified.

23 .INCLUDE  file  already open - An attempt was made to .INCLUDE a file  that

has already been openned (.INCLUDEd).

24 INCLUDE filename error - A syntax error has occurred in the filename  given

in an .INCLUDE directive.

25 Invalid drive name - The disk drive specifier in a filename to be .INCLUDEd

is invalid.

26 INCLUDE file not found - The file specified in an .INCLUDE directive cannot

be  found on the specified disk drive or currently logged disk if  no  disk

specifier is used.

APPENDIX A

SVASM02 LST Output File Description

The  output fragments which follow were extracted from the TEST02.LST file  to

illustrate  the  format of the assembler output.   In general,  the  first  33

spaces  on  a line are generated by SVASM02.   The remainder of the line is  a

copy of the input code.

.

.

18  = 0047            		QUOTE1='G'		;SINGLE QUOTE

.

.

Line 18 contains an equate.   The line number appears in the first four spaces

on the left.   The = signifies a valid equate, having the hexadecimal value of

0047.   The  several  spaces between the value and QUOTE1  are  the  remaining

spaces  up  to column 32,  followed by tab expansion spaces up to the  QUOTE1.

The spaces between the 'G' and the ; are tab fill spaces such that the comment

delimiter (;) starts in a column which is a multiple of 8,  plus 1.  The input

source code contained tabs in these spaces.

.

52                    	;

53                    	;  BYTE AND WORD DIRECTIVES

54                    	;

55    1006  0D0A5356  		.BYTE $0D,$0A,'SVENSON ELECTRONICS','$'

100A  454E534F

100E  4E20454C

1012  45435452

.

Lines  52  through 55 are the output from commentary information and  a  .BYTE

directive.   The ; in lines 52-54 are in column 33, having been in column 1 in

the  input  file.   The  byte  output begins in line  55.   The  1006  is  the

hexadecimal address (program counter) at which the bytes were assembled.   The

eight  hexadecimal  characters which follow represent the value of  the  first

four bytes of the data assembled.  The remainder of the line is as input.  The

three  unumbered  lines following are the remaining output for the  directive.

For  .BYTE  and  .WORD  directives,  the line number  is  not  incremented  if

additional  lines  are  required to complete the output  of  the  data.   Line

numbering resumes at the next statement.

.

65    1024  AD0230    		LDA PORT1

66    1027  29FF      		AND #$FF

67    1029  8D0530    		STA DAT1

.

Lines  65-67  are typical instruction code output.   Note that  addresses  are

listed in lowbyte/high byte order exactly the way they appear in memory.

.

86    102F  EAEA      		BRA *-126

*** 19 *** CMOS Assembly not selected

.

Line  86  contains an error,  with the error message on  the  following  line.

Since  the  branch  instruction is expected to generate two bytes  of  machine

code, two NOP's (EA) are inserted in place of the code that would be generated

had  there been no error.   In the error message line,  the 19 refers  to  the

error  number,  a discussion of which may be found in the  main  documentation

under ERRORS.

.

END OF SVENSON ELECTRONICS 6502/65C02 ASSEMBLER - VERSION 1.0

BYTES ASSEMBLED:    510         NO. OF ERRORS:   46

SYMBOL TABLE:

LINE

SYMBOL    VALUE   DEFINED             CROSS REFERENCES

BIAS      0007      48

BVAL1     009D      34       47

BVAL2     009D      35       51

CHECKP    2091     381

The  above output fragment appears at the end of the code output.   The number

of  bytes  asembled  is  given,  and represents  the  actual  count  of  bytes

assembled.    Gaps  in  the  address  space  of  the  program,   such  as  for

uninitialized storage, are not counted.  The number of errors is also stated.

The  alphabetically  sorted  symbol table  follows,  with  appropriate  column

headings for the symbol name,  its hexadecimal value,  the decimal line number

in which it was defined,  and decimal listing of cross-reference line numbers.

The above example shows only a single reference for two symbols,  but multiple

references  are  listed  on the same line up to the maximum number  of  spaces

allowed by the WIDTH directive.   Any additional references are listed on  the

following lines.

APPENDIX B

SVASM02 Object File Format

The  object  file consists of records of ASCII hexadecimal characters  in  the

Intel format.  A typical record looks like this:

:101006000D0A5356454E534F4E20454C45435452B8

Byte 0        A colon character, signifying the beginning of the record

Bytes 1-2     Hexadecimal value indicating the number of loadable bytes in the

record.  The maximum is 10 (16 decimal).

Bytes 3-6     Hexadecimal  address  where  first byte of the  record  will  be

loaded.

Bytes 7-9     Always 00.

Bytes 10-on   Hexadecimal characters representing the actual data bytes.

Last two      A  checksum.   The checksum is the two's complement of the 8-bit

bytes  sum of the value of all bytes (not the ASCII) with  carry

discarded.   It includes the byte count, load address bytes, and

loadable data bytes.

The  last record contains values of zero for byte  count,  load  address,  and

checksum.  It effectively is :0000000000.