**************************************************************

INTERRUPT 49H INTERFACE FOR MAGIC

**************************************************************

This interface allows external programs to control MAGIC TSR.

The default interrupt number is changable via command line switch

/Inn, e.g. /I64 would set interrupt 64 hex.

In order for caller to identify whether MAGIC holds the interrupt 49h,

@@--NEW-->@@

there is a 6 byte ID stored immediately preceeding the interrupt service.

The ID is: " MAGic" (case as shown, quotes are not part of the string).

For MAGic Deluxe the ID string is "xMAGic". The ID string will not wrap

around the segment boundaries, thus the offset of the interrupt vector

must be at least 6.

ID can be used to identify MAGIC and also to find which interrupt

vector is used - application can examine 256 interrupt vectors

and check 6 bytes preceeding the service routine. As an additional

check, in the same segment at offset 0 there will be standard

CodeRunneR ID: RT followed by 4 byte TSR ID: VMAG (for VGA MAGIC).

INT 49h OPERATIONAL CONVENTIONS

1. Caller's registers AX,BX,CX,DX are modified.

Registers DS,ES,SI,DI,BP,SS,SP and Flags are preserved.

2. Function number (1,2,..) is passed in AX register.  Any other

arguments are passed in BX,CX,DX.

4. Operation status is returned in AX. Any other return values

are passed in BX,CX,DX.

5. The following return status values (AX) are common to all

functions:

-1	TSR was busy, try same call later

-2	Invalid function number

-3	Function works only in magnified mode

-4  Function works only in unmagnified mode

-5  Function works only for MAGic Deluxe

6. Horizontal (X) and vertical (Y) coordinates or sizes are passed

(or returned) in registers DX for X and BX for Y.

7. The coordinates/sizes are measured in the same units as

those used by VGA hardware:

a) In graphics modes

X: 8 pixel units

Y: 1 pixel units

b) Text modes

X: character columns

Y: character lines

All coordinates are 0 based, sizes are 1 based.

8. Interrupts are enabled during command execution and any other

MAGIC operations are locked out.

*************************************************************************

FUNCTIONS

*************************************************************************

--01-- Magnify ON

ENTRY:	AX = 1

EXIT:   AX = 0	Cannot magnify in current video state

= 2	Magnified in text mode

= 3	Magnified in graphics mode

--02-- Magnify OFF

ENTRY:	AX = 2

EXIT:	AX = *	Same as function 1

--03-- Capture screen point (e.g. mouse cursor)

ENTRY:	AX = 3

DX = X

BX = Y

EXIT:	AX = 0	Ok

Specified X,Y point is made part of magnified window. No window

movement is done is the point is already within the window.

--04-- Reposition magnified window

ENTRY:	AX = 4

DX = X of upper left corner

BX = Y of upper left corner

EXIT:	AX = 0	Ok

Sets upper left corner of the magnified window to requested

coordinates. If the coordinates are outside of valid screen

boundaries, the nearest position is set.

--05--  Get position of magnified window

ENTRY:	AX = 5

EXIT:	AX = 0	Position in DX,BX is valid only if AX is 0.

DX = X

BX = Y

--06--  Get full screen size (valid only in magnified mode)

ENTRY:	AX = 6

EXIT:	AX = 0	Sizes in DX,BX is valid only if AX is 0.

DX = Horizontal size

BX = Vertical size

--07--  Get magnified window size

ENTRY:	AX = 7

EXIT:	AX = 0	Sizes in BX,DX is valid only if AX is 0.

DX = Horizontal size

BX = Vertical size

The sizes returned are always smaller or equal than those

of full screen (equal only in modes where no magnification

is possible).

--08--  Set text mode magnification size

ENTRY:	AX = 8

BX = Scaling factor 1,2,4,6,8,9

Values 4,6,8,9 are available only for MAGic Deluxe

EXIT:	AX =  0  Ok

= -4  MAGic is in magnified state, can't set size

= -5  Requested size works only for MAGic Deluxe

This function may be called only in unmagnified state to set

the size for subsequent magnify-on call (func AX=1).

Regular MAGic supports only BX=1 and 2 (magnifications 1.4 and

2 times). MAGic Deluxe supports additional values BX=4,6,8,9

(magnifications 4,6,8,12 times).