Packet Send Utility

FTP Packet Driver Interface, and Front End.

Author:        Patrick Whittle

Date:          November 4, 1994

Revisions:     February 24, 1995

January 6, 1997

Purpose:       Provide interface to network packet driver, and MAC level

communication for ethernet networks.  See documentation in

PKTSEND.C for more information.

This program is distributed in the hope that it will be useful,

but WITHOUT ANY WARRANTY; without even the implied warranty of

MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE.

Overview:

This program may be distributed as long as the code is not changed in any

way, and the author's name is intact.  The files in this archive are ready

to use; no 'install' needed.  Please support the shareware concept.

The program's purpose is to provide an interface to an FTP packet driver and

enable access to ethernet media access control frames.  Using an FTP packet

driver, PKTSEND.COM transmits raw packets directly to the ethernet via DOS

interrupt calls.  The text mode interface accepts addresses and data from the

user, and passes it to FTP via interrupt 60h (interrupts 60h through 7Fh

are used by FTP drivers).  The packets are sent as 802.3 media access control

frames directly to the NIC.  In addition to address and data, PKTSEND.COM uses

an ethernet type field to identify the type of protocol being used.  The default

type field used in PKTSEND is 0x0800 to indicate IP.

+----------------------------------------+

|             PKTSEND.COM                |

|  Ethernet Media Access Control Frames  |

|   Destination - Source - Type - Data   |

+----------------------------------------+

||

+------------------------------------+

|       FTP Packet Driver  /         |

|      board specific driver(s)      |

+------------------------------------+

||

+------------------------------+

|      Ethernet board(s)       |

+------------------------------+

||

=================================== Ethernet wire

Use PKTSEND.COM along with a network utility such as SNOOPER to observe

generated network activity.  Use this utility to demonstrate media access

control frame transmissions on ethernet networks.

Usage:

Addresses entered in PKTSEND are sent to an FTP packet driver as broadcasts

with specific source and destination addresses (broadcast here does not infer

network broadcast 255.255.255.255, but a MAC level ethernet transmission).

Observe the results of keystrokes using software on a second network terminal.

Install FTP network packet drivers on two terminals, run PKTSEND.COM on one

of them, and SNOOPER on a second.  Data sent over the ethernet via PKTSEND.COM

will be intercepted by SNOOPER to be recorded.

When no network is installed, load DUMMY.COM into memory.  This is a TSR

program loaded at DOS that accepts packets via FTP interrupts and discards

them (a null device driver).  Using PKTSEND.COM and DUMMY.COM together

demonstrates the intended use of this software.

Files in this archive:

PKTSEND.COM        Packet Send Utility, copyright (c) Patrick Whittle, 1995-7

PKTSEND.DOC        This documentation file

PKTSEND.C          Source code and main program

8086.H             Include file used by PKTSEND.C

DUMMY.COM          FTP Packet Driver Emulator v1.02 Copyright (c) Patrick Whittle, 1995-7

DUMMY.ASM          Source code for dummy packet driver (null device driver)

TRACE.COM          Packet driver tracer version 9.0 copyright 1988-89, Russell Nelson.

TRACE.ASM          Source code for packet driver tracer program

DEFS.ASM           Required by TRACE.ASM

DUMP.EXE           Dump program to view output from TRACE program

DUMP.C             Source code for DUMP program

PKTDRVR.C          Turbo C Driver for FTP Software's packet driver interface.

* Graciously donated to the public domain by Phil Karn.

DIS_PKT9.ZIP       Provides an Ethernet or a Token Ring Packet Driver interface to programs

built to operate over Packet Drivers.

WINPKT.ZIP         Provides a Packet Driver interface between Windows 3 Enhanced mode

applications and a real Packet Driver.

NDIS3PKT.ZIP       A Windows VxD (Virtual Device) which provides a packet driver interface

on top of NDIS version 3.0 or 3.1

Using PKTSEND when no network is present:

Load DUMMY.COM

Load TRACE.COM

Run PKTSEND.COM and transmit some data

Exit the Packet Send Utility, and type 'exit' at DOS to remove the TRACE TSR

Run DUMP.EXE to view resulting packet data

Using PKTSEND when a network is present:

Load an FTP Software FTP packet driver

Load TRACE.COM

Run PKTSEND.COM and transmit some data

Exit the Packet Send Utility, and type 'exit' at DOS to remove the TRACE TSR

Run DUMP.EXE to view resulting packet data

Contact the author via e-mail:

Internet:    Patrick_Whittle@PRAXAIR.COM    OR

patrick@xiris.com