page 1

----------- SERIOUS Users Guide -----------

SERIOUS version 2.21 -- serial communications device driver

Copyright (c) 1989,1990 by  Norman J. Goldstein

132 Powell Street

Vancouver BC

Canada  V6A 1G1

This software is distributed under the Shareware concept.

- You are encouraged to copy and distribute the file  SERUS221.ZIP ;

in its original, unmodified form only.

- No fee is to be charged for the software, other than the actual

cost of copying and distribution.

- If you end up using  SERIOUS , on other than an evaluation basis,

you are obliged to send in a registration fee to the copyright owner.

The last section of this document has details on this.

The copyright owner is not liable for damages of any kind resulting

from the use of this software.

page 2

Contents

Section                                  Page

1     Introduction                      3

2     How  SERIOUS  works               3

3     Function specifications           4

4     Obtaining the entry address       7

5     The inner workings                8

6     Revisions (future and past)       8

7     Registration                      9

page 3

Section 1 -- Introduction

SERIOUS is a device driver for the serial port of the IBM PC and

compatible computers.  It is not a communications program.  It

does provide the tools to write your own communications program.

There are, of course,  BIOS and DOS routines for serial com-

munications, but these are weak, and are not used in serious

programs.  The  SERIOUS  device driver is intended to fill this

gap.

Section 2 -- How  SERIOUS  works.

The program is a DOS installable device driver.  The syntax for its

entry in the config.sys file is

device=serious.sys  [name]  [ord]

The 2 parameters  name  and  ord  are optional, and may be placed

in any order, separated by at least 1 space.

name -- This is the file name by which DOS will recognize the

installed driver.  The default name is  SERIOUS0 . The 1st

character must not be a digit.  Otherwise, the usual DOS

file naming conventions apply; in particular, the name must

be at most 8 characters.

ord -- The driver may be configured to contain a buffer in

which to place incoming characters.  The default is that

there be no such buffer.  If  ord  is a number between

1 and 16 , then the driver will contain such a buffer.

Function 0 (open) of section 3  explains how users may

supply their own buffer for the driver to use.

The buffer has length  2  to the power of the order.  The

highest allowable order is  16 , which would give the buffer

the size of a complete segment -- 64K bytes. The smallest

allowable order is  1 , creating a meagre  2  byte buffer.

Two advantages to creating and using a default buffer within

the driver are 1) Less work to later open and use the driver,

and 2) The buffer remains in RAM even if the application

program is terminated; this is a convenient way for the driver

to be reaccessed without any incoming characters having been

lost, assuming that the terminated program did not close

the driver.

The disadvantage to creating a default buffer within the

driver is that it may not be deallocated.  The driver

code occupies less than  1300  bytes in RAM; having a 2K

buffer would more than double the RAM being used.

The driver contains 5 functions, which may be accessed by the

applications program.  They are described in the next section.

page 4

Section 3 -- Function specifications

The functions are accessed by the application filling CPU registers

with appropriate values, and then performing a far call to the

entry address of  SERIOUS .  The way to obtain the entry address

is explained in section 4 .  Examples are in the file  serface.c ,

which contains routines for interfacing with  Turbo C .  These

examples also illustrate how to construct an assembler interface

to  SERIOUS .  The file  serdemo.c  is a Turbo C example of an

application using the interface routines of  serface.c  ; it is a

dumb terminal.

Every function has the following in common.

Input:   bx = function number.

Returns  bx = error code.

A value of  0  indicates no error.

The file  serface.h  defines the numeric values of

all the error codes.

------------------------------------------------------------------

Function 0: Open

Purpose: Inform the driver of the user's configuration, and set up

the hardware interrupt routines.

Input:   bx = 0 .

ax = base port number of the serial chip.

ch = interrupt request number (IRQ) of the serial chip.

cl = order of the user supplied input buffer.

es = segment of the user supplied input buffer.

di = offset of the user supplied input buffer.

The 'order' and purpose of the input buffer is described in

section 2 . A reasonable order is  10 , for a buffer of  1K .

At  1200 baud, a continuous input stream from the serial port would

fill up a  1K  buffer in about 10 seconds.  A special case is

entering a value of  0  for the order.  This informs the driver to

use the default buffer, which is contained within the driver.  The

size of this default buffer is determined at installation as a

command paramater in the config.sys file, as explained in section 2 .

If  port  is specified as  0 , then this function  merely reports

whether the driver is currnetly open.  No other input information

is used, in this case.

Return errors: ERR_Active   - driver was already open.

ERR_BadOrder - the  order  is not in the allowable

range of between  1  and  16 , or

neither a default nor a user-defined buffer was

specified.

ERR_BadIrq    - The Interrupt Request number is

not between  0  and  15 .

ERR_NotActive - this is possible, if  port  is

specified as  0 .

------------------------------------------------------------------

page 5

------------------------------------------------------------------

Function 1: SetParms

Purpose: Set the transmission parameters.

Input:   bx = 1 .

ax = baud.

ch = stop , cl = data.

dx = parity.

baud = rate of transmission, between  2  and  65,535

bits/second.  Some common values are  300 , 1200  and

2400 .  The driver has been tested at  9600 , using a

hardwire connection to a network.

data = the number of data bits in each character,

between  5  and  8 .  Common values are  7  and  8 .

stop = the number of stop bits, either  1  or  2 . (Actually,

for  data  of length  5 , only  1 1/2  stop bits are

generated when  stop  is specified as  2 .)

parity is the code to specify the type of parity generated.

0,2,4,6: no parity bit.

1: odd parity.

3: even parity.

5: mark parity (Parity bit = 1).

7: space parity (Parity bit = 0).

When  SERIOUS  is opened, the default paramater settings are

1200 baud with 8 data bits, 1 stop bit and no parity.

Return errors: ERR_BadData , ERR_BadStop , ERR_BadParity , or

ERR_BadBaud  indicates the specified parameter is

out of range.

------------------------------------------------------------------

------------------------------------------------------------------

Function 2: Close

Purpose: Disable the UART, and restore the hardware interrupt

vectors to their values before the previous call to

Open .  It is important to close  SERIOUS  before

deallocating the input buffer.  Otherwise, characters might

be written to the old location of the input buffer!

Input    bx = 2 .

Return errors: ERR_NotActive - driver was not open.

------------------------------------------------------------------

page 6

------------------------------------------------------------------

Function 3: SendChar

Purpose: Send a character out through the serial port.

Input al = character to be sent.

bx = 3 .

Return errors: ERR_UARTsleeps - UART is not responding.

------------------------------------------------------------------

------------------------------------------------------------------

Function 4: RecvChar

Purpose: Receive a character from the serial port.

Inpput:   bx = 4 .

Output:  ax = the next character (ah=0), if there is one, or

ax = -1 , if there is no character.

Return errors: none

------------------------------------------------------------------

------------------------------------------------------------------

Function 5: SetMode

Purpose: Specify which interrupts to activate on the UART .

Inpput:   bx = 5 .

al = interrupt mask.

A bit is set if the interrupt is to be active.

bit 0 -- Received data.

1 -- Transmitter holding register empty.

2 -- Line error.

3 -- Modem status change.

The driver routines for reception and transmition require that

bits 0 and 1 be set.  When the driver is opened, these are the

only set bits in the mask.

Bits 2 and 3 are useful for debugging.  They have the following

effect, when the appropriate bit is set.

When a line error is detected, the byte  ^L  is inserted into the

input stream, followed by the byte content of the Line Status Register

of the UART .  The relevant bits of this register are described.

Bit   Meaning when set:

1 -- Overrun. A received character was overwritten before being read.

2 -- Parity. A wrong parity bit was received.

3 -- Framing. A stop bit was not received where expected.

4 -- Break interrupt.

page 7

cont Function 5 -- SetMode

When a modem status change is detected, the byte  ^S  is inserted into

the input stream, followed by the byte content of the Modem Status

Register of the UART.  The bits of this register are described.

Bit   Meaning when set:

0 -- Change in "Clear To Send" .

1 -- Change in "Data Set Ready" .

2 -- A ring has just completed.

3 -- Change in "Data Carrier Detect" .

4 -- "Clear To Send" .

5 -- "Data Set Ready" .

6 -- Ring indicator.

7 -- "Data Carrier Detect" .

When  ^L  or  ^S  is being used in this special fashion, and is

received through the serial port, the driver preceeds it  by the

backslash character  '\' ;  should  '\'  be received, it too is

preceeded by a backslash!  In other words, the backslash is used

as an escape character, to denote that the next character is to be

taken literally.  These complications are introduced only if either

bits 2 or 3 is set in the interrupt mask; otherwise all characters in

the input stream have been actually received through the serial port.

Return errors: none

------------------------------------------------------------------

Section 4 -- Obtaining the entry address

In the earlier versions of  SERIOUS , this was an awkward

procedure, as the driver was implemented as a TSR .  Not so,

any more!

The steps outlined in this section are implemented in the

function  SetEntryAddress()  in the file  serface.c .

The default device name for the driver is  SERIOUS0 , although

this may be changed at installation, as explained in section 2 .

This device must be opened, as any regular file, to obtain a

descriptor from  DOS .  This descriptor is used to perform an

"i/o control" read from the driver; this places the entry address

into a user supplied variable i.e. into a 4 byte buffer.

page 8

Section 5 -- The inner workings

The PC serial communications chip is called a UART (Universal

Asynchronous Receiver Transmitter).  As characters are received by

the UART through the serial port, SERIOUS  automatically places them

into an input buffer (specified by the user), using the hardware

interrupt system of the UART.  This buffer is implemented as a

circular buffer.  As soon as wrap-around occurs, the buffer

becomes 'empty'. The user must read characters from the input

buffer sufficiently often to ensure this does not happen. This is

not difficult.  A  1K  buffer takes about 10 seconds to fill up at

1200 baud.  There is plenty of time for the CPU to process

the characters.

When open, the driver intercepts 3 interrupts:

0Ch -- UART interrupt, mentioned above.

1Ch -- Clock interrupt, for timings.

23h -- DOS break (^C) interrupt.  The driver is automatically

closed.  This means that it is safe, as far as SERIOUS is

concerned, to terminate your application using  ^C .

Section 6 -- Revisions (future and past)

Some possible enhancements are:

- Allow changing the input buffer.

(Now, SERIOUS must first be closed, and then opened to change

the size or location of the input buffer.)

- Specify/change an output buffer, at any time.  This allows for

smoother transmission;  the interrupt system of the UART would,

more completely, handle the sending of characters out through

the serial port.

(Now, there is no output buffer, and spin waits are used until

the UART is ready to send the next character.)

- Support the break interrupt facility of the UART.

Revision history:

This is off to a late start.  Better late than never!

23 Aug 1990: Version 2.21 -- Bug fix.  Now ports other than COM1

can actually be used.

page 9

Section 7 -- Registration

Single users are asked to send $20 to the address on the cover page

of this document.  For evnironments with multiple machines, please

contact me to arrange an agreement.

In any event, I would like to hear your comments about this software.

For registered users, I certainly intend to give detailed responses,

as well as early update information.

----------------------------------------------------------------

IBM  is a registered trademark of

International Business Machines Corporation .

Turbo C  is a registered trademark of  Borland International .

I made heavy use of the document

"NS16450/INS8250A/NS16C450/INS82C50A

Asynchronous Communications Element" , Feb 1985

by National Semiconductor

which I obtained, along with other helpful information, from

Hamilton Avnet Electronics  in  Burnaby, BC .