WADE - Wagners Debugger

User Manual

Version 1.5 - 85-04-27

WADE  is an interactive symbolic Z80 debugger with  full

assembly and disassembly using standard ZILOG mnemonics.

Up to eight conditional and/or unconditional breakpoints

plus a temporary breakpoint may be defined. Full tracing

with  or  without list and with real-time  execution  of

subroutines  on  command or automatic  (using  protected

areas)  is provided.  Tracing may be controlled by inst-

ruction count or a conditional expression. A full set of

operators provides for arithmetic,  logical,  shift, and

relational operations on hex,  decimal, binary, and cha-

racter data,  and on registers,  variables, and symbols,

including  embedded assignments to registers and  varia-

bles.  Extended  addressing can be provided for  systems

with banking or memory management capabilities.

WADE supports parameterised command macros and conditio-

nal command execution.

WADE  is  supplied  with complete source  code  for  the

Digital Research RMAC assembler.  System dependent  rou-

tines  like  console i/o,   file i/o,  and  banking  are

collected in a single module to allow easy adaptation to

other  operating systems or stand-alone ROM-based appli-

cations.

No Copyright

Wade was written 1984-1985 by

Thomas Wagner

Patschkauer Weg 31

D-1000 Berlin 33

West Germany

BIX mail: twagner

It has been released to the public domain in 1987.

You may use and modify this program, its sources, or any

parts  of it,  in any way you choose.  No part  of  this

program  is  copyrighted any longer,  even if  copyright

notices still appear in some of the files.

No  contribution  in any way is  requested  or  expected

(although I wouldn't reject it :-)).  I've given up CP/M

and  Z80  programming,  so  this software is  no  longer

supported.

I'll  try  to  help  and  answer  questions  about  this

software, but I haven't touched it for quite a while, so

don't expect too much.

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 2

Command Summary

A  {addr}             Assemble

B                     Display Breakpoints

B  adr {adr..}        Set Breakpoints

BI mexpr ;adr {adr..} Set Conditional Breakpoints

BX                    Clear all Breakpoints

BX adr {adr..}        Clear specified Breakpoints

BXI                   Clear Break Condition

C  {N}{J} {count}     Trace over calls {Nolist}{Jumps}

C  {N}{J} W mexpr     ..While

C  {N}{J} U mexpr     ..Until

D  {from {to}}        Dump memory

E  mexpr ;command     Execute command conditionally

F  {command line}     Specify Filename & command line

G  {to} {; breakadr}  Go {with temp breakpoint}

H                     Display Low and High addr of file

H  expr {expr..}      Display result of expression(s)

I  {port}             Input from port

J  file {params}      Jump to macro file

K                     Kill macro file

L  {from {to}}        List disassembled code

M  begin end dest     Move memory

N                     Name (Symbol) list

N  expr symname ...   Define Names

NF filename           Define Symbol File Name

NS num                Reserve Space for symbols

NX                    Delete all Names

NX symname ...        Delete specified Names

NR {offset}           Read Symbol File

NW                    Write Symbol File

O  {byte {port}}      Output a byte to port

P                     Display protect condition

P  mexpr              Define protect condition

PX                    Delete protect condition

Q  {J} begin end str  Query {justified} for bytes

R  {offset}           Read a File

S  {addr}             Substitute memory

S  addr bytestring    Substitute immediate

T  {N}{J} {count}     Trace {Nolist} {Jumps only}

T  {N}{J} W mexpr     ..While

T  {N}{J} U mexpr     ..Until

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 3

U                     User input trap

V  begin end begin2   Verify (compare) memory

W  start end {offset} Write a file to disk

X                     Examine CPU state

X'                    Display alternate Registers

X  regname            Display & Change Register

X  regname expr       Change Register

Y                     Display Y-Variables

Y  n                  Display & Change Y-Variable n

Y  n  expr            Change Y-Variable n

Z  begin end bytestr  Zap (fill) memory with a string

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 4

Using WADE

Syntax:  WADE  { filename { symbol-filename }}

WADE is called like any other CP/M command. It relocates

itself  below the BDOS and sets the address at 6 corres-

pondingly (for CP/M 3,  this is done by the system using

the RSX mechanism).

If a filname is specified in the command line, this file

is  then read into memory,  as if an R-command had  been

entered. Please note that you have to issue an F-command

to  clear the filename from the default FCB if the  pro-

gram under test expects parameters.

If  a second (symbol-) filename is given in the  command

line,  this file is read as a Symbol File,  as if an  NR

command  had  been entered.  Note for CP/M 3,  that  the

program file is read first, so if a program with attach-

ed RSX is loaded,  there may be not enough space to read

all  symbols.  In this case,  the symbols should be read

before the program (see NF, NR and R commands).

Example:    WADE myprog.com myprog.sym

To  exit  WADE under CP/M,  issue a G command  with  the

parameter 0. This will reset the BDOS-pointers and warm-

boot the operating system.

Example:    g0

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 5

Command input

Commands  and  parameters  may be entered  in  upper- or

lowercase.  Spaces  may be used freely as separator  be-

tween operands,  commas may be used to separate  parame-

ters.  The  maximum  accepted  input line length  is  79

characters.

The  line  is interpreted only after a CR has  been  en-

tered,  so  you can edit the input using the DEL  or  BS

key.  A TAB is interpreted as a space,  a LF is ignored.

All other control characters are refused.

Auto Command Repeat

All  entered  commands except 'G' are saved and  re-exe-

cuted if a carriage return only is entered.  Parameters,

however,  are not saved,  so the default values will  be

used if applicable. If parameters must be specified with

a command, auto repeat is not possible.

Display Control

Display output can be stopped at each line end by  ente-

ring  ^S (control + S),  and continued by ^Q (control  +

Q).  Hitting  the space key once will also stop the dis-

play,  hitting it again (or entering ^Q) will let output

continue.

Command abort

Every  command  which produces output can be aborted  at

the  end of each output line by entering  any  character

other than space,  ^S,  or ^Q. Also, a trace with Nolist

can  be aborted at any time the program does not execute

in real-time.  Aborting a command will also kill a  cur-

rently active command macro.

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 6

Expressions

All  numbers  in commands and assembly lines may  be  an

expression,  except  for "X regname",  where a  register

name, not a register designation (Rx) must be used.

An expression has the general form

factor { operator factor ...}

where operator is one of the following:

+       addition

-       subtraction

*       multiplication

/       integer division

%       remainder of integer division (modulus)

&       bitwise AND

!       bitwise OR

|       bitwise OR  (alternate representation)

#       bitwise XOR

^       bitwise XOR (alternate representation)

<<      circular 16-bit left shift

>>      circular 16-bit right shift

<       less than

<=      less than or equal to

>       greater than

>=      greater than or equal to

=       equal

<>      not equal

&&      boolean AND

||      boolean OR

!!      boolean OR (alternate representation)

:=      assignment (word)

==      assignment (byte)

where  the factor which is assigned to may be

an  unsigned register or variable  specifica-

tion, or an address value. The result of this

operator  is the value on the right-hand side

of the assignment operator,  which is at  the

same time assigned to the register,  variable

or address specified on the left-hand side.

Size  adjustment  is automatic for  variables

and  registers.  If an address is used as the

destination,  a  word is stored.  To store  a

byte only,  use the operator '=='.  Note that

to assign to a register or variable,  no sign

or  expression may be used with the  register

or variable name, i.e. the expression

RHL := 1234  will assign 1234 to register HL,

whereas  +RHL := 1234 will assign 1234 to the

address contained in register HL.

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 7

:       extended memory specification

the  value  of this operator is the value  on

the  right-hand  side of  the  operator.  The

value on the left-hand side specifies a (sys-

tem  dependent) extended  address,  which  is

used  in memory references and with  commands

which expect an address.  In all other cases,

this value is ignored. An error occurs if the

extended  address is undefined in the  target

system,   or   if  extended   addressing   is

disabled.

A factor has the form

{ sign } number

where sign is

+ (plus), - (minus), or ~ (not)

and number is

(expression)    the byte at the memory location

addressed by "expression"

(expression).   the word at the memory location

addressed by "expression"

[expression]    the value of the expression

hhhh    hex number

dddd.   decimal number

bbbb"   binary number

'c'     character

string  character string (only the last 2 characters

are significant)

Rx      contents of CPU-Register x

Yn      contents of Variable Y0..Y9

H       special variable H(igh)

L       special variable L(ow)

M       special variable M(ax)

T       special variable T(op)

X       special variable eXtended address default

$       CPU-register PC

.symbol Value of the symbol

The  variable  L contains the standard  load  address

(100h  for CP/M) and is not changed by the  debugger,

but may be user assigned to a different value.

The  variable H contains the highest address read  on

the  last file.  It is updated each time an R-command

is executed.

The  variable M contains the highest address read  on

all previous R-commands.

The  variable T contains the top address of the  user

TPA. It is updated if symbol table space is expanded.

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 8

The  variable  X  contains the default bank  for  all

operations if extended addressing is enabled.  It may

be  changed by assignment or by changing the PC  with

an extended address.

The short-form '$' for PC may only be used in expres-

sions, not in an X-command.

The character '_' (underline) may be used in  numbers

to enhance readability. It is completely ignored.

String:

Any number of characters delimited by quotes (').

Use  a  two quotes ('') to represent a  single  quote

within a string: 'It''s a quote'.

Register names:

primary:    A, F, AF, B, C, BC, D, E, DE,

H, L, HL, IX, IY, SP, PC

alternate:  A', F', AF', B', C', BC', D', E', DE',

H', L', HL'

control:    IFF  (interrupt enable flip flop)

I    (interrupt register)

R    (refresh register, read only)

Symbols:

Any  number  of characters,  of which only the  first

eight  characters  are significant  (the  significant

length  of symbols can be changed  by  reassembling).

The first character must be non-numeric.  Symbols may

consist of letters,  digits,  and the special charac-

ters @, ?, _, and $. Lowercase letters are translated

to uppercase. Any underlines (_) are stripped.

Multiple Expressions

All  commands expecting a condition will accept multiple

expressions  in  sequence.  Only the value of  the  last

expression is used as the condition result.  The general

form for this "mexpr" is

expression  { {,} expression ... }

Byte-Strings

The commands Q(ery),  S(ubstitute),  and Z(ap) expect  a

byte-string  as  operand.  In a  byte-string,  character

strings  are  significant over their  full  length.  The

general form is

expression-or-string { {,} expression-or-string ... }

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 9

only  the lower byte is significant for  expressions  in

byte strings.  Note that a string is evaluated first, so

to  enter  the expression 'N'-40 in a  byte-string,  you

have to use brackets:

'N'-40  1+2*3  'A'+2      => 'N', -40, 7, 'A', 2

['N'-40] [1+2]*3 ['A'+2]  => '^N', 9, 'C'

Order of Evaluation

The precedence of operators is as follows (high to low):

expression delimiters       (), ()., []

signs                       +, -, ~

bitwise operators           &, !, |, #, ^

multiplication operators    *, /, %, <<, >>

addition operators          +, -

relational operators        =, <>, <, >, <=, >=

boolean operators           &&, ||, !!

extended address            :

assignment                  :=, ==

Operators  of  equal  precedence are evaluated  left  to

right,  and  must be bracketed if a different  order  of

evaluation  is desired.  Assignments are evaluated right

to left.

Examples:

1 + 2 * -3       => -5

5 - 2 : 2 * 3    => Bank 3, Address 6

3:5+2 == (3:6)   => Assign contents of 3:6 to 3:7

rhl := +rde := 1 => Assign 1 to register HL and the

address contained in DE

rhl := (rde). := 1 => same as above

The  nesting  depth  of expressions is  limited  by  the

available debugger stack space.  In the current release,

256  bytes  are available for the  stack,  so  that  any

reasonable expression should cause no problems.

Booleans

For commands expecting a conditional expression, and for

the  boolean AND/OR operators,  the value  0  represents

FALSE, while any value other than 0 represents TRUE.

The result of the relational and boolean operators is  0

for TRUE and FFFF for FALSE.

An  example  to show the difference between bitwise  and

boolean operators:

1 &  2  is 0

1 && 2  is FFFF.

1 |  2  is 3

1 || 2  is FFFF.

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 10

Commands

A - Assemble

Syntax:    A  { start-address }

If  a start address is not specified,  the last  address

displayed in an A or S command is used.

The  current  contents of the location are displayed  in

hex  and in disassembled form,  and a line  of  assembly

code is expected. The code must be specified in standard

ZILOG mnemonics,  with operands separated by a comma. No

abbreviations  are allowed.  The entered instruction re-

places the current one,  and the command will advance to

the next instruction.

If you enter an empty line,  the command will advance to

the next instruction.  To terminate the command, enter a

dot (.) as first character in a line.

If an input can be interpreted as both a register and  a

number  (as in   LD  A,C ),  the register interpretation

takes  precedence.  Use LD A,0C to specify an  immediate

value.

The  displacement  for relative jumps is entered  as  an

absolute address.  The debugger will calculate the  dis-

placement for you.

A  symbol  may  be  defined at the  current  address  by

entering the symbol with a terminating colon (':').

A  syntax error will exit assembly mode without changing

the address pointer.  Simply entering a carriage  return

will continue assembling at the same location.

Example:

:a 220

220  START:  LD    DE,2345          ld a,b

221          LD    B,L              here: ld d,'a'

223          JR    NZ,0227   .

:

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 11

B - Breakpoint Set/Delete/Display

Syntax:    a)  B

b)  B   address { address.. }

c)  BI  multexpr ; address { address.. }

d)  BX

e)  BX  address { address.. }

f)  BXI

a) All  breakpoints and the conditional break expression

(if present) are displayed.

b) The  addresses  are entered as  unconditional  break-

points. If an address is already defined as conditio-

nal  breakpoint,  the  condition is deleted for  this

address.

c) The expression is stored as break condition,  and the

addresses are defined as conditional breakpoints. Any

previously defined conditional breakpoints are set to

unconditional.

d) Delete all breakpoints and the condition expression.

e) Delete the specified breakpoints.

f) Delete   the   break   condition.   All   conditional

breakpoints are set to unconditional.

Example:

:b 4711 .start

:bi rc = 15; 5

:b

4711  0400  0005(If)

If: rc = 15

:

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 12

C - Trace over calls

Syntax:    a)  C {N} {J} {count}

b)  C {N} {J} W multexpr

c)  C {N} {J} U multexpr

This  command  works  the same as the  T(race)  command,

except  that routines CALLed are executed in  real-time,

i.e. they are not traced.

The called routine MUST return to the instruction  after

the Call-instruction, or tracing will fail. That is, you

can  not  use this command for routines which  expect  a

parameter list after the call instruction and modify the

return  address,  or  for routines which might  pop  the

return address and return to somewhere else.

See definition of T for more details on the parameters.

Example:   cnw rbc <> 0 && rde = 0

D - Dump

Syntax:    D   {from-address {to-address}}

Memory is displayed in hexadecimal and ASCII format from

the from-address up to,  and including,  the to-address.

If  the to-address is not  specified,  the  from-address

plus 7F is used,  resulting in an 8-line display. If the

from-address  is not given,  dumping continues from  the

last  address dumped.  If the to-address is smaller than

the from-address, one line is dumped.

Example:   d 110 140

d 110 1         (this will dump 110 to 11f)

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 13

E - Execute Command Conditionally

Syntax:    E   { multexpr } ; command

The  debugger command is executed only if the result  of

the  expression is TRUE.  If no expression is specified,

or if the input is not an expression,  the command  will

not  be  executed.  This  command is mainly for  use  in

command macros.

Example:   e rhl > 1000;j mac2, rhl

F - Specify Command Line

Syntax:    F   { command-line }

Execution of this command is system dependent.

For CP/M,  the command line is inserted at the  standard

location  80,  and  the default FCB's at 5C and  6C  are

filled. For CP/M Plus, the password fields at 51..56 are

also defined.

Example:   f test.com;pass -f -x

G - Start Real-Time Execution

Syntax:    G   {to-address} {; temporary-break-address }

The user program is entered at the specified to-address.

If the to-address is not specified,  execution begins at

the current PC.

A temporary breakpoint may be set, which is automatical-

ly deleted on any break.

Example:   g;111

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 14

H - Display Expression

Syntax:    a)  H

b)  H   expression  { {,} expression ... }

a) The  values of the special variables  L(ow),  H(igh),

M(ax),  and T(op) (see "expressions", above) are dis-

played.

b) The  expression  is evaluated and displayed  in  hex,

decimal, binary and charater form, with two's comple-

ment  for hex and decimal.  This is repeated for  all

given expressions.

This  command may also be used to assign to variables

and registers and to display their values.

Example:   h y0 + $ << 2 & fffe

I - Input from port

Syntax:    I   { port }

One  byte is read and displayed from the specified port.

If no port is given,  the last port number specified for

this command is used.

For systems which evaluate the upper half of the address

bus  on I/O,  the port may be specified as a 16-bit num-

ber. The upper byte will be output on A8..A15.

Example:

:i 2210

I(Port=10, B=22): ...

:

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 15

J - Jump to Command Macro

Syntax:    J   filename { , parameter ... }

Execution  and  format of this command is system  depen-

dent.

Command input will be read from the specified file.  The

commands  will  be  read until an error  or  end-of-file

occurs.  No nesting of command macros is possible, using

this  command  from within a macro  will  terminate  the

calling macro.

Macro   execution  may  be  prematurely  terminated   by

entering  a character at the console or by using the  K-

command from within the macro.

Macros may be parameterized. The parameters specified in

the  macro  call will be substituted for  the  parameter

identifier @n in the macro body, where n is the position

of  the  parameter,  counting from 0 to  9.  Spaces  are

significant  in  a parameter except after  a  comma,  so

commas  must  be used to separate parameters.  An  empty

parameter  is generated by two commas in  sequence.  The

parameter identifier will be substituted anywhere,  even

in strings.  To generate a @-character in a  macro,  use

two @'s.

Example:

Contents of file TESTMAC:

g;123

e rhl > @0; k

e ; *** HL is less than @0 ! ***      (Note 1)

e @1; j testmac, @1,@2,@3,@4,@5,@6    (Note 2)

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 16

Invocation:

j testmac, 100, 50

Notes:

1)  The  command in this line will never be executed,

but  the line will be displayed on the  terminal.

It thus may be used to display a message.

2)  This command will only be executed if the  second

parameter (number 1) is non-empty.

K - Kill Macro Execution

Syntax:    K

A currently active macro will be terminated. If no macro

is active, this command has no effect.

L - List Disassembled Code

Syntax:    L   { from-address { to-address }}

The  code  beginning  at the  from-address  up  to,  and

including,  the  to-address,  is listed in  disassembled

form. If no to-address is given, eight lines are listed.

If no from-address is specified,  listing continues from

the last listed instruction, or from the current PC if a

break has occurred.

If the to-address is smaller than the from-address,  one

line is listed.

Example:   l rpc rpc + 5

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 17

M - Move Memory

Syntax:    M   begin-address end-address destination

The   memory  starting  at  begin-address  up  to,   and

including,  the end-address, is moved to the destination

address.

Overlapping  moves  are  allowed and do  not  result  in

propagating values through memory.

An error will result if the end-address is smaller  than

the begin-address.

Example:   m rpc rpc+10 3000

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 18

N - Name (Symbol) Definition

Syntax:    a)  N

b)  N   address symbol ...

c)  NF  filename

d)  NX

e)  NX  symbol ...

f)  NS  number

g)  NR  { offset }

h)  NW

a) The  number  of defined symbols and the current  free

symbol  space is displayed in decimal,  then all  de-

fined symbols are listed in ascending address order.

b) The symbol is defined as having the value  "address".

If the symbol is already defined, it will be assigned

the new value.

c) Defines the name for the symbol file. The symbol file

name is not changed by the normal 'F'-command. The NF

command  is implied if a second filename is specified

on the initial command line.

d) All symbols are deleted. This will not release symbol

table memory space.

e) The specified symbol names are deleted.

f) The  symbol  table  is  expanded  to  make  room  for

"number" additional symbols.

g) The  file  specified by the last NF-command  will  be

read  as  symbol-file.  If an offset is  given,  this

offset  will be added to all symbol values.  The  re-

quired  format is:  First value,  then  symbol  name,

separated by spaces or tabs. Value and symbol must be

on  the  same  line.  One line may  contain  multiple

symbol  definitions.  No line may be longer  than  80

characters.

h) The  symbol  table  will  be  written  to  the   file

specified by the last NF-command.

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 19

Cautions:

Symbol  table  space is allocated  downwards  (toward

address  0) in memory,  directly below the  debugger.

Expansion of this space is automatic if more  symbols

are  defined  than can be contained in the  available

space.  If  the program stack is within 512 bytes  of

the top of the TPA, the stack will be moved down, and

the  SP will be changed accordingly.

If  the program under test uses space at the  top  of

the  TPA for data or program storage,  this space may

be  overwritten when symbol table space is  expanded,

or  the  program might overwrite  the  symbol  table,

causing the debugger to crash. To avoid this, reserve

enough  symbol  table space before starting the  pro-

gram. To be safe from symbol table expansion, you may

set the M-variable to the top of the TPA,  since WADE

will  never expand the symbol table below Max  (Exam-

ple:  h m := t).

For  CP/M 3 only,  programs with an attached RSX will

cause the M-variable to be set to the Top of the TPA,

since  all  RSXes will be  relocated  there.  Reserve

symbol  space before loading the program,  since  the

symbol  table  cannot  be expanded once  the  RSX  is

loaded.

Example:

:n 100 start, 5 bdos, 4711 stinks

:ns 40.

:nf test.sym

:nr 2000

:nx bdos

:nf new.sym

:nw

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 20

O - Output to Port

Syntax:    O   { data-byte { port-address }}

Output the data-byte to the specified port-address.  The

data-byte will also be displayed.

If the port-address is not given,  the last port-address

specified with an O-command is used. If the data-byte is

not entered, the data of the last O-command is used.

For systems which evaluate the upper half of the address

bus  on I/O,  the port may be specified as a 16-bit num-

ber. The high byte will be output on A8..A15.

Example:

:o 'x' 2210

O(Port=10, B=22): ...

:

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 21

P - Trace Protection

Syntax:    a)  P

b)  PX

c)  P   multexpr

a) Display current protect condition

b) Delete protect condition

c) Define protect condition

The  protect condition is evaluated on each  trace.  The

instruction will be executed in real-time,  with a break

set to the current return address,  if the protect  exp-

ression evaluates to a TRUE (nonzero) value.

CAUTION: If the value at the current stackpointer is NOT

a  return  address when the protect expression is  true,

the  breakpoint will be set at an invalid  address,  and

the program may fail, or it may not return to the debug-

ger.

The  default value of the protect expression is 'RPC  >=

xxxx',  where  xxxx  is  the  starting  address  of  the

debugger.  This  results in BDOS-calls being executed in

real  time.   Be  careful  when  changing  the   protect

expression, since tracing into the BDOS will most likely

not  work  (the debugger also uses BDOS-calls,  and  the

BDOS is not re-entrant).

Example:   p rpc >= (6).

Q - Query (search) for a byte string

Syntax:    Q   {J} begin-addr end-addr byte-string

Memory will be searched for the byte-string starting  at

the begin-addr up to, and including, the end-addr. Every

match  will be displayed as one line of dump,  with  the

dump beginning at the first matching byte,  or,  if J is

specified, at 8 bytes before the first matching byte.

Example:   qj l h 'Hello' 0d 0a

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 22

R - Read a File into Memory

Syntax:    R   { offset }

Execution of this command is system dependent.

The  file  specified by the last F-command will be  read

into memory.  An offset,  if specified, will be added to

the standard load address.  If a file has the  extension

HEX,  it is assumed to contain standard INTEL-Hex format

records.  All  other  filetypes are read into memory  as

they are without any editing.

An error occurs if the file cannot be found, if the load

address is below 80h (CP/M),  or if the file would over-

write the debugger.

For  CP/M 3,  files with an attached RSX will be  loaded

via the system's "load overlay" function,  which handles

the relocation of the RSX.  A file is assumed to have an

attached  RSX if the first byte contains a RET  instruc-

tion (C9),  and if the filetype is .COM.  Note that  for

files  with attached RSX,  the High address will not  be

set  to  the  end of the file,  since  this  address  is

unknown.

Example:   r h-100

S - Substitute Memory

Syntax:    a)  S   address byte-string

b)  S   { address }

a) The byte-string replaces the memory at address.

b) The  byte at the specified address is displayed,  and

input of a byte-string is expected.  The  byte-string

replaces  the  contents  of  memory,   and  the  next

location is displayed. An empty input advances to the

next location. Use a dot (.) as first input character

to terminate the command.

If no address is specified, substitution continues at

the  last  address  displayed by a previous  S  or  A

command.

Example:

:s rhl 'Hi, there' 0d 0a

T - Trace

Syntax:    a)  T   {N}{J}  {count}

b)  T   {N}{J}  W multexpr

c)  T   {N}{J}  U multexpr

a) "count"  instructions  are  traced.  If no  count  is

given, one instruction is traced.

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 23

b) tracing continues while the expression evaluates to a

TRUE value.

c) tracing continues until the expression evaluates to a

TRUE value.

If  N  is given,  the traced instructions  will  not  be

displayed.

If  J is given,  only instructions which modify the pro-

gram counter (Jump,  Call and Return instructions)  will

be  displayed and counted or cause the expression to  be

evaluated.

If NJ is specified, instructions will not be listed, and

only  instructions which modify the PC will decrease the

count or test the condition.

Tracing  will terminate independent of count or  expres-

sion if a breakpoint is encountered,  or if a  character

(other than ^S or ^Q) is entered at the console.

If  the  protect condition (see P,  above) becomes  TRUE

during trace,  tracing will be inhibited and the protec-

ted part will be executed in real-time.

CAUTION:  Since it is not possible to break at the  cur-

rent  PC,  instructions causing a loop to itself can  be

traced only when the loop is exited.  Instructions other

than DJNZ will cause the debugger to issue an error.

That is,

100       LD    B,10

102       DJNZ  102

will  execute  the DJNZ in real-time and then trace  the

next instruction, whereas

100       JP    NZ,100

will cause the debugger to refuse to trace,  even if the

condition is false. You have to change the PC by hand to

allow the program to continue.

Self-modifying  code can cause the trace to fail if  the

implicit   break   after  the  traced   instruction   is

overwritten by the instruction itself. For example,

100     LD (103),A

103     NOP

will cause the break at 103 to be missed,  and execution

of the program will continue in real-time.

Example:   tj u y0:=y0+1, rpc > 2000

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 24

U - User Input Trap

Syntax:    U

This command is user-defineable.  Execution and  parame-

ters are system dependent.

The  debugger will prompt for a character.  Entering  CR

will delete any trap character,  entering any other cha-

racter will define this char as input trap character. If

a trap char is defined, all console input by the program

under test,  except for the "read console buffer"  func-

tion of CP/M, will be checked for equality with the trap

character, and a break will be entered on a match.

The  break will be set up such that the program     will

again  read a character if a Go is issued.  This  second

character  read  will  then not again be checked  for  a

match with the trap character.

Example:

:u

Ch: ~

:

V - Verify Memory

Syntax:    V   begin-addr end-addr compare-addr

Memory starting at the begin-addr up to,  and including,

the  end-addr,  is compared with memory starting at  the

compare-addr.  Non-matching  bytes  are  displayed  with

their address.

Example:   v 100 1ff 200

WADE - Wagners Debugger - User Manual - V. 1.5 -  Page 25

W - Write memory to File

Syntax:    W   start-address end-address { offset }

Execution of this command is system dependent.

Memory  beginning  at  the  start-address  up  to,   and

including,  the  end-address  is  written  to  the  file

specified  by  the last F-command.  If the file type  is

An  offset is only accepted for files of type  HEX,  and

then  specifies  the offset to be added to  the  current

write address when generating the Hex-file address.

Example:   w l h

X - Examine CPU State/Registers

Syntax:    a)  X

b)  X'

c)  X   register-name

d)  X   register-name expression {...}

a) The primary registers,  the current instruction,  and

the bottom stack words are displayed.

b) The alternate register set is displayed.

c) The  contents of the register are displayed,  and  an

expression is expected.  The value of the  expression

is assigned to the register.

d) The  register  is set to the  specified  value.  This

specification may be repeated.

Example:

:x pc

0100  1ac

:x a 0a b 0c de 1234

X, Intel Hex format will be generated.

Y - Display Y-Variables

Syntax:    a)  Y

b)  Y   n

c)  Y   n  expression {...}

a) Display the contents of all 10 Y-Variables

b) The  contents of the variable n (n = 0..9)  are  dis-

played,  and an expression is expected.  The value of

the expression is assigned to the variable.

c) The  variable n is set to the specified  value.  This

specification may be repeated.

The Y-Variables are not modified by the debugger and are

thus  available  as counters or event markers  for  user

expressions and command macros.

Example:

:y 0

0000  123

:y 0 123  1 0c  2 0d

Z - Zap (fill) memory

Syntax:    Z   begin-addr end-addr byte-string

Memory starting at begin-addr up to, and including, end-

addr is filled with the byte-string.

Example:   z 2000 2fff 0d 0a

X, Intel Hex format will be generated.

Appendix

Extended Memory Considerations

The  distribution disk contains a version of WADE  which

supports  extended addressing under CP/M 3.  If you  are

running a banked version of CP/M 3,  this version should

run  on your system without modifications if,  and  only

if, the following is true:

-  there  is  at least 2 k space in common memory  below

the BDOS entry.  This space is required to allow WADE

to  switch  back into the default bank  (bank  1)  on

return from a breakpoint in a different bank.

-  the MOVE,  XMOVE and SELMEM entries of your BIOS  are

in common memory.

If  you  do  not intend to use  tracing  or  breakpoints

outside of the normal bank 1,  you should be able to use

the extended version even if the first condition is  not

true.

Please  note that the extended version is much slower in

all  memory access operations due to the need to  access

memory  via the MOVE and XMOVE BIOS routines instead  of

using direct access.  The extended version should there-

fore only be used where access to other banks is  neces-

sary.

To verify that you can run the extended version, use the

non-extended WADE and the command macro EXTEST:

WADE

J EXTEST

and  note where the macro stops execution.  If the macro

exits with a g0,  you can use the extended  version.  If

the  message  'OK to use in non-trace mode' appears  but

the macro then terminats with 'Less than 2 k free',  you

may not trace any routine outside the default bank.

Cautions

WADE always operates on a 'default bank',  which is  set

to  1  (the CP/M TPA bank) on entry.  Since there is  no

system  independent  way to obtain  the  current  active

bank,  you have to change the active bank by hand if you

intend  to trace any routine outside of bank 1.  Extreme

caution  must  be employed when tracing  routines  which

might change the current bank,  since tracing might fail

(most likely completely crashing the system) if WADE  is

not informed on the bank switch.  Please note, too, that

the restart location in all banks for which a breakpoint

has  been  defined will be changed while tracing  is  in

progress or any breakpoint has been set and the  program

is  executing in real time.  Although WADE will  restore

the  original contents of this location,  you should not

attempt  to trace any routine which might depend on  the

information  stored at this location (38h to 3ah in  the

current release).

X, Intel Hex format will be generated.

Changing the Bank

The  current  active bank is contained  in  the  special

register  'X'  and  may be displayed using the H  or  X'

command.  It may be changed by assignment:

H X := 0

or by changing the program counter with the X-command:

X PC 0:100

You  can  also  use all memory reference  commands  with

locations  outside  the current bank  by  specifying  an

extended address:

L 0:100         Lists Bank 0, Address 100

S 5:e000        Substitutes Bank 5, Address E000

M 0:0 ff 100    Moves  from Bank 0,  Address 0 to FF,

to Address 100 in the current Bank

Z 7:0 ff 0      Sets Locations 0 to FF in Bank 7 to 0

You may even assign to locations in other banks:

H 0:100 := (7:100).

Customization

If you own the source version of WADE,  you might possi-

bly  change  the  entry 'EBREAK' in  module  MONPEEK  to

obtain   the  currently selected bank in  some  hardware

dependent  way  annge  the  entry 'EBREAK' in  module  MONPEEK  to

obtain   the  currently selected bank in  some  hardware

dependent  way  and  store it in  the  variable  'CBANK'

before switching to the default bank.  This would  allow

tracing  into other banks without setting the X-register

by  hand.  If your BIOS returns the previously  selected

bank in register A on exit from the SELMEM routine,  you

may  simply  set the equate 'MEGA' to TRUE in  the  file

MONOPT.LIB and re-assemble the module MONPEEK.

###################################