American Information Systems Corporation

*** Company Confidential ***

Ram Monitor Specifications

This document describes the RAM monitor for the AIS 3200 system.

Information in this document subject to change without notice.

AIS makes no guarantees, express or implied, as to the accuracy

or completeness of this information.

The PROM of an AIS/3200 system contains 4 basic program

blocks; the initialization code, the RAMLESS monitor, the

RAMmonitor, and the VIOS.  The primary purpose of the

initialization code is twofold; hardware checkout and software

environment setup.  When an AIS/3200 system is powered up, the

initialization code checks all devices on the AIS/3200 cpu

board and sizes and checks all memory in the system, building

a map that will be used later in the booting process.

If the initialization code does not find enough usable memory

to start the VIOS or RAMmonitor it will branch to the RAMless

monitor.  The RAMless monitor provides a subset of the functions

of the RAMmonitor without using any memory.  Normally, the VIOS

is automatically booted from ROM when the AIS/3200 system is

supplied with power and the RAMless monitor and the RAMmonitor

are rarely executed.

The VIOS (Virtual I/O System) is an operating system in itself

and provides a constant environment to the AIS/3200 system

in regards to the host operating system and hardware.

It functions as an I/O dispatcher and in normal operation

fields all interrupts and traps.  It is the sole means

of communicating with any host hardware or software.

The RAMmonitor is symbolic debugging tool that provides for

controlled execution of programs and the ability to interrogate/change

the hardware/software environment of the AIS/3200 system.

The RAMmonitor automatically generates and uses symbols for some

of its most used values.  Additional symbols may be added and deleted

using the '+' and '#' commands respectively.  A synopsis of the

monitor commands can be printed by the '?' command.

The ouput format can be changed by a '$' command.

All input values to the RAMmonitor are taken to be in hexidecimal.

To input data in a format which is not hexidecimal type a ' (right

quote) followed by one of the format specifiers listed in the

CHANGE OUTPUT BASE command ('$'), followed by the input

value in the desired format.

The RAMmonitor interprets addresses using the state that was active

when a break in program execution is encountered.  This includes the

the mmu registers that specify the page tables for user or system state

if translation was enabled.  This feature can be disabled by enabling

physical translation with the 'e' (ENABLE OPTIONS) command.

Each of the following sections describes a monitor command.

A title in capital letters is followed by syntax example(s)

on the left and the interpretation of the example(s) on th

e right.

Syntax may be interpreted as follows:

<cr> means enter carriage return

arguments in {} are optional

arguments in <> are required

for arguments separated by comma, pick any one

The backspace and delete characters both are interpreted as erase.

The space before the last character in an input  command line is

optional.

SYMBOL MANIPULATON:

add symbol			<symbol> <value> +

delete symbol			<symbol> #

list symbols			<symbol specification> l

general information about symbols:

The symbol '.' stands for the current location.

Here's a sample listing of the default symbols and their values.

bitmap    = 000002e0     id        = 00041134     spl     = 00041138

fp        = 0004113c     sb        = 00041140     r7      = 00041144

r6        = 00041148     r5        = 0004114c     r4      = 00041150

r3        = 00041154     r2        = 00041158     r1      = 0004115c

r0        = 00041160     intbase   = 00041164     sp      = 00041168

sp0       = 00041168     pc        = 0004116c     mod     = 00041170

ps        = 00041172     psr       = 00041172     reset   = 00060000

_etext    = 00067090     _edata    = 000677d0     rom     = 00fa0000

rtc       = 00ff0003     icu       = 00fffe00

CHANGE OUTPUT FORMAT

<format specification> $	Change the output format to hex, decimal,

octal, character, or instruction disassembly

depending on the argument letter.

The argument letters and their meanings:

x or h	hexidecimal

d	decimal

o	octal

c	character

u	instruction disassembly

EVALUATE AN EXPRESSION

<expression>=			This command will evaluate the given

input expression using the defined

symbols and some basic arithmetic.

BREAKPOINT MANIPULATION

b				displays breakpoints

-<number> b			clears a breakpoint corresponding to number

-*b				clears all breakpoints

-<addr> b			clears breakpoint at addr

A set breakpoint includes information about the address translation in effect

when the breakpoint was set.  A problem can arise if a breakpoint is set

with physical addressing in effect and once this breakpoint is reached

the addressing mode is no longer physical.   The breakpoint will occur as

desired but the monitor will not be able to modify the breakpoint properly

because it will use the wrong addressing.  The monitor can properly clear

the breakpoint and it can be 'reset' with the new addressing mode.

COMPARE

<addr1> <addr2> <count> c<cr> 	Compare the block of memory starting

at addr1 to the block of memory starting

at addr2 for count bytes.

If there are any differences they

will be printed with the address in the

addr1-block with its value followed

by the address in the addr2-block

with its value.

DISPLAY

<addr> <count> d<cr>		Display a block of memory starting at

address addr for count bytes.  Both

hex and ascii interpretations are

printed.

ENABLE OPTIONS

<p,h,s,u,m> e<cr>		Enable the option(s) associated with the

given letter(s).

The option-letters are:

p	All addresses are interpreted

as physical addresses.

h	Print hidden symbols, those

normally used but not printed

in a list symbols command.

s	Print addresses symbolically.

Instead of printing a address

as a number try to interpret

it as a symbol.

u	Allow user-state debugging.

m	Enable MMU type breakpoints,

current proms do not contain

this code to save on space,

so this is a no-op.

FILL

<addr> <count> <val> F<cr>	Fill a block of memory with the

byte value val starting at address

addr for count bytes.

GO

<addr> G<cr>			Start execution at a specified address

or the current value of the symbol

pc if no address is given.  Interrupts

will be disabled.

HELP

?<cr>				Print a synopsis of all ramless monitor

commands.

{command letter} ?<cr>		Print a synopsis of the command associated

with the given letter.  In some cases more

information will be printed about a command

than with the previous example.

IMAGE LOAD

I<cr>			Start up the serial loader.

The serial loader is program to load binary

data over a serial line (tty01) into

memory.  It uses the following protocol:

(1) DTR is made active.

(2) Wait till receive the 4 sync characters

AISL.

(3) Read in 4 bytes to form load address

most significant byte first.

(4) Read in 4 bytes to form a image size

most significant byte first.

(5) Read in 4 bytes to form a checksum

most significant byte first.

(6) Read in image size bytes calculating

a checksum of received characters.

(7) Compare checksum of received characters

with that received earlier.

Print a message if they are not equal.

(8) Recalculate the checksum by using the

data loaded into memory and compare

with original checksum.

(9) If all is well print the load address

and the image size and return to the

ramless monitor.

(10) DTR is made inactive.

MOVE

<srcaddr> <destaddr> <count> M<cr>	 Move a block of memory

starting at address srcaddr to address

destaddr for count bytes.

This command checks for overlapping

ranges and adjusts its move to properly

transfer data so that the destination

will get a copy of the source even if

some of the destination range overlaps

the source.

PROCEED

{count} p		Proceed; continue program execution.

If a count is given that many breakpoints

will be passed before execution is stopped.

A proceed command without a count will

resume execution and continue until the

next breakpoint.

OPEN A LOCATION

"`" "\" "/"		Open a location in byte,word, or double mode.

[addr]/			Open a double location (32 bits)

at addr or last open address.

Subsequent open mode commands

will use 32-bit operations.

[addr]\			Open a word location (16 bits)

at addr or last open address.

Subsequent open mode commands

will use 16-bit operations.

[addr]`			Open a byte location (8 bits)

at addr or last open address.

Subsequent open mode commands

will use 8-bit operations.

open mode commands:	Once a location has been opened,

there are the following options:

reopen		Typing a /, \, or ` reopens the

current location in the size specified.

This rereads the specified location.

new-open	Typing a new address and then

one of /, \, or ` opens a new location.

write-close	Typing a new value and a <cr> writes

the value to the current location

and closes the location.  Typing

a <cr> with no value will close the

location without writing anything.

previous	Typing a ^ opens the proceeding

location after writing data to the

current location if any was entered.

next		Typing a line feed opens the next

location after writing data to the

current location if any was entered.

redirect	Typing a @ will use the current

location as a pointer and open a new

location at that address.

move-up		Typing a number followed by '+'

will add that number to the current

location and open that address.

move-down	Typing a number followed by '-'

will subtract that number from the

current location and open that address.

change-format	Typing a format specifier and '$'

will reopen the current location

in the new format.

Format specifiers are as follows:

x or h	hexidecimal

d	decimal

o	octal

c	character

u	instruction disassembly

s	Display as string starting at

the current address for up to

512 bytes.

In all cases except string the format

change is permanent and results in the

same action as a change format command.

REGISTER DISPLAY

{c,m,f} r<cr>		Display the cpu, mmu, or fpu registers.

Typing an 'r' without specifying the particular

type will default to cpu register display.

STEP

{count} s<cr>		Single-step count instructions.

,			Same as above, no <cr> needed.

If no count is given, one instruction

is single stepped.

<			Single-step a bsr,jsr,or cxp instruction.

Execute the subroutine and anything it

may possibly call and breakpoint when

it returns.

UNASSEMBLE INSTRUCTIONS

<addr> {count} u	Disassemble count instructions starting at

addr.

If no count is given a default of two

instructions will be disassembled.

caveat:

The disassembler knows nothing about

the validity of the data it is being

told to interpret.  It will start at

any address it is given even though

it may actually be in the middle of

a multi-byte instruction.

ZAP

<addr> <value> <b,w,d> Z<cr>	Write a location, without reading.

The letters b, w, d specify byte, word, and

double writes respectively.

This command is useful for writing

special device registers that interpret

reads different from writes.