SPECIFICATIONS VME/DM6

Form Factor    H         W         L              UNITS

22        7         17             inches

Weight         42 pounds

Above dimensions are initial dimensions and may change!

Weight depends on configuration.

Power:

115V +10%,-10% 6A max

230V +10%,-10% 3A max

Cooling:  2 DC Fans

Number of VME slots      6

Compatibelity            A32:D32

Available Power:

+5V  @45A

+12V @10A

-12V @5A

Available drive slots:

four 5 1/4 inch full hight, 2 with window for tape or floppy.

Controller Mounting:

1 Standard 5 1/4 Inch Hard floppy Controller.

2 Colex floppy controllers.

Switches/Fuses:

1 Power, 1 Reset Switch

1 Mains fuse holder 6A @ 115V 3A @230V

Connectors:

1    Mains

1    Amphenol 50 way for SASI extension.

1    BNC  for RS170 video.

1    5 pin DIN female for keyboard.

#########################################################################