UNISYS

Emergency 	Maintenance 	Update

CTOS III  s1.3.6

Jan 7, 1997

Released Version:	R1.3.1

Style ID:	B25V10B25VBU

Library Reference Number:

DESCRIPTION

This Emergency Maintenance Update (EMU) is provided on an asneeded basis only.  It is not an official release nor is it available through Corporate Software and Publications Operations (CSPO).  The information in this document is supplemental to the product's Software Release Announcement (SRA).

Contact your Unisys support center for more information concerning this EMU.

INSTALLATION INSTRUCTIONS

Use Installation Manager to install this software.  For Specific questions or information on restrictions, you should refer to  the CTOS III R1.3.1  SRA.

SOFTWARE FILES

This EMU is a complete replacement package for the released product.

ADDITIONAL HARDWARE SUPPORT

This release adds support for the SG1650 workstation equipped with hard disk.  An SG1650 workstation which is not equipped with hard disk is supported under both CTOS III R1.2.20 and up, as well as CTOS III R1.3.1 and up.  With a hard disk, the SG1650 requires CTOS III s1.3.3 or greater.

RESTRICTIONS

There are no known restrictions for this EMU.  However, see MANDATORY FIELD CHANGE below.

IMPORTANT RELATED HARDWARE INFORMATION

The SG451P card (486based Cluster IOP) requires a firmware change to correct a problem which manifested itself as one of several different ercs (below) occurring at BOOT time only.

One or more of the following error codes have been seen:

Erc 245 loading [Sys]<Sys>Nls.sys

Erc 80 in [Sys]<Sys>DisplayConfig.sys

Erc 80 in [Sys]<Sys>SignOn.run

Other ercs may have also been seen.  The problem occurs maybe one or two times in every ten system reboots.  However, it may also manifest itself in ways not listed above.

The mandatory Field Change Notice is MCN SG451P001 (orderable kit number 46173191000) and the CORRECT firmware part numbers and locations are:

Location U301  should have part number 46143731001

Location U302  should have part number 46143723001

If the part numbers on your SG451P in these locations do NOT match the above (i.e they end in 000), you will need to obtain and install the FCN listed above.

PROBLEMS CORRECTED in CTOS III s1.3.6

PLE 16949094	IMSP fails to install with 2 IDS slices on a B39.

PLE 16967891	Return erc 7 when Hardware ID is used on SG4200.

PROBLEMS CORRECTED in CTOS III s1.3.5

PLE 16914134	An IRQ conflict exists between the SG103A and SG206V, both are set to IRQ 10.

PLE 16914151	'Battery Backup Clock Invalid' being returned on B39 systems.

PROBLEMS CORRECTED in CTOS III s1.3.4

PLE 16883140	Semaphore calls fail to return error code when they fail.  This caused problems for LancePC32.

PLE (none)	Enforce the CTOS restriction of 4GB as the absolute maximum size for a disk volume.

PROBLEMS CORRECTED in CTOS III s1.3.3

PLE 16694380	SG451P (486based Cluster IOP) speeds up datetime.

PLE 16685941	Workstation hangs when exiting an application under VPC and Windows 3.1.  Was a Paging Service problem.

PLE 16702102	Config.sys :nMaxNetUsers: parameter was ignored, and Sysgen default was used instead.

PLE 16702161	Access Control hangs the system as soon as it is installed via SysInit.jcl.  Problem was in keyboard fix for PLE 16615366 that was introduced in s1.3.2.

PROBLEMS CORRECTED in CTOS III s1.3.2

PLE 16651346	SG451P (486based Cluster IOP) board with 32MB memory would not boot or poll cluster line.

PLE 16625001	Termination process changed to only call UnLockVideo if

user number is local.

PLE 16628816	Disabled automatic volume recognition (AVR) for disk volumes which were not flagged as removable devices.  This corrects erc 235 failures seen in various applications as well as in the Paging Service.

PLE 16615366	Occasional erc 80 crash in keyboard routine on machine with MCR device attached to the IBUS.

PROBLEMS CORRECTED in CTOS III s1.3.2 (continued)

PLE 16622109	Floating point routines return wrong value the FIRST time they are called on CTOS III 1.1, 1.2 and 1.3.

PLE 16636967	Do not write error message to PLOG if a configuration file does not exist (was annoying to customer).

PLE 16656283	ICC sent a request out of order when the requests were on the block wait queue.  Affected LANce operations.

PLE (none)	Updated VBU installation file to include missing file name: OsLabl3.obj (missing from R1.3.1 VBU).[jy>