CONVERGENT TECHNOLOGIES

RELEASE NOTICE FOR

12.1 GRAPHICS (STA1501)

Revised August 3, 1989

SECTION   TITLE	PAGE

1.0	Description of Programs	 2

2.0	Changes from Prior Version	 3

2.1  SPRs Closed in This Release	 3

2.2  Changes from Release 12.0	 3

3.0	Contents of Distribution Diskette	 5

4.0	Installation Procedures	 6

4.1  StandAlone Hard Disk Systems	 6

4.2  Master Hard Disk Systems	 7

4.3  StandAlone Floppy Disk Systems	 8

5.0	System Software Compatibility	 9

5.1  Workstation Environment	 9

6.0	Hardware Information	10

6.1  Hardware Configurations Supported	10

6.2  Special Hardware Requirements	10

7.0	Resource RequirementsUtilization	11

7.1  Memory RequirementsUtilization	11

8.0	Restrictions	12

9.0	Documentation Updates	13

10.0	Supplementary Information	14

10.1 New Procedures Added	14

11.0	Error Codes	21

12.0	Known Errors and Omissions	23

1.0	Description of Programs

This Release Notice describes Graphics Library version 12.1.  This section and the sections that follow provide a description of the product, contents of the Distribution Diskette, installation procedures, and other information pertinent to this release of Graphics Library.

2.0	Changes from Prior Version

2.1	SPRs Closed in This Release

12732 Nonsolid line types do not connect properly

14622 Absence of a .user file causes 7691.

12441 Colors not plotted properly on a HP7220T

2.2	Changes from Release 12.0

A.	Graphics library now runs completely in protected mode.

B.	Printing has been enhanced.  The new call gfx_SetPageSize sets the page size sent to GPS.  gfx_SetCopies sets the number of copies printed by a GPS printer.  Both of these calls function only on GPS printers.

C.	GCx04 graphics module is now supported with 256 colors also supported.

D.	There are two ways to specify picture memory requirements to the protected mode Graphics Library.  The first method is to allocate enough memory for the picture file plus at least 4K extra (using AllocMemorySL), and then provide the pointer and count of paragraphs to the OpenPicture call.  For example:

erc := AllocMemorySL (E000, ADS pMemory);

erc := OpenPicture (pbPicName, cbPicName, pbPswd, cbPswd, ModeRead, pMemory, 0E00);

This will allocate about 57Kb of memory, and provide it to the OpenPicture call.

The second method is to figure how much memory will be required, then pass a zero pointer and the count of paragraphs to the OpenPicture call.  When the second method is used, the graphics library will allocate its own memory to store the picture in, and then deallocate it when the picture is closed.  For example:

pMemory.R := 0;

pMemory.S := 0;

erc := OpenPicture (pbPicName, cbPicName, pbPswd, cbPswd, ModeRead, pMemory, 6000);

This will ask for about 384Kb of memory from the OpenPicture call.  When using the second method, no memory allocation or deallocation calls are needed for the OpenPicture call.

3.0	Contents of Distribution Diskette

The Graphics Library Distribution Diskette is your master copy, and has been shipped writeprotected.  It should not be writeenabled, nor should it be used as a working copy.

The Distribution Diskette contains the following files in the <CT> directory:

ComplexRoman.font

DuplexRoman.font

Gothic.font

SimplexRoman.font

SimplexPlot.font

Graphics.edf

Graphics.fonts

Graphics.lib

gfxGPAMDum.obj

RasterOpIwsAws.obj

HPPlotterConfig.sys

PlotterConfig.sys

StrobeConfig.sys

StrobePlotterConfig.sys

In addition to various standard system files, the Distribution Diskette contains the following files in the <Sys> directory:

HdInstall.sub

For documentation purposes the Distribution Diskette contains the following file in the <Release Note> directory:

GraphicsRelNot.doc

4.0	Installation Procedures

The Graphics Library should only be used on Convergent Information Processing Systems which are equipped with hard disk <or dual floppy disks>, or in clusters where the master is equipped with hard disk.

Use the installation procedures described below (after updating the Standard Software revision level, if required).  Characters that you must type are shown in boldface.  Special keys, such as RETURN and GO, are shown in upper case.

{4.1	StandAlone Hard Disk Systems

A.	Signon and set path at the standalone workstation.  If the Signon form is displayed, fill it in and press GO.  Set the path as follows:

Command   Path  RETURN

Path

[Volume]	Sys  RETURN

[Directory]	Sys  RETURN

[Default file prefix]	RETURN

[Password]	(if any)  GO

[Node]

If your hard disk has a volume password on [Sys], fill this password into the [Password] field before pressing GO.}

B.	Command   Submit  RETURN

Submit

File list	[f0]<Sys>HdInstall.sub  GO

[Parameters]

[Forced Expansion]

[Show Expansion?].

C.	If Graphics is to be used to build applications, install the Standard Software Development Diskette per the instructions in the Standard Software Release Notice.}

{4.2	Master Hard Disk Systems

A.	Signon and set path at the master workstation.  If the Signon form is displayed, fill it in and press GO.  Set the path as follows:

Command   Path  RETURN

Path

[Volume]	Sys  RETURN

[Directory]	Sys  RETURN

[Default file prefix]	RETURN

[Password]	(if any)  GO

[Node]

If your hard disk has a volume password on [Sys], fill this password into the [Password] field before pressing GO.

B.	Command   Submit  RETURN

Submit

File list	[f0]<Sys>HdInstall.sub  GO

[Parameters]

[Forced Expansion]

[Show Expansion?]5.

C.	If Graphics is to be used to build applications, install the Standard Software Development Diskette per the instructions in the Standard Software Release Notice.}

{4.3	StandAlone Floppy Disk Systems

<This section is required  If the product is not supported on floppy disk systems, then say so.>

A.	Signon and set path at the standalone workstation.  If the Signon form is displayed, fill it in and press GO.  Set the path as follows:

Command   Path  RETURN

Path

[Volume]	Sys  RETURN

[Directory]	Sys  RETURN

[Default file prefix]	RETURN

[Password]	(if any)  GO

[Node]

If your floppy disk has a volume password on [Sys], fill this password into the [Password] field before pressing GO.

B.	Command   Submit  RETURN

Submit

File list	[f0]<Sys>HdInstall.sub  GO

[Parameters]

[Forced Expansion]

[Show Expansion?]5.

C.	If Graphics is to be used to build applications, install the Standard Software Development Diskette per the instructions in the Standard Software Release Notice.}

{5.0	System Software Compatibility

5.1	Workstation Environment

If Graphics is to be used in building an application, 11.0 Standard Software (or higher) and 11.0 Ctos.lib (or higher) are required.  If the application must run on the GC003 module, 2.0 CtosVM (or higher).  If Graphics is to be used to run an application which is already built, Standard Software at revision level 11.0 or higher is required for the application to function correctly.  If you have not yet installed Standard Software at such a revision level, do so before installing Graphics.}

{6.0	Hardware Information

6.1	Hardware Configurations Supported

Graphics Library 12.1 works on all NGEN Workstations with graphics.}

{6.2	Special Hardware Requirements

An NGEN Workstation equipped with a GC001 module and color or monochrome display, or NGEN Workstation equipped with a GC003 module and VM001VM002 or VM003, or NGEN Workstation equipped with a GCx04 module and any compatible monitor,  is required to use Graphics Library 12.1.}

{7.0	Resource RequirementsUtilization

7.1	Memory RequirementsUtilization

The minimum memory size required for a Graphics workstation is 384K.  A substantial amount of memory must be allocated when using primitives to create picture files because pictures are memory resident.  The minimum amount of memory required for a picture file is 4096 bytes.  The memory overhead required for each object within a picture is 256 bytes.  This does not include the commands that go into a picture file.  Thus a complicated picture will require substantially more memory.  See the Graphics Programmer's Guide for more details.}

8.0	Restrictions

{9.0	Documentation Updates

In this release, the plotters and printers supported include the HP7470A plotter, the HP7220C plotter, the HP7220T plotter, the HP7475A plotter, the Gould DS10 plotter, the Strobe Model 100 plotter, the Printronix MVP printer, the Envision 420 printer, the IDS Prism printer, the Anadex 9620 printer, the Okidata Microline 93 printer, the Dataproducts SPG8010, and the Dataproducts SPG8050 printers.  The files included here (PlotterConfig.sys, StrobeConfig.sys, StrobePlotterConfig.sys, and HPPlotterConfig.sys) are the configuration files for these plotters and printers.}

Output Dev.	Comm Config File	Printer Config File

HP7470A	PlotterConfig.sys	HPPlotterConfig.sys

HP7220C	PlotterConfig.sys	HPPlotterConfig.sys

HP7475A	PlotterConfig.sys	HPPlotterConfig.sys

HP7220T	PlotterConfig.sys	HPPlotterConfig.sys

Gould DS10	PlotterConfig.sys	HPPlotterConfig.sys

Strobe	StrobeConfig.sys

StrobePlotterConfig.sys

Printronix	none	LPTConfig.sys

Anadex	none	LPTConfig.sys

Envision	none	LPTConfig.sys

Prism	none	LPTConfig.sys

Okidata	none	LPTConfig.sys

Dataproducts

8010	none	LPTConfig.sys

When using a plotter, the 'Comm Config File' should be used when going directly to the plotter, and the 'Printer Config File' should be used if spooling to the plotter.

The Generic Printing System (GPS) is also supported in this release. To output to GPS first call SetPlotterDevice with the name of the GPS device or file (this is a required step). Next call SetOutputType(100), to set the type to GPS. Finally call SetOutputDevice(1). Subsequent calls to DisplayPicture, or any Draw procedure will send the output to the requested GPS device. For details see GPS documentation. The use of GPS requires an additional library, GPAM.lib, available in the GPS software package. For those who do not wish to use GPS, a substititution module is provided in the Graphics library called gfxGPAMDum.obj. Include this module in the linker object line to resolve GPS calls without including the GPS package.

10.0	Supplementary Information

10.1	New Procedures Added

These procedures were added to support access to planes 3 through 7 on certain GC_x04 configurations.  When compared to SetRasterDestinationPlane and SetRasterSourcePlane source mask 8 is plane 3, and source mask 0 is all planes.

Note that if you set the raster source plane to a single plane and set the raster destination plane to all planes, or if you set the raster source plane to all planes and set the raster destination planes to a single plane, the raster operation will use the single plane as a default for the source and destination.

__________________________________________________

gfx_SetRasterDestinationPlane	device dependent

__________________________________________________

gfx_SetRasterDestinationPlane (wDestinationMask):

ErcType

HARDWARE REQUIREMENTS

This procedure affects DoRasterOp and DoRasterOpText only on workstations that have raster access and in which all graphics planes have the same address (see Table A1 in Appendix A, Hardware Requirements).

On certain raster access workstations, the screen coordinates used in the vector and arc manipulation procedures differ from the raster coordinates used in the bit manipulation procedures.  (See Table A1.)  GetRasterInfo returns the two sets of coordinates for these workstations.

DESCRIPTION

gfx_SetRasterDestinationPlane assigns the destination plane for bit manipulation procedures that follow.  SetRasterDestination must still be called to set the destination memory address.  The all planes selection is implemented for DoRasterOpText only.

__________________________________________________

gfx_SetRasterDestinationPlane	(continued)

__________________________________________________

PARAMETERS

gfx_SetRasterDestinationPlane (wDestinationMask):

ErcType

wDestinationMask

Specifies which plane is the destination for the bit manipulations that follow.

0 =	all planes

(see note below)

1 =	plane 0

2 =	plane 1

4 =	plane 2

8 =	plane 3

10=	plane 4

20=	plane 5

30=	plane 6

40=	plane 7

NOTE

On GC_x04 systems, all raster operations can use the all planes selection.  The all planes selection, however,  is implemented for DoRasterOpText only on other systems.

__________________________________________________

__________________________________________________

gfx_SetRasterDestinationPlane	(continued)

__________________________________________________

SEE ALSO

SetRasterDestination

SetRasterDestinationPlane

SetRasterSource

SetRasterSourcePlane

gfx_SetRasterSourcePlane

__________________________________________________

gfx_SetRasterSourcePlane	device dependent

__________________________________________________

gfx_SetRasterSourcePlane (wSourceMask):  ErcType

HARDWARE REQUIREMENTS

This procedure affects DoRasterOp and DoRasterOpText only on workstations that have raster access and in which all graphics planes have the same address.  (See Table A1 in Appendix A, Hardware Requirements.)

On certain raster access workstations, the screen coordinates used in the vector and arc manipulation procedures differ from the raster coordinates used in the bit manipulation procedures.  (See Table A1.)  GetRasterInfo returns the two sets of coordinates for these workstations.

DESCRIPTION

gfx_SetRasterSourcePlane assigns the source plane for bit manipulation procedures that follow.  SetRasterSource must still be called to set the source memory address.

__________________________________________________

gfx_SetRasterSourcePlane	(continued)

__________________________________________________

PARAMETERS

gfx_SetRasterSourcePlane (wSourceMask):  ErcType

wSourceMask

Specifies which plane is the source for the bit manipulations that follow.

0 =	all planes

(see note below)

1 =	plane 0

(lowest order plane)

2 =	plane 1

4 =	plane 2

8 =	plane 3

10=	plane 4

20=	plane 5

30=	plane 6

40=	plane 7

NOTE

On GC_x04 systems, all raster operations can use the allplanes selection.  The all planes selection, however,  is implemented for DoRasterOpText only on other systems.

__________________________________________________

__________________________________________________

gfx_SetRasterSourcePlane	(continued)

__________________________________________________

SEE ALSO

SetRasterSource

SetRasterSourcePlane

SetRasterDestination

SetRasterDestinationPlane

gfx_SetRasterDestinationPlane

11.0	Error Codes

Note that several of the following error codes will occur only when using graphics objects with the Document Designer.

7604 Bad Fill Algorithm.

This error occurs in the new area fill algorithm on calls to FillRectangle or FillScreenRectangle. It is an internal error and should not occur. Contact Technical Support.

7630 Bad Object Bounds

This error is returned from GetPictureBounds when no object exists in the current picture or an empty object is the only existant object.

7667 Picture Status Warning

This error is returned from DisplayPicture or DisplayCurrentObject when a foreign font was encountered or a fill was not completed due to insufficient memory.

7668 Invalid Plane

This error is returned from DoRasterOp when a previous SetRasterSourcePlane or SetRasterDestinationPlane specified an invalid plane for this device.

7678 Bad ICMS message type

This is a system error which occurs if Business Graphics Package recieves a message type it does not understand from the InterContext Message Service (ICMS).

7694 Odd VDM parameter count

VDM files contain entries that are word aligned. This error occurs when an entry containks and odd number of parameter bytes.

7695 Invalid VDM opcode

This indicates the opcode for a VDM entry is invalid. This is the normal error when attempting to read nonVDM files.

7596 Bad VDM parameter count

Some VDM entries require a fixed number of parameters. This error occurs when an entry contains an incorrect number of parameters.

12.0	Known Errors and Omissions

A.	The DataProducts 8050 printer is not supported.

B.	The color of the first object drawn is not being saved when the default color is used.  The workaround is to set the color of the first object using the SetColor call.

C.	The vector cursor is not holding its relative position with respect to other objects after transformations.

D.	DrawScreenArc will not be supported with this release.

E.	When DrawCircle and FillPolygon are used in complement mode, traces will be seen on the screen.

F.	When DrawLine is used in complement mode, there is a chance of getting some color mixing on the screen.

G.	When DrawLine is used in complement mode, there is a chance of getting some color mixing on the screen.

H.	Passing an angle with value greater that 2 pi to DrawArc results in inconsistent behavior.

I.	When running an application with a GC004 or GC104 graphics processor, graphics operations will run in parallel to other operations.  Example symptoms could be a change in the color display before the completion of graphics lines being drawn on the display, or display of alpha characters before the completion of a graphics bitmap operation.

J.	Subsequent calls to InitGraphics in a single application does not reset the default font.

(end)