SETPAL.DOC

This program allows the user to set the palette from MS-DOS.

The border color index is not changed.

SETPAL data data data... (sets the color palette).

where "data" is a hexidecimal number from 0->ffff.

SETPAL begins loading at palette index 0,

and continues loading palette locations until

out of data (maximum of 16).

The palette is defined as follows:

15  14  13  12  11  10   9   8   7   6   5   4   3   2   1   0

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+

| M | M | M | M | U | U | T | T | T | T | T | T | T | T | T | T |

| I | R | G | B | U | U | K | B | B | B | G | G | G | R | R | R |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+

where:

M = a bit used to define a monitor color

T = a bit used to define a television color

R = red

G = green

B = blue

I = intensity

K = key onto external video

U = unused