@(#) Version 5.1.140

GROUP boot

NAME		boot-automatically

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>Boot Automatically

SNMP NAME	ncdBootServerAutomatically

SNMP PATH	ncdBoot 1

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies whether the terminal automatically boots

or simply stops in the boot monitor with the > prompt.

This parameter is ignored if the terminal has been

put in one of the secure modes.

NAME		boot-custom-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>Custom File

SNMP NAME	ncdBootCustomFile

SNMP PATH	ncdBoot 20

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies a non-standard name for the server

download file.

If specified this parameter consumes space in

the optional string space of NVRAM.

This feature is only available with boot monitor

version 2.6.0 or later.

NAME		boot-default-server-when-prom-booting

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>Default Server when PROM Booting

SNMP NAME	ncdBootDefaultServer

SNMP PATH	ncdBoot 7

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

When the terminal is booted via the network, the

boot server is considered the default server to

be used when a particular server has not been

specified. This parameter specifies the default

server when the terminal is booted locally.

NAME		boot-desired-source

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>Primary Boot Source

SNMP NAME	ncdBootDesiredSource

SNMP PATH	ncdBoot 5

TYPE		CHOICE

CHOICES		{ tcpip tftp nfs ncdnet local prom }

DEFAULT		tcpip

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the desired source of the server code

on the next boot of the terminal.

'Local' describes a number of options including

PROM's, Flash Memory cards, battery-backed static

RAM cards, and OTP (One-Time Programmable) cards.

'PROM' is an alias for 'Local' provided for

backwards compatibility.

'TCPIP' is an alias for 'TFTP' provided for

backwards compatibility.

NAME		boot-monitor-inverse-video

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>Boot Monitor Inverse Video

SNMP NAME	ncdBootMonitorReverseVideo

SNMP PATH	ncdBoot 8

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies whether the boot monitor should use

inverse video -- dark characters on a light

background.

NAME		boot-mop

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>Try MOP

SNMP NAME	ncdBootMop

SNMP PATH	ncdBoot 18

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that MOP should be used to request

a server download file.

NAME		boot-mop-with-file-name

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>Try MOP with File Name

SNMP NAME	ncdBootMopWithFileName

SNMP PATH	ncdBoot 19

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that a MOP download request should

specify a file name.

Normally MOP requests do not specify the name of

the file to be loaded.

NAME		boot-nfs-directory

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>NFS Directory

SNMP NAME	ncdBootNfsDirectory

SNMP PATH	ncdBoot 22

TYPE		STRING

DEFAULT		/tftpboot/

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the UNIX directory where the terminal should

search when using NFS to download the server file.

If specified this parameter consumes space in

the optional string space of NVRAM.

This feature is only available with boot monitor

version 2.6.0 or later.

NAME		boot-persistent-loading

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>Persistent Loading

SNMP NAME	ncdBootPersist

SNMP PATH	ncdBoot 6

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies whether the terminal should continue

attempting to boot until it succeeds.

This feature is only available with boot monitor

version 2.3.3 or later.

NAME		boot-previous-server

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>Previous Server

SNMP NAME	ncdBootActualServer

SNMP PATH	ncdBoot 4

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the most recent boot server.

NAME		boot-previous-source

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>Previous Source

SNMP NAME	ncdBootActualSource

SNMP PATH	ncdBoot 3

TYPE		CHOICE

CHOICES		{ tcpip tftp nfs ncdnet local prom }

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the most recent boot source.

'Local' describes a number of options including

PROM's, Flash Memory cards, battery-backed static

RAM cards, and OTP (One-Time Programmable) cards.

'PROM' is an alias for 'Local' provided for

backwards compatibility.

'TCPIP' is an alias for 'TFTP' provided for

backwards compatibility.

NAME		boot-prom-force-update

HIDEBOX		No Widget

SNMP NAME	ncdBootPromForceUpdate

SNMP PATH	ncdBoot 28

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

If true, force the unit to accept the boot prom update file

specified in boot-prom-update-file even if it is a back rev

NAME		boot-prom-update-file

HIDEBOX		No Widget

SNMP NAME	ncdBootPromUpdateFile

SNMP PATH	ncdBoot 27

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

When a non-null path is specified the terminal

attempts to update the boot prom to the image

specified.  Do NOT turn off power to the terminal

while the boot prom is being updated

NAME		boot-second-source

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>Secondary Boot Source

SNMP NAME	ncdBootSecondSource

SNMP PATH	ncdBoot 23

TYPE		CHOICE

CHOICES		{ none tcpip tftp nfs ncdnet local prom }

DEFAULT		none

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies a fallback source of the server code

on the next boot of the terminal.

NAME		boot-send-broadcast-icmp-for-subnet-mask

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>Send Broadcast ICMP for Subnet Mask

SNMP NAME	ncdBootEnableICMPSubnetMaskRequest

SNMP PATH	ncdBoot 16

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Use an ICMP message sent to the broadcast address

to determine the appropriate subnet mask at server

boot. If this is disabled, only the boot server is

sent the ICMP message.

Note that this option can produce a lot of network

traffic.

This feature is only available with boot monitor

version 2.3.3 or later.

NAME		boot-tcpip-broadcast-boot-request

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>TCPIP Broadcast Boot Request

SNMP NAME	ncdBootBroadcastInetRequest

SNMP PATH	ncdBoot 17

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that the boot monitor should broadcast

download file requests.

Note that this option can produce a lot of network

traffic.

This feature is only available with boot monitor

version 2.2.0 or later.

NAME		boot-tcpip-desired-server

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>Primary TCPIP Server

SNMP NAME	ncdBootDesiredInetServer

SNMP PATH	ncdBoot 9

TYPE		STRING

DEFAULT		0.0.0.0

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the IP address of the boot server

when the desired boot source is TCP/IP or NFS.

NAME		boot-tcpip-product-name-and-memory-size-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>TCPIP Product Name and Memory Size File

SNMP NAME	ncdBootUseRelativeMemSizeFile

SNMP PATH	ncdBoot 12

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that the product model form of server

download file name with the memory size suffix

should be used to request a server download file.

This feature is only available with boot monitor

version 2.0.0 or later.

This feature can only be disabled when using boot

monitor version 2.2.0 or later.

NAME		boot-tcpip-product-name-and-memory-size-with-path-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>TCPIP Product Name and Memory Size with Path File

SNMP NAME	ncdBootUseAbsoluteMemSizeFile

SNMP PATH	ncdBoot 13

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that the product model form of server

download file name with the memory size suffix

prefixed with the string /tftpboot/ should be

used to request a server download file.

This feature is only available with boot monitor

version 2.0.0 or later.

This feature can only be disabled when using boot

monitor version 2.2.0 or later.

NAME		boot-tcpip-product-name-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>TCPIP Product Name File

SNMP NAME	ncdBootUseRelativeProductFile

SNMP PATH	ncdBoot 14

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that the product model form of server

download file name should be used to request a

server download file.

This feature is only available with boot monitor

version 2.0.0 or later.

This feature can only be disabled when using boot

monitor version 2.2.0 or later.

NAME		boot-tcpip-product-name-with-path-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>TCPIP Product Name with Path File

SNMP NAME	ncdBootUseAbsoluteProductFile

SNMP PATH	ncdBoot 15

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that the product model form of server

download file name with the prefix /tftpboot/

should be used to request a server download file.

This feature is only available with boot monitor

version 2.0.0 or later.

This feature can only be disabled when using boot

monitor version 2.2.0 or later.

NAME		boot-tcpip-second-server

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>Secondary TCPIP Server

SNMP NAME	ncdBootSecondInetServer

SNMP PATH	ncdBoot 25

TYPE		STRING

DEFAULT		0.0.0.0

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the IP address of the secondary boot server

when the desired boot source is TCP/IP, TFTP, or NFS.

NAME		boot-tcpip-third-server

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>Tertiary TCPIP Server

SNMP NAME	ncdBootThirdInetServer

SNMP PATH	ncdBoot 26

TYPE		STRING

DEFAULT		0.0.0.0

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the IP address of the tertiary boot server

when the desired boot source is TCP/IP, TFTP, or NFS.

NAME		boot-tcpip-unit-address-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>TCPIP Unit Address File

SNMP NAME	ncdBootUseRelativeHexFile

SNMP PATH	ncdBoot 10

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that the hexidecimal representation of

the IP address should be used to request a server

download file.

This feature can only be disabled when using boot

monitor version 2.2.0 or later.

NAME		boot-tcpip-unit-address-with-path-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>TCPIP Unit Address with Path File

SNMP NAME	ncdBootUseAbsoluteHexFile

SNMP PATH	ncdBoot 11

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that the hexidecimal representation of

the IP address prefixed with the string /tftpboot/

should be used to request a server download file.

This feature is only available with boot monitor

version 2.0.0 or later.

This feature can only be disabled when using boot

monitor version 2.2.0 or later.

NAME		boot-test-ram

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>Test RAM

SNMP NAME	ncdBootRamTest

SNMP PATH	ncdBoot 2

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies whether RAM self tests are performed

at power up.

It is suggested that this self test not be

disabled.

NAME		boot-tftp-directory

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>TFTP Directory

SNMP NAME	ncdBootTftpDirectory

SNMP PATH	ncdBoot 21

TYPE		STRING

DEFAULT		/tftpboot/

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the UNIX directory where the terminal should

search when using TFTP to download the server file.

If specified this parameter consumes space in

the optional string space of NVRAM.

This feature is only available with boot monitor

version 2.6.0 or later.

NAME		boot-third-source

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Booting=>Tertiary Boot Source

SNMP NAME	ncdBootThirdSource

SNMP PATH	ncdBoot 24

TYPE		CHOICE

CHOICES		{ none tcpip tftp nfs ncdnet local prom }

DEFAULT		none

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies a fallback source of the server code

on the next boot of the terminal.

GROUP config

NAME		config-access-control-enabled

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Enable Config Access Control

SNMP NAME	ncdAccessConfigdAccessControlEnabled

SNMP PATH	ncdAccess 8

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether requests for connections to the

configuration daemon from outside the terminal will

be checked against the host access control list

NAME		config-access-control-list

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Config Access Control List

SNMP NAME	ncdAccessConfigdHostTable

SNMP PATH	ncdAccess 9

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the host access control list for the

configuration daemon.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdAccessCHTIndex

SNMP PATH	ncdAccessConfigdHostTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Config Access Control List

NAME		host

SNMP NAME	ncdAccessCHTAddress

SNMP PATH	ncdAccessConfigdHostTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the network name/address

of a node granted permission to

remotely access the configuration

daemon.

NAME		family

SNMP NAME	ncdAccessCHTFamily

SNMP PATH	ncdAccessConfigdHostTable 3

TYPE		CHOICE

CHOICES		{ tcpip ncdnet }

DEFAULT		tcpip

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the type of network connection for

which this entry applies.

END TABLE ROW ENTRIES

NAME		config-add-domain-to-unit-name-as-filename

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>Add Domain to Unit Name as Filename

SNMP NAME	ncdConfigInitUseFQUnitName

SNMP PATH	ncdConfig 7

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that the terminal's symbolic name

should include the domain, e.g. ncdu4.ncd.com.

when being used as the file name of the initial

configuration file.

NAME		config-auto-save-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>Auto Save File

SNMP NAME	ncdConfigAutoSaveFile

SNMP PATH	ncdConfig 16

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that any changes made to the terminal's

configuration are automatically saved to a file.

NAME		config-auto-save-file-name

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>Auto Save File Name

SNMP NAME	ncdConfigAutoSaveFileName

SNMP PATH	ncdConfig 17

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the file to which automatic saves of

configuration changes are made.

NAME		config-auto-save-nvram

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>Auto Save NVRAM

SNMP NAME	ncdConfigAutoSaveNvram

SNMP PATH	ncdConfig 15

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that any changes made to the terminal's

configuration that are stored in NVRAM, are

automatically saved.

NAME		config-auto-saved-info

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>Auto Saved Info

SNMP NAME	ncdConfigAutoSaveChanges

SNMP PATH	ncdConfig 18

TYPE		CHOICE

CHOICES		{ user-changes all-changes all-info }

DEFAULT		user-changes

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies what information is automatically

saved to file when configuration changes are made.

NAME		config-console-display-password

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Console Display Password

SNMP NAME	ncdAccessConsoleDisplayPassword

SNMP PATH	ncdAccess 42

TYPE		STRING

DEFAULT		nil

PERMISSION	WRITE-ONLY

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the password required to display the console

NAME		config-console-enforce-password-locally

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Enforce Console Password Locally

SNMP NAME	ncdAccessConsoleEnforcePasswordLocally

SNMP PATH	ncdAccess 43

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that a password is required to access

the console at the terminal. Normally,

the local unit does not require a password to access

it's console.

NAME		config-custom-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>Custom File

SNMP NAME	ncdConfigInitFile

SNMP PATH	ncdConfig 5

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies a non-standard name for the initial

configuration file.

If specified this parameter consumes space in

the optional string space of NVRAM.

NAME		config-default-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>Default File

SNMP NAME	ncdConfigDfltFile

SNMP PATH	ncdConfig 14

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the configuration file which is

considered to define the default configuration

of the terminal. This will be initialized to

the name of the file that is loaded as the

initial configuration file.

NAME		config-enforce-passwords-locally

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Enforce Config Passwords Locally

SNMP NAME	ncdAccessConfigdEnforePasswdsLocally

SNMP PATH	ncdAccess 7

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that a password is required to access

configuration information at the terminal. Normally,

the local unit does not require a password to access

it's own configuration information.

NAME		config-generic-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>Generic File

SNMP NAME	ncdConfigInitUseGenericFile

SNMP PATH	ncdConfig 11

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that the standard generic file

should be used as the file name for the

initial configuration file.

NAME		config-load-initial-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>Load Initial File

SNMP NAME	ncdConfigInitFileLoad

SNMP PATH	ncdConfig 1

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies whether an initial configuration file

should be loaded.

NAME		config-ncdnet-object-name

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>NCDnet Object Name

SNMP NAME	ncdConfigDnetObject

SNMP PATH	ncdConfig 23

TYPE		STRING

DEFAULT		CONFIGD

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the NCDnet object on which the terminal

listens for NCDnet connections to the configuration

daemon.

NAME		config-persistent-loading

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>Persistent Loading

SNMP NAME	ncdConfigInitPersistentFileLoad

SNMP PATH	ncdConfig 2

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies whether the terminal is to persist in

attempting to load an initial configuration file

until it succeeds.

NAME		config-pref-enforce-passwords-locally

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Enforce User Preferences Passwords Locally

SNMP NAME	ncdAccessPrefdEnforcePasswdsLocally

SNMP PATH	ncdAccess 12

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that a password is required to access

user preferences information at the terminal. Normally,

the local unit does not require a password to access

it's own user preferences information.

NAME		config-pref-ncdnet-object-name

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>NCDnet Object Name

SNMP NAME	ncdConfigPrefdDnetObject

SNMP PATH	ncdConfig 21

TYPE		STRING

DEFAULT		PREFD

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the NCDnet object on which the terminal

listens for NCDnet connections to the user

preferences daemon.

NAME		config-pref-read-only-password

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>User Preferences Read-Only Password

SNMP NAME	ncdAccessPrefdReadOnlyPasswd

SNMP PATH	ncdAccess 10

TYPE		STRING

DEFAULT		nil

PERMISSION	WRITE-ONLY

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the password required to obtain read-only access

to the user preference information.

NAME		config-pref-read-write-password

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>User Preferences Read-Write Password

SNMP NAME	ncdAccessPrefdReadWritePasswd

SNMP PATH	ncdAccess 11

TYPE		STRING

DEFAULT		nil

PERMISSION	WRITE-ONLY

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the password required to obtain read-write access

to the user preferences information.

NAME		config-pref-tcp-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>TCP Port

SNMP NAME	ncdConfigPrefdTcpPort

SNMP PATH	ncdConfig 20

TYPE		INTEGER

MIN		1024

DEFAULT		5977

MAX		65535

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the TCP port on which the terminal

listens for raw TCP connections to the user

preferences daemon.

NAME		config-pref-telnet-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>Telnet Port

SNMP NAME	ncdConfigPrefdTelnetPort

SNMP PATH	ncdConfig 19

TYPE		INTEGER

MIN		1024

DEFAULT		5997

MAX		65535

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the TCP port on which the terminal

listens for telnet connections to the user

preferences daemon.

NAME		config-read-only-password

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Config Read-Only Password

SNMP NAME	ncdAccessConfigdReadOnlyPasswd

SNMP PATH	ncdAccess 5

TYPE		STRING

DEFAULT		public

PERMISSION	WRITE-ONLY

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the password required to obtain read-only access

to the configuration information.

NAME		config-read-write-password

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Config Read-Write Password

SNMP NAME	ncdAccessConfigdReadWritePasswd

SNMP PATH	ncdAccess 6

TYPE		STRING

DEFAULT		nil

PERMISSION	WRITE-ONLY

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the password required to obtain read-write access

to the configuration information.

NAME		config-tcp-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>TCP Port

SNMP NAME	ncdConfigTcpPort

SNMP PATH	ncdConfig 13

TYPE		INTEGER

MIN		1024

DEFAULT		5979

MAX		65535

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the TCP port on which the terminal

listens for raw TCP connections to the configuration

daemon.

NAME		config-telnet-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>Telnet Port

SNMP NAME	ncdConfigTelnetPort

SNMP PATH	ncdConfig 12

TYPE		INTEGER

MIN		1024

DEFAULT		5999

MAX		65535

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the TCP port on which the terminal

listens for telnet connections to the configuration

daemon.

NAME		config-unit-ethernet-address-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>Unit Ethernet Address File

SNMP NAME	ncdConfigInitUseEnetUnitAddrFile

SNMP PATH	ncdConfig 8

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that the terminal's Ethernet addresss,

e.g. 0000A7001234, should be used as the file name

for the initial configuration file.

NAME		config-unit-ip-address-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>Unit IP Address File

SNMP NAME	ncdConfigInitUseInetUnitAddrFile

SNMP PATH	ncdConfig 9

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that the terminal's IP addresss,

e.g. C02B9A04, should be used as the file name

for the initial configuration file.

NAME		config-unit-name-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>Unit Name File

SNMP NAME	ncdConfigInitUseUnitNameFile

SNMP PATH	ncdConfig 6

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that the terminal's symbolic name,

e.g. ncdu4, should be used as the file name

for the initial configuration file.

NAME		config-unit-ncdnet-address-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>Unit NCDnet Address File

SNMP NAME	ncdConfigInitUseDnetUnitAddrFile

SNMP PATH	ncdConfig 22

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that the terminal's NCDnet addresss,

e.g. 1.17, should be used as the file name

for the initial configuration file.

NAME		config-unix-directory

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>UNIX Directory

SNMP NAME	ncdConfigDfltUnixDirectory

SNMP PATH	ncdConfig 3

TYPE		STRING

DEFAULT		/usr/lib/X11/ncd/configs/

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the UNIX directory where the terminal should

search for it's initial configuration file.

If specified this parameter consumes space in

the optional string space of NVRAM.

NAME		config-use-decimal-ip-address-notation-as-filename

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>Use Decimal IP Address Notation as Filename

SNMP NAME	ncdConfigInitUseDecimalInetUnitAddr

SNMP PATH	ncdConfig 10

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that the terminal's IP address

should be used in decimal dotted notation,

e.g. 192.43.154.4, when being used as the

file name of the initial configuration file.

NAME		config-vms-directory

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Configuration=>VMS Directory

SNMP NAME	ncdConfigDfltVmsDirectory

SNMP PATH	ncdConfig 4

TYPE		STRING

DEFAULT		NCD_ROOT:[CONFIGS]

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the VMS directory where the terminal should

search for it's initial configuration file.

If specified this parameter consumes space in

the optional string space of NVRAM.

GROUP diag

NAME		diag-abort-on-exception

HIDEBOX		No Widget

SNMP NAME	ncdDiagAbortOnException

SNMP PATH	ncdDiag 10

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Determines whether the terminal should abort when an

exception

occurs in a local application or should just terminate the application.

NAME		diag-access-control-enabled

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Enable Diagnostic Access Control

SNMP NAME	ncdAccessDiagdAccessControlEnabled

SNMP PATH	ncdAccess 24

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether requests for connections to the

diagnostic daemon from outside the terminal will

be checked against the host access control list.

NAME		diag-access-control-list

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Diagnostic Access Control List

SNMP NAME	ncdAccessDiagdAccessControlTable

SNMP PATH	ncdAccess 25

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the host access control list

for the diagnostic daemon.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdAccessDiagdACTIndex

SNMP PATH	ncdAccessDiagdAccessControlTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Diagd Access Control List

NAME		host

SNMP NAME	ncdAccessDiagdACTAddress

SNMP PATH	ncdAccessDiagdAccessControlTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the network name/address

of a node granted permission to

remotely access the diagnostic

daemon.

NAME		family

SNMP NAME	ncdAccessDiagdACTFamily

SNMP PATH	ncdAccessDiagdAccessControlTable 3

TYPE		CHOICE

CHOICES		{ tcpip ncdnet }

DEFAULT		tcpip

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the type of network connection for

which this entry applies.

END TABLE ROW ENTRIES

NAME		diag-add-timestamps

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Diagnostics=>Add Timestamps

SNMP NAME	ncdDiagTimestampEnabled

SNMP PATH	ncdDiag 1

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether the diagnostic daemon should

add timestamps to the diagnostic log.

NAME		diag-buffer-size

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Diagnostics=>Buffer Size

SNMP NAME	ncdDiagConfiguredBufferSize

SNMP PATH	ncdDiag 3

TYPE		INTEGER

MIN		2048

DEFAULT		16384

MAX		16777216

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the maximum size (in bytes)

of the diagnostic log.

NAME		diag-log-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Diagnostics=>Log File

SNMP NAME	ncdDiagLogFile

SNMP PATH	ncdDiag 4

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the file name where to write a copy of the

diagnostic log.

NAME		diag-memory-debugging

HIDEBOX		No Widget

SNMP NAME	ncdMemoryDebugging

SNMP PATH	ncdDiag 9

TYPE		CHOICE

CHOICES		{ none low high }

DEFAULT		none

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Determines the level of memory debugging.

NAME		diag-ncdnet-object-name

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Diagnostics=>NCDnet Object Name

SNMP NAME	ncdDiagDnetObject

SNMP PATH	ncdDiag 7

TYPE		STRING

DEFAULT		DIAGD

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the NCDnet object on which the terminal

listens for NCDnet connections to the diagnostics

daemon.

NAME		diag-severe-system-errors

HIDEBOX		No Widget

SNMP NAME	ncdSevereSystemErrors

SNMP PATH	ncdDiag 8

TYPE		CHOICE

CHOICES		{ ignore warn abort }

DEFAULT		warn

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Determines the action when a severe system error occurs.

NAME		diag-tcp-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Diagnostics=>TCP Port

SNMP NAME	ncdDiagTcpPort

SNMP PATH	ncdDiag 6

TYPE		INTEGER

MIN		1024

DEFAULT		5978

MAX		65535

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the TCP port on which the terminal

listens for raw TCP connections to the diagnostics

daemon.

NAME		diag-telnet-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Diagnostics=>Telnet Port

SNMP NAME	ncdDiagTelnetPort

SNMP PATH	ncdDiag 5

TYPE		INTEGER

MIN		1024

DEFAULT		5998

MAX		65535

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the TCP port on which the terminal

listens for telnet connections to the diagnostics

daemon.

NAME		diag-timestamp-frequency

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Diagnostics=>Timestamp Frequency

SNMP NAME	ncdDiagTimestampGranularity

SNMP PATH	ncdDiag 2

TYPE		INTEGER

MIN		1

DEFAULT		1

MAX		600

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the maximum frequency (in seconds)

at which timestamps are added to the diagnostic log.

GROUP dps

NAME		dps-resource-path

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Display PostScript=>DPS/X resource Path

SNMP NAME	ncdDpsResourcePath

SNMP PATH	ncdDps 1

TYPE		STRING

DEFAULT		/usr/lib/X11/ncd/dps::

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

The path to search for DPS resource files.

GROUP enet

NAME		enet-interfaces-table

HIDEBOX		Console=>Statistics=>Show Statistics=>

Ethernet=>Interfaces Table

SNMP NAME	ncdEtherTable

SNMP PATH	ncdEther 1

TYPE		TABLE

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The table of Ethernet interfaces.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdEtherTableIndex

SNMP PATH	ncdEtherTable 11

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Ethernet Interfaces Table

NAME		packets-received-with-crc-errors

SNMP NAME	ncdEtherInCrcErrors

SNMP PATH	ncdEtherTable 1

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times a incoming frame failed the

Cyclic Redundancy Check (CRC).

NAME		packets-missed

SNMP NAME	ncdEtherInMisseds

SNMP PATH	ncdEtherTable 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times an incoming frame was

not received due to a lack of buffers.

NAME		packets-received-with-framing-errors

SNMP NAME	ncdEtherInFrameErrors

SNMP PATH	ncdEtherTable 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times an incoming frame had a bad bit

alignment.

NAME		single-collisions-on-packet-transmission

SNMP NAME	ncdEtherSingleCollisions

SNMP PATH	ncdEtherTable 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times an outbound frame had a single

collision occur during transmission.

NAME		multiple-collisions-on-packet-transmission

SNMP NAME	ncdEtherMultipleCollisions

SNMP PATH	ncdEtherTable 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times an outbound frame had multiple

collisions occur during transmission.

NAME		late-collisions-on-packet-transmission

SNMP NAME	ncdEtherLateCollisions

SNMP PATH	ncdEtherTable 6

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times the collision signal was asserted

after the preamble of a frame was transmitted.

NAME		deferrals-on-packet-transmission

SNMP NAME	ncdEtherOutDeferreds

SNMP PATH	ncdEtherTable 7

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times an outbound frame could not be

transmitted directly due to the Ethernet being busy,

and the Ethernet exponentail backoff algorithm was

invoked.

NAME		retry-failures-on-packet-transmission

SNMP NAME	ncdEtherOutRetryErrors

SNMP PATH	ncdEtherTable 8

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times an outbound frame could not be

transmitted after 16 backed-off attempts due to

repeated collisions on the Ethernet.

NAME		lost-carrier

SNMP NAME	ncdEtherLostCarriers

SNMP PATH	ncdEtherTable 9

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times carrier (Receiver Enable)

was lost during the transmission of a frame.

NAME		restarts

SNMP NAME	ncdEtherRestarts

SNMP PATH	ncdEtherTable 10

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times the software has restarted the

Ethernet controller, possibly due to a heuristic for

determining an inoperable controller.

NAME		fifo-overflow

SNMP NAME	ncdEtherOverflow

SNMP PATH	ncdEtherTable 12

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of FIFO overflows detected.

NAME		incoming-giant-packets

SNMP NAME	ncdEtherLongIn

SNMP PATH	ncdEtherTable 13

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of incoming packets larger than legal ethernet size.

NAME		outgoing-giant-packets

SNMP NAME	ncdEtherLongOut

SNMP PATH	ncdEtherTable 14

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of outgoing packets larger than legal ethernet size.

These packets are discarded before being transmitted to the ethernet.

NAME		ring-overflows

SNMP NAME	ncdEtherRingOverflow

SNMP PATH	ncdEtherTable 15

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times the receive ring overflowed.

NAME		fifo-underflow

SNMP NAME	ncdEtherUnderflow

SNMP PATH	ncdEtherTable 16

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of FIFO underflows detected.

END TABLE ROW ENTRIES

GROUP exec

NAME		exec-access-control-enabled

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Enable Execution Host Access Control

SNMP NAME	ncdAccessExecdAccessControlEnabled

SNMP PATH	ncdAccess 28

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether xhost-style access control

is on or off for remote requests for local

command execution.

NAME		exec-access-control-list

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Execution Access Control List

SNMP NAME	ncdAccessExecdAccessControlTable

SNMP PATH	ncdAccess 29

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the host access control list for the local

command execution daemon.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdAccessExecdACTIndex

SNMP PATH	ncdAccessExecdAccessControlTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Execution Access Control List

NAME		host

SNMP NAME	ncdAccessExecdACTAddress

SNMP PATH	ncdAccessExecdAccessControlTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the network name/address

of a node granted permission to

remotely request the execution

of local commands.

NAME		family

SNMP NAME	ncdAccessExecdACTFamily

SNMP PATH	ncdAccessExecdAccessControlTable 3

TYPE		CHOICE

CHOICES		{ tcpip ncdnet }

DEFAULT		tcpip

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the type of network connection for

which this entry applies.

END TABLE ROW ENTRIES

NAME		exec-command-menu

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Commands and Startup=>Command Menu

SNMP NAME	ncdExecCommandsMenuTable

SNMP PATH	ncdExec 3

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the contents of the command menu.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdExecCMIndex

SNMP PATH	ncdExecCommandsMenuTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Startup Commands Table

NAME		menu-label

SNMP NAME	ncdExecCMCommandsLabel

SNMP PATH	ncdExecCommandsMenuTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the label to be displayed in the menu.

NAME		command

SNMP NAME	ncdExecCMCommandsCommand

SNMP PATH	ncdExecCommandsMenuTable 3

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the command to be executed.

END TABLE ROW ENTRIES

NAME		exec-disabled-commands

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Commands and Startup=>Disabled Commands

SNMP NAME	ncdExecDisabledCommandsTable

SNMP PATH	ncdExec 2

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the list of local commands that should

be disabled so that the user of the unit is

prevented from using these commands.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdExecDCIndex

SNMP PATH	ncdExecDisabledCommandsTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Disabled Commands Table

NAME		command

SNMP NAME	ncdExecDCCommands

SNMP PATH	ncdExecDisabledCommandsTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the name of the local command

that should be disabled.

END TABLE ROW ENTRIES

NAME		exec-startup-commands

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Commands and Startup=>Startup Commands

SNMP NAME	ncdExecStartupCommandsTable

SNMP PATH	ncdExec 1

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the list of commands to be executed

when a session is started.  The commands can

be anything allowed by the exec daemon.

NB: the defaults for standard servers is to enable

login and console; xremote servers will have both

wm and serial as defaults

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdExecSTCIndex

SNMP PATH	ncdExecStartupCommandsTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Startup Commands Table

NAME		command

SNMP NAME	ncdExecSTCCommands

SNMP PATH	ncdExecStartupCommandsTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the command to be executed.

END TABLE ROW ENTRIES

NAME		exec-user-access-control

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Enable Execution User Access Control

SNMP NAME	ncdAccessExecdUserAccess

SNMP PATH	ncdAccess 37

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether remote requests for local command

execution should only be allowed if requested by the

user who is currently logged into the terminal.  Note

that this option is only effective if the NCD Display

Manager is being used.

GROUP file

NAME		file-default-vms-file-name-extension

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

File Service=>Default VMS File Name Extension

SNMP NAME	ncdFileDfltVmsExtension

SNMP PATH	ncdFile 11

TYPE		STRING

DEFAULT		.DAT

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the default file extension appended to file

names if one does not already exist when referencing

the file on a file server with VMS style names.

NAME		file-enable-floppy-filesystem

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

File Service=>Enable Floppy Filesystem

SNMP NAME	ncdFileDosFloppyEnabled

SNMP PATH	ncdFile 16

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

If true the local floppy drive will be accessible via

the local file manager or from another host via NFS.

NAME		file-enable-nfs-server

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

File Service=>Enable NFS Server

SNMP NAME	ncdFileNfsServerEnabled

SNMP PATH	ncdFile 15

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies that the NFS server daemon should be started

up at boot time so that file systems within the terminal

may be accessed by remote hosts via NFS.

NAME		file-export-directory-list

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

File Service=>Export Directory List

SNMP NAME	ncdFileExportDirectoryTable

SNMP PATH	ncdFile 17

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies file service access points accessible by remote

systems.  These access points do not physically exist in

the terminal's file system, but instead are links to real

directories.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdFileEDTIndex

SNMP PATH	ncdFileExportDirectoryTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Export Directory List.

NAME		export-directory

SNMP NAME	ncdFileEDTExportDirectory

SNMP PATH	ncdFileExportDirectoryTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies a directory accessible from

remote units.

NAME		equivalent-physical-directory

SNMP NAME	ncdFileEDTEquivalentDirectory

SNMP PATH	ncdFileExportDirectoryTable 3

TYPE		STRING

DEFAULT		/local/

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the physical directory found in a

filesystem local to the terminal.

END TABLE ROW ENTRIES

NAME		file-extended-diagnostics

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

File Service=>Extended Diagnostics

SNMP NAME	ncdFileExtendedDiagnostics

SNMP PATH	ncdFile 7

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies that the unit should generate more detailed

diagnostic messages regarding file service.

NAME		file-failed-server-ignore-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

File Service=>Failed Server Ignore Timeout

SNMP NAME	ncdFileServerHolddownTime

SNMP PATH	ncdFile 9

TYPE		INTEGER

MIN		0

DEFAULT		120

MAX		600

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) to ignore a file server

which has failed due to a network timeout error.

An ignored server is skipped over when attempt to open

a new file. This behavior speeds up unit booting and

session reset when the primary file server has failed.

NAME		file-initial-protocol-1

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

File Service=>Initial Protocol 1

SNMP NAME	ncdFileInitialFileServer1Protocol

SNMP PATH	ncdFile 2

TYPE		CHOICE

CHOICES		{ tftp nfs ncdnet nfs/tcp local smb use-boot-protocol }

DEFAULT		tftp

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the file service protocol to be used

in conjunction with the primary initial file server.

NAME		file-initial-protocol-2

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

File Service=>Initial Protocol 2

SNMP NAME	ncdFileInitialFileServer2Protocol

SNMP PATH	ncdFile 4

TYPE		CHOICE

CHOICES		{ tftp nfs ncdnet nfs/tcp local smb use-boot-protocol }

DEFAULT		tftp

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the file service protocol to be used

in conjunction with the secondary initial file server.

NAME		file-initial-server-1

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

File Service=>Initial Server 1

SNMP NAME	ncdFileInitialFileServer1

SNMP PATH	ncdFile 1

TYPE		STRING

DEFAULT		0.0.0.0

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the host to be used as the primary file server

at boot time to load the initial config file.

A value of 0.0.0.0 implies that the boot server should be

used for this value.

NAME		file-initial-server-2

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

File Service=>Initial Server 2

SNMP NAME	ncdFileInitialFileServer2

SNMP PATH	ncdFile 3

TYPE		STRING

DEFAULT		0.0.0.0

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the host to be used as the secondary file server

at boot time to load the initial config file.

A value of 0.0.0.0 implies that the boot server should be

used for this value.

NAME		file-manager-access-control-enabled

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Enable File Manager Access Control

SNMP NAME	ncdAccessFiledAccessControlEnabled

SNMP PATH	ncdAccess 34

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether xhost-style access control

is on or off for remote requests for file

manager daemon.

NAME		file-manager-access-control-list

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>File Manager Access Control List

SNMP NAME	ncdAccessFiledAccessControlTable

SNMP PATH	ncdAccess 35

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the host access control list for the

file manager daemon.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdAccessFiledACTIndex

SNMP PATH	ncdAccessFiledAccessControlTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the File Manager Access Control List

NAME		host

SNMP NAME	ncdAccessFiledACTAddress

SNMP PATH	ncdAccessFiledAccessControlTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the network name/address

of a node granted permission to

remotely request the execution

of local commands.

NAME		family

SNMP NAME	ncdAccessFiledACTFamily

SNMP PATH	ncdAccessFiledAccessControlTable 3

TYPE		CHOICE

CHOICES		{ tcpip ncdnet }

DEFAULT		tcpip

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the type of network connection for

which this entry applies.

END TABLE ROW ENTRIES

NAME		file-manager-ncdnet-object-name

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

File Manager=>NCDnet Object Name

SNMP NAME	ncdFileManagerDnetObject

SNMP PATH	ncdFile 14

TYPE		STRING

DEFAULT		FILED

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the NCDnet object on which the terminal

listens for NCDnet connections to the file manager

daemon.

NAME		file-manager-password

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Password

SNMP NAME	ncdAccessFileManagerPasswd

SNMP PATH	ncdAccess 33

TYPE		STRING

DEFAULT		nil

PERMISSION	WRITE-ONLY

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the password required to obtain access

to the file manager.

NAME		file-manager-tcp-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

File Manager=>TCP Port

SNMP NAME	ncdFileManagerTcpPort

SNMP PATH	ncdFile 13

TYPE		INTEGER

MIN		1024

DEFAULT		5976

MAX		65535

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the TCP port on which the terminal

listens for raw TCP connections to the file

manager daemon.

NAME		file-manager-telnet-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

File Manager=>Telnet Port

SNMP NAME	ncdFileManagerTelnetPort

SNMP PATH	ncdFile 12

TYPE		INTEGER

MIN		1024

DEFAULT		5996

MAX		65535

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the TCP port on which the terminal

listens for telnet connections to the file manager

daemon.

NAME		file-name-type-for-initial-tftp-servers

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

File Service=>Name Type for Initial TFTP Servers

SNMP NAME	ncdFileInitialFileServersUseVmsTftp

SNMP PATH	ncdFile 5

TYPE		CHOICE

CHOICES		{ unix vms }

DEFAULT		unix

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the type of file names to be sent via TFTP

in conjunction with the initial file servers.

NAME		file-nfs-access-control-default

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>NFS Server Access Control Default

SNMP NAME	ncdAccessNfsdAccessControlDefault

SNMP PATH	ncdAccess 39

TYPE		CHOICE

CHOICES		{ no-access read-only read-write }

DEFAULT		no-access

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Default access control policy for requests to the NFS

server daemon.

NAME		file-nfs-access-control-list

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>NFS Server Access Control List

SNMP NAME	ncdAccessNfsdAccessControlTable

SNMP PATH	ncdAccess 40

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the host access control list

for requests to the NFS server daemon.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdAccessNfsdACTIndex

SNMP PATH	ncdAccessNfsdAccessControlTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the NFS Server Access Control List

NAME		file-system

SNMP NAME	ncdAccessNfsdACTFileSystem

SNMP PATH	ncdAccessNfsdAccessControlTable 2

TYPE		CHOICE

CHOICES		{ /local /xfloppy }

DEFAULT		/local

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

File system in terminal that can be accessed

remotely via NFS.

NAME		host

SNMP NAME	ncdAccessNfsdACTHost

SNMP PATH	ncdAccessNfsdAccessControlTable 3

TYPE		STRING

DEFAULT		0.0.0.0

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the host name/IP address

of a node granted permission to

remotely access the NFS server

daemon.

NAME		permission

SNMP NAME	ncdAccessNfsdACTPermission

SNMP PATH	ncdAccessNfsdAccessControlTable 4

TYPE		CHOICE

CHOICES		{ no-access read-only read-write }

DEFAULT		no-access

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the type of access allowed for the remote

host to the NFS file system.

END TABLE ROW ENTRIES

NAME		file-nfs-gid

HIDEBOX		No Widget

SNMP NAME	ncdAccessNfsGroupId

SNMP PATH	ncdAccess 27

TYPE		INTEGER

MIN		-2147483648

DEFAULT		-2

MAX		2147483647

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the GID (Group ID) to be used for NFS and RPC

requests.

NAME		file-nfs-uid

HIDEBOX		No Widget

SNMP NAME	ncdAccessNfsUserId

SNMP PATH	ncdAccess 26

TYPE		INTEGER

MIN		-2147483648

DEFAULT		-2

MAX		2147483647

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the UID (User ID) to be used for NFS and RPC

requests.

NAME		file-nfs-unmount-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

File Service=>NFS Unmount Timeout

SNMP NAME	ncdFileUnmountTime

SNMP PATH	ncdFile 10

TYPE		INTEGER

MIN		1

DEFAULT		1800

MAX		3600

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) after the last reference

to an NFS file service entry that the unit automatically

notifies the server that it no longer requires access

to the file system.

NAME		file-service-table

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

File Service=>File Service Table

SNMP NAME	ncdFileServiceTable

SNMP PATH	ncdFile 6

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the file service table.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdFileSTIndex

SNMP PATH	ncdFileServiceTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the File Service Table

NAME		local-unix-mount-point

SNMP NAME	ncdFileSTLocalUnixMountPoint

SNMP PATH	ncdFileServiceTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the unit's UNIX style local name

for this file service access point.

NAME		local-vms-mount-point

SNMP NAME	ncdFileSTLocalVmsMountPoint

SNMP PATH	ncdFileServiceTable 3

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the unit's VMS style local name

for this file service access point.

NAME		server

SNMP NAME	ncdFileSTServerAddress

SNMP PATH	ncdFileServiceTable 4

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the file server.

NAME		protocol

SNMP NAME	ncdFileSTProtocol

SNMP PATH	ncdFileServiceTable 5

TYPE		CHOICE

CHOICES		{ tftp nfs ncdnet nfs/tcp local smb use-boot-protocol }

DEFAULT		tftp

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the file service protocol.

NAME		server-mount-point

SNMP NAME	ncdFileSTServerMountPoint

SNMP PATH	ncdFileServiceTable 6

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the name on the file server

of the file service access point.

NAME		file-name-type

SNMP NAME	ncdFileSTFileNameType

SNMP PATH	ncdFileServiceTable 7

TYPE		CHOICE

CHOICES		{ unknown unix vms }

DEFAULT		unknown

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the type of file names

used by the file server. This is not

used with NFS file servers.

NAME		retransmission-timeout

SNMP NAME	ncdFileSTRetransTimeout

SNMP PATH	ncdFileServiceTable 8

TYPE		INTEGER

MIN		0

DEFAULT		3

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) between

successive transmissions of a file service

request. Note this is only used with file

service protocols running over connectionless

transport services, e.g. NFS or TFTP.

NAME		transaction-timeout

SNMP NAME	ncdFileSTTransTimeout

SNMP PATH	ncdFileServiceTable 9

TYPE		INTEGER

MIN		0

DEFAULT		30

MAX		100000

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) to attempt

a file service request before a failure

situation is declared.

NAME		read-size

SNMP NAME	ncdFileSTReadSize

SNMP PATH	ncdFileServiceTable 10

TYPE		INTEGER

MIN		0

DEFAULT		8192

MAX		8192

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of data in bytes

to be requested in a single read request.

This parameter is only used with NFS or

NFS/TCP. If you are having problems reading

files with NFS across gateways, try decreasing

this parameter to 1024.

NAME		write-size

SNMP NAME	ncdFileSTWriteSize

SNMP PATH	ncdFileServiceTable 11

TYPE		INTEGER

MIN		0

DEFAULT		8192

MAX		8192

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of data in bytes

to be requested in a single write request.

This parameter is only used with NFS or

NFS/TCP. If you are having problems writing

files with NFS across gateways, try decreasing

this parameter to 1024.

END TABLE ROW ENTRIES

NAME		file-smb-default-password

HIDEBOX		No Widget

SNMP NAME	ncdSMBDefaultPassword

SNMP PATH	ncdFile 19

TYPE		STRING

DEFAULT

PERMISSION	WRITE-ONLY

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the default password for use in SMB

authorization.

NAME		file-smb-default-username

HIDEBOX		No Widget

SNMP NAME	ncdSMBDefaultUsername

SNMP PATH	ncdFile 18

TYPE		STRING

DEFAULT		guest

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the default username for use in SMB

authorization.

NAME		file-try-all-matches-on-open

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

File Service=>Try All Matches on Open

SNMP NAME	ncdFileRegressiveSearch

SNMP PATH	ncdFile 8

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that the unit should try all file service

entries that match when opening a file, until it

succeeds in opening the file, or there are no more

matches remain. If this is not enabled, then the unit

only tries the longest match(es) in the file service table.

GROUP font

NAME		font-cache-hits

HIDEBOX		Console=>Statistics=>Show Statistics=>

Font Usage=>Cache Hits

SNMP NAME	ncdFontServiceCacheHits

SNMP PATH	ncdFontService 7

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the number of fonts cache hits

NAME		font-cache-size

HIDEBOX		Console=>Statistics=>Show Statistics=>

Font Usage=>Cache Size

SNMP NAME	ncdFontServiceCacheSize

SNMP PATH	ncdFontService 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the total size of the font cache

NAME		font-cache-used

HIDEBOX		Console=>Statistics=>Show Statistics=>

Font Usage=>Cache Used

SNMP NAME	ncdFontServiceCacheUsed

SNMP PATH	ncdFontService 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the portion of cache memory that is in use

NAME		font-number-of-fonts-in-cache

HIDEBOX		Console=>Statistics=>Show Statistics=>

Font Usage=>Number of Fonts in Cache

SNMP NAME	ncdFontServiceNumCachedFonts

SNMP PATH	ncdFontService 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the number of fonts currently in cache

NAME		font-object-count

HIDEBOX		Console=>Statistics=>Show Statistics=>

Font Usage=>Object Count

SNMP NAME	ncdFontServiceFontsInUse

SNMP PATH	ncdFontService 9

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the current number of fonts objects

NAME		font-objects-currently-in-use

HIDEBOX		Console=>Statistics=>Show Statistics=>

Font Usage=>Objects Currently in Use

SNMP NAME	ncdFontServiceReferenceIncs

SNMP PATH	ncdFontService 6

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the number of fonts objects that are

in use currently

NAME		font-total-fonts-loaded

HIDEBOX		Console=>Statistics=>Show Statistics=>

Font Usage=>Total Fonts Loaded

SNMP NAME	ncdFontServiceNetLoads

SNMP PATH	ncdFontService 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the total number of fonts loaded

since the terminal has been in use

GROUP icmp

NAME		icmp-address-mask-replies-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Address Mask Replies Received

SNMP NAME	icmpInAddrMasksReps

SNMP PATH	icmp 13

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Address Mask Reply messages received.

NAME		icmp-address-mask-replies-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Address Mask Replies Transmitted

SNMP NAME	icmpOutAddrMaskReps

SNMP PATH	icmp 26

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Address Mask Reply messages sent.

NAME		icmp-address-mask-requests-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Address Mask Requests Received

SNMP NAME	icmpInAddrMasks

SNMP PATH	icmp 12

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Address Mask Request messages received.

NAME		icmp-address-mask-requests-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Address Mask Requests Transmitted

SNMP NAME	icmpOutAddrMasks

SNMP PATH	icmp 25

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Address Mask Request messages sent.

NAME		icmp-destination-unreachables-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Destination Unreachables Received

SNMP NAME	icmpInDestUnreachs

SNMP PATH	icmp 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Destination Unreachable messages received.

NAME		icmp-destination-unreachables-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Destination Unreachables Transmitted

SNMP NAME	icmpOutDestUnreachs

SNMP PATH	icmp 16

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Destination Unreachable messages sent.

NAME		icmp-echo-replies-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Echo Replies Received

SNMP NAME	icmpInEchoReps

SNMP PATH	icmp 9

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Echo Reply messages received.

NAME		icmp-echo-replies-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Echo Replies Transmitted

SNMP NAME	icmpOutEchoReps

SNMP PATH	icmp 22

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Echo Reply messages sent.

NAME		icmp-echo-requests-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Echo Requests Received

SNMP NAME	icmpInEchos

SNMP PATH	icmp 8

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Echo (request) messages received.

NAME		icmp-echo-requests-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Echo Requests Transmitted

SNMP NAME	icmpOutEchos

SNMP PATH	icmp 21

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Echo (request) messages sent.

NAME		icmp-packets-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Packets Received

SNMP NAME	icmpInMsgs

SNMP PATH	icmp 1

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of ICMP messages which the

entity received.  Note that this counter includes

all those counted by ICMP Packets Received with Errors.

NAME		icmp-packets-received-with-errors

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Packets Received with Errors

SNMP NAME	icmpInErrors

SNMP PATH	icmp 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP messages which the entity

received but determined as having ICMP-specific

errors (bad ICMP checksums, bad length, etc.).

NAME		icmp-packets-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Packets Transmitted

SNMP NAME	icmpOutMsgs

SNMP PATH	icmp 14

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of ICMP messages which this

entity attempted to send.  Note that this counter

includes all those counted by ICMP Packets Unable

to be Transmitted.

NAME		icmp-packets-unable-to-be-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Packets Unable to be Transmitted

SNMP NAME	icmpOutErrors

SNMP PATH	icmp 15

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP messages which this entity did

not send due to problems discovered within ICMP

such as a lack of buffers.  This value should not

include errors discovered outside the ICMP layer

such as the inability of IP to route the resultant

datagram.  In some implementations there may be no

types of error which contribute to this counter's value.

NAME		icmp-parameter-problems-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Parameter Problems Received

SNMP NAME	icmpInParamProbs

SNMP PATH	icmp 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Parameter Problem messages received.

NAME		icmp-parameter-problems-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Parameter Problems Transmitted

SNMP NAME	icmpOutParamProbs

SNMP PATH	icmp 18

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Parameter Problem messages sent.

NAME		icmp-redirects-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Redirects Received

SNMP NAME	icmpInRedirects

SNMP PATH	icmp 7

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Redirect messages received.

NAME		icmp-redirects-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Redirects Transmitted

SNMP NAME	icmpOutRedirects

SNMP PATH	icmp 20

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Redirect messages sent.  For an

NCD unit, this object will always be zero, since non-routers

do not send redirects.

NAME		icmp-source-quenches-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Source Quenches Received

SNMP NAME	icmpInSrcQuenchs

SNMP PATH	icmp 6

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Source Quench messages received.

NAME		icmp-source-quenches-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Source Quenches Transmitted

SNMP NAME	icmpOutSrcQuenches

SNMP PATH	icmp 19

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Source Quench messages sent.

NAME		icmp-time-exceededs-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Time Exceededs Received

SNMP NAME	icmpInTimeExcds

SNMP PATH	icmp 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Time Exceeded messages received.

NAME		icmp-time-exceededs-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Time Exceededs Transmitted

SNMP NAME	icmpOutTimeExcds

SNMP PATH	icmp 17

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Time Exceeded messages sent.

NAME		icmp-timestamp-replies-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Timestamp Replies Received

SNMP NAME	icmpInTimestampReps

SNMP PATH	icmp 11

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Timestamp Reply messages received.

NAME		icmp-timestamp-replies-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Timestamp Replies Transmitted

SNMP NAME	icmpOutTimestampReps

SNMP PATH	icmp 24

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Timestamp Reply messages sent.

NAME		icmp-timestamp-requests-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Timestamp Requests Transmitted

SNMP NAME	icmpOutTimestamps

SNMP PATH	icmp 23

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Timestamp (request) messages sent.

NAME		icmp-timestamps-requests-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

ICMP=>Timestamps Requests Received

SNMP NAME	icmpInTimestamps

SNMP PATH	icmp 10

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of ICMP Timestamp (request) messages received.

GROUP ip

NAME		ip-address

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

IP=>IP Address

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		YES

DESCRIPTION

Specifies the current IP address of the unit.

NAME		ip-address-at-next-boot

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

IP=>IP Address At Next Boot

SNMP NAME	ncdIpNewAddress

SNMP PATH	ncdIp 1

TYPE		STRING

DEFAULT		0.0.0.0

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the IP address for the unit

the next time it is booted.

Note that if the current IP address is

0.0.0.0, the value of this parameter is

used immediately as the current IP address.

Otherwise, the current IP address cannot

be modified while the unit is running because

this would lead to bizarre and unexpected behaviors

on both the unit and any connected hosts.

NAME		ip-address-table

SNMP NAME	ipAddrTable

SNMP PATH	ip 20

TYPE		TABLE

VIEW		SNMP ONLY

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

IP addressing information relative to the respective interfaces

BEGIN TABLE ROW ENTRIES

NAME		ip-entry-address

SNMP NAME	ipAdEntAddr

SNMP PATH	ipAddrTable 1

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the current IP address of the unit.

NAME		ip-entry-if-index

SNMP NAME	ipAdEntIfIndex

SNMP PATH	ipAddrTable 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Interface Table for this Interface

NAME		ip-entry-subnet-mask

SNMP NAME	ipAdEntNetMask

SNMP PATH	ipAddrTable 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the IP subnet mask that is used to

determine which portion of the unit's IP address

corresponds to the network number and which

portion to the host number.

NAME		ip-entry-broadcast-address

SNMP NAME	ipAdEntBcastAddr

SNMP PATH	ipAddrTable 4

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the IP broadcast address to be used

when the unit sends a broadcast packet, and in

recognizing the receipt of broadcast IP packets

to a non-standard IP broadcast address.

NAME		largest-ip-datagram-reassembled

SNMP NAME	ipAdEntReasmMaxSize

SNMP PATH	ipAddrTable 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The size of the largest IP datagram that

can be reassembled from incoming IP fragmented

datagrams received on this interface.

END TABLE ROW ENTRIES

NAME		ip-broadcast-address

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

IP=>Broadcast Address

TYPE		STRING

DEFAULT		255.255.255.255

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the IP broadcast address to be used

when the unit sends a broadcast packet, and in

recognizing the receipt of broadcast IP packets

to a non-standard IP broadcast address.

NAME		ip-default-packet-time-to-live

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

IP=>Default Packet Time-to-Live

SNMP NAME	ipDefaultTTL

SNMP PATH	ip 2

TYPE		INTEGER

MIN		1

DEFAULT		255

MAX		255

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

The default value inserted into the Time-To-Live

field of the IP header of datagrams originated at

this entity, whenever a TTL value is not supplied

by the transport layer protocol.

NAME		ip-equivalent-addresses

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

IP=>Equivalent IP Addresses

SNMP NAME	ncdIpEquivAddressesTable

SNMP PATH	ncdIp 11

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the list of equivalent IP addresses.

BEGIN TABLE ROW ENTRIES

NAME		equivalent-ip-address-table-index

SNMP NAME	ncdIpEATIndex

SNMP PATH	ncdIpEquivAddressesTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The index into the Equivalent IP Address Table

NAME		addresses

SNMP NAME	ncdIpEATAddresses

SNMP PATH	ncdIpEquivAddressesTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies a list of IP addresses or IP host

names that are to be considered as originating

from the same host system. This is useful

when the unit will be receiving UDP packets

from a multihomed host.

END TABLE ROW ENTRIES

NAME		ip-extended-routing-diagnostics

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

IP=>Extended Routing Diagnostics

SNMP NAME	ncdIpRouteExtendedDiagnostics

SNMP PATH	ncdIp 8

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that the unit should generate more detailed

diagnostic messages regarding IP routing.

NAME		ip-forwarding-type

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>IP Forwarding Type

SNMP NAME	ipForwarding

SNMP PATH	ip 1

TYPE		CHOICE

CHOICES		{ gateway host }

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The indication of whether this entity is acting

as an IP gateway in respect to the forwarding of

datagrams received by, but not addressed to, this

entity.  IP gateways forward datagrams.  IP hosts

do not (except those source-routed via the host).

(NCD units will never act as a gateway).

NAME		ip-initial-default-gateway-1

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

IP=>Initial Default Gateway 1

SNMP NAME	ncdIpInitDfltGatewy1

SNMP PATH	ncdIp 2

TYPE		STRING

DEFAULT		0.0.0.0

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the primary default IP gateway to

be used initially in the IP routing table.

The IP routing table can change over time due

to normal operation and various management events.

NAME		ip-initial-default-gateway-2

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

IP=>Initial Default Gateway 2

SNMP NAME	ncdIpInitDfltGatewy2

SNMP PATH	ncdIp 3

TYPE		STRING

DEFAULT		0.0.0.0

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the secondary default IP gateway to

be used initially in the IP routing table.

The IP routing table can change over time due

to normal operation and various management events.

NAME		ip-packet-fragmentations-completed

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Packet Fragmentations Completed

SNMP NAME	ipFragOKs

SNMP PATH	ip 17

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of IP datagrams that have been

successfully fragmented at this entity.

NAME		ip-packet-fragmentations-not-completed

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Packet Fragmentations Not Completed

SNMP NAME	ipFragFails

SNMP PATH	ip 18

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of IP datagrams that have been

discarded because they needed to be fragmented at

this entity but could not be, e.g., because their

Don't Fragment flag was set.

NAME		ip-packet-fragments-created

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Packet Fragments Created

SNMP NAME	ipFragCreates

SNMP PATH	ip 19

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of IP datagram fragments that have

been generated as a result of fragmentation at

this entity.

NAME		ip-packet-reassemblies-completed

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Packet Reassemblies Completed

SNMP NAME	ipReasmOKs

SNMP PATH	ip 15

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of IP datagrams successfully re-assembled.

NAME		ip-packet-reassemblies-not-completed

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Packet Reassemblies Not Completed

SNMP NAME	ipReasmFails

SNMP PATH	ip 16

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of failures detected by the IP re-

assembly algorithm (for whatever reason: timed

out, errors, etc).

NAME		ip-packet-reassemblies-required

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Packet Reassemblies Required

SNMP NAME	ipReasmReqds

SNMP PATH	ip 14

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of IP fragments received which needed

to be reassembled at this entity.

NAME		ip-packet-reassembly-timeout

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Packet Reassembly Timeout

SNMP NAME	ipReasmTimeout

SNMP PATH	ip 13

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The maximum number of seconds which received

fragments are held while they are awaiting

reassembly at this entity.

NAME		ip-packet-transmission-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Packet Transmission Requests

SNMP NAME	ipOutRequests

SNMP PATH	ip 10

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of IP datagrams which local IP

user-protocols (including ICMP) supplied to IP in

requests for transmission.  Note that this counter

does not include any datagrams counted in

ipForwDatagrams.

NAME		ip-packet-transmission-requests-discarded

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Packet Transmission Requests Discarded

SNMP NAME	ipOutDiscards

SNMP PATH	ip 11

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of output IP datagrams for which no

problem was encountered to prevent their

transmission to their destination, but which were

discarded (e.g., for lack of buffer space).  Note

that this counter would include datagrams counted

in Packets Forwarded if any such packets met this

(discretionary) discard criterion.

NAME		ip-packet-transmission-requests-without-routes

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Packet Transmission Requests without Routes

SNMP NAME	ipOutNoRoutes

SNMP PATH	ip 12

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of IP datagrams discarded because no

route could be found to transmit them to their

destination.  Note that this counter includes any

packets counted in Packets Forwarded which meet this

`no-route' criterion.  Note that this includes any

datagrams which a host cannot route because all of

its default gateways are down.

NAME		ip-packets-forwarded

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Packets Forwarded

SNMP NAME	ipForwDatagrams

SNMP PATH	ip 6

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of input datagrams for which this

entity was not their final IP destination, as a

result of which an attempt was made to find a

route to forward them to that final destination.

In entities which do not act as IP Gateways, this

counter will include only those packets which were

Source-Routed via this entity, and the Source-

Route option processing was successful.

NAME		ip-packets-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Packets Received

SNMP NAME	ipInReceives

SNMP PATH	ip 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of input datagrams received from

interfaces, including those received in error.

NAME		ip-packets-received-and-delivered

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Received Packets Delivered

SNMP NAME	ipInDiscards

SNMP PATH	ip 9

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of input datagrams successfully

delivered to IP user-protocols (including ICMP).

NAME		ip-packets-received-and-discarded

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Received Packets Discarded

SNMP NAME	ipInDiscards

SNMP PATH	ip 8

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of input IP datagrams for which no

problems were encountered to prevent their

continued processing, but which were discarded

(e.g., for lack of buffer space).  Note that this

counter does not include any datagrams discarded

while awaiting re-assembly.

NAME		ip-packets-received-with-errors-in-ip-header

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Packets Received with Errors in IP Header

SNMP NAME	ipInHdrErrors

SNMP PATH	ip 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of input datagrams discarded due to

errors in their IP headers, including bad

checksums, version number mismatch, other format

errors, time-to-live exceeded, errors discovered

in processing their IP options, etc.

NAME		ip-packets-received-with-incorrect-address

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Packets Received with Incorrect Address

SNMP NAME	ipInAddrErrors

SNMP PATH	ip 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of input datagrams discarded because

the IP address in their IP header's destination

field was not a valid address to be received at

this entity.  This count includes invalid

addresses (e.g., 0.0.0.0) and addresses of

unsupported Classes (e.g., Class E).  For entities

which are not IP Gateways and therefore do not

forward datagrams (read all NCD's), this counter

includes datagrams discarded because the destination

address was not a local address.

NAME		ip-packets-received-with-unknown-protocols

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Packets Received with Unknown Protocols

SNMP NAME	ipInUnknownProtos

SNMP PATH	ip 7

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of locally-addressed datagrams

received successfully but discarded because of an

unknown or unsupported protocol.

NAME		ip-respond-to-broadcast-icmp

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

IP=>Respond to Broadcast ICMP

SNMP NAME	ncdIpRspToBcastIcmp

SNMP PATH	ncdIp 9

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that the NCD should responsd to ICMP

request messages that were sent to an IP broadcast

address.

NAME		ip-routing-table

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

IP=>Routing Table

SNMP NAME	ipRouteTable

SNMP PATH	ip 21

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the IP routing table.

BEGIN TABLE ROW ENTRIES

NAME		destination

SNMP NAME	ipRouteDest

SNMP PATH	ipRouteTable 1

TYPE		STRING

DEFAULT		0.0.0.0

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

The destination IP address of this route.  An

entry with a value of 0.0.0.0 is considered a

default route.  Multiple routes to a single

destination can appear in the table.

NAME		gateway

SNMP NAME	ipRouteNextHop

SNMP PATH	ipRouteTable 7

TYPE		STRING

DEFAULT		0.0.0.0

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

The IP address of the next hop of this route.

(In the case of a route bound to an interface

which is realized via a broadcast media, the value

of this field is the agent's IP address on that

interface.)

NAME		preference

SNMP NAME	ipRouteMetric1

SNMP PATH	ipRouteTable 3

TYPE		INTEGER

MIN		-2147483648

DEFAULT		0

MAX		2147483647

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies a value for determining which route

is preferred when there are multiple routes

to a particular destination. The ICMP router

discovery messages will convey this information

dynamically, otherwise the values are statically

configured by network administration.

NAME		type

SNMP NAME	ipRouteType

SNMP PATH	ipRouteTable 8

TYPE		CHOICE

CHOICES		{ dynamic static }

DEFAULT		static

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether the route can be automatically

deleted if the unit detects failures using the route.

NAME		creation-method

SNMP NAME	ipRouteProto

SNMP PATH	ipRouteTable 9

TYPE		CHOICE

CHOICES		{ snmp icmp local proxy-arp }

DEFAULT		snmp

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the method by which the route was

created.

NAME		birth

SNMP NAME	ipRouteAge

SNMP PATH	ipRouteTable 10

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies when the route was created. The

time value is relative to the unit's boot

in seconds.

NAME		time-to-live

SNMP NAME	ipRouteMetric5

SNMP PATH	ipRouteTable 11

TYPE		INTEGER

MIN		-1

DEFAULT		-1

MAX		2147483647

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies how much time (in seconds) remains

before the route will be automatically deleted.

This information is conveyed in the ICMP router

discovery messages. A value of -1 indicates

that the route should not be automatically

deleted.

NAME		destination-type

SNMP NAME	ncdIpDestType

SNMP PATH	ipRouteTable 12

TYPE		CHOICE

CHOICES		{ network host }

DEFAULT		network

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether the route has a destination

of a host or a network. Most routes are network

routes, the exceptions being proxy arp routes,

and the routes used for SLIP connections.

NAME		route-mask

SNMP NAME	ipRouteMask

SNMP PATH	ipRouteTable 13

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the mask to be logical-ANDed with the

destination address before being compared to the

value in the gateway field.

NAME		interface-index

SNMP NAME	ipRouteIfIndex

SNMP PATH	ipRouteTable 2

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		1

DEFAULT		0

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Index that identifies the interface of this route

NAME		metric-2

SNMP NAME	ipRouteMetric2

SNMP PATH	ipRouteTable 4

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		1

DEFAULT		0

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Alternative routing metric not currently used

NAME		metric-3

SNMP NAME	ipRouteMetric3

SNMP PATH	ipRouteTable 5

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		1

DEFAULT		0

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Alternative routing metric not currently used

NAME		metric-4

SNMP NAME	ipRouteMetric4

SNMP PATH	ipRouteTable 6

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		1

DEFAULT		0

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Alternative routing metric not currently used

END TABLE ROW ENTRIES

NAME		ip-subnet-address

HIDEBOX		Console=>Statistics=>Show Statistics=>

IP=>Subnet Address

SNMP NAME	ncdIpSubnetAddr

SNMP PATH	ncdIp 10

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the current IP subnet address for this unit

NAME		ip-subnet-mask

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

IP=>Subnet Mask

TYPE		INTEGER

MIN		0

DEFAULT		4294967040

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the IP subnet mask that is used to

determine which portion of the unit's IP address

corresponds to the network number and which

portion to the host number.

NAME		ip-use-address-discovery

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

IP=>Use Address Discovery

SNMP NAME	ncdIpDynamicAddressDiscovery

SNMP PATH	ncdIp 4

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that IP addresses supplied via BOOTP

should be used, if available.

NAME		ip-use-proxy-arp

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

IP=>Use Proxy Arp

SNMP NAME	ncdIpUseProxyArp

SNMP PATH	ncdIp 5

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies that proxy arp should be used to

resolve routes to hosts for which there are

no routes in the IP routing table. Note that

using default gateways overrides any use of

proxy arp.

NAME		ip-use-router-discovery

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

IP=>Use Router Discovery

SNMP NAME	ncdIpUseRouterDiscovery

SNMP PATH	ncdIp 6

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies that the unit should modify its

IP routing table based on the receipt of

ICMP router discovery messages as detailed

in RFC 1256.

NAME		ip-use-router-solicit

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

IP=>Use Router Solicit

SNMP NAME	ncdIpUseRouterDiscoverySolicit

SNMP PATH	ncdIp 7

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that the unit should send ICMP

router discovery solicitation messages at

boot time as detailed in RFC 1256.

GROUP java

NAME		java-appletviewer-command

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Java=>AppletViewer Command

SNMP NAME	ncdJavaAppletViewerCmd

SNMP PATH	ncdJava 2

TYPE		STRING

DEFAULT		java ncd.applet.NCDAppletViewer

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the java command line that should be used to

start up the AppletViewer.

NAME		java-cache-class-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Java=>Cache Class File

SNMP NAME	ncdJavaCacheClassFile

SNMP PATH	ncdJava 3

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that the Java classes shipped with NCDWare should

be loaded and cached in memory.  This will improve the

runtime

performance of Java, especially if TFTP is being used, at the

expense of memory consumption.

NAME		java-directory

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Java=>Java Directory

SNMP NAME	ncdJavaDirectory

SNMP PATH	ncdJava 1

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the directory where the Java class files and

miscellaneous properties files are installed.  If not

specified, the unit will use the location of the boot

file as the default.

NAME		java-web-colors

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Java=>Use Web Palette

SNMP NAME	ncdJavaWebColors

SNMP PATH	ncdJava 4

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Java allocates a set of colors that it will use when

displaying

images for example.  If this configuration parameter is true, it will

allocate the set of colors that the Netscape browser normally allocates,

also known as the 'Web Palette'.  If set to false, colors will be

allocated using an algorithm that attempts to produce a broad range

of colors without using as much of the available colormap.

GROUP lat

NAME		lat-circuit-flush-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

LAT=>Circuit Flush Timeout

SNMP NAME	ncdLatCircuitFlushTimeout

SNMP PATH	ncdLat 2

TYPE		INTEGER

MIN		0

DEFAULT		80

MAX		100

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in milliseconds) during which input

characters should be queued before the unit flushes

queued data by sending a LAT packet to the host.

NAME		lat-connect-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

LAT=>Connect Timeout

SNMP NAME	ncdLatConnectTimeout

SNMP PATH	ncdLat 1

TYPE		INTEGER

MIN		1

DEFAULT		20

MAX		600

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) that must elapse

between a LAT connection attempt and a lack of

response for a failure situation to be declared.

NAME		lat-keepalive-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

LAT=>Keepalive Timeout

SNMP NAME	ncdLatKeepaliveTimeout

SNMP PATH	ncdLat 5

TYPE		INTEGER

MIN		0

DEFAULT		20

MAX		255

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) of inactivity on

an established LAT connection before a LAT keepalive

packet is sent.

NAME		lat-retransmission-limit

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

LAT=>Retransmission Limit

SNMP NAME	ncdLatRetransLimit

SNMP PATH	ncdLat 4

TYPE		INTEGER

MIN		4

DEFAULT		8

MAX		120

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the number of times a LAT packet is

to be retransmitted before the connection is

considered to have failed.

NAME		lat-retransmission-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

LAT=>Retransmission Timeout

SNMP NAME	ncdLatRetransTimeout

SNMP PATH	ncdLat 3

TYPE		INTEGER

MIN		100

DEFAULT		2000

MAX		60000

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in milliseconds) between consecutive

retransmissions of LAT data packets on an established

connection.

NAME		lat-service-directory-max-size

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

LAT=>Service Directory Max Size

SNMP NAME	ncdLatMaxDirEntries

SNMP PATH	ncdLat 7

TYPE		INTEGER

MIN		25

DEFAULT		100

MAX		1000

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the maximum number of entries

in the LAT service directory.

NAME		lat-service-groups

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

LAT=>Service Groups

SNMP NAME	ncdLatGroupCodes

SNMP PATH	ncdLat 6

TYPE		STRING

DEFAULT		0

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the LAT service groups from which service

advertisements will be added to the LAT service

directory.

GROUP login

NAME		login-default-hosts

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Login=>Default Hosts

SNMP NAME	ncdLoginDfltHostsTable

SNMP PATH	ncdLogin 1

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the default list of hosts to present

to the user in the login box.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdLoginDHTIndex

SNMP PATH	ncdLoginDfltHostsTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the index into the Default Hosts Table

NAME		network

SNMP NAME	ncdLoginDHTNetwork

SNMP PATH	ncdLoginDfltHostsTable 2

TYPE		CHOICE

CHOICES		{ tcp ncdnet ica wincenter }

DEFAULT		tcp

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the protocol to use.

NAME		host

SNMP NAME	ncdLoginDHTHost

SNMP PATH	ncdLoginDfltHostsTable 3

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the name of the login host.

NAME		description

SNMP NAME	ncdLoginDHTDescription

SNMP PATH	ncdLoginDfltHostsTable 4

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies a description of the login host.

END TABLE ROW ENTRIES

NAME		login-disabled-networks

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Login=>Disabled Networks

SNMP NAME	ncdLoginDisabledNetTable

SNMP PATH	ncdLogin 3

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the list of networks to disable.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdLoginDNTIndex

SNMP PATH	ncdLoginDisabledNetTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the index into the Disabled Networks Table

NAME		network

SNMP NAME	ncdLoginDNTName

SNMP PATH	ncdLoginDisabledNetTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the name of the network

to disable.

END TABLE ROW ENTRIES

NAME		login-dwlogin-name

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Login=>Login Name

SNMP NAME	ncdLoginDwLoginAccount

SNMP PATH	ncdLogin 13

TYPE		STRING

DEFAULT		NDS_LOGIN

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the login account name to use to invoke

the VMS DECWindows login box.

NAME		login-dwlogin-password

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Login=>Login Password

SNMP NAME	ncdLoginDwPassword

SNMP PATH	ncdLogin 14

TYPE		STRING

DEFAULT

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the login account password to use to invoke

the VMS DECWindows login box.

NAME		login-ica-enabled

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Login=>Enable ICA Broadcast

SNMP NAME	ncdLoginICAEnabled

SNMP PATH	ncdLogin 17

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that the ICA master browser should be

queried for server and published applications.

NAME		login-ncddm-enabled

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Login=>NCD Display Manager Enabled

SNMP NAME	ncdLoginNCDdmEnabled

SNMP PATH	ncdLogin 15

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that the NCD Display Manager should be

queried for user-specific information.

NAME		login-ncddm-tcp-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Login=>NCD Display Manager TCP Port Number

SNMP NAME	ncdLoginNCDdmTcpPort

SNMP PATH	ncdLogin 16

TYPE		INTEGER

MIN		0

DEFAULT		677

MAX		65535

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the TCP port on which the NCD Display

Manager is listening for requests.

NAME		login-restrict-host-choices

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Login=>Restrict Host Choices

SNMP NAME	ncdLoginRestrictConnectionChoices

SNMP PATH	ncdLogin 2

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that the user should only be allowed

to choose a login host specified in the default

login hosts list.

NAME		login-window-delay-time

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Login=>Window Delay Time

SNMP NAME	ncdLoginAutoMapDelay

SNMP PATH	ncdLogin 4

TYPE		INTEGER

MIN		0

DEFAULT		3

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) to wait before

displaying the login window when starting from

a command line.

NAME		login-xdm-action-on-disconnect

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Login=>What to do when XDM disconnects

SNMP NAME	ncdXdmcpWhenFirstClientExits

SNMP PATH	ncdXdmcp 8

TYPE		CHOICE

CHOICES		{ logout prompt ignore }

DEFAULT		prompt

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies what to do when the host XDM program

disconnects from the display.

NAME		login-xdm-authentication-key

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>XDM Authentication Key

SNMP NAME	ncdAccessXdmAuthCookie

SNMP PATH	ncdAccess 1

TYPE		STRING

DEFAULT		nil

PERMISSION	WRITE-ONLY

EFFECT		AT_SESSION_STARTUP

NVRAM		YES

DESCRIPTION

Specifies the key that must be used in conjunction

with authorized client connection requests.

NAME		login-xdm-broadcast-wait-time

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Login=>Broadcast Wait Time

SNMP NAME	ncdXdmcpBroadcastWaitTime

SNMP PATH	ncdXdmcp 5

TYPE		INTEGER

MIN		0

DEFAULT		3

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) to wait for responses

to the XDM broadcast query before displaying the results

to the user via the login box. Any responses which

arrive after this time are appended to the end of

the displayed list.

NAME		login-xdm-broadcasts

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Login=>What to do with XDMCP Broadcasts

SNMP NAME	ncdXdmcpBroadcasts

SNMP PATH	ncdXdmcp 6

TYPE		CHOICE

CHOICES		{ use-first prompt ignore }

DEFAULT		prompt

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies what to do with responses to XDM broadcast

and indirect queries.  If set to Ignore, then

broadcasts are not sent.

NAME		login-xdm-display-class

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Login=>XDMDisplayClass

SNMP NAME	ncdXdmcpcpDisplayClass

SNMP PATH	ncdXdmcp 3

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies a specific XDMCP Display Class to use

instead of the default, which is NCD-unittype.

NAME		login-xdm-indirects

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Login=>What to do with XDMCP Indirects

SNMP NAME	ncdXdmcpIndirects

SNMP PATH	ncdXdmcp 7

TYPE		CHOICE

CHOICES		{ use-first prompt }

DEFAULT		prompt

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies what to do with responses to XDM indirect

queries.

NAME		login-xdm-keepalives-begin-time

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Login=>SendKeepalivesTime

SNMP NAME	ncdXdmcpSendKeepAlivesTime

SNMP PATH	ncdXdmcp 1

TYPE		INTEGER

MIN		0

DEFAULT		3

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the length of time (in minutes) after which

the unit is to verify that the XDM session host

is still running by sending an XDMCP keepalive.

NAME		login-xdm-keepalives-wait-time

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Login=>KeepalivesDeadTime

SNMP NAME	ncdXdmcpKeepAlivesDeadTime

SNMP PATH	ncdXdmcp 2

TYPE		INTEGER

MIN		0

DEFAULT		30

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) from the start

of XDM dead session testing that must elapse

without receiving a response from the login

host to the XDM keepalive messages before a

dead session failure situation is declared.

NAME		login-xdm-manufacturer-display-id

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Login=>XDM Manufacturer Display ID

SNMP NAME	ncdXdmcpMfgDisplayId

SNMP PATH	ncdXdmcp 4

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the XDM Manufacturer Display ID

used for this unit.  This parameter is not settable.

GROUP ncdnet

NAME		ncdnet-address

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

NCDnet=>NCDnet Address

SNMP NAME	ncdDecnetAddress

SNMP PATH	ncdDecnet 8

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the current NCDnet address of the unit.

NAME		ncdnet-address-at-next-boot

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

NCDnet=>NCDnet Address At Next Boot

SNMP NAME	ncdDecnetNewAddress

SNMP PATH	ncdDecnet 9

TYPE		STRING

DEFAULT		0.0

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the NCDnet address for the unit

the next time it is booted.

Note that the current NCDnet address cannot

be modified while the unit is running because

this would require changing the unit's Ethernet

address while established connections are running.

This would lead to bizarre and unexpected behaviors

on both the unit and the connected hosts.

NAME		ncdnet-bytes-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet=>Bytes Received

SNMP NAME	ncdDecnetDataBytesRcv

SNMP PATH	ncdDecnet 16

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the number of bytes received over NCDnet

NAME		ncdnet-bytes-sent

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet=>Bytes Sent

SNMP NAME	ncdDecnetDataBytesXmt

SNMP PATH	ncdDecnet 15

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the number of bytes transmitted over NCDnet

NAME		ncdnet-connect-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

NCDnet=>Connect Timeout

SNMP NAME	ncdDecnetConnectTimeout

SNMP PATH	ncdDecnet 1

TYPE		INTEGER

MIN		1

DEFAULT		120

MAX		600

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) that must elapse

between a NCDnet connection attempt and a lack of a response

for a failure situation to be declared.

NAME		ncdnet-count-circuit-down

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet=>Count Circuit Down

SNMP NAME	ncdDecnetCntCircDown

SNMP PATH	ncdDecnet 19

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the number of circuit down events

NAME		ncdnet-data-blocks-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet=>Data Blocks Received

SNMP NAME	ncdDecnetDataBlocksRcv

SNMP PATH	ncdDecnet 14

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the number of data messages received over NCDnet

NAME		ncdnet-data-blocks-sent

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet=>Data Blocks Sent

SNMP NAME	ncdDecnetDataBlocksXmt

SNMP PATH	ncdDecnet 13

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the number of data messages transmitted over NCDnet

NAME		ncdnet-default-router

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

NCDnet=>Default Router

SNMP NAME	ncdDecnetDesignatedRouter

SNMP PATH	ncdDecnet 10

TYPE		STRING

DEFAULT		0.0

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the NCDnet address or symbolic name

of the default router on the NCDnet network.

NAME		ncdnet-hello-packets-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet=>Hello Packets Received

SNMP NAME	ncdDecnetHelloPacketsRcv

SNMP PATH	ncdDecnet 18

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the number of NCDnet hello packets received

NAME		ncdnet-hello-packets-sent

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet=>Hello Packets Sent

SNMP NAME	ncdDecnetHelloPacketsXmt

SNMP PATH	ncdDecnet 17

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the number of NCDnet hello packets transmitted

NAME		ncdnet-hello-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

NCDnet=>Hello Timeout

SNMP NAME	ncdDecnetHelloTimeout

SNMP PATH	ncdDecnet 4

TYPE		INTEGER

MIN		1

DEFAULT		60

MAX		600

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) between successive

transmissions of NCDnet Hello packets. As a part

of normal operation, routers send Hello packets to

end-node systems that specify a suggested Hello

timeout value. NCD units will use this value if it

is present.

NAME		ncdnet-inbound-errors

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet=>Inbound Errors

SNMP NAME	ncdDecnetInErrs

SNMP PATH	ncdDecnet 20

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the number of NCDnet receive errors

NAME		ncdnet-keepalive-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

NCDnet=>Keepalive Timeout

SNMP NAME	ncdDecnetKeepaliveTimeout

SNMP PATH	ncdDecnet 3

TYPE		INTEGER

MIN		1

DEFAULT		60

MAX		600

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) of inactivity

on an established NCDnet connection before an

NCDnet keepalive packet is sent.

NAME		ncdnet-mop-allow-reset

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

NCDnet=>Allow MOP Reset

SNMP NAME	ncdDecnetMopResetEnabled

SNMP PATH	ncdDecnet 12

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that the unit should allow remote reboot via MOP.

NAME		ncdnet-mop-identify-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

NCDnet=>MOP Identify Timeout

SNMP NAME	ncdDecnetMopIdTimeout

SNMP PATH	ncdDecnet 6

TYPE		INTEGER

MIN		1

DEFAULT		300

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) between successive

transmissions of NCDnet MOP identify packets.

NAME		ncdnet-mop-reset-password

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>MOP Reset Password

SNMP NAME	ncdAccessMopResetPasswd

SNMP PATH	ncdAccess 23

TYPE		STRING

DEFAULT		nil

PERMISSION	WRITE-ONLY

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the password that must be contained in the MOP

reset packet. Note that this value must be composed of 16

hexadecimal digits.

NAME		ncdnet-name-bad-name-errors

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet Name Service=>Bad Name Errors

SNMP NAME	ncdDnetNameResolverBadNameErrs

SNMP PATH	ncdDnetNameResolver 7

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of 'Bad Name' errors returned by the

NCDnet name servers for name requests. This indicates

a badly formed name.

NAME		ncdnet-name-cache-hits

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet Name Service=>Name Cache Hits

SNMP NAME	ncdDnetNameResolverCacheHits

SNMP PATH	ncdDnetNameResolver 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of invocations where the answer

was found in the local NCDnet name cache.

NAME		ncdnet-name-cache-max-lifetime

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

NCDnet Name Service=>Name Cache Max Lifetime

SNMP NAME	ncdDnetNameResolverNameCacheMaxTimeout

SNMP PATH	ncdDnetNameResolver 13

TYPE		INTEGER

MIN		0

DEFAULT		1800

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the maximum amount of time (in seconds) that a

NCDnet name service entry should be used before it

is automatically deleted.

NAME		ncdnet-name-cache-max-size

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

NCDnet Name Service=>Name Cache Max Size

SNMP NAME	ncdDnetNameResolverNameCacheMaxSize

SNMP PATH	ncdDnetNameResolver 12

TYPE		INTEGER

MIN		0

DEFAULT		32

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the maximum number of entries in the NCDnet

name cache.

NAME		ncdnet-name-cache-overflows

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet Name Service=>Name Cache Overflows

SNMP NAME	ncdDnetNameResolverCacheOverflows

SNMP PATH	ncdDnetNameResolver 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times a valid entry in the local NCDNet name cache

had to be deleted to make room for a new entry.

NAME		ncdnet-name-local-cache

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

NCDnet Name Service=>Local Name Cache

SNMP NAME	ncdDnetNameResolverNameCacheTable

SNMP PATH	ncdDnetNameResolver 14

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the NCDnet name service cache.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdDnetNameResolverIndex

SNMP PATH	ncdDnetNameResolverNameCacheTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the index to the NCDnet Name Table

NAME		name

SNMP NAME	ncdDnetNameResolverName

SNMP PATH	ncdDnetNameResolverNameCacheTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the symbolic name of the host.

NAME		address

SNMP NAME	ncdDnetNameResolverAddress

SNMP PATH	ncdDnetNameResolverNameCacheTable 3

TYPE		STRING

DEFAULT		0.0

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the NCDnet address of the host.

NAME		lifetime

SNMP NAME	ncdDnetNameResolverExpTime

SNMP PATH	ncdDnetNameResolverNameCacheTable 4

TYPE		INTEGER

MIN		0

DEFAULT		0

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the time (relative to the unit boot time

in seconds) at which the entry will become invalid.

NB: Zero (0) indicates an infinite lifetime.

END TABLE ROW ENTRIES

NAME		ncdnet-name-memory-allocation-failures

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet Name Service=>Memory Allocation Failures

SNMP NAME	ncdDnetNameResolverNoMemErrs

SNMP PATH	ncdDnetNameResolver 11

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times the NCDnet name resolver

was unable to allocated required memory.

NAME		ncdnet-name-no-server-responses

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet Name Service=>No Server Responses

SNMP NAME	ncdDnetNameResolverNoRspErrs

SNMP PATH	ncdDnetNameResolver 9

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times no response was received from a

NCDnet name server.

NAME		ncdnet-name-no-such-name-errors

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet Name Service=>No Such Name Errors

SNMP NAME	ncdDnetNameResolverNoNameErrs

SNMP PATH	ncdDnetNameResolver 6

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of 'No Such Name' errors returned by the

NCDnet name servers for name requests.

NAME		ncdnet-name-numerical-addresses

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet Name Service=>Numerical Addresses

SNMP NAME	ncdDnetNameResolverNumericalAddrs

SNMP PATH	ncdDnetNameResolver 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of invocations where the NCDnet name was

in numerical format, e.g. 1.17.

NAME		ncdnet-name-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet Name Service=>Name Requests

SNMP NAME	ncdDnetNameResolverRequests

SNMP PATH	ncdDnetNameResolver 1

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times the NCDnet name resolver

was invoked.

NAME		ncdnet-name-server-hits

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet Name Service=>Name Server Hits

SNMP NAME	ncdDnetNameResolverServerHits

SNMP PATH	ncdDnetNameResolver 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of invocations where the answer

was found by consulting the NCDnet name servers.

NAME		ncdnet-name-server-response-errors

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet Name Service=>Name Server Response Errors

SNMP NAME	ncdDnetNameResolverRspErrs

SNMP PATH	ncdDnetNameResolver 8

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of badly formed responses returned by the

NCDnet name servers for name requests.

NAME		ncdnet-name-server-transaction-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

NCDnet Name Service=>Name Server Transaction Timeout

SNMP NAME	ncdDnetNameResolverTransactTimeout

SNMP PATH	ncdDnetNameResolver 15

TYPE		INTEGER

MIN		1

DEFAULT		10

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) to attempt

an NCDnet name service request before a failure

situation is declared.

NAME		ncdnet-name-servers

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

NCDnet Name Service=>Name Servers

SNMP NAME	ncdDnetNameResolverServersTable

SNMP PATH	ncdDnetNameResolver 16

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the NCDnet name cache.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdDnetNameResolverServerIndex

SNMP PATH	ncdDnetNameResolverServersTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the index into the NCDnet Name Servers Table

NAME		server

SNMP NAME	ncdDnetNameResolverServer

SNMP PATH	ncdDnetNameResolverServersTable 2

TYPE		STRING

DEFAULT		0.0

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the NCDnet address of the name server.

END TABLE ROW ENTRIES

NAME		ncdnet-name-system-errors

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet Name Service=>System Errors

SNMP NAME	ncdDnetNameResolverSysErrs

SNMP PATH	ncdDnetNameResolver 10

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times the NCDnet name resolver

encountered an general system error.

NAME		ncdnet-node-address

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet=>Node Address

SNMP NAME	ncdDecnetCurrentAddress

SNMP PATH	ncdDecnet 23

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the current NCDnet address of the unit.

NAME		ncdnet-outbound-errors

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet=>Outbound Errors

SNMP NAME	ncdDecnetOutErrs

SNMP PATH	ncdDecnet 21

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the number of NCDnet transmit errors

NAME		ncdnet-retransmission-limit

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

NCDnet=>Retransmission Limit

SNMP NAME	ncdDecnetRetransLimit

SNMP PATH	ncdDecnet 2

TYPE		INTEGER

MIN		1

DEFAULT		10

MAX		32767

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the number of times NCDnet should

retransmit data on an established connection

before a failure situation is declared.

NAME		ncdnet-router-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

NCDnet=>Router Timeout

SNMP NAME	ncdDecnetRouterTimeout

SNMP PATH	ncdDecnet 5

TYPE		INTEGER

MIN		1

DEFAULT		60

MAX		32767

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) after which

a Router Hello message is received that the

sending router is used as the default router.

NAME		ncdnet-seconds-since-zeroed

HIDEBOX		Console=>Statistics=>Show Statistics=>

NCDnet=>Seconds Since Zeroed

SNMP NAME	ncdDecnetLastZeroTime

SNMP PATH	ncdDecnet 22

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the number of seconds since the NCDnet stats

were last reset to zero

NAME		ncdnet-user-account-name

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

NCDnet=>User Account Name

SNMP NAME	ncdDecnetDnetUserName

SNMP PATH	ncdDecnet 11

TYPE		STRING

DEFAULT		NCD_NDS

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the account name used in NCDnet

connection requests. This is used by the peer

to map to a proxy or default account.

GROUP net

NAME		net-arp-table-entry-buffers

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Buffers=>ARP Table Entry Buffers

SNMP NAME	ncdMbufATTable

SNMP PATH	ncdMbuf 11

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of network buffers allocated

to storing ARP table entries.

NAME		net-available-buffers

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Buffers=>Available Buffers

SNMP NAME	ncdMbufAvail

SNMP PATH	ncdMbuf 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of network buffers which are unallocated.

NAME		net-available-cluster-buffers

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Buffers=>Available Cluster Buffers

SNMP NAME	ncdMbufClusterAvail

SNMP PATH	ncdMbuf 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of large contiguous network buffers

which are unallocated.

NAME		net-buffer-not-available-errors

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Buffers=>Buffer Not Available Errors

SNMP NAME	ncdMbufDrops

SNMP PATH	ncdMbuf 6

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times the networking code was unable to

allocate a network buffer, and initiated error recovery

procedures due to this failure.

NAME		net-buffer-waits

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Buffers=>Buffer Waits

SNMP NAME	ncdMbufWaits

SNMP PATH	ncdMbuf 7

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times the networking code had to block waiting

for network buffers to become available.

NAME		net-connection-control-structure-buffers

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Buffers=>Connection Control Structure Buffers

SNMP NAME	ncdMbufConnCtrlStructs

SNMP PATH	ncdMbuf 10

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of network buffers allocated

to storing connection control structures.

NAME		net-connection-name-buffers

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Buffers=>Connection Name Buffers

SNMP NAME	ncdMbufConnNames

SNMP PATH	ncdMbuf 12

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of network buffers allocated

to storing connection names.

NAME		net-connection-option-buffers

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Buffers=>Connection Option Buffers

SNMP NAME	ncdMbufConnOptions

SNMP PATH	ncdMbuf 13

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of network buffers allocated

to storing connection options.

NAME		net-domain-channel

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Network Interfaces=>Domain Channel

SNMP NAME	ncdWirelessDomain

SNMP PATH	ncdWireless 12

TYPE		INTEGER

MIN		0

DEFAULT		0

MAX		15

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

All stations require the same 'Domain' number.  Radios on

different 'Domains' cannot communicate with each other.

NAME		net-ethernet-snap-encapsulation

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Network Interfaces=>Ethernet SNAP Encapsulation

SNMP NAME	ncdEtherUseSNAPEncap

SNMP PATH	ncdEther 11

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

For most networks, this field should be FALSE which

will cause Ethernet packets to be sent in the

common, original encapsulation for Ethernet.

However for a few networks Ethernet packets are

sent using SNAP (IEEE 802.2 LLC) encapsulation.

Setting this parameter to TRUE causes the NCD to send

Ethernet packets in this form.

NOTE: This parameter effects the unit's fundamental

ability to communicate with other stations on the

network and should only be enabled by an administrator

knowledgable about the technical details of the

network.

NAME		net-interface-address-buffers

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Buffers=>Interface Address Buffers

SNMP NAME	ncdMbufInterfaceAddr

SNMP PATH	ncdMbuf 15

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of network buffers allocated to storing network

interface address information.

NAME		net-interfaces-table

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Interfaces=>Interfaces Table

SNMP NAME	ifTable

SNMP PATH	if 2

TYPE		TABLE

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The table of network interfaces.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ifIndex

SNMP PATH	ifTable 1

TYPE		INTEGER

MIN		1

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

A unique value for each interface.  Its value

ranges between 1 and the value of Number of

Interfaces.  The value for each interface remains

constant.

NAME		description

SNMP NAME	ifDescr

SNMP PATH	ifTable 2

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

A textual string containing information about the

interface.

NAME		type

SNMP NAME	ifType

SNMP PATH	ifTable 3

TYPE		CHOICE

CHOICES		{ ethernet slip ppp token-ring software-loopback }

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The type of interface, distinguished according to

the physical/link protocol(s) immediately `below'

the network layer in the protocol stack.

NAME		maximum-transmission-unit

SNMP NAME	ifMtu

SNMP PATH	ifTable 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The size of the largest datagram which can be

sent/received on the interface, specified in

octets.  For interfaces that are used for

transmitting network datagrams, this is the size

of the largest network datagram that can be sent

on the interface.

NAME		speed

SNMP NAME	ifSpeed

SNMP PATH	ifTable 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

An estimate of the interface's current bandwidth

in bits per second.  For interfaces which do not

vary in bandwidth or for those where no accurate

estimation can be made, this object should contain

the nominal bandwidth.

NAME		physical-address

SNMP NAME	ifPhysAddress

SNMP PATH	ifTable 6

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The interface's address at the protocol layer

immediately `below' the network layer in the

protocol stack.  For interfaces which do not have

such an address (e.g., a serial line), this object

should contain an octet string of zero length.

NAME		administrative-status

SNMP NAME	ifAdminStatus

SNMP PATH	ifTable 7

TYPE		CHOICE

CHOICES		{ up down testing }

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The desired state of the interface.  The

testing state indicates that no operational

packets can be passed.

NAME		operational-status

SNMP NAME	ifOperStatus

SNMP PATH	ifTable 8

TYPE		CHOICE

CHOICES		{ up down testing }

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The current operational state of the interface.

The testing state indicates that no operational

packets can be passed.

NAME		last-administrative-change

SNMP NAME	ifLastChange

SNMP PATH	ifTable 9

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The time (in seconds relative to unit boot)

at which the interface entered its current

operational state.  If the current state was

entered prior to the last re-initialization

of the local network management subsystem,

then this object contains a zero value.

NAME		bytes-received

SNMP NAME	ifInOctets

SNMP PATH	ifTable 10

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of octets received on the

interface, including framing characters.

NAME		unicast-packets-received

SNMP NAME	ifInUcastPkts

SNMP PATH	ifTable 11

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of subnetwork-unicast packets

delivered to a higher-layer protocol.

NAME		multicast-packets-received

SNMP NAME	ifInNUcastPkts

SNMP PATH	ifTable 12

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of non-unicast (i.e., subnetwork-

broadcast or subnetwork-multicast) packets

delivered to a higher-layer protocol.

NAME		received-packets-discarded

SNMP NAME	ifInDiscards

SNMP PATH	ifTable 13

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of inbound packets which were chosen

to be discarded even though no errors had been

detected to prevent their being deliverable to a

higher-layer protocol.  One possible reason for

discarding such a packet could be to free up

buffer space.

NAME		packets-received-with-errors

SNMP NAME	ifInErrors

SNMP PATH	ifTable 14

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of inbound packets that contained

errors preventing them from being deliverable to a

higher-layer protocol.

NAME		packets-received-with-unknown-protocols

SNMP NAME	ifInUnknownProtos

SNMP PATH	ifTable 15

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of packets received via the interface

which were discarded because of an unknown or

unsupported protocol.

NAME		bytes-transmitted

SNMP NAME	ifOutOctets

SNMP PATH	ifTable 16

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of octets transmitted out of the

interface, including framing characters.

NAME		unicast-packets-transmitted

SNMP NAME	ifOutUcastPkts

SNMP PATH	ifTable 17

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of packets that higher-level

protocols requested be transmitted to a

subnetwork-unicast address, including those that

were discarded or not sent.

NAME		multicast-packets-transmitted

SNMP NAME	ifOutNUcastPkts

SNMP PATH	ifTable 18

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of packets that higher-level

protocols requested be transmitted to a non-

unicast (i.e., a subnetwork-broadcast or

subnetwork-multicast) address, including those

that were discarded or not sent.

NAME		packets-discarded-on-transmission

SNMP NAME	ifOutDiscards

SNMP PATH	ifTable 19

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of outbound packets which were chosen

to be discarded even though no errors had been

detected to prevent their being transmitted.  One

possible reason for discarding such a packet could

be to free up buffer space.

NAME		errors-on-packet-transmission

SNMP NAME	ifOutErrors

SNMP PATH	ifTable 20

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of outbound packets that could not be

transmitted because of errors.

NAME		packets-waiting-to-be-transmitted

SNMP NAME	ifOutQLen

SNMP PATH	ifTable 21

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The length of the output packet queue (in packets).

END TABLE ROW ENTRIES

NAME		net-minimum-data-buffers

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Network Interfaces=>Minimum Network Data Buffers

SNMP NAME	ncdMbufMinimumDataBuffers

SNMP PATH	ncdMbuf 17

TYPE		INTEGER

MIN		0

DEFAULT		0

MAX		16384

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Minimum number of data network buffers that should be pre-allocated.

Normally network buffers are allocated as needed.

NAME		net-number-of-interfaces

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Interfaces=>Number of Interfaces

SNMP NAME	ifNumber

SNMP PATH	if 1

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of network interfaces (regardless of

their current state) present on this system.

NAME		net-packet-data-buffers

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Buffers=>Packet Data Buffers

SNMP NAME	ncdMbufPacketDatas

SNMP PATH	ncdMbuf 8

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of network buffers allocated

to storing user data within the protocol stacks.

NAME		net-packet-fragment-reassembly-buffers

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Buffers=>Packet Fragment Reassembly Buffers

SNMP NAME	ncdMbufFrags

SNMP PATH	ncdMbuf 14

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of network buffers allocated

to storing IP packet fragments for reassembly.

NAME		net-packet-header-buffers

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Buffers=>Packet Header Buffers

SNMP NAME	ncdMbufPacketHeaders

SNMP PATH	ncdMbuf 9

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of network buffers allocated

to storing protocol header information.

NAME		net-protocol-control-structure-buffers

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Buffers=>Protocol Control Structure Buffers

SNMP NAME	ncdMbufProtoCtrlStructs

SNMP PATH	ncdMbuf 16

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of network buffers allocated

to storing protocol control structures.

NAME		net-reserved-data-buffers

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Buffers=>Reserved Data Buffers

SNMP NAME	ncdMbufReserved

SNMP PATH	ncdMbuf 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of network buffers that are reserved, but as yet

unallocated, for storing user data within the protocol stacks.

NAME		net-roaming-condition

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Network Interfaces=>Roaming Condition

SNMP NAME	ncdWirelessRoamConfig

SNMP PATH	ncdEther 14

TYPE		CHOICE

CHOICES		{ slow normal fast }

DEFAULT		slow

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Determine how quickly stations will roam from one to another.

The default setting is 'Normal' for most conditions.

Set to 'Fast' if there are many master units in the area.

Set to 'Normal' if the area is not overcrowded with master units.

Set to 'Slow' if there are few master units in the area.

NAME		net-token-ring-speed

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Network Interfaces=>Token Ring Speed

SNMP NAME	ncdTokenRingSpeed

SNMP PATH	ncdTokenRing 1

TYPE		CHOICE

CHOICES		{ 4-mbit 16-mbit }

DEFAULT		4-mbit

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

The speed at which to run the token ring interface (if present).

Note that at boot the unit will attempt to automatically

configure itself to run at the correct ring speed. It will

first attempt to enter the ring at the speed described by this

parameter. If that fails the unit will try the other speed, and

adjust this parameter to the new speed so that subsequent boots

occur more rapidly.

NAME		net-total-buffers

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Buffers=>Total Buffers

SNMP NAME	ncdMbufTotal

SNMP PATH	ncdMbuf 1

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of network buffers that currently

exist in the unit.

NAME		net-total-cluster-buffers

HIDEBOX		Console=>Statistics=>Show Statistics=>

Network Buffers=>Total Cluster Buffers

SNMP NAME	ncdMbufClusterTotal

SNMP PATH	ncdMbuf 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of large contiguous network buffers

that currently exist in the unit.

GROUP nfs

NAME		nfs-directory-contents-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Directory Contents Requests

SNMP NAME	ncdNfsClientReadDirs

SNMP PATH	ncdNfs 17

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Read Directory operations requested by

the NFS client.

NAME		nfs-directory-create-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Directory Create Requests

SNMP NAME	ncdNfsClientMakeDirs

SNMP PATH	ncdNfs 15

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Create Directory operations requested by

the NFS client.

NAME		nfs-directory-remove-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Directory Remove Requests

SNMP NAME	ncdNfsClientRemoveDirs

SNMP PATH	ncdNfs 16

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Remove Directory operations requested by

the NFS client.

NAME		nfs-file-create-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>File Create Requests

SNMP NAME	ncdNfsClientCreates

SNMP PATH	ncdNfs 10

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS File Create operations requested by

the NFS client.

NAME		nfs-file-remove-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>File Remove Requests

SNMP NAME	ncdNfsClientRemoves

SNMP PATH	ncdNfs 11

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS File Remove operations requested by

the NFS client.

NAME		nfs-file-rename-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>File Rename Requests

SNMP NAME	ncdNfsClientRenames

SNMP PATH	ncdNfs 12

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS File Rename operations requested by

the NFS client.

NAME		nfs-get-attributes-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Get Attributes Requests

SNMP NAME	ncdNfsClientGetAttributes

SNMP PATH	ncdNfs 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Get Attribute operations requested by

the NFS client.

NAME		nfs-get-file-system-stats-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Get File System Stats Requests

SNMP NAME	ncdNfsClientFileSysStats

SNMP PATH	ncdNfs 18

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Stat File System operations requested by

the NFS client.

NAME		nfs-get-root-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Get Root Requests

SNMP NAME	ncdNfsClientGetRoots

SNMP PATH	ncdNfs 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Get File System Root operations

requested by the NFS client.

NAME		nfs-link-create-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Link Create Requests

SNMP NAME	ncdNfsClientCreateLinks

SNMP PATH	ncdNfs 13

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Create Link operations requested by

the NFS client.

NAME		nfs-null-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Null Requests

SNMP NAME	ncdNfsClientNulls

SNMP PATH	ncdNfs 1

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Null operations requested by the

NFS client.

NAME		nfs-path-lookup-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Path Lookup Requests

SNMP NAME	ncdNfsClientLookups

SNMP PATH	ncdNfs 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS File Name Component Lookup

operations requested by the NFS client.

NAME		nfs-read-data-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Read Data Requests

SNMP NAME	ncdNfsClientReads

SNMP PATH	ncdNfs 7

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Read operations requested by the NFS client.

NAME		nfs-read-symlink-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Read Symlink Requests

SNMP NAME	ncdNfsClientReadLinks

SNMP PATH	ncdNfs 6

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Read Link operations requested

by the NFS client.

NAME		nfs-received-directory-contents-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Directory Contents Requests

SNMP NAME	ncdNfsServerReadDirs

SNMP PATH	ncdNfs 35

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Read Directory operations received by the terminal.

NAME		nfs-received-directory-create-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Directory Create Requests

SNMP NAME	ncdNfsServerMakeDirs

SNMP PATH	ncdNfs 33

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Create Directory operations received by the terminal.

NAME		nfs-received-directory-remove-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Directory Remove Requests

SNMP NAME	ncdNfsServerRemoveDirs

SNMP PATH	ncdNfs 34

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Remove Directory operations received by the terminal.

NAME		nfs-received-file-create-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>File Create Requests

SNMP NAME	ncdNfsServerCreates

SNMP PATH	ncdNfs 28

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS File Create operations received by the terminal.

NAME		nfs-received-file-remove-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>File Remove Requests

SNMP NAME	ncdNfsServerRemoves

SNMP PATH	ncdNfs 29

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS File Remove operations received by the terminal.

NAME		nfs-received-file-rename-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>File Rename Requests

SNMP NAME	ncdNfsServerRenames

SNMP PATH	ncdNfs 30

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS File Rename operations received by the terminal.

NAME		nfs-received-get-attributes-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Get Attributes Requests

SNMP NAME	ncdNfsServerGetAttributes

SNMP PATH	ncdNfs 20

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Get Attribute operations received by the terminal.

NAME		nfs-received-get-file-system-stats-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Get File System Stats Requests

SNMP NAME	ncdNfsServerFileSysStats

SNMP PATH	ncdNfs 36

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Stat File System operations received by the terminal.

NAME		nfs-received-get-root-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Get Root Requests

SNMP NAME	ncdNfsServerGetRoots

SNMP PATH	ncdNfs 22

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Get File System Root operations

received by the terminal.

NAME		nfs-received-link-create-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Link Create Requests

SNMP NAME	ncdNfsServerCreateLinks

SNMP PATH	ncdNfs 31

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Create Link operations received by the terminal.

NAME		nfs-received-null-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Null Requests

SNMP NAME	ncdNfsServerNulls

SNMP PATH	ncdNfs 19

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Null operations received by the terminal.

NAME		nfs-received-path-lookup-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Path Lookup Requests

SNMP NAME	ncdNfsServerLookups

SNMP PATH	ncdNfs 23

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS File Name Component Lookup

operations received by the terminal.

NAME		nfs-received-read-data-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Read Data Requests

SNMP NAME	ncdNfsServerReads

SNMP PATH	ncdNfs 25

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Read operations received by the terminal. the NFS client.

NAME		nfs-received-read-symlink-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Read Symlink Requests

SNMP NAME	ncdNfsServerReadLinks

SNMP PATH	ncdNfs 24

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Read Link operations received by the terminal.

NAME		nfs-received-set-attributes-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Set Attributes Requests

SNMP NAME	ncdNfsServerSetAttributes

SNMP PATH	ncdNfs 21

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Set Attribute operations received by the terminal.

NAME		nfs-received-symlink-create-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Symlink Create Requests

SNMP NAME	ncdNfsServerCreateSymlinks

SNMP PATH	ncdNfs 32

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Create Symbolic Link operations

received by the terminal.

NAME		nfs-received-write-data-cache-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Write Data Cache Requests

SNMP NAME	ncdNfsServerWriteCaches

SNMP PATH	ncdNfs 26

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Write Cache operations received by the terminal.

NAME		nfs-received-write-data-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Write Data Requests

SNMP NAME	ncdNfsServerWrites

SNMP PATH	ncdNfs 27

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Write operations received by the terminal.

NAME		nfs-set-attributes-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Set Attributes Requests

SNMP NAME	ncdNfsClientSetAttributes

SNMP PATH	ncdNfs 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Set Attribute operations requested by

the NFS client.

NAME		nfs-symlink-create-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Symlink Create Requests

SNMP NAME	ncdNfsClientCreateSymlinks

SNMP PATH	ncdNfs 14

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Create Symbolic Link operations

requested by the NFS client.

NAME		nfs-write-data-cache-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Write Data Cache Requests

SNMP NAME	ncdNfsClientWriteCaches

SNMP PATH	ncdNfs 8

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Write Cache operations requested by

the NFS client.

NAME		nfs-write-data-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

NFS=>Write Data Requests

SNMP NAME	ncdNfsClientWrites

SNMP PATH	ncdNfs 9

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of NFS Write operations requested by the NFS

client.

GROUP parallel

NAME		parallel-daemons-lat-service-passwords

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Parallel Daemon LAT Service Passwords

SNMP NAME	ncdAccessParalleldLatPasswdTable

SNMP PATH	ncdAccess 36

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the LAT service passwords required to

gain access to the parallel port daemon using LAT.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdAccessParalleldLPTIndex

SNMP PATH	ncdAccessParalleldLatPasswdTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Parallel Daemons LAT Passwds Table

NAME		port-number

SNMP NAME	ncdAccessParalleldLPTPortNumber

SNMP PATH	ncdAccessParalleldLatPasswdTable 2

TYPE		INTEGER

MIN		1

MAX		2

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The name of the parallel port.

NAME		password

SNMP NAME	ncdAccessParalleldLPTPassword

SNMP PATH	ncdAccessParalleldLatPasswdTable 3

TYPE		STRING

DEFAULT		nil

PERMISSION	WRITE-ONLY

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the LAT service password.

END TABLE ROW ENTRIES

NAME		parallel-daemons-statistics-table

HIDEBOX		Console=>Statistics=>Show Statistics=>

Parallel Daemons=>Parallel Daemon Statistics Table

SNMP NAME	ncdParallelDStatsTbl

SNMP PATH	ncdParallel 2

TYPE		TABLE

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the table of parallel daemon statistics.

BEGIN TABLE ROW ENTRIES

NAME		port-number

SNMP NAME	ncdParallelDStatsTblPortNumber

SNMP PATH	ncdParallelDStatsTbl 1

TYPE		INTEGER

MIN		1

MAX		2

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The name of the parallel port.

NAME		connections-accepted

SNMP NAME	ncdParallelDStatsTblConnAccepts

SNMP PATH	ncdParallelDStatsTbl 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of connections made to the parallel daemon.

NAME		connections-with-access-control-errors

SNMP NAME	ncdParallelDStatsTblConnBadAccesses

SNMP PATH	ncdParallelDStatsTbl 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of connections that were closed by the

parallel daemon due to the remote address not being

present in the parallel daemon's access control list.

NAME		connections-with-io-errors

SNMP NAME	ncdParallelDStatsTblConnIOErrors

SNMP PATH	ncdParallelDStatsTbl 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of connections that were closed by the

parallel daemon due to the a local I/O error. This

includes the user aborting the connection via local

mechanisms.

NAME		connections-with-normal-completion

SNMP NAME	ncdParallelDStatsTblConnEofCloses

SNMP PATH	ncdParallelDStatsTbl 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of connections that were closed by the

remote host. This is the normal mode of ending a

connection to the parallel daemon.

END TABLE ROW ENTRIES

NAME		parallel-daemons-table

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Parallel=>Parallel Daemons Table

SNMP NAME	ncdParallelDTable

SNMP PATH	ncdParallel 1

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the table of parallel daemon parameters.

BEGIN TABLE ROW ENTRIES

NAME		parallel-daemon-table-index

SNMP NAME	ncdParallelDTableIndex

SNMP PATH	ncdParallelDTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The index into the parallel daemons table

NAME		port-number

SNMP NAME	ncdParallelDTablePortNumber

SNMP PATH	ncdParallelDTable 2

TYPE		INTEGER

MIN		1

MAX		2

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The name of the parallel port.

NAME		use-parallel-protocol

SNMP NAME	ncdParallelDTableUseParaProt

SNMP PATH	ncdParallelDTable 3

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that the parallel daemon control protocol

should be used.  Note that this could cause some data

loss if enabled with old host software.

NAME		tcp-port

SNMP NAME	ncdParallelDTableTCPPort

SNMP PATH	ncdParallelDTable 4

TYPE		INTEGER

MIN		1

DEFAULT		5964

MAX		65535

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the TCP port on which the terminal

listens for raw TCP connections to the parallel

daemon.

NAME		ncdnet-object-name

SNMP NAME	ncdParallelDTableNCDnetObjName

SNMP PATH	ncdParallelDTable 5

TYPE		STRING

DEFAULT		PARALLELD

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the NCDnet object on which the terminal

listens for NCDnet connections to the parallel

daemon.

NAME		enable-lat-service

SNMP NAME	ncdParallelDTableEnableLAT

SNMP PATH	ncdParallelDTable 6

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies that the unit is to accept incoming

LAT solicit messages for a service which

allows access to the parallel daemon. This is used

to support VMS print services to the unit.

NAME		lat-service-name

SNMP NAME	ncdParallelDTableLATServName

SNMP PATH	ncdParallelDTable 7

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the name of the parallel daemon LAT service.

NAME		lat-service-rating

SNMP NAME	ncdParallelDTableLATServRating

SNMP PATH	ncdParallelDTable 8

TYPE		INTEGER

MIN		0

DEFAULT		50

MAX		255

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the rating of the parallel daemon LAT service.

END TABLE ROW ENTRIES

GROUP pex

GROUP ppp

NAME		ppp-interfaces-table

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

PPP and SLIP=>PPP Interfaces Table

SNMP NAME	ncdPppIfTable

SNMP PATH	ncdPpp 2

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies PPP configuration parameters.

BEGIN TABLE ROW ENTRIES

NAME		port-number

SNMP NAME	ncdPppIfTablePortNumber

SNMP PATH	ncdPppIfTable 1

TYPE		INTEGER

MIN		1

MAX		3

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The serial port number.

NAME		ctrl-char-map

SNMP NAME	ncdPppIfTableCtlCharMap

SNMP PATH	ncdPppIfTable 2

TYPE		INTEGER

MIN		0

DEFAULT		0

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Bit map specifying which control characters

should not be sent in plain text form.  The most

signficant bit corresponds to control character

0x1f, the least signficant bit corresponds to

character 0x00.  Not used for SLIP connections.

NAME		mtu

SNMP NAME	ncdPppIfTableMTU

SNMP PATH	ncdPppIfTable 3

TYPE		INTEGER

MIN		128

DEFAULT		1000

MAX		1500

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the maximum number of bytes that should

be sent/received in the information field of a PPP

frame.  Not used for SLIP connections.

NAME		compress-protocol-field

SNMP NAME	ncdPppIfTableProtocolComp

SNMP PATH	ncdPppIfTable 4

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that PPP frames may be sent using

compressed protocol fields.  Not used for

SLIP connections.

NAME		compress-addr-ctrl-fields

SNMP NAME	ncdPppIfTableACComp

SNMP PATH	ncdPppIfTable 5

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that PPP frames may be sent with

the address and control fields compressed.

Not used for SLIP connections.

NAME		passive-mode

SNMP NAME	ncdPppIfTablePassive

SNMP PATH	ncdPppIfTable 6

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Normally the NCD unit will attempt to initiate

the PPP negotiation with the remote host.  If this

option is enabled, the NCD unit will wait for

the remote host to initiate the negotiation.

Not used for SLIP connections.

NAME		use-old-format

SNMP NAME	ncdPppIfTableOldOptions

SNMP PATH	ncdPppIfTable 7

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

The format for exchanging some configuration options

has changed in the latest PPP RFC's (1331/1332).

Although the NCD unit should be able to negotiate

with older PPP implementations, it may sometimes be

necessary to enable this feature to allow correct

option negotiation.  Not used for SLIP connections.

END TABLE ROW ENTRIES

NAME		ppp-slip-interfaces-table

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

PPP and SLIP=>PPP and SLIP Interfaces Table

SNMP NAME	ncdPppSlipIfTable

SNMP PATH	ncdPpp 1

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies PPP and SLIP configuration parameters.

BEGIN TABLE ROW ENTRIES

NAME		port-number

SNMP NAME	ncdPppSlipIfTablePortNumber

SNMP PATH	ncdPppSlipIfTable 1

TYPE		INTEGER

MIN		1

MAX		3

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The serial port number.

NAME		ip-address

SNMP NAME	ncdPppSlipIfTableIpAddress

SNMP PATH	ncdPppSlipIfTable 2

TYPE		STRING

DEFAULT		0.0.0.0

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the IP address to be used for the PPP/SLIP

connection.  For PPP, if the address is 0.0.0.0,

then it is expected that the IP address will be

supplied by the remote host.

NAME		peer-ip-address

SNMP NAME	ncdPppSlipIfTablePeerIpAddress

SNMP PATH	ncdPppSlipIfTable 3

TYPE		STRING

DEFAULT		0.0.0.0

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the IP address of the system at the other

end of the PPP/SLIP link.  For PPP, if the address is

0.0.0.0, then the NCD unit will attempt to obtain

the address from the remote host.

NAME		dflt-gateway

SNMP NAME	ncdPppSlipIfTableDfltGateway

SNMP PATH	ncdPppSlipIfTable 4

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies if the remote host should be used as a

default gateway.

NAME		compress-headers

SNMP NAME	ncdPppSlipIfTableVJCompression

SNMP PATH	ncdPppSlipIfTable 5

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that TCP/IP packets may be sent using

Van Jacobsen compressed TCP/IP headers.

END TABLE ROW ENTRIES

GROUP pref

NAME		pref-audio-maximum-gain

HIDEBOX		Console=>Setup=>Change User Preferences=>

Audio=>Maximum Gain

SNMP NAME	ncdPrefMaximumGain

SNMP PATH	ncdPref 41

TYPE		INTEGER

MIN		0

DEFAULT		100

MAX		100

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Maximum volume of an audio client.

NAME		pref-bell-duration

HIDEBOX		Console=>Setup=>Change User Preferences=>

Bell=>Bell Duration

SNMP NAME	ncdPrefBellDuration

SNMP PATH	ncdPref 10

TYPE		INTEGER

MIN		0

DEFAULT		100

MAX		1800

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies bell duration in milliseconds.

NAME		pref-bell-enable

HIDEBOX		Console=>Setup=>Change User Preferences=>

Bell=>Enable Bell

SNMP NAME	ncdPrefBellEnabled

SNMP PATH	ncdPref 7

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Toggles bell on and off.

NAME		pref-bell-pitch

HIDEBOX		Console=>Setup=>Change User Preferences=>

Bell=>Bell Pitch

SNMP NAME	ncdPrefBellPitch

SNMP PATH	ncdPref 9

TYPE		INTEGER

MIN		0

DEFAULT		800

MAX		3000

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies bell pitch in Hertz  (Hz).

NAME		pref-bell-volume

HIDEBOX		Console=>Setup=>Change User Preferences=>

Bell=>Bell Volume

SNMP NAME	ncdPrefBellVolume

SNMP PATH	ncdPref 8

TYPE		INTEGER

MIN		0

DEFAULT		50

MAX		100

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the bell's base volume in percentage of max volume.

NAME		pref-compatibility-decwindows-images

HIDEBOX		Console=>Setup=>Change User Preferences=>

Compatibility=>Be Compatible With Old DECwindows Images

SNMP NAME	ncdPrefOldDwImageCompatibility

SNMP PATH	ncdPref 34

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether to byte swap incoming X images which

some old DECwindows applications fail to do.

NAME		pref-compatibility-decwindows-keyboards

HIDEBOX		Console=>Setup=>Change User Preferences=>

Compatibility=>Be Compatible With Old DECwindows Keyboards

SNMP NAME	ncdPrefOldDwKbdCompatibility

SNMP PATH	ncdPref 33

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether the right modifiers should pretend to be

left modifiers like old DEC keyboards.

NAME		pref-compatibility-decwindows-vendor-string

HIDEBOX		Console=>Setup=>Change User Preferences=>

Compatibility=>Be Compatible With Old DECwindows Vendor String

SNMP NAME	ncdPrefOldDwCompatibility

SNMP PATH	ncdPref 32

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether to not to prefix the display vendor

string with DECWINDOWS for older DECwindows applications.

NAME		pref-compatibility-permit-old-x-bugs

HIDEBOX		Console=>Setup=>Change User Preferences=>

Compatibility=>Permit Old X Bugs

SNMP NAME	ncdPrefPermitOldBugs

SNMP PATH	ncdPref 31

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether to bypass error checks on requests which old

X clients don't expect to fail.

NAME		pref-console-auto-lock-idle-time

HIDEBOX		Console=>Setup=>Change User Preferences=>

Console and Utilities=>Auto Lock Idle Time

SNMP NAME	ncdPrefConsoleAutoLockTime

SNMP PATH	ncdPref 37

TYPE		INTEGER

MIN		0

DEFAULT		0

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

How long the terminal should be idle  (in minutes)

before automatically locking the screen.

NAME		pref-console-auto-logout-cancel-delay

HIDEBOX		Console=>Setup=>Change User Preferences=>

Console and Utilities=>Auto Logout Cancel Delay

SNMP NAME	ncdPrefConsoleAutoLogoutCancelDelay

SNMP PATH	ncdPref 36

TYPE		INTEGER

MIN		0

DEFAULT		5

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

How long the terminal should wait  (in seconds)

for the user to cancel an automatic logout.

NAME		pref-console-auto-logout-idle-time

HIDEBOX		Console=>Setup=>Change User Preferences=>

Console and Utilities=>Auto Logout Idle Time

SNMP NAME	ncdPrefConsoleAutoLogoutIdleTime

SNMP PATH	ncdPref 35

TYPE		INTEGER

MIN		0

DEFAULT		0

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

How long the terminal should be idle  (in minutes)

before automatically logging out the user.

NAME		pref-console-blank-screen-delay

HIDEBOX		Console=>Setup=>Change User Preferences=>

Console and Utilities=>Console Blank Screen Delay

SNMP NAME	ncdPrefConsoleBlankScreenDelay

SNMP PATH	ncdPref 39

TYPE		INTEGER

MIN		0

DEFAULT		500

MAX		5000

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Time (in milliseconds) to wait after selecting Blank Screen.

NAME		pref-console-key-sequence

HIDEBOX		Console=>Setup=>Change User Preferences=>

Console and Utilities=>Console Key Sequence

SNMP NAME	ncdPrefConsoleKeyString

SNMP PATH	ncdPref 44

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

The key seqeunce used to bring up the Console.

NAME		pref-console-lock-screen-default-password

HIDEBOX		Console=>Setup=>Change User Preferences=>

Console and Utilities=>Lock Screen Default Password

SNMP NAME	ncdPrefConsoleLockScreenPasswdDflt

SNMP PATH	ncdPref 38

TYPE		STRING

DEFAULT		nil

PERMISSION	WRITE-ONLY

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

The password for the screen lock.

NAME		pref-console-show-timestamps

HIDEBOX		Console=>Setup=>Change User Preferences=>

Console and Utilities=>Show Console Timestamps

SNMP NAME	ncdPrefConsoleShowTimestamps

SNMP PATH	ncdPref 40

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether to display timestamps with console messages.

NAME		pref-environment

HIDEBOX		Console=>Setup=>Change User Preferences=>

Environment Variables=>Environment Variables

SNMP NAME	ncdPrefEnvVarTable

SNMP PATH	ncdPref 54

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the current environment variable settings for use

by local clients.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdPrefEnvVarTableIndex

SNMP PATH	ncdPrefEnvVarTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Environment Variable table

NAME		name

SNMP NAME	ncdPrefEnvVarTableName

SNMP PATH	ncdPrefEnvVarTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the Environment Variable name.

NAME		value

SNMP NAME	ncdPrefEnvVarTableValue

SNMP PATH	ncdPrefEnvVarTable 3

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the Environment Variable value.

END TABLE ROW ENTRIES

NAME		pref-font-extended-diagnostics

HIDEBOX		Console=>Setup=>Change User Preferences=>

Fonts=>Show Extended Font Diagnostics

SNMP NAME	ncdPrefFontExtendedDiagnostics

SNMP PATH	ncdPref 26

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether to report all font actions which require

file system or network access.

NAME		pref-font-path

HIDEBOX		Console=>Setup=>Change User Preferences=>

Fonts=>Current Font Path

SNMP NAME	ncdPrefFontPathTable

SNMP PATH	ncdPref 27

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the current font search path.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdPrefFontPathTableIndex

SNMP PATH	ncdPrefFontPathTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Font Path Table

NAME		font-path-entry

SNMP NAME	ncdPrefFontPathTableEntry

SNMP PATH	ncdPrefFontPathTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies a current font path entry in the table

END TABLE ROW ENTRIES

NAME		pref-keyboard-auto-repeat

HIDEBOX		Console=>Setup=>Change User Preferences=>

Keyboard=>Enable Keyboard Autorepeat

SNMP NAME	ncdPrefKbdAutoRepeat

SNMP PATH	ncdPref 2

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Toggles whether keys repeat.

NAME		pref-keyboard-auto-repeat-rate

HIDEBOX		Console=>Setup=>Change User Preferences=>

Keyboard=>Autorepeat Rate

SNMP NAME	ncdPrefKbdAutoRepeatRate

SNMP PATH	ncdPref 4

TYPE		INTEGER

MIN		2

DEFAULT		20

MAX		30

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Times per second that keys auto-repeat.

NAME		pref-keyboard-auto-repeat-start

HIDEBOX		Console=>Setup=>Change User Preferences=>

Keyboard=>Autorepeat Begin After

SNMP NAME	ncdPrefKdbAutoRepeatAfter

SNMP PATH	ncdPref 3

TYPE		INTEGER

MIN		0

DEFAULT		500

MAX		1000

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Number of milliseconds key must be held down before it starts

autorepeating.

NAME		pref-keyboard-function-key-style

HIDEBOX		Console=>Setup=>Change User Preferences=>

Keyboard=>Function Key Style

SNMP NAME	ncdPrefKbdFuncKeyMap

SNMP PATH	ncdPref 1

TYPE		CHOICE

CHOICES		{ vms unix }

DEFAULT		vms

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Determines how function keys should be interpreted.

NAME		pref-keyboard-keyclick-volume

HIDEBOX		Console=>Setup=>Change User Preferences=>

Keyboard=>Keyclick Volume

SNMP NAME	ncdPrefKeyClickVolume

SNMP PATH	ncdPref 5

TYPE		INTEGER

MIN		0

DEFAULT		0

MAX		100

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Volume of keyclick in percentage of maximum volume.

NAME		pref-keyboard-leds

HIDEBOX		Console=>Setup=>Change User Preferences=>

Keyboard=>LEDs

SNMP NAME	ncdPrefKbdLedsTable

SNMP PATH	ncdPref 6

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Describes what LEDs indicate.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdPrefLedsTableIndex

SNMP PATH	ncdPrefKbdLedsTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the LED Table

NAME		led-entry

SNMP NAME	ncdPrefLedsTableEntry

SNMP PATH	ncdPrefKbdLedsTable 2

TYPE		CHOICE

CHOICES		{ x1 x2 x3 x4 net warn ctrl shift caps num mod1 mod2 mod3 mod4 mod5 }

DEFAULT		x1

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the LED values

END TABLE ROW ENTRIES

NAME		pref-mouse-acceleration

HIDEBOX		Console=>Setup=>Change User Preferences=>

Pointing Devices=>Acceleration Ratio

SNMP NAME	ncdPrefMouseAcceration

SNMP PATH	ncdPref 12

TYPE		INTEGER

MIN		10

DEFAULT		300

MAX		1000

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the percentage by which physical mouse motion

past the mouse threshold has been passed should be multipled

to obtain the distance traveled by the pointer cursor.  A

value of less than 100 makes the mouse slower; a value over

than 100 makes it faster.

NAME		pref-mouse-arrangement

HIDEBOX		Console=>Setup=>Change User Preferences=>

Pointing Devices=>Button Arrangement

SNMP NAME	ncdPrefMouseButtonArragement

SNMP PATH	ncdPref 13

TYPE		CHOICE

CHOICES		{ right-handed left-handed }

DEFAULT		right-handed

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether the mouse buttons are mapped with Button1

at the right or left.

NAME		pref-mouse-threshold

HIDEBOX		Console=>Setup=>Change User Preferences=>

Pointing Devices=>Threshold Distance

SNMP NAME	ncdPrefMouseThresholdDistance

SNMP PATH	ncdPref 11

TYPE		INTEGER

MIN		1

DEFAULT		1

MAX		20

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the number of pixels the mouse must be moved

before the acceleration factor is applied.

NAME		pref-opengl-allow-alpha

HIDEBOX		Console=>Setup=>Change User Preferences=>

OpenGL=>Allow Alpha Test

SNMP NAME	ncdOpenglPrefAllowAlpha

SNMP PATH	ncdPrefOpengl 12

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Enables alpha test control.

If disabled, alpha test is not client-controllable.

The alpha test accepts or rejects an incoming pixel

for rendering based on its alpha value.

Alpha test is available only with RGBA visuals --

enabling this parameter has no effect with

Color-Index visuals.

NAME		pref-opengl-allow-blend

HIDEBOX		Console=>Setup=>Change User Preferences=>

OpenGL=>Allow Blending

SNMP NAME	ncdOpenglPrefAllowBlend

SNMP PATH	ncdPrefOpengl 10

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Enables color blending control and operations.

If disabled, blending is not client-controllable.

Blending combines an incoming pixel's color and

alpha values with those of the pixel already

stored at the location.

Blending is available only with RGBA visuals.

Enabling this parameter has no effect with

Color-Index visuals.

NAME		pref-opengl-allow-depth

HIDEBOX		Console=>Setup=>Change User Preferences=>

OpenGL=>Allow Depth Test

SNMP NAME	ncdOpenglPrefAllowDepth

SNMP PATH	ncdPrefOpengl 8

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Enables Z-buffering control and operations.

If disabled, depth test is not client-controllable.

Depth testing is normally used for hidden-surface

elimination.

NAME		pref-opengl-allow-dither

HIDEBOX		Console=>Setup=>Change User Preferences=>

OpenGL=>Allow Dithering

SNMP NAME	ncdOpenglPrefAllowDither

SNMP PATH	ncdPrefOpengl 9

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Enables color dithering control and operations.

If disabled, dithering is not client-controllable.

Dithering improves color resolution when rendering

to a visual with a small number of color bitplanes.

In this implementation, dithering is available only

with 8-bit deep visuals -- enabling this parameter

has no effect with 24-bit deep visuals.

NAME		pref-opengl-allow-logic-operations

HIDEBOX		Console=>Setup=>Change User Preferences=>

OpenGL=>Allow Logical Operations

SNMP NAME	ncdOpenglPrefAllowLogicOperations

SNMP PATH	ncdPrefOpengl 15

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Enables bit-wise logical operations on pixels.

If disabled, logic-ops are not client-controllable.

This mechanism combines incoming pixel values

with pixels that are already drawn using one

of sixteen binary logical operations.

Logical operations are available only with

Color-Index visuals -- enabling this parameter

has no effect with RGBA visuals.

NAME		pref-opengl-allow-polygon-stipple

HIDEBOX		Console=>Setup=>Change User Preferences=>

OpenGL=>Allow Polygon Stipple

SNMP NAME	ncdOpenglPrefAllowPolygonStipple

SNMP PATH	ncdPrefOpengl 14

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Enables stippling operations on polygons.

If disabled, stippling is not client-controllable.

Stippling is used to fill polygons with a non-solid

client-defined 32-bit by 32-bit window-aligned

pattern.

NAME		pref-opengl-allow-scissor

HIDEBOX		Console=>Setup=>Change User Preferences=>

OpenGL=>Allow Scissor Test

SNMP NAME	ncdOpenglPrefAllowScissor

SNMP PATH	ncdPrefOpengl 13

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Enables scissor test control and operations.

If disabled, scissor test is not client-controllable.

Scissoring is typically used to mask out a

rectangular region of the screen to prevent

drawing from occuring outside of it.

NAME		pref-opengl-allow-stencil

HIDEBOX		Console=>Setup=>Change User Preferences=>

OpenGL=>Allow Stencil Test

SNMP NAME	ncdOpenglPrefAllowStencil

SNMP PATH	ncdPrefOpengl 11

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Enables stencil test control & stenciling operations.

If disabled, stenciling is not client-controllable.

Stenciling is typically used to prevent drawing to an

irregularly shaped region of the screen.

Disabling this parameter results in memory savings as

the space for the stencil buffer is not allocated.

NAME		pref-opengl-debug

HIDEBOX		No Widget

SNMP NAME	ncdOpenglPrefDebug

SNMP PATH	ncdPrefOpengl 1

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Enables memory checks & various debug statements.

NAME		pref-opengl-fast-perspective

HIDEBOX		Console=>Setup=>Change User Preferences=>

OpenGL=>Fast Perspective Rendering

SNMP NAME	ncdOpenglPrefFastPerspective

SNMP PATH	ncdPrefOpengl 25

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Enables fast perspective rendering.

Enabling this parameter results in substantial

performance improvements when rendering primitives

in perspective viewing situations (as compared with

the performance when this parameter is disabled).

The disadvantage of enabling fast perspective

rendering is that under certain conditions,

texture-mapping is reduced in quality.  The specific

condition where this occurs is with relatively large

polygons with small textures rendered in a

perspective view that produces extreme foreshortening

of the displayed polygon.

Disabling this parameter is equivalent to

specifying 'Best Texture Mapping Quality'.

NAME		pref-opengl-have-24rgbdb

HIDEBOX		Console=>Setup=>Change User Preferences=>

OpenGL=>Enable 24-bit RGBA double buffered Visual

SNMP NAME	ncdOpenglPrefHave24RGBDB

SNMP PATH	ncdPrefOpengl 23

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Advertise the 24-bit RGBA double buffered Visual

type to clients.

Takes effect only upon the next occurrence of

loading the OpenGL extension.

NAME		pref-opengl-have-24rgbsb

HIDEBOX		Console=>Setup=>Change User Preferences=>

OpenGL=>Enable 24-bit RGBA single buffered Visual

SNMP NAME	ncdOpenglPrefHave24RGBSB

SNMP PATH	ncdPrefOpengl 24

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Advertise the 24-bit RGBA single buffered Visual

type to clients.

Takes effect only upon the next occurrence of

loading the OpenGL extension.

NAME		pref-opengl-have-8cidb

HIDEBOX		Console=>Setup=>Change User Preferences=>

OpenGL=>Enable 8-bit Color-Index double buffered Visual

SNMP NAME	ncdOpenglPrefHave8CIDB

SNMP PATH	ncdPrefOpengl 17

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Advertise the 8-bit Color-Index double buffered Visual

type to clients.

Takes effect only upon the next occurrence of

loading the OpenGL extension.

NAME		pref-opengl-have-8cisb

HIDEBOX		Console=>Setup=>Change User Preferences=>

OpenGL=>Enable 8-bit Color-Index single buffered Visual

SNMP NAME	ncdOpenglPrefHave8CISB

SNMP PATH	ncdPrefOpengl 18

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Advertise the 8-bit Color-Index single buffered Visual

type to clients.

Takes effect only upon the next occurrence of

loading the OpenGL extension.

NAME		pref-opengl-have-8rgbdb

HIDEBOX		Console=>Setup=>Change User Preferences=>

OpenGL=>Enable 8-bit RGBA double buffered Visual

SNMP NAME	ncdOpenglPrefHave8RGBDB

SNMP PATH	ncdPrefOpengl 21

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Advertise the 8-bit RGBA double buffered Visual

type to clients.

Takes effect only upon the next occurrence of

loading the OpenGL extension.

NAME		pref-opengl-have-8rgbsb

HIDEBOX		Console=>Setup=>Change User Preferences=>

OpenGL=>Enable 8-bit RGBA single buffered Visual

SNMP NAME	ncdOpenglPrefHave8RGBSB

SNMP PATH	ncdPrefOpengl 22

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Advertise the 8-bit RGBA single buffered Visual

type to clients.

Takes effect only upon the next occurrence of

loading the OpenGL extension.

NAME		pref-opengl-have-alpha-buffer

HIDEBOX		Console=>Setup=>Change User Preferences=>

OpenGL=>Activate Alpha Buffer for RGBA Visuals

SNMP NAME	ncdOpenglPrefHaveAlphaBuffer

SNMP PATH	ncdPrefOpengl 16

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Allows usage of built-in non-zero-depth alpha buffers.

In this implementation, RGBA visuals are predefined

in pairs, with one having an 8-bit deep alpha buffer,

and the other a zero-bit deep alpha buffer.  Enabling

this parameter allows use of the non-zero-depth alpha

buffers, whereas disabling this parameter causes the

visuals with 8-bit deep alpha buffers not to be

advertised to clients.  Furthermore, disabling this

parameter also results in memory savings as the space

for the alpha buffer is not allocated.

This parameter has no effect for Color-Index visuals.

Takes effect only upon the next occurrence of

loading the OpenGL extension.

NAME		pref-power-manage-enable

HIDEBOX		Console=>Setup=>Change User Preferences=>

Power Management=>Enable VESA Monitor Power Management

SNMP NAME	ncdPrefPowerManageEnable

SNMP PATH	ncdPref 53

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether the power management should be used

NAME		pref-power-manage-powerdown-time

HIDEBOX		Console=>Setup=>Change User Preferences=>

Power Management=>Power Down After

SNMP NAME	ncdPrefPowerManagePowerdownTime

SNMP PATH	ncdPref 52

TYPE		INTEGER

MIN		0

DEFAULT		60

MAX		240

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

How long the terminal needs to be idle  (in minutes)

before the transition to the power down state.

NAME		pref-power-manage-standby-time

HIDEBOX		Console=>Setup=>Change User Preferences=>

Power Management=>Standby After

SNMP NAME	ncdPrefPowerManageStandbyTime

SNMP PATH	ncdPref 50

TYPE		INTEGER

MIN		0

DEFAULT		20

MAX		240

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

How long the terminal needs to be idle  (in minutes)

before the transition to the standby state.

NAME		pref-power-manage-suspend-time

HIDEBOX		Console=>Setup=>Change User Preferences=>

Power Management=>Suspend After

SNMP NAME	ncdPrefPowerManageSuspendTime

SNMP PATH	ncdPref 51

TYPE		INTEGER

MIN		0

DEFAULT		40

MAX		240

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

How long the terminal needs to be idle  (in minutes)

before the transition to the suspend state.

NAME		pref-screen-background-bitmap-background

HIDEBOX		Console=>Setup=>Change User Preferences=>

Screen Background=>Bitmap Background Color

SNMP NAME	ncdPrefScreenBackBitmapBackground

SNMP PATH	ncdPref 18

TYPE		STRING

DEFAULT		white

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the color to use for the background of the

screen background bitmap.

NAME		pref-screen-background-bitmap-file

HIDEBOX		Console=>Setup=>Change User Preferences=>

Screen Background=>Bitmap File

SNMP NAME	ncdPrefScreenBitmapFile

SNMP PATH	ncdPref 16

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the bitmap file to be used if the screen background

is Bitmap.

NAME		pref-screen-background-bitmap-foreground

HIDEBOX		Console=>Setup=>Change User Preferences=>

Screen Background=>Bitmap Foreground Color

SNMP NAME	ncdPrefScreenBackBitmapForeground

SNMP PATH	ncdPref 17

TYPE		STRING

DEFAULT		black

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the color to use for the foreground of the

screen background bitmap.

NAME		pref-screen-background-color

HIDEBOX		Console=>Setup=>Change User Preferences=>

Screen Background=>Solid Color

SNMP NAME	ncdPrefScreenSolidColor

SNMP PATH	ncdPref 15

TYPE		STRING

DEFAULT		#0183c6

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the solid color used if the screen background is Solid Color.

NAME		pref-screen-background-type

HIDEBOX		Console=>Setup=>Change User Preferences=>

Screen Background=>Screen Background Type

SNMP NAME	ncdPrefScreenBackground

SNMP PATH	ncdPref 14

TYPE		CHOICE

CHOICES		{ default solid-color bitmap }

DEFAULT		default

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies what the screen backround should look like.

NAME		pref-screensaver-bitmap-file

HIDEBOX		Console=>Setup=>Change User Preferences=>

Screen Saver=>Screen Saver Bitmap File

SNMP NAME	ncdPrefScreenSaverBitmapFile

SNMP PATH	ncdPref 25

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the bitmap file to use if the screen saver is using

Bitmap mode.

NAME		pref-screensaver-enable

HIDEBOX		Console=>Setup=>Change User Preferences=>

Screen Saver=>Enable Screen Saver

SNMP NAME	ncdPrefScreenSaverEnabled

SNMP PATH	ncdPref 20

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether the screen saver should work.

NAME		pref-screensaver-exposures

HIDEBOX		Console=>Setup=>Change User Preferences=>

Screen Saver=>Allow Exposures After Screen Saver

SNMP NAME	ncdPrefScreenSaverAllowExposures

SNMP PATH	ncdPref 21

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether the screen saver should have clients redraw

themselves when it restores the screen.

NAME		pref-screensaver-interval

HIDEBOX		Console=>Setup=>Change User Preferences=>

Screen Saver=>Change Screen Saver After

SNMP NAME	ncdPrefScreenSaverChangeAfter

SNMP PATH	ncdPref 23

TYPE		INTEGER

MIN		0

DEFAULT		60

MAX		3000

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

How many seconds the screen saver waits

before modifying its pattern.

NAME		pref-screensaver-style

HIDEBOX		Console=>Setup=>Change User Preferences=>

Screen Saver=>Screen Saver Style

SNMP NAME	ncdPrefScreenSaverStyle

SNMP PATH	ncdPref 24

TYPE		CHOICE

CHOICES		{ blank bitmap x-logo hardware }

DEFAULT		blank

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the type of screen saver image to use.

NAME		pref-screensaver-time

HIDEBOX		Console=>Setup=>Change User Preferences=>

Screen Saver=>Save Screen After

SNMP NAME	ncdPrefScreenSaverAfter

SNMP PATH	ncdPref 22

TYPE		INTEGER

MIN		0

DEFAULT		600

MAX		3000

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

How long the terminal needs to be idle  (in seconds)

before the screen saver starts.

NAME		pref-sie-contrast-threshold

HIDEBOX		Console=>Setup=>Change User Preferences=>

X, Graphics, and SIE=>SIE Contrast Threshold

SNMP NAME	ncdPrefSIEContrast

SNMP PATH	ncdPref 43

TYPE		INTEGER

MIN		0

DEFAULT		50

MAX		100

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Contrast value used in SIE scaling operations.

Applies only to Bilinear-Interp & 4x4-Area-methods.

NAME		pref-sie-sampling-method

HIDEBOX		Console=>Setup=>Change User Preferences=>

X, Graphics, and SIE=>SIE Sampling Method

SNMP NAME	ncdPrefSIESamplingMethod

SNMP PATH	ncdPref 42

TYPE		CHOICE

CHOICES		{ nearest-neighbor any-neighbor bilinear-interp 4x4-area }

DEFAULT		nearest-neighbor

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the SIE sampling method to use when scaling.

NAME		pref-xserver-backing-store

HIDEBOX		Console=>Setup=>Change User Preferences=>

X, Graphics, and SIE=>Use Backing Store

SNMP NAME	ncdPrefUseBackingStore

SNMP PATH	ncdPref 30

TYPE		CHOICE

CHOICES		{ when-mapped by-request disabled }

DEFAULT		by-request

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the default backing store given to client windows.

NAME		pref-xserver-benchmark-optimizations

HIDEBOX		Console=>Setup=>Change User Preferences=>

X, Graphics, and SIE=>Enable Benchmark Optimizations

SNMP NAME	ncdPrefBenchmarkOptimizationsEnabled

SNMP PATH	ncdPref 19

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

When enabled, some graphics operations are optimized for

benchmarks.

NAME		pref-xserver-current-pointing-device

HIDEBOX		Console=>Setup=>Change User Preferences=>

Pointing Devices=>Current Pointing Device

SNMP NAME	ncdPrefCurrentPointingDevice

SNMP PATH	ncdPref 46

TYPE		CHOICE

CHOICES		{ mouse mouse-and-extension-device input-extension-device-only }

DEFAULT		mouse

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether the pointer is a mouse

or one of the input extension devices

NAME		pref-xserver-graphics-optimizations

HIDEBOX		Console=>Setup=>Change User Preferences=>

X, Graphics, and SIE=>Enable Graphics Optimizations

SNMP NAME	ncdPrefGraphicsOptimizationsEnabled

SNMP PATH	ncdPref 28

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

When enabled, some graphics operations are optimized with

a possible loss in accuracy.

NAME		pref-xserver-screen-resolution

HIDEBOX		Console=>Setup=>Change User Preferences=>

X, Graphics, and SIE=>Screen Resolution

SNMP NAME	ncdPrefScreenRes

SNMP PATH	ncdPref 29

TYPE		INTEGER

MIN		20

DEFAULT		100

MAX		300

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies what the server reports for its resolution

(in dots per inch) to X clients and the font server.

NAME		pref-xserver-touchscreen-button-press-threshold

HIDEBOX		Console=>Setup=>Change User Preferences=>

Pointing Devices=>Button Press Threshold

SNMP NAME	ncdPrefTouchscreenButtonPressThreshold

SNMP PATH	ncdPref 47

TYPE		INTEGER

MIN		1

DEFAULT		8

MAX		255

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the relative force which the user must exert

on the touchscreen before a button press event is generated.

The button press threshold should be greater than the button

release threshold

NAME		pref-xserver-touchscreen-button-release-threshold

HIDEBOX		Console=>Setup=>Change User Preferences=>

Pointing Devices=>Button Release Threshold

SNMP NAME	ncdPrefTouchscreenButtonReleaseThreshold

SNMP PATH	ncdPref 45

TYPE		INTEGER

MIN		1

DEFAULT		5

MAX		255

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the relative force which the user must exert

on the touchscreen before a button release event is generated.

GROUP serial

NAME		serial-access-control-enabled

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Enable Serial and Parallel Access Control

SNMP NAME	ncdAccessSerialdAccessControlEnabled

SNMP PATH	ncdAccess 31

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether xhost-style access control

is on or off for requests to connect to the

serial or parallel port daemon.

NAME		serial-access-control-list

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Serial and Parallel Access Control List

SNMP NAME	ncdAccessSerialdAccessControlTable

SNMP PATH	ncdAccess 32

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the host access control list for the serial

or parallel port daemon.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdAccessSerialdACTIndex

SNMP PATH	ncdAccessSerialdAccessControlTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Serial Access Control List

NAME		host

SNMP NAME	ncdAccessSerialdACTAddress

SNMP PATH	ncdAccessSerialdAccessControlTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the network name/address

of a node granted permission to

remotely access the serial or parallel

port daemon.

NAME		family

SNMP NAME	ncdAccessSerialdACTFamily

SNMP PATH	ncdAccessSerialdAccessControlTable 3

TYPE		CHOICE

CHOICES		{ tcpip ncdnet lat }

DEFAULT		tcpip

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the type of network connection for

which this entry applies.

END TABLE ROW ENTRIES

NAME		serial-daemons-lat-service-passwords

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Serial Daemon LAT Service Passwords

SNMP NAME	ncdAccessSerialdLatPasswdTable

SNMP PATH	ncdAccess 30

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the LAT service passwords required to

gain access to the serial port daemons using LAT.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdAccessSerialdLPTIndex

SNMP PATH	ncdAccessSerialdLatPasswdTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Serial Daemons LAT Passwds Table

NAME		port-number

SNMP NAME	ncdAccessSerialdLPTPortNumber

SNMP PATH	ncdAccessSerialdLatPasswdTable 2

TYPE		INTEGER

MIN		1

MAX		3

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The name of the serial port.

NAME		password

SNMP NAME	ncdAccessSerialdLPTPassword

SNMP PATH	ncdAccessSerialdLatPasswdTable 3

TYPE		STRING

DEFAULT		nil

PERMISSION	WRITE-ONLY

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the LAT service password.

END TABLE ROW ENTRIES

NAME		serial-daemons-statistics-table

HIDEBOX		Console=>Statistics=>Show Statistics=>

Serial Daemons=>Serial Daemon Statistics Table

SNMP NAME	ncdSerialDaemonStatsTable

SNMP PATH	ncdSerialDaemon 1

TYPE		TABLE

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the table of serial daemon statistics.

BEGIN TABLE ROW ENTRIES

NAME		port-number

SNMP NAME	ncdSerialDaemonPortNumber

SNMP PATH	ncdSerialDaemonStatsTable 1

TYPE		INTEGER

MIN		1

MAX		3

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The name of the serial port.

NAME		connections-accepted

SNMP NAME	ncdSerialDaemonConnAccepts

SNMP PATH	ncdSerialDaemonStatsTable 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of connections made to the serial daemon.

NAME		connections-with-access-control-errors

SNMP NAME	ncdSerialDaemonConnBadAccesses

SNMP PATH	ncdSerialDaemonStatsTable 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of connections that were closed by the

serial daemon due to the remote address not being

present in the serial daemon's access control list.

NAME		connections-with-io-errors

SNMP NAME	ncdSerialDaemonConnIOErrors

SNMP PATH	ncdSerialDaemonStatsTable 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of connections that were closed by the

serial daemon due to the a local I/O error. This

includes the user aborting the connection via local

mechanisms.

NAME		connections-with-normal-completion

SNMP NAME	ncdSerialDaemonConnEofCloses

SNMP PATH	ncdSerialDaemonStatsTable 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of connections that were closed by the

remote host. This is the normal mode of ending a

connection to the serial daemon.

END TABLE ROW ENTRIES

NAME		serial-daemons-table

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Serial=>Serial Daemons Table

SNMP NAME	ncdSerialSDTable

SNMP PATH	ncdSerial 3

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the table of serial daemon parameters.

BEGIN TABLE ROW ENTRIES

NAME		serial-daemon-table-index

SNMP NAME	ncdSerialSDTableIndex

SNMP PATH	ncdSerialSDTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The index into the serial daemons interface table

NAME		port-number

SNMP NAME	ncdSerialSDTablePortNumber

SNMP PATH	ncdSerialSDTable 2

TYPE		INTEGER

MIN		1

MAX		3

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The name of the serial port.

NAME		use-serial-protocol

SNMP NAME	ncdSerialSDTableUseSerialProt

SNMP PATH	ncdSerialSDTable 3

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that the new serial daemon control protocol

should be used.  Note that this could cause some data

loss if enabled with old host software.

NAME		tcp-port

SNMP NAME	ncdSerialSDTableTCPPort

SNMP PATH	ncdSerialSDTable 4

TYPE		INTEGER

MIN		1

DEFAULT		87

MAX		65535

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the TCP port on which the terminal

listens for raw TCP connections to the serial

daemon.

NAME		ncdnet-object-name

SNMP NAME	ncdSerialSDTableUseNCDnetObjName

SNMP PATH	ncdSerialSDTable 5

TYPE		STRING

DEFAULT		SERIALD

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the NCDnet object on which the terminal

listens for NCDnet connections to the serial

daemon.

NAME		enable-lat-service

SNMP NAME	ncdSerialSDTableEnableLAT

SNMP PATH	ncdSerialSDTable 6

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies that the unit is to accept incoming

LAT solicit messages for a service which

allows access to the serial daemon. This is used

to support VMS print services to the unit.

NAME		lat-service-name

SNMP NAME	ncdSerialSDTableLATServName

SNMP PATH	ncdSerialSDTable 7

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the name of the serial daemon LAT service.

NAME		lat-service-rating

SNMP NAME	ncdSerialSDTableLATServRating

SNMP PATH	ncdSerialSDTable 8

TYPE		INTEGER

MIN		0

DEFAULT		50

MAX		255

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the rating of the serial daemon LAT service.

END TABLE ROW ENTRIES

NAME		serial-interface-statistics-table

HIDEBOX		Console=>Statistics=>Show Statistics=>

Serial=>Interface Statistics Table

SNMP NAME	ncdSerialTable

SNMP PATH	ncdSerial 1

TYPE		TABLE

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The table of serial interfaces.

BEGIN TABLE ROW ENTRIES

NAME		port-number

SNMP NAME	ncdSerialInPortNumber

SNMP PATH	ncdSerialTable 5

TYPE		INTEGER

MIN		1

MAX		3

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The name of the serial port.

NAME		reception-overruns

SNMP NAME	ncdSerialInOverruns

SNMP PATH	ncdSerialTable 1

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times an incoming octet overwrote a

previously received octet in the UART's buffer.

NAME		reception-frame-errors

SNMP NAME	ncdSerialInFrameErrors

SNMP PATH	ncdSerialTable 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times an incoming octet had bad bit

alignment.

NAME		reception-parity-errors

SNMP NAME	ncdSerialInParityErrors

SNMP PATH	ncdSerialTable 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times the parity bit of an incoming

octet was incorrect.

NAME		reception-break-errors

SNMP NAME	ncdSerialInBreakErrors

SNMP PATH	ncdSerialTable 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times a stop bit was not received

at the expected time after a start bit, and the

intervening bits were all zero.

NAME		total-characters-transmitted

SNMP NAME	ncdSerialInTotCharXmit

SNMP PATH	ncdSerialTable 6

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Total characters transmitted

NAME		total-transmit-interrupts

SNMP NAME	ncdSerialInTotXmitInt

SNMP PATH	ncdSerialTable 7

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Total transmission interrupts

NAME		total-characters-received

SNMP NAME	ncdSerialInTotCharRcv

SNMP PATH	ncdSerialTable 8

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Total of characters received

NAME		total-receive-interrupts

SNMP NAME	ncdSerialInTotRcvInt

SNMP PATH	ncdSerialTable 9

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Total of reception interrupts

NAME		total-receive-errors

SNMP NAME	ncdSerialInTotRcvErr

SNMP PATH	ncdSerialTable 10

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Total of reception errors

END TABLE ROW ENTRIES

NAME		serial-interfaces-table

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Serial=>Serial Interfaces Table

SNMP NAME	ncdSerialIfTable

SNMP PATH	ncdSerial 2

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the table of serial port parameters.

BEGIN TABLE ROW ENTRIES

NAME		serial-interface-table-index

SNMP NAME	ncdSerialIfTableIndex

SNMP PATH	ncdSerialIfTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The index into the serial interface table

NAME		port-number

SNMP NAME	ncdSerialIfTablePortNumber

SNMP PATH	ncdSerialIfTable 2

TYPE		INTEGER

MIN		1

MAX		3

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The name of the serial port.

NAME		mode

SNMP NAME	ncdSerialIfTableMode

SNMP PATH	ncdSerialIfTable 3

TYPE		CHOICE

CHOICES		{ terminal printer serial-daemon slip console input-device xremote ppp }

DEFAULT		terminal

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies what the serial port should be used for, but takes

effect the next time the unit

is booted.  The choices

'Printer' and 'Serial Daemon' are equivalent.

NAME		current-mode

SNMP NAME	ncdSerialIfTableCurMode

SNMP PATH	ncdSerialIfTable 9

TYPE		CHOICE

CHOICES		{ terminal printer serial-daemon slip console input-device xremote ppp }

DEFAULT		terminal

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies what the serial port should be used for.  Any

change takes effect immediately, but the value of this

parameter is ignored at boot time in favor of the 'mode'

value.

NAME		baud-rate

SNMP NAME	ncdSerialIfTableBaudRate

SNMP PATH	ncdSerialIfTable 4

TYPE		CHOICE

CHOICES		{ 50 75 110 134.5 150 200 300 600 1050 1200 1800 2000 2400 4800 7200 9600 14400 19200 38400 57600 76800 115200 }

DEFAULT		9600

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the baud rate of the serial port.

NAME		data-bits

SNMP NAME	ncdSerialIfTableDataBits

SNMP PATH	ncdSerialIfTable 5

TYPE		CHOICE

CHOICES		{ 8 7 }

DEFAULT		8

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the number of data bits per

character of the serial port.

NAME		stop-bits

SNMP NAME	ncdSerialIfTableStopBits

SNMP PATH	ncdSerialIfTable 6

TYPE		CHOICE

CHOICES		{ 1 2 }

DEFAULT		1

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the number of stop bits per

character of the serial port.

NAME		parity

SNMP NAME	ncdSerialIfTableParity

SNMP PATH	ncdSerialIfTable 7

TYPE		CHOICE

CHOICES		{ none odd even space mark }

DEFAULT		none

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the form of parity generated by

and expected by the serial port.

NAME		handshake

SNMP NAME	ncdSerialIfTableHandshake

SNMP PATH	ncdSerialIfTable 8

TYPE		CHOICE

CHOICES		{ none xon/xoff dtr/dsr rts/cts }

DEFAULT		none

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the type of flow control

of the serial port.

NAME		hangup

SNMP NAME	ncdSerialIfTableActionOnDiscon

SNMP PATH	ncdSerialIfTable 10

TYPE		CHOICE

CHOICES		{ none drop-dtr send-break }

DEFAULT		none

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies what a local NCDterm client will do

when closing the serial port.  Also specifies

what the serial daemon will do to

signal the end of a network connection.

END TABLE ROW ENTRIES

GROUP snmp

NAME		snmp-allow-reset

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Allow SNMP Reset

SNMP NAME	ncdAccessSnmpResetEnabled

SNMP PATH	ncdAccess 21

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether remote reset via SNMP is enabled.

NAME		snmp-bad-variable-name-error-responses-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Bad Variable Name Error Responses Received

SNMP NAME	snmpInNoSuchNames

SNMP PATH	snmp 9

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP PDUs which were

delivered to the SNMP protocol entity and for

which the value of the error-status field is

`noSuchName'.

NAME		snmp-bad-variable-name-error-responses-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Bad Variable Name Error Responses Transmitted

SNMP NAME	snmpOutNoSuchNames

SNMP PATH	snmp 21

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP PDUs which were

generated by the SNMP protocol entity and for

which the value of the error-status is `noSuchName'.

NAME		snmp-bad-variable-values-error-responses-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Bad Variable Values Error Responses Received

SNMP NAME	snmpInBadValues

SNMP PATH	snmp 10

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP PDUs which were

delivered to the SNMP protocol entity and for

which the value of the error-status field is

`badValue'.

NAME		snmp-bad-variable-values-error-responses-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Bad Variable Values Error Responses Transmitted

SNMP NAME	snmpOutBadValues

SNMP PATH	snmp 22

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP PDUs which were

generated by the SNMP protocol entity and for

which the value of the error-status field is

`badValue'.

NAME		snmp-general-error-responses-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>General Error Responses Received

SNMP NAME	snmpInGenErrs

SNMP PATH	snmp 12

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP PDUs which were

delivered to the SNMP protocol entity and for

which the value of the error-status field is

`genErr'.

NAME		snmp-general-error-responses-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>General Error Responses Transmitted

SNMP NAME	snmpOutTooBigs

SNMP PATH	snmp 24

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP PDUs which were

generated by the SNMP protocol entity and for

which the value of the error-status field is

`genErr'.

NAME		snmp-get-next-requests-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Get-Next Requests Received

SNMP NAME	snmpInGetNexts

SNMP PATH	snmp 16

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP Get-Next PDUs which have

been accepted and processed by the SNMP protocol

entity.

NAME		snmp-get-next-requests-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Get-Next Requests Transmitted

SNMP NAME	snmpOutGetNexts

SNMP PATH	snmp 26

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP Get-Next PDUs which have

been generated by the SNMP protocol entity.

NAME		snmp-get-requests-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Get Requests Received

SNMP NAME	snmpInGetRequests

SNMP PATH	snmp 15

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP Get-Request PDUs which

have been accepted and processed by the SNMP

protocol entity.

NAME		snmp-get-requests-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Get Requests Transmitted

SNMP NAME	snmpOutGetRequests

SNMP PATH	snmp 25

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP Get-Request PDUs which

have been generated by the SNMP protocol entity.

NAME		snmp-get-responses-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Get Responses Received

SNMP NAME	snmpInGetResponses

SNMP PATH	snmp 18

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP Get-Response PDUs which

have been accepted and processed by the SNMP

protocol entity.

NAME		snmp-get-responses-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Get Responses Transmitted

SNMP NAME	snmpOutGetResponses

SNMP PATH	snmp 28

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP Get-Response PDUs which

have been generated by the SNMP protocol entity.

NAME		snmp-packets-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Packets Received

SNMP NAME	snmpInPkts

SNMP PATH	snmp 1

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of Messages delivered to the

SNMP entity from the transport service.

NAME		snmp-packets-received-with-asn1-errors

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Packets Received with ASN.1 Errors

SNMP NAME	snmpInASNParseErrs

SNMP PATH	snmp 6

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of ASN.1 or BER errors

encountered by the SNMP protocol entity when

decoding received SNMP Messages.

NAME		snmp-packets-received-with-bad-community-names

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Packets Received with Bad Community Names

SNMP NAME	snmpInBadCommunityNames

SNMP PATH	snmp 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP Messages delivered to

the SNMP protocol entity which used a SNMP

community name not known to said entity.

NAME		snmp-packets-received-with-bad-community-uses

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Packets Received with Bad Community Uses

SNMP NAME	snmpInBadCommunityUses

SNMP PATH	snmp 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP Messages delivered to

the SNMP protocol entity which represented an SNMP

operation which was not allowed by the SNMP

community named in the Message.

NAME		snmp-packets-received-with-bad-variable-types

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Packets Received with Bad Variable Types

SNMP NAME	snmpInBadTypes

SNMP PATH	snmp 7

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP PDUs delivered to the SNMP

protocol entity which had an unknown PDU type.

NAME		snmp-packets-received-with-bad-versions

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Packets Received with Bad Versions

SNMP NAME	snmpInBadVersions

SNMP PATH	snmp 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP Messages which were

delivered to the SNMP protocol entity and were for

an unsupported SNMP version.

NAME		snmp-packets-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Packets Transmitted

SNMP NAME	snmpOutPkts

SNMP PATH	snmp 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP Messages which were

passed from the SNMP protocol entity to the

transport service.

NAME		snmp-read-only-access-control-enabled

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Enable SNMP Read-Only Access Control

SNMP NAME	ncdAccessSnmpReadOnlyAccessControl

SNMP PATH	ncdAccess 15

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether requests for connections to the

SNMP daemon from outside the unit will

be checked against the read-only host access control list

NAME		snmp-read-only-access-control-list

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>SNMP Read-Only Access Control List

SNMP NAME	ncdAccessSnmpReadOnlyAccessControlTable

SNMP PATH	ncdAccess 16

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the list of IP hosts that allowed

to use the SNMP read-only community to access

network management information via SNMP.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdAccessSnmpROAccessControlTableIndex

SNMP PATH	ncdAccessSnmpReadOnlyAccessControlTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the SNMP RO Access Control Table

NAME		host

SNMP NAME	ncdAccessSnmpROAccessControlTableHost

SNMP PATH	ncdAccessSnmpReadOnlyAccessControlTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the network name/address

of a node granted permission to

remotely access the SNMP daemon.

END TABLE ROW ENTRIES

NAME		snmp-read-only-community

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>SNMP Read-Only Community

SNMP NAME	ncdAccessSnmpReadOnlyCommunity1

SNMP PATH	ncdAccess 13

TYPE		STRING

DEFAULT		public

PERMISSION	WRITE-ONLY

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the first of two possible community names that can

be specified in SNMP requests to obtain read-only access

to configuration information.

NAME		snmp-read-only-community-alt

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>SNMP Read-Only Alternative Community

SNMP NAME	ncdAccessSnmpReadOnlyCommunity2

SNMP PATH	ncdAccess 14

TYPE		STRING

DEFAULT		nil

PERMISSION	WRITE-ONLY

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the second of two possible community names that can

be specified in SNMP requests to obtain read-only access

to configuration information.

NAME		snmp-read-only-variable-error-responses-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Read-Only Variable Error Responses Received

SNMP NAME	snmpInReadOnlys

SNMP PATH	snmp 11

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number valid SNMP PDUs which were

delivered to the SNMP protocol entity and for

which the value of the error-status field is

`readOnly'.  It should be noted that it is a

protocol error to generate an SNMP PDU which

contains the value `readOnly' in the error-status

field, as such this object is provided as a means

of detecting incorrect implementations of SNMP.

NAME		snmp-read-only-variable-error-responses-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Read-Only Variable Error Responses Transmitted

SNMP NAME	snmpOutReadOnlys

SNMP PATH	snmp 23

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number valid SNMP PDUs which were generated by

the SNMP protocol entity and for which the value of the

error-status field is `readOnly'.

NAME		snmp-read-write-access-control-enabled

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Enable SNMP Read-Write Access Control

SNMP NAME	ncdAccessSnmpReadWriteAccessControl

SNMP PATH	ncdAccess 19

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether requests for connections to the

SNMP daemon from outside the unit will

be checked against the read-write host access control list

NAME		snmp-read-write-access-control-list

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>SNMP Read-Write Access Control List

SNMP NAME	ncdAccessSnmpReadWriteAccessControlTable

SNMP PATH	ncdAccess 20

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the list of IP hosts that allowed

to use the SNMP read-write community to access

network management information via SNMP.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdAccessSnmpRWAccessControlTableIndex

SNMP PATH	ncdAccessSnmpReadWriteAccessControlTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the SNMP RW Access Control Table

NAME		host

SNMP NAME	ncdAccessSnmpRWAccessControlTableAddress

SNMP PATH	ncdAccessSnmpReadWriteAccessControlTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the network name/address

of a node granted permission to

remotely access the SNMP daemon.

END TABLE ROW ENTRIES

NAME		snmp-read-write-community

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>SNMP Read-Write Community

SNMP NAME	ncdAccessSnmpReadWriteCommunity1

SNMP PATH	ncdAccess 17

TYPE		STRING

DEFAULT		nil

PERMISSION	WRITE-ONLY

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the first of two possible community names that can

be specified in SNMP requests to obtain read-write access

to configuration information.

NAME		snmp-read-write-community-alt

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>SNMP Read-Write Alternative Community

SNMP NAME	ncdAccessSnmpReadWriteCommunity2

SNMP PATH	ncdAccess 18

TYPE		STRING

DEFAULT		nil

PERMISSION	WRITE-ONLY

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the second of two possible community names that can

be specified in SNMP requests to obtain read-write access

to configuration information.

NAME		snmp-set-requests-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Set Requests Received

SNMP NAME	snmpInSetRequests

SNMP PATH	snmp 17

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP Set-Request PDUs which

have been accepted and processed by the SNMP

protocol entity.

NAME		snmp-set-requests-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Set Requests Transmitted

SNMP NAME	snmpOutSetRequests

SNMP PATH	snmp 27

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP Set-Request PDUs which

have been generated by the SNMP protocol entity.

NAME		snmp-too-big-error-responses-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Too Big Error Responses Received

SNMP NAME	snmpInTooBigs

SNMP PATH	snmp 8

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP PDUs which were

delivered to the SNMP protocol entity and for

which the value of the error-status field is

`tooBig'.

NAME		snmp-too-big-error-responses-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Too Big Error Responses Transmitted

SNMP NAME	snmpOutTooBigs

SNMP PATH	snmp 20

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP PDUs which were

generated by the SNMP protocol entity and for

which the value of the error-status field is

`tooBig.'

NAME		snmp-trap-monitors

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>SNMP Trap Monitors

SNMP NAME	ncdAccessSnmpTrapMonitorsTable

SNMP PATH	ncdAccess 22

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the list of IP hosts to which SNMP traps are sent.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdAccessSnmpTMTIndex

SNMP PATH	ncdAccessSnmpTrapMonitorsTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Trap Monitors Table

NAME		host

SNMP NAME	ncdAccessSnmpTMTHost

SNMP PATH	ncdAccessSnmpTrapMonitorsTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the network name/address

of a node granted permission to

remotely access the SNMP daemon.

NAME		community

SNMP NAME	ncdAccessSnmpTMTCommunity

SNMP PATH	ncdAccessSnmpTrapMonitorsTable 3

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the community name to use

in the trap message sent to the host.

END TABLE ROW ENTRIES

NAME		snmp-traps-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Traps Received

SNMP NAME	snmpInTraps

SNMP PATH	snmp 19

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP Trap PDUs which have

been accepted and processed by the SNMP protocol

entity.

NAME		snmp-traps-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Traps Transmitted

SNMP NAME	snmpOutTraps

SNMP PATH	snmp 29

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of SNMP Trap PDUs which have

been generated by the SNMP protocol entity.

NAME		snmp-variable-gets-completed

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Variable Gets Completed

SNMP NAME	snmpInTotalReqVars

SNMP PATH	snmp 13

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of MIB objects which have been

retrieved successfully by the SNMP protocol entity

as the result of receiving valid SNMP Get-Request

and Get-Next PDUs.

NAME		snmp-variable-sets-completed

HIDEBOX		Console=>Statistics=>Show Statistics=>

SNMP=>Variable Sets Completed

SNMP NAME	snmpInTotalSetVars

SNMP PATH	snmp 14

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of MIB objects which have been

altered successfully by the SNMP protocol entity

as the result of receiving valid SNMP Set-Request

PDUs.

GROUP tcp

NAME		tcp-ack-only-packets-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Ack-Only Packets Transmitted

SNMP NAME	ncdTcpOutBareAcks

SNMP PATH	ncdTcp 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of TCP packets received which acknowledged

1 or more octets of unacknowledged data.

NAME		tcp-ack-packets-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Ack Packets Received

SNMP NAME	ncdTcpInAcks

SNMP PATH	ncdTcp 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of TCP packets transmitted which contained

no data, and only the ACK flag turned on.

NAME		tcp-active-connect-attempts

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Active Connect Attempts

SNMP NAME	tcpActiveOpens

SNMP PATH	tcp 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times TCP connections have made a

direct transition to the SYN-SENT state from the

CLOSED state.

NAME		tcp-connect-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP=>Connect Timeout

SNMP NAME	ncdTcpConnectTimeout

SNMP PATH	ncdTcp 6

TYPE		INTEGER

MIN		1

DEFAULT		75

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) that must elapse

between a TCP connection attempt and a lack of a response

for a failure situation to be declared.

NAME		tcp-connection-table

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Connection Table

SNMP NAME	tcpConnTable

SNMP PATH	tcp 13

TYPE		TABLE

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The table of TCP connections.

BEGIN TABLE ROW ENTRIES

NAME		local-ip-address

SNMP NAME	tcpConnLocalAddress

SNMP PATH	tcpConnTable 2

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The local IP address for this TCP connection.  In

the case of a connection in the listen state which

is willing to accept connections for any IP

interface associated with the node, the value

0.0.0.0 is used.

NAME		local-port

SNMP NAME	tcpConnLocalPort

SNMP PATH	tcpConnTable 3

TYPE		INTEGER

MIN		0

MAX		65535

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The local port number for this TCP connection.

NAME		remote-ip-address

SNMP NAME	tcpConnRemAddress

SNMP PATH	tcpConnTable 4

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The remote IP address for this TCP connection.

NAME		remote-port

SNMP NAME	tcpConnRemPort

SNMP PATH	tcpConnTable 5

TYPE		INTEGER

MIN		0

MAX		65535

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The remote port number for this TCP connection.

NAME		state

SNMP NAME	tcpConnState

SNMP PATH	tcpConnTable 1

TYPE		CHOICE

CHOICES		{ closed listen syn-sent syn-received established fin-wait-1 fin-wait-2 close-wait last-ack closing time-wait }

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The state of this TCP connection.

END TABLE ROW ENTRIES

NAME		tcp-connections-reset

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Connections Reset

SNMP NAME	tcpEstabResets

SNMP PATH	tcp 8

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times TCP connections have made a

direct transition to the CLOSED state from either

the ESTABLISHED state or the CLOSE-WAIT state.

NAME		tcp-connections-timed-out-on-keepalives

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Connections Timed Out on Keepalives

SNMP NAME	ncdTcpKeepAliveDrops

SNMP PATH	ncdTcp 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of connections progressing directly to the

CLOSED state due to too many keep alives being sent.

NAME		tcp-connections-timed-out-on-retransmissions

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Connections Timed Out on Retransmissions

SNMP NAME	ncdTcpRetransDrops

SNMP PATH	ncdTcp 1

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of connections progressing directly to the

CLOSED state due to too many retransmissions being sent.

NAME		tcp-default-linger-time

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP=>Default Linger Time

SNMP NAME	ncdTcpDfltLingerTime

SNMP PATH	ncdTcp 11

TYPE		INTEGER

MIN		1

DEFAULT		120

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the default time (in seconds) TCP continues to attempt

reliable transmission of outstanding data on

the connection's transmit queue after local software

closes a connection. This timer is optional and can be

configure on a per connection basis by the software by

higher level software.

NAME		tcp-default-mss-for-non-local

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP=>Use default maximum segment size for non-local hosts

SNMP NAME	ncdTcpDfltMssForNonLocal

SNMP PATH	ncdTcp 14

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

By default, when communicating with non-local hosts, the default TCP maximum

segment size of 536 bytes is used.  Setting this parameter to false

allows larger segment sizes to be used.

NAME		tcp-established-connections

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Established Connections

SNMP NAME	tcpCurrEstab

SNMP PATH	tcp 9

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of TCP connections for which the

current state is either ESTABLISHED or CLOSE-WAIT.

NAME		tcp-failed-connect-attempts

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Failed Connect Attempts

SNMP NAME	tcpAttemptFails

SNMP PATH	tcp 7

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times TCP connections have made a

direct transition to the CLOSED state from either

the SYN-SENT state or the SYN-RCVD state, plus the

number of times TCP connections have made a direct

transition to the LISTEN state from the SYN-RCVD

state.

NAME		tcp-keepalive-idle-time

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP=>Keepalive Idle Time

SNMP NAME	ncdTcpKeepIdle

SNMP PATH	ncdTcp 8

TYPE		INTEGER

MIN		1

DEFAULT		75

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) that must elapse

on an established TCP connection with no activity

before a TCP keepalive packet is generated.

NOTE: TCP keepalive packets are currently not generated by

NCD terminals.

NAME		tcp-keepalive-interval

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP=>Keepalive Interval

SNMP NAME	ncdTcpKeepAliveIntvl

SNMP PATH	ncdTcp 9

TYPE		INTEGER

MIN		1

DEFAULT		75

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) between successive

transmissions of TCP keepalive packets when no

response to the keepalive packets has been received.

NOTE: TCP keepalive packets are currently not generated by

NCD terminals.

NAME		tcp-keepalive-packets-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Keepalive Packets Transmitted

SNMP NAME	ncdTcpOutKeepAlives

SNMP PATH	ncdTcp 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of TCP Keep Alive packets transmitted.

NAME		tcp-max-keepalive-idle-time

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP=>Max Keepalive Idle Time

SNMP NAME	ncdTcpMaxKeepAliveIdeTime

SNMP PATH	ncdTcp 10

TYPE		INTEGER

MIN		1

DEFAULT		600

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the maximum amount of time (in seconds) that

TCP keepalives are transmitted with no response

before a failure situation is declared.

NOTE: TCP keepalive packets are currently not generated by

NCD terminals.

NAME		tcp-max-retransmissions

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP=>Max Retransmissions

SNMP NAME	ncdTcpMaxRxt

SNMP PATH	ncdTcp 7

TYPE		INTEGER

MIN		12

DEFAULT		12

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the number of retransmission that must

occur on a TCP connection before a failure situation is

declared.

NAME		tcp-maximum-number-of-connections

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Maximum Number of Connections

SNMP NAME	tcpMaxConn

SNMP PATH	tcp 4

TYPE		INTEGER

MIN		-1

MAX		2147483647

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The limit on the total number of TCP connections

the entity can support.  In entities where the

maximum number of connections is dynamic, this

object should contain the value -1.

NAME		tcp-maximum-retransmission-time

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Maximum Retransmission Time

SNMP NAME	tcpRtoMax

SNMP PATH	tcp 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The maximum value (in seconds) permitted for

the retransmission timeout.

NAME		tcp-minimum-retransmission-time

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Minimum Retransmission Time

SNMP NAME	tcpRtoMin

SNMP PATH	tcp 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The minimum value (in seconds) permitted for

the retransmission timeout.

NAME		tcp-packets-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Packets Received

SNMP NAME	tcpInSegs

SNMP PATH	tcp 10

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of segments received, including

those received in error.  This count includes

segments received on currently established connections.

NAME		tcp-packets-retransmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Packets Retransmitted

SNMP NAME	tcpRetransSegs

SNMP PATH	tcp 12

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of segments retransmitted - that

is, the number of TCP segments transmitted

containing one or more previously transmitted

octets.

NAME		tcp-packets-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Packets Transmitted

SNMP NAME	tcpOutSegs

SNMP PATH	tcp 11

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of segments sent, including

those on current connections but excluding those

containing only retransmitted octets.

NAME		tcp-packets-with-errors-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Packets with Errors Received

SNMP NAME	tcpInErrs

SNMP PATH	tcp 14

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of segments received in error

(e.g., bad TCP checksums).

NAME		tcp-passive-connects-attempts

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Passive Connects Attempts

SNMP NAME	tcpPassiveOpens

SNMP PATH	tcp 6

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times TCP connections have made a

direct transition to the SYN-RCVD state from the

LISTEN state.

NAME		tcp-receive-buffer-size

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP=>Receive Buffer Size

SNMP NAME	ncdTcpRecvBufferSize

SNMP PATH	ncdTcp 12

TYPE		INTEGER

MIN		1024

DEFAULT		4096

MAX		9216

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the maximum amount of received data in bytes that a TCP

connection will buffer in the unit. This corresponds to

the TCP receive window advertised by the unit to the peer.

NAME		tcp-reset-packets-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Reset Packets Transmitted

SNMP NAME	tcpOutRsts

SNMP PATH	tcp 15

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of TCP segments sent containing the RST flag.

NAME		tcp-retransmission-algorithm-type

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP=>Retransmission Algorithm Type

SNMP NAME	tcpRtoAlgorithm

SNMP PATH	tcp 1

TYPE		CHOICE

CHOICES		{ other constant rsre van-jacobson }

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The algorithm used to determine the timeout value

used for retransmitting unacknowledged octets.

NAME		tcp-send-buffer-size

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP=>Send Buffer Size

SNMP NAME	ncdTcpSendBufferSize

SNMP PATH	ncdTcp 13

TYPE		INTEGER

MIN		1024

DEFAULT		2048

MAX		65535

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the maximum amount of data in bytes

awaiting transmission that a TCP connection will buffer

in the unit. This corresponds to the maximum amount of

the peer's TCP send window that will be used by the unit.

GROUP tcpip

NAME		tcpip-arp-cache

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

ARP=>ARP Cache

SNMP NAME	ipNetToMediaTable

SNMP PATH	ip 22

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the ARP table which is used to map from IP addresses

to media dependent physical addresses.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ipNetToMediaIfIndex

SNMP PATH	ipNetToMediaTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Interface index for each entry equivalence

NAME		ethernet-address

SNMP NAME	ipNetToMediaPhysAddress

SNMP PATH	ipNetToMediaTable 2

TYPE		STRING

DEFAULT		00:00:00:00:00:00

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the physical address (Ethernet).

NAME		ip-address

SNMP NAME	ipNetToMediaNetAddress

SNMP PATH	ipNetToMediaTable 3

TYPE		STRING

DEFAULT		0.0.0.0

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the IP address.

NAME		type

SNMP NAME	ipNetToMediaType

SNMP PATH	ipNetToMediaTable 4

TYPE		CHOICE

CHOICES		{ incomplete dynamic static }

DEFAULT		incomplete

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the state of the entry. A static

entry is one that is not subject to the usual

automatic deletion process.

NAME		time-since-last-use

SNMP NAME	ncdIpTimeSinceLastUse

SNMP PATH	ipNetToMediaTable 13

TYPE		INTEGER

MIN		0

DEFAULT		0

MAX		255

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the last time (in minutes) the entry

was used by the unit.

END TABLE ROW ENTRIES

NAME		tcpip-arp-complete-entry-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

ARP=>Complete Entry Timeout

SNMP NAME	ncdArpCompleteEntryTimeout

SNMP PATH	ncdArp 2

TYPE		INTEGER

MIN		1

DEFAULT		20

MAX		255

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies how long (in minutes) a completed ARP table entry should

be allowed to exist without being used before it is

automatically deleted.

NAME		tcpip-arp-incomplete-entry-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

ARP=>Incomplete Entry Timeout

SNMP NAME	ncdArpIncompleteEntryTimeout

SNMP PATH	ncdArp 1

TYPE		INTEGER

MIN		1

DEFAULT		1

MAX		255

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies how long (in minutes) an incomplete ARP table

entry should be allowed to exist without being used before it is

automatically deleted. Incomplete entries exist when

an IP address cannot be resolved to a Ethernet address.

NAME		tcpip-dns-default-domain

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP/IP Name Service=>DNS Default Domain

SNMP NAME	ncdNameResolverDnsDefaultDomain

SNMP PATH	ncdNameResolver 19

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the default domain to use with DNS

name service requests.

NAME		tcpip-dns-domain-search

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP/IP Name Service=>DNS Domain Search

SNMP NAME	ncdNameResolverDnsDoNameSearch

SNMP PATH	ncdNameResolver 20

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that when attempting to resolve a name

with DNS try successively wider scopes of the name

space. i.e. if the default domain is eng.ncd.com,

and the host name is hansen, then we first try

hansen.eng.ncd.com., and then hansen.ncd.com.

NAME		tcpip-name-bad-name-errors

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP/IP Name Service=>Bad Name Errors

SNMP NAME	ncdNameResolverBadNameErrs

SNMP PATH	ncdNameResolver 7

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of 'Bad Name' errors returned by the

TCP/IP name servers for name requests. This indicates

a badly formed name.

NAME		tcpip-name-cache-hits

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP/IP Name Service=>Name Cache Hits

SNMP NAME	ncdNameResolverCacheHits

SNMP PATH	ncdNameResolver 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of invocations where the answer

was found in the local TCP/IP name cache.

NAME		tcpip-name-cache-ignore-case

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP/IP Name Service=>Name Cache Ignore Case

SNMP NAME	ncdNameResolverNameCacheIgnoreCase

SNMP PATH	ncdNameResolver 21

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether case should be ignored when searching for a name

in the local name cache.

NAME		tcpip-name-cache-max-lifetime

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP/IP Name Service=>Name Cache Max Lifetime

SNMP NAME	ncdNameResolverNameCacheMaxTimeout

SNMP PATH	ncdNameResolver 13

TYPE		INTEGER

MIN		0

DEFAULT		1800

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the maximum amount of time (in seconds) that a

TCP/IP name service entry should be used before it

is automatically deleted.

NAME		tcpip-name-cache-max-size

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP/IP Name Service=>Name Cache Max Size

SNMP NAME	ncdNameResolverNameCacheMaxSize

SNMP PATH	ncdNameResolver 12

TYPE		INTEGER

MIN		0

DEFAULT		32

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the maximum number of entries in the TCP/IP

name cache.

NAME		tcpip-name-cache-overflows

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP/IP Name Service=>Name Cache Overflows

SNMP NAME	ncdNameResolverCacheOverflows

SNMP PATH	ncdNameResolver 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times a valid entry in the local TCP/IP name cache

had to be deleted to make room for a new entry.

NAME		tcpip-name-local-cache

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP/IP Name Service=>Local Name Cache

SNMP NAME	ncdNameResolverNameCacheTable

SNMP PATH	ncdNameResolver 14

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the TCP/IP name service cache.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdNameResolverTableIndex

SNMP PATH	ncdNameResolverNameCacheTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the index into the IP Local Name Cache Table

NAME		name

SNMP NAME	ncdNameResolverTableName

SNMP PATH	ncdNameResolverNameCacheTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the symbolic name of the host.

NAME		address

SNMP NAME	ncdNameResolverTableAddress

SNMP PATH	ncdNameResolverNameCacheTable 3

TYPE		STRING

DEFAULT		0.0.0.0

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the IP address(es) of the host.

NAME		lifetime

SNMP NAME	ncdNameResolverTableExpTime

SNMP PATH	ncdNameResolverNameCacheTable 4

TYPE		INTEGER

MIN		0

DEFAULT		0

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the time (relative to the unit boot time

in seconds) at which the entry will become invalid.

NB: Zero (0) indicates an infinite lifetime

END TABLE ROW ENTRIES

NAME		tcpip-name-memory-allocation-failures

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP/IP Name Service=>Memory Allocation Failures

SNMP NAME	ncdNameResolverNoMemErrs

SNMP PATH	ncdNameResolver 11

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times the TCP/IP name resolver

was unable to allocated required memory.

NAME		tcpip-name-no-server-responses

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP/IP Name Service=>No Server Responses

SNMP NAME	ncdNameResolverNoRspErrs

SNMP PATH	ncdNameResolver 9

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times no response was received from a

TCP/IP name server.

NAME		tcpip-name-no-such-name-errors

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP/IP Name Service=>No Such Name Errors

SNMP NAME	ncdNameResolverNoNameErrs

SNMP PATH	ncdNameResolver 6

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of 'No Such Name' errors returned by the

TCP/IP name servers for name requests.

NAME		tcpip-name-numerical-addresses

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP/IP Name Service=>Numerical Addresses

SNMP NAME	ncdNameResolverNumericalAddrs

SNMP PATH	ncdNameResolver 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of invocations where the TCP/IP name was

in numerical format, e.g. 192.43.154.7.

NAME		tcpip-name-requests

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP/IP Name Service=>Name Requests

SNMP NAME	ncdNameResolverRequests

SNMP PATH	ncdNameResolver 17

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times the TCP/IP name resolver

was invoked.

NAME		tcpip-name-server-hits

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP/IP Name Service=>Name Server Hits

SNMP NAME	ncdNameResolverServerHits

SNMP PATH	ncdNameResolver 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of invocations where the answer

was found by consulting the TCP/IP name servers.

NAME		tcpip-name-server-protocol

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP/IP Name Service=>Name Server Protocol

SNMP NAME	ncdNameResolverProtocol

SNMP PATH	ncdNameResolver 15

TYPE		CHOICE

CHOICES		{ dns ien-116 both }

DEFAULT		ien-116

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the TCP/IP name service protocol to use

when contacting the name servers.

NAME		tcpip-name-server-response-errors

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP/IP Name Service=>Name Server Response Errors

SNMP NAME	ncdNameResolverRspErrs

SNMP PATH	ncdNameResolver 8

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of badly formed responses returned by the

TCP/IP name servers for name requests.

NAME		tcpip-name-server-retransmission-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP/IP Name Service=>Name Server Retransmission Timeout

SNMP NAME	ncdNameResolverRtxTimeout

SNMP PATH	ncdNameResolver 16

TYPE		INTEGER

MIN		1

DEFAULT		2

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) between

retransmissions of a TCP/IP name service

request.

NAME		tcpip-name-server-transaction-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP/IP Name Service=>Name Server Transaction Timeout

SNMP NAME	ncdNameResolverTransactTimeout

SNMP PATH	ncdNameResolver 1

TYPE		INTEGER

MIN		1

DEFAULT		10

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) to attempt

a TCP/IP name service request before a failure

situation is declared.

NAME		tcpip-name-servers

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

TCP/IP Name Service=>Name Servers

SNMP NAME	ncdNameResolverNameServersTable

SNMP PATH	ncdNameResolver 18

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the list of TCP/IP name servers.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdInetNameServersTableIndex

SNMP PATH	ncdNameResolverNameServersTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the index to the IP Name Servers Table

NAME		server

SNMP NAME	ncdInetNameServersTableAddress

SNMP PATH	ncdNameResolverNameServersTable 2

TYPE		STRING

DEFAULT		0.0.0.0

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the IP address of the name server.

END TABLE ROW ENTRIES

NAME		tcpip-name-system-errors

HIDEBOX		Console=>Statistics=>Show Statistics=>

TCP/IP Name Service=>System Errors

SNMP NAME	ncdNameResolverSysErrs

SNMP PATH	ncdNameResolver 10

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times the TCP/IP name resolver

encountered an general system error.

GROUP term

NAME		term-default-connect-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

VT320 Terminal Emulation=>Terminal Default Connect Timeout

SNMP NAME	ncdTermDfltConnectTimeout

SNMP PATH	ncdTerm 4

TYPE		INTEGER

MIN		0

DEFAULT		20

MAX		600

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the default amount of time (in seconds) that

the terminal emulator should attempt to establish a

connection before declaring an error situation.

NAME		term-default-hosts

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

VT320 Terminal Emulation=>Default Hosts

SNMP NAME	ncdTermDfltConnectionTable

SNMP PATH	ncdTerm 1

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the default list of hosts to present

to the user in the terminal chooser.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdTermDCTIndex

SNMP PATH	ncdTermDfltConnectionTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Default Hosts table for NCDterm

NAME		protocol

SNMP NAME	ncdTermDCTProtocol

SNMP PATH	ncdTermDfltConnectionTable 2

TYPE		CHOICE

CHOICES		{ telnet lat cterm serial }

DEFAULT		telnet

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the protocol to use.

NAME		host

SNMP NAME	ncdTermDCTHost

SNMP PATH	ncdTermDfltConnectionTable 3

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the name of the terminal host.

NAME		description

SNMP NAME	ncdTermDCTDescription

SNMP PATH	ncdTermDfltConnectionTable 4

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies a description of the terminal host.

END TABLE ROW ENTRIES

NAME		term-delay-lat-display

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

VT320 Terminal Emulation=>Delay LAT Display after Boot

SNMP NAME	ncdTermDelayLATDisplay

SNMP PATH	ncdTerm 5

TYPE		INTEGER

MIN		0

DEFAULT		3

MAX		120

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) to wait after

reboot to display the terminal chooser lat services.  This delay is

useful to allow time for the LAT service directory to be

filled.

NAME		term-restrict-host-choices

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

VT320 Terminal Emulation=>Restrict Host Choices

SNMP NAME	ncdTermRestrictConnectionChoices

SNMP PATH	ncdTerm 2

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that the user should only be allowed

to choose a terminal host specified in the default

terminal hosts list.

GROUP tftp

NAME		tftp-ack-packets-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

TFTP=>Ack Packets Received

SNMP NAME	ncdTftpClientInAcks

SNMP PATH	ncdTftp 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of TFTP Ack packets received by the TFTP client.

NAME		tftp-ack-packets-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

TFTP=>Ack Packets Transmitted

SNMP NAME	ncdTftpClientOutAcks

SNMP PATH	ncdTftp 10

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of TFTP Ack packets sent by the TFTP client.

NAME		tftp-bad-access-error-responses-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

TFTP=>Bad Access Error Responses Received

SNMP NAME	ncdTftpClientInAccessErrors

SNMP PATH	ncdTftp 6

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times an TFTP Error packet was received

by the TFTP client with the error code set to

'Access violation.'

NAME		tftp-data-packets-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

TFTP=>Data Packets Received

SNMP NAME	ncdTftpClientInDataPkts

SNMP PATH	ncdTftp 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of TFTP Data packets received by the TFTP client.

NAME		tftp-data-packets-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

TFTP=>Data Packets Transmitted

SNMP NAME	ncdTftpClientOutDataPkts

SNMP PATH	ncdTftp 9

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of TFTP Data packets sent by the TFTP client.

NAME		tftp-disk-full-error-responses-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

TFTP=>Disk Full Error Responses Received

SNMP NAME	ncdTftpClientInDiskFullErrors

SNMP PATH	ncdTftp 7

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times an TFTP Error packet was received

by the TFTP client with the error code set to 'Disk full'.

NAME		tftp-file-read-requests-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

TFTP=>File Read Requests Transmitted

SNMP NAME	ncdTftpClientReadReqs

SNMP PATH	ncdTftp 1

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of file read requests made by the TFTP client.

NAME		tftp-file-write-requests-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

TFTP=>File Write Requests Transmitted

SNMP NAME	ncdTftpClientWriteReqs

SNMP PATH	ncdTftp 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of file write requests made by the TFTP client.

NAME		tftp-no-available-buffer-errors

HIDEBOX		Console=>Statistics=>Show Statistics=>

TFTP=>No Available Buffer Errors

SNMP NAME	ncdTftpClientNoBufs

SNMP PATH	ncdTftp 8

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times the TFTP client failed to allocate

a buffer.

NAME		tftp-no-file-error-responses-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

TFTP=>No File Error Responses Received

SNMP NAME	ncdTftpClientInNoFileErrors

SNMP PATH	ncdTftp 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times an TFTP Error packet was received

by the TFTP client with the error code set to

'File not found'.

NAME		tftp-packets-retransmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

TFTP=>Packets Retransmitted

SNMP NAME	ncdTftpClientRetrans

SNMP PATH	ncdTftp 11

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times a TFTP packet was retransmitted by

the TFTP client.

GROUP tokring

NAME		tokring-interface-table

HIDEBOX		Console=>Statistics=>Show Statistics=>

Token Ring=>Interface Table

SNMP NAME	dot5Table

SNMP PATH	dot5 1

TYPE		TABLE

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

A list of Token Ring status and parameter

values for an 802.5 interface.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	dot5IfIndex

SNMP PATH	dot5Table 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The value of this object identifies the

802.5 interface for which this entry

contains management information.

NAME		commands

SNMP NAME	dot5Commands

SNMP PATH	dot5Table 2

TYPE		CHOICE

CHOICES		{ none open reset close }

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

This object may be written with SNMP

to open, close and reset the interface.

NAME		ring-status

SNMP NAME	dot5RingStatus

SNMP PATH	dot5Table 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The current interface status which can

be used to diagnose fluctuating problems

that can occur on token rings, after a

station has successfully been added to

the ring.

Before an open is completed, this

object has the value for the 'no status'

condition.  The dot5RingState and

dot5RingOpenStatus objects provide for

debugging problems when the station

can not even enter the ring.

The object's value is a sum of

values, one for each currently applicable

condition.  The following values are

defined for various conditions:

0 = No Problems detected

32 = Ring Recovery

64 = Single Station

256 = Remove Received

512 = reserved

1024 = Auto-Removal Error

2048 = Lobe Wire Fault

4096 = Transmit Beacon

8192 = Soft Error

16384 = Hard Error

32768 = Signal Loss

131072 = no status, open not completed.

NAME		ring-state

SNMP NAME	dot5RingState

SNMP PATH	dot5Table 4

TYPE		CHOICE

CHOICES		{ opened closed opening closing open-failure ring-failure }

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The current interface state with respect

to entering or leaving the ring.

NAME		last-open-status

SNMP NAME	dot5RingOpenStatus

SNMP PATH	dot5Table 5

TYPE		CHOICE

CHOICES		{ none unknown cable-fault signal-loss timeout ring-failure beaconing duplicate-addr request-failed remove-received successful }

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

This object indicates the success, or the

reason for failure, of the station's most

recent attempt to enter the ring.

NAME		ring-speed

SNMP NAME	dot5RingSpeed

SNMP PATH	dot5Table 6

TYPE		CHOICE

CHOICES		{ unknown 1000000 4000000 16000000 }

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The speed or bandwidth of the ring

specified in (bits/second).

NAME		upstream-neighbor

SNMP NAME	dot5UpStream

SNMP PATH	dot5Table 7

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The MAC-address of the nearest active

up stream neighbor station in the ring.

NAME		active-monitor-participate

SNMP NAME	dot5ActMonParticipate

SNMP PATH	dot5Table 8

TYPE		CHOICE

CHOICES		{ true false }

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Whether or not this station will participate

in the active monitor selection process

NAME		functional-addresses

SNMP NAME	dot5Functional

SNMP PATH	dot5Table 9

TYPE		CHOICE

CHOICES		{ none }

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The bit mask of all Token Ring functional

addresses for which this interface will

accept frames.

END TABLE ROW ENTRIES

NAME		tokring-statistics-table

HIDEBOX		Console=>Statistics=>Show Statistics=>

Token Ring=>Statistics Table

SNMP NAME	dot5StatsTable

SNMP PATH	dot5 2

TYPE		TABLE

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

A table containing Token Ring statistics,

one entry per 802.5 interface.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	dot5StatsIfIndex

SNMP PATH	dot5StatsTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The value of this object identifies the

802.5 interface for which this entry

contains management information.

NAME		line-errors

SNMP NAME	dot5StatsLineErrors

SNMP PATH	dot5StatsTable 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

This counter is incremented when a frame

or token is copied or repeated by a

station, the E bit is zero in the frame

or token and one of the following

conditions exists: 1) there is a

non-data bit (J or K bit) between the SD

and the ED of the frame or token, or

2) there is an FCS error in the frame.

NAME		burst-errors

SNMP NAME	dot5StatsBurstErrors

SNMP PATH	dot5StatsTable 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

This counter is incremented when a station

detects the absence of transitions for five

half-bit timers (burst-five error).

NAME		ac-errors

SNMP NAME	dot5StatsACErrors

SNMP PATH	dot5StatsTable 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

This counter is incremented when a station

receives an AMP or SMP frame in which A is

equal to C is equal to 0, and then receives

another SMP frame with A is equal to C is

equal to 0 without first receiving an AMP

frame. It denotes a station that cannot set

the AC bits properly.

NAME		aborted-transmissions

SNMP NAME	dot5StatsAbortTransErrors

SNMP PATH	dot5StatsTable 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

This counter is incremented when a station

transmits an abort delimiter while

transmitting.

NAME		internal-errors

SNMP NAME	dot5StatsInternalErrors

SNMP PATH	dot5StatsTable 6

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

This counter is incremented when a station

recognizes an internal error.

NAME		lost-frames

SNMP NAME	dot5StatsLostFrameErrors

SNMP PATH	dot5StatsTable 7

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

This counter is incremented when a station

is transmitting and its TRR timer expires.

This condition denotes a condition where a

transmitting station in strip mode does not

receive the trailer of the frame before the

TRR timer goes off.

NAME		receive-congestions

SNMP NAME	dot5StatsReceiveCongestions

SNMP PATH	dot5StatsTable 8

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

This counter is incremented when a station

recognizes a frame addressed to its

specific address, but has no available

buffer space indicating that the station

is congested.

NAME		frame-copied-errors

SNMP NAME	dot5StatsFrameCopiedErrors

SNMP PATH	dot5StatsTable 9

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

This counter is incremented when a station

recognizes a frame addressed to its

specific address and detects that the FS

field A bits are set to 1 indicating a

possible line hit or duplicate address.

NAME		token-errors

SNMP NAME	dot5StatsTokenErrors

SNMP PATH	dot5StatsTable 10

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

This counter is incremented when a station

acting as the active monitor recognizes an

error condition that needs a token

transmitted.

NAME		soft-errors

SNMP NAME	dot5StatsSoftErrors

SNMP PATH	dot5StatsTable 11

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of Soft Errors the interface

has detected. It directly corresponds to

the number of Report Error MAC frames

that this interface has transmitted.

Soft Errors are those which are

recoverable by the MAC layer protocols.

NAME		hard-errors

SNMP NAME	dot5StatsHardErrors

SNMP PATH	dot5StatsTable 12

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times this interface has

detected an immediately recoverable

fatal error.  It denotes the number of

times this interface is either

transmitting or receiving beacon MAC

frames.

NAME		signal-loss

SNMP NAME	dot5StatsSignalLossErrors

SNMP PATH	dot5StatsTable 13

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times this interface has

detected the loss of signal condition from

the ring.

NAME		transmit-beacons

SNMP NAME	dot5StatsTransmitBeacons

SNMP PATH	dot5StatsTable 14

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times this interface has

transmitted a beacon frame.

NAME		ring-recoveries

SNMP NAME	dot5StatsRecoveries

SNMP PATH	dot5StatsTable 15

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of Claim Token MAC frames

received or transmitted after the interface

has received a Ring Purge MAC frame.  This

counter signifies the number of times the

ring has been purged and is being recovered

back into a normal operating state.

NAME		cable-faults

SNMP NAME	dot5StatsLobeWires

SNMP PATH	dot5StatsTable 16

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times the interface has

detected an open or short circuit in the

lobe data path.  The adapter will be closed

and dot5RingState will signify this

condition.

NAME		removes-received

SNMP NAME	dot5StatsRemoves

SNMP PATH	dot5StatsTable 17

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times the interface has

received a Remove Ring Station MAC frame

request.  When this frame is received

the interface will enter the close state

and dot5RingState will signify this

condition.

NAME		single-stations

SNMP NAME	dot5StatsSingles

SNMP PATH	dot5StatsTable 18

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times the interface has

sensed that it is the only station on the

ring.  This will happen if the interface

is the first one up on a ring, or if

there is a hardware problem.

NAME		frequency-errors

SNMP NAME	dot5StatsFreqErrors

SNMP PATH	dot5StatsTable 19

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of times the interface has

detected that the frequency of the

incoming signal differs from the expected

frequency by more than that specified by

the IEEE 802.5 standard.

END TABLE ROW ENTRIES

GROUP pwireless

NAME		pwireless-interface-table

HIDEBOX		Console=>Statistics=>Show Statistics=>

Wireless LAN=>Interface Table

SNMP NAME	ncdWirelessTable

SNMP PATH	ncdWireless 1

TYPE		TABLE

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

A list of Token Ring status and parameter

values for an 802.5 interface.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdWirelessIfIndex

SNMP PATH	ncdWirelessTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The value of this object identifies the

wireless interface for which this entry

contains management information.

NAME		domain

SNMP NAME	ncdWirelessDomain

SNMP PATH	ncdWirelessTable 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

This object specifies the domain used

by the wireless PCMCIA card.

NAME		master-name

SNMP NAME	ncdWirelessMasterName

SNMP PATH	ncdWirelessTable 3

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The current master name.

NAME		master-node-id

SNMP NAME	ncdWirelessMasterNode

SNMP PATH	ncdWirelessTable 4

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The current master node.

NAME		channel

SNMP NAME	ncdWirelessChannel

SNMP PATH	ncdWirelessTable 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

This object indicates the current

channel.

NAME		subchannel

SNMP NAME	ncdWirelessSubChannel

SNMP PATH	ncdWirelessTable 6

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

This object indicates the current

sub channel.

NAME		rom-version

SNMP NAME	ncdWirelessROMVersion

SNMP PATH	ncdWirelessTable 7

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The ROM Version of the PCMCIA Card.

NAME		country-code

SNMP NAME	ncdWirelessCountryCode

SNMP PATH	ncdWirelessTable 8

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

This object indicates the Country Code.

NAME		signal-strength

SNMP NAME	ncdWirelessSignalStrength

SNMP PATH	ncdWirelessTable 9

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

This object indicates the Signal

Strength of the last packet received.

END TABLE ROW ENTRIES

NAME		pwireless-statistics-table

HIDEBOX		Console=>Statistics=>Show Statistics=>

Wireless LAN=>Statistics Table

SNMP NAME	ncdWirelessStatsTable

SNMP PATH	ncdWireless 2

TYPE		TABLE

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

A table containing Wireless statistics,

one entry per interface.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdWirelessStatsIfIndex

SNMP PATH	ncdWirelessStatsTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The value of this object identifies the

wireless interface for which this entry

contains management information.

NAME		number-of-hops

SNMP NAME	ncdWirelessStatsHops

SNMP PATH	ncdWirelessStatsTable 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

16 bit counter indicating the number of

hop periods.

NAME		number-of-searches

SNMP NAME	ncdWirelessStatsSearches

SNMP PATH	ncdWirelessStatsTable 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

16 bit counter indicating the number of

searches for other Masters performed.

NAME		times-master

SNMP NAME	ncdWirelessStatsTimesMaster

SNMP PATH	ncdWirelessStatsTable 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

16 bit counter indicating the number of

times the terminal has been Master.

NAME		times-resync

SNMP NAME	ncdWirelessStatsTimesResync

SNMP PATH	ncdWirelessStatsTable 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

16 bit counter indicating the number of

times the terminal synchronized to a new

Master.

NAME		missed-syncs

SNMP NAME	ncdWirelessStatsMissedSyncs

SNMP PATH	ncdWirelessStatsTable 6

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

16 bit counter indicating the number of

sync messages missed.

NAME		rx-packets

SNMP NAME	ncdWirelessStatsRXPackets

SNMP PATH	ncdWirelessStatsTable 7

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

16 bit counter indicating the number of

data packets received.

NAME		tx-packets

SNMP NAME	ncdWirelessStatsLostTXPackets

SNMP PATH	ncdWirelessStatsTable 8

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

16 bit counter indicating the number of

data packets transmitted.

NAME		lost-cts

SNMP NAME	ncdWirelessStatsLostCTS

SNMP PATH	ncdWirelessStatsTable 9

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

16 bit counter indicating the number of

times RTS was sent without recieving a CTS.

NAME		cts-errors

SNMP NAME	ncdWirelessStatsCTSErrors

SNMP PATH	ncdWirelessStatsTable 10

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

16 bit counter indicating the number of

times CTS retries expired.

NAME		ack-errors

SNMP NAME	ncdWirelessStatsACKErrors

SNMP PATH	ncdWirelessStatsTable 11

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

16 bit counter indicating the number of

times ACT retries expired.

NAME		sid-mismatch

SNMP NAME	ncdWirelessStatsSIDMismatch

SNMP PATH	ncdWirelessStatsTable 12

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

16 bit counter indicating the number of

security ID mismatched.

NAME		missed-sobs

SNMP NAME	ncdWirelessStatsMissedSOBs

SNMP PATH	ncdWirelessStatsTable 13

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

16 bit counter indicating the number of

Start of B messages missed.

NAME		corrupted-packets

SNMP NAME	ncdWirelessStatsCorruptedPackets

SNMP PATH	ncdWirelessStatsTable 14

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

16 bit counter indicating the number of

data packets passing CRC, but not length

check.

NAME		adapter-resets

SNMP NAME	ncdWirelessStatsAdapterReset

SNMP PATH	ncdWirelessStatsTable 15

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

32 bit counter indicating the number of

times the wireless adapter needed to reset

for initialization or error recovery.

NAME		send-timeouts

SNMP NAME	ncdWirelessStatsSendTimeouts

SNMP PATH	ncdWirelessStatsTable 16

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

32 bit counter indicating the number of

transmit timeouts.

NAME		receive-timeouts

SNMP NAME	ncdWirelessStatsReceiveTimeouts

SNMP PATH	ncdWirelessStatsTable 17

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

32 bit counter indicating the number of

receive timeouts.

NAME		antenna-status-changes

SNMP NAME	ncdWirelessStatsAntennaStatusChange

SNMP PATH	ncdWirelessStatsTable 18

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

32 bit counter indicating the number of

antenna status change (connect/disconnect)

interrupts received.

NAME		roam-alarms

SNMP NAME	ncdWirelessStatsRoamAlarms

SNMP PATH	ncdWirelessStatsTable 19

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

32 bit counter indicating the number of

roaming alarm packets recevied, which

indicates that the signal strength is

weak.

NAME		in-syncs

SNMP NAME	ncdWirelessStatsInSyncs

SNMP PATH	ncdWirelessStatsTable 20

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

32 bit counter indicating the number of

times the terminal was able to synchronize

to a Master station.

NAME		out-syncs

SNMP NAME	ncdWirelessStatsOutSyncs

SNMP PATH	ncdWirelessStatsTable 21

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

32 bit counter indicating the number of

times the terminal has lost contact with

the Master station.

END TABLE ROW ENTRIES

GROUP udp

NAME		udp-listener-table

HIDEBOX		Console=>Statistics=>Show Statistics=>

UDP=>Listener Table

SNMP NAME	udpTable

SNMP PATH	udp 5

TYPE		TABLE

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

A table containing UDP listener information.

BEGIN TABLE ROW ENTRIES

NAME		local-ip-address

SNMP NAME	udpLocalAddress

SNMP PATH	udpTable 1

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The local IP address for this UDP listener.  In

the case of a UDP listener which is willing to

accept datagrams for any IP interface associated

with the node, the value 0.0.0.0 is used.

NAME		local-port

SNMP NAME	udpLocalPort

SNMP PATH	udpTable 2

TYPE		INTEGER

MIN		0

MAX		65535

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The local port number for this UDP listener.

END TABLE ROW ENTRIES

NAME		udp-packets-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

UDP=>Packets Received

SNMP NAME	udpInDatagrams

SNMP PATH	udp 1

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of UDP datagrams delivered to UDP applications.

NAME		udp-packets-received-with-errors

HIDEBOX		Console=>Statistics=>Show Statistics=>

UDP=>Packets Received with Errors

SNMP NAME	udpInErrors

SNMP PATH	udp 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of received UDP datagrams that could

not be delivered for reasons other than the lack

of an application at the destination port.

NAME		udp-packets-received-with-unknown-port

HIDEBOX		Console=>Statistics=>Show Statistics=>

UDP=>Packets Received with Unknown Port

SNMP NAME	udpNoPorts

SNMP PATH	udp 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of received UDP datagrams for

which there was no application at the destination port.

NAME		udp-packets-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

UDP=>Packets Transmitted

SNMP NAME	udpOutDatagrams

SNMP PATH	udp 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of UDP datagrams sent from this entity.

GROUP unit

NAME		unit-administrative-status

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Unit=>Administrative Status

SNMP NAME	ncdSysAdminStatus

SNMP PATH	ncdSystem 7

TYPE		CHOICE

CHOICES		{ running session-reset last-client-close-reset unit-reset }

DEFAULT		running

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

The administrative status of the unit.  Options are:

Running                 : Unit is running; no resets pending

Session Reset           : Session reset pending

Last Client Close Reset : Reset on last client close pending

Unit Reset              : Unit reset pending

NAME		unit-blocks-of-available-memory

HIDEBOX		Console=>Statistics=>Show Statistics=>

Unit=>Blocks of Available Memory

SNMP NAME	ncdSysMemFrags

SNMP PATH	ncdSystem 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of discontiguous blocks of unallocated memory.

NAME		unit-boot-monitor-version

HIDEBOX		Console=>Statistics=>Show Statistics=>

Unit=>Boot Monitor Version

SNMP NAME	ncdSysBootPromVersion

SNMP PATH	ncdSystem 5

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

A textual description of the boot PROM's installed

in the unit.

NAME		unit-code-memory-installed

HIDEBOX		Console=>Statistics=>Show Statistics=>

Unit=>Code Memory Installed

SNMP NAME	ncdSysMemCode

SNMP PATH	ncdSystem 8

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total amount of code RAM in bytes installed in the system.

NAME		unit-contact

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Unit=>Administrative Contact

SNMP NAME	sysContact

SNMP PATH	system 4

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the administrative contact for the unit.

This is provided as a convience for

system administration personnel.

NAME		unit-floating-point-exceptions

HIDEBOX		Console=>Statistics=>Show Statistics=>

Unit=>Floating point exceptions

SNMP NAME	ncdSysFPEtotal

SNMP PATH	ncdSystem 15

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total number of floating point exceptions

NAME		unit-global-password

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Unit Global Password

SNMP NAME	ncdAccessGlobalPasswd

SNMP PATH	ncdAccess 38

TYPE		STRING

DEFAULT		nil

PERMISSION	WRITE-ONLY

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the password required to obtain read-write access

to the Config, Pref, SNMP, and File Daemons.

NAME		unit-idle-time

HIDEBOX		Console=>Statistics=>Show Statistics=>

Unit=>Idle Time

SNMP NAME	ncdSysIdleTime

SNMP PATH	ncdSystem 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The estimated amount of time (in seconds) since boot

that the system was not executing any useful task.

NAME		unit-keyboard-controller-version

HIDEBOX		Console=>Statistics=>Show Statistics=>

Unit=>Keyboard Controller Version

SNMP NAME	ncdSysKeyboardControllerVersion

SNMP PATH	ncdSystem 6

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

A textual description of the keyboard controller

installed in the unit.

NAME		unit-license-agent-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Licenses=>License Agent Port Number

SNMP NAME	ncdSysLicenseAgentPort

SNMP PATH	ncdSystem 17

TYPE		INTEGER

MIN		0

DEFAULT		5129

MAX		65536

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the UDP port on which the license agent is listening.

The terminal will send messages to this port to get licenses.

NAME		unit-license-agent-retries

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Licenses=>Maximum Retries

SNMP NAME	ncdSysLicenseAgentRetries

SNMP PATH	ncdSystem 19

TYPE		INTEGER

MIN		1

DEFAULT		3

MAX		100

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the maximum number of attempts the terminal should make

to get a license from the license agent.

NAME		unit-license-agent-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Licenses=>Transaction Timeout

SNMP NAME	ncdSysLicenseAgentTimeout

SNMP PATH	ncdSystem 18

TYPE		INTEGER

MIN		1

DEFAULT		2

MAX		100

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the maximum amount of time the terminal should wait

for a reply from the license agent.

NAME		unit-license-agents

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Licenses=>License Agents

SNMP NAME	ncdSysLicenseAgentTable

SNMP PATH	ncdSystem 16

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the list of IP hosts where an NCD license agent

is running.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdSysLicenseAgentTableIndex

SNMP PATH	ncdSysLicenseAgentTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the license agents table

NAME		host

SNMP NAME	ncdSysLicenseAgentTableAddress

SNMP PATH	ncdSysLicenseAgentTable 2

TYPE		STRING

DEFAULT		0.0.0.0

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the IP host where the license agent is running.

END TABLE ROW ENTRIES

NAME		unit-license-key

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Licenses=>License Key

SNMP NAME	ncdSysLicenseKey

SNMP PATH	ncdSystem 11

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

A license key that defines the enabled

optional licensed software features.

NAME		unit-licensed-features

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Licenses=>Licensed Features

SNMP NAME	ncdSysLicensedFeatures

SNMP PATH	ncdSystem 12

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The list of software features that are licensed

currently enabled for this unit.

NAME		unit-location

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Unit=>Physical Location

SNMP NAME	sysLocation

SNMP PATH	system 6

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the physical location of the unit.

This is provided as a convience for

system administration personnel.

NAME		unit-low-memory-level

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Unit=>Show Low Memory Warning Box at  (bytes)

SNMP NAME	ncdSysLowMemWarningThreshold

SNMP PATH	ncdSystem 10

TYPE		INTEGER

MIN		0

DEFAULT		50000

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Resets the threshold (in bytes) for displaying the

Low on Memory warning box. If free memory drops below

this value, the warning box will appear with an

appropriate message.

NAME		unit-memory-available

HIDEBOX		Console=>Statistics=>Show Statistics=>

Unit=>Memory Available

SNMP NAME	ncdSysMemAvail

SNMP PATH	ncdSystem 2

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The amount of RAM memory in bytes which is currently unallocated.

NAME		unit-memory-installed

HIDEBOX		Console=>Statistics=>Show Statistics=>

Unit=>Memory Installed

SNMP NAME	ncdSysMemTotal

SNMP PATH	ncdSystem 1

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The total amount of RAM in bytes installed in the system.

This does not include special purpose RAM such as video memory.

NAME		unit-name

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Unit=>Unit Name

SNMP NAME	sysName

SNMP PATH	system 5

TYPE		STRING

DEFAULT		unknown

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the symbolic name of the NCD unit.

NAME		unit-optional-features

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Licenses=>Optional Features

SNMP NAME	ncdSysOptionalFeatures

SNMP PATH	ncdSystem 13

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The list of optional software features that

are currently enabled for this unit.

NAME		unit-query-for-name-at-boot

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Unit=>Query for Unit Name at Boot

SNMP NAME	ncdSysQueryNameAtBoot

SNMP PATH	ncdSystem 9

TYPE		CHOICE

CHOICES		{ none tcpip ncdnet }

DEFAULT		none

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies that the unit should send a reverse name

request at boot to the name servers in an

attempt to determine the unit's symbolic name.

Choices:

None - Do not try reverse query.

TCPIP - Try reverse query to TCP/IP name servers,

if none are defined try the TCPIP boot server if

that is defined. Note TCPIP name server protocol

must be DNS for query to succeed. IEN-116 does not

support reverse queries.

NCDnet - Try reverse query to MOP boot server if

that is defined.

NAME		unit-snmp-system-object-id

HIDEBOX		Console=>Statistics=>Show Statistics=>

Unit=>SNMP System Object Id

SNMP NAME	sysObjectID

SNMP PATH	system 2

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The authoritative identification of the SNMP

network management subsystem contained in the entity.

NAME		unit-tcpip-service-level

HIDEBOX		Console=>Statistics=>Show Statistics=>

Unit=>TCP/IP Service Level

SNMP NAME	sysServices

SNMP PATH	system 7

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

A value which indicates the set of network services that

this entity primarily offers. For NCD units, the value is

always 72.

The value is a sum.  This sum initially takes the

value zero, Then, for each layer, L, in the range

1 through 7, that this node performs transactions

for, 2 raised to (L - 1) is added to the sum.  For

example, a node which performs primarily routing

functions would have a value of 4 (2^(3-1)).  In

contrast, a node which is a host offering

application services would have a value of 72

(2^(4-1) + 2^(7-1)).  Note that in the context of

the Internet suite of protocols, values should be

calculated accordingly:

layer  functionality

1  physical (e.g., repeaters)

2  datalink/subnetwork (e.g., bridges)

3  internet (e.g., IP gateways)

4  end-to-end  (e.g., IP hosts)

7  applications (e.g., mail relays)

For systems including OSI protocols, layers 5 and

6 may also be counted.

NAME		unit-terminal-model

HIDEBOX		Console=>Statistics=>Show Statistics=>

Unit=>Terminal Model

SNMP NAME	ncdSysTermModel

SNMP PATH	ncdSystem 14

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

A textual representation of the current terminal model

NAME		unit-time-since-boot

HIDEBOX		Console=>Statistics=>Show Statistics=>

Unit=>Time Since Boot

SNMP NAME	sysUpTime

SNMP PATH	system 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The time since the system was last booted (in seconds).

NAME		unit-version

HIDEBOX		Console=>Statistics=>Show Statistics=>

Unit=>Version

SNMP NAME	sysDescr

SNMP PATH	system 1

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

A textual description of the entity.  This value

includes the full name and version identification

of the unit's hardware, and software.

GROUP video

NAME		video-fast-mpeg-decoder

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Video=>Fast MPEG decoder(requires license)

SNMP NAME	ncdFastMPEGDecoder

SNMP PATH	ncdVideo 1

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Use the licensed fast MPEG video decoder

NAME		video-starworks-volume-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Video=>Starworks volume file

SNMP NAME	ncdVideoVolumeFile

SNMP PATH	ncdVideo 2

TYPE		STRING

DEFAULT		/usr/lib/X11/ncd/volumes.sw

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Starworks volume information file.

GROUP xremote

NAME		xremote-additional-escape-chars

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

XRemote=>Additional Escape Chars

SNMP NAME	ncdXremoteEscTable

SNMP PATH	ncdXremote 18

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Although the XRemote transport protocol has a pre-defined list of

characters that will be escaped, additional escape characters may be

defined.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdXremoteEscIndex

SNMP PATH	ncdXremoteEscTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the list of additional escape characters when using the

XRemote transport protocol.

NAME		escape-character

SNMP NAME	ncdXremoteEscChar

SNMP PATH	ncdXremoteEscTable 2

TYPE		INTEGER

MIN		0

DEFAULT		0

MAX		255

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

A character which should not be sent in the clear across the serial

line because it will be recognized as

a control character by intervening

software/hardware.  If this character is embedded in the data stream,

it will be translated into an escape sequence.

END TABLE ROW ENTRIES

NAME		xremote-allow-xremotetcp-connection

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

XRemote=>Allow XRemoteTCP Connection

SNMP NAME	ncdXremoteNetEnabled

SNMP PATH	ncdXremote 19

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

If true, an XRemote/TCP connection is permitted,

subject to any host-based access control.

NAME		xremote-authorize-xremote-clients

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

XRemote=>Authorize XRemote Clients

SNMP NAME	ncdXremoteAuthEnabled

SNMP PATH	ncdXremote 17

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

If true, XRemote clients will be authenticated

like any other X client.  If false, no

authentication is performed and any XRemote

client will be allowed access to the X terminal

NAME		xremote-data-compression-mode

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

XRemote=>Data Compression Mode

SNMP NAME	ncdXremoteCompressType

SNMP PATH	ncdXremote 16

TYPE		CHOICE

CHOICES		{ default lzw-14 lzw-12 none }

DEFAULT		default

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies how data in XRemote packets should be

compressed.

NAME		xremote-next-receive-sequence-number

HIDEBOX		Console=>Statistics=>Show Statistics=>

XRemote=>Next Receive Sequence Number

SNMP NAME	ncdXremoteNxtReceiveSequenceNumber

SNMP PATH	ncdXremote 15

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The expected sequence number of the next received

packet.

NAME		xremote-next-transmit-sequence-number

HIDEBOX		Console=>Statistics=>Show Statistics=>

XRemote=>Next Transmit Sequence Number

SNMP NAME	ncdXremoteTransSequenceNumber

SNMP PATH	ncdXremote 12

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The sequence number of the next packet to be

transmitted.

NAME		xremote-oldest-unacknowledged-packet

HIDEBOX		Console=>Statistics=>Show Statistics=>

XRemote=>Oldest Unacknowledged Packet

SNMP NAME	ncdXremoteUnackPkt

SNMP PATH	ncdXremote 13

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The sequence number of the oldest transmitted packet

for which an acknowledgment has not been received.

NAME		xremote-packet-round-trip-time

HIDEBOX		Console=>Statistics=>Show Statistics=>

XRemote=>Packet Round Trip Time

SNMP NAME	ncdXremoteRoundTripTime

SNMP PATH	ncdXremote 9

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The current approximation of the amount of time (in

milliseconds) from when a packet is sent until the

corresponding acknowledgment is received.  The round

trip time is used to compute the time-out length.

NAME		xremote-packet-transmission-errors

HIDEBOX		Console=>Statistics=>Show Statistics=>

XRemote=>Packet Transmission Errors

SNMP NAME	ncdXremotePktTransErrors

SNMP PATH	ncdXremote 4

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of errors encountered while trying to

transmit a packet.

NAME		xremote-packets-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

XRemote=>Packets Received

SNMP NAME	ncdXremotePktsReceived

SNMP PATH	ncdXremote 6

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of XRemote packets received.

NAME		xremote-packets-received-out-of-sequence

HIDEBOX		Console=>Statistics=>Show Statistics=>

XRemote=>Packets Received Out of Sequence

SNMP NAME	ncdXremotePktsOutOfSequence

SNMP PATH	ncdXremote 8

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of XRemote packets received with a bad

sequence number.

NAME		xremote-packets-retransmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

XRemote=>Packets Retransmitted

SNMP NAME	ncdXremotePktsRetrans

SNMP PATH	ncdXremote 5

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of XRemote packets that have been

retransmitted.

NAME		xremote-packets-transmitted

HIDEBOX		Console=>Statistics=>Show Statistics=>

XRemote=>Packets Transmitted

SNMP NAME	ncdXremotePktsTransmitted

SNMP PATH	ncdXremote 3

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of XRemote packets that have been

transmitted, including retransmitted packets.

NAME		xremote-packets-with-errors-received

HIDEBOX		Console=>Statistics=>Show Statistics=>

XRemote=>Packets with Errors Received

SNMP NAME	ncdXremotePktsWithErrorsReceived

SNMP PATH	ncdXremote 7

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of XRemote packets received in error.

NAME		xremote-receive-window-size

HIDEBOX		Console=>Statistics=>Show Statistics=>

XRemote=>Receive Window Size

SNMP NAME	ncdXremoteReceiveWinSize

SNMP PATH	ncdXremote 14

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of packets that may be legally received

from the remote host without an acknowledgment

being sent.

NAME		xremote-state

HIDEBOX		Console=>Statistics=>Show Statistics=>

XRemote=>State

SNMP NAME	ncdXremoteState

SNMP PATH	ncdXremote 2

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The state of the XRemote connection.

NAME		xremote-tcp-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

XRemote=>TCP Port

SNMP NAME	ncdXremoteListenPort

SNMP PATH	ncdXremote 20

TYPE		INTEGER

MIN		1024

DEFAULT		5900

MAX		65535

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the TCP port number on which the NCD unit

listens for XRemote connections.

NAME		xremote-time-out-length

HIDEBOX		Console=>Statistics=>Show Statistics=>

XRemote=>Time-out Length

SNMP NAME	ncdXremoteTimeOut

SNMP PATH	ncdXremote 10

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The current time-out length (in milliseconds).

Packets which are not acknowledged within the

time-out period are retransmitted.

NAME		xremote-transmit-window-size

HIDEBOX		Console=>Statistics=>Show Statistics=>

XRemote=>Transmit Window Size

SNMP NAME	ncdXremoteTransWinSize

SNMP PATH	ncdXremote 11

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The number of packets that may be transmitted

before an acknowledgment from the remote host is

required.

NAME		xremote-xremote-version

HIDEBOX		Console=>Statistics=>Show Statistics=>

XRemote=>XRemote version

SNMP NAME	ncdXremoteVersion

SNMP PATH	ncdXremote 1

TYPE		INTEGER

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Revision level of the XRemote protocol being used.

GROUP xserver

NAME		xserver-access-control-enabled

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Enable X Access Control

SNMP NAME	ncdAccessAccessEnabled

SNMP PATH	ncdAccess 3

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies whether xhost-style access control

is on or off for client connections.

NAME		xserver-access-control-enabled-default

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>Default for Enable X Access Control

SNMP NAME	ncdAccessDfltAccessEnabled

SNMP PATH	ncdAccess 2

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_SESSION_STARTUP

NVRAM		NO

DESCRIPTION

Specifies whether xhost-style access control

is on or off by default for client connections.

NAME		xserver-access-control-list

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Access Control=>X Access Control List

SNMP NAME	ncdAccessAccessControlTable

SNMP PATH	ncdAccess 4

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the host access control list for the X server.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdAccessACTIndex

SNMP PATH	ncdAccessAccessControlTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the X Access Control List

NAME		host

SNMP NAME	ncdAccessACTAddress

SNMP PATH	ncdAccessAccessControlTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the name of the host that is

granted access to make connections

to the server.

NAME		family

SNMP NAME	ncdAccessACTFamily

SNMP PATH	ncdAccessAccessControlTable 3

TYPE		CHOICE

CHOICES		{ tcpip ncdnet vms-decwindows }

DEFAULT		tcpip

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

The access control family to which

this entry applies.

END TABLE ROW ENTRIES

NAME		xserver-arc-cache-max-size

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

X and Graphics=>Arc Cache Maximum Size

SNMP NAME	ncdXserverArcCacheSize

SNMP PATH	ncdXserver 20

TYPE		INTEGER

MIN		0

DEFAULT		524288

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the maximum amount of memory (in bytes)

that can be used for arc caching. If the arc cache

memory is needed to satisfy an X client request, the

arc cache may be flushed.

NAME		xserver-black-pixel

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

X and Graphics=>Black Pixel

SNMP NAME	ncdXserverXBlackPixel

SNMP PATH	ncdXserver 24

TYPE		INTEGER

MIN		0

DEFAULT		0

MAX		255

PERMISSION	READ-WRITE

EFFECT		AT_SESSION_STARTUP

NVRAM		NO

DESCRIPTION

specifies the pixel to be reported as BlackPixel

NAME		xserver-color-depth

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

X and Graphics=>Color Depth

SNMP NAME	ncdXServerColorDepth

SNMP PATH	ncdXserver 34

TYPE		CHOICE

CHOICES		{ 8-bit 16-bit 24-bit }

DEFAULT		8-bit

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

specifies the X servers color depth

NAME		xserver-default-colormap

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

X and Graphics=>Default Colormap

SNMP NAME	ncdXserverColormapTable

SNMP PATH	ncdXserver 23

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

specifies the initial contents of the default

colormap

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdXserverCTIndex

SNMP PATH	ncdXserverColormapTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Default Colormap Table

NAME		pixel

SNMP NAME	ncdXserverCTPixel

SNMP PATH	ncdXserverColormapTable 2

TYPE		INTEGER

MIN		0

DEFAULT		0

MAX		255

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

TBD

NAME		rgb

SNMP NAME	ncdXserverCTRgb

SNMP PATH	ncdXserverColormapTable 3

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

TBD

NAME		read-only

SNMP NAME	ncdXserverCTReadOnly

SNMP PATH	ncdXserverColormapTable 4

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_SESSION_STARTUP

NVRAM		NO

DESCRIPTION

TBD

END TABLE ROW ENTRIES

NAME		xserver-default-font

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Fonts=>Default Font

SNMP NAME	ncdXserverDefaultFont

SNMP PATH	ncdXserver 7

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		AT_SESSION_STARTUP

NVRAM		NO

DESCRIPTION

Specifies the X server's default font.

NAME		xserver-default-font-path

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Fonts=>Default Font Path

SNMP NAME	ncdXserverDefaultFontPathTable

SNMP PATH	ncdXserver 6

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the default set of directories where the unit

should look for fonts. The current X server font path is

reset to this value on session reset.

Special values are:

'built-ins' which refers to fonts built

into the NCD server.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdXserverFontPathTableIndex

SNMP PATH	ncdXserverDefaultFontPathTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the default font path entry table

NAME		font-path-entry

SNMP NAME	ncdXserverFontPathTableEntry

SNMP PATH	ncdXserverDefaultFontPathTable 2

TYPE		STRING

DEFAULT		built-ins

PERMISSION	READ-WRITE

EFFECT		AT_SESSION_STARTUP

NVRAM		NO

DESCRIPTION

Specifies the default font path directories set

at server reset

END TABLE ROW ENTRIES

NAME		xserver-default-visual

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

X and Graphics=>Default Visual

SNMP NAME	ncdXserverXDefaultVisual

SNMP PATH	ncdXserver 22

TYPE		CHOICE

CHOICES		{ static-gray gray-scale static-color pseudo-color true-color direct-color true-color-24 direct-color-24 }

DEFAULT		static-gray

PERMISSION	READ-WRITE

EFFECT		AT_SESSION_STARTUP

NVRAM		NO

DESCRIPTION

specifies the X server's default visual

NAME		xserver-disabled-extensions

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

X and Graphics=>Disabled X Extensions

SNMP NAME	ncdXserverXDisabledExt

SNMP PATH	ncdXserver 17

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the extensions that should not be made available

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdXserverDisExtTableIndex

SNMP PATH	ncdXserverXDisabledExt 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the disabled X extensions table

NAME		extension-name

SNMP NAME	ncdXserverDisExtTableName

SNMP PATH	ncdXserverXDisabledExt 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies a disabled extension

END TABLE ROW ENTRIES

NAME		xserver-enable-local-dead-key-processing

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Input Devices=>Local Dead Key Support

SNMP NAME	ncdXserverDeadKeySupport

SNMP PATH	ncdXserver 4

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Allows some keyboards to use local

dead key processing

NAME		xserver-enable-local-lightpen-blue-flood

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Input Devices=>Light Pen Blue Flood

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Allows blue flooding of the screen when

when the Light Pen's button is pressed.

NAME		xserver-font-alias-file-name

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Fonts=>Font Alias File Name

SNMP NAME	ncdXserverFontAliasFileName

SNMP PATH	ncdXserver 11

TYPE		STRING

DEFAULT		fonts.alias

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the file name to use for the font aliases.

NAME		xserver-font-cache-max-size

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Fonts=>Font Cache Maximum Size

SNMP NAME	ncdXserverFontCacheSize

SNMP PATH	ncdXserver 8

TYPE		INTEGER

MIN		0

DEFAULT		100000

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the maximum amount of memory (in bytes)

that can be used for font caching. If the font cache

memory is needed to satisfy an X client request, the

font cache may be flushed.

NAME		xserver-font-directory-file-name

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Fonts=>Font Directory File Name

SNMP NAME	ncdXserverFontDirFileName

SNMP PATH	ncdXserver 10

TYPE		STRING

DEFAULT		fonts.dir

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the file name to use for the font directory.

NAME		xserver-font-path-alias-table

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Fonts=>Font Name Table

SNMP NAME	ncdXserverFontPathAliasTable

SNMP PATH	ncdXserver 12

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies a symbolic mapping between user-specified

strings and font path names.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdXserverAliasTableIndex

SNMP PATH	ncdXserverFontPathAliasTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the default font path alias table

NAME		font-path-alias

SNMP NAME	ncdXserverAliasTableAlias

SNMP PATH	ncdXserverFontPathAliasTable 2

TYPE		STRING

DEFAULT		100dpi

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies a font path aliases for the given value

NAME		font-path-value

SNMP NAME	ncdXserverAliasTableValue

SNMP PATH	ncdXserverFontPathAliasTable 3

TYPE		STRING

DEFAULT		/usr/lib/X11/ncd/fonts/pcf/100dpi/

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the value for the given font alias

correspondence

END TABLE ROW ENTRIES

NAME		xserver-fontserver-open-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Fonts=>Open Timeout

SNMP NAME	ncdXserverFontserverOpenTimeout

SNMP PATH	ncdXserver 15

TYPE		INTEGER

MIN		0

DEFAULT		30

MAX		100

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) to attempt to establish

a connection to a font server before declaring an error

situation.

NAME		xserver-fontserver-read-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Fonts=>Read Timeout

SNMP NAME	ncdXserverFontserverReadTimeout

SNMP PATH	ncdXserver 14

TYPE		INTEGER

MIN		0

DEFAULT		20

MAX		100

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in seconds) to attempt

to read information from a font server before declaring

an error situation.

NAME		xserver-fontserver-reopen-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Fonts=>Reopen Timeout

SNMP NAME	ncdXserverFontserverReopenTimeout

SNMP PATH	ncdXserver 16

TYPE		INTEGER

MIN		0

DEFAULT		10

MAX		100

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the timeout (in seconds) before failing

to reopen the font server connection.

NAME		xserver-fontserver-retry-attempts

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Fonts=>Retry Attempts

SNMP NAME	ncdXserverFontserverRetryAttempts

SNMP PATH	ncdXserver 13

TYPE		INTEGER

MIN		0

DEFAULT		5

MAX		25

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the number of times to retry a font server

request before declaring an error situation.

NAME		xserver-initial-x-resources

HIDEBOX		No Widget

SNMP NAME	ncdXserverInitialXResources

SNMP PATH	ncdXserver 19

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the initial resources of the RESOURCE_MANAGER

root window property. Resources listed in this parameter

are stored in the X server, where they are accessible by

X clients.

NAME		xserver-initialize-web-palette-colors

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

X and Graphics=>Pre-initialize Web Palette Colors

SNMP NAME	ncdXserverInitializeWebPaletteColors

SNMP PATH	ncdXserver 35

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_SESSION_STARTUP

NVRAM		NO

DESCRIPTION

Specifies whether to preload the 216 web palette colors into

the default colormap

NAME		xserver-input-extension-device

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Input Devices=>Input Extension Device

SNMP NAME	ncdXserverInputExtensionDevice

SNMP PATH	ncdXserver 26

TYPE		CHOICE

CHOICES		{ tablet 3d-mouse touchscreen }

DEFAULT		tablet

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specify which input extension device is

attached to serial port.

NAME		xserver-keyboard-description-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Input Devices=>Keyboard Description File

SNMP NAME	ncdXserverKeyboardDescFile

SNMP PATH	ncdXserver 30

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the keyboard description file

NAME		xserver-keyboard-narrow-keysym-tables

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Input Devices=>Narrow Keysym Tables

SNMP NAME	ncdXserverNarrowKeyTable

SNMP PATH	ncdXserver 3

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies whether to use R3 2-entry keysym tables

instead of R4-style 4-entry keysym tables

NAME		xserver-keyboard-type

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Input Devices=>Keyboard Type

SNMP NAME	ncdXserverCurrentKeyboard

SNMP PATH	ncdXserver 1

TYPE		CHOICE

CHOICES		{  }

DEFAULT

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		YES

DESCRIPTION

Specifies the type of keyboard in use.

NOTE: Choices depend on attached keyboard.

NAME		xserver-keyboard-type-choices

HIDEBOX		No Widget

SNMP NAME	ncdXserverKeyboardTypeChoices

SNMP PATH	ncdXserver 2

TYPE		TABLE

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies the list of currently allowed choices

for the type of keyboard in use.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdXserverKeyboardChoiceIndex

SNMP PATH	ncdXserverKeyboardTypeChoices 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the keyboard choice table

NAME		choice

SNMP NAME	ncdXserverCurrentKeyboardChoice

SNMP PATH	ncdXserverKeyboardTypeChoices 2

TYPE		STRING

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Specifies an available choice for the type

of the current keyboard in use.

END TABLE ROW ENTRIES

NAME		xserver-keysym-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

X and Graphics=>Keysym File

SNMP NAME	ncdXserverKeysymDBPath

SNMP PATH	ncdXserver 27

TYPE		STRING

DEFAULT		/usr/lib/X11/ncd/XKeysymDB

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the file name of the keysym database

used by local clients.

NAME		xserver-mouse-buttons

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Input Devices=>Mouse Buttons

SNMP NAME	ncdXserverMouseButtons

SNMP PATH	ncdXserver 5

TYPE		CHOICE

CHOICES		{ auto-detect 2 }

DEFAULT		auto-detect

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the number of buttons on the mouse

attached to the unit. The choice 'Auto' allows

the unit to attempt to automatically determine

the number of buttons. Note that the choice '2'

is only needed with a Microsoft mouse.

With all other mice the number of buttons is

automatically detected.

NAME		xserver-retain-font-path

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Fonts=>Retain Font Path Across Sessions

SNMP NAME	ncdXserverRetainFonts

SNMP PATH	ncdXserver 9

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that the X server should retain the font path

the last client disconnects from the server, and it

undergoes a session reset.

NAME		xserver-retain-x-settings

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

X and Graphics=>Retain X Settings Across Sessions

SNMP NAME	ncdXserverXServerRetainSettings

SNMP PATH	ncdXserver 18

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that the X server should retain the settings

of the resource database, keyboard, mouse, and screen

saver settings when the last client disconnects from

the server, and it undergoes a session reset.

NAME		xserver-rgb-file

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

X and Graphics=>RGB File

SNMP NAME	ncdXserverRgbPath

SNMP PATH	ncdXserver 21

TYPE		STRING

DEFAULT		/usr/lib/X11/ncd/rgb.txt

PERMISSION	READ-WRITE

EFFECT		AT_BOOT

NVRAM		NO

DESCRIPTION

Specifies the file name of the color database.

NAME		xserver-touchscreen-calibration-limits

HIDEBOX		No Widget

SNMP NAME	ncdXServerTouchscreenCalibrationLimitsTable

SNMP PATH	ncdXserver 28

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Touchscreen calibration limits -- Lower Left( x,y), Upper Right (x,y)

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdXserverTouchscreenCalibrationLimitsIndex

SNMP PATH	ncdXServerTouchscreenCalibrationLimitsTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Touchscreen Calibration Limits List

NAME		x-min

SNMP NAME	ncdXserverTouchscreenCalibrationLimitsXMin

SNMP PATH	ncdXServerTouchscreenCalibrationLimitsTable 2

TYPE		INTEGER

MIN		0

DEFAULT		28

MAX		16383

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the minimum touchscreen x coordinate which

is in the active area of the screen

NAME		y-min

SNMP NAME	ncdXserverTouchscreenCalibrationLimitsYMin

SNMP PATH	ncdXServerTouchscreenCalibrationLimitsTable 3

TYPE		INTEGER

MIN		0

DEFAULT		12

MAX		16383

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the minimum touchscreen y coordinate which

is in the active area of the screen

NAME		x-max

SNMP NAME	ncdXserverTouchscreenCalibrationLimitsXMax

SNMP PATH	ncdXServerTouchscreenCalibrationLimitsTable 4

TYPE		INTEGER

MIN		0

DEFAULT		420

MAX		16383

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the maximum touchscreen x coordinate which

is in the active area of the screen

NAME		y-max

SNMP NAME	ncdXserverTouchscreenCalibrationLimitsYMax

SNMP PATH	ncdXServerTouchscreenCalibrationLimitsTable 5

TYPE		INTEGER

MIN		0

DEFAULT		319

MAX		16383

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		YES

DESCRIPTION

Specifies the maximum touchscreen y coordinate which

is in the active area of the screen

END TABLE ROW ENTRIES

NAME		xserver-vendor-string

HIDEBOX		No Widget

SNMP NAME	ncdXserverVendorString

SNMP PATH	ncdXserver 29

TYPE		STRING

DEFAULT

PERMISSION	READ-WRITE

EFFECT		AT_SESSION_STARTUP

NVRAM		NO

DESCRIPTION

Specifies the vendor string reported by the X server.

NAME		xserver-white-pixel

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

X and Graphics=>White Pixel

SNMP NAME	ncdXserverXWhitePixel

SNMP PATH	ncdXserver 25

TYPE		INTEGER

MIN		0

DEFAULT		0

MAX		255

PERMISSION	READ-WRITE

EFFECT		AT_SESSION_STARTUP

NVRAM		NO

DESCRIPTION

specifies the pixel to be reported as WhitePixel

GROUP ncd3270

NAME		ncd3270-default-hosts

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

3270 Terminal Emulation=>Default Hosts

SNMP NAME	ncd3270DfltConnectionTable

SNMP PATH	ncd3270 1

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the default list of hosts to present

to the user in the terminal chooser.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncd3270DCTIndex

SNMP PATH	ncd3270DfltConnectionTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Default Hosts table for NCD3270

NAME		protocol

SNMP NAME	ncd3270DCTProtocol

SNMP PATH	ncd3270DfltConnectionTable 2

TYPE		CHOICE

CHOICES		{ telnet pu2.1 }

DEFAULT		telnet

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

The protocol to use.

NAME		host

SNMP NAME	ncdTermDCTHost

SNMP PATH	ncd3270DfltConnectionTable 3

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

The name of the terminal host.

NAME		port

SNMP NAME	ncdTermDCTPort

SNMP PATH	ncd3270DfltConnectionTable 4

TYPE		STRING

DEFAULT		23

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

The port number to use.

NAME		model

SNMP NAME	ncd3270DCTModel

SNMP PATH	ncd3270DfltConnectionTable 5

TYPE		CHOICE

CHOICES		{ model-2 model-3 model-4 model-5 }

DEFAULT		model-2

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

The model to use.

NAME		description

SNMP NAME	ncdTermDCTDescription

SNMP PATH	ncd3270DfltConnectionTable 6

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

A description of the host.

END TABLE ROW ENTRIES

NAME		ncd3270-default-model

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

3270 Terminal Emulation=>NCD3270 Default Model

SNMP NAME	ncd3270DefaultModel

SNMP PATH	ncd3270 4

TYPE		CHOICE

CHOICES		{ model-2 model-3 model-4 model-5 }

DEFAULT		model-2

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Default NCD3270 Model Type

NAME		ncd3270-default-protocol

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

3270 Terminal Emulation=>NCD3270 Default Protocol

SNMP NAME	ncd3270DefaultProtocol

SNMP PATH	ncd3270 3

TYPE		CHOICE

CHOICES		{ telnet pu2.1 }

DEFAULT		telnet

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Default NCD3270 protocol

NAME		ncd3270-ehllapia-default-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

3270 Terminal Emulation=>EHLLAPI A Default tcp Port

SNMP NAME	ncd3270EhllapiaDefaultPort

SNMP PATH	ncd3270 9

TYPE		INTEGER

MIN		0

DEFAULT		5021

MAX		65535

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

EHLLAPI A default tcp port

NAME		ncd3270-enable-3270-emulation

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

3270 Terminal Emulation=>Enable 3270 Terminal Emulation

SNMP NAME	ncd3270Enable3270Emulation

SNMP PATH	ncd3270 2

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Enable 3270 Terminal Emulation

NAME		ncd3270-pu21-default-host

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

3270 Terminal Emulation=>PU2.1 Default Host

SNMP NAME	ncd3270PU21DefaultHost

SNMP PATH	ncd3270 7

TYPE		STRING

DEFAULT		local

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Default Brixton PU2.1 Host

NAME		ncd3270-pu21-default-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

3270 Terminal Emulation=>PU2.1 Default tcp Port

SNMP NAME	ncd3270PU21DefaultPort

SNMP PATH	ncd3270 8

TYPE		INTEGER

MIN		0

DEFAULT		5020

MAX		65535

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Default Brixton PU2.1 tcp Port

NAME		ncd3270-tn-default-host

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

3270 Terminal Emulation=>TN3270 Default Host

SNMP NAME	ncd3270TNDefaultHost

SNMP PATH	ncd3270 5

TYPE		STRING

DEFAULT		local

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Default TN3270 host

NAME		ncd3270-tn-default-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

3270 Terminal Emulation=>TN3270 Default tcp Port

SNMP NAME	ncd3270TNDefaultPort

SNMP PATH	ncd3270 6

TYPE		INTEGER

MIN		0

DEFAULT		23

MAX		65535

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Default TN3270 tcp Port

GROUP modules

NAME		modules-directory

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Loadable Modules=>Loadable Modules Directory

SNMP NAME	ncdModulesDirectory

SNMP PATH	ncdModules 1

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the directory containing the loadable modules.

An appropriate file table entry must be established.

NAME		modules-load-policy

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Loadable Modules=>Load Policy

SNMP NAME	ncdModulesLoadPolicyTable

SNMP PATH	ncdModules 2

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

The Load Policy table iteslf

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdModulesLoadPolicyIndex

SNMP PATH	ncdModulesLoadPolicyTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Load Policy Table

NAME		name

SNMP NAME	ncdModulesLoadPolicyName

SNMP PATH	ncdModulesLoadPolicyTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

The name of the loaded module

NAME		policy

SNMP NAME	ncdModulesLoadPolicyLoadAction

SNMP PATH	ncdModulesLoadPolicyTable 3

TYPE		CHOICE

CHOICES		{ on-demand at-boot disable }

DEFAULT		on-demand

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

The policy for loading this module:

On Demand - the module is loaded when needed.

At Boot   - the module is loaded when the unit is booted.

Disable   - the module is never loaded.

END TABLE ROW ENTRIES

NAME		modules-loaded

HIDEBOX		Console=>Statistics=>Show Statistics=>

Loadable Modules=>Currently Loaded

SNMP NAME	ncdModulesLoadedTable

SNMP PATH	ncdModules 4

TYPE		TABLE

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Shows information about loaded modules.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdModulesLoadedIndex

SNMP PATH	ncdModulesLoadedTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

The index into the table of Currently Loaded Modules.

NAME		name

SNMP NAME	ncdModulesLoadedName

SNMP PATH	ncdModulesLoadedTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Contains the module name.

NAME		memory-used

SNMP NAME	ncdModulesLoadedMemoryUsed

SNMP PATH	ncdModulesLoadedTable 3

TYPE		INTEGER

MIN		0

DEFAULT		0

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Contains the memory used by this module.

NAME		version

SNMP NAME	ncdModulesLoadedVersion

SNMP PATH	ncdModulesLoadedTable 5

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Contains the 'what' information of the module.

END TABLE ROW ENTRIES

GROUP time

NAME		time-offset-from-gmt

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Time=>Offset from GMT

SNMP NAME	timeOffsetFromGMT

SNMP PATH	ncdTime 4

TYPE		INTEGER

MIN		-719

DEFAULT		-480

MAX		720

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the offset (in minutes) from Greenwich Mean Time (GMT).  Positive

values represent locations East of GMT and negative values represent

locations West of GMT.

NAME		time-server

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Time=>Time Server

SNMP NAME	timeServerAddress

SNMP PATH	ncdTime 1

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the IP host to use as a time server.

NAME		time-server-retries

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Time=>Time Server Retries

SNMP NAME	timeServerRetries

SNMP PATH	ncdTime 3

TYPE		INTEGER

MIN		0

DEFAULT		3

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies how many attempts should be made to contact the time server.

NAME		time-server-timeout

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Time=>Time Server Timeout

SNMP NAME	timeServerTimeout

SNMP PATH	ncdTime 2

TYPE		INTEGER

MIN		1

DEFAULT		3

MAX		4294967295

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies how long (in seconds) to wait for a reply from the time server.

NAME		time-zone

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Time=>Timezone

SNMP NAME	timeTimeZone

SNMP PATH	ncdTime 5

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Time zone for location of unit.

GROUP wincenter

NAME		wincenter-audio

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

WinCenter=>Audio Enabled

SNMP NAME	ncdWinCenterAudioEnabled

SNMP PATH	ncdWinCenter 5

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		AT_SESSION_STARTUP

NVRAM		NO

DESCRIPTION

NAME		wincenter-numcolors

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

WinCenter=>Number of Colors

SNMP NAME	ncdWinCenterNumColors

SNMP PATH	ncdWinCenter 4

TYPE		CHOICE

CHOICES		{ 16 256 truecolor }

DEFAULT		16

PERMISSION	READ-WRITE

EFFECT		AT_SESSION_STARTUP

NVRAM		NO

DESCRIPTION

NAME		wincenter-resolution

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

WinCenter=>Window Size

SNMP NAME	ncdWinCenterWindowSize

SNMP PATH	ncdWinCenter 3

TYPE		CHOICE

CHOICES		{ default full-screen 640x480 800x600 1024x768 1152x900 1280x1024 1600x1200 }

DEFAULT		default

PERMISSION	READ-WRITE

EFFECT		AT_SESSION_STARTUP

NVRAM		NO

DESCRIPTION

NAME		wincenter-resources

HIDEBOX		No Widget

SNMP NAME	ncdWinCenterResources

SNMP PATH	ncdWinCenter 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		AT_SESSION_STARTUP

NVRAM		NO

DESCRIPTION

Specifies the WinCenter resources of the RESOURCE_MANAGER

root window property. Resources listed in this parameter

are stored in the X server, where they are accessible by

X clients.

GROUP browser

NAME		browser-auto-load-images

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>Auto Load Images

SNMP NAME	ncdBrowserAutoLoadImages

SNMP PATH	ncdBrowser 5

TYPE		CHOICE

CHOICES		{ user-preference enable disable }

DEFAULT		user-preference

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies automatically load images when loading a new Web page.

NAME		browser-cache-size

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>Cache Size

SNMP NAME	ncdBrowserCacheSize

SNMP PATH	ncdBrowser 2

TYPE		INTEGER

MIN		0

DEFAULT		0

MAX		100000

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the Maximum size of the memory cache in

killo bytes.

NAME		browser-environment

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>Environment Variables

SNMP NAME	ncdBrowserEnvVarTable

SNMP PATH	ncdBrowser 24

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the current Browser environment variable settings for use

by local clients.

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdBrowserEnvVarTableIndex

SNMP PATH	ncdBrowserEnvVarTable 21

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Browser Environment Variable table

NAME		name

SNMP NAME	ncdBrowserEnvVarTableName

SNMP PATH	ncdBrowserEnvVarTable 22

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the Browser Environment Variable name.

NAME		value

SNMP NAME	ncdBrowserEnvVarTableValue

SNMP PATH	ncdBrowserEnvVarTable 23

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the Browser Environment Variable value.

END TABLE ROW ENTRIES

NAME		browser-ftp-proxy-host

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>FTP Proxy Host

SNMP NAME	ncdFTP_ProxyHost

SNMP PATH	ncdBrowser 13

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the FTP (File Transfer protocol) proxy

hostname whch runs the FTP server deamons.

NAME		browser-ftp-proxy-host-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>FTP Proxy Host Port

SNMP NAME	ncdBrowserFTPProxyHostPort

SNMP PATH	ncdBrowser 14

TYPE		INTEGER

MIN		0

DEFAULT		80

MAX		65535

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the FTP proxi TCP/IP port that the FTP

server application listens on.

NAME		browser-ftp-proxy-overrides

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>FTP Proxy Overrides

SNMP NAME	ncdFTPProxyHostOverrides

SNMP PATH	ncdBrowser 15

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the FTP Proxy domains that avoid/override proxy

NAME		browser-gopher-proxy-host

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>Gopher Proxy Host

SNMP NAME	ncdGopherProxyHost

SNMP PATH	ncdBrowser 16

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the Gopher proxy

hostname whch runs the Gopher server deamons.

NAME		browser-gopher-proxy-host-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>Gopher Proxy Host Port

SNMP NAME	ncdBrowserGopherProxyHostPort

SNMP PATH	ncdBrowser 17

TYPE		INTEGER

MIN		0

DEFAULT		80

MAX		65535

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the Gopher proxi TCP/IP port that the Gopher

server application listens on.

NAME		browser-gopher-proxy-overrides

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>Gopher Proxy Overrides

SNMP NAME	ncdGopherProxyHostOverrides

SNMP PATH	ncdBrowser 18

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the Gopher Proxy domains that avoid/override proxy

NAME		browser-home-page

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>Home Page

SNMP NAME	ncdHomePage

SNMP PATH	ncdBrowser 9

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the Default home page that the browser will

automatically load once started.

NAME		browser-http-proxy-host

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>HTTP Proxy Host

SNMP NAME	ncdHTTP_ProxyHost

SNMP PATH	ncdBrowser 10

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the HTTP Proxy hostname which runs the

HTTP proxy service.

NAME		browser-http-proxy-host-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>HTTP Proxy Host Port

SNMP NAME	ncdBrowserHTTPProxyHostPort

SNMP PATH	ncdBrowser 11

TYPE		INTEGER

MIN		0

DEFAULT		80

MAX		65535

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Stecifies the HTTP Proxy TCP/IP port fo the HTTP

proxy server application.

NAME		browser-http-proxy-overrides

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>HTTP Proxy Overrides

SNMP NAME	ncdHTTP_ProxyHostOverrides

SNMP PATH	ncdBrowser 12

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the HTTP Proxy domains that avoid/override proxy

NAME		browser-java-applets

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>Java Applets

SNMP NAME	ncdBrowserJavaApplets

SNMP PATH	ncdBrowser 7

TYPE		CHOICE

CHOICES		{ user-preference enable disable }

DEFAULT		user-preference

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies if Jaba applets ar to be executed when

included in web contents.

NAME		browser-jscript-active

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>Jscript Active

SNMP NAME	ncdBrowserJscriptActive

SNMP PATH	ncdBrowser 6

TYPE		CHOICE

CHOICES		{ user-preference enable disable }

DEFAULT		user-preference

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies if Jscript/Javascript routines ar to be executed

when web contents are loaded.

NAME		browser-max-connections

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>Max Connections

SNMP NAME	ncdBrowserMaxConnections

SNMP PATH	ncdBrowser 3

TYPE		INTEGER

MIN		1

DEFAULT		5

MAX		32

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the Maximum number of TCP/IP connections

the browser can open concurrently.

NAME		browser-module

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>Module

SNMP NAME	ncdBrowserModule

SNMP PATH	ncdBrowser 1

TYPE		CHOICE

CHOICES		{ no-encryption ssl-40-bit-encryption ssl-128-bit-encryption }

DEFAULT		no-encryption

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies which browser module should be loaded.

NAME		browser-smtp-email-server

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>SMTP Email Server

SNMP NAME	ncdBrowserEmailServer

SNMP PATH	ncdBrowser 4

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the SMTP electronic mail server.

NAME		browser-socks-host

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>Socks Host

SNMP NAME	ncdSocksHost

SNMP PATH	ncdBrowser 19

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the SOCKS server which runs the SOCKS

deamon which apporves requests outside the local

network to a remote network resource.

NAME		browser-socks-port

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Browser=>Socks Port

SNMP NAME	ncdBrowserSocksPort

SNMP PATH	ncdBrowser 20

TYPE		INTEGER

MIN		0

DEFAULT		1080

MAX		65535

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the SOCKS TCP/IP port that the SOCKS

daemon will listen on for requests from a

SOCKS enable application.

GROUP print

NAME		print-lpr-servers

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

Print=>Lpr Servers

SNMP NAME	ncdPrintLprServersTable

SNMP PATH	ncdPrint 1

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Print server information

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdPrintLprServersTableIndex

SNMP PATH	ncdPrintLprServersTable 1

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the Print Lpr Servers table

NAME		host

SNMP NAME	ncdPrintLprServersTableHost

SNMP PATH	ncdPrintLprServersTable 2

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

host name

NAME		queue-name

SNMP NAME	ncdPrintLprServersTableQueueName

SNMP PATH	ncdPrintLprServersTable 3

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

queue name

NAME		datastream-type

SNMP NAME	ncdPrintLprServersTableDatastreamType

SNMP PATH	ncdPrintLprServersTable 4

TYPE		CHOICE

CHOICES		{ ascii postscript nt-postscript }

DEFAULT		ascii

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

datastream type

NAME		description

SNMP NAME	ncdPrintLprServersTableDescription

SNMP PATH	ncdPrintLprServersTable 5

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

description

NAME		request-banner-page

SNMP NAME	ncdPrintLprServersTableRequestBannerPage

SNMP PATH	ncdPrintLprServersTable 10

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

request banner page

END TABLE ROW ENTRIES

GROUP ica

NAME		ica-chooser-entries

HIDEBOX		No Widget

SNMP NAME	ncdICAConfiguration

SNMP PATH	ncdICA 5

TYPE		TABLE

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

ICA Chooser entry table

BEGIN TABLE ROW ENTRIES

NAME		index

SNMP NAME	ncdICAChooserEntryIndex

SNMP PATH	ncdICAConfiguration 3

TYPE		INTEGER

VIEW		SNMP ONLY

MIN		0

MAX		4294967295

PERMISSION	READ-ONLY

NVRAM		NO

DESCRIPTION

Index into the ICA chooser entry table

NAME		ica-chooser-entry

SNMP NAME	ncdICAChooserEntry

SNMP PATH	ncdICAConfiguration 4

TYPE		STRING

DEFAULT		nil

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

ICA Chooser entry options

END TABLE ROW ENTRIES

NAME		ica-enable-chooser-editing

HIDEBOX		No Widget

SNMP NAME	ncdICAEditingEnabled

SNMP PATH	ncdICA 2

TYPE		BOOL

DEFAULT		true

PERMISSION	READ-WRITE

EFFECT		AT_SESSION_STARTUP

NVRAM		NO

DESCRIPTION

Enables ICA Chooser editing

NAME		ica-extended-diagnostics

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

ICA=>Extended ICA Diagnostics

SNMP NAME	ncdICAExtendedDiagnostics

SNMP PATH	ncdICA 8

TYPE		BOOL

DEFAULT		false

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies that the ICA should generate more detailed

diagnostic messages regarding ICA service.

NAME		ica-retry-attempts

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

ICA=>Retry Attempts

SNMP NAME	ncdICARetryAttempts

SNMP PATH	ncdICA 7

TYPE		INTEGER

MIN		0

DEFAULT		1

MAX		6

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the number of retries to find

ICA master browser/server before timiming out.

NAME		ica-wait-time

HIDEBOX		Console=>Setup=>Change Setup Parameters=>

ICA=>ICA Browser Wait Timeout

SNMP NAME	ncdICABrowserWaitTime

SNMP PATH	ncdICA 6

TYPE		INTEGER

MIN		100

DEFAULT		500

MAX		5000

PERMISSION	READ-WRITE

EFFECT		IMMEDIATE

NVRAM		NO

DESCRIPTION

Specifies the amount of time (in miliseconds) the

ICA client waits for response from the ICA master browser

or server before timiming out.