.CENTERGRAPHBOX DATA SHEETTHE ^^GRAPHBOX IS A HIGHLY VERSATILE, LOST COST DESK-TOP RASTER-SCANGRAPHICS TERMINAL.  IT PROVIDES BUILT-IN FACILITIES FORDISPLAYING CONVENTIONAL UPPER/LOWER CASE ^^ASCII CHARACTERS,STRAIGHT LINES, POINTS, EMPTY AND SOLID BOXES, CIRCLES OF ARBITRARYRADIUS, OR ARBITARY SIZE CHARACTERS FROM A USER-LOADEDALTERNATE CHARACTER SET.A BLINKING CURSOR DISPLAYS THE POSITION IN WHICH THE NEXT GRAPHICSOBJECT WILL BE DISPLAYED.  COMMANDS ARE PROVIDED TO MANIPULATETHE CURSOR ON A CHARACTER ROW/COLUMN BASIS, OR ON A RESOLUTION-LIMITEDBASIS.A SET OF CROSSHAIRS CONTROLLED BY A JOYSTICK ARE USED TO PROVIDE GRAPHICSFEEDBACK TO AN APPLICATION PROGRAM, EITHERIN THE ^^GRAPHBOX ITSELF OR EXTERNAL TO THE GRAPHBOX. AN OPTIONALLIGHT-PEN CAN ALSO BE USED FOR FEEDBACK.^^GRAPHBOX CAN ACT AS "DUMB" ALPHANUMERIC TERMINAL, A DUMB GRAPHICSTERMINAL, A LOCALLY INTELLIGENT TERMINAL, OR ASA STANDALONE PROCESSING SYSTEM BECAUSE OF THE STRUCTURE OF THEINTERNAL MICROPROCESSOR DRIVEN SOFTWARE.USER-WRITTEN APPLICATION PROGRAMS CAN BE EXECUTED IN THE OPTIONALUSER-PROGRAM MEMORY, AND PERIPHERALS CAN BE CONNECTEDLOCALLY TO THE ^^GRAPHBOX..PAGE^^FEATURES.BREAK.LMARGIN 20256 BY 512 DOT MATRIX DISPLAY.BREAK^SMALL SIZE.BREAK^COMPATIBLE WITH MOST DUMB TERMINALS.BREAK^BLINKING CURSOR.BREAK.LMARGIN 10^^ALPHANUMERIC ^^FUNCTIONS.BREAK.LMARGIN 20^UPPER/LOWER CASE WITH TAILS BELOW LINE.BREAK28 LINES BY 86 CHARACTER DISPLAY.BREAK^AUTOMATIC SCREEN SCROLL, WRAP-AROUND.BREAK^KEYBOARD DATA ENTRY.BREAK^CURSOR ADDRESSING.BREAK^SCREEN CLEAR.BREAK^AUTOMATIC BUFFER OVERFLOW NOTIFICATION OF ^HOST ^^CPU(SUPPLIES "SLOWDOWN!" FEEDBACK).BREAK^ALTERNATE CHARACTER SET STORAGE/DISPLAY.BREAK^BACKSPACING.BREAK^ERASE LINE OR REGION OF SCREEN.BREAK^REMOTE CURSOR MOVEMENT UP/DOWN/LEFT/RIGHT.BREAK^CHARACTER DISPLAY NOT LIMITED TO CHARACTER ROWS AND COLUMNS.BREAK.LMARGIN 10^^KEYBOARD ^^FUNCTIONS.BREAK.LMARGIN 20^^TTY LOCK.BREAK^SELECT BAUD RATE/WORD STRUCTURE.BREAK^TERMINAL RESET.BREAK^JOYSTICK POSITIONING OF CROSSHAIRS (CROSSHAIR ACCELERATION/INSTANT STOP/SINGLE STEP).BREAK^DEBUGGER "WAKEUP" KEY IF DEBUGGER LOADED.BREAK^DUMP MODE (DISPLAYS GRAPHICS COMMANDS AS ^^ASCII SEQUENCES FOR DEBUGGING).BREAK^NOTIFICATION TO LOCAL INTELLIGENCE OF REQUEST FOR GRACEFUL STOP.BREAK.LMARGIN 10^^GRAPHICS ^^FUNCTIONS.BREAK.LMARGIN 20^ALL GRAPHICS FUNCTIONS TRIGGERED BY ^E^S^CAPE SEQUENCES.BREAK^ALL PARAMETERS TO GRAPHICS FUNCTIONS TRANSMITTED AS NUMERIC ^^ASCIIFOR SIMPLE INTERFACE TO ^HOST APPLICATION PROGRAMS.BREAK^MOVE CURSOR AND DRAW DOT.BREAK^DRAW DOT AT CURSOR.BREAK^DRAW DOT AT CURSOR AND MOVE CURSOR.BREAK^DRAW LINE FROM ANY POINT TO ANY OTHER POINT.BREAK^DRAW BOX OUTLINE.BREAK^DRAW SOLID BOX.BREAK^DRAW ARC OF CIRCLE.BREAK^DRAW REMOTELY SUPPLIED PICTURE.BREAK^DRAW STORED PICTURE.BREAK^ERASE REGION.BREAK^ERASE DOT/LINE/BOX/ARC/PICTURE.BREAK^SELECT WHITE BACKGROUND.BREAK^SELECT BLACK BACKGROUND.BREAK^SET CURSOR ^X.BREAK^SET CURSOR ^Y.BREAK^SET CURSOR ^X AND ^Y.BREAK^READ CURSOR ^X AND ^Y AS NUMERIC ^^ASCII COORDINATES.BREAK^READ CROSSHAIRS ^X AND ^Y.BREAK^ERASE CURSOR.BREAK^SCROLL SCREEN LEFT/RIGHT/DOWN/UP.BREAK^SCROLL SCREEN AND DRAW IMAGE LINE (STRIP CHART MODE).BREAK^STORE REMOTELY SUPPLIED PICTURE.BREAK^GRAPHICS AND ALPHANUMERICS SCROLL TOGETHER.BREAK^LIMITED REAL-TIME GRAPHICS.BREAK.LMARGIN 10^^LOCAL ^^INTELLIGENCE.BREAK.LMARGIN 20^OPTIONAL 16/32K USER RAM.BREAK^DOWNLOAD USER PROGRAM BINARY.BREAK^PASS CONTROL TO DOWN-LOADED PROGRAM.BREAK^DIRECT USER PROGRAM ACCESS TO SCREEN MEMORY.BREAK^ALL GRAPHICS PRIMITIVES AVAILABLE AS SUBROUTINES.BREAK^BUILT-IN REAL-TIME OPERATING SYSTEM.BREAK^EXTERNAL ^^I/O BUS CONNECTS TO OTHER PERIPHERALS --FLOPPY DISK, PRINTER, ETC, FOR COMPLETE LOCAL PROCESSING POWER.BREAK^RUNS ^SOFTWARE ^DYNAMICS SOFTWARE.BREAK.LMARGIN 10^^APPLICATIONS.LMARGIN 20.BREAK^BAR AND CURVE PLOTS FOR ENGINEERING.BREAK^LOGIC AND PRINTED CIRCUIT DESIGN AIDS.BREAK^GRAPHIC DISPLAY IN COMPUTER-AIDED INSTRUCTION.BREAK^REAL-TIME WAVEFORM DISPLAYS FOR HOSPITAL OR TELEMETRY ENVIRONMENTS.BREAK^GENERAL PURPOSE DATA ENTRY/DISPLAY FOR MANAGEMENT INFORMATION SYSTEMS.BREAK.LMARGIN 10^^ELECTRICAL ^^SPECIFICATIONS.BREAK.LMARGIN 20110^V SINGLE PHASE 60^H^Z.BREAK20^W DISSIPATED POWER.BREAK3 AMP FUSE.BREAK.LMARGIN 10^^GRAPHBOX ^^MECHANICAL.BREAK.LMARGIN 20^HEIGHT 12".BREAK^DEPTH 21".BREAK^WIDTH 15.5".BREAK.LMARGIN 10^^INTERFACE ^^ELECTRONICS.BREAK.LMARGIN 20^^EIA ^^RS232 INTERFACE.BREAK50-19200 ^BAUD, KEYBOARD SELECTABLE.BREAK^EXTERNAL 50 PIN COMPUTER DATA-^I/^O BUS FOR LOCAL PERIPHERALS.BREAK.LMARGIN 10022178.END