.ND

.TL

Bootstrapping 2.9BSD on the Pro350

.sp

.LP

.SH

Preliminaries

.sp

.PP

The Pro350 is essentially an 11/23+ with floating pt.,

a bit mapped screen, 3 serial ports, two disk subsystems

and a weird little time of day clock.  For 2.9, I've named

the devices as follows:

.sp

.TS

center;

l c l l l.

Device	Driver Name	/dev name	Special	File #'s

_

b	c

.sp

-bit mapped	cn.c	/dev/console	--	5,0

screen and

keyboard

.sp

-serial port	pc.c	/dev/tty00	--	1,0

& com. port		& /dev/tty01		1,1

.sp

a

-RX50 floppies	r5.c	/dev/[r]r50	11,0	16,0

& /dev/[r]r51	11,1	16,1

.sp

RD5X hard disk	rd.c	/dev/[r]rd0[a-h]	10,0-7	10,0-7

.TE

.sp

The hard disk partitions are:

.sp

.TS

center;

c c c c

c l n l.

name	filesystem	start (512 blocks)	size (512 blocks)

offset

_

.sp

a	/	64	4480

.sp

b	swap	4544	1920

.sp

c	/usr for RD51	6464	13056

.sp

d	/usr for RD50	6464	3264

.sp

h	whole disk	0	19584 (RD51)

or	9792 (RD50)

.TE

.sp

The printer and com. ports can both be used as

ordinary terminal lines.  If pins 8-9 are jumpered

on the printer port it can also be used as a

maintenance terminal (i.e., hit <break> and you

halt the cpu and end up in ODT).  Also, DSR is used

as carrier on the printer port.

.sp

.bp

.TL

Bootstrapping 2.9 on the Pro350

.sp

.SH

Phase 1

.LP

(requires a running 2.9BSD system)

.sp

.PP

Generate a bootable kernel and an appropriate system disk

using the modified sources.  Briefly, this involves

the following:

.in+2

-kernel sysgen:

.in+2

.sp

.ti-1

--install the new source files in the appropriate directories and modify

the files ../sys/sys according to diffs.sys

.sp

--set up an appropriate config file

.br

eg.

.nf

#  Machine configuration file.

#  Format:

#	name	value		comments

#

#  The value field may not contain blanks;

#  blank lines are not allowed.

#

#	General parameters

pdp11		21

ident		jay

maxusers	2

bootdev		rd

timezone	5			# Pacific

dst		1

#

#	Filesystem configuration

rootdev		makedev(10,0)		# rd0a

swapdev		makedev(10,1)		# rd0b

dumpdev		NODEV

pipedev		makedev(10,0)		# rd0a

swplo		0

nswap		1920

dumplo		0

dumproutine	nodev

#

#	Device configuration

NXP		0

NXP_CONTROLLER 	0

NHK		0

NRK		0

NRL		0

NRP		0

NR5		2

NRD		1

NHT		0

NTM		0

NTS		0

NDZ		0		# in units of boards (8 each)

NDH		0		# in units of boards (16 each)

NDM		0		# in units of boards (16 each)

LOWDM		0		# LOWDM is in units of lines

NKL		0

NLP		0

NPC		2

.fl

note pdp11 type of '21'

.sp

.ti-1

--run config

.ti+5

-config PRO

.br

.ti+5

-cd ../PRO

.sp

.ti-1

--edit localopts as required:

.in-2

.sp

.ti-1

--edit param.c to set HZ to 64 (The rate of the PRO's weird

clock) "make" to generate the kernel

.sp

.ti-1

--run "make" in directory ../sys/stand to generate

the stand alone utilities that know how to talk to

the RX50 and RD5X.

.sp

.ti-1

--run "make.sh" in directory, ../mdec to generate

the boot blocks for the RX50 and RD5X.

.sp

.ti-1

--now make a root file system on an appropriate

disk partition

.sp

# mkfs /dev/r??? 2240

.sp

.in+2

-copy all the normal root stuff

including "rdfmt", "getfile" and "rdboot" just

created into the '/' directory

.in-2

.sp

.ti-1

--now make the file system for the maintenance floppy

.sp

# mkfs /dev/r??? proto where "proto" is the following:

.nf

/mdec/rkuboot

200 128

d--777 0 0

$

.fi

# mount /dev/r??? /mnt

.sp

.in+2

-copy the following to /mnt

.in+2

.sp

boot, restor, mkfs, rdfmt and getfile

.br

/unix with root and swap on the r50 (swaplo 400, nswap 390)

.br

(will have to be stripped to fit on floppy)

.sp

and put the r5 bootstrap block in block 0

.sp

# umount /dev/r???

.br

# dd if= ... mdec/r5uboot of=/dev/r??? count=1

.br

.

.br

.

.br

.

.br

#

.sp

--now just 'dd' the floppy filesystem into

an ordinary file in preparation for downloading

.sp

# dd if=/dev/r??? of=d bs=10b count=40

.br

#

.in0

.sp

.SH

Phase 2

.LP

.sp

.in+2

.ti-1

--downloading the maintenance floppy requires

.sp

.in+2

.ti-1

--make up an mtc (maintenance terminal cable, see Appendix B)

.sp

.ti-1

--free up a 9600 baud serial port on your running 2.9BSD system

(no login etc.) with a male end ready to plug the mtc. into it

.sp

.ti-1

a scratch rx50 floppy

.sp

.ti-1

an ascii terminal-setup, 9600 baud, 8 bit-no parity

.sp

.in-2

.ti-1

--plug the cable into the pro's printer port and the terminal

.sp

.ti-1

--power up the pro

.sp

.ti-1

--hit "break"

.br

you should now get an '@' indicating you are in ODT

.sp

.ti-1

--insert the scratch floppy in drive 0

.sp

.ti-1

--toggle in the primitive form of 'getfile' as per Appendix A

.sp

.ti-1

unplug the cable from the terminal and connect it to the serial

port on the 2.9BSD system

.sp

.ti-1

type '# putfile d /dev/tty??' on the 2.9BSD system

to initiate downloading

.sp

.in+2

-the floppy should be writing away for about 10 minutes

.in-2

.sp

.ti-1

--when 'putfile' returns, you should have a bootable

maintenance floppy

.sp

.ti-1

--boot the floppy via

.in+2

<break>

.br

@160000g

.br

.

.br

.

.br

.

.br

40boot

.br

:

.in0

.sp

.SH

Phase 3

.LP

.sp

.in+2

.ti-1

--setting up the hard disk

.sp

.ti-1

--format the hard disk

.sp

:r5(0,0)rdfmt

.br

.

.br

.

.br

.

.br

(0-RD51, 1-RD50)?0 <cr>

.br

.

.br

.

.br

.

.br

:

.sp

.ti-1

--download the / filesystem

.sp

:r5(0,0)getfile

.br

Controller#(0-R5, 1-RD, 2-RA)?1 <cr>

.br

Drive #0 <cr>

.br

Start Offset #64 <cr>

.br

Ready for putfile command

.sp

and now type "# putfile /dev/??? /dev/tty??

.br

to start the download

.br

NOTE: Use the block special file for the disk.

.br

(this takes about 1 1/2 hours)

.in0

.sp

.in+3

.ti-1

--now boot unix

.br

:rd(0,64)unix

.br

.

.br

.

.br

.

.sp

.ti-1

--it should come up single user on the console

screen/keyboard

.sp

.ti-1

--copy in RD bootstrap

.sp

# dd if=/rdboot of=/dev/rrd0h

.sp

--create a full dump of / on floppies

.sp

# sync; dump 0fs /dev/rr50 23 /dev/rrd0a

.sp

.ti-1

--you may now choose to download /usr in a similar

manner to / or to use 'mkfs' to make an empty file

system and add stuff as you go along

.sp

.ti-1

--to download with 'getfile' use start offset of 6464

.sp

.ti-1

--for 'mkfs' use

.sp

# mkfs /dev/rrd0c 6528

.sp

.in-3

.ul

Final Remarks

.in+3

.sp

.ti-1

--if you need to recover from a bad crash, you can

boot unix on the floppy and run mkfs and restore from the

maintenance floppy to recover the full dump on floppies

.sp

.ti-1

--you will then have to boot /unix using the floppy boot and use 'dd'

to put the RD bootstrap back.

.sp

.ti-1

--you can move individual files using the modified 'cu'

on the printer or com. port.

.sp

.ti-1

--for major transfers, getfile/putfile to a floppy

is the easiest.

.sp

.SH

Appendix A

.LP

.nf

@0/17777 12737<lf>

@2/17777 340<lf>

...

using the following data:

0000000 012737 000340 177776 005002 012706 157776 032737 000010

0000020 174204 001774 004567 000234 022700 000005 001373 005003

0000040 012701 001000 005037 174222 004567 000210 060003 010037

0000060 174226 077106 004567 000174 042703 177400 020003 001046

0000100 010205 010446 006305 005004 071427 000012 060405 011604

0000120 006304 060405 005004 071427 000012 005205 010537 174210

0000140 012604 005204 010437 174206 012737 000160 174204 005037

0000160 174224 032737 000010 174204 001774 105737 174204 100406

0000200 005202 012700 000007 004567 000036 000704 012737 000060

0000220 174204 005037 174224 032737 000010 174204 001774 012700

0000240 000011 004567 000002 000666 105737 177564 002375 010037

0000260 177566 000205 105737 177560 002375 013700 177562 000205

.sp

until 276/177777 205<cr>

then start execution with:

@R7/0 0<cr>

@p<cr>

the system should be now ready to connect the cable to the 2.9BSD line and

issue the 'putfile d /dev/tty??' command.

.sp

.SH

Appendix B

.LP

.sp

The mtc is cabled as follows:

pins	9 pin D		25 pin D

1		1

2		3

3		2

7		7

plus jumpers on 9 pin

5-6

8-9 (for maintenance purposes only)