VINST11.DOC

-----------

[Supplement to VDE.DOC]

Instuctions for VINSTALL 1.1 (01/88)

Installation Utility for Video Display Editor

MSDOS Version

(c)1988 E. Meyer

VDE.EXE comes installed for an IBM PC compatible computer.  This instal-

lation gives the best performance, but requires an IBM standard BIOS and video

adapter.  However, VDE can be re-installed for a generic MSDOS computer that

supports ANSI standard screen control sequences (eg, the ANSI.SYS screen

driver).  This generic installation can also be used on IBM compatibles, in

the event that direct video access is undesirable.

VINSTALL also gives you a number of User Options that can be changed to

adjust VDE's performance to your tastes; lets you install a default set of

Macro Key definitions; and customizes the VDE Print routines to take advantage

of your printer's special features.

Note: always keep an unmodified copy of both VDE.EXE and VINSTALL.EXE.

=============================  RUNNING VINSTALL  =============================

VINSTALL SYNTAX:    A>vinstall {d:}vdename{.exe}          "{}"=optional

Typically, you can use VINSTxx to modify VDExx.EXE (of course, feel free

to rename these files more conveniently) simply by typing:

A>VINSTxx VDExx.EXE

and selecting the values you want to change.

VINSTALL can use auxiliary data files to make life easier: printer

definition files and key definition files.  The recommended file types for

these are ".VDP" and ".VDK".  (VINSTALL can read VDK files created by CP/M

VINSTALL version 2.6.  The format is larger, but compatible as long as no more

than 512 bytes (0200h) are used.)

The version numbers of VDE, VINSTALL, and any ".VDx" data files you try

to read in must "agree" or you will get an error.  The match need not be

exact, as there may be a new release of VDE without modifying the installation

areas.

VINSTALL is largely self-documenting, and steps you through each item

individually.  If this is your first time running the program, take advantage

of its "Help" option before you start in, for some basic instructions.  You

may also find a good ASCII chart helpful.  VINSTALL uses several different

types of data entry:

TEXT - Just type in the character(s).

LOGICAL - Yes/No.  Just type "Y" or "N".

DECIMAL - Enter a number from 0 to 255.

HEX - Enter a two-digit hex byte value 00-FF.  (Don't omit lead zeros.)

BYTE STRINGS - fixed length: enter the required number of hex bytes.

variable: enter the (hex) count for the number of bytes,

followed by the bytes themselves.  Example:  (02)1B3D.  The max-

imum length is given in square brackets [#].  This format is

referred to below as "(#),bytes".

VINSTALL can't check for illegal values; be careful what you enter!

What follows is a more detailed explanation of installation items, in

about the same order as in the VINSTALL procedure.

INSTALLATION

Clock speed in MHz (01-FF) - Set this hex value to your CPU clock

speed; most are either 4.77, 8, or 10 MHz (use 48, 80, or A0 here).

Adjust the value if desired to change the length of delays.

IBM PC compatible? - Set this to "Y" for fast direct video access with

IBM compatible computers; or "N" for MSDOS standard character output.  The

rest of the installation depends on your choice here.

NON-IBM PARAMETERS

This group of items is generally relevant only if you answered "N" to the

IBM PC question above.  You can skip through them if you answered "Y".

(Exception:  change the "Lines" setting if your screen on entry to VDE isn't

25 lines.)

Viewable columns (80-255) - normally 80; set to the column width

of your display, if different.  (Automatically determined for IBM PC.)

Viewable lines (16-255) - normally 25; if you have a shorter or

longer display, enter the number of lines VDE should use.

Enable fast scroll (Y/N) - Does your computer respond to the ANSI

sequences for insert/delete line (ESC[L,M)?  If so, set this to "Y" for a

faster display.  (Default is "N", for MSDOS ANSI.SYS.  The VDE display

will not work properly if you select "Y" when your console driver does not

support these codes.)

IBM PC PARAMETERS

This group of items is relevant only if you answered "Y" to the IBM PC

question above.  You should skip through them if you answered "N".

Text and highlight attributes - you can select two video attribute

bytes, one for normal text and one for highlights (eg, header and menus).

Each is two hex digits, for a background and foreground color.  The color

digits are:  *0 = black   4 = red      *8 = grey        C = brt red

1 = blue    5 = magenta   9 = brt blue    D = brt magenta

2 = green   6 = brown     A = brt green   E = yellow

3 = cyan   *7 = white     B = brt cyan   *F = brt white

(*-Only 0,7,8,F work on monochrome systems.)  Any digit may be used for

the foreground, but only 0-7 for background (8-F produce blinking video...

ugh).  The defaults are 07 (white text on black) for normal video and 7F

(bright text on white) for highlights; this works on all systems.

Snow removal (Y/N) - Some older CGA systems experience ugly "snow" when

programs write directly to video memory.  If this happens to you, set this

option to "Y", and VDE will time its video access according to horizontal

retrace intervals.  (Will decrease display speed somewhat; default is "N".)

Use block cursor (Y/N) - When returning from EGA 43-line text mode,

VDE has to reset the cursor type.  Answer Yes for block, No for underline.

(A block cursor is always used in 43-line mode; an underline is too hard

to see.)  This parameter has no effect unless 43-line mode is used.

USER OPTIONS

Create BAKup files (Y/N) - Should a backup file be made when you edit

an existing file?  Choose "Y" to play safe, "N" to save disk space.

Default file mode (W/A/N) -  Which of the three file modes "W","A","N"

described in VDE.DOC do you want as the default?  (Originally "A".)

Default insert on (Y/N) - Do you want the INSERT toggle to start out ON

when first entering VDE?  (Originally No.)

Default ruler on (Y/N) - Do you want the Ruler line (^OT) to start out

ON when first entering VDE?  (Originally No.)

Default HCR display on (Y/N) - Do you want Hard CR display (^OD) to

start out ON when entering a Document ("W/A" modes) with VDE?  (Originally

Yes.)  The default is always OFF in "N" mode.

Enable hyphenation (Y/N) - Do you want hyphenation (^OH) to start out

ON when first entering VDE?  (Originally Yes.)

Left and Right margin columns (1-255) - Default margin settings for

"W/A" modes.  Set both to 1 to entirely disable wordwrap and reformatting.

Page length (0-255) - Set the number of text lines to print on a page.

(Does not include another 3 lines for a header, if used.)  Setting to 0

disables pagination entirely.

Scroll overlap (0-255) - This is the number of lines overlap between

screens when scrolling with ^R/^C.  The smaller, the larger the scroll.

Keep this smaller than your screen size!

Horizontal scroll delay (01-FF) - This hex parameter creates a delay

between scrolling the cursor line and the rest of the screen, when the

screen shifts horizontally.  For IBM PC compatibles this is unnecessary;

leave it set at 01.  For slower displays you may want to increase this

value to provide such a delay.

Ring bell on error (Y/N) - Do you want your terminal bell to ring when

a VDE error message displays?  (Default is No.)

BS (^H) deletes to left (Y/N) - Do you want the Backspace (^H) key to

delete characters?  Default is No - it just moves the cursor left.

DEL deletes to right (Y/N) - Do you want the Delete key to remove the

character to the right?  Default is No - it deletes to the left.

Block mark character (00-1F) - Usually 02 (^B), but if you need to

imbed that code in your files for any reason you can change this to some

other (otherwise unused) hex ASCII code, such as 00 (^@).

Wildcard character - Normally "_", this is the character that will be

taken as a wildcard (and case ignore flag) in FIND strings.

Hard Tab display spacing  -  This determines how any hard Tabs (^I)

in your text will display.  Usually 8, this can be made 2, 4, 16, or

any other power of two.  Other values will produce erratic results.

Variable tab stops -  Enter up to 8 variable tab stop columns, in

ascending order.  Fill unused values with 0.  Defaults: 6, 15, 35, 55.

Show time as AM/PM (Y/N) - Do you want the Time stamp to display in

12 hour format (eg 1:21 PM)?  Originally Yes.  If No, the time shows in

24 hour format (eg 13:21).

Show date as Day,Month (Y/N) - Do you want the Date stamp to display in

European format (eg 15 January)?  Originally No, meaning US Format (Month,

Day, eg January 15).

PRINTER INSTALLATION

VINSTALL can use Printer Installation files to store printer definitions.

These have a file type of "VDP".  You have the option to read in or write out

such a file, as well as editing the following values individually.

Printer ID - Identify the printer you are installing.  (16 characters)

Printer initialization - Enter a string of code (#),bytes to send to

the printer before each print job, to set margins, print modes etc.

Printer uninit - Enter another string (#),bytes to send to the printer

after each print job, to restore printer to desired state on exit.

Send LF after CR (Y/N) - Normally Yes.  But with some printers, which

manufacture their own LF on receiving a CR, you will want No here.

Toggles and switches - These are the ^P-codes that will be used to call

up predefined sequences.  There are six toggles, then six switches, all

entered as hex ASCII values.  The defaults (^D,^S,^Y,^T,^V; ^Q,^W,^E,^R)

are chosen for WordStar compatibility, but can be changed if you want.

(Avoid ^H,^I,^J,^L,^M,^Z, and your Block character, which are used for

other purposes.)  Fill in any unused values with FF.

Codes for toggles (on/off) - Enter the actual hex codes (#),bytes to

send to the printer on encountering each of the toggle codes defined

above, first to turn a feature on, then off.

Codes for switches - Enter the actual hex codes (#),bytes to send to

the printer on encountering each of the switches defined above.

MACRO KEYS

VINSTALL can use Key Definition files to store macro key sets.  These

have a file type of "VDK", and are compatible with those used by the CP/M VDE,

except that they will extend to 1024 bytes rather than 512.  You have the

option to read in or write out such a file, as well as editing the keys

individually.

The default macro definitions are those attached to the commands ESC-

0..9 when VDE is first entered.  By changing them you can configure VDE to

suit your own specialized writing requirements.  (Any Keys defined with

ESC-K while running VDE override the defaults.)

VINSTALL gives you a display much like this:

MACRO KEYS:  (03C6 bytes free)  <---- note room (up to 03F4 = 1012)

<0><>

<1><>                           <---- "<>" flag means empty

<2><>

<3>^[;^C                        <---- normal macro key

<4><>

<5><N>World Wide Widgets Inc.   <---- "<N>" flag means no-rpt key

<6><>

<7><Q>^QR^KB^QC^KK^KC           <---- "<Q>" flag means quiet key

<8><>

<9><>

This is pretty straightforward; you just select the number of the Key you

wish to redefine, then type in the new definition, much as you would in

VDE.  All the same rules apply (but VINSTALL will accept longer keys, up

to 128 bytes).  If the new definition is too long to fit it will be

rejected.

RESTORE DEFAULTS

This option is provided in the event that someone has so mucked up your

copy of VDE that you can't sort out what's wrong.  This should return VDE

to nearly the state it was distributed in: all User Options restored to

their defaults, printer set to a basic Teletype, no Macro Keys.

NOTE:  If you want, you can customize VINSTALL itself to return to any

set of default options (including printer and Macro Keys) you like.  Just

run VINSTALL on itself:        A>VINSTxx VINSTxx.EXE

When you save, the values you have selected will become the "defaults"

used by the Restore option in the future.

Eric Meyer, 427 N. Washington, Bloomington IN 47401  ---  CIS [74415,1305]

==============================================================================