August 12, 1982

TO: All CP/M assembly programmers

FROM: Thomas Hill

200 Oklahoma

Anchorage, Ak.  99504

(907) - 337-1984

SUBJECT: Undocumented CP/M BDOS Features

Just a short note to aquaint you with an "undocumented feature" I have

encountered in the CP/M 2.2 BDOS.  While developing an assembly program

which read and wrote disk files, an early version did not open the

output file before writing to it.  Oddly enough, the BDOS accepted the

write and did not return an error condition.  Being a curious soul

(and cautious), I sidetracked to investigate this effect.  A call to

Digital Research resulted in a letter informing me that they knew of the

effect and told me it was an "undocumented feature" of CP/M.  They also

told me that it was the programmer's responsibility to open and close

his files properly, to which a heartily agree.

However.  I wrote some test programs to determine WHERE on the disk the

information was going, and WHAT happened to the valid data on the disk.

Writing to an unopened file apparently writes information beginning at

Group 0, sector 1 and continues in a sequential manner thru the

allocation map.  (I lost three directories that way).  No change is made

in the allocation map, however, and the only change in the File Control

Block is the Current Record and Next Record fields are incremented.  NO

CHANGE occurs in the FCB allocation map.

While it is, of course, the programmer's responsibility to control the

file accesses, and proper opening and closing is mandatory, in some

cases (particularly during program development), proper file access may

not take place.  If this occurs, a possible loss of data may result.

There may be a BDOS patch which will clear this up, or someone out there

may already have one.  If anyone knows more about this, I would

appreciate it if you would drop me a line at the above address.

Thanks,

Thomas Hill

#########################################################