THE PDA PURE DATA CARD V.S.THE APPLE  COMM CARD(OR OTHER 6850 CARDS).

THERE ARE MANY KINDS OF APPLE COMM CARDS ON THE MARKET THESE DAYS AND ONE

OF THEM IS THE PURE DATA CARD CALLED THE PDA 232C CARD.  THIS CARD HAS

COME UNDER ATTACK FOR ITS INCOMPATABILITY WITH MOST OF THE SOFTWARE PACKAGES

AVAILABLE FOR COMMUNICATIONS.  THIS IS TRUE BECAUSE THE PDA CARD IS A

LOCALLY PRODUCED CARD AND IS NOT KNOWN IN THE STATES. HOWEVER, I BELIEVE

THAT IN THIS CASE THE LOCAL PRODUCT IS DEFINITELY THE BEST PIECE OF HARDWARE.

LETS LOOK AT THE HARDWARE ON THE PDA CARD AS OPPOSED TO THE APPLE COMM

CARD.  THE PDA CARD HAS A 8250 USART ON ITS BOARD.  THE APPLE COMM CARD

HAS A 6850 USART ON ITS BOARD.  THESE USARTS DO THE SAME BASIC JOB BUT

THE PDA'S USART DOES IT WITH FLAIR.  WHAT THIS REALLY MEAN IS THAT THE

8250 HAS MORE CONTROLLING REGISTERS ON IT THAN THE 6850.  THIS ALLOWS

BETTER CONTROL IF YOU ONLY KNOW THE  PROPER REGISTERS TO TICKLE.  IN THE

FOLLOWING REGISTER COMPARISON, I WILL ASSUME SLOT #2 AS THIS IS THE GENERALLY

ACCEPTED COMM SLOT FOR THE APPLE.

LETS COMPARE THE 8250 AND 6850 REGISTERS:

6850               8250

----               ----

ADDRESS   NAME    ADDRESS   NAME

-------   ----    -------   ----

HEX AE     STATUS  HEX A8    DATA

HEX AF     DATA    HEX A9    BAUD/RATE

HEX AA    INTERRUPT

HEX AB    LINE CTL

HEX AC    MODEM CTL

HEX AD    LINE STAT

HEX AE    MOD. STAT

AS YOU CAN SEE THERE ARE A FEW MORE REGISTERS IN THE 8250 THEN IN THE

6850 (THERE MAY BE MORE REGISTERS IN THE  6850, BUT THESE ARE THE WORK

HORSES). YOU WILL ALSO NOTICE THAT I HAVE ONLY GIVEN THE LOW ORDER ADDRESS

BYTE.   THIS IS BECAUSE WHEN USING APPLESOFT, THE HIGH ORDER BYTE WOULD

BE "C0", BUT WHEN ADDRESSING THE THESE BYTES FROM A CPM ASSEMBLER IT WOULD

BE "0E0". THUS THE DATA BYTE IN THE 8250 IS AT LOCATION C0A8 UNDER APPLE-

SOFT AND AT 0E0A8 IN A CPM 80 ASSEMBLER SOURCE CODE LISTING (MORE LATER).

LETS CARRY ON WITH THE 8250 AND LOOK  MORE EXTENSIVELY AT THE BITS IN THE

STATUS REGISTERS. HEX "A8" WE SAID WAS THE DATA REGISTER, WELL IT ALSO

DOES DOUBLE DUTY AS THE LOW ORDER BAUD RATE BYTE.  THIS MEANS THAT WE MUST

PREPARE THE 8250 BY SETTING  SOME VALUES INTO IT BEFORE WE USE IT TO ACTUALLY

DO SOME COMMUNICATION. I WILL SHOW YOU HOW THIS IS DONE IN AN EXAMPLE LATER.

ON WITH THE BITS:

REGISTER BITS     USE

-------- ----     ---

A8        0-7     BAUD RATE LSB/DATA

A9        0-7     BAUD RATE MSB/INTR.

AA        0-2     INTERRUPT IDENT.

AB        0       WORD LENTH SELECT

AB        1       WORD LENTH SELECT

AB        2       NUMBER STOP BITS

AB        3       PARITY ENABLE

AB        4       EVEN PARITY SELECT

AB        5       STICK PARITY

AB        6       SET BREAK

AB        7       BAUD RATE SET LATCH

AC        0       DATA TERM READY

AC        1       REQ TO SEND

AD        0       RECEIVE BUFF FULL

AD        1       OVER RUN ERROR

AD        2       PARITY ERROR

AD        3       FRAME ERROR

AD        4       BREAK INTERRUPT

AD        5       TX BUFFER EMPTY

AD        6       TX SHIFT REG EMPTY

AE        0       DELTA CTS

AE        1       DELTA DSR

AE        2       TRAILING EDGE RI

AE        3       DELTA RLSD

AE        4       CTS

AE        5       DSR

AE        6       RI

AE        7       RLSD

SLIPPED A FEW ACRONYMS IN ON YOU.

TO BE SUCCESSFUL AT USING THE 8250 USART CARD YOU MUST FIRST RESET THE

CARD, PLACE THE PROPER BAUD RATE INTO IT, AND THEN RESET IT AGAIN TO READY

IT TO FOR TX/RX DATA. WHEN THE 8250 IS RESET AND GOING YOU MUST MONITOR

THE LINE STATUS REGISTER "AD" BITS TO SEE WHEN DATA IS READY TO TRANSMIT

OR BE RECEIVED.  THIS IS DONE BY  MONITORING THE 0 AND 5 BITS IN THIS REGISTER.

SINCE A PICTURE IS WORTH A THOUSAND WORDS THEN MAYBE THE SAME IS TRUE

ABOUT AN EXAMPLE.  HERE IS AN EXAMPLE OF HOW TO RESET THE 6850 IN A CPM 80

ASSEMBLER LISTING.

*************************************

EXAMPLE OF AN 8080 LISTING

*************************************

;THE NEXT TWO INSTRUCTIONS RESETS THE

;MODEM CONTROL REGISTER AC

00N B:PDA80:OULD LI

STA   0E0ACH;TO MODEM CTR REG

;THE NEXT TWO INSTRUCTIONS SETS THE

;DIVISOR BIT LATCH ON IN THE LINE

;CONTROL REGISTER SO WE CAN SET UP

;THE PROPER BAUD RATE INTO A8 AND

;A9 REGISTERS.

MVI   A,83H ;SEND 10000011 TO

STA   0E0ABH;LINE CONTROL REG

;THE NEXT TWO INSTRUCTIONS SETS THE

;MSB OF THE BAUD RATE DIVISOR TO SET

;UP FOR 300 BAUD

MVI   A,80H ;SEND 10000000 TO

STA   0E0A9H;THE MSB/DATA REG

;THE NEXT TWO INSTRUCTIONS SETS THE

;LSB OF THE BAUD RATE DIVISOR TO SET

;UP FOR 300 BAUD

MVI   A,01H ;SEND 00000001 TO

STA   0E0A8H;THE LSB BAUD REG

;THE NEXT TWO INSTRUCTIONS SETS THE

;8250 UP TO GO BY RESETING THE BAUD

;RATE DIVISOR BIT AND SETTING THE

;WORD LENGTH BITS (FOR 8 BITS), IN

;THE LINE CONTROL REGISTER AB

MVI   A,03H ;SEND 00000003 TO

STA   0E0ABH;THE LSB BAUD REG

;THE NEXT INSTRUCTIONS DISABLE

;INTERRUPTS UNLESS YOU WANT TO GET

;FANCY AND USE THEM

MVI   A,00H ;SEND 00000000 TO

STA   0E0A9H;A9 REG

************************************

END OF FIRST PART OF EXAMPLE

************************************

THAT TAKES CARE OF THE RESET NOW ALL

WE NEED TO DO IS TO MONITOR THE 0 BIT

FOR RECEIVE DATA AND THE 5 BIT FOR

TRANSMIT DATA IN THE LINE STATUS

REGISTER AD

********************************

MORE LISTING

********************************

;IN THIS I/O ROUTINE THE NEXT TWO

;INTRUCTIONS WOULD MONITOR FOR

;INCOMING DATA

INPUT    LDA   0E0ADH;GET LINE STATUS

ANI   01H   ;RECEIVE DATA?

JZ    INPUT ;LOOP IF NOT

;IN THIS TRANSMIT ROUTINE THE NEXT

;TWO INTRUCTIONS TEST THE TX BIT 5

;

OUTPUT   LDA   0E0ADH;GET LINE STATUS

ANI   20H  H;XMIT BUFF EMPTY

JZ    OUTPUT;LOOP IF YES

*************************************

END OF EXAMPLE LISTING

*************************************

THE PREVIOUS EXAMPLES ARE ONLY PIECES OF 8080 ASSEMBLER AND NOT COMPLETE,

BUT ITS ONLY THE SETUP AND TESTING IDEA THAT I'M TRYING TO SHOW.

WELL THATS THE IDEA OF HOW YOU CAN SET UP THE REGISTERS IN THE 8250

USART AND GET THE THING GOING.  THE BEST WAY TO GET THE HANG OF THE THING

IS TO GET SOME SOFTWARE THAT HAS SOURCE CODE AVAILABLE AND GO TO IT.

WITH THE REGISTERS UNDER YOUR CONTROL YOU CAN MAKE THE PDA STAND ON ITS

EAR.

IN CONCLUSION I WOULD LIKE TO SAY THAT THE HARDWARE OF THE PURE DATA

CARD CONTAINS THE SAME 8250 USART AS THE IBM PC, WHICH SHOULD SAY SOMETHING

ABOUT ITS QUALITY. THE FACT THAT THERE IS A SHORTAGE OF AVAILABLE SOFTWARE

MAY BE A CONSIDERATION; HOWEVER, PURE DATA HAS SOME AVAILABLE FOR CPM USE($20).

RON BLOOR

#################################################################