NorthStar Phantom Line Modification:

In order to configure memory in the upper 8K of memory, it is

necessary to "phantom out" the 1K block of address space where the

bootstrap PROM resides.  First, the memory board which will overlap

the disk controller (E800H-EBFFH) needs to have the phantom option

enabled by installing the "PH" jumper near the edge connector.  The

following modifications should then be made to the ZPB processor

board:

1.  Install the PROM address decoders (74LS136) at locations 5C

and 5D.

2.  Install the PROM address socket and header at location 5E.

The header should be addressed for E800H by connecting:

a.  Pin 1 to 8, 10

b.  Pin 7 to 9, 11, 12, 13

Pin 13 represents the most significant bit of the address and pin

8 represents address bit 8.  Pins connected to pin 7 are 1's and to

pin 1 0's.  If the PROM were located at FC00H, pins 8-13 would be

connected to pin 7.

3.  Install pullup resistor R10 (1K).

4.  Remove or cut the "PE" jumper (top of board) to enable the

PROM-select signal.

5.  Cut the trace on the solder side between 7C pin 5 and the

feed through at 5C.  This enables the processor board to read data

from the disk controller memory area.

6.  Connect a jumper from the feedthrough at 5C (from step 5) to

6B (74367) pin 10.  This is the PROM-select signal.

7.  Install a jumper from 6B pin 9 to pin 67 (phantom line) on

the edge connector.

8.  Bend out pin 3 on 6E (74LS03) or cut the trace on the

component side between 6E pin 3 and 6F (74LS02) pin 3.  This will

disable the circuitry which generates a wait state for the PROM

option.

This hardware modification should "phantom out" a 1K block of memory.

North Star's engineers disagree, but it seems to work.

#######################################################################################