ONZCPR21.DOC					February 4, 1983

ONZCPR21.HEX is a version of NZCPR21 configured for use on an Osborne 1

with ROM version 1.4 or 1.41 -- previous versions found on a number of

RCPM's didn't work on the double density Osborne 1 for the following

reasons.

The double density boot program loads CP/M as furnished at CB00h since it has

a TPA 1k less than the standard CP/M for single density machines (59k vs.

60k).   In addition, the double density boot program expects to see C3,5C as

the first 2 bytes of the CCP (CB00/1h for 59k CP/M).   If not it prints BOOT

ERROR and tries again.   The third byte of the CCP is the load address + 3 in

addition to being part of the initial jump.   Thus C3,5C,CE found as the first

three bytes of the 59k CCP satisify the boot program and cause CP/M to

be loaded at CB00h (CEh - 3 = CBh).

To configure NZCPR for the DD Osborne, it is necessary to "ORG" NZCPR at

0CB00h and edit the initial jump (to CPR) to 0CE5Ch.  The easiest way to

fix the jump is to insert a JMP CPR at the source location which will assemble

to 0CE5Ch even if you have to use an ORG 0CE5Ch to do it.   You will also need

to insert JR LABEL and LABEL: statements on either side of the JMP CPR

instruction in order to maintain program flow around the added JMP CPR.

You'll probably want to do a trial assembly first in order to find

the best location since NZCPR must fit into 800h bytes.

When you have a assembled file that looks OK you can follow the

integration instructions in ZCPR.DOC using SYSGEN and DDT.  The offset

when using DDT to overlay the original CCP in CPM59.COM with ONZCPR is

03E80h.

Please excuse the hacked up code necessary to make NZCPR work with

the 1.4 ROM, it seemed like the only way that others could easily

use in assembling their own versions of NZCPR.

It may also be of help in adapting ZCPR2 (when available) to the Osborne 1.

ONZCPR21.HEX also includes Matt Wilson's fix for a bug in ERA: which was in

NZCPR-16 and is still in NZCPR-21 (see NZCPRERA.FIX on the Allentown RCPM

and perhaps other systems).

While you're modifying CP/M, if you want a version that will skip the AUTOST

routine, change 201Ch from 01 to 00 in cpm59.com (the next bytes should be

07,AUTOST).   This will allow you to boot directly to the CP/M prompt when

you log on to the system using <CR> or <">.   If you want to automatically

print the directory when you boot, you can leave 201Ch as 01 and change the

following bytes to 04,DIR (04 = the number of letters in the command + 1).

Obviously you can cause other commands or programs to load and execute auto-

matically by making similar changes.   If you want to make the changes directly

on the system tracks of a disk, you can use your favorite disk utility program

to change T1,S6 (PS5), byte 1c, etc.

Having just had a chance to start looking through a copy of the Osborne

technical manual ($49.95 at many Osborne dealers), I see that much of what I've

covered above is shown in much more detail including schematics, prom and cbios

listings.   If I'd had the manual earlier, it would have saved a lot of work

and eyestrain -- but then I wouldn't have had the fun of discovering for my-

self some of what goes on in the prom and cbios.

WARNING!!

If you're not experienced in making changes of these types, I would recommend

caution and the use of a disk copy that you can afford to lose.   If you are

experienced, then you've probably already learned the need for caution and the

use of a disk copy that you can afford to lose.

Good luck.

Jim Cornelius

Broomall, Pa.

January 16, 1983

February 4,1983  -- DOC file version 2 -- Clarified text; added AUTOST info.;

added trademark acknowledgements; added comments regarding

Osborne Technical Manual.

------------------------------------------------------------------------------

Osborne 1 is a trademark of Osborne Computer Corporation

CP/M is a registered trademark of Digital Research, Inc.###############################################################################################################################################################################################################################################################################################################################################