!@#$   INTRODUCTION

WARNING ++ this pac expands to 102K ++ have you the room?

YAM (Yet Another Modem program) by Chuck Forsberg is a modem

program providing a superset of the MODEM7 commands.  It will

do almost anything MODEM7 will do, and a great many other things

besides.  It has been adapted to support a large number of modems

and computers.  This distribution package has extra files needed

to support the PMMI MM-103 modem, but support for other modems,

including those commonly used with APPLE and TRS80-II are avail-

able.  (For APPLE and TRS80 support call the Forsberg RCPM in

Oregon, the phone number is listed in the comments in YAM.H).

YAM will allow you to switch drives, switch disks, erase files (on

your own system), estimate the time required to send a file or

group of files, and send or receive files without disturbing the

save buffer from the terminal mode.  If you configure it for a

UART serial port instead of a modem you can dump file between com-

puters at up to 19200 baud.  There is an XMODEM like version (XYAM)

available from the same files by recompiling the same component files

and changing one header file #define.  And there is more.

YAM will support batch file transfers, but is NOT compatible with

MODEM7 in this mode.  This YAM with the PMMI modem insists, that it

dial the phone from its phone number file, and not you with your

fingers.  (Someone is likely to fix that however, fairly soon).

The files in this concatenation are separated by the "!@#$",

CR,LF character string.  All files except yam1.dif and

yamphone.t are identified in the first or second line with

>>:name.  To use these files to get a working copy of yam,

you will have to break this file into its component parts,

edit a version of YAMSYS.H to match your system, and compile

the files using the BDS C compiler version 1.44, or newer.

The yam[12345789(10)] files were the most recent versions of

yam as of mid May 1982, and were taken from Chuck Forsberg's

RCPM system in Oregon.  A note on the Forsberg system warned

that the files containing yam were the newest ones available,

experimental, not the offically distributed version, and not

guaranteed.  But fear not, they do seem to work all right.

Don't call Chuck and complain, though.

Yam5pmmi.c is based on the CPMUG yampmmi.doc with enhancements

by:

Paul Homchick

One Radnor Station #300

Radnor, PA 19087

These enhancements include ringback-autodial, a flip command

(between orginate and answer modes), timing loop bug fixes,

and modem error checking using the MIREADYERROR version of

readline().

To use the flip command do this: >>>a0:flip ans

or this: >>>a0:flip org

To implement the flip feature you must modify the yam1.c file

using dif.com with yam1.dif.  Also, be sure to use the yam.h

file included in this pac, as it has a new FLAG (Ansorg) defined.

The following files are included in this package:

yam.sub

yaml2.sub

yam.h

yamz89.h  (an example of a yamsys.h file)

yampmmi.h (another example)

yam1.c

yam2.c

yam3.c

yam4.c

yam5.c

yam5pmmi.c (special pmmi dependent routines installed)

yam7.c

yam8.asm

yam9.asm

yam10.c

yamphone.t (note tab chars after name and before baud. You

gotta have them.)

yam1.dif (use with SSED to use pmmi flip feature)

Because of the complexity of this program, unless you normally

practise ESP, you will probably need other documentation.  See

YAM.DOC and YAM.RNO, available on many RCPM systems.

######