Heath/Zenith Disk Capacities

and patch for "FORMAT.COM"

By Larry Shipinski  02/14/82

The following information has been compiled for your reference.

For the Z89-37 Dual Density Disk Controller: (5")

48 TPI Drives (Wango/Seimans)

Single	Double	Extended  <--Density

Tracks		  40      40       40

Sect/Track	  20	  32	   40

Grp size	   8	   8	    8

Tot. Grps	   91	 151	  189

Dir Enrty	  64	 128	  128

Sys Tracks	   3	   2	    2

Disk Capacity    90K     148K     186K

96 TPI Drives (Tandon FD100-4)

Single	    Double	 Extended  <--Density

1      2    1      2      1      2  <--# sides

Tracks	       80   160    80   160      80   160

Sect/Track     20    20    32    32      40    40

Grp size        8    16    16    16      16    16

Tot. Grps     191   195   155   315     194   394

Dir Entry      64   128   128   256     128   256

SYS tracks      3     3     2     2       2     2

Disk Capacity 190K  388K  308K  624K    386K  782K

NOTE: Sides and Density are operator options of "FORMAT.COM".

You can select "Extended Double Density" on your 5"

disks by typing an "E" to the density prompt or apply

the patch below to your format program.

Patch for FORMAT.COM :

Use DDT to make the following changes in FORMAT.COM.

ddt format.com     <-- get FORMAT.COM with DDT

DDT VERS 2.2

NEXT  PC

1780 0100

s13e7              <-- start replacing our data

13E7  73  53

13E8  69  44

13E9  6E  2C

13EA  67  20

13EB  6C  44

13EC  65  3D

13ED  2C  44

13EE  20  44

13EF  44  2C

13F0  3D  20

13F1  64  45

13F2  6F  3D

13F3  75  45

13F4  62  44

13F5  6C  44

13F6  65  29

13F7  29  20

13F8  3A  .       <-- end

g0		  <-- get out of DDT

save 23 FORMAT.COM    <-- save the revised program

Now when you use FORMAT.COM you will be prompted for the

"Extended Double Density" when using 5" drives.

(end)			L. Shipinski

###############################################################################################