DTMF tone generation and detection

An implementation using the TMS320C54x

Gunter Schmer, MTSA

SC Group Technical Marketing

Rev. 2.0

10/16/96

[The important notice page is required in all application reports. This page is the reverse side of the title page and contains the TI disclaimer and the copyright clause. Always use the latest important notice as provided from the Relational Document Manager (RDM) file. This notice is not typed, changed, corrected, or in any way altered from the RDM file.]

abstract

This application note describes the implementation of a DTMF tone generator and detector for the TMS320C54x. This application note provides some theoretical background on the algorithms used for tone generation and detection. It documents the actual implementation in detail. Finally the code is benchmarked in terms of its speed and memory requirements.

DTMF touch tone dialing: background

A DTMF (Dual Tone Multiple Frequency) codec incorporates an encoder that translates key strokes or digit information into dual tone signals, as well as a decoder detecting the presence and the information content of incoming DTMF tone signals. Each key on the keypad is uniquely identified by its row and its column frequency as shown in Figure 1. The DTMF generating and decoding scheme is not too computationally extensive and can easily be handled by a DSP concurrently with other tasks. This article describes an implementation of the DTMF codec on the TMS320C54x, TI’s fixed-point DSP designed especially for telecommunication applications.

Column Frequency Group

1209Hz    1336Hz    1477Hz    1633Hz

1

2

3

A

4

5

6

B

7

8

9

C

*

0

#

D

697 Hz

770 Hz

Row

Frequency

Group

852 Hz

941 Hz

Figure 1 	Touch-tone telephone keypad: A row and a column tone is associated with each digit

DTMF tone generator

The encoder portion and tone generation part of a DTMF codec is based on two programmable, second order digital sinusoidal oscillators, one for the row the other one for the column tone. Two oscillators instead of eight facilitate the code and reduce the code size. Of course for each digit that is to be encoded, each of the two oscillators needs to be loaded with the appropriate coefficient and initial conditions before oscillation can be initiated. As typical DTMF frequencies range from approx. 700 Hz to 1700 Hz, a sampling rate of 8 kHz for this implementation puts us in a safe area of the Nyquist criteria. Figure 2 displays the block diagram of the digital oscillator pair. Table 1 specifies the coefficients and initial conditions necessary to generate the DTMF tones.

For the one who is interested in some more detail, appendix A gives some refreshing theoretical background and a guideline for determining coefficients and initial conditions for digital sinusoidal oscillators.

Tone duration specifications by AT&T state the following: 10 digits/sec are the maximum data rate for touch tone signals. For a 100 msec time slot the duration for the actual tone is at least 45 msec and not longer than 55 msec. The tone generator must be quiet during the remainder of the 100 msec time slot.

| f/Hz | a1 | y(-1) | y(-2)/A |
| --- | --- | --- | --- |
| 697 -1 | 0.85382 | 0 | -0.52047 |
| 770 | 0.82263 | 0 | -0.56857 |
| 852 | 0.78433 | 0 | -0.62033 |
| 941 | 0.73911 | 0 | -0.67358 |
| 1209 row tone column tone a1 a1 | 0.58206 | 0 | -0.81314 |
| 1336 | 0.49820 | 0 | -0.86706 |
| 1477 -1 | 0.39932 | 0 | -0.91680 |
| 1633 | 0.28424 | 0 | -0.95874 |

Figure 2	Two second-order digital 	Table 1 Coefficients and initial conditions    sinusoidal oscillators

Program flow description of the DTMF generator

For the following description of the program flow it is helpful to simultaneously consult the flowchart given in Figure 3.    Essentially the series of keypad entries (digits) will be translated into a series of dual-tones of certain duration which are interrupted by pauses of certain duration. Later the dual-tones will enable the decoder to identify the associated digits. The pauses are also necessary to discriminate between two or more identical digits entered successively.

Therefore the program flow incorporates two tasks, which are swapped after certain time intervals. One task (the “tone-task”) is to generate dual-tone samples, the other one (the “quiet-task”) is to generate pause samples. Each task is assigned a certain duration which is controlled by a timer variable. At the end of each task, the task has to initialize the timer variable and the task-name (tone or quiet) for the next task to be invoked. At the end of the quiet-task one very important component is added: A new digit is retrieved from the digit buffer and is unpacked. Unpacking means that the digit is mapped to the row/column tone properties (oscillator coefficients, initial conditions) and pointers are loaded, pointing to the appropriate locations in the oscillator property table.

The entire program flow is synchronized to the receive interrupt service routine, which provides us with a perfect clock for real-time processing and constant sample output. On completion of the RINT_ISR the task scheduler is invoked, which determines the particular task (tone or quiet) that needs to be executed. Both tone-task and quiet-task check on the timer variable to determine if the end of the task duration is already reached. If not, a tone or quiet sample respectively is generated. If yes, the next task name and duration is initialized and will start to execute with the completion of the next RINT_ISR. The quiet-task additionally unpacks the next digit at the end of its duration.

Task scheduler	“quiet-task”	“tone-task”

START

TASK

== 0 ?

1

0

set

TASK = 1

TIMER = TIME_LG

load new digit

increment DIGIT_PTR

unpack

new digit

decrement

TIMER

generate

quiet sample

RET

TIMER

== 0

0

set

TASK = 0

TIMER = TIME_SH

decrement

TIMER

generate

dual tone sample

RET

TIMER

== 0

1

Figure 3      Flowcharts of the DTMF encoder implementation

DTMF tone detector

The task to detect DTMF tones in a incoming signal and convert them into actual digits is certainly more complex then the encoding process. The decoding process is by its nature a continuous process, meaning it needs to search an ongoing incoming data stream for the presence of DTMF tones continually.

Collecting spectral information

The Goertzel algorithm is the basis of the DTMF detector. This method is a very effective and fast way to extract spectral information from an input signal. This algorithm essentially utilizes two-pole IIR type filters to effectively compute DFT values. It thereby is a recursive structure always operating on one incoming sample at a time, as compared to the DFT (or FFT) which needs a block of data before being able to start processing. The IIR structure for the Goertzel filter incorporates two complex-conjugate poles and facilitates the computation of the difference equation by having only one real coefficient. For the actual tone detection the magnitude (here squared magnitude) information of the DFT is sufficient. After a certain number of samples N (equivalent to a DFT block size) the Goertzel filter output converges towards a pseudo DFT value vk(n), which can then be used to determine the squared magnitude. See Figure 4 for a short mathematical description of the algorithm. More detail is provided in Appendix B.

Goertzel Algorithm in short:

1. Recursively compute for n = 0 .. N

2. Compute once every N

Figure 4      Goertzel Algorithm

The Goertzel algorithm is much faster than a true FFT, as only a few of the set of spectral line values are needed and only for those values are filters provided. Squared magnitudes are needed for 8 row/column frequencies and for their 8 second harmonics. The second harmonics information will later enable us to discriminate DTMF tones from speech or music. Table 2 contains a list of frequencies and filter coefficients. The choice of N is mainly driven by the frequency resolution needed, which sets a lower boundary. N also is chosen so that (k/N)fs most accurately coincides with the actual DTMF frequencies (see Table 1) assuming ks are integer values and fs is a sampling frequency of 8 ksps.

| 1st Harmonics (N = 102) fs = 8ksps |  |  |  | 2nd Harmonics (N = 102) fs = 8ksps |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DTMF | k | frequency (k/N)fs/Hz | coefficient cos(2pi k/N) | 2nd Harm. | k | frequency (k/N)fs/Hz | coefficient cos(2pi k/N) |
| 697 | 9 | 706 (+1.2%) | 27860 | 1394 | 18 | 1412 (+1.2%) | 14606 |
| 770 | 10 | 784 (+1.9%) | 26745 | 1540 | 20 | 1569 (+1.9%) | 10891 |
| 852 | 11 | 863 (+1.3%) | 25529 | 1704 | 22 | 1725 (+1.3%) | 7010 |
| 941 | 12 | 941 (0.0%) | 24216 | 1882 | 24 | 1882 (+0.0%) | 3023 |
| 1209 | 15 | 1176 (-2.7%) | 19747 | 2418 | 31 | 2431 (+0.5%) | -10891 |
| 1336 | 17 | 1333 (-0.2%) | 16384 | 2672 | 34 | 2667 (-0.2%) | -16384 |
| 1477 | 19 | 1490 (+0.9%) | 12773 | 2954 | 38 | 2980 (+0.9%) | -22811 |
| 1633 | 21 | 1647 (+0.9%) | 8967 | 3266 | 42 | 3294 (+0.9%) | -27860 |

Table 2    List of frequencies, frequency deviations and filter coefficients (true DFT)

The graphs of Figure 5 are from a Mathcad simulation. We can clearly see, how the Goertzel filters resonate with an input signal of matching row/column frequency (case 1) and how they do not resonate, when the input signal does not match their frequency (case 2).    The Mathcad simulation data is provided in Appendix C.

800 Hz

1483 Hz

squared

magnitude

squared

magnitude

858 Hz

1483 Hz

n=102

n=102

11                          19

k

11                            19

k

n

n

Figure 5	Mathcad simulation case 1:    858Hz, 1483Hz    --> digit = 9 case 2:    800Hz, 1483Hz    --> digit = undefined

Modifications to the Goertzel Algorithm

The coefficients of Table 2 reflect the coefficients needed to recursively compute the true DFT. The evenly spaced frequency bins of a true DFT present an inherent drawback in the DTMF tone detection process. The DFT frequency bins mostly deviate from the true DTMF frequency by an amount in the range of up to 2% off center frequency. To be able to meet the acceptable bandwidth specifications a modification of the algorithm departs from the true DFT and tunes frequency bins exactly with the DTMF tone frequencies. This modification gives up the DFT property of evenly spaced frequency bins and with that takes two calculated risks: (1) A frequency bin is possibly moved inside of the mainlobe of its neighboring frequency bin. Therefore neighboring frequency bins can affect each other. Note, that the mainlobe of the continuous magnitude spectrum of a rectangular windowed sinewave (window is N wide) is exactly the distance of 2 DFT frequency bins. (2) This is especially true when column frequencies and 2nd harmonics of row frequencies lie close to oneanother. Note that column frequencies and 2nd harmonics of row frequencies share the same frequency band.

To minimize these risks, two rules were applied: (1) Frequency bins shall be spaced apart by more than 0.9/fs. (2) If rule 1 can not be achieved, 1st harmonics frequency bin placement shall have a higher priority than 2nd harmonic bin placement. With these rules applied, the coefficient table given in Table 2 is modified. Essentially all necessary fundamental frequency bin can be tuned to their DTMF frequency and only in two cases of 2nd harmonics a small frequency deviation needed to be accepted. The new method of using tuned frequency bins facilitates the checking of acceptable bandwidths of DTMF tones and helps meet    the acceptable bandwidth specifications. The modified table of coefficients is given in Table 3.

| 1st Harmonics (N = 102) fs = 8ksps |  |  |  | 2nd Harmonics (N = 102) fs = 8ksps |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DTMF | k | frequency (k/N)fs/Hz | coefficient cos(2pi k/N) | 2nd Harm. | k | frequency (k/N)fs/Hz | coefficient cos(2pi k/N) |
| 697 | 8.88 | 697 | 27980 | 1394 | 17.93 | 1406 (+0.9%) | 14739 |
| 770 | 9.82 | 770 | 26956 | 1540 | 19.72 | 1546 (+0.4%) | 11414 |
| 852 | 10.86 | 852 | 25701 | 1704 | 21.72 | 1704 | 7549 |
| 941 | 12.00 | 941 | 24216 | 1882 | 24.00 | 1882 | 3032 |
| 1209 | 15.42 | 1209 | 19073 | 2418 | 30.83 | 2418 | -10565 |
| 1336 | 17.03 | 1336 | 16325 | 2672 | 34.07 | 2672 | -16503 |
| 1477 | 18.83 | 1477 | 13085 | 2954 | 37.66 | 2954 | -22318 |
| 1633 | 20.82 | 1633 | 9315 | 3266 | 41.64 | 3266 | -27472 |

Table 3    List of tuned frequencies and modified filter coefficients

Validity checks

Once the spectral information in form of squared magnitude at each of the row and column frequencies and their second harmonics is collected, a series of tests need to be executed to determine the validity of tone and digit results.

A first check makes sure the signal strength of the possible DTMF tone pair is sufficient. The sum of the squared magnitudes of the peak spectral row component and the peak spectral column component need to be above a certain threshold (THR_SIG). Since already small twists (row and column tone strength are not equal) result in significant row and column peak differences, the sum of row and column peak provides a better parameter for signal strength than separate row and column checks. Tone twists are investigated in a separate check to make sure the twist ratio specifications are met.

The spectral information can reflect to types of twists. The more likely one, called “reverse twist”, assumes the row peak to be larger than the column peak. Row frequencies (lower frequency band) are typically less attenuated as than column frequencies (higher frequency band), assuming a low-pass filter type telephone line. The decoder computes therefore a reverse twist ratio and sets a threshold (THR_TWIREV) of 8dB acceptable reverse twist. The other twist, called “standard twist”, occurs, when the row peak is smaller than the column peak. Similarly, a “standard twist ratio” is computed and its threshold (THR_TWISTD) is set to 4dB acceptable standard twist.

The program makes a comparison of spectral components within the row group as well as within the column group. The strongest component must stand out (in terms of squared amplitude) from its proximity tones within its group by more than a certain threshold ratio (THR_ROWREL, THR_COLREL).

Finally, the program checks on the strength of the second harmonics in order to be able to discriminate DTMF tones from possible speech or music. It is assumed that the DTMF generator generates tones only on the fundamental frequency, however speech will always have significant even-order harmonics added to its fundamental frequency component. This second harmonics check therefore makes sure that the ratio of second harmonics component and fundamental frequency component is below a certain threshold (THR_ROW2nd, THRCOL2nd). If the DTMF signal pair passes all these checks, we say, a valid DTMF tone pair, which corresponds to a digit, is present.

We now need to determine if the valid DTMF tone information contains stable digit information. This is done by mapping the tone-pair to its corresponding digit and comparing it with the previously detected digit. We call the digit information stable if it has been detected twice successively.

Finally we compare the detected digit with the previous to last digit. Only if the last digit was preceded by a pause then we accept the current digit as a valid digit. The detector is then forced into a state where it is waiting for a pause before being able to accept a new digit. This last step is necessary to ensure the discrimination of identical keystrokes succeeding one another.

Program flow description of the DTMF detector

This DTMF decoder implementation uses a task scheduler in a similar fashion as the encoder, again synchronized to the completion of the receive interrupt service routine. The receive ISR is continuously filling a buffer of N = 102 words with incoming data. Once the buffer is full it signals the full buffer by setting the input data status word indatastat to 1. The task scheduler essentially keeps polling indatastat and once it recognizes a full input data buffer (indatastat = 1) it initiates the DTMF detection process by calling the appropriate functions. (again consulting the flowcharts of Figure 6 might be very helpful at this point):

First the input data status word is reset to signal that the detection process has been initiated. Then the content of the input data buffer is copied into an intermediate buffer for processing (double buffering). All the detection functions will then operate on the intermediate buffer. The gain control function attenuates strong signal inputs and protects the succeding functions from overflow of the accumulators. Then the Goertzel filters are executed. Since the preceding gain control ensures that overflow cannot occur, overflow checking was removed and optimized loops allow fast execution. The output of the Goertzel function are the delay states of the 16 filters which are collected in an array.

On completion of the Goertzel function the digit validation checks are invoked. The spectral information is computed from the filter delay states and collected in an energy template. For the next round of execution the filter delay states are initialized to zero. The energy template is then searched for row and column energy peaks. From then on the detector essentially operates in two modes: The tone/digit detection mode or the pause detection mode. In the tone/digit detection mode the detector searches for DTMF tone presence and executes all the digit validation tests. In the pause detection mode DTMF tone detection is disabled and the decoder first has to await a pause signal. Tone/digit or pause modes are controlled by the detectstat variable. Digit validation checks include signal strength, reverse and standard twist, relative peaks, second harmonics and digit stablility. With the successful completion of these tests the valid digit is stored into the digit output buffer.

Multichannel DTMF tone detection

The software is written as C-callable reentrant functions. This enables the user to setup multichannel tone detectors in C without adding significant additional code. In order to facilitate and structure multichannel applications, the code uses a structure to hold all the global variables and pointers to various arrays for a single channel. All the user has to do, is to define a structure for each channel and initialize it properly. A pointer to the structure of a desired channel is passed to the C-callable functions and the detection takes place.

new flow-

charts!!!

digit validation

checks

buffer ready

indatastat == 1 ?

gain control

attenuate

strong signals

Goertzel filters

for 1st and 2nd

harmonics

Yes

No

set

indatastat = 0

RET

START

1

Energy

Computation

Initialize filter taps

to zero

Search for

energy maxima

set

detectstat = 1

Map

detected tone pair

to digit

Store digit in buffer

store digit

as valid result

Pause detection mode

Tone/Digit

detection mode

Yes

No

No

Yes

No

Yes

RET

valid digit !

valid tones !

same digit

as last ?

Detector status:

detectstat == 1 ?

1

Sig. ener

ok?

Twists

ok?

Rel. Peaks

ok?

2nd Harm.

ok?

Is this a

Pause ?

Figure 6      Flowcharts of the DTMF decoder implementation

Speed and memory requirements

The following Table 3 summarizes the speed and memory requirements of the DTMF encoder/decoder implementation. The encoder as well as the decoder occupy only a very small portion of the computing capability of this 50 MIPS DSP. The maximal MIPS count for the actual DTMF encoder is approx. 0.45 MIPS. The isolated DTMF decoder uses approx. 0.82 MIPS. These speed specifications include all processing necessary after the completion of the receive interrupt service routine. Also a sampling rate of 8000 samples/sec is assumed. The MIPS performance of the DTMF decoder is achieved using a buffered concept instead of a sample by sample concept. The speed critical Goertzel-DFT function has the 16 filters coded inline for fast execution. Due to an improved gain control the typical overflow check within the goertzel routine was omitted. Drawback of the buffered concept is the need for additional data memory due to double buffering of data. The amount of data memory is to a large degree given by the size of the input data buffers, which are in this application set to N = 102 length. Tables 4 and 5 summarize the benchmarks for the DTMF encoder as well as decoder

| Module Name | Tasks included | Program Mem. | Data Mem. | max. cycles per sample | MIPS |
| --- | --- | --- | --- | --- | --- |
| DTMF encoder (sample by sample) | task scheduler tone task quiet task | 129 | 49 | 57 | 0.45 |
| HW/SW initialization I/O | hwinit swinit RINT_ISR | 90 | -- | -- | -- |
| TOTAL |  | 219 | 49 | 57 | 0.45 |

Table 4	Speed and memory requirements for the DTMF encoder

| Module Name | C-functions | Program Mem. | Data Mem. (n channels) | max. cycles per 102 samp | MIPS |
| --- | --- | --- | --- | --- | --- |
| DTMF decoder (buffered) | some overhead gain control Goertzel-DFT DTMF checks | 572 | 260*n + 32 | 100 705 8802 788 | 0.008 0.055 0.690 0.062 |
| HW/SW initialization I/O | ISRs inits C-environment | 351 | -- | -- | -- |
| TOTAL |  | 923 | 260*n + 32 | 10395 | 0.82 |

Table 5	Speed and memory requirements for the DTMF decoder

For very tight MIPS requirements in multichannel applications it is possible to even further reduce the MIPS count of the DTMF decoder to within a range of 0.55 to 0.6 MIPS per full DTMF channel. The modifications of the code would involve the conditional execution of the 2nd harmonics portion of the Goertzel DFT. In the “tone present phase” only two 2nd harmonic spectral lines need to be computed, whereas in the “pause present phase” 2nd harmonic spectral information can be omitted. These changes to the code would not affect the performance of the decoder.

Performance

Two well-known tests to evaluate the performance of DTMF decoders are available through MITEL and Bell Communications Research (Bellcore), which both supply the associated test tapes and test procedures.

MITEL tests

The DTMF tone decoder has been tested using the MITEL test procedures. For a preliminary test a set of files with the digitized signal data for the various MITEL tests was acquired through TI internal sources. The files essentially contain a subset of the signal data of the real MITEL test tapes. Since non of the files contained more than 32k words of data, the files were reformatted as “.inc”-files and then included in the source code. At linktime the data of a given test file was mapped into external memory of the TMS320C54xEVM occupying at maximum 32k words of data space. The code was then tested in real-time using the contents of the 32k-words test buffer as its input source. During this preliminary test the various decoder thresholds were set to proper levels.

The complete MITEL test was then executed according to the test procedures specified in the MITEL test document. A digital audio tape (DAT) recording of the test tapes was used as input signal source. The decoder was executed on the TMS320C54xEVM utilizing a TMS320AC01 analog interface to convert the incoming signal into the digital domain. The MITEL test essentially has two sections. The first section measures the DTMF tone decoder in terms of Recognition Bandwidths (RBW), Recognition Center Frequency Offset (RCFO), Standard Twist, Reverse Twist, Dynamic Range (DR), Guard time and Signal-to-Noise Ratio (SNR). The second section, called “talk-off test”, consists of recordings of conversations on telephone trunks made over a long period of time and condensed into a 30 minute period. In this section the decoder’s capabilities to reject other sources such as speech and music is measured. MITEL specifies a maximum of 30 responses of the DTMF decoder as acceptable speech-rejection.

Table 1 summarizes the MITEL test results. With the exception of the recognition bandwidth results for the low-band frequencies, the DTMF decoder passes all the tests and exceeds the specifications. The slightly higher relative bandwidths for the low-band frequencies were accepted in favor of a higher robustness. Essentially the signal strength threshold THR_SIG2 determines to a large part the recognition bandwidths. THR_SIG2 has been set to leave slightly more room for digit acceptance and higher robustness. The threshold settings for acceptable twists helped pass the twist test and exceed the specifications. The dynamic range of the decoder of 28dB is better than the specification. The guard time of 30ms leaves enough head room to safely detect digits with a specified 45ms tone duration. The decoder was able to correctly detect all the 1000 tone bursts for each of the given noise environments of -24dBV, -18dBV and -12dBV AWGN. When the decoder was exposed to the 30 minutes of speech and music samples, it did not respond a single time and exceeded the MITEL talk-off specification of 30 permissible responses by a significant margin.

Table 1.    MITEL test results

|  | BW tests | frequency | RBW% | RCFO% |
| --- | --- | --- | --- | --- |
|  | specification |  | 1.5% < RBW < 3.5% |  |
|  | Low band | 697 Hz | 5.8% | 0.10% |
|  |  | 770 Hz | 5.5% | 0.05% |
|  |  | 852 Hz | 5.1% | 0.05% |
|  |  | 941 Hz | 4.5% | 0.05% |
|  | High band | 1209 Hz | 2.8% | 0.10% |
|  |  | 1336 Hz | 2.5% | 0.05% |
|  |  | 1477 Hz | 2.3% | 0.15% |
|  |  | 1633 Hz | 2.4% | 0.00% |
|  | TWIST tests | Std twist | rev twist |  |
|  | specification | > 4 dB | > 8 dB |  |
|  | DIGIT 1 | 4.7 dB | 8.5 dB |  |
|  | DIGIT 5 | 4.6 dB | 8.6 dB |  |
|  | DIGIT 9 | 4.5 dB | 8.3 dB |  |
|  | DIGIT 16 | 4.6 dB | 8.5 dB |  |
|  | DR tests | dyn range |  |  |
|  | specification | > 25 dB |  |  |
|  | DIGIT 1 | 28 dB |  |  |
|  | DIGIT 5 | 28 dB |  |  |
|  | DIGIT 9 | 28 dB |  |  |
|  | DIGIT 16 | 28 dB |  |  |
|  | guard time | min pause time | min tone time |  |
|  | specification |  | 45 ms |  |
|  | DIGIT 1 | 13 ms | 30 ms |  |
|  | SNR tests | Noise | result |  |
|  | specification | - 24 dBV |  |  |
|  | DIGIT 1 | - 24 dBV | passed |  |
|  | DIGIT 1 | - 12 dBV | passed |  |
|  | DIGIT 1 | - 18 dBV | passed |  |
|  | Talk-off test | decoder responses | result |  |
|  | specification | < 30 |  |  |
|  | result | none | very robust |  |

BELLCORE talk-off test

Through TI internal sources the Bellcore series-1 Digit Simulation Test Tapes for DTMF receivers were available for testing. These tapes consist of six half-hour sequences of speech samples, designated parts 1 through 6, which are known to contain energy at or near valid DTMF frequency pairs. This test exhaustively measures the speech-rejection capabilities of DTMF receivers in telecommunication systems. There are over 50,000 speech samples including some music. It is estimated that the six parts of the series-1 tapes are equivalent to the exposure of one million customer dialing attempts in a local central office. In other words, exposing a DTMF receiver to all the speech samples in series-1, will produce the same number of digit simulations that the receiver would experience if it were exposed to customer speech and room noise present during network control signaling on one million calls. The Bellcore talk-off test is far more exhaustive than the MITEL talk-off test.

The test setup was identical to the one used for the MITEL talk-off. Table 2 summarizes the test results for the Bellcore talk-off. In the 3 hours of testing the decoder responded only in 6 cases to digit simulations. This is far less than the specifications require to pass the talk-off. The decoder proved to be very robust in terms of its speech-rejection capabilities.

Table 2. Bellcore talk-off test results

|  | test | digits | specification | results |
| --- | --- | --- | --- | --- |
|  | part 1 through 6 (3 hours) | 0 - 9 | per 1,000,000 calls < 333 responses | 6 responses |
|  | part 1 through 6 (3 hours) | 0 - 9, *, # | per 1,000,000 calls < 500 responses | 6 responses |
|  | part 1 through 6 (3 hours) | 0 - 9, *, #, A, B, C, D | per 1,000,000 calls < 666 responses | 6 responses |

Summary

DTMF tone encoding and decoding concepts and algorithms were described here in some detail. Further theoretical background is provided in the appendix. The DTMF encoder and decoder implementations were explained and the associated speed and memory requirements were presented. The DTMF tone decoder has been tested according to the MITEL and BELLCORE test specifications and the results are documented. It is important to note that the decoder was implemented as reentrant, C-callable functions, which facilitate setting up a multi-channel DTMF decoder system. The code is modular and easy to integrate into any given telephony application. The decoder algorithm was optimized to meet the test specifications as well as offer a very attractive MIPS count of approximately 0.82 MIPS per channel.

Appendix A — background: Digital oscillators

Digital Sinusoidal Oscillators

A digital sinusoidal oscillator may in general be viewed as a form of a two pole resonator for which the complex-conjugate poles lie on the unit circle. It can be shown that the poles of a second order system with system function

(A.1)

with parameters

are exactly located at the unit circle. That is

(A.2)

The discrete-time impulse response

(A.3)

corresponding to the above second order system clearly indicates a clean sinusoidal output due to a given impulse input. We can therefore term our system a digital sinusoidal oscillator or digital sinusoidal generator.

For the actual implementation of a digital sinusoidal oscillator the corresponding difference equation is the essential system descriptor, given by

(A.4)

where initial conditions y(-1) and y(-2) are zero. We note that the impulse applied at the system input serves the purpose of beginning the sinusoidal oscillation. Thereafter the oscillation is self sustaining as the system has no damping and is exactly marginally stable. Instead of applying a delta impulse at the input, we let the initial condition y(-2) be the systems oscillation initiator and remove the input. With this in mind our final difference equation is given by

(A.5)

where

with fs being the sampling frequency, fo being the frequency and A the amplitude of the sinusoid to be generated. We note that the initial condition y(-2) solely determines the actual amplitude of the sinewave.

Appendix B — background: Goertzel Algorithm

Goertzel Algorithm

Body Text for definition

As the first stage in the tone detection process the Goertzel Algorithm is one of the standard schemes used to extract the necessary spectral information from an input signal. Essentially the Goertzel algorithm is a very fast way to compute DFT values under certain conditions. It takes advantage of two facts:

(1) 	The periodicity of phase factors  allows to express the computation 	of the DFT as a linear filter operation utilizing recursive difference 	equations

(2)	Only few of the set of spectral values of an actual DFT are needed (in 	this application we have 8 row/column tones plus an additional 8 tones 	for corresponding 2nd harmonics)

Having in mind that a DFT of size N is defined as

(B.1)

we can indeed find the sequence of a one-pole resonator

(B.2)

which has a sample value at n = N coinciding exactly with the actual DFT value. In other words each DFT value X(k) can be expressed in terms of the sample value at n = N resulting from a linear filter process (one-pole filter).

We can verify, that

(B.3)

The difference equation corresponding to the above one-pole resonator sequence (B.2), which is essential for the actual implementation, is given by

(B.4)

with y(-1) = 0 and pole location . Being a one-pole filter, this recursive filter description yet contains complex multiplications, not very convenient for a DSP implementation. Instead we utilize a two-pole filter with complex conjugate poles  and only real multiplications in its difference equation

(B.5)

where vk(-1) and vk(-2) are zero.

Only in the Nth iteration a complex multiplication is needed to compute the DFT value, which is

(B.6)

However the DTMF tone detection process does not need the phase information of the DFT and squared magnitudes of the computed DFT values in general suffices. After some mathematical manipulation we find

(B.7)

In short: For the actual DSP implementation equations (B.5) and (B.7) will be used retrieve the spectral information from the input signal x(n) for further evaluation. Note that equation (B.5) is the actual recursive linear filter expression, which is looped through for n = 0 .. N.    Equation (B.7) is only computed once every N samples to determine the squared magnitudes.

Appendix C — DTMF tone generator code listing

DTMF tone generator executable on a TMS320C54x EVM

**************************************************************

*   (C) COPYRIGHT TEXAS INSTRUMENTS, INC. 1996               *

**************************************************************

*   Program Name:     DTMF tone generator                    *

*   File Name:        dtmf100e.asm                           *

*   File Description: This file contains the code for a      *

*                     DTMF tone generator                    *

*                     (TMS320C54x EVM version)               *

*                                                            *

*   Author:   Gunter Schmer                                  *

*   Date:     08/20/96                                       *

*   Revision: 2.0                                            *

*   Latest working date:  08/20/96                           *

**************************************************************

.title	"dtmf encoder"

.mmregs

.include  "varse.inc"		;define global vars and consts

.include  "sectse.inc"		;define global tables and buffers

.include  "globalse.inc"	;globalized labels

******************************************

*       CODE STARTS HERE

******************************************

.text

START

ssbx	INTM	;global interrupt disable

ld	#0,DP	;initialize data pointer

;------------ Initialize Hardware ------------------------------------

call	hwinit	;hardware initialization subroutine

;------------ Initialize Software ------------------------------------

ssbx    SXM	;data sign ext. before usage

rsbx    OVM	;no saturation of accu if overflow

ssbx	FRCT	;fractional mode bit, left shift of

;  multiplier to compensate for extra

;  sign bit

stm	#0040h,IMR	;enable RINT1 interrupt

stm	#00c8h,IFR	;clear all pending serial port or timer interrupts

rsbx	INTM	;global interrupt enable

ld	#aic_conf,DP	;DP to variables

mvmd	DRR1,rcv	;dummy read

mvdm	tra,DXR1	;dummy write

******************************************

*       Main Program

******************************************

Main

st	#PH_NBR,DIGIT_PTR  ;initialize pointer to PHNBR table

st	#0,TASK	;initialize beginning task

st	#0,TIMER	;initialize timer to zero

stm	#DATA,AR5	;AR5 points to testbuffer

next	idle	1

call	tasks	;actual processing following RINT interrupt

nop

;--- store to test buffer ----

;stm	#7fffh,BK

;ld	tra,A

;stl	A,*AR5+%

b	next

done	b	done

************************************************************

* Task Scheduler:

* Run this task scheduler section at the beginning of

* each RINT_ISR process

*

*

************************************************************

tasks	cmpm	TASK,#00h

bc	task1,NTC	;if(TASK!=0) branch to task1

;  else branch to task0

task0	cmpm	TIMER,#00h

bc	task01,NTC	;branch if timer not zero

mvdm	DIGIT_PTR,AR1	; AR1 points to digit in PHNBR table

st	#01h,TASK	; TASK of next RINT_ISR is task1

st	#TIME_LG,TIMER	; set timer to long duration

ld	*AR1+,A	; load A with digit

bc	done,alt	; done if digit is -1 !

mvmd	AR1,DIGIT_PTR	; save new pointer to digit in PHNBR table

call	unpack	; unpack digit: A --> T1_OFS, T2_OFS

b	task02

task01	ld	TIMER,A

sub	#1,A

stl	A,TIMER	;decrement timer

task02	call	quiet

b	task3	;branch to task3

task1	cmpm	TIMER,#00h

bc	task11,NTC	;branch if timer not zero

st	#00h,TASK	; TASK of next RINT_ISR is task0

st	#TIME_SH,TIMER	; set timer to short duration

b	task12

task11	ld	TIMER,A

sub	#1,A

stl	A,TIMER	;decrement timer

task12	mvdm	T1_OFS,AR2	;AR2 is offset for row-tone

mvdm	T2_OFS,AR3	;AR3 is offset for column-tone

call	tone

task3	ret

******************************************

* Interrupt Service Routines

******************************************

RINT1_ISR:

ld	#rcv,DP

mvmd	DRR1,rcv

andm	#0fffch,tra

mvdm	tra,DXR1

popm	ST1

popm	ST0

rete

.end

;varse.inc

***************************************

***   Constants

***************************************

TIME_SH	.set	440	;pause duration (55 msec)

TIME_LG	.set	360	;tone duration  (45 msec)

DAC_OFS	.set	000h	;DAC offset

***************************************

***   Variables

***************************************

.bss    aic_conf,1

.bss	rcv,1	;receive variable

.bss	tra,1	;transmit variable

.bss	DIGIT,1	;current digit

.bss	DIGIT_PTR,1	;points to current digit in PHNBR table

.bss	TIMER,1	;timer counter

.bss	T1_OFS,1	;offset for tone 1 (row tone)

.bss	T2_OFS,1	;offset for tone 2 (column tone)

.bss	TASK,1	;holds the nbr of the task that is to be executed

;sectse.inc

***************************************

***   Tables

***************************************

*******************************************************

* The following table assembles the coefficients and  *

* initial conditions for the difference equations of  *

* the digital sinusoidal oscillators.                 *

*                                                     *

* In general:                                         *

*   DEQ:    y(n) = 2*cos(2pi*f/fs)*y(n-1) - y(n-2)    *

*   I.C.:   y(-1) = 0                                 *

*           y(-2) = -A*sin(2pi*f/fs)                  *

*   where   A  = desired amplitude of sine wave       *

*           f  = desired frequency of sine wave       *

*           fs = sampling frequency                   *

*                                                     *

* Example:                                            *

*       .word    cos(2pi*f/fs)*32768	;coefficient  *

*       .word    0			;y(-1)        *

*       .word    -A*sin(2pi*f/fs)*32768	;y(-2)        *

*                                                     *

*******************************************************

.sect   "tbl_tone"

TONES   .word	27980	;row 1	coef

.word	0          	;	y(n-1)

.word	-1024*5204/10000 ;	y(n-2)

.word	26956	;row 2	coef

.word	0	;	y(n-1)

.word	-1024*5686/10000 ;	y(n-2)

.word	25701	;row 3	coef

.word	0	;	y(n-1)

.word	-1024*6203/10000 ;	y(n-2)

.word	24219	;row 4	coef

.word	0	;	y(n-1)

.word	-1024*6736/10000 ;	y(n-2)

.word	19073	;col 1	coef

.word	0	;	y(n-1)

.word	-1024*8132/10000 ;	y(n-2)

.word	16325	;col 2	coef

.word	0	;	y(n-1)

.word	-1024*8671/10000 ;	y(n-2)

.word	13085	;col 3	coef

.word	0	;	y(n-1)

.word	-1024*9168/10000 ;	y(n-2)

.word	9315	;col 4	coef

.word	0	;	y(n-1)

.word	-1024*9587/10000 ;	y(n-2)

*****************************************************************

* The following table contains offsets into the                 *

* tone table tbl_tone.                                          *

* The offset for the row-tone is the (upper byte) of each word  *

* The offset for the column-tone is the (lower byte + #12)      *

*                                                               *

*****************************************************************

.sect	"tbl_keys"

KEYS	.word	0903h	; '0'  example: 0903h --> 3.row, 1.column (*3)

.word	0000h	; '1'

.word	0003h	; '2'

.word	0006h	; '3'

.word	0300h	; '4'

.word	0303h	; '5'

.word	0306h	; '6'

.word	0600h	; '7'

.word	0603h	; '8'

.word	0606h	; '9'

.word	0009h	; 'A'

.word	0309h	; 'B'

.word	0609h	; 'C'

.word	0909h	; 'D'

.word	0900h	; 'E' = '*'

.word	0906h	; 'F' = '#'

****************************************************************

* The following table contains the phone number to be          *

* encoded. The format is as follows                            *

*                                                              *

* PH_NBR   .word   4,2,7,0Eh,0Fh,0Ah,3,-1                      *

*                                                              *

* is the foll nbr: 4 2 7 * # A 3   (-1 terminates the encoding)*

****************************************************************

.sect	"tbl_phnr"

PH_NBR	.word	1,2,3,4,5,6,7,8,9,0,0Ah,0Bh,0Ch,0Dh,0Eh,0Fh

.word	1,2,3,4,5,6,7,8,9,0,0Ah,0Bh,0Ch,0Dh,0Eh,0Fh

.word	1,2,3,4,5,6,7,8,9,0,0Ah,0Bh,0Ch,0Dh,0Eh,0Fh

.word	1,2,3,4,5,6,7,8,9,0,0Ah,0Bh,0Ch,0Dh,0Eh,0Fh

.word	1,2,3,4,5,6,7,8,9,0,0Ah,0Bh,0Ch,0Dh,0Eh,0Fh

.word	1,2,3,4,5,6,7,8,9,0,0Ah,0Bh,0Ch,0Dh,0Eh,0Fh

.word	1,2,3,4,5,6,7,8,9,0,0Ah,0Bh,0Ch,0Dh,0Eh,0Fh

.word	1,2,3,4,5,6,7,8,9,0,0Ah,0Bh,0Ch,0Dh,0Eh,0Fh

.word	1,2,3,4,5,6,7,8,9,0,0Ah,0Bh,0Ch,0Dh,0Eh,0Fh

.word	1,2,3,4,5,6,7,8,9,0,0Ah,0Bh,0Ch,0Dh,0Eh,0Fh

.word	1,2,3,4,5,6,7,8,9,0,0Ah,0Bh,0Ch,0Dh,0Eh,0Fh

.word	1,2,3,4,5,6,7,8,9,0,0Ah,0Bh,0Ch,0Dh,0Eh,0Fh

.word	1,2,3,4,5,6,7,8,9,0,0Ah,0Bh,0Ch,0Dh,0Eh,0Fh

.word	1,2,3,4,5,6,7,8,9,0,0Ah,0Bh,0Ch,0Dh,0Eh,0Fh

.word	1,2,3,4,5,6,7,8,9,0,0Ah,0Bh,0Ch,0Dh,0Eh,0Fh

.word	1,2,3,4,5,6,7,8,9,0,0Ah,0Bh,0Ch,0Dh,0Eh,0Fh

.word	-1

******************************************

* TEST BUFFER

******************************************

DATA	.usect	"testbuf",8000h

******************************************

*       Stack setup

*****************************************

BOS     .usect  "stack",20h	; setup stack

TOS     .usect  "stack",1	; Top of stack at reset

**************************************************************

*   (C) COPYRIGHT TEXAS INSTRUMENTS, INC. 1996               *

**************************************************************

*   Program Name:     DTMF tone generator                    *

*   File Name:        dtmfsube.asm                           *

*   File Description: This file contains subroutines for a   *

*                     DTMF tone generator                    *

*                     (TMS320C54x EVM version)               *

*                                                            *

*   Author:   Gunter Schmer                                  *

*   Date:     08/20/96                                       *

*   Revision: 2.0                                            *

*   Latest working date:  08/20/96                           *

**************************************************************

.mmregs

.include  "globalse.inc"

*****************************************************************

* SUBROUTINE:  unpack                                           *

* Description: Maps the key value (in A) into two offsets for   *

*              two tones and writes results into variables      *

*              T1_OFS, T2_OFS                                   *

* Uses:        AR2                                              *

* Input:       A                                                *

* Output:      none                                             *

*****************************************************************

unpack	stlm	A,AR2	;AR2 keys-map offset for unpacking

ld	#DIGIT,DP	;DP to variables

nop		;pipeline conflict(1) + latency(1) with AR2

ld	*AR2(KEYS),A	;load A with keys-map value

ld	*AR2(KEYS),B	;load B with keys-map value

and	#0f00h,A	;mask out the row portion

and	#000fh,B	;mask out the column portion

sftl	A,-8	;right shift A by 8

add	#12,B	;add 4*3 words offset to point into column portion

stl	A,T1_OFS	;store tone 1 offset (row tone)

stl	B,T2_OFS	;store tone 2 offset (column tone)

ret

*****************************************************************

* SUBROUTINE:  tone                                             *

* Description: Generates the dual tone samples for DTMF         *

*              using offsets T1_OFS, T2_OFS                     *

* Uses:        AR2, AR3                                         *

* Input:       none                                             *

* Output:      none                                             *

*****************************************************************

tone	ld	#0,B	;clear B  ----ROW TONE----

sub	*AR2(TONES+2),15,B ; (B) = -(1/2)y(n-2), in high accumulator

ltd	*AR2(TONES+1)	; load (T) with y(n-1) and y(n-1) --> y(n-2)

mpy	*AR2(TONES),A  ; (A) = coef*y(n-1)

add	A,B	; (B) = coef*y(n-1) - (1/2)y(n-2)

sth	B,1,*AR2(TONES+1) ; 2*(B) --> y(n-1)

ld	#0,B	;clear B  ----COLUMN TONE----

; b	tone1	; test only one oscillator

sub	*AR3(TONES+2),15,B ; (B) = -(1/2)y(n-2), in high accumulator

ltd	*AR3(TONES+1)	; load (T) with y(n-1) and y(n-1) --> y(n-2)

mpy	*AR3(TONES),A  ; (A) = coef*y(n-1)

add	A,B	; (B) = coef*y(n-1) - (1/2)y(n-2)

sth	B,1,*AR3(TONES+1) ; 2*(B) --> y(n-1)

tone1	add	*AR2(TONES+1),15,B ; add two tone samples

add	#DAC_OFS,B	; add DAC offset

sth	B,tra	; write to transmit variable

ret

*****************************************************************

* SUBROUTINE:  quiet                                            *

* Description: Generates a pause of specified duration          *

*                                                               *

* Uses:        AR2, AR3                                         *

* Input:       TIME_SH                                          *

* Output:      none                                             *

*****************************************************************

quiet	ld	#0,B	; clear B

add	#DAC_OFS,B	; add DAC offset

sth	B,tra	; write to transmit variable

ret

Appendix D — DTMF tone detector code listing

DTMF tone detector executable on a TMS320C54x EVM

/*************************************************************

*   (C) COPYRIGHT TEXAS INSTRUMENTS, INC. 1996               *

**************************************************************

*   Program Name:     DTMF tone decoder                      *

*   File Name:        globals.h                              *

*   File Description: defines channel information            *

*                                                            *

*   Author:   Gunter Schmer                                  *

*   Date:     10/15/96                                       *

*   Revision: 2.0                                            *

*   Latest working date: 10/15/96                            *

*************************************************************/

/* globals */

/* struct used to pass on channel info to functions */

typedef struct

{

int *indata;	/* ptr to indata 			*/

int *taps;	/* ptr to filter states 		*/

int *energy;	/* ptr to energy template 		*/

int *digitptr;	/* ptr to digit output array 		*/

int digitlast;	/* last detected digit 		*/

int detectstat;	/* status of detector 			*/

/*  0 - detector waiting for pause 	*/

/*  1 - detector ready to detect digit */

int err_flags;	/* error flags for dtmf checks	*/

/* b0 - error signal strength		*/

/* b1 - error reverse twist		*/

/* b2 - error standard twist		*/

/* b3 - error row's relative peak	*/

/* b4 - error col's relative peak	*/

/* b5 - error row's 2nd harmonic	*/

/* b6 - error col's 2nd harmonic	*/

/* b7 - error OVERFLOW of ACCA	*/

/* b8..b15 are zero			*/

} DTMFCHANNEL;

/* --------------- channel#1 information --------------------------------*/

int  indata1[102];	/* receive buffer 			*/

int  indata2[102];	/* process buffer 			*/

int  digits[128];	/* output buffer for digit results 	*/

int  taps[32];		/* filter states  			*/

int  energy[16];	/* energy template 			*/

/* used for interrupt servicing and signaling a full buffer */

int  *indataptr = &indata1[0];/* ptr to receive buffer 		*/

int  indatastat = 0;	/* status of receive buffer 		*/

/*   0 = buffer not ready to process 	*/

/*   1 = buffer ready to process 	*/

/* initialized DTMFCHANNEL */

DTMFCHANNEL 	channel1 = {	&indata2[0],

&taps[0],

&energy[0],

&digits[0],

0xFFFF,

1,

0	};

DTMFCHANNEL 	*ptrchannel1 = &channel1;

/* -------------- end of channel#1 --------------------------------------*/

/* used for testing */

volatile unsigned int *testdataptr = (volatile unsigned int *) 0x8000;

/*************************************************************

*   (C) COPYRIGHT TEXAS INSTRUMENTS, INC. 1996               *

**************************************************************

*   Program Name:     DTMF tone decoder                      *

*   File Name:        main.c                                 *

*   File Description: calls init functions                   *

*                     calls decoder functions                *

*                                                            *

*   Author:   Gunter Schmer                                  *

*   Date:     10/15/96                                       *

*   Revision: 2.0                                            *

*   Latest working date: 10/15/96                            *

*************************************************************/

#include "init.h"

#include "globals.h"

#include "mmregs.h"

#include "dtmfsub.h"

/* prototypes */

void tasks(void);

void initArrays(void);

main()

{

/*********** init HW and SW ************/

initWSGen();		/* initialize WS generator */

initArrays();	/* initialize arrays	*/

initSP1();		/* initialize SP1          */

initAC01();		/* initialize AIC          */

initInts(0x0040);	/* allow only RINT1        */

/*********** synch tasks() to RINT0 *********/

for(;;)   {

asm("	IDLE  1");	/* wait for interrupts             */

tasks();		/* run tasks on completion of ISR  */

}

}

void tasks(void)

{

int n;

if(indatastat==1)  {

/* reset indata status bit */

indatastat = 0;

/* copy indata1 to indata2 */

for(n=0;n<102;n++)

indata2[n] = indata1[n];

/* perform gain control */

gaincntrl(ptrchannel1);

/* perform goertzel     */

goertzel(ptrchannel1);

/* perform dtmf digit validation */

dtmfchecks(ptrchannel1);

}

}

void initArrays(void)

{

int n;

for(n=0;n<102;n++)   {

indata1[n] = 0;

indata2[n] = 0;

}

for(n=0;n<128;n++)

digits[n] = 0;

for(n=0;n<32;n++)

taps[n] = 0;

for(n=0;n<16;n++)

energy[n] = 0;

}

**************************************************************

*   (C) COPYRIGHT TEXAS INSTRUMENTS, INC. 1996               *

**************************************************************

*   Program Name:     DTMF tone decoder                      *

*   File Name:        dtmfsub.asm                            *

*   File Description: This file contains the subroutines     *

*                     used for a DTMF tone detector          *

*                     (TMS320C54x EVM version)               *

*   Author:   Gunter Schmer                                  *

*   Date:     10/15/96                                       *

*   Revision: 2.0                                            *

*   Latest working date: 10/15/96                            *

**************************************************************

.mmregs

.include  "globals.inc" ;global labels

.include  "tables.inc"	 ;various tables

*******************************************************************

* SUBROUTINE: gaincntrl (approx. 705 cycles)

*

* Input: none

* Output: none

* Uses:

*

* Description: Scales a block of input data in order to ensure

*              no overflow when running the goertzel routine

*              approx. 490 cycles per N=102 sample frame

*              approx. 5 cycles   per sample (negligable)

*

*              gain_power = (1/64) * SUM{0,N-1}[x(n)*x(n)]

*                         = (1/64) * N * signalpower

*

*              gain_lim   = (1/64) * (1/N)

*                         = minimum signalpower (-..dB)

*

*              gain_const = 0.5*sqrt(2)*sqrt(64/N)

*              gain_amp   = gain_const * sqrt(gain_power)

*              gain_lev   = 1/N

*              gain_scale = gain_lev/gain_amp

*

*              x(n) = x(n) * gain_scale

*

* FRCT=1, SXM=1

*******************************************************************

.text

_gaincntrl:

nop

pshm	ST0

pshm	ST1

pshm	AR1

pshm	AR6

pshm	AR7

nop

frame	#-10	;---- local variables -----

; *SP(0) = gain_pow (HI)

; *SP(1) = gain_pow (LO)

; *SP(2) = gain_lim (HI)

; *SP(3) = gain_lim (LO)

; *SP(4) = gain_amp

; *SP(5) = gain_lev

; *SP(6) = gain_scale

; *SP(7) = gain_const

; *SP(8) = gain_delta

; *SP(9) = ptr to indata

;---- arguments -----------

; (A)    = ptr to dtmfchannel

stlm	A,AR5

nop

ld	*AR5,A

stl	A,*SP(9)

;---- value initializations --------------------------

ld	#0005h,16,A	;(A)  = 00050000h

or	#0505h,A	;(A)  = 00050505h = 1/(64*N) (Q31)

sth	A,*SP(2)	;(A)  --> gain_lim  (32bit)

stl	A,*SP(3)

ld	#00E3h,A	;(AL) = 00E3h = (0.5*sqrt(2))/N

stl	A,*SP(5)	;(AL) --> gain_lev  (16bit)

ld	#6564h,A	;(AL) = 6564h = sqrt(64/102)

stl	A,*SP(7)	;(AL) --> gain_const

ld	#4000h,A	;(AL) = 4000h = 0.5

stl	A,*SP(8)	;(AL) --> gain_delta

ssbx	FRCT

;---- compute signal power ---------------------------

ld	*SP(9),A

stlm	A,AR2	;AR2 points to indata2 buffer

rptz	B,#(N-1)	;for(n=0;n<N-1;n++)

squra	*AR2+,B	;  (B)=(B)+x(n)*x(n)

sfta	B,-6	;(B)=(1/64)*(B)

sth	B,*SP(0)	;(B) --> gain_pow   (32bit)

stl	B,*SP(1)	;        gain_pow = (N/64)*sigpower

ld	*SP(2),16,A	;(A) = gain_lim = 1/(64*N)  (32bit)

or	*SP(3),A

sub	A,B	;(B) = gain_pow - gain_lim

bc	gain2,BLEQ	;if(gain_pow > gain_lim)

;{

;---- compute gain_amp = sqrt(gain_pow) -------------

st	#4000h,*SP(4)	;   initialize gain_amp estimate

st	#4000h,*SP(8)	;   initialize gain_delta to 0.5

mvmm	SP,AR2

mar	*+AR2(8)	;   AR2 points to gain_delta

stm	#(sqrt_iterations-1),BRC

rptbd	sqrtloopend-1

squr	*SP(4),A	;   (A) = yold*yold

dsub	*SP(0),A	;   (A) = yold*yold - x

sqrtloop

bcd	sqrt1,AGT

ld	*SP(4),16,B	;   (B) = yold

sub	*AR2,15,B	;   (B) = yold - (1/2)*delta

;   if(y needs to be larger)

add	*AR2,16,B	;     (B) = yold + (1/2)*delta

sqrt1	sth	B,*SP(4)	;   (BH) --> ynew

ld	*AR2,15,A	;   (AH) = (1/2)delta

sth	A,*AR2	;   (AH) --> delta new

squr	*SP(4),A

dsub	*SP(0),A	;   (A) = yold*yold - x

sqrtloopend

;---- compute gain_amp = sqrt(64/N)*gain_amp --------

ld	*SP(7),T	;   (T) = gain_const

mpy	*SP(4),A	;   (A)=sqrt(64/102)*gain_amp

sth	A,*SP(4)	;   (AH) --> gain_amp

;---- compute gain_lev/gain_amp ---------------------

ld	*SP(5),16,A	;   (AH) = gain_lev

rpt	#(16-1)

subc	*SP(4),A	;   compute (gain_lev/gain_amp)

and	#0ffffh,A	;   retain quotient (AL), mask remainder

sfta	A,15	;   shift quotient into high accu

sth	A,*SP(6)	;   (AH) --> gain_scale

;---- scale indata with (gain_lev/gain_amp) ---------

ld	*SP(9),A	;

stlm	A,AR2	;   AR2 points to indata

mvmm	SP,AR3	;

mar	*+AR3(6)	;   AR3 points to gain_scale

stm	#(N-1),BRC	;

rptb	gain1-1	;   for(n=0;n<N;n++)

mpy	*AR2,*AR3,A	;      (A) = x(n)*(gain_lev/gain_amp)

sth	A,*AR2+	;      (AH) --> x(n)

gain1			;}

gain2	frame	#10

nop

popm	AR7

popm	AR6

popm	AR1

popm	ST1

popm	ST0

ret

***********************************************************************

* SUBROUTINE: goertzel         (w/o OV check approx. 8861 cycles)

*

* Input:  none

* Output: none

* Uses:

*

* Description:

* This new version of the subroutine operates on a block of data

* 102 samples long and computes the following difference equation

* It does thereby not check for overflow and assumes a prescaling

* gain control function to protect from possible overflow.

*

* vk(n) = 2*coef*vk(n-1) - vk(n-2) + x(n)

*

* Coefficients are in order starting with location COEF1st

* COEF1st .word	xxxx	;coef1

*	  .word	yyyy	;coef2

*	  ....

*

* Data vk(n-1), vk(n-2) is ordered as follows

*	v15(n-1)

*	v15(n-2)

*	v14(n-1)

*	v14(n-2)

*	....

*	v0(n-1)

*	v0(n-2)  <--AR2

*

* FRCT=1, SXM=1

*

***********************************************************************

_goertzel:

nop

pshm	ST0

pshm	ST1

pshm	AR1

pshm	AR6

pshm	AR7

;---- local variables -----

; none

;---- arguments -----------

; (A)    = ptr to DTMFCHANNEL

rsbx	OVA	;clear overflow bit for A

ssbx	FRCT	;set fractional mode

stlm	A,AR5	;AR5 is used as pointer to struct elements

; *AR5(0) = ptr to indata

; *AR5(1) = ptr to taps

; *AR5(2) = ptr to energy template

; *AR5(3) = digitptr

; *AR5(4) = digitlast

; *AR5(5) = detectstat

; *AR5(6) = err_flags

ld	*AR5,A	;

stlm	A,AR1	;AR1 points to indata buffer

ld	*AR5(1),A	;

add	#31,A	;

stlm	A,AR6	;AR6 points to end of taps block

stm	#(N-1),BRC

goer1	rptb	goer2-1	;for(n=0;n<N;n++)     {

mvmm	AR6,AR2	;   AR2 points to end of taps block

stm	#COEF1st,AR3	;   AR3 points to Coefficients

ld	*AR1+,16,A	;   (A) = x(n)

;   ------ filter k=0 -----------------

sub	*AR2-,16,A,B	;   (B) = -vk(n-2) + x(n)

mac	*AR2,*AR3,B	;   (B) = coef*vk(n-1) - vk(n-2) +x(n)

mac	*AR2,*AR3+,B	;   (B) = 2*coef*vk(n-1) - vk(n-2) +x(n)

delay	*AR2	;   vk(n-1) ---> vk(n-2)

sth	B,*AR2-	;   (B)     ---> vk(n-1)

;   ------ filter k=1 -----------------

sub	*AR2-,16,A,B	;   (B) = -vk(n-2) + x(n)

mac	*AR2,*AR3,B	;   (B) = coef*vk(n-1) - vk(n-2) +x(n)

mac	*AR2,*AR3+,B	;   (B) = 2*coef*vk(n-1) - vk(n-2) +x(n)

delay	*AR2	;   vk(n-1) ---> vk(n-2)

sth	B,*AR2-	;   (B)     ---> vk(n-1)

;   ------ filter k=2 -----------------

sub	*AR2-,16,A,B	;   (B) = -vk(n-2) + x(n)

mac	*AR2,*AR3,B	;   (B) = coef*vk(n-1) - vk(n-2) +x(n)

mac	*AR2,*AR3+,B	;   (B) = 2*coef*vk(n-1) - vk(n-2) +x(n)

delay	*AR2	;   vk(n-1) ---> vk(n-2)

sth	B,*AR2-	;   (B)     ---> vk(n-1)

;   ------ filter k=3 -----------------

sub	*AR2-,16,A,B	;   (B) = -vk(n-2) + x(n)

mac	*AR2,*AR3,B	;   (B) = coef*vk(n-1) - vk(n-2) +x(n)

mac	*AR2,*AR3+,B	;   (B) = 2*coef*vk(n-1) - vk(n-2) +x(n)

delay	*AR2	;   vk(n-1) ---> vk(n-2)

sth	B,*AR2-	;   (B)     ---> vk(n-1)

;   ------ filter k=4 -----------------

sub	*AR2-,16,A,B	;   (B) = -vk(n-2) + x(n)

mac	*AR2,*AR3,B	;   (B) = coef*vk(n-1) - vk(n-2) +x(n)

mac	*AR2,*AR3+,B	;   (B) = 2*coef*vk(n-1) - vk(n-2) +x(n)

delay	*AR2	;   vk(n-1) ---> vk(n-2)

sth	B,*AR2-	;   (B)     ---> vk(n-1)

;   ------ filter k=5 -----------------

sub	*AR2-,16,A,B	;   (B) = -vk(n-2) + x(n)

mac	*AR2,*AR3,B	;   (B) = coef*vk(n-1) - vk(n-2) +x(n)

mac	*AR2,*AR3+,B	;   (B) = 2*coef*vk(n-1) - vk(n-2) +x(n)

delay	*AR2	;   vk(n-1) ---> vk(n-2)

sth	B,*AR2-	;   (B)     ---> vk(n-1)

;   ------ filter k=6 -----------------

sub	*AR2-,16,A,B	;   (B) = -vk(n-2) + x(n)

mac	*AR2,*AR3,B	;   (B) = coef*vk(n-1) - vk(n-2) +x(n)

mac	*AR2,*AR3+,B	;   (B) = 2*coef*vk(n-1) - vk(n-2) +x(n)

delay	*AR2	;   vk(n-1) ---> vk(n-2)

sth	B,*AR2-	;   (B)     ---> vk(n-1)

;   ------ filter k=7 -----------------

sub	*AR2-,16,A,B	;   (B) = -vk(n-2) + x(n)

mac	*AR2,*AR3,B	;   (B) = coef*vk(n-1) - vk(n-2) +x(n)

mac	*AR2,*AR3+,B	;   (B) = 2*coef*vk(n-1) - vk(n-2) +x(n)

delay	*AR2	;   vk(n-1) ---> vk(n-2)

sth	B,*AR2-	;   (B)     ---> vk(n-1)

;   ------ filter k=8 -----------------

sub	*AR2-,16,A,B	;   (B) = -vk(n-2) + x(n)

mac	*AR2,*AR3,B	;   (B) = coef*vk(n-1) - vk(n-2) +x(n)

mac	*AR2,*AR3+,B	;   (B) = 2*coef*vk(n-1) - vk(n-2) +x(n)

delay	*AR2	;   vk(n-1) ---> vk(n-2)

sth	B,*AR2-	;   (B)     ---> vk(n-1)

;   ------ filter k=9 -----------------

sub	*AR2-,16,A,B	;   (B) = -vk(n-2) + x(n)

mac	*AR2,*AR3,B	;   (B) = coef*vk(n-1) - vk(n-2) +x(n)

mac	*AR2,*AR3+,B	;   (B) = 2*coef*vk(n-1) - vk(n-2) +x(n)

delay	*AR2	;   vk(n-1) ---> vk(n-2)

sth	B,*AR2-	;   (B)     ---> vk(n-1)

;   ------ filter k=10 -----------------

sub	*AR2-,16,A,B	;   (B) = -vk(n-2) + x(n)

mac	*AR2,*AR3,B	;   (B) = coef*vk(n-1) - vk(n-2) +x(n)

mac	*AR2,*AR3+,B	;   (B) = 2*coef*vk(n-1) - vk(n-2) +x(n)

delay	*AR2	;   vk(n-1) ---> vk(n-2)

sth	B,*AR2-	;   (B)     ---> vk(n-1)

;   ------ filter k=11 -----------------

sub	*AR2-,16,A,B	;   (B) = -vk(n-2) + x(n)

mac	*AR2,*AR3,B	;   (B) = coef*vk(n-1) - vk(n-2) +x(n)

mac	*AR2,*AR3+,B	;   (B) = 2*coef*vk(n-1) - vk(n-2) +x(n)

delay	*AR2	;   vk(n-1) ---> vk(n-2)

sth	B,*AR2-	;   (B)     ---> vk(n-1)

;   ------ filter k=12 -----------------

sub	*AR2-,16,A,B	;   (B) = -vk(n-2) + x(n)

mac	*AR2,*AR3,B	;   (B) = coef*vk(n-1) - vk(n-2) +x(n)

mac	*AR2,*AR3+,B	;   (B) = 2*coef*vk(n-1) - vk(n-2) +x(n)

delay	*AR2	;   vk(n-1) ---> vk(n-2)

sth	B,*AR2-	;   (B)     ---> vk(n-1)

;   ------ filter k=13 -----------------

sub	*AR2-,16,A,B	;   (B) = -vk(n-2) + x(n)

mac	*AR2,*AR3,B	;   (B) = coef*vk(n-1) - vk(n-2) +x(n)

mac	*AR2,*AR3+,B	;   (B) = 2*coef*vk(n-1) - vk(n-2) +x(n)

delay	*AR2	;   vk(n-1) ---> vk(n-2)

sth	B,*AR2-	;   (B)     ---> vk(n-1)

;   ------ filter k=14 -----------------

sub	*AR2-,16,A,B	;   (B) = -vk(n-2) + x(n)

mac	*AR2,*AR3,B	;   (B) = coef*vk(n-1) - vk(n-2) +x(n)

mac	*AR2,*AR3+,B	;   (B) = 2*coef*vk(n-1) - vk(n-2) +x(n)

delay	*AR2	;   vk(n-1) ---> vk(n-2)

sth	B,*AR2-	;   (B)     ---> vk(n-1)

;   ------ filter k=15 -----------------

sub	*AR2-,16,A,B	;   (B) = -vk(n-2) + x(n)

mac	*AR2,*AR3,B	;   (B) = coef*vk(n-1) - vk(n-2) +x(n)

mac	*AR2,*AR3+,B	;   (B) = 2*coef*vk(n-1) - vk(n-2) +x(n)

delay	*AR2	;   vk(n-1) ---> vk(n-2)

sth	B,*AR2-	;   (B)     ---> vk(n-1)

goer2			;}

goer3	popm	AR7

popm	AR6

popm	AR1

popm	ST1

popm	ST0

ret

**************************************************************************

* SUBROUTINE: dtmf_checks             (approx 788 cycles)

*

* Input:  none

* Output: A (status if valid digit)

* Uses:

*

* Description:

*  - energy computation

*  - initialize filter taps to zero

*  - maximum search

*  - DTMF status check

*  - signal strength check

*  - twist check

*  - relative peak check

*  - 2nd harmonic check

*  - digit validation

*

**************************************************************************

_dtmfchecks:

nop

pshm	ST0

pshm	ST1

pshm	AR1

pshm	AR6

pshm	AR7

nop

frame	#0	;---- local variables -----

; none

;---- arguments -----------

; (A)    = ptr to dtmfchannel struct

ssbx	FRCT

stlm	A,AR5	;AR5 is used as ptr to dtmfchannel struct

; *AR5(0) = ptr to indata

; *AR5(1) = ptr to taps

; *AR5(2) = ptr to energy template

; *AR5(3) = digitptr

; *AR5(4) = digitlast

; *AR5(5) = detectstat

; *AR5(6) = err_flags

;-------------------------------------------------------------------------

; Energy computation

;

;  y(N)y*(N) = vk(N)*vk(N) - 2*coef*vk(N)*vk(N-1) + vk(N-1)*vk(N-1)

;

; Coefficients are in pmem in order starting with location COEF1st

; COEF1st .word	xxxx	;coef0  <--AR3

; 	  .word	yyyy	;coef1

;	  ....

;

; Data vk(n-1), vk(n-2) is ordered as follows in dmem

;	v15(n-1)

;	v15(n-2)

;	v14(n-1)

;	v14(n-2)

;	....

;	v0(n-1)

;	v0(n-2)  <--AR2

;

; FRCT = 1 , OVLY = 1

; AR2: points to v1(N-1)

; AR3: points to COEF1st

;

;-------------------------------------------------------------------------

ener	ld	*AR5(1),A

add	#31,A

stlm	A,AR2	;AR2 points to end of filter taps block

stm	#COEF1st,AR3	;AR3 points to beg of coefficient table

ld	*AR5(2),A

stlm	A,AR4	;AR4 points to energy template

stm	#(16-1),BRC

rptb	ener1-1	;for(k=0;k<16;k++)   {

ld	*AR2,16,A	; (A) = vk(N-1), A(32:16) loaded

mpya	*AR2-	; (B) = vk(N-1)*vk(N-1), (T) = vk(N-1)

mpy	*AR2,A	; (A) = vk(N)*vk(N-1)

ld	*AR3+,T	; (T) = coef

mpya	A	; (A) = coef*vk(N)*vk(N-1)

sub	A,1,B	; (B) = -2*coef*vk(N)*vk(N-1) + vk(N-1)*vk(N-1)

ld	*AR2,T	; (T) = vk(N)

mac	*AR2-,B	; (B) = vk(N)*vk(N) - 2*coef*vk(N)*vk(N-1) + vk(N-1)*vk(N-1)

sth	B,*AR4+	; store result E[k] into energy data block

ener1			;}

;------------------------------------------------------------------------

; Initialize Filter Taps to zero for next round

;

;------------------------------------------------------------------------

ld	*AR5(1),A

stlm	A,AR2	;AR2 points to beginning of filter taps

ld	#0,A

rpt	#(32-1)	;for(i=0;i<32;i++)

stl	A,*AR2+	;   initialize filter taps for next round

BREAK	nop		;<--- breakpoint for testing

;------------------------------------------------------------------------

; Alternative Maximum search

; result: AR3 points to max row energy ROWMAX

;         AR4 points to max col energy COLMAX

;------------------------------------------------------------------------

max_search:

ld	*AR5(2),A

stlm	A,AR2	;AR2 points to ENERGY table (row energies)

mvmm	AR2,AR3	;AR3 points to ENERGY table

stm	#(4-1),BRC

rptb	max1-1	;for(k=0;k<4;k++)

ld	*AR3,A	;   (A)=oldmax

sub	*AR2,A	;   (A)=oldmax-E[k]

bc	max11,AGT	;   if(oldmax < E[k])

mvmm	AR2,AR3	;      AR3 points to newmax

max11	mar	*AR2+	;   increment pointer

max1			;result: AR3 points to ROWMAX

;        AR2 points to column energies

mvmm	AR2,AR4	;AR4 points to ENERGY table (column energies)

stm	#(4-1),BRC

rptb	max2-1	;for(k=0;k<4;k++)

ld	*AR4,A	;   (A)=oldmax

sub	*AR2,A	;   (A)=oldmax-E[k]

bc	max21,AGT	;   if(oldmax < E[k])

mvmm	AR2,AR4	;      AR4 points to newmax

max21	mar	*AR2+	;   increment pointer

max2			;result: AR4 points to COLMAX

;        AR2 points to column energies

;----------------------------------------------------------------------

; DTMF Status Check:

;   if(detectstat == 0)

;      if( max(ROWMAX,COLMAX) < THR_PAU )

;         detectstat = 1, detector enabled

;      terminate all checks

;   else

;      continue

;

; AR3 points to ROWMAX

; AR4 points to COLMAX

;----------------------------------------------------------------------

stat_check

andm	#0080h,*AR5(6)	;clear error flags, except OVA

bitf	*AR5(5),#0001h	;

bc	sig_check,tc	;if(detectstat == 1) continue with sig_check

ld	*AR3,A	;

ld	*AR4,B	;else

max	A	;  (A)=max(ROWMAX,COLMAX)

sub	#THR_PAU,A	;  (A)=max(ROWMAX,COLMAX) - THR_PAU

bc	stat1,ageq	;  if((A)<0)

st	#0001h,*AR5(5)	;     detectstat = 1

stat1	b	ende	;  terminate all checks

;----------------------------------------------------------------------

; Signal strength check:

;    if(ROWMAX+COLMAX > THR_SIG)  continue

;    else  terminate all checks

;

; AR3 points to ROWMAX

; AR4 points to COLMAX

;----------------------------------------------------------------------

sig_check:

ld	*AR3,A

add	*AR4,A	;(A) = ROWMAX+COLMAX

sub	#THR_SIG1,A,B	;

bc	ende,BLEQ	;if(ROWMAX+COLMAX <= THR_SIG1)

;    signal is not possible tone

;    terminate checks

sub	#THR_SIG2,A,B	;

bc	err_sig,BLEQ	;if(ROWMAX+COLMAX <= THR_SIG2)

;    signal is possible tone

;    however does not meet THR_SIG2

;    branch to err_sig

b	twist_check

err_sig:		;**** record signal strength error ****

orm	#0001h,*AR5(6)	; set signal strength error flag (b0)

b	ende	; terminate checks

;----------------------------------------------------------------------

; Twist check:

;    if(ROWMAX > COLMAX)

;       check reverse twist:

;       if((COLMAX/ROWMAX) > THR_REVTWI)   continue

;       else terminate all checks

;    else

;       check standard twist:

;       if((ROWMAX/COLMAX) > THR_STDTWI)   continue

;       else  terminate all check

;

; AR3 points to ROWMAX

; AR4 points to COLMAX

;----------------------------------------------------------------------

twist_check:

ld	*AR3,A

sub	*AR4,A

bc	twist2,ALEQ	;if(ROWMAX > COLMAX)

twist1	ld	*AR4,16,A	;   (AH)=COLMAX

rpt	#(16-1)	;

subc	*AR3,A	;   compute (COLMAX/ROWMAX)

and	#0ffffh,A	;   retain quotient (AL), mask remainder

sfta	A,15	;   shift quotient into AH

sub	#THR_REVTWI,16,A;

bc	err_revtwi,ALEQ;   if((COLMAX/ROWMAX) <= THR_REVTWI)

;      branch to err_revtwi

b	rel_check

;else

twist2	ld	*AR3,16,A	;   (AH)=ROWMAX

rpt	#(16-1)	;

subc	*AR4,A	;   compute (ROWMAX/COLMAX)

and	#0ffffh,A	;   retain quotient (AL), mask remainder

sfta	A,15	;   shift quotient into AH

sub	#THR_STDTWI,16,A;

bc	err_stdtwi,ALEQ;   if((COLMAX/ROWMAX) <= THR_STDTWI)

;      branch to err_stdtwi

b	rel_check

err_revtwi:		;**** record reverse twist error ****

orm	#0002h,*AR5(6)	; set reverse twist error flag (b1)

b	ende	; terminate checks

err_stdtwi:		;**** record standard twist error ****

orm	#0004h,*AR5(6)	; set standard twist error flag (b2)

b	ende	; terminate checks

;----------------------------------------------------------------------

; Relative peak check:

;

;    find rel row peak --> RELPEAK

;    if(RELPEAK/ROWMAX < THR_ROWREL)  continue

;    else terminate all checks

;

;    find rel col peak --> RELPEAK

;    if(RELPEAK/COLMAX < THR_ROWREL)  continue

;    else terminate all checks

;

; AR3 points to ROWMAX

; AR4 points to COLMAX

;----------------------------------------------------------------------

rel_check:		;------ relative row peak ratio check -------

ld	*AR5(2),A	;

stlm	A,AR0	;AR0 points to ENERGY table (row frequencies)

ld	#0,B	;(BH)=0

stm	#(4-1),BRC	;

rptb	rel12-1	;for(k=0;k<4;k++)

ld	*AR0,16,A	;  (AH)=E[k]

cmpr	0,AR3	;

bc	rel11,TC	;  if(AR0==AR3)

max	B	;    (B)=newmax=max(oldmax,E[k])

rel11	 mar	*AR0+	;

rel12	rpt	#(16-1)	;

subc	*AR3,B	;compute (RELPEAK/ROWMAX)

and	#0ffffh,B	;retain quotient (BL), mask remainder

sfta	B,15	;shift quotient into BH

sub	#THR_ROWREL,16,B;

bc	err_rowrel,BGEQ;if((RELPEAK/ROWMAX) >= THR_ROWREL)

;      branch to err_rowrel

;------ relative column peak ratio check ----

rel2	ld	#0,B	;(BH)=0

stm	#(4-1),BRC	;

rptb	rel22-1	;for(k=4;k<8;k++)

ld	*AR0,16,A	;  (AH)=E[k]

cmpr	0,AR4	;

bc	rel21,TC	;  if(AR0==AR4)

max	B	;    (B)=newmax=max(oldmax,E[k])

rel21	 mar	*AR0+	;

rel22	rpt	#(16-1)	;

subc	*AR4,B	;compute (RELPEAK/COLMAX)

and	#0ffffh,B	;retain quotient (BL), mask remainder

sfta	B,15	;shift quotient into BH

sub	#THR_COLREL,16,B;

bc	err_colrel,BGEQ;if((RELPEAK/COLMAX) >= THR_COLREL)

;      branch to err_colrel

b	sec_check

err_rowrel:		;**** record row's rel peak error ****

orm	#0008h,*AR5(6)	; set row's rel peak error flag (b3)

b	ende	; terminate checks

err_colrel		;**** record col's rel peak error ****

orm	#0010h,*AR5(6)	; set col's rel peak error flag (b4)

b	ende	; terminate checks

;----------------------------------------------------------------------

; Second Harmonics check

;    if( (ROW2nd/ROWMAX) < THR_ROW2nd )  continue

;    else terminate all checks

;

;    if( (COL2nd/COLMAX) < THR_COL2nd )  continue

;    else terminate all checks

;

; AR3 points to ROWMAX

; AR4 points to COLMAX

;----------------------------------------------------------------------

sec_check:

stm	#8h,AR0	;offset into 2nd harmonics energies

;------ 2nd harmonics check for rows -------

mar	*AR3+0	;AR3 points to corresponding ROW2nd

ld	*AR3-0,16,A	;(AH)=ROW2nd, AR3 points to ROWMAX

sub	*AR3,16,A,B	;(BH)=ROW2nd-ROWMAX

bc	err_row2nd,BGEQ;if(ROW2nd<ROWMAX)

rpt	#(16-1)	;

subc	*AR3,A	;   compute (ROW2nd/ROWMAX)

and	#0ffffh,A	;   retain quotient (AL), mask remainder

sfta	A,15	;   shift quotient into AH

sub	#THR_ROW2nd,16,A;

bc	err_row2nd,AGEQ;   if((ROW2nd/ROWMAX) >= THR_ROW2nd)

;      branch to err_row2nd

;------ 2nd harmonics check for columns ----

sec2	mar	*AR4+0	;AR4 points to corresponding COL2nd

ld	*AR4-0,16,A	;(AH)=COL2nd, AR4 points to COLMAX

sub	*AR4,16,A,B	;(BH)=COL2nd-COLMAX

bc	err_col2nd,BGEQ;if(COL2nd<COLMAX)

rpt	#(16-1)	;

subc	*AR4,A	;   compute (COL2nd/COLMAX)

and	#0ffffh,A	;   retain quotient (AL), mask remainder

sfta	A,15	;   shift quotient into AH

sub	#THR_COL2nd,16,A;

bc	err_col2nd,AGEQ;   if((COL2nd/COLMAX) >= THR_COL2nd)

;      branch to err_col2nd

b	digit_map

err_row2nd:		;**** record row's 2nd harm error ****

orm	#0020h,*AR5(6)	; set row's 2nd harm error flag (b5)

b	ende	; terminate checks

err_col2nd:		;**** record col's 2nd harm error ****

orm	#0040h,*AR5(6)	; set col's 2nd harm error flag (b6)

b	ende	; terminate checks

;----------------------------------------------------------------------

; Map detected tone pair to digit

;

; AR3 points to ROWMAX

; AR4 points to COLMAX

;----------------------------------------------------------------------

digit_map:

;---------- find row and col numbers ----------------------------

ld	*AR5(2),A

stlm	A,AR0	;load offset

nop

nop

mar	*AR3-0	;AR3 contains row#

mar	*+AR0(4)

mar	*AR4-0	;AR4 contains col#

;---------- map row and column numbers to actual digit -------------

ld	*(AR3),8,A	;load row nbr in upper byte of AL

add	*(AR4),A	;load column nbr in lower byte of AL

stm	#KEYS,AR2	;AR2 points to key mapping table

stm	#0,AR3	;AR3 is used as counter

stm	#(16-1),BRC

rptb	digit3-1	;for(k=0;k<16;k++)   {

sub	*AR2+,A,B

bc	digit31,bneq	;   if(A == KEYS[k])  insert delay

b	digit4	;       branch to ende

digit31	 mar	*AR3+	;  increment counter

digit3			;}

;------------------------------------------------------------------------

; Validate digit from previous digit

;

;------------------------------------------------------------------------

digit4			;now AR3 contains decoded digit

ld	*AR5(3),A

stlm	A,AR4	;AR4 used as digitptr

ldm	AR3,A	;load current digit

sub	*AR5(4),A,B	;compare with last digit

bc	digit5,bneq	;if current digit and last digit are the same

stl	A,*AR4+	;   store VALID DIGIT into DIGITS

ldm	AR4,A	;

stl	A,*AR5(3)	;   update pointer

st	#0,*AR5(5)	;   disable detector and wait for pause

digit5	ldm	AR3,A	;store decoded digit

stl	A,*AR5(4)	; into digitlast

ende	frame	#0

popm	AR7

popm	AR6

popm	AR1

popm	ST1

popm	ST0

nop

ret

**************************************************************

*   (C) COPYRIGHT TEXAS INSTRUMENTS, INC. 1996               *

**************************************************************

*   Program Name:     DTMF tone decoder                      *

*   File Name:        tables.inc                             *

*   File Description: tables for dtmfsub.asm                 *

*                                                            *

*   Author:   Gunter Schmer                                  *

*   Date:     10/15/96                                       *

*   Revision: 2.0                                            *

*   Latest working date: 10/15/96                            *

**************************************************************

***************************************

***   Constants

***************************************

N		.set	102			;max count for DFT loops

THR_SIG1	.set 1900*32768/10000	;threshold for possible tone

THR_SIG2	.set 2150*32768/10000	;threshold for definite tone

THR_PAU		.set 458*32768/10000	;threshold for pause energy

THR_STDTWI	.set 3162*32768/10000	;threshold for standard twist (-5dB)

THR_REVTWI	.set 1259*32768/10000	;threshold for reverse twist (-9dB)

THR_ROWREL	.set 2000*32768/10000	;threshold for row's relative peak ratio

THR_ROW2nd	.set 2000*32768/10000	;threshold for row's 2nd harmonic ratio

THR_COLREL	.set 2000*32768/10000	;threshold for col's relative peak ratio

THR_COL2nd	.set 2000*32768/10000	;threshold for col's 2nd harmonic ratio

sqrt_iterations .set 16		;iterations for sqrt in gain control function

***************************************

***   Tables

***************************************

*******************************************************

* The following table holds the coefficients for      *

* the 16 Goertzel filters, computed as follows        *

*                                                     *

*       .word    cos(2pi*k/N)*32768	;coefficient  *

*                                                     *

* with  N = 102  (1st and 2nd Harmonics)              *

*       k =  9, 10, 11, 12, 15, 17, 19, 21  (1. Harm) *

*       k = 18, 20, 22, 24, 31, 34, 38, 42  (2. Harm) *

*                                                     *

*******************************************************

.sect   "tbl_coef"

COEF1st	.word	27980	; 1st Harmonics

.word	26956

.word	25701

.word	24219

.word	19073

.word	16325

.word	13085

.word	9315

COEF2nd	.word	14739	; 2nd Harmonics

.word	11414

.word	7549

.word	3032

.word	-10565

.word	-16503

.word	-22318

.word	-27472

*****************************************************************

* Digit mapping table                                           *

*   Low byte : row number                                       *

*   High byte: column number                                    *

*****************************************************************

.sect	"tbl_keys"

KEYS	.word	0301h	; '0'

.word	0000h	; '1'

.word	0001h	; '2'

.word	0002h	; '3'

.word	0100h	; '4'

.word	0101h	; '5'

.word	0102h	; '6'

.word	0200h	; '7'

.word	0201h	; '8'

.word	0202h	; '9'

.word	0003h	; 'A'

.word	0103h	; 'B'

.word	0203h	; 'C'

.word	0303h	; 'D'

.word	0300h	; 'E' = '*'

.word	0302h	; 'F' = '#'

****************************************************************

* Test Buffer for encoding results                             *

****************************************************************

DATA	.usect	"buffer",8000h  ;length 32k words

References

- Proakis, J.G., Manolakis, D.G., Digital Signal Processing, Macmillan Publishing Company, New York, 1992,    Pages 737-739

- Mock, P., Add DTMF Generation and Decoding to DSP-mP Designs,    DSP Applications with the TMS320 Family, Vol. 1, Texas Instruments, 1989

- Ziemer, R.E., Tranter, W.H., Principles of Communications, Houghton Mifflin Company, Boston, 1995

- MITEL Technical Data, Tone Receiver Test Cassette CM7291, 1980

- Bell Communications Research, Digit Simulation Test Tape, Technical Reference TR-TSY-000763, Issue 1, July 1987