TERMINAL ACCESS PACKAGE (TAP)

USER'S	GUIDE

VENIX VERSION

INTRODUCTION

The Terminal Access Package (TAP) allows the C programmer full

exploitation of	the AED	512/767/1024's capabilities. For every instruction in

the 512/767/1024's command set there is	a corresponding	C callable subroutine

to issue the instruction.  Operands, or	arguments, are encoded by TAP

into the appropriate format.

LEGEND

1.  All	arguments not ending in	S are 16-bit integers or pointers

to the same.

2.  Arguments ending in	S are 8-bit one	dimensional arrays of type char.

(There is one exception - see DFP below).

3.  Arguments preceded by a slash (/) are returned arguments.

4.  Arguments not preceded by a	slash are used by, but not modified by the

subprogram.

5.  CAP	is an abbreviation for Current Access Position.

SERIAL INTERFACE

Terminal acts like a TTY in raw	mode.

PARALLEL INTERFACE FUNCTIONS

The following subprograms are to be used with a 16-bit bidirectional

data bus port, except for IBYTE	and OBYTE which	can be serial.	The user

is cautioned to	ensure that no information is sent over	the parallel

interface while	there is still serial data to be processed by the terminal.

DMA FUNCTIONS:	RRD, RDA, WRD, WDA and SCD must	specify	the transfer

length in words.  For more detail, refer to the	PARALLEL COMMUNICATION and

TERMINAL COMMAND PROTOCOL Sections of the manual.

IBYTE (/bytes,n)  -  Read (n) characters into array (bytes).  Normally

used internally by	TAP, but can be	invoked	by the user for	special

purposes.

OBYTE (bytes,n)	 -  Output (n) characters from array (bytes).  Normally

used internally by	TAP, but can be	invoked	by the user for	special

purposes.

SELECT (byte)  -  Select terminal corresponding	to bits	#0-7 of	(byte).

( Not	implemented in Venix).

SKS (/bytes,n)	-  Send	(n) keystroke characters into array (bytes).

( Not implemented in	Venix).

DMA FUNCTIONS

Note :	DMA functions require (data) to	start on an

integer	boundary.  Also	note that the parameter

(n) is the number of WORDS of data to be sent,

(SCD is	the exception to the rule).

This implies that the AOI accessed in RDA or WDA

must contain an	even number of bytes.

RRD (/data,n)  -  Read raster direct.  Perform terminal	to host	DVMA

transfer of (n) words into	array (data).

RDA (/data,n)  - Read direct from area of interest specified by	DAI

function.	Perform	terminal to host DVMA transfer of (n) words into

array (data).

WRD (data,n)  -	 Write raster direct.  Perform host to terminal	DVMA

transfer of (n) words from	array (data).

WDA (data,n)  -	 Write direct into area	of interest specified by DAI

function.	Perform	host to	terminal DVMA transfer of (n) words from

array (data).

SCD (data,n)  -	 Start Command DMA.  Terminal receives commands	from the

DMA channel.  Command array (data)	should be (n) BYTES long.  Termi-

nating byte should	= 60 (omit if using Venix driver).  Commands

which request data	to be sent from	the terminal to	the host must

not be sent with command DMA.  DMA	commands must not be sent with

command DMA.

TERMINAL COMMAND FUNCTIONS

The following subprograms are equivalent to the AED 512/767/1024 terminal

instructions which carry the same name.	 For more detail regarding their

action,	the user is referred to	the "Terminal Control Protocol"

section	of the manual.

AAV (code)  -  Anti-alias vectors.  Code = disable, Code=1 enables anti-

aliasing.	Code = 2 enables and sets anti-aliasing	to ramp	with each

of	the 8 default colors.

NA	512.

AED -  Prints the general status of the	AED terminal on	the screen. Status

information includes: version #, CAP, current color , etc...

This command puts the interface in	"display" mode (see SIF), so a

subsequent	sif('P') is needed to reenable terminal	to host	transfers.

1024 only.

BFL (color)  -	Boundary fill.	Fill in	current	color at CAP,

Boundary pixels are those with value (color).

BSO (x,y)  -  Set both screen origins, horizontal to (x), vertical to (y).

CAI (x,y)  -  Copy pixels within the area of interest to an area of equal

size and shape with lower left corner at x,y.

1024 only.

CHR (n)	-  Set character size. n=1,2,3,4 for 5x6,7x9,5x6(double),7x9(double).

1024 only.

DAI(x1,y1,x2,y2)  -  Define Area of Interest with corners at (x1,y1)

and (x2,y2).

DCA (x,y)  -  Draw Cursor Absolute, centered at	(x,y).

DCL (rad)  -  Draw CircLe, centered at CAP, of radius rad.

0	< rad <	128.

DFC (rad)  -  Draw "Fat" Circle, centered at CAP, of radius rad.

0	< rad <	128.

DFP (Flag_color, n, coords)  -	Draw Filled Polygon of n vertices defined

by	coords.	 Flag color is a color which the user will not use in

the background prior to drawing the polygon.  The polygon is filled

in	the current color.

NA	512.

Note : int	coords[n][2];  coords[i][0], coords[i][1] are the x, y

coordinates of the	i'th vertex, 0 < i < n.

DFR (x,y)  -  Draw Filled Rectangle, with one corner including CAP, opposite

corner including (x,y), which becomes new CAP.

DJC  -	Disable	Joystick Cursor.

DMV (dxs,dys)  -  Draw multiple	Vectors, beginning at CAP.  Will draw

vectors until dxs(i)=dys(i)=0.  Each dxs(i) and dys(i) is a signed

number in the range (-128,127).

DPA  -	Disable	joystick pan.

DPK (keyno,icodes)  -  Define Programmable Key.	 (keyno) specifies one of

the 8 special function keys.  (icodes) gives the sequence to send each

time the key is hit.  For example:

dpk (3, "EXIT");

will send the sequence "EXIT" each	time key number	3 is hit.

(icodes) must be terminated by a null (0) byte, unless it is

8 characters long (the maximum).

DRL (display)  -  Loads	the LED	array with the binary encoding of (display).

DSF (code,xsize,ysize,nplane,plane_masks,bits)	-  Define user symbol

("Special Font").	(code) is unique for this symbol, must be between

0 and 127,	inclusive.  (xsize) and	(ysize)	give the character matrix

dimensions	in pixels.  (nplane) gives the number of planes	in which

the character is defined.	(plane_masks) is a list	of these planes,

each specified by a one in	the corresponding bit position.	 (bits)

contains the binary matrices to be	scanned	left to	right, top to

bottom, plane to plane.

DSK (keyno,size,icodes)	 -  Define software key.  (keyno) specifies one

of	key 0 through key 7.  (icodes) gives the sequence to send each time

the key is	hit, with (size) being the length of the sequence.

DSP (pat#,patterns)  -	Define a 'stipple' pattern.  The pattern will be

used by the	terminal to fill rectangles drawn with subsequent

FRR	and DFR	commands.

DTM (origin,yorigin,xscale,yscale)  -  Define Tablet Mapping.  (xorigin,

yorigin) are the tablet coordinate	of the lower left-hand corner of

rectangular area to be mapped onto	the display screen.  (xscale)

and (yscale) are inverse scale factors which determine the	rectangle's

size according to the following relationship:

xscale=65536/xsize

yscale=65536/ysize

where (xsize) and (ysize) give the	size of	the rectangle in tablet

units.

DVA (x,y)  -  Draw Vector Absolute, from CAP to	(x,y),which becomes new

CAP.

DVR (dx,dy)  -	Draw Vector Relative, from CAP to (CAPx+dx,CAPy+dy), which

becomes new CAP. -129 < dx,dy < 128.

ECU  -	Erase Cursor Unconditionally.

EJC  -	Enable Joystick/Cursor.

ELP (xsize,ysize)  -  Draw Ellipse at CAP with radii defined by	xsize, ysize.

0 <	xsize, ysize < 256.

EPA  -	Enable Joystick	Pan

ERS  -	Erase video memory, write-enabled plans	only, to the background

color, and	remain in interpreter.

ESC  -	Escape.	 Enter interpreter.  Does not normally have to be issued

when using	TAP.  However if the programmer	uses his own routine to

write text, he must re-enter the interpreter with esc();.

ESF (xsize,ysize,dx,dy)	 -  Erase Special Font.	 Actually erases a

rectangle xsize by	ysize with lower left-hand corner at CAP.  Updates

CAP to (CAPx+dx,CAPy+dy).	(dx) and (dy) must be in the range (-128,

127).  This function will erase all write-enabled planes, so the

user must mask out	the planes to be saved with the	SWM function.

ETC (/key,/flag,/code,/x,/y)  -	 Enable	Tablet Cursor.	The cursor light

will come on and the operator is free to move the cursor about.  When

the pen is	depressed, or a	cursor button hit, TAP will return screen

coordinates if (flag) is 0.  Tablet coordinates will be returned

if	the pen	or cursor is outside of	rectangle defined with the DTM

subroutine	when the hit is	made, and in this case (flag) will

be	non-zero.

(code) Gives the code for the combination of cursor buttons depressed.

If	no key was struck 0 will be returned in	(code).

Note:  This subroutine differs considerably from the TCP definition of

ETC.

ETP (Idchar)  -	 Enable	tablet polling if Idchar does not equal	zero.  Data

will be sent to the host only when	requested with the RTP command.

(see below).

FFD  -	Erase the entire memory, write-enabled planes only, to the background

color, and exit interpreter.

FRR (dx,dy)  -	Filled Rectangle Relative from CAP to CAP+dx,dy, which

becomes new CAP.  -129 < dx,dy < 128.

GFL (mask,color)  -  Generalized Overlay Fill.	Fill in	current	color, through

current write mask.	Boundary pixels	are those whose	bitwise	AND with

(mask) is equal to (color).	 The fill starts at CAP.

HOM  -	Return the alpha cursor	to the first character of the top line,

according to the current font size.

HSR (dx)  -  Horizontal	Scroll Relative.  Modify the current horizontal

origin by addition of (dx),	which must be in the range (-128,127).

IFL  -	Interior Fill.	Fill begins at CAP, in the current color.

Boundary pixels	are those not equal to the pixel at CAP.

JUS (addr)  -  Jump to User Subroutine at location (addr).

LAT (Base,values)  -  Load Anti-alias color Table.  Sets anti-alias

table with ramping at color	base which is defined by values.

See	TCP section of manual for details.

LMR (addr,n,bytes)  -  Load Microprocessor RAM,	with (n) (bytes)

starting at	address	(addr).

MOV (x,y)  -  Move CAP to (x,y).

MVR (dx,dy)  -	Move CAP to (CAPx+dx,CAPy+dy).	(dx) and (dy) must be in

the	range (-128-127).

OFL  -	Overlay	fill.  Fill begins at CAP.  Boundary is	formed by pixels

whose bitwise AND with the current color is non-zero. The fill

is in the current color.  During the fill, the write mask is

set to the current color.

OPT (optnum,value)  -  Set option number (optnum) to (value).

RCP (/x,/y)  -	Return CAP in (x,y).  If cursor	is enabled this	will be

the	pixel which the	cursor is centered upon.

RCT (first,n,/reds,/greens,/blues) -  Read Color Table.	 Return	(n) r,g,b

triples, starting with color number	(first).  (first) + (n)	< 257.

n=RHO (/xorigin)  -  Return Horizontal Origin as function value	and in

(xorigin).

r=RHR (n,/bytes)  -  Read Horizontal Runs.  Return number of runs as

function value.  The runs are sequenced in (bytes) identically to the

output format.  (n)	is the number of bytes available in (bytes).

If the terminal returns more than (n) bytes	TAP will

report an error and	halt.

See	TCP for	the encoding of	(bytes).

RHS (n,/pixels)	 -  Read Horizontal Scan.  (n) must be the number of

bytes in the AOI.	(pixels) should	be at least (n)	bytes long;

it will be filled with the pixels in the AOI ordered according

to the AOI parameters.

RJP (/x,/y)  -	Return Joystick	Position.  (x) and (y) will be in the

range (0,511).

ROT (n)	-  Rotate memory planes	n times	within the current

area of interest. 0<= (n) <	8 .  ROT(0) restores the AOI to	its

original appearance.

1024 only.

n=RPX (/value)	-  Read	Pixel.	Pixel at CAP is	returned as function value

and in (value).

RST  -	Reset.	Perform	a full reset of	the AED	terminal equivalent to

the	Power-on reset.

RTP (/Idchar,/stat,/x,/y)  -  Read Tablet Polling.Idchar,pen status and

x, y coordinates are sent to the host if ETP is enabled.

n=RVO (/yorigin)  -  Return Vertical Origin as function	value and in

(yorigin).

RZR (/x,/y) - Read zoom	registers into (x,y). See SZR command.

1024 only.

SAC (color)  -	Set alpha cursor color.

SAP (size,font,xspace,yspace,link)  -  Set Alphanumeric	Parameters as

follows:

size = '1'	 Normal	Sized		 font =	'5'    5 x 8 font

size = '2'	 Double	Sized		 font =	'7'    7 x 12 font

size = '.'	 Same Size as before.	 font =	'.'    Same font as before.

hspace - horizontal	distace	between	character centers.

yspace - vertical distance between character centers.

link = 'L'	Alpha cursor position identical	with CAP (default).

link = 'U'	Alpha cursor position not identical with CAP.

SAR (xysigns,xrates,yrates)  -	Set auto roam.	Horizontal rate	- xrates

*60/32 pixels per second; analogous	for vertical.  Bits 7 and 6 of

xysigns determine respectively the sign of horizontal and vertical

roam.

512	only.

SBC (color)  -	Set Background Color.

SBL (value,red,green,blue,ontime,offtime)  -  Set Blink.  Blink	pixels

of color (value) between their current color definition and

(red,green,blue).  Current definition will be in effect for	(ontime)

fields; alternate definition for (offtime)	number of fields.

(offtime) =	0 will disable blink.  Up to eight colors may be blink-

enabled simultaneously.

SBR (main,aux)	-  Set Baud Rates.  Override baud rates	selected by back

panel DIP switch for main and aux. serial interfaces.  Set rates as

follows:

'0'	   '1'	  '2'	 '3'	'4'    '5'    '6'     '7'	 '8'

300	   600	  1200	 2400	4800   9600   19200   External	 Leave

Baud   Baud	  Baud	 Baud	Baud   Baud   Baud    /16	 Current

Rate

Unchanged

Note:  The arguments are 1-character ASCII in all cases.  Also, a

a programming error can change the main baud	rate, garbling

further serial communication	until a	manual reset.

SCC (color1,color2,bltime)  -  Set Cursor Color(s).  The cursor	will blink

between (color1) and (color2). (bltime) specifies number of	consecutive

fields between color changes (1 field=approx. 1/60 sec).  If (color1) =

(color2) cursor will not appear to blink.

SCP (ctype,hsize,vsize)	 -  Set	Cursor Parameters.  (ctype) select cursor

type as follows:

ctype='X'		Crosshair, x-shaped, 13	units large

ctype='+'		Crosshair, +-shaped, 13	units large

ctype='L'		Full screen-width crosshairs

If crosshair cursor	style is selected (hsize) does not give	cursor

size, but is complemented to give a	display	memory address bit mask.

If full screen crosshair is	selected (vsize) specifies the plane in

which to write the cursor; (visze) is in binary decoded format.

SCR  -	Send Carriage Return.  Included	in TAP for sake	of completeness

only.  TAP will wait for the terminal to send a carriage return.

One	way to be sure that any	serial I/O is complete ( before

switching to parallel) is to issue the SCR command as the last of

a sequence of commands.  On	return serial I/O is complete and

the	switch can be done.

SCS (value)  -	Set Console Status, to (value).	 See TCP for details.

SCT (first,n,reds,greens,blues)	 -  Set	up Color Table.	 Define	(n) colors

starting with (first). ( reds), (greens), (blues) are the rgb values

for	the colors to be defined.  ( See RCT).

SEC (color)  -	Set Current Color.

SHO (xorigin)  -  Set Horizontal Origin	to (xorigin).

SIF ('P','S','M','D')  -  Select Interface.  SIF determines the	communications

interface the terminal will	use for	sending	to the host.

See	TCP for	details.

SLS (pattern,scale)  -	Set Line Style.	 The binary encoding of	(pattern)

specifies a	basic broken line pattern, or template,	with 1's correspond-

ing	to visible portions and	0's to invisible portions.  The	scan of

pattern is from most significant bit to least.  A value of 255 will

accordingly	revert the terminal to the default solid line vectors.

(scale) allows the basic pattern to	be scaled up as	follows:

255	  times	1

85	  times	2

17	  times	4

1	  times	8

SPF (pat#)  -  Set Patterned Fill.  Enable stipple fill	of pat#	defined

by DSP.

SRM (mask0,mask1,mask2,mask3)  -  Set Read Masks.  See TCP for details.

SSE (addr)  -  Set Stack End address to	(addr).	 See Application Note

'User Firmware' for	details

STD (delay)  -	Set turnaround delay for serial	communication.

(delay) is in units	of 1/60	second.

STP (code)  -  Set tablet parameters.  If code = 0, data will be sent to

the	host only when the pen is depressed.  If code =	1, date	is sent

when the pen is depressed or released.  The	tablet must have been

previously enabled with the	ETP command.

STW (code)  -  Set Tektronix Window to 9-bit if	code = 0 and set window

to 10-bit if code =	1.

SUC (byte)  -  Set Up counters for DVMA.  Bit assignments in the encoding

of (byte): (1 = true, 0 = false).

0:	X autoincrement	(default=1)

1:	Y autoincrement	(default = 0)

2:	X up counter (left to right) (default =	1)

3:	Y up counter (bottom to	top) (default =	0)

4:	Unlinked counters.  Overflow from one counter will not

increment the other.  (default = 0)

5:	16-bit transfers (as opposed to	8-bit) (default=1)

6:	0 (reserved)

7:	0 (reserved)

The DAI	command	sets all bits to the appropriate values,

except for bit 5.  If you need bit 5 to	be 0, make it so

before you issue a DAI instruction.

SUP (mode)  -  Set Superroam.  (mode) =	1 enables Superroam.  (mode = 0)

disables Superroam.	 See TCP for details.

512	only.

SVO (yorigin)  -  Set Vertical Origin to (yorigin).

SWM (mask)  -  Set Write Mask.

SZR (xzoom,yzoom)  -  Set-up Zoom Registers.  Zoom factors (xzoom) and

(yzoom) should be in the range (1,16).

VSR (dy)  -  Vertical Scroll Relative.	Modify the current horizontal

origin by addition of (dy),	which should be	in the range (-128,127).

WHC (n,pixels)	-  Write horizontal scan, non-AOI. The (n) pixels in

(pixels) will be written starting at CAP, proceeding to the	right

and	down.

WHR (bytes)  -	Write horizontal runs, beginning at CAP.

See TCP for encoding of the byte stream (bytes).

WHS (n,pixels)	-  Write Horizontal Scan of the	entire AOI.  (n) should	be

equal to the number	of pixels in the AOI.

WHU (n,bytes)  -  Write	horizontal runs	alternate.  (n)	should be equal	to

the	number of bytes	in (bytes).  See TCP for the encoding of (bytes).

WIP (n,bytes)  -  Write	Incremental Plotter mode.  (n) gives the total

number of direction	codes.	(bytes)	holds the codes, packed	two per

byte.  See TCP for more information.

WMP (dxs,dys)  -  Write	multiple pixels, beginning at CAP.  Write pixels

until dxs(i) = dys(i) = 0.

WPX (color)  -	Write (color) at CAP.

WSF (nchar,codes,dxs,dys)  -  Write user symbols ('Special Font	Characters').

Write (nchar) symbols with (codes) defined by DSF, above.  First

character is written based at CAP, which is	updated	by addition of

(dxs(i)) for each charcter written.	 The numbers in	(dxs) and (dys)

should be in the range (-128,127).

SPECIAL	PURPOSE	FUNCTIONS

TAPINT (option1, option2)  -  Initialize TAP.  Must be called before any

other call to TAP.	(option1) and (option2)	have no	current	usage;

they should	have the value 0.

TXT (message,dir)-  Draw a text	string beginning at CAP. (message) is an

array of type byte.	 If dir	= 0, the message is written left to

right, else	up to down.  Message must be terminated	with a null (0)

byte.  Non-printing	characters (unfortunately including tab, CR, LF)

are	ignored.