767 FIRMWARE MODIFICATIONS

AUTHOR: SCOTT NEAR

THIS FILE CONTAINS THE DOCUMENTATION OF THE 767 FIRMWARE CHANGES

***************************************************************************

V03:	LAST VERSION BY DAVE ARNOLD

V8308	LAST VERSION BY SCOTT NEAR

V8308+	Added SAI in a preliminary rev for Moorhead 6/6/84.

V82.7:	CHANGED ETC BACK TO V91 COMPATIBLE  ETC(IDCHAR) DISABLE IF 0.

ADDED ETP COMMAND-ENABLE TABLET POLLING.  ETP(IDCHAR) DISABLE IF 0.

V82.9:	USED NEW DFP ALGORITHM (DIFFERENT FORMAT). DFP FLAG,N,V1.....VN.

ADDED STP COMMAND. DEFAULT TO SINGLE PEN HITS. STP(1) FOR UP/DN HITS.

ADDED STW COMMAND. STW(0) 9-BIT TEK MODE, STW(1) 10-BIT TEK MODE.

STW INITIALIZES TO 0.

REMOVED STYLUS BEEP.

FIXED THE MVR COMMAND SO THAT IT PROPERLY CROSSES THE 512X512 BOUNDRY.

V83.01:	FIXED BLG GLITCH AND CORRECTED IMPROPER BLG LED CONTROL

CHANGED WHS FOR CORRECT ENCODING SCHEME (0 INSTEAD OF CNTRL-SH-P)

CORRECTED BIT PAD PROBLEM: SINGLE STYLUS HIT CAUSING

MULTIPLE HITS WHEN CALLING THE CIRCLE ROUTINE

CHANGED SRM FOR PRIME. SRM(N,0,0,0)= SRM(N,N,N,N). OTHERWISE SAME

ADDED MAR (SET TEXT MARGINS). FORMAT: MAR (LEFT MAR) , (RIGHT MAR)

THE DFR COMMAND WILL NOW FILL 1023,1023 CORRECTLY

THE CURSOR NOW SCALES PROPERLY WITH BOTH TEK WINDOWS (SUB)

V83.08:	FIXED REMAINING TEKTRONICS PROBLEM: IN 1K MODE RAP-AROUND

OCCURS WHEN USING SUB PAST 512X512.

FIXED WMP PROBLEMS (WRAP AT 512X512).

FIXED POWER UP PROBLEMS: BAD BYTE IN BUFFER (UNIT CAN HANG).

'SEND' ROUTINE NOW REQUESTS TWO SI'S BEFORE HANDSHAKING.

RESET ROUTINE DISABLES READ MASKS UNTIL AED IS DISLAYED.

*************************************************************************

V8408:

6/6/84

VM01	Added new command SAI (specify AOI).  This is a more

powerful version of DAI.  SAI(flag,x1,y1,x2,y2) defines

an AOI with the obvious corners.  If flag is 0, this is

the same as MOV x1, y1  DAI x2, y2.  If flag is <> 0, subsequent

DMA transfers (WDA, RDA) are done a column at a time rather

than by rows.   It looks like it should be simple to add this

'column' capability to the rest of the AOI commands as well.

It should also be possible to clean up and simplify ALL the

AOI code in the process.

The flag value is stored in AOIDIR.  The DAI command sets

this flag to 0.

PAGE0	Added AOIDIR to extended page.

VX01	Modified WDA, RDA commands to take AOIDIR into account.

VN01	Added SAI command.

8/18/84

VA01	Total rewrite of GET: (parallel byte section).  Old version

ignored MSB of command out (loc .29) if option sw.5 was

down.  This was not correct.  New version will ignore

multi-terminal select and reset bits in .29 (as it should).

Also changed the meaning of TSBAR.  TSBAR = 0 used to mean

the terminal was selected.  Now bit 7 of TSBAR is 1 if the

terminal is selected, bit 6 is 1 if opt. sw. 5 is up.  This

change made GET a little faster (and easier to write).  The

reset routines in VI01 were changed to initialize TSBAR

properly.

VI01	Added code to initialize TSBAR.  On reset bit 7 of TSBAR

is set - terminal is always selected after reset.  Bit 6

is set if option switch 5 is up.

Also speeded up reset routine by doing program checksum

once instead of 5 times.

PAGE0	Moved GTCHAR and OUTRAN to extended page.

VP01	'Added 2' to instructions using GTCHAR, OUTRAN.

VC01	Removed check for AFIFO empty at CUR2:.  This was done

so that cursor would be refreshed even if mouse hadn't

moved (mouse only sends data when moved).

Added code to init QUOH before calling SCALE routine.

VD01	Fixed problem with lower case descender characters

(gjpqy,;) - they dropped to bottom of screen when they

should have been displayed around Y=512.

ERASE routine now works even if zoomed - wrong routine was

being called to unzoom.

VE01	SEND routine modified to wait for 2 SIs before clearing

byte ready flag.

VF01	Fixed bug in GFL - it was having trouble terminating

because it didn't recognize fill color as a boundary.

VL01	Modified DFP so that it works with write mask <> 255.

This version only looks in the write enabled planes

for boundary color.

VM01	Modified SCT command so that changes to color table are

made during vertical blank - this reduces the annoying

flicker seen when writing the color table DACs during

screen refresh.

Fixed bug in WIP command - point counts that were multiples

of 256 weren't treated properly.

VO01	Color table wasn't set up correctly when dejag enabled -

DEJAGA routine was assuming clt locations 128-255 were 0.

Modified DEJAGA to store 0 where needed.

Also routines DAC, SETANT were doing dejag setup for

colors 0-15.  This was harmless, but a waste of time

for colors 8-15, since we don't dejag them.  Changed to do

setup for colors 0-7.

VS01	Changed use of PADCR and PADFL.  PADCR now points

to the first byte of the most recent report from the

mouse or pad, not the last.

VP01	Modified PAD routine in accordance with new meaning of

PADCR (see above).

VT01	Fixed bug in ZOOMS routine (SZR command).  Funny

things happened if SZR args were greater than 16, added

code to set args<17.

VU01	Fixed bug in LMR command - byte counts that were multiples

of 256 weren't treated properly.

Note:	Things that still need to be done are

1) Fix scroll routines in VD01.

2) Modify WHU, WHS, RHS, WHR, RHR to support SAI command.

3) Add typeahead.

4) Incorporate Colorware VP01.

5) Find out why the screen scrolls (sometimes) after doing

COPY DL3:IMAGE1.DAT GD:  - is this Robins' problem ?

8/19/84

VD01	Fixed scrolling problem (routines FLF, SCROL).

PAGE0	Added SCRBOT to extended page.  This is a temp used

by VD:FLF in calculating current bottom of screen.

8/22/84

VK01	Typeahead finished.  Added routines GETKEY, PUTKEY

to do the buffering (they work like GET, PUT).

GETKEY should be called only if KROUT=KRIN, i.e.

when the fifo is not empty.  Keyboard input is

buffered if keybd not locked out (via OPT command)

and not in LOCAL mode (in local keys go to FIFO).

Programmed key codes (DSK, DPK) go into KFIFO

just as if the codes were typed.  If KFIFO becomes

full, further keystrokes are silently ignored by PUTKEY.

Modified routines SKS, SPSUB, KMAP.  Deleted SPSEN,

all keystrokes are now sent at idle loop time via SPSUB.

The variables SPREQ, KPEND, KFLAG are all now obsolete.

VC01	Modified for typeahead.  Setting Tek GINMOD causes

any keys in KFIFO to be discarded immediately - the

next keystroke is the important one.  Places in the

code that used to check for KPEND or KFLAG <> 0 and

do something with KEYNO if true now check for

KROUT <> KRIN, and call GETKEY if true.

PAGE0	Added KRIN, KROUT to extended page.

6502	Added KFIFO, 64 bytes just after blink table BLTAB.

That page is still 1/2 empty, if 64 keystrokes of

typeahead isn't enough ...

8/27

VN01	Fixed bug associated with 'HERE IS' key.  Hitting this

key causes screen erase, however, the erase was done

after drawing the cursor.  The result was

that after cursor was drawn and HERE IS hit, next time

cursor was erased the stuff it was hiding (before the screen

erase) re-appeared on the (otherwise empty) screen.  Solution

was to erase cursor before erasing screen.

8/30

VP01	Added support for low-res packed binary data format used

by Summa mouse, KURTA pad.  Modified PAD to verify the

correctness of every byte sent by GTCO, if everything isn't

as it should be the report is ignored.  Made a subroutine

out of the code that did the ASCII decimal to binary conversion

and put it in VE01, where it was copied from in the first place.

Deleted the DECDIG subroutine, it was inefficient and isn't

needed anymore.

VS01	Modified AXINT to support binary data format

used by Kurta pad, Summa mouse.

VE01	Made a subroutine out of DEC01, renamed it MUL10.  Changed

DECIN to call MUL10 instead of branching to DEC01 etc.

9/6

VK01	Reset key routine now flushes pending keystrokes in KFIFO.

9/20

VD01	Added ERD as in Colorware terminals.

VN01	Added ERD.

VF01	Fill routines now preserve CAP.