CTS Monitor (CTSMON) User's Guide

Version 14.32

28 February 1994

LEGAL NOTICES

CTSMON.EXE, CTSMON.HLP, and this document are Copyright (C)

1990-1994 Exabyte Corporation.  All rights reserved.  EXABYTE is

a registered trademark of Exabyte Corporation.  Trademarks

belonging to other companies are used for identification

purposes only, and belong to their respective owners.

Microsoft, Microsoft Windows, and MS-DOS are registered

trademarks of Microsoft Corporation.  IBM is a trademark of

International Business Machines Corporation.  In this document,

DOS refers to both IBM DOS and MS-DOS.  Desqview is a trademark

of Quarterdeck Office Systems.

U. S. Government Restricted Rights.  This software and

documentation is provided with RESTRICTED RIGHTS.  Use,

duplication, or disclosure by the Government is subject to

restrictions as set forth in subdivision (b)(3)(ii) of The

Rights in Technical Data and Computer Software clause at

252.227-7013.  Contractor/manufacturer is Exabyte

Corporation/1685 38th Street/Boulder CO 80301-2630.

Disclaimer of Warranty.  Neither Exabyte Corporation nor anyone

else who has been involved in the creation, production, or

delivery of this product shall be liable for any direct,

indirect, consequential, or incidental damages (including

damages for loss of business profits, business interruption,

loss of business information, and the like) arising out of the

use of or inability to use such product, even if Exabyte

Corporation has been advised of the possibility of such damages.

Because some states do not allow the exclusion or limitation of

liability for consequential or incidental damages, the above

limitation may not apply to you.  This disclaimer of Warranty

and Limited Warranty is governed by the laws of the State of

Colorado.

Export Restrictions.  This software is subject to U. S. Export

Regulations and it shall be the responsibility of the user to

determine what requirements apply with respect to this program.

Legal Questions.  If you have any questions about this Agreement

or if you want to contact Exabyte Corporation for any reason,

please call us at 800-447-3920 or 303-442-4333 or write to us

at

Exabyte Corporation

Attention: Service Administration

1685 38th Street

Boulder CO 80301-2630

INTRODUCTION

This document describes the features and functions of the CTS

Monitor program (CTSMON) for the EXB-8200, EXB-8200SX, EXB-8500,

EXB-8500c, EXB-8205, and EXB-8505.  CTSMON is provided to

EXABYTE customers to enable them to perform diagnostic, test,

firmware upgrade, and configuration functions with the above

Cartridge Tape Subsystem (CTS) products.

The CTS Monitor program communicates over an RS-232 connection

from a PC to the diagnostic port on the CTS to get diagnostic

dump information, load new firmware, and set configuration

options that affect the way the CTS operates.  Note that because

of differences in the supported models of CTS, not all functions

work with all drives.  For example, configuration and firmware

upgrades cannot be done over the serial port on the EXB-8200.

Functions that don't work with the attached CTS will be shaded

out on the menu or will return a message stating that the

function is not available for that CTS.

INSTALLATION & CUSTOMIZATION

If you are familiar with DOS, PCs, and Exabyte CTS products, and

don't want to spend a lot of time reading, refer the the Quick

Start section, below.  More detailed information on installation

and customization follows that section.

QUICK START

Install the CTS Monitor by specifying the directory that you

want to install the CTS Monitor to on the command line of

CTSnnnn.EXE.  The default installation directory is the current

directory.  For example, to install version 14.32 from A:\ to

C:\CTSMON, type:

A:\CTS1432 C:\CTSMON

This will create the directory C:\CTSMON if it does not already

exist, and extract the files CTSMON.EXE, CTSMON.HLP, CTSMON.DOC,

CTSMON.PS, CTSMON.PIF, CTSMON.ICO, and CTSMON.NEW to that

directory.

To start the CTS Monitor program from the DOS prompt, type:

CTSMON

Press F1 for help or Alt-D to read the online documentation.

To print a copy of the manual, copy CTSMON.PS to a PostScript

printer, or if you don't have a PostScript printer, copy

CTSMON.DOC to a printer.

HARDWARE REQUIREMENTS

To run the CTS Monitor program, you need:

An IBM PC, XT, AT, PS/2 or compatible 80x86 based

computer.

An MDA, CGA, EGA, or VGA monitor.

At least one serial port on COM1:, COM2:, COM3:, COM4:,

or one of the eight PS/2 serial ports, hardware compatible with

the IBM PC (8250, 16450, or 16550AFN UART).  If your serial port

has a 9 pin connector, you also need a 9 pin to 25 pin serial

port adapter.

A hard disk drive.

A monitor cable (4-pin to RS-232, available from

EXABYTE).

At least 640 KBytes RAM (500 KBytes free).

In addition, the following options are helpful:

An EGA or VGA display will result in much better

performance than a CGA display, and allows you the option of

viewing more information on the screen at once in 43 or 50 line

mode.

Additional memory in the form of EMS will substantially

improve performance.

A mouse allows for smoother menu operation.

HARDWARE INSTALLATION

Connect the Monitor cable to the serial port on the back of the

computer and to the Monitor port on the back of the CTS.

Exabyte sells a 25 pin to 4 pin adapter cable for this purpose.

If your serial port has a 9 pin connector, you will also need a

9 pin to 25 pin adapter, which can be obtained at most computer

and electronic stores.

SOFTWARE INSTALLATION

CTSMON is distributed in a set of four files:

CTSnnnn.DOC -- quick installation instructions,

CTSnnnn.EXE -- a decompression program,

CTSnnnn.S00 -- installation data for disk 1,

CTSnnnn.S01 -- installation data for disk 2 of 2 or disk

1 of 1,

where the nnnn is the version number of the program.

In the following instructions, the "source directory" refers to

the directory where these four distribution files are stored.

If the source directory happens to be a 360K floppy drive, the

.S01 file must be on a separate diskette, otherwise all four

files should be kept together.  "Destination directory" means

the directory that you want to install the CTS Monitor program

files into.

To install the program files, execute CTSnnnn.EXE with a command

line parameter of the destination directory.  If no destination

directory is specified on the command line, the current

directory is used.  For example, to install version 14.32 of the

CTS Monitor program from two 360K floppy disks to C:\CTSMON,

insert the disk with CTS1432.EXE and CTS1432.S01 on it in drive

A: and type

A:CTS1432 C:\CTSMON

You may wish to add the directory containing the CTS Monitor

program to your DOS path so that you can run it from any

directory.

CUSTOMIZING CTSMON

To start up the CTS Monitor from the DOS prompt, type:

CTSMON

You can customize your installation by selecting System, then

Setup from the menu.  This allows you to:

Specify default file locations

Select a serial port other than COM1:

Force the use of the PC BIOS for video writes

Customize screen mode and colors

See the description of the Setup menu below for more information

on the options available.

CGA & MONCHROME DISPLAYS

If you are experiencing "snow" on a CGA monitor, start the CTS

Monitor program with the /S option on the command line or select

Snow elimination from the System | Setup menu.  Note that many

CGA adapter cards turn off interrupts for extended periods of

time, thus causing dropouts during high speed serial port

communications.  This causes much slower operation of the

monitor when taking diagnostic dumps.

If you are having trouble reading an LCD or monochrome display,

use the /M command line option or select Black & White under the

System | Setup | Colors menu.

OPERATING ENVIRONMENT

The following sections describe some considerations with respect

to your system software.

MEMORY MANAGEMENT AND SPEED

CTSMON will use EMS, if available.  Be sure that the number of

EMS handles available is equal to the number of bytes of EMS

divided by 16384.  Lesser numbers may cause system hangs.  See

the documentation for your EMS driver for details.  If you don't

have EMS, but have a RAM disk, CTSMON performance will be

improved if you set up the temporary files directory (System |

Setup | Temporary files) on the RAM disk.

DOS ENVIRONMENT VARIABLES

The PATH environment variable is used to find data files, like

CTSMON.HLP, when a file cannot be found in the current directory

or in the directory where CTSMON.EXE was started.

COMSPEC is used to perform the System | OS shell command.  If

this variable does not specify the location and name of your

command interpreter (normally COMMAND.COM), then this function

will not work.

If there is no CTSMON.CFG file, and if a TEMP environment

variable exists, it is used to determine where to put temporary

files.  If CTSMON.CFG exists, the TEMP environment variable is

ignored.  See System | Setup above.

RUNNING UNDER MICROSOFT WINDOWS

The CTS Monitor is not a Microsoft Windows application.

However, some people have had good results running the CTS

Monitor under Microsoft Windows 3.1 in 386 Enhanced mode.  This

kind of operation is dependent on:

Manually avoiding serial port conflicts

Having a fast enough computer to avoid dropouts on the

serial port

Assigning a high enough priority to ensure that high

speed serial communications can occur.

The CTSMON.PIF file that comes with the CTS Monitor can be used

for this purpose.  CTSMON.ICO is a Microsoft Windows icon for

the CTS Monitor.  See your Windows documentation for information

on the use of these files.

To use your mouse properly in the CTS Monitor while running

under Windows 3.1 in 386 Enhanced mode, you may want to edit the

SYSTEM.INI file under the [NonWindowsApp] section to add the

line:

MouseInDosBox=1

as well as ensuring that your mouse driver is installed in

AUTOEXEC.BAT or CONFIG.SYS.

Note that running in a multitasking environment will slow down

the operation of the CTS Monitor program.

RUNNING UNDER DESQVIEW

To run the CTS Monitor under DESQview, ensure that your setup

allows enough RAM for CTSMON.EXE, and allows direct access to

the serial ports.  Do not allow the CTSMON.EXE window to be

swapped out of RAM while it is active, since that would be

incompatible with the serial port interrupt handling done by the

CTS Monitor.

Note that running in a multitasking environment will slow down

the operation of the CTS Monitor program.

DOS VERSIONS

CTSMON.EXE has been tested with MS-DOS 3.30, 4.01, 5.00, and

6.00, as well as DR-DOS 6.0 and the OS/2 2.0 DOS compatibility

box.  Other versions of DOS may work, but versions prior to 3.0

probably do not work.

When using DR-DOS 6.0, the disk cache must be configured not to

do block writes, which will interfere with the high speed serial

communications of the CTS Monitor program.

To find out what version of DOS you are running, type VER at the

DOS prompt.

RUNNING UNDER THE OS/2 DOS BOX

This mode of operation might work.  See your OS/2 manual for

instructions on reserving a serial port for a DOS program.  If

you can't get the CTS Monitor to run in the OS/2 DOS box, try

booting under regular DOS.

Note that running in a multitasking environment will slow down

the operation of the CTS Monitor program.

RUNNING CTSMON

The CTS Monitor program is normally run as an interactive, menu

driven application.  It also has a command line interface to

allow loading firmware and taking of diagnostic dumps to be done

from a batch file.

To run CTSMON interactively, start it up by typing

CTSMON

SCREEN AREAS

The screen is divided into several areas.  The top area contains

a description of the currently highlighted menu entry, an arrow

that points up or down, and a digital clock.  The arrow points

up when the control processor is running the main control

program anda points down when it is stopped (running the

monitor-only program).  The digital clock shows the current time

of day (24-hour format).  The clock is updated at least once a

minute while the program is running.  This area might also

display:

R to indicate keystroke macro recording is in progress

P to indicate that keystroke macro play back is in

progress

A small block to indicate the system is logging to the

disk file CTSMON.LOG.

The second area of the screen is the Main Menu.  Except for

Quit, each item on the Main Menu has an associated pull-down

menu.

Below the Main Menu is the message area or dialog window, where

status and error messages are displayed.

USER INTERFACE BASICS

The CTSMON user interface is based on pull-down menus.  You can

select items on the menu using four methods:

Using the arrow keys, move the wide bar cursor over the

item, then press Enter to select it.

Type the highlighted character in the menu item (the

selection key).

Move the mouse cursor over the item you want to select,

and click the left mouse button.

Press the shortcut key (shown to the right of the menu

item name, if that menu item has a shortcut key).  You can use a

shortcut key without displaying the menu item first.

To back out of the current operation, a pull-down menu or data

entry window, or to exit from the text file viewer, press Esc.

As each item on the menu is highlighted with the wide bar

cursor, a help line appears at the top of the screen.  This is a

one-line description of that menu item.

While any menu or data entry screen is active, you can display

help for that menu or screen by pressing the F1 key.

If you are prompted for a yes or no answer, the default answer

is indicated by an upper case letter, as in (y/N).  If you want

to answer yes, type Y. If you want to answer no, press any other

key.

FILE NAME ENTRY

To enter the name of a file that exists on disk (when loading

code or selecting a binary dump file to convert to a text dump,

for example), you can type or edit the file name presented to

you, including drive and path if necessary.

You can also use one or more wild card characters (? and *) in

the file name, then press Enter.  For example, you could enter

A:*.BIN to see all of the files on drive A: with the .BIN

extension.

When you leave a wild card in the file name, a file selection

list appears on the screen.  If you select the name of a

directory in the list, a list of files in that directory that

match that file specification is displayed. Likewise, if you

select ".." in the list, you get a list of files matching your

file specification, one level up in the directory.

SHORT CUT KEYS

Short cut keys allow you to go straight to a function without

navigating throught the menu system.

The table below describes currently assigned short cut keys:

Key     Function

Alt-A   Options -- set EEPROM options

Alt-B   Breakpoint set

Alt-D   Doc file -- display CTSMON.DOC

Alt-E   XRAM display -- Examine External RAM.

Alt-G   Goto breakpoint

Alt-H   Help (same as F1)

Alt-J   Select next available serial port.

Alt-L   Load .BIN file.

Alt-N   Make code load tape (FEPROM contents only).

Alt-O   OS Shell.  Type EXIT to return to CTSMON.

Alt-P   Print -- control background print function.

Alt-Q   Quit -- Exit CTS Monitor.

Alt-R   Run the CTS control processor main program.

Alt-T   sTop the CTS control processor main program.

Alt-V   Enter LEOT test mode.

Alt-X   Exit CTS Monitor program.

F1              Help

F2              Create dump file

F3              View text file

F4              Binary to text

F9              SCSI command/status trace

Alt-F1  Reset servo

Alt-F2  Load tape

Alt-F3  Unload tape

Alt-F4  Find LBOT

Alt-F5  Go track

Alt-F6  Stop tape

Alt-F7  rewind t o PBOT

Alt-F8  rewind Unload

Ctrl-F1 Display version information and memory status.

Ctrl-F2 Start keyboard recording to create a macro.

Ctrl-F3 Play back macro recorded earlier.

Ctrl-F4 Play back macro recorded earlier in a loop.

Ctrl-F5 Stop macro play back.

Ctrl-F6 Toggle logging to disk.

Ctrl-F7 Comment

Ctrl-F8 Select screen capture name.

Ctrl-F9 Capture screen to file.

COMMAND LINE SWITCHES

A file name on the command line means to load that .BIN file

into the CTS.

Command line switches may be preceded either with a - or /, and

may be upper or lower case.

/1 - /17 = Use serial port number 1 through 17:

0 = SERIAL 4

1 = COM1:

2 = COM2:

3 = COM3:

4 = COM4:

5 = SERIAL 5

6 = SERIAL 6

7 = SERIAL 7

8 = SERIAL 8

9 = SERIAL 3

10 = MSO 0

11 = MSO 1

12 = MSO 2

13 = MSO 3

14 = MSO 4

15 = MSO 5

16 = MSO 6

17 = MSO 7

/B      Use BIOS for screen writes (eliminates CGA

snow).

/C1     Use COM1:

/C2     Use COM2:

/C3     Use COM3:

/C4     Use COM4:

/D      Dump to SNAPSHOT.85 and SNAPSHOT.TXT.

/Dfilename      Dump to filename.85 and filename.TXT

/E      Use EGA 43 line or VGA 50 line mode.

/M      Force monochrome color mapping.

/S      Use BIOS for screen writes (eliminates CGA

snow).

*MSO means Multiple Serial Output Card, like the one from

Personal computing Tools, Inc., configured on IRQ3.  Bank 0

starts at 280H, and bank 1 starts at 2A0H.  Use of 16550 AFN

UARTs instead of 16450 UARTs will result in better performance.

The default settings for serial port and display mode settings

are the same as what you used the last time you ran CTSMON.

These settings are stored in CTSMON.CFG.  If CTSMON.CFG does not

exist, the defaults are COM1:, fast screen writes (not using the

BIOS), and allow color (don't force monochrome).  If the

specified serial port does not exist when the program is

started, another serial port will be tried.  These options can

also be set with the System/Setup menu.

The following examples demonstrate how command line switches can

be used:

CTSMON /1

Starts CTS Monitor on COM1:

CTSMON /2

Starts CTS Monitor on COM2:

CTSMON /M

Starts the CTS Monitor in monochrome mode.

CTSMON /E

Starts the monitor in 43 or 50 line color mode.

CTSMON /DDUMP

Starts CTS Monitor, creates dump files DUMP.85 and DUMP.TXT (or

DUMP.DMP if the CTS is an EXB-8200 or EXB-8200sx), then exits.

CTSMON 85C-0001.BIN

Loads the code file 85C-0001.BIN, then exits.

CTSMON /E /2

Starts the monitor in 43-line or 50-line mode on COM2:.

DOS ERRORLEVEL EXIT CODES

If you use the CTS Monitor in a batch file or call it from

another program, it is useful to know what the following

ERRORLEVEL numbers mean:

0:      Normal exit.  No problem.

1:      Command line confused or help requested.

2:      Break pressed, program halted.

3:      Out of memory.

4:      Windowing function failure -- out of memory.

5:      COM port not found.

6:      Window stack underflow (programming error).

7:      Main menu definition or memory allocation error.

8:      Program damaged -- get a new copy!  (CRC check failed).

9:      Unable to write to output file.  Disk full?

10:     Array write out of bounds (programming error).

11:     Window stack overflow (programming error).

12:     Virtual array error!  Not enough RAM or disk space!

13:     Virtual array index out of range (programming error).

14:     Dump failed.

15:     Load failed.

16:     Function not available on EXB-8200.

17:     Memory corruption error.

18:     Out of RAM.

19 to 255:      Generic program error trap numbers.

TELEPHONE TRANSFER OF DATA

Dumps can be transferred to the Exabyte Technical Support

Bulletin Board System (BBS) at 303-447-7100.

To transfer a dump file or a code load file over a modem

requires a separate telecommunications program that supports a

protocol like XMODEM, YMODEM, ZMODEM, or Kermit.  To transfer

the CTS Monitor program over a modem, it is better to transfer

the CTSnnnn.EXE, CTSnnnn.S00, and CTSnnnn.S01 "install" program

(actually just a self-extracting archive) than to transfer the

individual pieces that make up the monitor. This saves on

telephone connect time and keeps the files together.

The CTS Monitor program is not designed for remote connection to

a CTS by way of modem.  Line noise and communications delays may

damage your CTS or firmware.  It is possible, however, to use a

remote connection type program like PC Remote or PC Anywhere to

remotely operate a PC that is running CTSMON.  If you do this,

be careful not to connect CTSMON to the same port as the modem.

MAIN MENU

The Main menu contains seven options:

System -- CTS Monitor setup, customization, help, and

keyboard macro functions.

Firmware -- load new firmware, set configuration

options, make code load tapes, and make self test tapes.

Dump -- extract diagnostic dumps from a CTS and view

them.

eXamine -- view various internals of the CTS.

Control -- send instructions to the CTS control

processor.

Servo -- send instructions to the CTS servo processor.

Quit -- exit the CTS Monitor and return to the operating

system.

SYSTEM

The System menu allows you to execute operating system commands,

select the serial port, and do other CTS Monitor setup

functions.  It contains the following options:

OS Shell -- execute one or more DOS commands.

COM port -- select which serial port you want to use.

Doc file -- view the on line, plain ASCII text file

version of this documentation.

Help -- use the hypertext style help system.

Setup -- set configuration options.

Macros -- record or play back timed keystrokes.

Loopback test -- check your serial port hardware.

eXit -- return to the operating system.

System | OS shell  (Alt-O)

OS Shell leaves the CTS Monitor program active in memory while

returning you to the DOS prompt.  You can return to the CTS

Monitor program with the EXIT command.

Do not install any TSR programs with the OS Shell command. TSR

programs include commands like DOS PRINT that stay in memory

when the program ends.  If you want to print something without

exiting the CTS Monitor, use the Print command under the Dump

file menu.

The serial port is "live," and received characters will be

buffered while the other command is executing.

Running another program that uses the serial port with "OS

Shell" is NOT recommended.  This could result in serial

communications not functioning when you return to the CTS

Monitor program.

The shortcut key to shell out to DOS is Alt-O.

If the program or command you are trying to run fails because

not enough memory is available, quit the CTS Monitor to remove

it from RAM, then try running the program or command again from

the DOS prompt.

System | COM port

Use COM port if you have more than one supported serial port and

want to switch to another.  This brings up a submenu that allows

you to select the serial port that you want to connect to.

Although many serial ports are supported, only one serial port

can be used at one time.  If another serial port (for example, a

mouse) is being used at the same time the CTS Monitor is

communicating with the CTS, the other serial port must not use

the same IRQ number. For example, using COM1: or COM3: while a

mouse is on COM2: is OK, but using COM1:  while a mouse is on

COM3: is not.   (See the table below for IRQ numbers.)

The CTS Monitor also supports nonstandard serial port

configurations, such as the Multiple Serial Output Card

(RS-232/8s or RS-232/8si) sold by Personal Computing Tools,

Inc.

Note:  Connecting the CTS Monitor to a serial port that is in

use for a mouse may cause the computer to lock up.

COM1:/SERIAL 1, etc.

Select the port name that you would like to connect to.  If it

is not available, the CTS Monitor will reconnect to the port

that it was last connected to.

MSO Card

This entry causes a pop up entry window to prompt you for the

MSO port (0 to 23) that you want to connect to.  If the port is

available, CTS Monitor will connect to it.  If not, CTS Monitor

reconnects to the original port.

Next port  (Alt-J)

This entry causes CTS Monitor to connect to the next available

serial port. Serial ports are scanned in the following order:

COM1:

COM2:

COM3:

COM4:

SERIAL 5

SERIAL 6

SERIAL 7

SERIAL 8

MSO 0 through MSO 23

SERIAL 3

SERIAL 4

CTS Monitor skips over any of the above serial ports that are

not present with an error message.  Not all of the above serial

ports can coexist in the same PC, so you should expect some

missing serial ports.

By default, CTS Monitor connects to the serial port you

connected to last time, unless you specify otherwise on the

command line.  If it finds no CTSMON.CFG and no port specified

on the command line, it uses COM1.

System | Doc file  (Alt-D)

"Doc file" allows you to read online documentation (this file).

You can read this document if the file CTSMON.DOC is available

in the default directory, in the directory where CTSMON.EXE was

started, or somewhere in your DOS search path.  Press Alt-D from

the main menu to read the documentation.

NOTE:  Although you can print CTSMON.DOC, you will get higher

quality hard copy of this document if you connect to a

PostScript printer, then issue the DOS command

COPY CTSMON.PS LPT1

System | Help  (F1)

"Help" brings up the help system, as does pressing F1.

The help displayed usually says something relating to what is on

the screen.  If you want to look at something different, select

one of the highlighted topics on the screen to get you to

another help screen.  To select another topic, move the cursor

to the topic you want and press Enter, or move the mouse cursor

over the topic and click the left mouse button.

To exit the help system, press Esc.

System | Setup

Setup displays a submenu of customization options.

When CTS Monitor starts up, it looks for the CTSMON.CFG setup

file in the current directory.  If it doesn't find the file, it

looks in the directory where the program was started from, then

in all directories in your DOS path.  If it finds the file, it

uses that information to set up your system.  It then scans the

command line for options after reading the configuration file,

so command line options override specifications in the

configuration file.

When you change any option from this menu, the change is stored

in that file.   If you want to have one setup used all the time,

no matter what directory you are running the CTS Monitor from,

select all of your setup options from the directory where you

keep CTSMON.EXE.  Those options will then be used when you start

up the monitor later, even if you start CTS Monitor from a

different directory, unless there is a CTSMON.CFG file in that

directory as well.

Temporary files

Temporary files allows you to determine where you want temporary

files stored. In order of fastest to slowest, this should be (1)

a RAM disk that has at least 384 KBytes free, (2) a hard disk,

or (3) a floppy disk.  The default value is the contents of the

TEMP environment variable if there is no CTSMON.CFG file, or the

current directory if there is no TEMP environment variable.

.BIN file directory

If you did not specify a directory for .BIN files during

firmware load, you can use BIN file directory to specify where

CTS Monitor looks for .BIN files.

Dump file directory

Dump file directory determines where dump files (.85 and .TXT)

go when you don't specify a path when prompted for a file name

to dump to.

Macro directory

Macro directory specifies the default location of keystroke

macro files.  See System | Macros, below.

Snow elimination is ON/OFF

Snow elimination causes all screen writes to go through the BIOS

instead of writing directly to video RAM.  Turn this function on

if your display isn't working in CTS Monitor, or if you have a

CGA monitor and see "snow" on your screen every time something

changes (like the clock in the corner). Otherwise, leave this

off for faster performance.

43/50 line mode is ON/OFF

43/50 line mode allows you to take advantage of the EGA 43 line

mode or the VGA 50 line mode to display more information on your

screen.  ON means use 43 or 50 line mode (depending on the type

of video display card you have). OFF means leave the video mode

as it was (as long as the number of columns is 80).  The default

setting is OFF.

This option has no effect on MDA or CGA cards. If the display

mode is set to 35, 40, 43, or 50 line mode before the CTS

Monitor is started, it will stay in that mode regardless of

this setting.

Colors

Colors brings up a submenu that allows you to set colors.  If

you are using a monochrome display, turn Black & White ON and

select Green colors.

If you are using a color (CGA, EGA, or VGA) display, and don't

like the default green colors, try bLue colors.  If you don't

like those, you can play with the other menu selections until

you get a combination you like.

Color changes do not completely take effect until you quit and

restart the CTS Monitor.

MSO serial Port address

CTSMON supports up to 24 nonstandard serial port addresses.

Since they are nonstandard, you will probably have to

reconfigure both the serial port card and the CTS Monitor to

select IRQ and I/O Port addresses that do not conflict with

other hardware in your computer.

When you select this menu item, a pop-up window prompts you for

the MSO serial port number that you wish to modify.  When you

enter that number, another pop-up window appears that allows you

to edit the current settings for that serial port.

You should select an IRQ value of 3 or 4, and an I/O port

address that does not conflict with other PC hardware.  If you

wish to lock out a serial port so that it is not tried when the

CTS Monitor cycles through serial ports, enter an I/O address of

0.

Caution

Make sure that no two devices in your PC use the same I/O port

address, and that the serial port addresses configured with this

option are accurate.

Also, avoid using the same IRQ at the same time, for example,

for a mouse and a CTS.

The table below shows suggested serial port configuration

values.  Your hardware installation may require the use of

different values if you have more than one MSO card.  A port

address of 0 disables that port.

NAME    IRQ     PORT ADDRESS (HEX)

COM1:   	4       3F8

SERIAL 1        4       3F8

COM2:   	3       2F8

SERIAL 2        3       2F8

COM3:   	4       3E8

COM4:   	3       2E8

SERIAL 3        3       3220

SERIAL 4        3       3228

SERIAL 5        3       4220

SERIAL 6        3       4228

SERIAL 7        3       5220

SERIAL 8        3       5228

MSO 0   	3       280

MSO 1   	3       288

MSO 2   	3       290

MSO 3   	3       298

MSO 4   	3       2A0

MSO 5   	3       2A8

MSO 6   	3       2B0

MSO 7   	3       2B8

MSO 8   	3       2C0

MSO 9   	3       2C8

MSO 10  	3       2D0

MSO 11  	3       2D8

MSO 12  	3       220

MSO 13  	3       228

MSO 14  	3       230

MSO 15  	3       238

MSO 16  	3       240

MSO 17  	3       248

MSO 18  	3       250

MSO 19  	3       258

MSO 20  	3       300

MSO 21  	3       308

MSO 22  	3       310

MSO 23  	3       318

System | Macros

To automate repetitive tasks, there is a keystroke macro

recorder built into the CTS Monitor.  Because many actions

involving the monitor depend a great deal on timing, the

keystroke recorder remembers not only which key was pressed, but

how long of a delay there was since the last key was pressed.

Times are recorded with a resolution of 54.95 milliseconds, and

a maximum time interval of one day. Keystroke macros are

recorded in binary files (6 bytes per key stroke) with a .KEY

suffix. These files are not modifiable with a text editor.  You

may have as many macros of any size as you want.

This menu item also contains functions for logging CTS Monitor

actions to disk and capturing the current screen to a disk file.

These functions may also be useful outside the context of using

keystroke macros.

Macros may be chained together.  This means that one macro

passes control to another macro.  You can do this by selecting a

macro to play back while recording a macro.  The recording stops

before the actual play back starts, so when the first macro is

played again, it will start the second one up just as it quits.

Note that the keystroke recorder and player is just that --

there is no provision for error detection.  This means that you

must put some thought into the macro recording process.  To make

reliable macros:

1.      Begin the macro with about five Escapes.  This ensures

that you are back to the main menu, giving you a predictable

starting point. Because macro playback can be started from

anywhere with a hot key (Ctrl-F3 or Ctrl-F4), you don't know

what menu is active at the beginning of the play back process

without this step.

2.      Select menu items with the highlighted letters or hot

keys, rather than with cursor movements and the Enter key.  The

starting position of the cursor when you run the macro may not

match its starting position when you recorded the macros.

3.      Use the Comment function to put reminders, comments, or

prompts into a macro.  While a macro is being played back, both

the macro and the keyboard are active.  Therefore, you can pause

for the user to enter some data.

Record begin/end  (Ctrl-F2)

When you first select this option, you are prompted for the file

name for storing the macro.  The file name must end in .KEY.  As

soon as the file is opened, the keystroke (and time) recording

begins.  Macro recording continues until you select this

function again, start up another macro, or quit the CTS Monitor

program.  An "R" on the display, next to the arrow, indicates

that keystroke recording is in progress.

Play back once  (Ctrl-F3)

When you select this function, you are prompted for the file

name of the macro.  A "P" will appear on the display next to the

arrow to indicate that play back is in progress.  To stop play

back before the end of the keystroke macro file is reached,

press Ctrl-F5.

Loop play back  (Ctrl-F4)

This function is just like the Play back once function, except

that the macro is played back for the number of times you

specify at the prompt.

Stop macro  (Ctrl-F5)

This function allows you to stop a macro before it is finished

playing back.

The hot key, Ctrl-F5, is available for the same purpose.

Disk log ON/OFF  (Ctrl-F6)

When you use a macro for an unattended test, you can record what

went on while you were gone.  When you turn disk logging on, the

ASCII text file CTSMON.LOG is opened in the current directory,

and much of what is written to the dialog window on the screen

is written to this file.  If the file already exists, new

entries are appended to the end of the file.  Each time the file

is opened, the date and time are written to the file.

When disk logging is on, a small square appears next to the

clock on the display.

Note: If you are going to be running a macro repetitively, and

you toggle disk logging on and off in the macro, be sure that

you invoke this function an even number of times so that the

disk logging will be on at the right times.

Comment  (Ctrl-F7)

This function opens up a four-line text window on the screen for

you to type any comment you want.  The text window remains on

the screen until you press Ctrl-Enter, Esc, or Enter from the

last line of the text window.  This can be used for adding

comments to a macro file.

set capture File name  (Ctrl-F8)

This function allows you to specify the name of the file that

you want screen image files to be written to when you press the

~ key.  The default name is CTSMON.CAP in the dump directory.

Grab screen to file  (~)

Pressing ~ causes the contents of the current screen to be

appended to the file specified with the Set Capture File Name

function.  You can also do this from this menu selection, but

the menu will be written to the file as well. Line draw

characters and other graphics characters will be written as they

appear on the screen, so it is possible that some screen images

may need to be filtered by another program before printing or

importing to another program.

System | Loopback test

The loop back test is useful if you want to make sure that your

PC serial port is functioning properly.  To run the loop back

test,

1.      Unplug the serial port cable from the CTS.

2.      Electrically connect pins 2 and 3 of the connector on

the CTS end of the cable together using a small piece of wire.

3.      Select the loop back test function, then press Y to

confirm that you want to do the test.

The test sends 1024 randomly generated characters at 9600 baud

and checks to see if they were received successfully.  If they

were, a message stating that the test passed will be displayed

on the screen in a small over one second.  If not, a message

stating that the test failed will appear.

System | eXit  (Alt-X or Alt-Q)

There are several ways to exit the CTS Monitor program.  You can

select "Quit" from the main menu or "eXit" from the System menu;

you can press the shortcut keys Alt-X or Alt-Q; or you can press

Ctrl-Break (hold the Ctrl key and press the Break key).  The

last method is intended as an unconditional exit, without regard

to the condition of the CTS.

FIRMWARE

The Firmware menu allows you to load code updates to the CTS's

firmware (from .BIN files) or to change configuration options.

It also allows you to create special tapes for loading firmware

or testing a CTS.

Firmware | Option setting  (Alt-A)

Selecting Option setting loads the current selectable options

from the CTS, then displays these options on the screen for you

to change if you want.  After setting options on screen, you can

write them into the nonvolatile memory of the CTS.  These

changes go into effect after a power-on reset and stay in effect

until they are changed with a MODE SELECT command.

The configuration options enable you to specify the power-on

default values for:

Autosize the Tape

Buffered Mode

Button Action

Cartridge Type

Command Queuing

Default Density Reporting Code

Directory Support

Even Byte Disconnect

Gap Threshold

Lock Write Density to Power-up Value

Logical Block Size

Maximum Burst Size

Mode SEL/SENS

Monitor Data Compression

Motion Threshold

No Autoload

No Disconnect in Data Transfer

Prevent Media Removal

Reconnect Threshold

Report Actual Subproduct ID

Report Early Warning

Report Media Type

Request Sense Style

SCSI Bus Parity Checking

Sync Negotiate

Tape Density Legal at non-LBOT

True (areal) Density Selection

Write Delay Time

Write Mode

Note: Some options are not applicable to older versions of CTS

firmware.

Autosize the Tape

Sets the operation of the tape autosize function. If set to N,

the tape size is determined by the initial estimate at LBOT

along with the setting of the Cartridge Type option. If set to

Y, the tape is sized as tape motion proceeds.

Buffered Mode

Sets the buffer mode for write data. If set to Y, a write or

write file mark command will complete with Good status when the

data is written into the buffer; it does not indicate that the

data was successfully written to the tape. If set to N,

completion is indicated after the data has been written to the

tape. For write streaming, this must be set to Y.

You can override this power-on default option using the MODE

SELECT command.

Button Action

Controls what the CTS does when the front panel button is

pressed.  Select one of the following:

Complete command, clr queue, write buf & EOD, rewind & eject

Abort command, clr queue, write buf & EOD, rewind & eject

Abort command, clr queue, write buf & EOD, eject w/o rewind

Cartridge Type

Specifies the type of cartridge that the CTS expects when

loading a tape. Select one of the following:

15m/54m

Unknown

112m

You can override this power-on default option using the MODE

SELECT command.

Command Queuing

Controls queuing option for the CTS. The CTS allows tape

commands to queue when an immediate command is executing

(including power-on self-test and tape loading), and allows

non-tape commands to execute immediately.  Select one of the

following:

Queue new command

Busy (Send BUSY status when busy. No queuing)

Check Condition - Sense Key = Not Ready (No queuing)

Default Density Report Code

When set to Y, the EXB8500c never reports 00h for the Density

Code; instead it reports the actual tape format (e.g., 14h, 15h,

8Ch, or 90h). Setting the Density Code in MODE SELECT to 00h

will select the default tape format for the CTS.

When set to N, the EXB8500c reports 00h for the Density Code

instead of the default tape format. Note that this option does

not affect how the Density Code in MODE SELECT functions.

Directory Support

Configures operation of directory support by the CTS. Select

from the following:

Directories allowed, without file marks

Directories allowed, including file marks

No directories allowed

Even Byte Disconnect

Select one of the following:

4-on

Sets the even byte disconnect boundary to a multiple of 4 and

enables the even byte disconnect function.

4-off

Sets the even byte disconnect boundary to a multiple of 4 and

disables the even byte disconnect function

2-on

Sets the even byte disconnect boundary to a multiple of 2 and

enables the even byte disconnect function.

2-off

Sets the even byte disconnect boundary to a multiple of 2 and

disables the even byte disconnect function.

The even byte disconnect function power-on default value can be

overridden by the MODE SELECT command; the boundary value (2 or

4) cannot be overridden by the MODE SELECT command.

Gap Threshold

Specifies the gap threshold for the CTS. This value sets the

maximum number of consecutive gap blocks that can be written on

the tape.

Enter the value in hexadecimal (0x...) or decimal. The legal

range for this value is 0x0 (0) to 0x7 (7).

You can override this power-on default option using the MODE

SELECT command.

Lock Write Density to Power-up Value

Configures whether or not the host can change tape density with

a MODE SELECT command.

If set to Y, the density may not be changed from the power-up

default value. A MODE SELECT command to change density will be

ignored.

If set to N, the host may select another density with a MODE

SELECT command regardless of the power-up default setting.

Logical Block Size

Specifies the power-on default fixed block size. A value of 0

means variable block mode.

Enter the value in hexadecimal (0x...) or decimal. The legal

range for this value is 0x0 (0) to 0x3C000 (245760). Block sizes

lower than 0x400 (1024) bytes result in reduced performance.

You can override this power-on default option using the MODE

SELECT command.

Maximum Burst Size

Specifies the maximum burst size. This is the maximum number of

bytes that can be transferred across the SCSI bus (either to or

from the CTS) before a disconnect is performed.

Enter the value in hexadecimal (0x...) or decimal. The legal

range for this value is 0x0 (0) to 0xFFFF (65535).

You can override this power-on default option using the MODE

SELECT command.

Mode SEL/SENS

Configures the operation of the MODE SELECT and MODE SENSE

commands. Select one of the following:

8200 style (SCSI-1), no pages

8500 style, with length check in MODE SENSE

SCSI-2, without length check in MODE SENSE

Note: Depending on the firmware version and product

identification, you might see a subset of those options, or

additional options specific to your installation.

Monitor Data Compression

Configures compression mode processing. If set to N, compression

is always done, regardless of a previous block enlarging. If set

to Y, compression will automatically be turned off if a

compressed block gets larger, and will be turned back on when a

compressed block once again becomes smaller.

This option will be displayed only if the CTS is compression-

capable.

Motion Threshold

Specifies the motion threshold for the CTS. The value is given

in 4-KByte units. It specifies the number of bytes that must be

present in the buffer before a write causes tape motion or the

number of bytes available in the buffer before a read causes

tape motion. The value can also be interpreted as a fractional

part of 256 for the percentage of buffer space available for

data.

Enter the value in hexadecimal (0x...) or decimal. The legal

range for this value is 0x20 (32) to 0xD0 (208).

You can override this power-on default option using the MODE

SELECT command.

No Autoload

Sets the no-autoload state of the CTS. If set to Y, the CTS will

not load a tape that is inserted. Also, if the CTS is reset with

a tape inserted, the tape will remain unloaded. Only the

LOAD/UNLOAD command will load the tape. If set to N, the CTS

will load an inserted tape automatically.

You can override this power-on default option using the MODE

SELECT command.

No Disconnect in Data Transfer

If set to N, disconnects will be allowed in a data stream, which

maximizes efficient use of the SCSI bus.

If set to Y, disconnects will not be allowed in the middle of a

data stream.  This option may be better in the case of a

single-host, single-target SCSI bus.

You can override this power-on default option using the MODE

SELECT command.

Prevent Media Removal

Allows you to prevent media removal. If set to Y, the front

panel button is disabled and tape will not be ejected when an

unload operation is performed.

You can override these power-on default options using the

PREVENT/ALLOW MEDIUM REMOVAL command.

Reconnect Threshold

Specifies the reconnect threshold for the CTS. The value is

given in 4-KByte units. It specifies the number of bytes that

must be free in the buffer before a write operation causes the

CTS to reconnect to the SCSI bus, or the number of bytes present

in the buffer before a read operation causes the CTS to

reconnect.  The value can also be interpreted as a fractional

part of 256 for the percentage of buffer space available for

data.

Enter the value in hexadecimal (0x...) or decimal. The legal

range for this value is 0x20 (32) to 0xD0 (208).

You can override this power-on default option using the MODE

SELECT command.

Report Actual Subproduct ID

Configures reporting of the Subproduct ID field (bytes 24 to 31)

in inquiry data. If set to Y, the normal bytes are reported (the

name of the EEPROM). If set to N, this field is set to blanks.

Report Early Warning

Sets the status of reporting early warnings on read operations.

For write operations, early warnings are reported after LEOT is

passed on the tape. For read operations, reporting passing LEOT

is optional. If set to Y, early reporting for read operations is

enabled. If set to N, early reporting for read operations is

disabled.

You can override this power-on default option using the MODE

SELECT command.

Report Media Type

Controls reporting of media type in mode sense data. If set to

Y, the media type is included in the sense data. If set to N,

the media type is reported as zero (0) (SCSI-2).

Request Sense Style

Specifies the format for request sense data. This value sets the

number of additional bytes of sense data that are returned.

Select one of the following:

0x12 - Deferred error code never returned in byte 0

(8200)

0x14 - FSC not included

0x15 - Standard number of bytes (8500)

SCSI Bus Parity Checking

Enter Y to check for parity errors on the SCSI bus (recommended)

or N to ignore parity errors (not recommended).

You can override this power-on default option using the MODE

SELECT command.

Sync Negotiate

Configures operation of synchronous negotiation messages. Select

from the following:

Synchronous negotiation messages accepted, not

initiated

Synchronous negotiation messages rejected

Synchronous negotiation initiated by CTS after reset

Tape Density Legal at non-LBOT

Configures the legal density selection when not at LBOT.

If set to Y, the density required by MODE SELECT at non- LBOT is

the same as the density returned by MODE SENSE. This is the

density of the tape. MODE SENSE followed by MODE SELECT will

always work without error in this mode.

If set to N, the density required by MODE SELECT at nonLBOT is

the write density of the CTS. This may be different than the

tape density. MODE SENSE followed by MODE SELECT may return

error status due to an illegal attempt to change density.

True (areal) Density Selection

Configures how the Density Code is used with compression.

If set to Y, the density code selects only the true density

(areal, i.e., bits per area).  To change the format between

densities 14 and 90 or 15 and 8C requires the use of page F.

This option has no effect on a CTS that does not allow

compression.

If set to N, the Density Code selects the compression format as

well as the true density.

Write Delay Time

Specifies the write delay time in units of 100ms. This sets the

maximum length of time that data remains in the buffer before

the CTS writes it to tape. When the time is exceeded, the data

is written. If set to zero (0), there is no maximum write delay

time.

Enter the value in hexadecimal (0x...) or decimal. The legal

range for this value is 0x0 (0) to 0x3FFF (16383).

You can override this power-on default option using the MODE

SELECT command.

Write Mode

If the CTS is capable of compression operations, select one of

the following:

8500 (15h format) - Compression capable - Disabled

8500c (8Ch format) - Normal compression

8200 (14h format) - Compression capable - Disabled

8200c (90h format) - Normal compression

If the CTS is not capable of compression operations, select one

of the following:

8500 (15h format) - No compression/decompression capability

8200 (14h format) - No compression/decompression capability

Firmware | Load code from .BIN  (Alt-L)

You can load firmware into the Flash EPROM (FEPROM) of a CTS in

three ways:

Use the CTS Monitor, CTS Monitor, CODELOAD, or 8POPS to

load code from a .BIN file over the serial port.

Load a special tape that contains instructions to the

CTS to load its firmware from tape.

Send the code over the SCSI bus using the Write Buffer

command.  See the CTS Interface User's Manual for details.

Using CTS Monitor to load firmware is the most reliable method,

because this method doesn't require the present of good

firmware in the FEPROM before the load begins.  Thus, you can

retry a code load operation if it fails.  This is the only

method that allows you to recover.  Some versions of firmware do

not support the tape or SCSI bus option.

Loading code from tape is the most convenient method.  Simply

insert the tape into the CTS while it is powered on and running,

and the new firmware is automatically loaded with no further

action needed on the SCSI bus or the monitor port.  A firmware

load tape can only be reused as a data tape on an EXB-8200 or

EXB-8200SX.

Loading code from the SCSI bus is the fastest method because of

the speed of the bus.  The disadvantage of this approach is that

it probably requires a different program for each computer and

SCSI bus adapter combination.  SCSI bus loads use .TAP files

instead of .BIN files.

CTS firmware for loading by CTS Monitor (control and servo

programs, and/or some of the EEPROM data) is distributed as a

DOS file with the extension .BIN.

1.      Select Firmware, then Load code from the menu. You are

prompted for the name of the file containing the firmware.

2.      Either edit the file name, or, if you leave any wild

cards (* or ?) in the file name, select the file from a list of

files matching that description. A prompt asks, "Are you sure,"

giving you a last chance not to change the firmware.

3.      Enter "Y" to continue.  Press any other key to leave

your firmware unchanged.

4.      Remove any data cartridge from the CTS before loading

code. If the load fails for any reason (for example, a power

failure), the servo mechanism may damage any tape left in the

CTS.  The CTS attempts to rewind and unload the tape before a

load, but may not have enough time for a long rewind.

Firmware | Make code load tape

A firmware code load tape is a special tape that causes firmware

and/or EEPROM configuration information to be loaded into a CTS

simply by inserting the tape into the CTS.

To make a code load tape:

1.      Unload any data cartridge currently in the CTS.

2.      Ensure the CTS is idle (no SCSI bus activity).

3.      Select the Make code load tape option from the Firmware

menu.

4.      Select the type of code load tape you want to make by

answering "Y" or "N" to the three questions:

Include control & servo FEPROM code?

Include mode select defaults from CTS?

Include vendor unique options from .BIN?

If you include vendor unique options from a .BIN file, mode

select defaults will also be loaded from that .BIN file unless

you also select mode select defaults from CTS.  Mode select

defaults from the CTS can only be written to tape in combination

with control & servo FEPROM code and/or vendor unique options

from a .BIN file.

If you want to make more than one code load tape with the same

contents, enter the number you wish to make.

5.      If you are making a code load tape that includes any

vendor unique configuration information from a .BIN file on

disk, enter the name of the .BIN file in response to the prompt.

You can also enter a file specification that includes one or

more DOS wild cards if you want to select the file from a list.

6.      Insert a new blank tape in the CTS.  A 15 meter tape is

adequate, since less than one meter is actually used.

7.      Wait for the tape to be ejected and a message to appear

on the screen indicating if the operation was successful or

not.

8.      Label the code load tape with the version and type of

firmware that is on it.

To copy a firmware load tape, first load the firmware from the

tape by inserting the load tape into the CTS.  Then follow the

instructions above for creating a firmware load tape.

Once a tape is encoded as a firmware load tape, it cannot be

used for a data tape unless it is first erased in an EXB-8200 or

bulk erased since any attempt to do so with a CTS would result

in a code load operation.  Firmware load tapes can be

overwritten with a new version of firmware, however.

If you don't have a .BIN file with your current version of

firmware in it, then it is a good idea to make a code load tape

with your current firmware on it before loading new code with

CTS Monitor.  That way you have a backup version of this level

of firmware in case you need it for any reason.

MASS PRODUCING CODE LOAD TAPES

To produce large quantities of code load tapes, it is useful to

use an EXB-10, EXB-10i, or EXB-10e Cartridge Handling System

(CHS) in sequential mode to produce up to 10 at a time.  To do

this,

1.      Set up the CHS in sequential mode following the

instructions in its user's guide.  Set up the CHS to cycle only

once by turning the loop option off and the restart option on.

This prevents the CHS from continuing to load code into the CTS

forever after the tenth tape is loaded.

2.      Load ten new tapes into the CHS and turn the CHS power

on, but keep the CHS door open.

3.      After the CTS power on sequence is complete (as

indicated by the lights on the CTS), follow the instructions in

the above section for starting the code load tape creation

process, specifying the number of tapes you wish to write in

step 4.

4.      When you are prompted to insert the first tape, shut the

CHS door.  The CHS will load all ten tapes sequentially, then

stop.

USING A CODE LOAD TAPE

Warning

Removing power or resetting the CTS while it is loading code

into the Flash EPROM will result in an inoperable CTS.  If this

occurs, reload code using CTS Monitor. A reload from the code

tape cartridge will not work.

1.      Ensure that the CTS power is on and that the CTS is

unloaded and idle.

2.      Ensure that no SCSI bus reset will occur -- disconnect

the SCSI connector if it is accessible.

3.      Insert the code tape and wait for it to be ejected.  The

CTS loads the new code from the tape to its buffer and verifies

that the code load is good.  If the code passes its checksum

tests, the cartridge is ejected.

4.      After the code is loaded in the FEPROM, the CTS executes

a power-on reset.  After this reset is complete, you can resume

normal operation with the CTS.  CTS Monitor is not required to

load code with a code load tape, but if CTS Monitor is running,

it indicates with a message when the load is complete.

Firmware | Fix bad head sync

This option is not normally needed.  If the head sync value in

your CTS EEPROM image (the first two bytes) gets corrupted to

the extent that an automatic head sync tape will not function,

use this menu selection to change the value to the default head

sync value.  After doing this, turn the CTS power off, then on.

After the reset sequence, insert an automatic head sync setting

tape into the CTS. This will restore proper operation.

If you use this menu selection when the head sync value is

already legal, CTS Monitor does not change the head sync value

and reports the current head sync value.  If you want to ensure

that the head sync is properly set, use an automatic head sync

tape to reset the optimum value.

Firmware | Create self test tape

A self test tape is a special tape that causes the CTS to

perform self diagnostics when inserted.  To make a self test

tape:

1.      Ensure that the CTS is powered on, finished with its

power-on self test, idle, unloaded, and has no SCSI bus activity

going on.

2.      Select Create self test tape.  When prompted, answer "Y"

to indicate that the CTS is ready.

3.      When prompted, insert a blank data cartridge (15 Meter

size is adequate) and wait for it to be ejected.

Firmware | Show contents of .BIN file

Select this option to find out what is in a .BIN file without

loading it. CTS Monitor prompts you to choose the .BIN file,

then displays the contents on the screen. You are also asked if

you want to extract any release notes embedded in the .BIN file

to a disk file.

DIAGNOSTIC DUMPS

There are two dump file formats:  binary format and ASCII text.

The binary format is a compact representation of all of the most

important data in the CTS processors and can be used to

reproduce the ASCII text format.  The text format is a

translation of part of the information in the binary dump to a

readable ASCII format (though it contains hexadecimal).  The

EXB-8200 and EXB-8200sx dumps are available only in an ASCII

format.  All other dumps are available in both binary and ASCII

format.

When sending a dump file back to Exabyte Corporation for

analysis, send only the binary file.  This file is required for

Engineering analysis and the text format can be created from

that.  This saves disk space and modem time.  (A binary file

cannot be created from an ASCII file.)

NOTE:  The process of creating a dump file might cause servo

errors in your CTS.  After creating a dump file, you might have

to power the CTS off and back on again to make it function

normally.

Creating or translating a dump file has the side effect of

placing the CTS Monitor program in analysis mode, so that

functions in the Examine menu which do not involve the one

megabyte buffer will refer to the static dump data instead of an

attached CTS.  In analysis mode, the run/stop arrow in the upper

right hand corner of the display turns to a diamond.

Create dump file  (F2)

The Create Dump File menu option prompts you for a base file

name (drive, path, and name without extension).  Do not enter an

extension for the file name, it will be ignored.  If the

connected CTS is an EXB-8200 or EXB-8200sx, a single ASCII dump

file is automatically created with a .DMP extension.  If the

connected CTS is an EXB-8500, EXB-8500c, EXB-8505, or EXB-8205,

then two dump files are created.  One of the files is a binary

dump with the extension  ".85", and the other is a text dump

with the extension ".TXT".  If either of these files already

exists, you are asked if you wish to overwrite them.

When you create a new dump file with the Create dump file

option, you are prompted for a problem description.  This

problem description is included in the dump file to help EXABYTE

Technical Support organize dump files when they receive them for

analysis.

While entering this description, you can use the following

editing capabilities:  arrow keys, Backspace, and the Insert

toggle.  Pressing Enter from the last of the four lines or

Ctrl-Enter from any line completes the entry.

After the files are created, the text file displays on the

screen. You can read through the file at your leisure, using the

arrow keys and PgUp and PgDn keys.  When you are done, press

Esc.  You can view the file again later with the View text file

option.

While viewing a text file, you can use the following keys:

S

Search for specific text (case sensitive)

PgUp

Go up one page

PgDn, Enter or space bar

Go down one page

Home or Ctrl-PgUp

Go to top of file

End or Ctrl-PgDn

Go to bottom of file

Up arrow

Go up one line

Down arrow

Go down one line

Esc

Exit from the file viewer and return to the menu

If you don't want to wait for a dump to finish, you can

interrupt the process with a keystroke, then answer Y to the

pop-up prompt asking if you want to discard the dump.

Dump | View text file  (F3)

The View text file option allows you to view a text dump (or

other ASCII file) that was created previously.  You can read

through the file at your leisure, using the cursor keys and PgUp

and PgDn keys.  When you are done, press Esc.

While viewing a text file, you can use the following keys:

S

Search for specific text (case sensitive)

PgUp

Go up one page

PgDn, Enter or space bar

Go down one page

Home or Ctrl-PgUp

Go to top of file

End or Ctrl-PgDn

Go to bottom of file

Up arrow

Go up one line

Down arrow

Go down one line

Esc

Exit from the file viewer and return to the menu

Dump | Binary to text  (F4)

This option allows you to create a text dump file (.TXT) from a

binary dump file (.85), then view the text dump file.  If the

text dump file already exists, you are asked if you want to

overwrite it.  You will then be able to view the text dump with

the arrow keys.  When you are finished, press Esc. You can look

at the text dump file later with the View Text File option.

Dump | Print file  (Alt-P)

To print any plain ASCII text file (like CTSMON.DOC) to LPT1:,

select this option and specify the file name that you want to

print.

Printing occurs in the background while the CTS Monitor program

waits for keyboard input.  If you select this option right after

starting up the CTS Monitor program, there will be a slight

delay before printing starts while other background

initialization tasks are completed.  Only one file can be in the

print queue at a time. If you exit the CTS Monitor program, all

print activity is canceled.

If you want to stop a print job, you can either (1) exit the CTS

Monitor, or (2) select the print option again (Alt-P), then

answer Y when you are asked if you want to cancel the print

job.

EXAMINE

Examine allows you to view items included in the dump files

without taking the time for a complete dump.

eXamine | SCSI cmd/status trace  (F9)

The SCSI trace table consists of five sections.

The first section is a compact, raw dump of the trace table.

Interpretation of this table assumes that you are familiar with

the firmware that generates it.

The second section lists the most recent SCSI commands and

status replies, along with their corresponding host numbers.

The third section contains the last "action" CDB in its

entirety.  If there was no last "action" CDB, TEST UNIT READY

(all zeros) is listed.

The fourth section contains the last CDB to return a Check

Condition status, along with its sense key and fault symptom

code.  If there was no last command to return a Check Condition

status, TEST UNIT READY (all zeros) is displayed, with a fault

symptom code of zero.

The last section contains a more detailed trace of the CTS's

SCSI phases.

eXamine | XRAM display  (Alt-E)

This command allows you to examine any range of control

processor external memory and memory mapped I/O areas with

addresses ranging from 0 to FFFFH.

If the range of addresses you specify is less than or equal to

1024 bytes, then the memory is read only once before it is

displayed, and only the memory specified is read.  This is

significant when reading some of the memory mapped I/O

locations.

If you specify more than 1024 bytes of memory to read, the

memory is read in 1024 byte chunks, and the first screen of data

is displayed as soon as that part of memory is read in.  Memory

continues to be read as you read the data, until it has all been

read in.  If you want to look at a part of the memory that

hasn't been read yet, it will be read first, then the background

reading will continue where it left off.

While looking at the XRAM display, you can use the following

keystrokes:

PgUp

Go up one page

PgDn, Spacebar, or Enter

Go down one page

Home or Ctrl-PgUp

Go to the top of the buffer

End or Ctrl-PgDn

Go to the bottom of the buffer

Up arrow

Go up one line

Down arrow

Go down one line

Right arrow

Go right one byte

Left arrow

Go left one byte

g or G

Go to a hexadecimal address

F1

Help

Esc

Quit viewing external RAM

eXamine | Read SCSI chip register

This function allows you to read the contents of any register of

the SCSI chip.  Register addresses range from 0 to 1F

hexadecimal.

CONTROL

This menu allows you to stop (sTop) and start (Run) the control

processor main program manually.  When the control processor

stops, the CTS becomes inactive on the SCSI bus and sends basic

data about its state (the "standard context") to the screen.

NOTE:  When the control processor runs again, you might have to

power the CTS off and then on again to make it function

normally, depending on what happened while it was inactive.

All of the dump and examine functions automatically stop the

control processor if needed, then (if it was running before)

tell it to run when the dump is done.  The servo functions

automatically stop, but do not restart, the control processor.

The control menu also contains functions to do LEOT and

diagnostic testing and to set and use break points or trigger

points.  These functions typically require the help of an

EXABYTE Technical Support representative.

Control | sTop  (Alt-T)

This function stops the control processor main program (if it

was running) and displays the state of the internal registers

and RAM on the screen.  If you are using EGA 43-line or VGA

50-line mode, 48 bytes of external RAM and 48 bytes of program

memory (the boot PROM copyright message) also display.

When the control processor stops, the small arrow in the upper

right corner of the screen (next to the clock) points down.  On

a color display, this arrow turns red.  On a CGA or VGA display,

the screen border turns red.

Control | Run  (Alt-R)

This function causes the control processor to run again after it

has been stopped either manually or automatically (for example,

by issuing a servo command).

When the control processor is running, the small arrow in the

upper right corner of the screen (next to the clock) points up.

On a color monitor, it turns green.  On a CGA or VGA display,

the border turns green.

NOTE:  When you start the control processor again, you might

have to power the CTS off and then on again to make it function

normally, depending on what happened while it was inactive.

Control | diZplay front/back

This function is active in EXB-8200/EXB-8200sx mode only.

Display was spelled with a Z to allow the same selection key to

be used for this function as on the original EXB-8200 monitor.

This function is only for the EXB-8200 and EXB-8200sx, which

cannot display the "front" (0-127) and "back" (128-255) internal

RAM at the same time.  Selecting this function toggles back and

forth between the two.

NOTE:  Never exit the CTS Monitor program with the "back" RAM

displayed, as this will cause the CTS and Monitor to be out of

sync the next time the monitor program is started up, and the

dump function will not report correct data.

Control | LEOT test  (Alt-V)

To test the way a driver program and the CTS interact at LEOT

without actually writing all the way to LEOT, you can use this

function to move the LEOT point to a spot closer to LBOT.  The

units of measure here are physical 1024-byte blocks on the tape

(not logical blocks).  Once a fake LEOT point is set with this

function, the CTS must be reset to restore normal operation.

Control | Breakpoint set  (Alt-B)

This function sets three break points (addresses in the control

processor program to stop at when they are reached).  After

setting the break points, issue the Goto breakpoint command.

This function is for debugging, and not for normal CTS

operation.  When a breakpoint is reached, a message is sent over

the monitor port to the PC.  Break point selection depends on

the type of troubleshooting you are doing.  This is designed for

use by the firmware authors, but may be useful to end users

trying to solve a specific problem with guidance from an EXABYTE

Technical Support representative.

Control | trIgger set

This function sets three trigger points (addresses in the

control processor program that cause a pulse to be sent out on

monitor pin 4 when they are reached).  After setting the trigger

points, issue the Goto breakpoint command.

Control | Goto breakpoint  (Alt-G)

After setting break points or trigger points, use this function

to run the control processor with these break or trigger points

enabled.

Control | start/stop Diagnostics

To run diagnostics from the serial port that cause the CTS to

read and write with start/stop motions:

1.      Insert a scratch data cartridge into the CTS.

2.      Wait for the CTS to be idle, and ensure that no commands

or reset signals come over the SCSI bus.

3.      Select the Start/stop Diag menu item.

4.      Press Y to confirm that the CTS is loaded and idle, then

wait for one of the following responses:

Y       Yes, send diagnostics (from monitor) PASSED!

U       Unsuccessful (FAILED) diagnostic test!

B       Can't start operation until self test and

autoload are

done!

E       Can't write on tape -- tape is write protected!

Control | strEaming diagnostics

This is the same as the Control | start/stop Diagnostics

selection, except that the data transfer rate tested is higher

and the test completes faster due to fewer stop/start motions.

Control | Fake cleaning needed

This function causes the CTS to simulate the internal timer

condition where 30 hours of tape motion have passed and a

cleaning tape needs to be inserted. To clear this condition,

insert a cleaning tape or reset the CTS.

SERVO

The servo commands allow you to give basic motion commands to

the CTS for emergency tape extraction, troubleshooting hardware,

and testing.

NOTE:  After using these functions, you must perform a power-on

reset to continue normal operation of the CTS.

Servo | Reset servo  (Alt-F1)

This command tells the servo processor to reset itself. For this

command to work, the servo processor must be able to receive and

process the command. This command does not reset the control

processor, and will likely confuse the control processor if you

attempt to continue normal operation after the servo reset.

Servo | Load tape  (Alt-F2)

This command causes the CTS to load tape into the tape path from

a cartridge that has been inserted.

Servo | Unload tape  (Alt-F3)

This command causes the CTS to unload the tape without rewinding

it first.  This is useful if you suspect that your CTS is

damaged and might cause tape damage during a rewind operation.

Under normal operating conditions, unload operations at the

beginning of tape cause less tape damage to the data area of the

tape.

Servo | Find LBOT  (Alt-F4)

When the tape is at PBOT, this command causes the CTS to move

the tape forward to find the logical beginning of tape (LBOT).

Servo | Go track  (Alt-F5)

This command causes the servo processor to go into CTS native

tape format tracking (read) mode and move the tape forward at

normal speed.

Servo | Stop tape  (Alt-F6)

This command stops the tape motion.

Servo | rewind to PBOT  (Alt-F7)

This command rewinds the tape to physical beginning of tape

(PBOT).

Servo | rewind uNload  (Alt-F8)

This command rewinds the tape to physical beginning of tape

(PBOT), then unloads and ejects the data cartridge.

QUIT  (Alt-Q)

This command is the same as System | eXit (Alt-X).  Both menu

items terminate the CTS Monitor program and return you to DOS.

ERROR MESSAGES

Program error messages are described below.

File Errors

The following errors relate to reading and writing disk files.

Error writing to output file (disk full?).

Unable to close [file name] ?!

The program was unable to write to a disk file for some reason.

The most common reason is that the disk was full. To correct a

disk full error, shell out to DOS with ALT-O, delete any

unneeded files, type EXIT to return to the monitor, then retry

whatever you were doing.  If the disk that filled up was a

floppy disk, replace it.

[file name] is not a CTS binary dump file.

Binary file format error!

Unrecognized dump file format.  Get an upgrade to CTSMON.EXE.

Unrecognized servo data format.  File corrupted or CTSMON.EXE

out of date. Command buffer size too large.

Unable to open [file name]

Unable to open [file name] to print.

File name may have been incorrectly entered or the file may not

exist.

Error closing [file name]

Error flushing [file name] to disk.

Disk may be full, removed, write protected, or there may have

been a network failure.

Bad servo data format.

File corrupted or CTSMON.EXE out of date.

These messages may appear while trying to translate a binary

dump file (extension of .85) to an ASCII text file if the dump

file was damaged or the input file really wasn't a binary dump

file.

Unable to reopen [file name]

File wouldn't open, even though it was here a second ago . . .

not likely unless the file was on a network or you are running

multitasking system software.

Serial Communications Errors

The following errors pertain to failure to communicate on the

serial port.

Trying to stop control processor again (number)...

Whenever the CTS Monitor program commands the main control

program in the CTS to stop running and go into monitor mode,

certain basic information is supposed to be sent back.  If it is

not received, then several more attempts are made.  The most

obvious cause of this problem is a possible bad connection

between the PC and the CTS or that there is no power to the CTS.

Other possible causes are that the CTS firmware and the CTS

Monitor program are out of synchronization with each other.  In

this case, it may be necessary to restart the CTS Monitor

program and/or do a power on reset of the CTS by cycling the

poweroff, then on again.

Overrun error!

Parity error!

Framing error!

Break detected!

Receive error

Receive queue overrun error!

Break detected

Framing error

Parity error

Overrun error

These messages refer to the things that can go wrong with the

serial communications between the PC and the CTS.

XRAM receive framing error!  [number]

The CTS and CTS Monitor program are out of synchronization.

While waiting for a framing character, CTS Monitor received a

different character from the CTS.  Restarting the CTS Monitor

program and/or resetting the CTS may be required.

Requested COM port not found.

Connected to COMn:

The program couldn't switch to the serial port number you

requested.

COMn not found!

The serial port noted could not be located a program start-up.

If you have COM2: but not COM1:, use /2 on the CTSMON command

line.  DOS ERRORLEVEL = 5.

Dropout detected reading external RAM!

The CTS and monitor program are out of synchronization.  A

framing character was expected from the CTS, but nothing was

received. Monitor program restart and/or resetting the CTS may

be required.

Code Loading Errors

The following errors indicate a failure to load firmware, either

due to a bad file format, wrong version of the CTS Monitor

program, a hardware failure in the CTS, or a serial port

failure.

Invalid code file header or wrong version of monitor.

.BIN file format not supported by this monitor.

File too short!

File validation failure

All of the above error messages indicate that the code load

(.BIN) file that you attempted to load into the CTS was not

valid.  Either it was damaged, or it never was a valid code load

file, or there has been a .BIN file format change and this

version of the CTS Monitor program does not support it.  No

change to the CTS firmware is made if the .BIN file cannot be

verified to be OK first.

Hardware error!  Unable to complete erasure!

Servo program error!  Unable to complete erasure!

TIME OUT ERROR!  Unable to complete erasure!

0x01: Erase failed.

Erase failed.

These error messages indicate that an FEPROM or EEPROM erasure

did not complete successfully.  Your CTS might be nonfunctional.

If retries do not correct the problem, you probably have a

hardware problem.  FEPROMS and EEPROMS both have a finite number

of erase/program cycles that they can endure, and one of these

parts may need to be replaced.

Loading address [number] Dropout [number]

While loading code or data into the CTS, the CTS failed to

respond to the last packet sent, indicating that it probably did

not get enough bytes in. If sending more bytes to the CTS does

not unlock it, it will be necessary to reset the CTS (power off

then on or press the tiny white button near the power connector

on the CTS) and restart the CTS Monitor program to retry the

load operation.

Transfer failed -- giving up.

Giving up.

Gave up after x:xx.

Too many errors were detected in the firmware download.  Reset

the CTS by cycling its power off then on, then try the firmware

load again.

N: Transfer failed -- giving up.  Delay was [number]

The CTS got too many bad packets of data in a row.

A: [number] retries.

The CTS called the last data packet bad, and requested a retry.

0x02: Program operation failed.

Error sending end record.

Program operation failed.

Unable to program FEPROM or EEPROM -- possible hardware

failure.

0x04: FEPROM checksum error.

Firmware is damaged -- reload it.

Configuration upload failure!

Configuration upload integrity check failure!

Something went wrong in trying to get the current configuration

option information from the CTS.  No changes will be made unless

the existing options can be uploaded correctly, since part of

the data must be downloaded again.

Option change failure.

EEPROM options were not correctly reprogrammed.  It may be

necessary to reload your default options from your EEPROM

configuration file (85E-xxxxx.BIN), then try to change your

configuration options from the menu again.

Memory Shortage Errors

The following errors indicate a lack of memory available to run

the CTS Monitor program.

ERROR:  Out of memory.

Program halted due to lack of memory (in the main 640K).  This

is possible if you "shell" out to the CTS Monitor from another

program, if you have too many TSR programs resident, or if you

are running some multitasking system software that didn't

allocate at least 400K of RAM for the CTS Monitor. DOS

ERRORLEVEL = 3.

ERROR:  Windowing function failure.

The program couldn't open a new window on the screen, possibly

due to the lack of memory. DOS ERRORLEVEL = 4.

Main menu definition or memory allocation error!

Give the program more room in your PC RAM.  DOS ERRORLEVEL = 7.

Information Messages

The following messages do not necessarily indicate error

conditions with the CTS Monitor program.

Action terminated by keystroke.

Some of the error recovery loops allow the current action to be

terminated by hitting any key.  This message means that whatever

you were doing has terminated before finishing.  It may be

necessary to use several keystrokes or a Ctrl-Break to get out

of some loops in a reasonable period of time. Interrupting some

operations, like loading firmware, is not recommended in normal

circumstances, since this could leave the CTS without any valid

firmware and nonfunctional.

Clearing servo command in progress.

Clearing pending servo interrupt.

Checking for servo interrupt...

These messages may appear while the CTS Monitor is communicating

with the servo processor in the CTS.  They aren't really error

messages, they just indicate that the servo processor may be

busy with something else, like running through a reset

sequence.

Break!

Program halted because someone pressed Ctrl-Break.  DOS

ERRORLEVEL = 2.

[file name] is in the print queue.

Printer is timed out.

Printer I/O error!

Printer is off line.

Printer is out of paper.

Printer is busy.

Please make sure printer power is on and

printer is on line.

The preceding messages give the status of the background print

job.  One or more of them appear when you select the "Print

file" menu option while a print job is active.

!: Lost sync uploading bulk data

@: Breakpoint reached

C: FEPROM CRC of firmware failure

H: Programmed Hang -- see program counter

I: Primary interrupt from spare device?

O: Boot prom firmware is OBSOLETE -- get new boot prom or older

.BIN file P: Power on reset, FULL testing in progress

Q: CTS control processor OS task queue FULL

R: Tape drive reset

S: Stack over/underflow error

T: Firmware error:  control processor timeout without vector V:

Control processor OS error: Void link return

W: Watchdog timer time out

X: Monitor executing in monitor only mode

Z: unit self test done

b: Firmware hit a blue moon window!

h: Handshake went awry?

m: Mail linkage got executed

r: Handshake failure?

t: Hardware error -- DPORT read transfer error

0xxx (x) received from drive!

0xxx received from drive!

The above indicate unsolicited characters sent from the CTS to

the PC over the serial port.  These messages convey information

concerning the status of the CTS.

PROBLEM REPORTING

If you find a problem with the CTS Monitor software, or have a

suggestion for an improvement, please contact Exabyte Technical

Support at:

CTSMON Technical Support

EXABYTE CORPORATION

1685 38TH ST

BOULDER CO 80301-2630

BBS 303-447-7100

Voice 303-442-4333 or 800-442-4392

Explain the symptoms of the problem and write down the exact

sequence of keystrokes and/or other significant information

needed to reproduce the problem.  Also, please indicate what

hardware you are running the program on (type of CPU, graphics

card, monitor, and so on.) and the contents of your CONFIG.SYS

and AUTOEXEC.BAT.

CTS Monitor User's Guide