HARDCOPY PRINTOUT FOR THE NPC-764 LOGIC ANALYSIS SYSTEM

EQUIPMENT NEEDED: -EPSON MX-80 PRINTER

-EPSON 2K RS-232 BUFFER CARD (OR EQUIVALENT)

-EIA RS-232C 25 CONDUCTOR CABLE

IN THE RS-232C SECTION OF THE I/O CONFIGURATION MENU, SELECT 4800 BAUD

ONLY. NO OTHER PARAMETERS NEED CHANGING.

CONNECT THE PRINTER TO NPC-764 WITH THE CABLE. NOTE: PINS 6 AND 20 MUST

BE REVERSED AT ONE END TO FUNCTION PROPERLY.           IF A MENU OR A

TIMING DIAGRAM IS BEING DISPLAYED, CNTRL P WILL DUMP THE SCREEN.    IN

THE STATE DATA DISPLAY MODE, CNTRL P WILL DUMP FROM THE PRESENT TOP OF

THE SCREEN TO THE END OF THE MEMORY.          IN THE DIFFERENCE MODE,

DIFFERENCES NORMALLY HIGHLIGHTED WILL BE TAGGED WITH AN ASTERICK (*).

TO STOP, DEPRESSING ANY KEY WILL ABORT THE PROGRAM. THE 2K BUFFER

WILL, HOWEVER, EMPTY.

NOTE: WHEN USING DISASSEMBLY LOAD THE LADISA.COM FILE

FIRST AND THEN LOAD THE MX-80 PRINTER DRIVER PROGRAM  (IN THAT ORDER).

##########################################################