DOCUMENT NUMBER :  10008                   DATE: 07/18/94

SUBJECT:	Cirrus Logic CL-GD542X Data Sheet

| Features | CL-GD5420 |  | CL-GD5422 |  |  | CL-GD5424 | CL-GD5426 |  | CL-GD5428 |  | CL-GD5429 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Performance |  |  |  |  |  |  |  |  |  |  |  |  |
| VESAâVL-Busä and direct ‘386 or ‘486 CPU interface |  |  |  |  |  | ü | ü |  | ü |  | ü |  |
| BITBLT Engine |  |  |  |  |  |  | ü |  | Enhanced |  | Memory-Mapped I/O |  |
| Zero-wait-state operation | ü |  | ü |  |  | ü | ü |  | ü |  | ü |  |
| Maximum display memory | 1 Mbytes |  | 1 Mbytes |  |  | 1 Mbytes | 2 Mbytes |  | 2 Mbytes |  | 2 Mbytes |  |
| Display memory interface | 16-bit |  | 32-bit |  |  | 32-bit | 32-bit |  | 32-bit |  | 32-bit |  |
| Hardware Courser (in pixels) | up to 32 x 32 |  | up to 64 x 64 |  |  | up to 64 x 64 | up to 64 x 64 |  | up to 64 x 64 |  | up to 64 x 64 |  |
| Maximum dot clock frequency | 75 MHz |  | 80 MHz |  |  | 80 MHz | 80 MHz |  | 80 MHz |  | 86 MHz |  |
| High Integration |  |  |  |  |  |  |  |  |  |  |  |  |
| Integrated palette DAC and dual-frequency synthesizer | ü |  | ü |  |  | ü | ü |  | ü |  | ü |  |
| Motherboard VGA solution with only two ICs | ü |  | ü |  |  | ü | ü |  | ü |  | ü |  |
| Built-in port for VESAâ passthrough feature connector | ü |  | ü |  |  | ü | ü |  | ü |  | ü |  |
| Built-in ISA (up to 12.5 MHz), Microchannel â bus support | ü |  | ü |  |  | ü | ü |  | ü |  | ü |  |
| Flexibility |  |  |  |  |  |  |  |  |  |  |  |  |
| Support for x4-, x8- and x16-bit wide memory | ü |  | ü |  |  | ü | ü |  | ü |  | ü |  |
| 8- or 16-bit host bus I/O and memory interface | ü |  | ü |  |  | ü | ü |  | ü |  | ü |  |
| 8-bit gray and 3-3-2 RGB DAC modes |  |  |  |  |  |  |  |  | ü |  | ü |  |
| General |  |  |  |  |  |  |  |  |  |  |  |  |
| 100% hardware- and BIOS-compatible with IBMâ VGA display standards | ü |  | ü |  |  | ü | ü |  | ü |  | ü |  |
| ‘Green-PC’ compliant | ü |  | ü |  |  | ü | ü |  | ü |  | ü |  |
| 132-column text mode support | ü |  | ü |  |  | ü | ü |  | ü |  | ü |  |
| 46E8 or 3C3 sleep mechanism |  |  | ü |  |  | ü | ü |  | ü |  | ü |  |
| Video overlay and color key support |  |  | ü |  |  | ü | ü |  | ü |  | ü |  |
| VESAâ VAFC Baseline support (for video overlay) |  |  |  |  |  |  |  |  |  |  | ü |  |
| Support for C:-PX2080 MediaDACä |  |  |  |  |  |  |  |  | ü |  | ü |  |
| Low-power CMOS, 160-pin PQFP package | ü |  | ü |  |  | ü | ü |  | ü |  | ü |  |
| Screen Resolution and Colors |  |  |  |  |  |  |  |  |  |  |  |  |
| 640 x 480 | up to 256 |  | up to 16.5M |  |  | up to 16.5M | up to 16.5M |  | up to 16.5M |  | up to 16.5M |  |
| 800 x 600 | up to 256 |  | up to 64K |  |  | up to 64K | up to 64K |  | up to 64K |  | up to 64K |  |
| 1024 x 768 (interlaced) | up to 256 |  | up to 256 |  |  | up to 256 | up to 256 |  | up to 64K |  | up to 64K |  |
| 1024 x 768 (non-interlaced) | up to 256 |  | up to 256 |  |  | up to 256 | up to 256 |  | up to 256 |  | up to 256 |  |
| 1280 X 1024 (interlaced) |  |  | up to 16 |  |  | up to 16 | up to 256 |  | up to 256 |  | up to 256 |  |

# IBMâ Standard VGA Video Modes

| Mode No. | VESAâNo. | No. of Colors | Char. x Row | Char. Cell | Screen Format | Display Mode | Pixel Freq. MHz | Horiz. Freq. kHz | Vert. Freq. Hz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0, 1 | 0, 1 | 16/256K | 40 x 25 | 9 x 16 | 360 x 400 | Text | 14 | 31.5 | 70 |
| 2, 3 | 2, 3 | 16/256K | 80 x 25 | 9 x 16 | 720 x 400 | Text | 28 | 31.5 | 70 |
| 4, 5 | 4, 5 | 4/256K | 40 x 25 | 8 x 8 | 320 x 200 | Graphics | 12.5 | 31.5 | 70 |
| 6 | 6 | 2/256K | 80 x 25 | 8 x 8 | 640 x 200 | Graphics | 25 | 31.5 | 70 |
| 7 | 7 | Monochrome | 80 x 25 | 9 x 16 | 720 x 400 | Text | 28 | 31.5 | 70 |
| D | D | 16/256k | 40 x 25 | 8 x 8 | 320 x 200 | Graphics | 12.5 | 31.5 | 70 |
| E | E | 16/256k | 80 x 25 | 8 x 14 | 640 x 200 | Graphics | 25 | 31.5 | 70 |
| F | F | Monochrome | 80 x 25 | 8 x 14 | 640 x 350 | Graphics | 25 | 31.5 | 70 |
| 10 | 10 | 16/256k | 80 x 25 | 8 x 14 | 640 x 350 | Graphics | 25 | 31.5 | 70 |
| 11 | 11 | 2/256k | 80 x 25 | 8 x 16 | 640 x 480 | Graphics | 25 | 31.5 | 60 |
| 12 | 12 | 16/256k | 30 x 30 | 8 x 16 | 640 x 480 | Graphics | 25 | 31.5 | 60 |
| 12+ | 12+ | 16/256k | 30 x 30 | 8 x 16 | 640 x 480 | Graphics | 31.5 | 37.9 | 72 |
| 13 | 13 | 256/256k | 40 x 25 | 8 x 8 | 320 x 200 | Graphics | 12.5 | 31.5 | 70 |

# Cirrus Logic Extended Video Modes

| Mode No. | VESAâNo. | No. of Colors | Char. x Row | Char. Cell | Screen Format | Display Mode | Pixel Freq. MHz | Horiz. Freq. kHz | Vert. Freq. Hz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | - | 16/256K | 132 X 25 | 8 X 16 | 1056 X 400 | Text | 41.5 | 31.5 | 70 |
| 54 | 10A | 16/256K | 132 X 43 | 8 X 8 | 1056 X 350 | Text | 41.5 | 31.5 | 70 |
| 55 | 109 | 16/256K | 132 X 25 | 8 X 14 | 1056 X 350 | Text | 41.5 | 31.5 | 70 |
| 58,6A | 102 | 16/256K | 100 X 37 | 8 X 16 | 800 X 600 | Graphics | 36 | 35.2 | 56 |
| 58,6A | 102 | 16/256K | 100 X 37 | 8 X 16 | 800 X 600 | Graphics | 40 | 37.8 | 60 |
| 58,6A | 102 | 16/256K | 100 X 37 | 8 X 16 | 800 X 600 | Graphics | 50 | 48.1 | 72 |
| 5C | 103 | 256/256K | 100 X 37 | 8 X 16 | 800 X 600 | Graphics | 36 | 35.2 | 56 |
| 5C | 103 | 256/256K | 100 X 37 | 8 X 16 | 800 X 600 | Graphics | 40 | 37.9 | 60 |
| 5C | 103 | 256/256K | 100 X 37 | 8 X 16 | 800 X 600 | Graphics | 50 | 48.1 | 72 |
| 5D† | 104 | 16/256K | 128 X 48 | 8 X 16 | 1024 X 768 | Graphics | 44.9 | 35.5 | 87† |
| 5D | 104 | 16/256K | 128 X 48 | 8 X 16 | 1024 X 768 | Graphics | 65 | 48.3 | 60 |
| 5D | 104 | 16/256K | 128 X 48 | 8 X 16 | 1024 X 768 | Graphics | 75 | 56 | 70 |
| 5D | 104 | 16/256K | 128 X 48 | 8 X 16 | 1024 X 768 | Graphics | 77 | 58 | 72 |
| 5F | 101 | 256/256K | 80 X 30 | 8 X 16 | 640 X 480 | Graphics | 25 | 31.5 | 60 |
| 5F | 101 | 256/256K | 80 X 30 | 8 X 16 | 640 X 480 | Graphics | 31.5 | 37.9 | 72 |
| 60† | 105 | 256/256K | 128 X 48 | 8 X 16 | 1024 X 768 | Graphics | 44.9 | 35.5 | 87† |
| 60 | 105 | 256/256K | 128 X 48 | 8 X 16 | 1024 X 768 | Graphics | 65 | 48.3 | 60 |
| 60 | 105 | 256/256K | 128 X 48 | 8 X 16 | 1024 X 768 | Graphics | 75 | 56 | 70 |
| 60 | 105 | 256/256K | 128 X 48 | 8 X 16 | 1024 X 768 | Graphics | 77 | 58 | 72 |
| 64 | 111 | 64K | - | - | 640 X 480 | Graphics | 25 | 31.5 | 60 |
| 64 | 111 | 64K | - | - | 640 X 480 | Graphics | 31.5 | 37.9 | 72 |
| 65 | 114 | 64K | - | - | 800 X 600 | Graphics | 36 | 35.2 | 56 |
| 65 | 114 | 64K | - | - | 800 X 600 | Graphics | 40 | 37.8 | 60 |
| 66 | 110 | 32K‡ | - | - | 640 X 480 | Graphics | 25 | 31.5 | 60 |
| 66 | 110 | 32K‡ | - | - | 640 X 480 | Graphics | 31.5 | 37.9 | 72 |
| 67 | 113 | 32K‡ | - | - | 800 X 600 | Graphics | 36 | 31.5 | 56 |
| 6C† | 106 | 16/256K | 160 X 64 | 8 X 16 | 1280 X 768 | Graphics | 75 | 48 | 87† |
| 6D† | - | 256/256K | 160 X 64 | 8 X 16 | 1280 X 768 | Graphics | 75 | 48 | 87† |
| 71 | 112 | 16M | 80 X 30 | 8 X 16 | 640 X 480 | Graphics | 25 | 31.5 | 60 |
| 74 | - | 64K | - | - | 1024 X 768 | Graphics | 44.9 | 35.5 | 87† |

Notes:

1) ‘‡ ‘ indicates 32K Direct-Color/256-color Mixed mode

2)  ‘†‘ indicates interlaced mode

Summary of Screen Resolutions

| Resolutions | CL-GD5420 | CL-GD5422 | CL-GD5424 | CL-GD5426,28,29 |
| --- | --- | --- | --- | --- |
| 640 X 480 X 256 color, 8 bits/pixel | ü | ü | ü | ü |
| 640 X 480 X 32K color, 16 bits/pixel |  | ü | ü | ü |
| 640 X 480 X 64K color, 16 bits/pixel |  | ü | ü | ü |
| 640 X 480 X 16M, 24 bits/pixel |  | ü | ü | ü |
| 800 X 600 X 16 color, 4 bit planes | ü | ü | ü | ü |
| 800 X 600 X 256 color, 8 bits/pixel | ü | ü | ü | ü |
| 800 X 600 X 32K, 16 bits/pixel |  | ü | ü | ü |
| 800 X 600 X 64K, 16 bits/pixel |  | ü | ü | ü |
| 1024 X 768 X 16 color, 4 bit planes (interlaced and non-interlaced) | ü | ü | ü | ü |
| 1024 X 768 X 256 color, 8 bits/pixel (interlaced and non-interlaced) | ü | ü | ü | ü |
| 1024 X 768 X 32K color, 16 bits/pixel (interlaced) |  |  |  | ü |
| 1024 X 768 X 64K, 16 bits/pixel (interlaced) |  |  |  | ü |
| 1280 X 1024 X 16 color, 4 bit planes (interlaced) |  | ü | ü | ü |
| 1280 X 768 X 256 color, 8 bits/pixel (interlaced) |  |  |  | ü |