This is version 1.14 of the RVGNDIS driver, dated 6-Jun-96.     It is for

use by 100VG/AnyLAN Ethernet cards running NDIS Version 2 systems.

Beginning with version 1.14, "shared-memory" support has been deleted for

all cards  (it was confusing and did NOT give a performance improvement).

Version 1.14 of the driver will therefore ignore  any SHAREDMEMORY or MEM

keywords in the user's PROTOCOL.INI file and will initialize all cards to

run in "I-O mode".

PCI cards may share an IRQ level with other devices, if required.    Note

that this should be done only on "loaded" systems where there is no other

way of configuring the system,  since sharing an IRQ level among multiple

devices is inefficient and costs some performance.   Note also that older

OS/2 versions will REFUSE to allow a shared interrupt for PCI cards (such

versions assume only an EISA or MicroChannel card may share interrupts!).

In this case, the driver will attempt to set up a PCI card's interrupt as

"unshared" and,  if this is successful,  the driver displays a message to

that effect and runs the PCI card anyway.     On such older OS/2 systems,

the user must ensure that each 100VG PCI card is actually assigned to its

own IRQ level, or the system will crash!!

A sample PROTOCOL.INI file section for 100VG cards is as follows:

[RVGNDIS]

DRIVERNAME = R100VG$

IOADDRESS = 0xNNNN

IRQ = NN

NETADDRESS = 0xNNNNNNNNNNNN

NOTE:  The IOADDRESS parameter is REQUIRED for most CPUs; if unspecified,

a default ISA address of 0x300 will be used!

The R100VG$ driver name becomes R100VG$2, R100VG$3, or R100VG$4 when more

than one 100VG card is to run in a single CPU.   The IOADDRESS must match

that of the desired 100VG card;  if unspecified, a default ISA address of

300 will be assumed.   The IRQ and NETADDRESS should be specified only if

the default values programmed into the card's EEPROM  need to be changed.

Note that a user-specified IRQ value will be ignored for PCI cards, since

such cards must run with the IRQ which was assigned them by the PCI BIOS.

Revision History --

Version 1.10:  Added 9001 PCI card support.

Version 1.11:  Fixed 100VG configuration-register usage during resets.

Version 1.12:  Added adapter re-initializations in case of input errors.

Version 1.13:  Fixed certain OS/2 initialization errors.