This is version 1.14 of the RVGNDIS driver, dated 6-Jun-96.     It is for

use by 100VG/AnyLAN Ethernet cards running NDIS Version 2 systems.

Beginning with version 1.14, "shared-memory" support has been deleted for

all cards  (it was confusing and did NOT give a performance improvement).

Version 1.14 of the driver will therefore ignore  any SHAREDMEMORY or MEM

keywords in the user's PROTOCOL.INI file and will initialize all cards to

run in "I-O mode".

PCI cards may share an IRQ level with other devices, if required.    Note

that this should be done only on "loaded" systems where there is no other

way of configuring the system,  since sharing an IRQ level among multiple

devices is inefficient and can cost some performance.

A sample PROTOCOL.INI file section for 100VG cards is as follows:

[RVGNDIS]

DRIVERNAME = R100VG$

IOADDRESS = 0xNNNN

IRQ = NN

NETADDRESS = 0xNNNNNNNNNNNN

BUFFERSIZE = N

NOTE:  The IOADDRESS parameter is REQUIRED for most CPUs; if unspecified,

a default ISA address of 0x300 will be used!

The R100VG$ driver name becomes R100VG$2, R100VG$3, or R100VG$4 when more

than one 100VG card is to run in a single CPU.   The IOADDRESS must match

that of the desired 100VG card;  if unspecified, a default ISA address of

300 will be assumed.   The IRQ and NETADDRESS should be specified only if

the default values programmed into the card's EEPROM  need to be changed.

Note that a user-specified IRQ value will be ignored for PCI cards, since

such cards must run with the IRQ which was assigned them by the PCI BIOS.

The BUFFERSIZE parameter is only for the DOS driver.   BUFFERSIZE=2  will

reserve 60K for input buffers, and  BUFFERSIZE=1  reserves 34K for input.

If unspecified, 34K will be used.    This parameter must NOT be specified

when running Windows for Workgroups, since that system loads a DOS driver

into a fixed-size region and will CRASH if a driver requests more memory.

Revision History --

Version 1.10:  Added 7001 PCI card support.

Version 1.11:  Fixed 100VG configuration-register usage during resets.

Version 1.12:  Added adapter re-initializations in case of input errors.

Version 1.13:  Fixed certain OS/2 initialization errors.