This is version 1.24 of the RVGODI driver, dated 6-Apr-96.    It is to be

used by CPL    100VG/AnyLAN Ethernet cards running in DOS workstations.

Beginning with version 1.24,  the driver will run the various 100VG cards

in accordance with the following rules --

1)	ISA cards will always be run in "I-O" mode.

2)  5001 EISA cards will be run in "bus-master" mode if their EEPROMs

are so configured; if not, EISA cards will use "I-O" mode.

3)	Standard 7001 PCI cards will always be run in "I-O" mode.

4)  "Bus-master" 7001 PCI cards  will be run  in "bus-master" mode by

default.     If the IOMODE keyword is specified, these cards

will then be run in "I-O" mode.

Note that the old "shared-memory" mode  has been removed from the version

1.24 driver (it did not improve ISA/EISA performance and in fact DEGRADED

the performance of PCI cards!).    Note also that, for EISA and for "bus-

master" PCI cards, the use of "bus-master" mode is highly recommended due

to its much-higher performance  ("I-O" mode should only be used for diag-

nostics and testing with these cards).

Otherwise, the V1.24 driver operates as before  and can be used without a

NET.CFG file on systems having only a single 100VG card.    In this case,

the driver will test the system for (a) the first PCI card, (b) the first

EISA card, or (d) an ISA card at address 300.      The driver initializes

and runs whichever card it finds first.     For multi-card systems, or on

systems where different parameters apply, the user can specify a  NET.CFG

file, a sample of which is as follows:

LINK DRIVER RVGODI

PORT=nnnn

INT=nn

IOMODE

...

...

Note that the  IOMODE  parameter is only for "bus-master" PCI cards which

are to be forced into "I-O" mode (it is ignored by other cards);  and the

INT=  parameter is an override of the ISA or EISA interrupt-request level

set within the card's EEPROM, to be used only as needed (PCI cards always

use the IRQ level set by the PCI BIOS).   For more details about  NET.CFG

file parameters, refer to the Novell documentation for ODI workstations.

History of Changes:

Version 1.16 -- Added support for 7001 PCI cards.

Version 1.17 -- Send-packet logic changes to pass Novell test suites.

Version 1.18 -- Interrupt-logic changes to pass Novell test suites.

Version 1.19 -- Corrected PCI "shared-interrupt" problem.

Version 1.20 -- Added "device-enable" logic for PCI BIOS V2.1 and above.

Version 1.21 -- Added "I-O" mode support for "bus-master" 7001 PCI cards.

Version 1.22 -- Corrected 10-MB problems with "bus-master PCI cards.

Version 1.23 -- Added full "bus-master" support for EISA and "bus-master"

PCI cards.

Version 1.24 -- Deleted "shared-memory" logic for all cards.    Corrected

re-init logic and "bus-master" PCI card latency timings.