How to setup Unix network node so that it serves as a tftp

boot host for PS and PSII firmware.

If system does not already have one, mkir /tftpboot.

Move portserver firmware file to /tftpboot. Make sure that it

has read-write permissions.

Edit /etc/inetd.conf or /etc/inet/inetd.conf (depending

on the OS), remove comment (if it is commented out) from the

tftp	dgram	udp	..... etc. line. Then restart the

inetd process; kill -1 "inetd_process_number".

For AIX only, type smit lsinetdconf, then do startup tftp.

To verify, type tftp "host_tftp_system_name"

tftp> get "tftp_file_name"

tftp> recieved file --- tftp is running.

If it comes back with an error message or the tftp file is

now size 0, tftp is still not running.

On the PS or PSII; set config boothost=<tftpboot_host_ip_address>

bootfile=<tftp_boot_file>. Additionally for PSII add tftpboot=yes

or smart.

Reset Portserver.