# Chapter 3 - Telephony Services

This chapter describes the telephony services available to OS/2 and Microsoft Windows 3.1 application developers for the purpose of developing Mwave compatible telephony based applications.

## Mwave Telephony Architecture

The Mwave system hardware has the ability to play and record data to and from a telephone line, an external microphone/speaker, a telephone handset (just the ear piece and microphone), or a telephone deskset (standard analog telephone). For the purposes of this document, no distinction is made between a telephone handset and a telephone deskset. The term "handset" refers to either one. The data obtained from these devices can be in a variety of formats (voice data, fax data, etc.) as supported by corresponding DSP code tasks. Along with the ability to record and play telephony media, the system (with the required software tasks) has the ability to decode touch-tone type key presses coming either from the telephone line or handset.

Building a complete application from a library of various DSP functionality would be tedious at best. For this reason, the DSP tasks have been grouped into categories of applications, and have been integrated with software device drivers. The types of telephone formats addressed by the current device drivers include voice and fax carrier data transmission.  In future software releases, data transmission may be integrated with voice, e.g. voice and data.

The following figure shows the Mwave telephony architecture:

MWAVE-RESIDENT

HOST-RESIDENT

Mwave O/S

AT Tasks

Figure 3-1:  Mwave Telephony Architecture

Misc. Telephony Tasks

TAM Tasks

Mwave Host  O/S Level

DRIVER LEVEL

APPLICATION LEVEL

Modem Application

Discriminator Driver

Mwave Windows Manager

TAM Application

Fax Application

Mwave VxD

Fax Device Driver

Comm Driver

TPL Device Driver

TPS Device Driver

Fax Tasks

The following standard MCI drivers, provided in the OS/2 MMPM and Microsoft Windows 3.1, are used for processing high-level MCI calls to Mwave telephony hardware:

| Device Type | Driver Filename | Description |
| --- | --- | --- |
| fax | mcifax.drv fax.dll (OS/2) | An MCI device driver for sending and receiving faxes |
| tps | mcimsg.drv tps.dll (OS/2) | An MCI device driver for local message record and playback |
| tpl | mciphone.drv tpl.dll (OS/2) | An MCI device driver for message record and play through the phone line only |
| Table 3-1: Telephony MCI Driver Summary |  |  |

The Telephony Device Drivers

Two types of telephony drivers are supplied with the Mwave subsystem. These include fax send and receive (FAX) and telephone answering machine (TAM). They are designed to comply with standard MCI type command protocols, and are implemented as MCI extensions.

### Common Telephone Interface

Because both drivers make use of the telephone device, potential conflicts arise when multiple applications are active at once. Although the telephone can be physically used by only one type of driver at a time, the Mwave telephony drivers were designed with the ability to share control of the telephone device, and in essence, to virtualize the telephone line. This virtualization process has been integrated into the MCI telephone interface used by both drivers, and is performed transparently to the application.

Using a common telephone interface has some significant advantages. The common interface presents a standardized view of the telephone device to the application programmer. This allows the programmer to create a personal library of telephony functions, and use them in a variety of applications. Migrating from a FAX device to a TAM device does not require any retraining on programming the telephone.

Most importantly in today's multi-tasking environment, having a common telephone interface allows for transparent implementation of a virtual telephone device. Although multiple applications cannot use the telephone simultaneously, they can constantly monitor for incoming calls. The job of sharing the telephone device, discriminating between calls, and signaling the corresponding waiting application is performed transparently by the device driver. This allows the application programmer to treat the telephone device as a sharable resource, and does not require inter-communication between separate FAX, TAM, and Modem applications. An application can wait for a call, and know that it will gain control of the telephone when the call arrives. An application that owns the telephone knows that it can complete the call without fear that another application will try to "steal" the telephone line. The common telephone interface greatly simplifies any environment where the telephone line is shared.

Multiple applications can monitor for an incoming telephone call at any time.  However, only one application may monitor for any one particular type of call. For example, two applications can not be simultaneously waiting for a voice call, but one application can wait for a voice call, and another application can wait for a fax or modem call. Only one application can own the telephone line at any given time. Ownership of the telephone is the ability to actually use the telephone line (make a call).

An application takes ownership of the phone line in three ways:

·	The application receives an incoming phone call.

·	The application executes a command to take the phone off-hook.

·	The application executes a dial command.

Once an application takes ownership of the phone line, other applications can continue to wait for incoming telephone calls, or wait for the phone line to free up, but these other applications are not allowed to use the telephone device. If an application is just waiting for a call, active use of the telephone by other applications is transparent to the waiting application. Any application can still execute MCI commands to control the driver environment, even when the telephone is in use by another application. The telephone line is owned by an application until it places the telephone on-hook, or until the device driver detects that the call has been completed.

### MCI Event Handler

One difficulty in using a telephone device is the random nature of telephone events. At any one time, one or more applications might be waiting for a call, determining a data transmission baud rate, checking to see if the handset is on-hook, and looking for touch tone key presses from either the handset or the telephone line. Obviously, it is impractical to constantly poll for these types of events, especially in a non-real-time environment such as Microsoft Windows. Ideally, an application would be notified (via messages) when any of these randomized real-time events occur. Under MCI for OS/2 and Microsoft Windows,  an application notification message, MM_MCINOTIFY, is used to notify an application when a function call has been completed, but unfortunately, no mechanism exists to signal an application when a defined event occurs.

To handle the need for an on-demand messaging system, the MCI drivers for Fax and TAM include a message posting system, which when combined with an application supplied message handler, can signal telephony applications when a defined event occurs. The receiving application might or might not act on this message. Some of the message events defined include:

·	Receiving a telephone call

·	Detecting call termination

·	Incoming caller identification string

·	Handset hook status

·	Handset touch-tone key press

·	Telephone line hook status

·	Telephone line touch-tone key press

·	Telephone ring detected

There are additional messages defined for the Fax and TAM drivers which notify an application about more application specific events. The Microsoft Windows message chosen to signal these events is MM_MCIEVENT, because of the function's similarity to its OS/2 counterpart. "Initializing the Application" on page 1-6 describes the MM_MCIEVENT message in detail.

By using the supplied event handler, a telephony application need not poll the status of any of the important telephony peripherals.

## Developing an Mwave Telephony Application

The Mwave telephony device drivers are compliant with the guide-lines of the MCI command interface of MMPM and Microsoft Windows. The drivers implement all required MCI commands, and include additional MCI extensions to provide a simple yet comprehensive interface to the telephone device. Information specific to the functionality of the individual drivers is available in their corresponding chapters.

Before developing an Mwave telephony application, you need to become familiar with the procedures involved in using the MCI interface. This document assumes you are familiar with MCI. The following manuals, provided in the Microsoft Windows 3.1 Software Development Kit, describe MCI command execution in detail, and also explain the various commands and messages involved in writing an MCI-based Microsoft Windows application:

·	The Microsoft Windows Software Development Kit Multimedia Programmer's Guide contains an excellent overview of MCI, and provides code examples detailing the use of the MCI interface.

·	The Microsoft Windows Software Development Kit Multimedia Programmer's Reference provides a summary of the Microsoft Windows Multimedia API, including function and message descriptions, data types and structures, and Multimedia file formats.

The following manuals, provided in the Multimedia Presentation Manager Toolkit/2,  describe this information for OS/2.

·	The MMPM/2 Sample Application Programming Guide (S71G-2221)  contains an excellent overview of MCI, and provides code examples detailing the use of the MCI interface.

·	The MMPM/2 Programming Reference (S71G-2222) provides a summary of the MMPM  API, including function and message descriptions, data types and structures, and multimedia file formats.

This section illustrates the use of the common telephone interface, which is integrated into every telephony device. The information supplied here applies to all of the supplied telephony device drivers, although the code examples have references to specific drivers. More information on programming each individual device driver is supplied in a separate chapter.

### Initializing the Application

The most significant difference between traditional MCI devices and the telephony drivers supplied here is the use of an event handing routine. Communication of real-time status information from the MCI device to the application is performed through this application event handler. The handler should be able to service messages posted by the MCI device, which contain real-time status information about the device. The message, MM_MCIEVENT, is not a standard MCI message under Microsoft Windows. Thus, a Microsoft Windows application must call the RegisterWindowMessage function with the string "MM_MCIEVENT" to obtain the numeric value of the notification message.

MM_MCIEVENT

In addition to the message itself, wParam and lParam are used to pass information to the application.

WPARAM   wParam

Contains a device specific event message wEvent.

LPMCI_EVENT_PARMS    lParam

Specifies a far pointer to the following MCI_EVENT_PARMS structure:

typedef struct {

DWORD	dwDataParam1;

DWORD	dwEventData;

} MCI_EVENT_PARMS;

The data parameters are defined as follows:

DWORD   dwDataParam1

The low-order word specifies the device specific event message wEvent (same as wParam). The high-order word specifies the device ID of the device initiating the message.

DWORD   dwEventData

Contains a data parameter, which is dependent on the message type. This parameter is usually an on / off indication, or a pointer to string data.

The message type contained in wEvent, and the message data (or pointer to data) contained in dwEventData, comprise the event message. The value and meaning of the message data varies according to the individual message. The individual telephony device messages for each device are detailed in their corresponding MCI command reference chapters of this document.

The following code example illustrates the initialization of a Microsoft Windows application, including the MCI event handler. Modifications to the code include the implementation of three distinct functions to handle the initialization of the MCI driver environment. These functions have been isolated in the example for the sake of clarity, and could be integrated into the main program logic of an actual Microsoft Windows application. The functions are as follows:

InitDriverEnv()	Initializes the driver environment by opening the driver and installing the event handler.

UninitDriverEnv()	Uninitializes the driver, by simply closing the device.

EventHandler()	Receives messages from MCI (both MM_MCINOTIFY and MM_MCIEVENT), and acts on these messages. This routine is shown as a separate window procedure, but the code could easily be merged into the main window procedure.

An entire OS/2 and Microsoft Windows startup example is shown below.  Some of the modifications made to the generic application startup routines have been highlighted for easier reference.

//----------------------------------------------------------------------

// Windows Sample Code                                         GENAPP.C

//----------------------------------------------------------------------

//

// 	This example shows how to open the MCI device, and initialize the

// 	event handler.  The event handler routine shown in this example,

//  receives MM_MCINOTIFY messages as well as MM_MCIEVENT messages.

//

// WinMain()			- Invokes initialization & contains message loop

// InitApplication()	- Register window classes

// InitInstance()	- Create application & event handler windows

// InitDriverEnv()	- Initialize MCI driver & register event handler

// UninitDriverEnv()	- Close the MCI driver

// EventHandler()	- Process incoming MCI messages

// MainWndProc()		- Main program window proc

//----------------------------------------------------------------------

#include <windows.h>

#include <mmsystem.h>

#include <mciftdd.h>

#include <stdio.h>

HANDLE hInst;

HWND   hMainWnd,hEventHandler;

static UINT wOurDeviceID = 0;

//

// WinMain - Program entry point

//

int PASCAL WinMain(hInstance, hPrevInstance, lpCmdLine, nCmdShow)

HANDLE hInstance;

HANDLE hPrevInstance;

LPSTR lpCmdLine;

int nCmdShow;

{

MSG msg;

// Register the window classes if first time through, else abort

if( hPrevInstance || !InitApplication(hInstance) )

return (FALSE);

// Create the main windows and event handler

if( !InitInstance(hInstance, nCmdShow) )

return (FALSE);

// Initialize the MCI driver and begin execution

if( InitDriverEnv() )

{

while (GetMessage(&msg,NULL,NULL,NULL))

{

TranslateMessage(&msg);

DispatchMessage(&msg);

}

// Close and clean up the MCI driver environment

UninitDriverEnv();

}

return (msg.wParam);

}

//

// InitApplication - Register the window classes to be used

//

BOOL InitApplication(hInstance)

HANDLE hInstance;

{

WNDCLASS  wc;

BOOL  bTmp;

wc.style				= CS_OWNDC | CS_VREDRAW | CS_HREDRAW;

wc.lpfnWndProc		= MainWndProc;

wc.cbClsExtra		= 0;

wc.cbWndExtra		= 0;

wc.hInstance			= hInstance;

wc.hIcon				= LoadIcon(hInstance,"AppIcon");

wc.hCursor			= LoadCursor(NULL, IDC_ARROW);

wc.hbrBackground		= GetStockObject(BLACK_BRUSH);

wc.lpszMenuName		= "AppMenu";

wc.lpszClassName		= "AppWClass";

bTmp = RegisterClass(&wc);

wc.style				= 0;

wc.lpfnWndProc		= EventHandler;

wc.cbClsExtra		= 0;

wc.cbWndExtra		= 0;

wc.hInstance			= hInstance;

wc.hIcon				= 0;

wc.hCursor			= 0;

wc.hbrBackground		= 0;

wc.lpszMenuName		= 0;

wc.lpszClassName		= "HandlerWClass";

return( RegisterClass(&wc) && bTmp );

}

//

// InitInstance - Create the main window and the event handler window

//

BOOL InitInstance(hInstance, nCmdShow)

HANDLE hInstance;

intnCmdShow;

{

hInst = hInstance;

hMainWnd = CreateWindow("AppWClass","Event Handler Example",

WS_OVERLAPPEDWINDOW,CW_USEDEFAULT,

CW_USEDEFAULT,CW_USEDEFAULT,CW_USEDEFAULT,

NULL,NULL,hInstance,NULL );

hEventHandler = CreateWindow("HandlerWClass",0,0,0,0,0,0,

NULL,NULL,hInstance,NULL);

if (!hMainWnd || !hEventHandler )

return (FALSE);

ShowWindow(hMainWnd, nCmdShow);

UpdateWindow(hMainWnd);

return (TRUE);

}

//

// InitDriverEnv - Initialize the MCI driver environment

//

UINT InitDriverEnv()

{

MCI_OPEN_PARMS		mciOpenParms;

MCI_FAX_SET_PARMS 	mciSetParms;

// Open the MCI Driver of choice (in this case FAX), and register

// the message handler using MCI_SET...

mciOpenParms.dwCallback = hEventHandler; //set handle in dwCallback for open

mciOpenParms.lpstrDeviceType = "Mwavefax";

if( mciSendCommand(0,MCI_OPEN,MCI_OPEN_TYPE,

(DWORD)(LPVOID)&mciOpenParms) )

MessageBox(hMainWnd,"MCI Open Error","MCI_OPEN",MB_OK);

else

{

wOurDeviceID = mciOpenParms.wDeviceID;

mciSetParms.dwItem= MCI_FAX_SET_EVENT_HANDLER;

mciSetParms.dwSetData = hEventHandler;

mciSendCommand(wOurDeviceID,MCI_SET,

MCI_SET_ITEM,(DWORD)(LPVOID)&mciSetParms);

}

return(wOurDeviceID);

}

//

// UninitDriverEnv - Close down the MCI driver

//

void UninitDriverEnv()

{

MCI_GENERIC_PARMS mciGenericParms;

// Here we'll simply close the driver...

mciGenericParms.dwCallback = hEventHandler; //Set handle in dwCallback for Close

mciSendCommand( wOurDeviceID, MCI_CLOSE, MCI_WAIT, &mciGenericParms );

}

//

// EventHandler - Handle messages from MCI

//

long FAR PASCAL EventHandler(hWnd, message, wParam, lParam)

HWND hWnd;

unsigned message;

WPARAM wParam;

LPARAM lParam;

{

static UINT		uMCIMessage = 0xffff;

unsigned short	wDeviceID;

unsigned short	wEvent;

DWORD			dwEventData;

char				tmpstr[80];

switch (message)

{

case WM_CREATE:

// Register the new event message to be received

uMCIMessage = RegisterWindowMessage("MM_MCIEVENT");

break;

case MM_MCINOTIFY:

// *** Received a NOTIFY message ***

// Get DeviceID of the driver which is sending the message

wDeviceID = LOWORD( lParam );

// Check the message...

switch( wParam )

{

.

Handle MM_MCINOTIFY messages here.

.

}

break;

default:

if( message == uMCIMessage )

{

LPMCI_EVENT_PARMS mep = (LPMCI_EVENT_PARMS)lParam;

// *** Received an EVENT message ***

// Get DeviceID of the driver which is sending the message

wDeviceID = HIWORD( mep->dwDataParam1 );

// Get message being sent (wEvent). We could simply assign

// wEvent = wParam;, but for illustration we'll use...

wEvent = LOWORD( mep->dwDataParam1 );

// Get the data associated with the message (dwEventData)

dwEventData = mep->dwEventData;

// Check the message...

switch( wEvent )

{

.

Handle MM_MCIEVENT messages here.

.

}

}

else

return (DefWindowProc(hWnd, message, wParam, lParam));

}

return (NULL);

}

//

// MainWndProc - This is the window procedure for our main window

//

long FAR PASCAL MainWndProc(hWnd, message, wParam, lParam)

HWND hWnd;

unsigned message;

WPARAM wParam;

LPARAM lParam;

{

.

Standard Window Procedure

.

}

// OS/2 MMPM Sample Code                                          genapp.c

//

// This example shows how to open an MCI device and initialize the

// event handler.

//

// main

// MyWindowProc()      Process messages from MCI

// InitDriverEnv()     Initialize the MCI driver and register event handler

// UninitDriverEnv()   Close the MCI driver

// main()              Main program window procedure

//

#include <os2.h>                        // PM header file

#include "tam.h"                        // also includes mciftdd.h

#include <string.h>

#include <stdlib.h>

#include <stdio.h>

#include <stdarg.h>

#define STRINGLENGTH 80                 // Length of string

// Function prototypes

MRESULT EXPENTRY MyWindowProc( HWND hwnd, ULONG msg, MPARAM mp1, MPARAM

mp2 );

static int InitDriverEnv(void);

static void UninitDriverEnv(void);

char *lpAppName    = "Mwave TAM";

char *lpIniName    = "MWTAM.INI";

char *AnnounceFile = "\\announce.tam";

char *AnnounceTmp  = "\\announce.tmp";

long dwBFE;

// Define parameters by type

HAB  hab;                               // PM anchor block handle

CHAR szTAM[11] = "TAM Sample";          // String parameters, set in

CHAR szString[STRINGLENGTH];            // procedure.

PSZ  pszErrMsg;

HWND hwndClient = NULLHANDLE;           // Client area window handle

HWND hwndFrame = NULLHANDLE;            // Frame window handle

HWND hwndMenu;                          // Handle for the Menu bar

WORD mci_cmd_ctr = 1;

HWND    hEventHandler;

UINT    wTplDeviceID = 0;

UINT    wTpsDeviceID = 0;

//

//  lines omitted for clarity - see tam.c for complete code

//

// ----------------------------------------------------------------

// Main window procedure

INT main (VOID)

{

HMQ  hmq;                             // Message queue handle

QMSG qmsg;                            // Message from message queue

ULONG flCreate;                       // Window creation control flags

if ((hab = WinInitialize(0)) == 0L)   // Initialize PM

AbortTam(hwndFrame, hwndClient);   // Terminate the application

if ((hmq = WinCreateMsgQueue( hab, 0 )) == 0L)// Create a msg queue

AbortTam(hwndFrame, hwndClient);   // Terminate the application

if (!WinRegisterClass(                // Register window class

hab,                               // Anchor block handle

(PSZ)"MyWindow",                   // Window class name

(PFNWP)MyWindowProc,               // Address of window procedure

CS_SIZEREDRAW,                     // Class style

0                                  // No extra window words

))

AbortTam(hwndFrame, hwndClient);   // Terminate the application

flCreate = FCF_STANDARD &            // Set frame control flags to

~FCF_SHELLPOSITION &

~FCF_MAXBUTTON &

~FCF_SIZEBORDER &

~FCF_ACCELTABLE | FCF_DLGBORDER;

if ((hwndFrame = WinCreateStdWindow(

HWND_DESKTOP,            // Desktop window is parent

WS_VISIBLE,              // STD. window styles

&flCreate,               // Frame control flag

"MyWindow",              // Client window class name

szTAM,                   // No window text

0,                       // No special class style

(HMODULE)0L,             // Resource is in .EXE file

ID_WINDOW,               // Frame window identifier

&hwndClient              // Client window handle

)) == 0L)

AbortTam(hwndFrame, hwndClient);   // Terminate the application

if (!WinSetWindowPos( hwndFrame,      // Shows and activates frame

HWND_TOP,            // window at position 100, 100,

100, 100, 550, 80,   // and size 550,  70.

SWP_SIZE | SWP_MOVE | SWP_ACTIVATE | SWP_SHOW

))

AbortTam(hwndFrame, hwndClient); // Terminate the application

hEventHandler = hwndFrame;

// Get and dispatch messages from the application message queue

// until WinGetMsg returns FALSE, indicating a WM_QUIT message.

if (InitDriverEnv()) {

hwndMenu = WinWindowFromID( hwndFrame, FID_MENU );

while( WinGetMsg( hab, &qmsg, 0L, 0, 0 ) )

WinDispatchMsg( hab, &qmsg );

}

UninitDriverEnv();

WinDestroyWindow(hwndFrame);           // Tidy up...

WinDestroyMsgQueue( hmq );             // Tidy up...

WinTerminate( hab );                   // Terminate the application

} // End of main

// --------------------------------------------------------

// InitDriverEnv  -  initialize the MCI driver environment

static int InitDriverEnv(void)

{

// Open the MCI driver (in this case, Mwavetpl)

mciOpenParms.dwCallback = hEventHandler;

mciOpenParms.lpstrDeviceType = (INT *) "Mwavetpl";

dwBFE = mciSendCommand(0,MCI_OPEN,MCI_WAIT | MCI_OPEN_TYPE,

(DWORD)&mciOpenParms, mci_cmd_ctr++);

if( dwBFE )

{

error_box();

return(0);

}

// Get the device ID & register the Event Handler

wTplDeviceID = mciOpenParms.wDeviceID;

mciSetParms.dwCallback = hEventHandler;

mciSetParms.dwItem     = MCI_TAM_SET_EVENT_HANDLER;

mciSetParms.dwSetData  = hEventHandler;

mciSendCommand( wTplDeviceID,MCI_SET,MCI_WAIT | MCI_SET_ITEM,

(DWORD)&mciSetParms, mci_cmd_ctr++);

// Open the MCI Driver (in this case, Mwavetps)

mciOpenParms.dwCallback = hEventHandler;

mciOpenParms.lpstrDeviceType = (INT *)"Mwavetps";

dwBFE = mciSendCommand(0,MCI_OPEN,MCI_WAIT | MCI_OPEN_TYPE,

(DWORD)&mciOpenParms, mci_cmd_ctr++);

if( dwBFE )

{

error_box();

return(0);

}

// Get the device ID & register the Event Handler

wTpsDeviceID = mciOpenParms.wDeviceID;

mciSetParms.dwCallback = hEventHandler;

mciSetParms.dwItem     = MCI_TAM_SET_EVENT_HANDLER;

mciSetParms.dwSetData  = hEventHandler;

mciSendCommand( wTpsDeviceID,MCI_SET,MCI_WAIT | MCI_SET_ITEM,

(DWORD)&mciSetParms, mci_cmd_ctr++);

// Set to receive TAM phone calls

mciSetParms.dwItem    = MCI_TAM_SET_CALL_FILTER;

mciSetParms.dwSetData = 1;

mciSendCommand( wTplDeviceID,MCI_SET,MCI_WAIT | MCI_SET_ITEM,

(DWORD)&mciSetParms, mci_cmd_ctr++);

return(wTplDeviceID);

}

// -------------------------------------------------------

// UninitDriverEnv  - close the MCI driver

//

static void UninitDriverEnv(void)

{

mciGenericParms.dwCallback = hEventHandler;

dwBFE = mciSendCommand( wTplDeviceID, MCI_CLOSE, MCI_WAIT,

(DWORD)&mciGenericParms, mci_cmd_ctr++);

if( dwBFE )

error_box();

dwBFE = mciSendCommand( wTpsDeviceID, MCI_CLOSE, MCI_WAIT,

(DWORD)&mciGenericParms, mci_cmd_ctr++);

if( dwBFE )

error_box();

}

// --------------------------------------------------

// MyWindowProc - event handler for messages from MCI

//

MRESULT EXPENTRY MyWindowProc( HWND hwnd, ULONG msg, MPARAM mp1, MPARAM

mp2 )

{

HDC hdc;

static int          InitEnv    = 0;

static short        wKeys[3];               // Last 3 keys entered

static short        wQuiet;                 // Count for QUIET messages

static short        wKeysPressed;           // Count for 3 key command

static short        wCmdKey;                // Flag for 5-x play ctrl

unsigned short      wEvent;

unsigned long       dwEventData;

static int          FlashState = 0;

switch( msg )

{

case MM_MCINOTIFY:

switch( SHORT1FROMMP(mp1) )

{

case MCI_NOTIFY_FAILURE:

case MCI_NOTIFY_SUCCESSFUL:

case MCI_NOTIFY_SUPERSEDED:

case MCI_NOTIFY_ABORTED:

switch( wTamState )

{

case TS_COMMAND_MODE:

case TS_PLAY_MESSAGE:

PlayComplete();

break;

case TS_REMOTE_PLAY:

ContinueRemote();

wQuiet = 0;

break;

case TS_PLAY_ANNOUNCEMENT:

RecordMessage();

wQuiet = 0;

break;

case TS_RECORD_MESSAGE:

SaveMessage();

break;

case TS_ARCHIVE_PLAY:

PlayComplete();

break;

default:

break;

}

break;

}

break;

case MM_MCIEVENT:

mep = (LPMCI_EVENT_PARMS)mp2;

wEvent      = LOWORD( mep->dwDataParam1 );  // or wParam

dwEventData = mep->dwEventData;

if (dwEventData >= '0')

dwEventData -= '0';

else if (dwEventData == '#')

dwEventData = 35;

else if (dwEventData == '*')

dwEventData == 42;

switch(wEvent)

{

case PHONE_EVENT_CALL_TAM:

wKeysPressed = 0;

AnswerCall();

break;

case PHONE_EVENT_CALL_TERMINATED:

CallTerminated();

break;

case PHONE_EVENT_CALL_PROGRESS:

if( wTamState == TS_RECORD_MESSAGE ||

(wTamState==TS_REMOTE_PLAY && wRemoteState==RS_WAITING) )

switch( dwEventData )

{

case DIALTONE:

case SLOWBUSY:

case FASTBUSY:

CallTerminated();

break;

}

break;

case PHONE_EVENT_LINE_KEY:

if( wTamState == TS_REMOTE_PLAY )

{

if( wCmdKey == 5 )  // Check for play ctrl sequence

{

wCmdKey = -1;

switch( dwEventData )

{

case 1:

SeekMessage(TB_BACK);

break;

case 2:

if(!(wPause^=1))

mciSendCommand( wTpsDeviceID, MCI_RESUME, MCI_WAIT,

(DWORD)&mciGenericParms, mci_cmd_ctr++);

else

mciSendCommand( wTpsDeviceID, MCI_PAUSE, MCI_WAIT,

(DWORD)&mciGenericParms, mci_cmd_ctr++);

break;

case 3:

SeekMessage(TB_FORWARD);

break;

}

}

else                // Standard Remote Play command

{

switch( dwEventData)

{

case 1:

RemoteNext();

break;

case 2:

RemoteRemove();

break;

case 3:

RemoteRepeat();

break;

case 4:

RemoteArchive();

break;

case 5:       // Initiate play ctrl sequence

wCmdKey = (short)dwEventData;

break;

}

}

}

else                    // Check for 3 digit command code

{

wKeys[2] = wKeys[1];

wKeys[1] = wKeys[0];

wKeys[0] = (short)dwEventData;

if( ++wKeysPressed > 2 )

{

if((wKeys[2]*100+wKeys[1]*10+wKeys[0])==wCommandCode)

{

BeginRemote();

wCmdKey = -1;

}

}

}

break;

default:

break;

}

break;

case WM_CREATE:

WinStartTimer(hab, hwnd, 1000,1000UL);

break;

case WM_TIMER:

if( !wNewMessages )

{

if( FlashState )

{

WinSetWindowText(hwndFrame, szTAM);

FlashState = 0;

}

}

else if( FlashState ^= 1 )

WinSetWindowText(hwndFrame," ");

else

WinSetWindowText(hwndFrame, szTAM);

break;

case WM_COMMAND:

//

// When the user chooses option 1, 2, or 3 from the Options pull-

// down, the text string is set to 1, 2, or 3, and

// WinInvalidateRegion sends a WM_PAINT message.

// When Exit is chosen, the application posts itself a WM_CLOSE

// message.

{

USHORT command;                   // WM_COMMAND command value

command = SHORT1FROMMP(mp1);      // Extract the command value

switch (command)

{

case ID_RECANNOUNCE:

WinDlgBox( HWND_DESKTOP,      // Place anywhere on desktop

hwndFrame,         // Owned by frame

RecordAnnounce,    // Address of dialog procedure

(HMODULE)0,        // Module handle

RECANNOUNCE,       // Dialog identifier in resource

NULL);             // Initialization data

WinInvalidateRegion( hwnd, NULLHANDLE, FALSE ); // Force a repaint

break;

case ID_SETRING:

WinDlgBox( HWND_DESKTOP,      // Place anywhere on desktop

hwndFrame,         // Owned by frame

SetRingCount,      // Address of dialog procedure

(HMODULE)0,        // Module handle

RINGCOUNT,         // Dialog identifier in resource

NULL);             // Initialization data

WinInvalidateRegion( hwnd, NULLHANDLE, FALSE ); // Force a repaint

break;

case ID_COMMANDCODE:

WinDlgBox( HWND_DESKTOP,      // Place anywhere on desktop

hwndFrame,         // Owned by frame

SetCommandCode,    // Address of dialog procedure

(HMODULE)0,        // Module handle

COMMANDCODE,       // Dialog identifier in resource

NULL);             // Initialization data

WinInvalidateRegion( hwnd, NULLHANDLE, FALSE ); // Force a repaint

break;

case ID_RESET:

mp1 = (MPARAM)ID_PHONE;

case ID_PHONE:

case ID_TAM:

case ID_HANDSET:

case ID_SPEAKER:

case ID_FIRST:

case ID_PREVIOUS:

case ID_AGAIN:

case ID_NEXT:

case ID_ERASE:

case ID_REVERSE:

case ID_PAUSE:

case ID_FORWARD:

case ID_FAST:

case ID_NORMAL:

case ID_SLOW:

hdc = WinOpenWindowDC( hwnd );

ButtonAction(hdc,Menu2Button[SHORT1FROMMP(mp1)-ID_PHONE]);

break;

case ID_V0:

case ID_V1:

case ID_V2:

case ID_V3:

case ID_V4:

case ID_V5:

case ID_V6:

case ID_V7:

case ID_V8:

case ID_V9:

SetVolume(SHORT1FROMMP(mp1)-ID_V0);

break;

case ID_QUIT:

WinPostMsg( hwnd, WM_QUIT, (MPARAM)0,(MPARAM)0 );// Cause termination

break;

default:

return WinDefWindowProc( hwnd, msg, mp1, mp2 );

}

break;

}

case WM_ERASEBACKGROUND:

//

// Return TRUE to request PM to paint the window background

// in SYSCLR_WINDOW.

return (MRESULT)( TRUE );

case WM_PAINT:

// Window contents are drawn here in WM_PAINT processing.

{

HPS    hps;                       // Presentation Space handle

RECTL  rc;                        // Rectangle coordinates

POINTL pt;                        // String screen coordinates

// Create a presentation space

if( !InitEnv ) {

InitEnv = 1;

InitTamState();

ButtonAction(hdc,TB_TELEPHONE);

SetVolume(wVolume);

}

hps = WinBeginPaint( hwnd, 0L, &rc );

pt.x = 1; pt.y = 5;             // Set the text coordinates,

GpiSetColor( hps, CLR_NEUTRAL );         // colour of the text,

GpiSetBackColor( hps, CLR_BACKGROUND );  // its background and

GpiSetBackMix( hps, BM_OVERPAINT );      // how it mixes,

// and draw the string...

GpiCharStringAt( hps, &pt, (LONG)strlen( szString ), szString );

WinEndPaint( hps );                      // Drawing is complete

break;

}

case WM_CLOSE:

//

// This is the place to put your termination routines

//

sprintf(szString,"%d", wVolume);

PrfWriteProfileString(hini, lpAppName,"VOL",szString);

sprintf(szString,"%d", wMsgOut);

PrfWriteProfileString(hini, lpAppName,"MSGOUT",szString);

sprintf(szString,"%d", dwMsgIndex);

PrfWriteProfileString(hini, lpAppName,"MSGIDX",szString);

PrfCloseProfile(hini);

WinPostMsg( hwnd, WM_QUIT, (MPARAM)0,(MPARAM)0 );// Cause termination

break;

default:

//

// Everything else comes here.  This call MUST exist

// in your window procedure.

return WinDefWindowProc( hwnd, msg, mp1, mp2 );

}

return (MRESULT)FALSE;

} // End of MyWindowProc

// --------------------------------------------

// AbortTam

VOID AbortTam(HWND hwndFrame, HWND hwndClient)

{

PERRINFO  pErrInfoBlk;

PSZ       pszOffSet;

DosBeep(100,10);

if ((pErrInfoBlk = WinGetErrorInfo(hab)) != (PERRINFO)NULL)

{

pszOffSet = ((PSZ)pErrInfoBlk) + pErrInfoBlk->offaoffszMsg;

pszErrMsg = ((PSZ)pErrInfoBlk) + *((PSHORT)pszOffSet);

if((INT)hwndFrame && (INT)hwndClient)

WinMessageBox(HWND_DESKTOP,         // Parent window is desk top

hwndFrame,            // Owner window is our frame

(PSZ)pszErrMsg,       // PMWIN Error message

"Error Msg",          // Title bar message

MSGBOXID,             // Message identifier

MB_MOVEABLE | MB_CUACRITICAL | MB_CANCEL ); // Flags

WinFreeErrorInfo(pErrInfoBlk);

}

WinPostMsg(hwndClient, WM_QUIT, (MPARAM)NULL, (MPARAM)NULL);

} // End of AbortTam

The switch statements in the event handler routine are of special interest. As further code examples are provided in the Fax and TAM sections of this document, the case code for the switch statements will be filled in with code specific to the operation of the application. The event handler is the key section to this example. First, it is new to even the experienced MMPM programmer, and secondly, it is the foundation on which to build message driven applications.