# APPENDIX B - Programmer’s Notes

## B1 - Fax Notes

Warning:  The fax and TAM drivers are different.  Do not assume they work the same way.

Do not hang up the phone in any situation other than PHONE_EVENT_CALL_TERMINATED.

NOTE:  If the user wants to abort the call then the app should issue a MCI_STOP.  The app will then receive a PHONE_EVENT_CALL_TERMINATED.

## B2 - TAM Notes

Warning:  The fax and TAM drivers are different.  Do not assume they work the same way.

The app should hang up the phone for the following reasons:

- 5 uninterrupted CALL_PROGRESS_QUIET events in a row

- 5 CALL_PROGRESS_DIAL_TONE events in a row

- A CALL_PROGRESS_QUIET event during play

Warning:  Not all phone systems put out a dial tone when a person has hung up.  The application should count the number of uninterrupted quiets when not in a record or play state then hang up.  The TAM driver will not give you a phone event call terminated.

An unidentified tone event should reset the quiet and dial tone counter.

The application can load a file before a call in order to save processing time during the call.

Call progress event unidentified tone can be received during a transition state i.e. between ring tone and answer tone.

Beware of connection conflicts (handset , audio) between the TPS and TPL drivers

When the TPL driver is opened it will come up connected to normal phone.

When the TPS driver is opened it will come up connected to audio.

If the dial fails then the application needs to hang up the phone (DO NOT DO THIS WITH THE FAX!!).

A mci-stop should be made with the notify flag set so that it will be queued up behind the outstanding play or record.

The application should be aware that the discriminator can cause the record to be aborted.  In such a case the app will receive a PHONE_EVENT_CALL_TERMINATED as well.

Set audio volume vs. handset volume

Application MUST perform 	WinCreateMsgQueue(HAB, 100)  - OS/2

SetMessageQueue(100) - Windows

This will ensure that the application does not lose event messages.  The windows default is only 8!!!

If a record or play fails the application still needs to do a MCI_STOP

Set speed takes nearest match (rounding down) if no exact match.

Set speakerphone mode disables discrimination on calling tones.

The application can not load a file while a play or record is in progress.

If MCI_OPEN fails and the error code > 512 then just print out the number because GetErrorString will not provide the correct information.

MciGetErrorString requires a device id along with the error code.  When MCI_OPEN fails no device id is returned therefore calling MciGetErrorString with the error code > 512 (fax/tam error codes) will return an incorrect error string.  The error string that will be returned will be for a device driver currently in the system and your error code may be something like ( cdaudio error ).

The microphone should not be right next to the monitor, it will cause problems.

If in speakerphone mode answer tone will not work properly.

On hook and Off hook is electrical.

TAM set audio volume cant be done during dial.

The application can only get handset key events in TPS if TPS is connected to handset.  Currently TPL only gets flash “!”.

Status position during Play From is not updated until play gets going.  To get around this the application should seek to 0 before play to get current position in right place.

Application can not set volume during dial

There is no volume change for the phoneline record or play.

Mci set quality only with new file loaded.

Rule:  If app takes phone off hook it is responsible for putting it back on hook, else the driver does it.

## B3 - Integrated Application Notes

If the app chooses to use the same event handler for both fax and tam, care should be taken as to which device an event is from.

i.e.  PHONE_EVENT_CALL_TERMINATED will be sent to the tam app when a call has been discriminated as a fax call.  The app should issue an on hook (making sure to use the TAM DEVICE ID).

The app writer should be aware that mci_set and mci_status use the same constants.

i.e. MCI_TAM_SET_EVENT_HANDLER is different than MCI_FAX_SET_EVENT_HANDLER