# Chapter 4 - Fax Services

This chapter describes the telephony services available to application developers for the purpose of developing Mwave compatible Fax based applications.

## Mwave Fax Device Driver Architecture

In order to develop a functional OS/2 MMPM or Microsoft Windows fax application, three items are required: 1) hardware capable of providing fax send/receive functions, 2) a fax device driver to control the fax hardware based on inputs from the application, and 3) a device-independent programming interface between the application and the fax device driver to isolate the application from specific device driver and hardware differences.

The following block diagram illustrates this architecture as provided by the Mwave system:

Windows application

Fax API

Mwave Fax device driver

Discriminator

Mwave compliant hardware

Remote fax machine

Figure 4-1

This section provides an overview of the FAX Application Programming Interface and the Mwave Fax device driver used to develop an OS/2 or Microsoft Windows Mwave fax application.

### The Fax Application Programming Interface (API)

The FAX Application Programming Interface (API), described fully in Chapter 6 of this manual, provides the interface between an OS/2 MMPM or Microsoft Windows application and a fax device driver compliant with the FAX API. The Mwave system provides such a fax device driver, enabling any application calling the FAX API to access the fax capabilities of Mwave compliant hardware.

The FAX API was designed to be very similar to the  Media Control Interface (MCI) standard used in MMPM and Windows. The fax specific extensions to the MCI API were designed to provide a 'hands-off' interface to a fax driver's send and receive capabilities, while supplying a rich set of features and options. The MCI command message format is ideal for setting and tracking device status in an orderly manner, and with a couple of command extensions, allows the applications programmer to easily incorporate fax send and receive capabilities into an application.

The FAX API provides support for the following basic operations:

·	Receiving a Fax Document File

·	Sending a Fax Document File

·	Converting from Fax Document File format to Device Independent Bitmap (DIB) format and visa-versa

The following sections describe various aspects and features of the FAX API, and how they relate to the Mwave Fax device driver implementation.

### Fax Document File Format

For simplicity, the FAX API command extensions to the standard MCI functions include file send and file receive commands. Fax documents (single and multiple pages) are treated as a single Fax Document File to simplify the process of communicating with the fax device driver. Under the single file scheme, the application need only call a single send or receive command to send or receive an entire fax document, allowing the application to monitor the progress of the operation, but not requiring constant maintenance of the transmission procedure.

The native Fax Document File format used by the Mwave Fax device driver to send and receive fax image data is TIFF Class F. This format was chosen because it provides efficient, multiple-page storage capability.

An Mwave fax application can work directly with fax files in their native TIFF Class F format. Alternatively, the FAX API provides commands to convert from device independent Fax Document File format to Device Independent Bitmap (DIB) format and visa-versa. Thus, the application is able to use DIB format when displaying and printing fax images, but use the more efficient Fax Document File format to store the fax image data to disk.

The file format conversion commands provided by the Fax API enable an application to construct Fax Document Files from  DIB files, and extract a DIB format file from a Fax Document File. A Fax Document File is composed of multiple pages of fax image data, while a DIB file represents a single page from a multi-page Fax Document File. Using the file format conversion commands allows the extraction, insertion, and/or replacement of any page within a Fax Document File.

### Command Message Summary

The main design goals when defining the MCI commands for the FAX API were ease of use and hands-off operation. The following table provides a summary of the MCI commands available through the FAX API and Mwave Fax device driver. For complete details on these commands, see the FAX API Reference, Chapter 6 of this document.

| MCI Command | Description |
| --- | --- |
| MCI_CLOSE | Close the device driver |
| MCI_CONVERT | Convert to / from device dependent file data |
| MCI_DIAL | Dial the telephone |
| MCI_GETDEVCAPS | Get the capabilities of the device |
| MCI_INFO | Get device string identifier |
| MCI_OPEN | Open the device driver |
| MCI_RECEIVE | Receive a fax file |
| MCI_SEND | Send a fax file |
| MCI_SET | Configure the device |
| MCI_STATUS | Query device configuration |
| Table 4-1: MCI Command Summary |  |

Programmers familiar with the standard MCI specification will note the addition of the following MCI commands to the FAX API:

·	MCI_CONVERT

·	MCI_DIAL

·	MCI_RECEIVE

·	MCI_SEND

The MCI_CONVERT command is required to convert device independent Fax Document Files (TIFF Class F in the case of the Mwave Fax device driver) to/from DIB files. The MCI_DIAL command is required to dial the telephone device. The other two new commands, MCI_RECEIVE and MCI_SEND, provide generic multi-page file receive and send capability.

### Event Message Summary

The Mwave Fax device driver uses event messages to inform an application when various telephony-related events occur. The following table provides a summary of the MCI event messages which an application can receive from the Mwave Fax device driver. For complete details on these event messages, see Chapter 6 of this manual.

| Event Message | Description |
| --- | --- |
| PHONE_EVENT_CALL_PROGRESS | Call progress state has changed |
| PHONE_EVENT_CALL_FAX | An incoming fax call has been received |
| PHONE_EVENT_CALL_TERMINATED | Call terminated (supplies termination code ) |
| PHONE_EVENT_CALLER_ID | Caller ID string detected (supplies string pointer) |
| PHONE_EVENT_FAX_CONNECT | Returns connection parameters |
| PHONE_EVENT_FAX_HEADER | Supplies fax header from calling machine |
| PHONE_EVENT_FAX_PAGE_COMPLETE | Signals that a fax page has been completed |
| PHONE_EVENT_FAX_PAGE_STATUS | Supplies individual page completion status |
| PHONE_EVENT_FAX_POLL | Request to poll received |
| PHONE_EVENT_HANDSET | Change in handset status (supplies status) |
| PHONE_EVENT_HANDSET_KEY | Keypad press from handset (supplies character) |
| PHONE_EVENT_LINE | Change in hook status (supplies status) |
| PHONE_EVENT_LINE_KEY | Keypad press from line (supplies character) |
| PHONE_EVENT_RING | Telephone ring status (supplies ring on/off) |
| Table 4-2: MCI Event Message Summary |  |

## Developing an Mwave Fax Application

This section describes how to develop an application which calls the FAX API to access the Mwave Fax device driver, providing fax send and receive capabilities.

Throughout this section, code snippets are used to illustrate the basic concepts of how to use the Mwave Fax device driver. These code snippets are part of a complete  fax application example, fax.exe (referred to as FAXAPP throughout the remainder of this section). included on the companion diskette. One example is provided for OS/2 and one for Microsoft Windows. The source code is provided for reference, and can also be used as a starting point from which you can develop your own Mwave fax application.

The code snippets in this book are accompanied by the filename and function name (indicated by [ filename: function() ] ) of the FAXAPP source module where the corresponding source code can be located.

### FAXAPP Application Definition

FAXAPP demonstrates the basic concepts required to add fax send/receive/view capability to an application through the use of the Mwave Fax device driver. These concepts are illustrated by providing support for the following capabilities:

·	Send a Fax Document File or DIB file to a remote fax machine via a user specified phone number. The send operation runs in the background, allowing other operations to occur.

·	Receive a Fax Document File from a remote fax machine. The receive operation runs in the background, allowing other operations to occur.

·	View a user specified page from a Fax Document File. The view operation  runs in the foreground.

The purpose of FAXAPP is to provide an example of using the basic fax send and receive capabilities of the FAX API and the Mwave Fax device driver. As a result, there are many other capabilities provided by the FAX API and Mwave Fax device driver which are not demonstrated in FAXAPP, but might be useful for your particular application.  Additionally, although the sample is a functional fax machine, it doesn't contain the error handling or feature set of a complete robust application.

### How to run FAXAPP

This section provides a brief overview of how to run the example fax application FAXAPP provided in the Mwave system.

Starting FAXAPP

Copy the \fax subdirectory (for OS/2 or Windows) from the companion diskette to a \fax subdirectory (or other convenient subdirectory) on your system. Add a fax icon to the desktop if you wish. To start FAXAPP, simply double-click on the Fax  program icon (or start from an OS/2 command line or the Windows File|Run menu).  When the fax application starts, a screen appears to tell you that its initialization has completed.  Click OK to continue the operation.

Sending a fax

Facsimile machines send and receive files in a format called TIFF Class F format.  Many PC programs generate and manipulate image files in a different format called BMP (bitmapped).  The fax application supports both these formats   BMP files are converted to TIFF format before being sent   The Mwave FAX driver supports conversion of BMP files to TIFF (and vice-versa), and the application uses this conversion support.  The application supports only black and white (monochrome) files. Color BMP files are not supported by the Mwave FAX device driver.

To send a file:

1.  Select the Send command from the Options menu.

2.  Select the file (either a BMP or TIFF file) to be sent.  If the selected file is a BMP file, the application will convert the file from BMP format to TIFF format before being sent.  The application prompts you to enter the destination filename where the converted TIFF file is to be stored.  Be sure and specify a .TIF filename extension for the destination file.  Also, make sure the BMP file is monochrome.  Color BMP files will not be sent correctly.

3.  Enter the phone number of the remote fax machine when prompted. The specified file is then sent, in the background, to the remote fax machine, allowing you to continue to use other applications.  A message box is displayed after successful completion or call termination due to an error.

Receiving a fax

The application  automatically receives fax data from incoming fax calls.

The receive operation proceeds in the background, allowing you to continue working with other applications.  Message boxes are used to notify you that a fax call has been received,  to display the receive operation's completion status (either success or failure), and to indicate the name of the received TIFF file.

Viewing a fax

FAXAPP enables you to view fax data from a BMP file or a single page from a TIFF FAX Document File.  To view a fax:

1.  Select the View command from the Options menu.

2.  Select the file (either a BMP or TIFF file) to be viewed.  TIFF files will be converted to BMP format before being viewed.  For TIFF files, the application prompts you to enter the destination filename where the converted BMP file will be stored.  Be sure and specify a .BMP filename extension for the destination file.  If there are multiple pages of fax data within the specified TIFF file, you are prompted to select the number of the page you wish to view.

3. When the conversion completes (if conversion was required...if you asked to view a TIFF file), the fax application displays the file.  (The application actually displays a negative image of the file.  Where the original image is dark, the displayed image is light and vice-versa).

Other commands

Two additional commands are available from the FAXAPP Options menu.  They are:

Hang up

This command hangs up the phone (places the phone device on hook).  You can

hang up the phone any time.

Clear screen

This command clears the fax image currently displayed (if any) in the fax application window.  Select the View command if you want to view another fax.

### FAXAPP Code Model Design

This section briefly describes a few of the design considerations used to develop FAXAPP.

The primary goal of FAXAPP is to illustrate the operation of a fax device, which is event (message) driven, and thus is able to execute as a background task. There are some functions which are performed in the foreground, but these include only those functions (such as viewing a BMP file) invoked when the user is using FAXAPP in the foreground.

Because FAXAPP is designed to be message and event driven, a brief review of the types of messages and events which can be sent to FAXAPP by the host PC and the Mwave Fax device driver is useful.

MM_MCIEVENT

The MM_MCIEVENT event message is sent by the Mwave Fax device driver as a direct result of an external telephony event. All event messages are for notification purposes only, and the application is not required to perform any action to handle any of these events. The event messages are very useful however for writing event driven applications. The messages that are handled in FAXAPP are:

·	PHONE_EVENT_CALL_FAX

An incoming fax call has been received. The application must begin receiving the incoming fax data.

·	PHONE_EVENT_CALL_TERMINATED

An active call (send or receive) has been terminated, either successfully, or due to some error condition.

MM_MCINOTIFY (Windows only)

The MCI notification message MM_MCINOTIFY is the standard method for MCI to notify an application that an MCI command has been completed. This message is sent to an application whenever an MCI command is called with the MCI_NOTIFY flag specified. In FAXAPP, the MCI_NOTIFY flag is used instead of the MCI_WAIT flag for those MCI commands (MCI_DIAL and MCI_RECEIVE) which can take a substantial amount of time to complete, thus freeing the Microsoft Windows system to respond to other actions.

The MM_MCINOTIFY message is handled in the WndProc() function in fax.c. Receipt of MM_MCINOTIFY messages cause a change in the FAXAPP state machine.

#### The FAXAPP State Machine (Windows only)

FAXAPP's send and receive operations are implemented using a very simple state machine. FAXAPP can be in only one of the following states at any given time:

| State | Description |
| --- | --- |
| STATE_IDLE | No send or receive operation is in progress. |
| STATE_DIALING | Fax driver is dialing and attempting to connect with a remote fax machine. |
| STATE_SENDING_FAX | Connection with remote fax machine complete. Fax data is being sent. |
| STATE_RECEIVE_SETUP | An incoming fax call has been detected. The fax driver is now being set up to receive incoming fax data from a remote fax machine. |
| STATE_RECEIVING_FAX | Fax driver is receiving incoming fax data. |
| Table 4-4: FAXAPP States |  |

The FAXAPP state machine proceeds from state-to-state based on MM_MCINOTIFY messages received from the MCI_DIAL and MCI_RECEIVE commands (which are called with the MCI_NOTIFY flag specified). We'll get into more detail about these state changes in later sections, which deal with how to send and receive fax files.

FAXAPP uses the same window procedure (WndProc() in fax.c) to handle messages sent by both Microsoft Windows and the Mwave Fax device driver. The single message handling procedure implemented in FAXAPP was done purely for demonstration purposes. A dual procedure approach (one procedure handling Microsoft Windows messages and the other handling event message from the Mwave Fax device driver) could just as easily been used. For an example of a dual-procedure approach, see the TAM sample application provided on the companion diskette.

#### Received Fax Document Filenames

In order for FAXAPP to receive files in the background without requiring the user to specify a destination filename, a simple file naming scheme is used to automatically store received Fax Document Files.

Each time a fax call is received, the corresponding fax image data (either a single page or multi-page fax) is stored in a file with a file name format of FAX??.TIF, where ?? is a sequential decimal value which is incremented after the completion of every fax call, and is reset to zero whenever FAXAPP is started. Thus, FAXAPP overwrites an existing FAX0.TIF, FAX1.TIF, etc. whenever it receives new incoming fax calls after being restarted.

#### FAXAPP Source File Descriptions

The following source files comprise the FAXAPP example application:

| File | Description |
| --- | --- |
| makefile | Microsoft C 7.0 / Windows 3.1 or IBM C Set/2 compatible make file |
| fax.c | Contains FAXAPP initialization code and procedure to handle all window messages |
| faxdlgs.c | Contains functions to display and process dialog boxes |
| faxops.c | Contains all functions which interface to the FAX API |
| view.c | Contains functions to enable viewing of fax image data |
| fax.def | Linker definition file |
| fax.h | FAXAPP specific include file |
| mciftdd.h | Mwave Fax/TAM device driver include file |
| fax.rc | FAXAPP resource file |
| Table 4-5 |  |

### Opening and Initializing the Mwave Fax Driver - Windows

The first thing FAXAPP must do before sending and/or receiving fax files is to open and initialize the Mwave Fax device driver. The steps required are:

Step 1.	Register the MM_MCIEVENT message.

Step 2.	Open the Mwave Fax device driver.

Step 3.	Set up the fax driver event handler.

Step 4.	Set up the call filter.

This sequence of steps is performed automatically whenever FAXAPP is started (see the WinMain() function in fax.c). Let's take a closer look at each step in the Mwave Fax device driver open and initialization process.

Step 1.	Register the MM_MCIEVENT message

The Mwave Fax device driver communicates events to the application through the use of the "MM_MCIEVENT" message (see "Error: Reference source not found" on page 1-Error: Reference source not found for complete details on the "MM_MCIEVENT" message). Because this is not a standard MCI message under Microsoft Windows, it must be registered.

In addition, the "MM_MCIEVENT" message must be registered prior to setting the event handler window procedure (Step 3 in the open/initialize process) which  handles messages sent to our application from the Mwave Fax device driver. This is to insure that our application does not miss any "MM_MCIEVENT" messages which can be sent by the driver.

The following call registers the "MM_MCIEVENT" message and assigns the numeric value returned to the global variable uMCIMessage.

[ fax.c: WinMain() ]

/*------------------------------------------------------------------*/

/*  Register the MM_MCIEVENT message                                */

/*------------------------------------------------------------------*/

uMCIMessage = RegisterWindowMessage( "MM_MCIEVENT" );

The Mwave Fax device driver issues a "MM_MCIEVENT" message to Microsoft Windows whenever the driver needs to inform the application that some telephony event has occurred. Microsoft Windows then translates the "MM_MCIEVENT" message request and send the corresponding numeric value returned from the RegisterWindowMessage() function to our application's event handling procedure.

Now that the "MM_MCIEVENT" message has been registered, we can safely open the Mwave Fax device driver.

Step 2.	Open the Mwave Fax device driver

The Mwave Fax device driver is identified by the device type "Mwavefax" (case is not sensitive). This device type is used with the MCI_OPEN command to open the driver.

[ faxops.c: InitFax() ]

/*------------------------------------------------------------------*/

/*  Open the device by specifying only the "Mwavefax" device type   */

/*------------------------------------------------------------------*/

ResourceMessageBox( hWnd, IDS_MSG_INIT_DRIVER, 0, szAppName, MB_OK);

SetCursor(hcWaitCursor);

mciOpenParms.dwCallback = (DWORD)hWnd;

mciOpenParms.lpstrDeviceType = "Mwavefax";

dwReturn = mciSendCommand( NULL,                    // device ID

MCI_OPEN,                   // command

MCI_WAIT | MCI_OPEN_TYPE,   // flags

(DWORD)lpmciOpenParms );    // parameter block

if( dwReturn )

{

ResourceMessageBox(hWnd, IDS_ERR_INIT_DRIVER, (UINT)dwReturn, NULL, MB_OK);

return( FALSE );

}

wMwaveFaxID = lpmciOpenParms->wDeviceID;

NOTE:	For Windows, the handle of the window procedure responsible for processing MM_MCINOTIFY messages MUST be specified by assigning it to mciOpenParms.dwCallback prior to calling the MCI_OPEN command, regardless of whether the MCI_WAIT or MCI_NOTIFY flag is specified in the MCI_OPEN call. Failure to do so when using versions earlier than 2.1 will result in erratic behavior of the Mwave Fax device driver.

If the MCI_OPEN command completes successfully, the device ID of the Mwave Fax device driver (returned in lpmciOpenParms->wDeviceID) is assigned to the variable wMaveFaxID. This variable is specified in the remaining mciSendCommand calls to identify the Mwave Fax device driver.

As illustrated in the code example above, you should always check the return value from the MCI_OPEN command for an error. There are a number of conditions (insufficient memory or MIPS available on the Mwave board) which can cause the Mwave Fax device driver to fail opening, and these cases should be handled properly by the application.

Step 3.	Set up the fax driver event handler

After opening the driver, the next step is to set the window procedure our application uses to handle incoming "MM_MCIEVENT" event messages sent by the Mwave Fax device driver. This should be done immediately after opening the driver to minimize the chance of missing any driver event messages. The MCI_SET command with the MCI_FAX_SET_EVENT_HANDLER item is used to set the event handler procedure.

Recall that FAXAPP uses the same window procedure (WndProc() in fax.c) to process messages sent by both Microsoft Windows and the Mwave Fax device driver. This window procedure is assigned to our main application window, identified as hWnd. Thus, we specify hWnd as the window to receive "MM_MCIEVENT" messages.

[ faxops.c: InitFax() ]

/*------------------------------------------------------------------*/

/*  Set up the FAX driver event handler to our main application     */

/*  window procedure, since this is where we will process event     */

/*  messages sent from the FAX driver.                              */

/*------------------------------------------------------------------*/

mciSetParms.dwItem = MCI_FAX_SET_EVENT_HANDLER;

mciSetParms.dwSetData = (DWORD)hWnd;

SetCursor(hcWaitCursor);

dwReturn = mciSendCommand( wMwaveFaxID,

MCI_SET,

MCI_WAIT | MCI_SET_ITEM,

(DWORD)lpmciSetParms );

if( dwReturn )

{

ResourceMessageBox(hWnd, IDS_ERR_SET_EVENT_HANDLER, (UINT)dwReturn, NULL, 				MB_OK);

return( FALSE );

}

Step 4.	Set up the call filter

The last step required in the Mwave Fax device driver initialization process involves setting up the call filter. Setting the call filter to TRUE informs the Mwave Fax device driver that it is to receive fax calls.

Setting the call filter also provides a mechanism to insure that no other application is expecting to receive a fax call. Attempting to enable the call filter when another application has already enabled the filter results in an error return.

[ faxops.c: InitFax() ]

/*------------------------------------------------------------------*/

/*  Set the call filter                                             */

/*------------------------------------------------------------------*/

mciSetParms.dwItem = MCI_FAX_SET_CALL_FILTER;

mciSetParms.dwSetData = TRUE;

SetCursor(hcWaitCursor);

dwReturn = mciSendCommand( wMwaveFaxID,

MCI_SET,

MCI_WAIT | MCI_SET_ITEM,

(DWORD)lpmciSetParms );

if( dwReturn )

{

ResourceMessageBox(hWnd, IDS_ERR_SET_FILTER, (UINT)dwReturn, NULL, MB_OK);

return( FALSE );

}

### Opening and Initializing the Mwave Fax Driver - OS/2

The first thing FAXAPP must do before sending and/or receiving fax files is to open and initialize the Mwave FAX device driver. The steps required are:

1.  Open the Mwave FAX device driver.

2.  Set up the fax driver event handler.

3.  Set up the call filter.

This sequence of steps is performed automatically whenever FAXAPP is started (see the main function in fax.c). Let's take a closer look at each step in the Mwave FAX device driver open and initialization process.

Step 1. Open the Mwave FAX device driver

The Mwave FAX device driver is identified by the device type "Mwavefax" (case is not sensitive). This device type is used with the MCI_OPEN command to open the driver.

faxops.c: InitFAX

int InitFax(HWND hWnd)

{

char messagestring[255];

MessageBox (hWnd, "Initializing the fax driver",

szAppName, MB_OK|MB_ICONEXCLAMATION);

WinSetPointer (HWND_DESKTOP,

WinQuerySysPointer(HWND_DESKTOP,

SPTR_WAIT, FALSE));

mciOpenParms.lpstrDeviceType = (LPSTR) "Mwavefax";

mciOpenParms.dwCallback = (DWORD) hWnd;

dwReturn = mciSendCommand(wDeviceID,

MCI_OPEN,

MCI_WAIT | MCI_OPEN_TYPE,

(DWORD) lpmciOpenParms, ++mciCall);

if (dwReturn)

{       /* Error, unable to open device  */

if (!(mciGetErrorString(dwReturn,

(int *)messagestring, sizeof(messagestring))))

{

MessageBox(hWnd, messagestring, NULL,

MB_OK|MB_ERROR);

}

else

{

sprintf(messagestring,

"Unable to open device or

GetErrorString.RC= %d",

(LOWORD(dwReturn)));

MessageBox(hWnd, messagestring, NULL,

MB_OK|MB_ERROR);

}

return FALSE;

}

/* Device opened successfully, get the device ID */

wDeviceID = lpmciOpenParms->wDeviceID;

If the MCI_OPEN command completes successfully, the device ID of the Mwave FAX device driver  is assigned to the variable wMwaveFaxID. This variable is specified in the remaining mciSendCommand calls to identify the Mwave FAX device driver.

As illustrated in the code example above, you should always check the return value from the MCI_OPEN command for an error. There are a number of conditions (such as insufficient memory or MIPS available in  the Mwave DSP) that can cause the Mwave FAX device driver to fail opening.   These cases should be handled properly by the application.

Step 2.	Set up the fax driver event handler

After opening the driver, the next step is to set the window procedure our application uses to handle incoming "MM_MCIEVENT" event messages sent by the Mwave FAX device driver. This should be done immediately after opening the driver to minimize the chance of missing any driver event messages. The MCI_SET command with the MCI_FAX_SET_EVENT_HANDLER item is used to set the event handler procedure.

Recall that FAXAPP uses the same window procedure (WndProc() in fax.c) to process messages sent by both MMPM and the Mwave FAX device driver. This window procedure is assigned to our main application window, identified as hWnd. Thus, we specify hWnd as the window to receive MM_MCIEVENT messages.

faxops.c: InitFax

mciSetParms.dwCallback = (DWORD) hWnd;

mciSetParms.dwItem = MCI_FAX_SET_EVENT_HANDLER;

mciSetParms.dwSetData = (DWORD) hWnd;

WinSetPointer (HWND_DESKTOP,                WinQuerySysPointer(HWND_DESKTOP, SPTR_WAIT, FALSE));

dwReturn = mciSendCommand(wDeviceID, MCI_SET,

MCI_WAIT | MCI_SET_ITEM,

(DWORD) lpmciSetParms, ++mciCall);

if (dwReturn)

{

if (!(mciGetErrorString(dwReturn,

(int *)messagestring, sizeof(messagestring))))

{

MessageBox (hWnd, messagestring,

NULL, MB_OK|MB_ERROR);

}

else

{

MessageBox(hWnd,"Unable to set the event handler",

NULL, MB_OK);

}

return FALSE;

}

Step 3. Set up the call filter

The last step required in the Mwave FAX device driver initialization process involves setting up the call filter. Setting the call filter to TRUE informs the Mwave FAX device driver that it is to receive fax calls.

Setting the call filter also provides a mechanism to insure that no other application is expecting to receive a fax call. Attempting to enable the call filter when another application has already enabled the filter  results in an error return.

mciSetParms.dwItem = MCI_FAX_SET_CALL_FILTER;

mciSetParms.dwSetData = TRUE;

WinSetPointer (HWND_DESKTOP, WinQuerySysPointer(HWND_DESKTOP,

SPTR_WAIT, FALSE));

dwReturn = mciSendCommand (wDeviceID, MCI_SET,

MCI_WAIT | MCI_SET_ITEM,

(DWORD) lpmciSetParms, ++mciCall);

if (dwReturn)

{

if (!(mciGetErrorString (dwReturn,

(int *)messagestring, sizeof(messagestring))))

{

MessageBox(hWnd, messagestring,

NULL, MB_OK|MB_ERROR);

}

else

{

MessageBox (hWnd,

"Another telephony application is in use",

NULL, MB_OK);

}

return FALSE;

}

return TRUE;

This concludes the steps required to properly open and initialize the Mwave FAX device driver. We can now proceed with other driver operations, such as sending and receiving FAX Document Files.

### Sending a Fax - Windows

Sending a fax using the FAX API and the Mwave Fax device driver requires the following steps:

Step 1.	Inform the Fax driver of the names of the Fax Document File(s) to be sent.

Step 2.	Take the phone off-hook and dial the phone number of the destination fax machine.

Step 3.	Respond to a change (either completion, status change, or error) to the send operation.

The send command is initiated by selecting the Send command from the FAXAPP Options menu. The message procedure for the send command is:

[ fax.c: WndProc() ]

case IDM_SEND:

/*--------------------------------------------------*/

/*  Send a fax.                                     */

/*--------------------------------------------------*/

strcpy(FileName, "*.tif");

lpFileName = (GetFileName(hInst, hWnd, "Send Fax", FileName,

FileNamesz, TIF_FILTERSTRING));

if (lpFileName!=NULL)

{

if (tif = IsTif(lpFileName))

SendFax(hWnd, lpFileName);

else if (bmp = IsBmp(lpFileName))

if (ConvertBMP2TIF(hWnd, lpFileName))

SendFax(hWnd, lpFileName);

else if (!(tif || bmp))

ResourceMessageBox(hWnd, IDS_ERR_FILE_FORMAT, 0, NULL, MB_OK);

}

break;

Please note the following in the message procedure above:

a.	FAXAPP is designed to allow only one file (single or multiple pages) to be sent at a time. This is a limitation of FAXAPP, and not of the FAX API and Mwave Fax device driver, both of which provide support for sending multiple files at the same time.

b.	FAXAPP allows the user to send either TIF files (TIFF Class F format) or BMP files (DIB format). Since the Mwave FAX driver supports only TIF file sending, FAXAPP converts BMP files to TIF format via the ConvertBMP2TIF() function. See "Converting Fax Document Files to/from DIB format" on page 2-31 for more information.

The SendFax() function initiates the three step procedure required to send a fax. Let's examine how each of these steps is implemented.

Step 1.	Inform the Fax driver of the names of the Fax Document File(s) to be sent.

The MCI_SEND command is used to specify the name(s) of the Fax Document File(s) to be sent. The filenames are specified by providing to MCI_SEND a pointer to an array of pointers to strings containing the name of each file to be sent. For example, assume lpSendPtr is the array of pointers to the 'n' number of filename strings. It is initialized as follows:

LPSTR lpSendPtr[n+1];

lpSendPtr[0]   = address of string containing file #1 filename

lpSendPtr[1]   = address of string containing file #2 filename

:

lpSendPtr[n-1] = address of string containing file #n filename

lpSendPtr[n]   = (LPSTR)NULL;

Note that the filename list is terminated by a NULL filename pointer.

In FAXAPP, we declare a two-dimensional array SendBuff to store up to two filenames (although only one is used), and then assign the address of the SendBuff strings to the lpSendPtr array. The filename to send (the address of which is passed as the argument srcFileName to the SendFax() function), is copied into the first element of the SendBuff array (pointed to by lpSendPtr[0]). Finally, the address of the lpSendPtr array is sent to the MCI_SEND command.

[ faxops.c: SendFax() ]

LPSTR   lpSendPtr[10];

char    SendBuff[2][128];

/*------------------------------------------------------------------*/

/*  Send the fax file                                               */

/*------------------------------------------------------------------*/

lpSendPtr[0] = SendBuff[0];

lpSendPtr[1] = SendBuff[1];

lstrcpy( lpSendPtr[0], srcFileName );   // send file name

lpSendPtr[1] = '\0';

mciSendParms.lpstrFilename = (LPSTR)lpSendPtr;

SetCursor(hcWaitCursor);

dwReturn = mciSendCommand( wMwaveFaxID,

MCI_SEND,

MCI_WAIT | MCI_SEND_FILE,

(DWORD)lpmciSendParms );

if( dwReturn )

ResourceMessageBox( hWnd, IDS_ERR_SEND_FILE, (UINT)dwReturn, NULL, MB_OK );

else ...

The MCI_SEND command causes the Mwave Fax device driver to be configured for a send.  Once the MCI_SEND command completes, the driver has prepared the Fax Document Files for transmission to a remote fax machine.

Step 2.	Take the phone off-hook and dial the phone number of the destination fax machine.

The next step is to dial and connect to the remote fax machine. This is done via the MCI_DIAL command.

[ faxops.c: SendFax() ]

/*--------------------------------------------------------------*/

/*  Prompt user for the phone number                            */

/*--------------------------------------------------------------*/

lpfnGetNbr = (DLGPROC)MakeProcInstance((FARPROC)GetNbr_DlgProc, hInst);

if( DialogBox(hInst, "phonenumdlg", hWnd, lpfnGetNbr) )

{

/*----------------------------------------------------------*/

/*  Dial the phone number                                   */

/*----------------------------------------------------------*/

mciDialParms.lpstrDialString = (LPSTR)PhoneNumber;

mciDialParms.dwCallback = (DWORD)hWnd;

SetCursor(hcWaitCursor);

uAppState = STATE_DIALING;

mciSendCommand( wMwaveFaxID,

MCI_DIAL,

MCI_NOTIFY | MCI_DIAL_STRING | MCI_DIAL_MONITOR | MCI_DIAL_VERIFY,

(DWORD)lpmciDialParms );

}

In FAXAPP, the user is prompted to enter the phone number of the destination fax machine. The phone number is then supplied to the MCI_DIAL command.

Note that MCI_DIAL is called using the MCI_NOTIFY flag instead of the MCI_WAIT flag. This was done for two reasons. The first is that the dial and connect operation could be a lengthy one, and we do not want to tie up the Microsoft Windows system waiting for this operation to complete. Secondly, it enables our message procedure to track the machine state via the MM_MCINOTIFY message. Note that the FAXAPP state is set to STATE_DIALING prior to calling MCI_DIAL. A transition from the dialing state to the send fax data state is handled by the message procedure.

Hint:  In Microsoft Windows, it is best to use MCI_WAIT with MCI commands when debugging your code.  Using MCI_WAIT allows the application to get more descriptive error messages.

Also note that the MCI_DIAL_MONITOR flag is specified in the MCI_DIAL command. This allows the user to monitor the connection negotiation and call progress via speakers or headphones attached to the Mwave board's audio output connector.

After dialing and connecting to the destination fax machine, the Mwave Fax device driver begins sending the fax data to the destination fax machine. At the same time, MCI sends a MM_MCINOTIFY message (since MCI_DIAL was called with the MCI_NOTIFY flag specified) to indicate either successful completion or failure of the MCI_DIAL command. Our message procedure (see below) responds to a successful connection by changing the machine state to STATE_SENDING_FAX. If a dial or connection failure occurred, FAXAPP displays an error message, hangs up the line, and resets the machine state to STATE_IDLE.

[ fax.c: WndProc() ]

case MM_MCINOTIFY:

switch(wParam)

{

case MCI_NOTIFY_ABORTED:

case MCI_NOTIFY_FAILURE:

if( uAppState == STATE_DIALING )

idResource = IDS_ERR_DIALING;

else if( uAppState == STATE_RECEIVE_SETUP )

idResource = IDS_ERR_RECEIVE;

else

idResource = IDS_ERR_UNKNOWN_STATE;

ResourceMessageBox(hWnd, idResource, 0, "MM_MCINOTIFY", MB_OK);

SetOnHook();

uAppState = STATE_IDLE;

break;

case MCI_NOTIFY_SUCCESSFUL:

if( uAppState == STATE_DIALING )

{

uAppState = STATE_SENDING_FAX;

ResourceMessageBox(hWnd, IDS_MSG_SENDING_FILE, 0, szAppName, 				MB_OK);

}

else if( uAppState == STATE_RECEIVE_SETUP )

uAppState = STATE_RECEIVING_FAX;

break;

default:

break;

}

break;

Step 3.	Respond to a change (either completion, status change, or error) to the send operation.

After completing a successful connection, the Mwave Fax device driver sends the fax data to the destination fax machine (in the background) without requiring any support from the application. Upon completion, either successful or due to an error, the driver sends a MM_MCIEVENT event message of type PHONE_EVENT_CALL_TERMINATED to our application.

This message is processed by our event handling procedure as follows:

[ fax.c: WndProc() ]

if( Message == uMCIMessage )

{

LPMCI_EVENT_PARMS lpMciEventParms = (LPMCI_EVENT_PARMS)lParam;

/*------------------------------------------------------*/

/*  A MM_MCIEVENT message was issued by the Mwave FAX   */

/*  driver.                                             */

/*------------------------------------------------------*/

switch(wParam)

{

case PHONE_EVENT_CALL_TERMINATED:

/*----------------------------------------------*/

/*  Call was terminated.  Check termination     */

/*  code (in dwEventData) for cause.            */

/*----------------------------------------------*/

switch( lpMciEventParms->dwEventData )

{

case TERMINATION_NORMAL:

if( uAppState == STATE_SENDING_FAX )

idResource = IDS_MSG_SEND_OK;

else if( uAppState == STATE_RECEIVING_FAX )

idResource = IDS_MSG_RECEIVE_OK;

else

idResource = IDS_ERR_UNKNOWN_STATE;

break;

case TERMINATION_UNEXPECTED:

case TERMINATION_ERROR_XMIT:

case TERMINATION_ERROR_RECV:

if( uAppState == STATE_SENDING_FAX )

idResource = IDS_ERR_SEND_FAIL;

else if( uAppState == STATE_RECEIVING_FAX )

idResource = IDS_ERR_RECEIVE_FAIL;

else

idResource = IDS_ERR_UNKNOWN_STATE;

break;

default:

idResource = 0;

break;

}

if( idResource )

ResourceMessageBox( hWnd, idResource, 0, "MM_MCIEVENT", MB_OK );

/*----------------------------------------------*/

/*  Hang up the phone                           */

/*----------------------------------------------*/

SetOnHook();

uAppState = STATE_IDLE;

break;

Upon receipt of the PHONE_EVENT_CALL_TERMINATED case of the MM_MCIEVENT message, FAXAPP displays either a success or failure message (using the context of the current machine state), hangs up the phone, and resets the machine state back to STATE_IDLE.

You might want to respond to other types of MM_MCIEVENT messages (for example, PHONE_EVENT_FAX_PAGE_STATUS) to provide real-time status information to the user during the send operation.

### Sending a Fax - OS/2

Sending a fax using the FAX API and the Mwave FAX device driver requires the following steps:

Step 1.	Inform the FAX driver of the names of the FAX Document File(s) to be sent.

Step 2.	Take the phone off-hook and dial the phone number of the destination fax machine.

Step 3.	Respond to a change (either completion, status change, or error) to the send operation.

The send command is initiated by selecting the Send command from the FAXAPP Options menu. The message procedure for the send command is:

fax.c: WndProc

case IDM_SEND:

/*---------------------------------*/

/*  Send a fax.                    */

/*---------------------------------*/

strcpy(FileName, "*.tif");

lpFileName = (GetFileName("Send Fax",

(LPSTR) FileName));

if (strlen((char *)lpFileName) == 0)

break;

if (IS_TIF(lpFileName))

{

SendFax(hWnd, lpFileName, "");

break;

}

if (IS_BMP(lpFileName))

if (ConvertBMP2TIF(hWnd, wDeviceID,

lpFileName))

{

SendFax(hWnd, lpFileName, "");

break;

}

MessageBox(hWnd, "Unsupported File Format",

NULL, MB_OK);

break;

Please note the following in the message procedure above:

·	FAXAPP is designed to allow only one file (single or multiple pages) to be sent at a time. This is a limitation of FAXAPP, and not of the FAX API and Mwave FAX device driver, both of which provide support for sending multiple files at the same time.

·	FAXAPP allows the user to send either TIF files (TIFF Class F format) or BMP files (DIB format). Since the Mwave FAX driver supports only TIF file sending, FAXAPP converts BMP files to TIF format via the ConvertBMP2TIF function. See Section Converting Fax Document Files to/from DIB format on page 3-31 for more information.

The SendFax function initiates the three step procedure required to send a fax. Let's examine how each of these steps is implemented.

Step 1.	Inform the FAX driver of the names of the FAX Document File(s) to be sent.

The MCI_SEND command is used to specify the name(s) of the FAX Document File(s) to be sent. The filenames are specified by providing to MCI_SEND a pointer to an array of pointers to strings containing the name of each file to be sent. For example, assume lpSendPtr is the array of pointers to the 'n' number of filename strings. It is initialized as follows:

LPSTR lpSendPtr[n+1];

lpSendPtr[0]   = address of string containing file #1

lpSendPtr[1]   = address of string containing file #2

:

lpSendPtr[n-1] = address of string containing file #n 
lpSendPtr[n]   = (LPSTR)NULL;

Note that the filename list is terminated by a NULL filename pointer.

In FAXAPP, we declare a two-dimensional array SendBuff to store up to two filenames (although only one is used), and then assign the address of the SendBuff strings to the lpSendPtr array. The filename to send (the address of which is passed as the argument srcFileName to the SendFax function), is copied into the first element of the SendBuff array (pointed to by lpSendPtr[0]). Finally, the address of the lpSendPtr array is sent to the MCI_SEND command.

faxops.c: SendFax

void SendFax(HWND hWnd, LPSTR srcFileName, char phonenbr[25])

{

char * lpSendPtr[10];

char SendBuff[2][256];

/* 1.  Send the fax file */

mciSendParms.dwCallback = hWnd;

lpSendPtr[0] = SendBuff[0];

lpSendPtr[1] = SendBuff[1];

strcpy((CHAR *)lpSendPtr[0],(CHAR *)srcFileName );

lpSendPtr[1] = '\0';

mciSendParms.lpstrFilename = (char *)lpSendPtr;

WinSetPointer(HWND_DESKTOP,

WinQuerySysPointer(HWND_DESKTOP,

SPTR_WAIT,FALSE));

dwReturn = mciSendCommand(wDeviceID, MCI_SEND,

MCI_WAIT | MCI_SEND_FILE,

(DWORD) lpmciSendParms,

mciCall++ );

if (dwReturn)

{

/* Error, unable to send file  */

MessageBox(hWnd, "Unable to send file", NULL, MB_OK);

}

else . . .

The MCI_SEND command causes the Mwave Fax device driver to be configured for a send.  Once the MCI_SEND command completes, the driver has prepared the Fax Document Files for transmission to a remote fax machine.

Step 2.	Take the phone off-hook and dial the phone number of the destination fax machine.

The next step is to dial and connect to the remote fax machine. This is done via the MCI_DIAL command.

faxops.c: SendFax

if (WinDlgBox(HWND_DESKTOP, hWnd,

GetNbr_DlgProc, 0, PHONENUMDLG, NULL))

{

mciDialParms.lpstrDialString = PhoneNumber;

mciDialParms.dwCallback = hWnd;

WinSetPointer(HWND_DESKTOP,

WinQuerySysPointer(HWND_DESKTOP,

SPTR_WAIT, FALSE));

ulRC = mciSendCommand(wDeviceID, MCI_DIAL,

MCI_WAIT |

MCI_'DIAL_STRING |

MCI_DIAL_VERIFY |

MCI_DIAL_MONITOR_HANDSHAKING_ONLY,

(DWORD) lpmciDialParms,

mciCall++);

}

In FAXAPP, the user is prompted to enter the phone number of the destination fax machine. The phone number is then supplied to the MCI_DIAL command.

Also note that the MCI_DIAL_MONITOR_HANDSHAKING_ONLY flag is specified in the MCI_DIAL command. This allows the user to monitor the connection negotiation via speakers or headphones attached to the Mwave board's audio output connector.

After dialing and connecting to the destination fax machine, the Mwave FAX device driver begins sending the fax data to the destination fax machine.  If a dial or connection failure occurred, FAXAPP displays an error message, and hangs up the line.

faxevnts.c: ProcessEvent

#include "mciftdd.h"

#include <stdio.h>

#include <string.h>

#include "fax.h"

extern int mciCall;

void ProcessEvent(HWND hWnd, WORD wParam,

LPMCI_EVENT_PARMS lParam)

{

char   buf[256];

switch(wParam)

{

case PHONE_EVENT_CALL_FAX:

/* incoming call--  receive fax and notify user */

++NumFax;

ReceiveFax(hWnd);

break;

case PHONE_EVENT_CALL_TERMINATED:

/* notify user, enter into log */

switch (lParam->dwEventData)

{

case TERMINATION_NORMAL:

sprintf(buf, "Call completed normally");

break;

case TERMINATION_UNEXPECTED:

sprintf(buf, "Call terminated unexpectedly");

break;

case TERMINATION_ERROR_XMIT:

sprintf(buf,"Callterminated:transmit error");

break;

case TERMINATION_ERROR_RECV:

sprintf(buf,"Callterminated:receive error");

case TERMINATION_REQUESTED:

sprintf(buf,"Callterminated:type requested");

break;

default:

break;

} /* switch */

/* hang up the phone    */

MessageBox(hWnd, buf, "Fax Informations", MB_OK);

SetOnHook(hWnd);

break;

case PHONE_EVENT_CALLER_ID:

case PHONE_EVENT_FAX_HEADER:

case PHONE_EVENT_FAX_PAGE_COMPLETE:

case PHONE_EVENT_FAX_PAGE_STATUS:

case PHONE_EVENT_FAX_CONNECT:

case PHONE_EVENT_LINE:

case PHONE_EVENT_HANDSET:

case PHONE_EVENT_CALL_PROGRESS:

break;

default:

break;

}

}

Step 3.	Respond to a change (either completion, status change, or error) to the send operation.

After completing a successful connection, the Mwave FAX device driver sends the fax data to the destination fax machine without requiring any support from the application. Upon completion, either successful or due to an error, the driver sends a MM_MCIEVENT event message of type PHONE_EVENT_CALL_TERMINATED to our application. This message is processed by our event handling procedure as follows:

faxevnts.c: ProcessEvent

#include "mciftdd.h"

#include <stdio.h>

#include <string.h>

#include "fax.h"

extern int mciCall;

void ProcessEvent(HWND hWnd, WORD wParam, LPMCI_EVENT_PARMS lParam)

{

char   buf[256];

switch(wParam)

{

case PHONE_EVENT_CALL_FAX:

/* incoming call, receive fax and notify user */

++NumFax;

ReceiveFax(hWnd);

break;

case PHONE_EVENT_CALL_TERMINATED:

/* notify user, enter into log  */

switch (lParam->dwEventData)

{

case TERMINATION_NORMAL:

sprintf(buf, "Call completed normally");

break;

case TERMINATION_UNEXPECTED:

sprintf(buf, "Call terminated unexpectedly");

break;

case TERMINATION_ERROR_XMIT:

sprintfbuf,"Call terminated: transmit error");

break;

case TERMINATION_ERROR_RECV:

sprintf(buf,"Call terminated: receive error");

case TERMINATION_REQUESTED:

sprintfbuf,"Call terminated: type requested");

break;

default:

break;

} /* switch  */

/*  Hang up the phone  */

MessageBox(hWnd, buf, "Fax Informations", MB_OK);

SetOnHook(hWnd);

break;

case PHONE_EVENT_CALLER_ID:

case PHONE_EVENT_FAX_HEADER:

case PHONE_EVENT_FAX_PAGE_COMPLETE:

case PHONE_EVENT_FAX_PAGE_STATUS:

case PHONE_EVENT_FAX_CONNECT:

case PHONE_EVENT_LINE:

case PHONE_EVENT_HANDSET:

case PHONE_EVENT_CALL_PROGRESS:

break;

default:

break;

}

}

Upon receipt of PHONE_EVENT_CALL_TERMINATED , FAXAPP displays either a success or failure message  and hangs up the phone.

You might want to respond to other types of MM_MCIEVENT messages (for example, PHONE_EVENT_FAX_PAGE_STATUS) to provide real-time status information to the user during the send operation.

This concludes the steps required to send fax data.

### Receiving a Fax - Windows

Receiving a fax using the FAX API and the Mwave Fax device driver requires the following steps:

Step 1.	Respond to the MM_MCIEVENT message PHONE_EVENT_CALL_FAX

Step 2.	Initiate the receive operation in the Mwave Fax device driver.

Step 3.	Respond to a change (either completion, status change, or error) to the receive operation.

Let's look at how FAXAPP implements each of these steps.

Step 1.	Respond to the MM_MCIEVENT message PHONE_EVENT_CALL_FAX

The Mwave Fax device driver notifies FAXAPP that an incoming fax call has been detected by sending the PHONE_EVENT_CALL_FAX type of MM_MCIEVENT message. This message is processed by our event handling procedure as follows:

[ fax.c: WndProc() ]

if( Message == uMCIMessage )

{

LPMCI_EVENT_PARMS lpMciEventParms = (LPMCI_EVENT_PARMS)lParam;

/*------------------------------------------------------*/

/*  A MM_MCIEVENT message was issued by the Mwave FAX   */

/*  driver.                                             */

/*------------------------------------------------------*/

switch(wParam)

{

case PHONE_EVENT_CALL_FAX:

/*----------------------------------------------*/

/*  Incoming call. Receive fax and notify user. */

/*----------------------------------------------*/

ReceiveFax(hWnd);

NumFax++;   // counter for received fax's filenames

break;

On receipt of this message, the application must initiate the receive operation of the Mwave Fax device driver as soon as possible. FAXAPP calls the ReceiveFax() function which performs this operation.

Step 2.	Initiate the receive operation in the Mwave Fax device driver.

The Mwave Fax device driver begins to receive incoming fax data after the application calls the MCI_RECEIVE command. FAXAPP uses file names of the form "FAX??.TIF" to store received fax data. See"" on page 4-8 for more information on this file naming convention.

[ faxops.c: ReceiveFax() ]

void ReceiveFax( HWND hWnd )

{

char    buffer[32];

wsprintf( (LPSTR)buffer, "Fax%d.tif", NumFax);

mciReceiveParms.lpstrFilename = (LPSTR)buffer;

mciReceiveParms.dwCallback = (DWORD)hWnd;

SetCursor(hcWaitCursor);

uAppState = STATE_RECEIVE_SETUP;

mciSendCommand( wMwaveFaxID,

MCI_RECEIVE,

MCI_NOTIFY | MCI_RECEIVE_FILE,

(DWORD)lpmciReceiveParms );

MessageBox(hWnd, (LPSTR)buffer, "RECEIVING FAX FILE", MB_OK);

}

Note that MCI_RECEIVE is called using the MCI_NOTIFY flag instead of the MCI_WAIT flag. This was done for the sole purpose of allowing our message procedure to track the machine state via the MM_MCINOTIFY message. Note that the FAXAPP state is set to STATE_RECEIVE_SETUP prior to calling MCI_RECEIVE. A transition from the receive setup state to the receive fax data state is handled by the message procedure.

After initiating the receive operation, the Mwave Fax device driver begins to receive the fax data from the remote fax machine. At the same time, MCI sends a MM_MCINOTIFY message (since MCI_RECEIVE was called with the MCI_NOTIFY flag specified) to indicate either successful completion or failure of the MCI_RECEIVE command. Our message procedure (see below) responds to a successful receive setup by changing the machine state to STATE_RECEIVING_FAX. If a failure occurred during receive setup, FAXAPP displays an error, hangs up the line, and resets the machine state to STATE_IDLE.

[ fax.c: WndProc() ]

case MM_MCINOTIFY:

switch(wParam)

{

case MCI_NOTIFY_ABORTED:

case MCI_NOTIFY_FAILURE:

if( uAppState == STATE_DIALING )

idResource = IDS_ERR_DIALING;

else if( uAppState == STATE_RECEIVE_SETUP )

idResource = IDS_ERR_RECEIVE;

else

idResource = IDS_ERR_UNKNOWN_STATE;

ResourceMessageBox(hWnd, idResource, 0, "MM_MCINOTIFY", MB_OK);

SetOnHook();

uAppState = STATE_IDLE;

break;

case MCI_NOTIFY_SUCCESSFUL:

if( uAppState == STATE_DIALING )

{

uAppState = STATE_SENDING_FAX;

ResourceMessageBox(hWnd, IDS_MSG_SENDING_FILE, 0, szAppName, 				MB_OK);

} else if( uAppState == STATE_RECEIVE_SETUP )

uAppState = STATE_RECEIVING_FAX;

break;

default:

break;

}

break;

Step 3.	Respond to a change (either completion, status change, or error) to the receive operation.

After initiating the receive operation, the Mwave Fax device driver receives incoming fax data from the remote fax machine (in the background) without requiring any support from the application.

Upon receive completion, either successful or due to an error, the driver sends a MM_MCIEVENT event message of type PHONE_EVENT_CALL_TERMINATED to our application. This message is processed by our event handling procedure as follows:

[ fax.c: WndProc() ]

if( Message == uMCIMessage )

{

LPMCI_EVENT_PARMS lpMciEventParms = (LPMCI_EVENT_PARMS)lParam;

/*------------------------------------------------------*/

/*  A MM_MCIEVENT message was issued by the Mwave FAX   */

/*  driver.                                             */

/*------------------------------------------------------*/

switch(wParam)

{

case PHONE_EVENT_CALL_TERMINATED:

/*----------------------------------------------*/

/*  Call was terminated.  Check termination     */

/*  code (in dwEventData) for cause.            */

/*----------------------------------------------*/

switch( lpMciEventParms->dwEventData )

{

case TERMINATION_NORMAL:

if( uAppState == STATE_SENDING_FAX )

idResource = IDS_MSG_SEND_OK;

else if( uAppState == STATE_RECEIVING_FAX )

idResource = IDS_MSG_RECEIVE_OK;

else

idResource = IDS_ERR_UNKNOWN_STATE;

break;

case TERMINATION_UNEXPECTED:

case TERMINATION_ERROR_XMIT:

case TERMINATION_ERROR_RECV:

if( uAppState == STATE_SENDING_FAX )

idResource = IDS_ERR_SEND_FAIL;

else if( uAppState == STATE_RECEIVING_FAX )

idResource = IDS_ERR_RECEIVE_FAIL;

else

idResource = IDS_ERR_UNKNOWN_STATE;

break;

default:

idResource = 0;

break;

}

if( idResource )

ResourceMessageBox( hWnd, idResource, 0, "MM_MCIEVENT", MB_OK );

/*----------------------------------------------*/

/*  Hang up the phone                           */

/*----------------------------------------------*/

SetOnHook();

uAppState = STATE_IDLE;

break;

Upon receipt of the PHONE_EVENT_CALL_TERMINATED case of the MM_MCIEVENT message, FAXAPP displays either a success or failure message (using the context of the current machine state), hangs up the phone, and resets the machine state back to STATE_IDLE.

### Receiving a FAX - OS/2

Receiving a fax using the FAX API and the Mwave FAX device driver requires the following steps:

Step 1.	Respond to the MM_MCIEVENT message PHONE_EVENT_CALL_FAX

Step 2.	Initiate the receive operation in the Mwave FAX device driver.

Step 3.	Respond to a change (either completion, status change, or error) to the receive operation.

Let's look at how FAXAPP implements each of these steps.

Step 1.	Respond to the MM_MCIEVENT message PHONE_EVENT_CALL_FAX

The Mwave FAX device driver notifies FAXAPP that an incoming fax call has been detected by sending the PHONE_EVENT_CALL_FAX type of MM_MCIEVENT message. This message is processed by our event handling procedure as follows:

faxevnts.c: ProcessEvent

void ProcessEvent(HWND hWnd, WORD wParam,

LPMCI_EVENT_PARMS lParam)

{

char   buf[256];

switch(wParam)

{

case PHONE_EVENT_CALL_FAX:

/* incoming call - receive fax and notify user */

++NumFax;

ReceiveFax(hWnd);

break;

On receipt of this message, the application must initiate the receive operation of the Mwave FAX device driver as soon as possible. FAXAPP calls the ReceiveFax function which performs this operation.

Step 2.	Initiate the receive operation in the Mwave FAX device driver.

The Mwave FAX device driver begins to receive incoming fax data after the application calls the MCI_RECEIVE command. FAXAPP uses file names of the form "FAX??.TIF" to store received fax data..

faxops.c: ReceiveFax

void ReceiveFax(HWND hWnd)

{

char buff[256];

LPSTR lpBuff = (LPSTR) buff;

sprintf(buff,"c:\\Fax%d.tif",NumFax);

mciReceiveParms.lpstrFilename = (char *) lpBuff;

mciReceiveParms.dwCallback = (DWORD)hWnd;

WinSetPointer(HWND_DESKTOP,

WinQuerySysPointer(HWND_DESKTOP,

SPTR_WAIT, FALSE));

ulRC = mciSendCommand (wDeviceID, MCI_RECEIVE,

MCI_WAIT | MCI_RECEIVE_FILE,

(DWORD) lpmciReceiveParms,

mciCall++);

MessageBox(hWnd, buff, "RECEIVING FAX FILE", MB_OK);

} /* End ReceiveFax */

After initiating the receive operation, the Mwave FAX device driver begins to receive the fax data from the remote fax machine.  If a failure occurred during receive setup, FAXAPP displays an error and hangs up the line.

Step 3.	Respond to a change (either completion, status change, or error) to the receive operation.

After initiating the receive operation, the Mwave FAX device driver receives incoming fax data from the remote fax machine (in the background) without requiring any support from the application.

Upon receive completion, either successful or due to an error, the driver sends a MM_MCIEVENT event message of type PHONE_EVENT_CALL_TERMINATED to our application. This message is processed by our event handling procedure as follows:

faxevnts.c:

void ProcessEvent(HWND hWnd, WORD wParam, LPMCI_EVENT_PARMS lParam)

{

char   buf[256];

switch(wParam)

{

case PHONE_EVENT_CALL_FAX:

/* incoming call time to receive fax and notify user */

++NumFax;

ReceiveFax(hWnd);

break;

case PHONE_EVENT_CALL_TERMINATED:

/* notify user, enter into log                      */

switch (lParam->dwEventData)

{

case TERMINATION_NORMAL:

sprintf(buf, "Call completed normally");

break;

case TERMINATION_UNEXPECTED:

sprintf(buf, "Call terminated unexpectedly");

break;

case TERMINATION_ERROR_XMIT:

sprintf(buf, "Call terminated: transmit error");

break;

case TERMINATION_ERROR_RECV:

sprintf(buf,"Call terminated: receive error");

case TERMINATION_REQUESTED:

sprintf(buf,"Call terminated: type requested");

break;

default:

break;

} /* switch                                     */

/*  Hang up the phone                               */

MessageBox(hWnd, buf, "Fax Informations", MB_OK);

SetOnHook(hWnd);

break;

case PHONE_EVENT_CALLER_ID:

case PHONE_EVENT_FAX_HEADER:

case PHONE_EVENT_FAX_PAGE_COMPLETE:

case PHONE_EVENT_FAX_PAGE_STATUS:

case PHONE_EVENT_FAX_CONNECT:

case PHONE_EVENT_LINE:

case PHONE_EVENT_HANDSET:

case PHONE_EVENT_CALL_PROGRESS:

break;

default:

break;

}

}

Upon receipt of the PHONE_EVENT_CALL_TERMINATED case of the MM_MCIEVENT message, FAXAPP displays either a success or failure message and hangs up the phone,

This concludes the steps required to receive fax data.

### Converting Fax Document Files to/from DIB format

As mentioned previously, the FAX API specifies several different Fax Document File formats which might be supported by a compliant fax driver. In the case of the Mwave Fax device driver, the supported Fax Document File format is TIFF Class F. However, in the Microsoft Windows environment, it is much more convenient to view, print, and edit image data in Device Independent Bitmap (DIB) format. For this reason, the FAX API supports the command MCI_CONVERT which enables conversion from Fax Document File format to DIB format and visa versa.

FAXAPP uses the MCI_CONVERT command to enable sending of DIB files and viewing of TIFF Class F files by first converting a given file into the correct format. To illustrate the use of MCI_CONVERT, we'll use the example of sending a BMP file (a type of DIB file) to a destination fax machine. See "Sending a Fax" on page 31  for an overview of the fax send operation. Recall that the Mwave Fax device driver can only send files in TIFF Class F format, so we must first convert the BMP file into TIFF Class F format.

FAXAPP uses the ConvertBMP2TIF() function to convert a BMP format source file to a TIFF Class F format destination file using the MCI_CONVERT command as follows:

[ faxops.c: ConvertBMP2TIF() ] - Windows

int ConvertBMP2TIF(HWND hWnd, LPSTR SrcFileName)

{

DLGPROC lpfnGetDest;

char    buffer[32];

wFileConvertType = BMP_TO_TIF;

/*------------------------------------------------------------------*/

/*  Prompt user for destination file name                           */

/*------------------------------------------------------------------*/

lpfnGetDest = (DLGPROC)MakeProcInstance((FARPROC)GetDest_DlgProc, hInst);

if (!DialogBox(hInst, "DestinationFile", hWnd, lpfnGetDest))

{

FreeProcInstance(lpfnGetDest);

return( FALSE );

}

lstrcpy( (LPSTR)buffer, (LPCSTR)DestFile );

mciConvertParms.lpstrDestFilename = (LPSTR)buffer;

mciConvertParms.dwDestFormat = MCI_FAX_CONVERT_FMT_DEVFAX;

mciConvertParms.lpstrSrcFilename = SrcFileName;

SetCursor(hcWaitCursor);

dwReturn = mciSendCommand(  wMwaveFaxID,

MCI_CONVERT,

MCI_WAIT | MCI_CONVERT_SOURCE_FILE |

MCI_CONVERT_CREATE |

MCI_CONVERT_DESTINATION_FILE |

MCI_CONVERT_DESTINATION_FORMAT,

(DWORD) lpmciConvertParms);

:

faxops.c: ConvertBMP2TIF - OS/2

int ConvertBMP2TIF(HWND hWnd, WORD wDeviceID,

LPSTR SrcFileName)

{

static HANDLE hBuff;

LPSTR  lpBuff;

char buf[255];

char messagestring[255];

/* 1.  Get Destination File Name */

hBuff = (HANDLE) malloc(32);

lpBuff = (LPSTR) hBuff;

if (!WinDlgBox(HWND_DESKTOP, hWnd,

GetDest_DlgProc, 0, DestinationFile, NULL))

{

return FALSE;

}

strcpy((CHAR *)lpBuff, DestFile);

mciConvertParms.dwCallback = (DWORD)hWnd;

mciConvertParms.lpstrDestFilename = (char *) lpBuff;

mciConvertParms.dwDestFormat =

MCI_FAX_CONVERT_FMT_DEVFAX;

mciConvertParms.lpstrSrcFilename = (char *) SrcFileName;

WinSetPointer(HWND_DESKTOP,

WinQuerySysPointer(HWND_DESKTOP,

SPTR_WAIT, FALSE));

ulRC = mciSendCommand(wDeviceID, MCI_CONVERT,

MCI_WAIT | MCI_CONVERT_SOURCE_FILE |

MCI_CONVERT_CREATE |

MCI_CONVERT_DESTINATION_FILE |

MCI_CONVERT_DESTINATION_FORMAT,

(DWORD) lpmciConvertParms, mciCall++);

free((void *) hBuff);

if (ulRC)

{

/* Error, unable to convert file  */

mciGetErrorString(ulRC, (int *)messagestring,

sizeof(messagestring));

sprintf(buf,

"ERROR: %d:  Unable to convert image file. %s",

(LOWORD(ulRC)),        messagestring );

MessageBox(hWnd, buf, NULL, MB_OK);

return FALSE;

}

strcpy((CHAR *)lpFileName,(CHAR *)DestFile);

MessageBox(hWnd, "Bitmap file converted to TIFF",

szAppName, MB_OK);

return TRUE;

}  /* end ConvertBMP2TIF */

Note that MCI_CONVERT is called with the MCI_CONVERT_CREATE flag. This flag specifies that the destination file should be created if it doesn't exist. If the destination file does exist, its contents are destroyed and overwritten with the converted data (the same effect as using the MCI_CONVERT_OVERWRITE flag).

Alternatively, converted data can be inserted into (or appended onto) an existing destination file. Set the dwDestFrom field of the MCI_CONVERT_PARMS structure to the document page number (starting at page zero) where you want the converted data to be written into the destination file. You'll also need to include the MCI_CONVERT_DESTINATION_FROM flag and remove the MCI_CONVERT_CREATE flag in the MCI_CONVERT call.

### Closing the Mwave Fax Device Driver

Before exiting your application, you should always close the Mwave Fax device driver. Closing the driver frees memory, processor, and connection resources on the Mwave board, making them available for use by other Mwave applications.

When closing the FAX driver, always assign the window procedure handle to mciGenericParms.dwCallback prior to issuing the MCI_CLOSE.  Failure to do so will result in erratic behavior.  In FAXAPP, CloseFax looks like this:

[ faxops.c: CloseFax() ] - Windows

MCI_GENERIC_PARMS   mciGenericParms;

mciGenericParms.dwCallBack=(DWORD)hWnd;

mciSendCommand( wMwaveFaxID, 		// device ID

MCI_CLOSE, 		// MCI command

MCI_WAIT, 		// flags

(DWORD)(LPVOID) &mciGenericParms);

faxops.c: CloseFax - OS/2

void CloseFax(HWND hWnd )

{

MCI_GENERIC_PARMS   mciGenericParms;

mciGenericParms.dwCallback = (DWORD)hWnd;

mciSendCommand( wDeviceID,

MCI_CLOSE,

MCI_WAIT,

(DWORD)&mciGenericParms, mciCall++);

}

## Summary

FAXAPP provides a simple example of using the basic send and receive capabilities of the FAX API and the Mwave Fax device driver. Using the programming techniques outlined in this chapter, you should be able to add additional capabilities, such as providing real-time send and receive status information, into your own Mwave fax application. Be sure to check the FAX API Reference for a complete description of the available capabilities.