September 1992

FTP PC/TCP Installation

for

PE-EPP Ethernet Pocket LAN Adapter

==================================

The following files will be used in this installation:

Files	      Contents

-----------   ---------------------------------

EPPPD.SYS     PE-EPP series Driver Program

EPPPD.COM     PE-EPP series Driver Program

The PE-EPP packet driver programs above let you run FTP's PC/TCP

with the PE-EPP series Ethernet Card.  Two methods can be used:

METHOD 1:

If you use the EPPPD.SYS program, the following statement must

be included in your CONFIG.SYS file:

DEVICE = EPPPD.SYS <packet_int_no>

This statement may be placed anywhere in the CONFIG.SYS file.

METHOD 2:

If you use the EPPPD.COM program, follow the procedure below:

1. Execute the EPPPD.COM as follows:

>EPPPD <packet_int_no>

2. Execute the ETHDRV.EXE device driver program (provided by

FTP).

You are now ready to use FTP's PC/TCP.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.