How to Install the Netware OS/2 Client Driver

==================================================

ETHER-32V/P also supports the Novell OS/2 ODI client configuration for the

family Adapter cards.To install the OS/2 ODI driver, the Novell OS/2

Requester should be used.

Load Netware OS/2 ODI driver from the OS/2 CONFIG.SYS  file in the

sequence below.For more information on installing the OS/2 ODI driver,

refer to the NetWare Requester for OS/2 installation documentation.

After installing the OS/2 Requester, follow the steps below:

1.Insert the ETHER-32V/P driver diskette into the system's floppy drive.

2.Using the Netware OS/2 disks provided by Novell, install the Netware

OS/2 requester files on the target hard disk and the file server.

Follow the instructions on screen to configure the Netware OS/2

requester for a Netware-supported LAN adapter board.

3.Copy the OS/2 ODI driver,PCNTNW.OS2, from the ETHER-32V/P Driver Diskette

to the C:\NETWARE directory of the boot drive through path:

A:\NETWARE\WKS\ODI.OS2

4.Edit the CONFIG.SYS file and replace the Netware-supported LAN adapter

driver with the name of the OS/2 ODI driver, PCNTNW.SYS.  For example:

DEVICE=C:\NETWARE\PCNTNW.OS2

However,if the Novell OS/2 Requester is used to install the driver,

the requester will automatically make modifications to CONFIG.SYS.

The following is an example of NetWare 3.x of the CONFIG.SYS load

sequence for OS/2 ODI driver support for the IPX protocol stack.

REM ---Netware Requester Statements Begin ---

DEVICE=C:\NETWARE\LSL.SYS

RUN=C:\NETWARE\DDAEMON.EXE

DEVICE=C:\NETWARE\PCNTNW.OS2

(Located in OS2ODI directory on Driver Diskette)

DEVICE=C:\NETWARE\IPX.SYS

DEVICE=C:\NETWARE\NWREQ.SYS

IFS=DEVICE=C:\NETWARE\NWIFS.IFS

RUN=DEVICE=C:\NETWARE\NWDAEMON.EXE

REM ---Netware Requester Statements End ---

The default settings may be modified when the driver loads by creating

the file NET.CFG. Also, the OS/2 ODI driver may also be installed to

support multiple protocol stacks and Ethernet frame types. refer to

the Netware Requester for OS/2 installation documentation for more

information on configuring the NET.CFG file.