README file

Product Name:	   ETHER-32V/P family NDIS driver

for IBM OS/2 2.x.

Release Date:	   DEC. 01, 1994

Product Version:   2.00

-----------------------------------------------------------------------------

TABLE OF CONTENTS

This file contains information about ETHER-32V/P family card

NDIS driver for IBM OS/2 2.x.  It includes the following

topics:

1. WHICH ETHERNET ADAPTER DO YOU HAVE?

2. INSTALLING THE DRIVER

3. EDITING PROTOCOL.INI TO CHANGE THE CONFIGURATION

4. DISKETTE INVENTORY

------------------------------------------------------------------------------

1. WHICH ETHERNET ADAPTER DO YOU HAVE?

=======================================

ETHER-32V support VL-BUS system.

ETHER-32P support PCI bus system.

2. INSTALLING THE DRIVER

========================

2.1 Installing the driver for IBM LAN Services 2.0

--------------------------------------------------

To install the ETHER-32V/P driver for IBM LAN Services 2.0 (LAN Requester or

LAN Server), follow these steps:

1  Insert the floppy with ETHER-32V/P files into the floppy drive.

2  Open the LAN Services folder( or run "laps" command from your IBMCOM

subdirectory).

3  Run OS/2 LAN Services Installation/Configuration.

4  When the Basic or Advanced Installation/Configuration dialog box is

displayed, choose Advanced.

5  When the Install Type dialog box is displayed, choose Install or

Configure this workstation.

6  When the Installation and Configuration dialog box is displayed, choose

Configure a component. Then choose OK.

7  When the Configure dialog box is displayed, choose LAN Adapter and

Protocol Support and choose Configure. Then choose OK.

8  At the Options dialog box, choose Copy additional network adapter

drivers. Then choose OK.

9  At the Copy Additional Network Adapter Drivers dialog box, type the

path . Then choose OK.

10  When the Copy Diskette dialog box is displayed, choose OK.

11  When the Options dialog box is displayed, choose Configure Workstation.

Then choose OK.

12  When the Configure Workstation dialog box is displayed, remove the

current configurations from the Current Configuration list box.

13  Choose the Ethernet Adapter from the Network Adapter list box,

by highlighting the entry. Then choose Add to add the Ether-32V/P

PCnet family to Current Configuration list box.

14  Highlight the appropriate protocols in the Protocols list box. Then

choose Add to add them to the Current Configuration list box.

15  Choose the ETHER-32V/P PCnet family driver at the Current Configuration

list box. Then choose Edit. The Parameters for ETHER-32V/P PCnet family

dialog box is displayed.

16  View the default configuration. If the default configuration is

appropriate, choose OK.

If you need to change ETHER-32V/P driver configuration, change the

options in this dialog box.

You have finished installing the ETHER-32V/P driver for IBM LAN Services 2.0.

2.2  Installing the driver for IBM Extended Services 1.0

--------------------------------------------------------

To install the driver for IBM Extended Services 1.0, follow these steps:

1   Insert the floppy with ETHER-32V/P files into the floppy drive.

2  Display the Communication Manager main menu and choose Advanced.

3  Choose Configuration.

4  Choose LAN Adapter and Protocol Support (LAPS).

5  At the Options dialog box, choose Copy additional network adapter

drivers. Then choose OK.

6  At the Copy Additional Network Adapter Drivers dialog box, type the

path (a:). Then choose OK.

7  When the Copy Diskette dialog box is displayed, choose OK.

8  When the Options dialog box is displayed, choose Configure Workstation.

Then choose OK.

9  When the Configure Workstation dialog box is displayed, remove the

current configurations from the Current Configuration list box.

10  Choose the ETHER-32V/P PCnet family Ethernet Adapter from the Network

Adapters list box, by highlighting the entry. Then choose Add to add the

ETHER-32V/P PCnet family to Current Configuration list box.

11  Highlight the appropriate protocols in the Protocols list box. Then

choose Add to add them to the Current Configuration list box.

12  Choose the ETHER-32V/P PCnet family at the Current Configuration list box.

Then choose Edit. The Parameters for ETHER-32V/P PCnet family dialog box

is displayed.

14  View the default configuration. If the default configuration is

appropriate, choose OK.

If you need to change ETHER-32V/P's configuration, change the options in

this dialog box.

You have finished installing the driver for IBM Extended Services 1.0.

3. EDITING PROTOCOL.INI TO CHANGE THE CONFIGURATION

===================================================

To change ETHER-32V/P's configuration, you can edit keyword entries in the

IRMATR_NIF section of PROTOCOL.INI.  If you are using several ETHERNET

adapters, you will have [PCNTND_NIF] and [PCNTND2_NIF] sections. Here's a sample

PCNTND_NIF section in PROTOCOL.INI:

[PCNTND_NIF]

DRIVERNAME=PCNTND$

BUS_TO_SCAN=PCI {PCI1, PCI2, VESA}

RXBUFFERS=8

TXBUFFERS=8

For keywords description, please see Network Driver

Installation Guide.

-----------------------------------------------------------------------

4. DISKETTE INVENTORY

======================

The following files are provided:

File			Description

--------------		--------------------------------------------------

README.DOC		This file.

PCNTND.NIF		The network information file that supports

ETHER-32V/P adapters under OS/2 2.x; Extended Services

1.0 and LAN Services 2.0/3.0.

PCNTND.OS2		The device driver for OS/2 2.x; Extended Services 1.0

and LAN Services 2.0/3.0.