STB MVP DRIVER for MICROSOFT NT 3.1

BETA RELEASE REV (A)

--------------------

DATE: 3/22/94

* Driver is 640x480 256 Colors, Supports 2x1 Mode Only.

* No Paint Style calls are supported yet.

Obvious Problems:

* Block Copy can Block Distortion.

BETA RELEASE REV (B)

--------------------

DATE: 3/28/94

* Driver is 640x480 256 Colors, Supports 2x1 Mode Only.

Obvious Problems:

* Block Moves will occationally cause the moved

window to "Shatter".  - Move block off, then back on screen to fix.

* Wintach - Paint, Word Processing, and Spreadsheet will all Hang !

BETA RELEASE REV (C)

--------------------

DATE: 4/4/94

* Driver is 640x480 256 Colors, Supports 2x1 Mode Only.

Obvious Problems:

* Block Moves will occationally cause the moved

window to "Shatter".  - Move block off, then back on screen to fix.

BETA RELEASE REV (D)

--------------------

DATE: 4/11/94

* Driver is 640x480 256 Colors,

Supports 2x1,1x2,4x1,1x4,2x2 Modes Only.

(Separate DLL's must be copied to VGA256.DLL).

Corrected Issues:

* Analog Clock Now has Hands !

Obvious Problems:

* Block Moves will occationally cause the moved

window to "Shatter".  - Move block off, then back on screen to fix.

* Wintach causing problems again <Sigh>

BETA RELEASE REV (E)

--------------------------

DATE: 5/3/94

* Driver is 640x480, 800x600, 1024x768 in 256 Colors,

- Resolutions are set by first selecting a

ET-4000 Driver of the prefered resoultion, then

install the new MVP driver on top of the installed

ET-4000 driver.  This will ensure that the Registry

values are correct.

* Supports All Screen Layout Modes:

- Edit the Registry (c:\windows\system32\regedt32.exe)

SYSTEM->CurrentControlSet->Services->Vga->Device0

- Add

STB.ScreenWidth

STB.ScreenHeight

- and set the values to the prefered screen layout.

Corrected Issues:

* Window Shattering.

* Blank Holes in Full Screen Redraw.

* Window Pieces left on screen.

* Wintach Works Fine !

Obvious Problems: NONE !

BETA RELEASE REV (F)

--------------------------

DATE: 5/31/94

* Driver is 640x480, 800x600, 1024x768 in 256 Colors,

* Supports All Screen Layout Modes

Corrected Issues: (Reference Bug Report 05/19/94)

* Wintach/WinBench Issues on 4 Screens lockups

* MicroGraphics and AutoCad lockups

* Analog Clock lockup

Obvious Probles:

* Paintbrush tool bar.

BETA RELEASE REV (G)

--------------------------

DATE: 6/20/94

* Driver is 640x480, 800x600, 1024x768 in 256 Colors,

* Supports All Screen Layout Modes

Corrected Issues:

* Lockup on window move from screen 3.5 to 1.5

* Screen Chopping on background bitmaps.

Obvious Probles:

* Paintbrush tool bar.

BETA RELEASE REV (H)

--------------------------

DATE: 7/5/94

* Driver is 640x480, 800x600, 1024x768 in 256 Colors,

* Supports All Screen Layout Modes

Corrected Issues:

* Winbench 4.0 Lockups

Obvious Probles:

* Paintbrush tool bar.

BETA RELEASE REV (I)

--------------------------

DATE: 7/10/94

* Driver is 640x480, 800x600, 1024x768 in 256 Colors,

* Supports All Screen Layout Modes

Corrected Issues:

* Paintbrush tool bar.

Obvious Probles:

* NONE !!

BETA RELEASE REV (J)

--------------------------

DATE: 7/11/94

* Driver is 640x480, 800x600, 1024x768 in 256 Colors,

* Supports All Screen Layout Modes

Corrected Issues:

* Paintbrush lockup during Zoom of a bmp file.

Obvious Probles:

* NONE !!

BETA RELEASE REV (K)

--------------------------

DATE: 7/25/94

* Driver is 640x480, 800x600, 1024x768 in 256 Colors,

* Supports All Screen Layout Modes

Corrected Issues:

* Wintach not filling Paint Screen

Obvious Problems:

* NONE !!

Please WRITE up a detailed description of any problems encountered, to

include applications used, versions, steps performed to produce the

failure, etc. and FAX it to Richard Goad @ (214) 234-1306.

The more detail you provide the faster we can correct the problem.

THANK YOU!