| System Unit | Dimensions (HxWxD) |  |
| --- | --- | --- |
|  | Armada 7790DMT and 7792DMT | 2.0 x 12.6 x 9.7 in/5.08 x 32 x 24.7 cm |
|  | All other models | 2.0 x 12.6 x 9.5 in/5.08 x 32 x 24.2 cm |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Weight |  |
|  | Without Power Cord |  |
|  | Armada 7710MT and 7730MT | 7.9 lb/3.59 kg |
|  | Armada 7750MT | 7.8 lb/3.54 kg |
|  | Armada 7770DMT | 8.03 lb/3.64 kg |
|  | Armada 7790DMT and 7792DMT | 8.01 lb/3.63 kg |
|  | With Power Cord |  |
|  | Armada 7710MT and 7730MT | 8.2 lb/3.72 kg |
|  | Armada 7750MT | 8.1 lb/3.67 kg |
|  | Armada 7770DMT | 8.4 lb/3.81 kg |
|  | Armada 7790DMT and 7792DMT | 8.4 lb/3.81 kg |
|  | Stand-Alone Power Requirements |  |
|  | Average Operating Power | 15 W |
|  | Peak Operating Power | 25 W |
|  | Power Supply |  |
|  | Rated Input Voltage | 100-120/220-240 VAC |
|  | Rated Input Current | 1.2/0.6A |
|  | Rated Frequency | 50-60 Hz |
|  | Temperature |  |
|  | Operating | 50 to 104° F/10 to 40° C |
|  | Nonoperating | -4 to 140° F/-20 to 60° C |
|  | Relative Humidity (noncondensing) |  |
|  | Operating | 10 to 90% |
|  | Nonoperating | 5 to 95% |
|  | Shock |  |
|  | Operating | 10 G, 11 ms, half sine |
|  | Non operating | 240 G, 2 ms, half sine |
|  | Vibration |  |
|  | Operating | 0.5 G |
|  | Nonoperating | 1.5 G |
|  | Maximum Altitude (unpressurized) |  |
|  | Operating | 10,000 ft/3.05 km |
|  | Nonoperating | 30,000 ft/9.14 km |
| 13.3-Inch Color TFT 1024 x 768 Display | Dimensions (HxW) Diagonal Size | 7.98 x 10.64 in/20.28 x 27.03 cm 13.3 in/33.79 cm |
|  | Mounting | Internal |
|  | Number of Colors | Up to 64,000 |
|  | Contrast Ratio | 150:1 |
|  | Brightness | 125 CD/M2 |
|  | Pixel Resolution |  |
|  | Pitch | 0.264 x 0.264 mm |
|  | Format | 1024 x 768 |
|  | Configuration | RGB stripe |
|  | Backlight | Edge Lit |
|  | Character Display | 80 x 25 |
|  | Total Power Consumption (maximum) | 4.0 W |

| 12.1-Inch Color TFT 1024 x 768 Display | Dimensions (HxW) Diagonal Size | 7.3 x 9.7 in/18.43 x 24.58 cm 12.1 in/30.72 cm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mounting | Internal |  |  |  |  |  |
|  | Number of Colors | Up to 64 K |  |  |  |  |  |
|  | Contrast Ratio | 100:1 |  |  |  |  |  |
|  | Brightness | 85 CD/M2 @ 6.0 mA |  |  |  |  |  |
|  | Pixel Resolution |  |  |  |  |  |  |
|  | Pitch | 0.240 x 0.240 mm |  |  |  |  |  |
|  | Format | 1024 x 768 |  |  |  |  |  |
|  | Configuration | RGB stripe |  |  |  |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Backlight | Edge Lit |  |  |  |  |  |
|  | Character Display | 80 x 25 |  |  |  |  |  |
|  | Total Power Consumption (maximum) | 3.8 W |  |  |  |  |  |
| 12.1-Inch Color TFT 800 x 600 (SVGA) Display | Dimensions (HxW) Diagonal Size |  | 7.3 x 9.7 in/18.45 x 24.60 cm 12.1 in/30.75 cm |  |  |  |  |
|  | Mounting |  | Internal |  |  |  |  |
|  | Number of Colors |  | Up to 16M |  |  |  |  |
|  | Contrast Ratio |  | 100:1 |  |  |  |  |
|  | Brightness |  | 95 CD/M2 @ 5.75 mA |  |  |  |  |
|  | Pixel Resolution |  |  |  |  |  |  |
|  | Pitch |  | 0.3075 x 0.3075 mm |  |  |  |  |
|  | Format |  | 800 x 600 |  |  |  |  |
|  | Configuration |  | RGB Stripe |  |  |  |  |
|  | Backlight |  | Edge Lit |  |  |  |  |
|  | Character Display |  | 80 x 25 |  |  |  |  |
|  | Total Power Consumption (maximum) |  | 3.8 W |  |  |  |  |
| Hard Drives1 |  |  | 1.6-GB | 2.1-GB | 3-GB | 5-GB |  |
|  | Capacity Per Drive |  | 1631.9 MB | 2167.6 MB | 3080.0 MB | 5132.1 MB |  |
|  | SMART, drive fault prediction capable |  | Yes | Yes | Yes | Yes |  |
|  | Drive Height |  | 19 mm | 19 mm | 19 mm | 19 mm |  |
|  | Transfer Rate |  |  |  |  |  |  |
|  | Media |  | 48.7 to 74.5 Mb/s | 48.7 to 74.5 Mb/s | 48.7 to 74.5 Mb/s | 60.2 to 98.5 Mb/s |  |
|  | Interface |  | 16.6 MB/s | 16.6 MB/s | 16.6 MB/s | 16.6 MB/s |  |
|  | Sector Interleave |  | 1:1 | 1:1 | 1:1 | 1:1 |  |
|  | Seek Times (typical, including settling) |  |  |  |  |  |  |
|  | Track-to-track |  | 4 ms | 4 ms | 4 ms | 4 ms |  |
|  | Average |  | 12 ms | 12 ms | 12 ms | 12 ms |  |
|  | Full Stroke |  | 23 ms | 23 ms | 23 ms | 23 ms |  |
|  | Disk Rotational Speed |  | 4900 rpm | 4900 rpm | 4900 rpm | 4902 rpm |  |

| Hard Drives1 (cont’d) |  | 1.6-GB | 2.1-GB | 3-GB | 5-GB |
| --- | --- | --- | --- | --- | --- |
|  | Physical Configuration |  |  |  |  |
|  | Cylinders | 4927 | 4927 | 5119 | 6975 |
|  | Heads | 5 | 6 | 8 | 8 |
|  | Sectors per track | 178 to 112 | 178 to 112 | 175 to 110 | 225 to 137 |
|  | Bytes per sector | 512 | 512 | 512 | 512 |
|  | Logical Configuration |  |  |  |  |
|  | Cylinders | 3215 | 4200 | 5968 | 10592 |
|  | Heads | 16 | 16 | 16 | 15 |
|  | Sectors per track | 63 | 63 | 63 | 63 |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Bytes per sector | 512 | 512 | 512 | 512 |
|  | Buffer Size | 96 K | 96 K | 96 K | 512 K |
| Diskette Drive | Diskette Size Light | 3.5-inch On System |  |  |  |
|  | Drive Height | .5 in /12.7 mm |  |  |  |
|  | Bytes per Sector | 512 |  |  |  |
|  | Sectors per Track |  |  |  |  |
|  | High density | 18 (1.44-MB)/15 (1.2-MB) |  |  |  |
|  | Low density | 9 |  |  |  |
|  | Tracks per side |  |  |  |  |
|  | High density | 80 (1.44-MB)/80 (1.2-MB) |  |  |  |
|  | Low density | 80 |  |  |  |
|  | Read/Write heads | 2 |  |  |  |
|  | Access Times |  |  |  |  |
|  | Track-to-track (high/low) | 3 ms/6 ms |  |  |  |
|  | Average (high/low) | 94 ms/173 ms |  |  |  |
|  | Settling time | 15 ms |  |  |  |
|  | Latency average | 100 ms |  |  |  |
| 20X Max2 CD-ROM Drive | Applicable Disc | CD-DA, CD-ROM Mode 1, Mode 2 CD-XA (Mode 2, Form 1, Form 2) CD-I (Mode 2, Form 1, Form 2) CD-I Ready, CD Extra, Video CD, CD-Bridge PhotoCD (Single and Multisession) CD-WO (Fixed packets only) |  |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Center Hole Diameter | .59 in/1.5 cm |  |  |  |
|  | Disc Diameter | 12 cm, 8 cm |  |  |  |
|  | Track Pitch | 1.6 µm |  |  |  |
|  | Access Times (typical including settling) Random | Less than 170 ms |  |  |  |
|  | Full Stroke | Less than 600 ms |  |  |  |
|  | Audio Output Level |  |  |  |  |
|  | Line Out | .7 Vrms |  |  |  |
|  | Cache Buffer | 128 KB |  |  |  |

| 20X Max2 CD-ROM Drive | Data Transfer Rates (typical, including settling) |  |
| --- | --- | --- |
| (cont’d) | Sustained (1X) | 150 KB/s |
|  | Sustained (20X) | 3.0 MB/s |
|  | Startup Time | Less than 10 seconds |
| Li-Ion Battery Pack . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Dimensions (HxWxD) Weight | .92 x 3.46 x 5.44 in/2.34 x 8.80 x 13.82 cm .89 lb/.40 kg |
|  | Energy |  |
|  | Voltage | 14.4 V |
|  | Capacity | 2.7 Ah |
|  | Battery Life | Up to 3 hours3 |
|  | Temperature |  |
|  | Operating | 50° to 104° F/10° to 40° C |
|  | Recommanded Storage | 32° to 113° F/0° to 45° C |
| ArmadaStation Expansion Base (optional) | Dimensions (HxWxD) Without Monitor Support Cover | 5 x 17.3 x 16 in/12.7 x 46.5 x 40.6 cm |
|  | With Monitor Support Cover | 5.6 x 17.3 x 16 in/14.2 x 46.5 x 40.6 cm |
|  | Weight |  |
|  | ArmadaStation | 22.7 lb/10.3 kg |
|  | Monitor Support Cover | 3.3 lb/1.5 kg |
|  | Power Supply (Input) |  |
|  | Operating Voltage | 90-260 VAC |
|  | Operating Current | 5.0 Amp Maximum |
|  | Rated Voltage | 100-120 to 220-240 VAC |
|  | Rated Current | 2.5-5.0 Amp |
|  | Line Frequency | 50-60 Hz |
|  | Temperature |  |
|  | Operating | 5 to 95° F/10 to 35° C |
|  | Nonoperating | -22 to 140° F/-30 to 60° C |
|  | Relative Humidity |  |
|  | Operating | 20 to 80% |
|  | Nonoperating | 5 to 90% |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Maximum Altitude |  |
|  | Operating | 10,000 ft/3048 m |
|  | Nonoperating | 30,000 ft/9144 m |
|  | Shock Operating | 5 G, 11 ms, half sine |
|  | Nonoperating | 140 G, 2 ms, half sine |
|  | Vibration |  |
|  | Operating | .25 G, 10 to 500 Hz sine .25 octave/minute sweep |
|  | Nonoperating | .75 G, 10 to 500 Hz sine .5 octave/minute sweep |
|  | Monitor Support (maximum) | 55 lb/25 kg |

| Armada MiniStation E Convenience Base (optional) (cont’d) | Dimensions (HxWxD) Without Monitor Support Cover With Monitor Support Cover | 5.0 x 14.75 x 15.37 in/12.7 x 37.47 x 39.05 cm 5.6 x 14.75 x 15.37 in/14.2 x 37.47 x 39.05 cm |
| --- | --- | --- |
|  | Weight |  |
|  | Armada MiniStation E | 7.67 lb/3.48 kg |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Monitor Support Cover Option | 3.3 lb/1.5 kg |
|  | Power Supply (Input) |  |
|  | Operating Voltage | 90-260 VAC |
|  | Operating Current | 3.0 Amp Maximum |
|  | Rated Voltage | 100-120 VAC to 220-240 VAC |
|  | Rated Current | 1.5-3.0 Amps Maximum |
|  | Line Frequency | 50-60 Hz |
|  | Temperature |  |
|  | Operating | 5 to 95° F/10 to 35° C |
|  | Nonoperating | -22 to 140° F/-30 to 60° C |
|  | Relative Humidity |  |
|  | Operating | 20 to 80% |
|  | Nonoperating | 5 to 90% |
|  | Maximum Altitude |  |
|  | Operating | 10,000 ft/3048 m |
|  | Nonoperating | 30,000 ft/9144 m |
|  | Shock |  |
|  | Operating | 5 G, 11 ms, half sine |
|  | Nonoperating | 140 G, 2 ms, half sine |
|  | Vibration |  |
|  | Operating | .25 G, 10 to 500 Hz sine .25 octave/minute sweep |
|  | Nonoperating | .75 G, 10 to 500 Hz sine .5 octave/minute sweep |
|  | Monitor Support (Maximum) | 55 lb/25 kg |
|  | 1 Hard drive specifications may vary slightly depending on vendor. 2 Data transfer rates will vary from 1275 KB/s to 3000 KB/s 3 When tested in standard configuration using the Ziff-Davis BatteryMark test version 1.0, the battery life of the Armada 7750MT will range from 1 hour and 30 minutes to 2 hours and 30 minutes; the 7730MT and 7710MT will range from 1 hour and 30 minutes to 3 hours and 00 minutes; the 7770DMT will range from 1 hour and 45 minutes to 3 hours; the 7790DMT and 7792DMT will range from 1 hour and 30 minutes to 2 hours and 30 minutes; depending on individual usage. Compaq, registered United States Patent and Trademark Office. Armada, Armada Station, MiniStation, MultiLock, QuickBlank, QuickLock, are trademarks and/or service marks of Compaq Computer Corporation. Other product names mentioned herein may be trademarks and/or registered trademarks of their respective companies. ©1998 Compaq Computer Corporation. All rights reserved. Printed in the U.S.A. |  |