| System Unit | Dimensions (HxWxD) | 6.6 x 17.65 x 17.11 in/16.76 x 44.83 x 43.46 cm |  |
| --- | --- | --- | --- |
|  | Weight (approximate) | 23.7 lbs/10.74 kg |  |
|  | Weight Supported (in a Desktop Configuration) | 100 lbs/45.5 kg |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Power Supply |  |  |
|  | Operating Voltage Range | 90 - 132 VAC |  |
|  | Rated Voltage Range | Switched 110/220V |  |
|  | Rated Line Frequency | 50 - 60 Hz |  |
|  | Rated Input Current | 145W 4A/2A (115V/230V) |  |
|  | Maximum Rated Power | 145 W |  |
|  | Temperature |  |  |
|  | Operating | 50° to 95°F/10° to 35°C |  |
|  | Maximum Rate of Change | 50 F or 10 C/hr |  |
|  | Nonoperating | -22° to 140°F/-30° to 60°C |  |
|  | Maximum Rate of Change | 68 F or 20 C/hr |  |
|  | Humidity (noncondensing) |  |  |
|  | Operating | 20% to 80% |  |
|  | Nonoperating | 5% to 90% (38.7 C maximum wet bulb temperature) |  |
|  | Maximum Altitude (unpressurized) |  |  |
|  | Operating | 0 to 10,000 ft/0 to 3048 m |  |
|  | Nonoperating | 0 to 30,000 ft/0 to 9144 m |  |
|  | Maximum Rate of Change | 1,500 feet or 457 meters per minute |  |
| Enhanced Keyboard | Dimensions (HxWxD) Weight | 1.5 x 18.0 x 6.5 in/3.81 x 45.72 x 16.51 cm 3.5 lbs/1.59 kg |  |
| 1.44-MB Diskette Drive | Diskette Size | 3.5-in |  |
|  | LED Indicators | Green |  |
|  | Read/Write Capacity/Diskette (high/low density) | 1.44-MB/720 KB |  |
|  | Drive Height | 1 in |  |
|  | Drive Rotation | 300 rpm |  |
|  | Transfer Rate (high/low density) | 500/250 Kb/s |  |
|  | Bytes/Sector | 512 |  |
|  | Sectors/Track (high/low density) | 18/9 |  |
|  | Tracks/Side (high/low density) | 80/80 |  |
|  | Seek Time |  |  |
|  | Track-to-Track (high/low density) | 3/6 ms |  |
|  | Average (high/low) | 94/173 ms |  |
|  | Settling Time | 15 ms |  |
|  | Latency Average | 100 ms |  |
|  | Cylinders (high/low) | 80/80 |  |
|  | Read/Write Heads | 2 |  |

| SMART II Ultra ATA Hard Drives1 | Formatted Capacity | 4.3 GB2 4300 MB |  | 6.4 GB2 6400 MB |  |  | 10.0 GB2 10000MB |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Drive Type | 65 (soft) |  | 65 (soft) |  |  | 65 (soft) |
|  | Interface | Ultra ATA |  | Ultra ATA |  |  | Ultra ATA |
|  | Transfer Rate |  |  |  |  |  |  |
|  | Heads | 18.6 Mb/s |  | 10.6-17 Mb/s |  |  | 18.6 Mb/s |
|  | Interface | 33.3 Mb/s |  | 33.3 Mb/s |  |  | 33.3 Mb/s |
|  | Seek Time (typical including settling) |  |  |  |  |  |  |
|  | Single Track | .09 ms |  | 2.0 ms |  |  | < 1.0 ms |
|  | Average | 9.0 ms |  | 9.7 ms |  |  | 9.5 ms |
|  | Full Stroke | < 20 ms |  | 20 ms |  |  | < 20 ms |
|  | Disk Rotational Speed | 5400 rpm |  | 5400 rpm |  |  | 5400 rpm |
|  | Cylinders |  |  |  |  |  |  |
|  | Physical | 11530 |  | 7913 |  |  | 11658 |
|  | Logical | 8912 |  | 13328 |  |  | 16383 |
|  | Data Heads . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | 15 |  | 15 |  |  | 16 |
|  | Physical | 3 |  | 6 |  |  | 6 |
|  | Logical | 15 |  | 15 |  |  | 16 |
|  | Sectors per Track |  |  |  |  |  |  |
|  | Physical | 269-320 |  | 168-312 |  |  | 216-320 |
|  | Logical | 63 |  | 63 |  |  | 63 |
|  | Buffer Size | 256 KB |  | 256 KB |  |  | 256 KB |
| 32X Max3 trayload CD-ROM Drive | Disk Applicable Disk |  | CD-ROM (Mode 1 and 2), CD-DA, CD-XA |  |  |  |  |
|  |  |  | (Mode 2, Form 1 and 2), Photo CD (single and multi session), Mixed Mode (audio and data combined), CDI ready |  |  |  |  |
|  | Capacity |  | 650 MB |  |  |  |  |
|  | Block Size |  | 2048 bytes (Mode 1) |  |  |  |  |
|  |  |  | 2340, 2336 bytes (Mode 2) |  |  |  |  |
|  |  |  | 2352 bytes (CD-DA) |  |  |  |  |
|  |  |  | 2328 bytes (CD-XA) |  |  |  |  |
|  | Rotational Speed |  | 5200 rpm |  |  |  |  |
|  | Diameter |  | 12 cm, 8 cm |  |  |  |  |
|  | Center Hole |  | 15 mm |  |  |  |  |
|  | Thickness |  | 1.2 mm |  |  |  |  |
|  | Track Pitch |  | 1.6 µm |  |  |  |  |
|  | Interface |  | IDE |  |  |  |  |
|  | Access Time (typical) |  |  |  |  |  |  |
|  | Random |  | < 100 ms |  |  |  |  |
|  | Full Stroke |  | < 150 ms |  |  |  |  |
|  | Audio Output Level |  |  |  |  |  |  |
|  | Line Out |  | 0.7 V @ 47 ohm |  |  |  |  |
|  | Headphone |  | 0.6 V @ 32 ohm |  |  |  |  |
|  | Data Transfer Rate |  |  |  |  |  |  |
|  | Sustained |  | 150 KB/s |  |  |  |  |
|  | Burst |  | 3.6 MB/s (variable) |  |  |  |  |
| 32X Max3 trayload CD-ROM Drive (cont’d) | Cache Buffer |  | 128 KB |  |  |  |  |
|  | Startup Time |  | < 7 seconds (typical); < 30 sec with multisession |  |  |  |  |
|  | Stop Time |  | < 4 seconds |  |  |  |  |
|  | Laser Parameters |  |  |  |  |  |  |
|  | Type |  | Semiconductor Laser Con A1 As |  |  |  |  |
|  | Wave Length |  | 780 +/- 25 nm |  |  |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Divergence Angle |  | 53.5° +/- 1.5° |  |  |  |  |
|  | Output Power |  | 0.14 mW @ objective lens |  |  |  |  |
|  | Operating Conditions Temperature |  | 41° to 113° F/5° to 45° C |  |  |  |  |
|  | Humidity |  | 10 to 80% |  |  |  |  |
|  | Dimensions (HxWxD) |  | 1.69 x 5.91 x 8.2 in/4.29 x 15.01 x 20.8 cm |  |  |  |  |
|  | Weight |  | 2.65 lbs/1.2 kg |  |  |  |  |
| Matrox MGA G100 AGP Graphics Controller (66 MHz) |  |  | 4-MB SGRAM |  |  |  |  |
|  | Resolution (Maximum) |  | Colors Supported |  |  | Refresh Rate |  |
|  | 1920 x 1200 |  | 65K |  |  | 65Hz |  |
|  | Acceleration |  | 3 ROP BitBLT, Line Draw, Text, Color Expansion, Basic Video Color Conversion and Scaling |  |  |  |  |
|  | Memory |  | 4-MB SGRAM standard, not upgradable |  |  |  |  |
| Matrox Millennium G200-SD AGP Graphics |  |  | 8-MB SDRAM |  |  |  |  |
| Controller (100 MHz) | Resolution (Maximum) |  | Colors Supported |  | Refresh Rate |  |  |
|  | 1920 x 1080 |  | 16.7M |  | 65Hz |  |  |
|  | Acceleration |  | 3 ROP BitBLT, Line Draw, Text, Color Expansion, Basic Video Color Conversion and Scaling, and Triangle Setup Engine |  |  |  |  |
|  | Memory |  | 8-MB SDRAM, upgradable to 16-MB |  |  |  |  |
|  | 1 Hard drive specifications will vary depending on vendor. 2 For Hard Drives, GB = Billion Bytes. 3 Data transfer rate will vary from 150 KB/s to 4800 KB/s. Compaq, registered United States Patent and Trademark Office. Armada, Armada Station, Ministation, MultiLock, QuickBlank, QuickLock, are trademarks and/or service marks of Compaq Computer Corporation. Intel, the Intel Inside Logo, and Pentium are registered trademarks of Intel Corporation. Microsoft, MS, Windows and Windows NT are registered trademarks of the Microsoft Corporation. Other product names mentioned herein may be trademarks and/or registered trademarks of their respective companies. ©1998 Compaq Computer Corporation. All rights reserved. Printed in the U.S.A. |  |  |  |  |  |  |