|  | Compaq Deskpro 2000 Model 5166X | Compaq Deskpro 2000 Model 5200X | Compaq Deskpro 2000 Model 5233X | Compaq Deskpro 2000 Model 6233X | Compaq Deskpro 2000 Model 6266X |
| --- | --- | --- | --- | --- | --- |
| System Unit |  |  |  |  |  |
| Microprocessor | 166-MHz Intel Pentium® Processor with MMXÔ technology | 200-MHz Intel Pentium Processor with MMXÔ technology | 233-MHz Intel Pentium Processor with MMXÔ technology | 233-MHz Intel Pentium II Processor | 266-MHz Intel Pentium II Processor |
| Upgradability | To other Pentium Processors with MMXÔ technology | To other Pentium Processors with MMXÔ technology | No upgrade available | To other Pentium II Processors1 | Intel Pentium II Processor 300 MHz1 |
| Cache Standard | 256-KB level 2 pipeline burst synchronous | 256-KB level 2 pipeline burst synchronous | 256-KB level 2 burst synchronous | 512-KB level 2 pipeline burst synchronous | 512-KB level 2 burst synchronous |
| Optional | 256-KB upgrade | 256-KB upgrade | 256-KB upgrade | not upgradeable | not upgradeable |
| Architecture | PCI/ISA | PCI/ISA | PCI/ISA | PCI/ISA | PCI/ISA |
| Slots |  |  |  |  |  |
| Shared ISA/PCI | 1 | 1 | 1 | 1 | 1 |
| PCI | 2 | 2 | 2 | 2 | 2 |
| ISA | 2 | 2 | 2 | 2 | 2 |
| Total | 5 | 5 | 5 | 5 | 5 |
| Bays |  |  |  |  |  |
| Internal | 2 | 2 | 2 | 2 | 2 |
| External | 3 | 3 | 3 | 3 | 3 |
| Total | 5 | 5 | 5 | 5 | 5 |
| RAM |  |  |  |  |  |
| Standard | 16 MB | 16 MB (Model 2100), 32 MB (Model 3200) | 16 MB (Model 2100), 32 MB (Model 3200/CDS) | 32 MB | 32 MB |
| Maximum | 384 MB | 384 MB | 384 MB | 384 MB | 384 MB |
| Intelligent Manageability |  |  |  |  |  |
| AssetControl | Standard | Standard | Standard | Standard | Standard |
| Fault Management with SMART II Hard Drives | Standard | Standard | Standard | Standard | Standard |
| Security Management | Standard | Standard | Standard | Standard | Standard |
| Configuration Management | Standard | Standard | Standard | Standard | Standard |
| Integration Management | Standard | Standard | Standard | Standard | Standard |
| Storage |  |  |  |  |  |
| Diskette Drives |  |  |  |  |  |
| Standard | 1.44-MB | 1.44-MB | 1.44-MB | 1.44-MB | 1.44-MB |
| Optional | LS-120 | LS-120 | LS-120 | LS-120 | LS-120 |
| CD-ROM Drives |  |  |  |  |  |
| Standard on CDS models |  |  | 16X Max2 CD-ROM Drive IDE (slot-load) | 24X Max3 CD-ROM Drive IDE (slot-load) | 24X Max3 CD-ROM Drive IDE (slot-load) |
| Optional | 16X Max2 CD-ROM 8X PD-CD | 16X Max2 CD-ROM 8X PD-CD | 8X PD-CD | 8X PD-CD | 8X PD-CD |

|  | Compaq Deskpro 2000 Model 5166X |  | Compaq Deskpro 2000 Model 5200X | Compaq Deskpro 2000 Model 5233X | Compaq Deskpro 2000 Model 6233X | Compaq Deskpro 2000 Model 6266X |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Storage (cont’d) |  |  |  |  |  |  |  |
| Hard Drives |  |  |  |  |  |  |  |
| Standard | 2.1-GB Ultra ATA4 (Model 2100), 3.2-GB Ultra ATA4 (Model 3200) |  | 2.1-GB Ultra ATA4 (Model 2100), 3.2-GB Ultra ATA4 (Model 3200) | 2.1-GB Ultra ATA4 (Model 2100), 3.2-GB Ultra ATA4 (Model 3200/CDS) | 2.1-GB Ultra ATA (Model 2100), 3.2-GB Ultra ATA (Model 3200) | 3.2-GB Ultra ATA (Model 3200) |  |
| Optional | 2.4 and 4.3 GB Ultra ATA, 2.1 and 4.3 GB UltraSCSI |  | 2.4 and 4.3 GB Ultra ATA, 2.1 and 4.3 GB UltraSCSI | 2.4 and 4.3 GB Ultra ATA, 2.1 and 4.3 GB UltraSCSI | 2.4 and 4.3 GB Ultra ATA, 2.1 and 4.3 GB UltraSCSI | 2.4 and 4.3 GB Ultra ATA, 2.1 and 4.3 GB UltraSCSI |  |
| Tape Drives |  |  |  |  |  |  |  |
| Optional | TR4 4-GB IDE |  | TR4 4-GB IDE | TR4 4-GB IDE | TR4 4-GB IDE | TR4 4-GB IDE |  |
| Controllers |  |  |  |  |  |  |  |
| Standard | Integrated Ultra ATA |  | Integrated Ultra ATA | Integrated Ultra ATA | Integrated Ultra ATA | Integrated Ultra ATA |  |
| Optional | Plug and Play SCSI (ISA), PCI UltraSCSI |  | Plug and Play SCSI (ISA), PCI UltraSCSI | Plug and Play SCSI (ISA), PCI UltraSCSI | Plug and Play SCSI (ISA), PCI UltraSCSI | Plug and Play SCSI (ISA), PCI UltraSCSI |  |
| Interfaces |  |  |  |  |  |  |  |
| Parallel | 1 |  | 1 | 1 | 1 | 1 |  |
| Serial | 1 |  | 1 | 1 | 1 | 1 |  |
| Pointing Device (mouse) | 1 |  | 1 | 1 | 1 | 1 |  |
| Graphics | 1 |  | 1 | 1 | 1 | 1 |  |
| Keyboard | 1 |  | 1 | 1 | 1 | 1 |  |
| USB | 2 |  | 2 | 2 | 2 | 2 |  |
| Graphics |  |  |  |  |  |  |  |
| S3 Trio 64V2/GX Matrox MGA 1064/SG Matrox MGA Millennium II Graphics Controller | Integrated — Optional |  | Integrated — Optional | Integrated — Optional | — Integrated Optional | — Integrated Optional |  |
| Monitors |  |  |  |  |  |  |  |
| V50 Color | Optional |  | Optional | Optional | Optional | Optional |  |
| V70 Color | Optional |  | Optional | Optional | Optional | Optional |  |
| V90 Color | Optional |  | Optional | Optional | Optional | Optional |  |
| P50 Color | Optional |  | Optional | Optional | Optional |  | Optional |
| P70 Color | Optional |  | Optional | Optional | Optional |  | Optional |
| P110 Color | Optional | Optional |  | Optional | Optional |  | Optional |
| P1610 | Optional | Optional |  | Optional | Optional |  | Optional |
| TFT500 Flat Panel | Optional | Optional |  | Optional | Optional |  | Optional |

|  | Compaq Deskpro 2000 Model 5166X | Compaq Deskpro 2000 Model 5200X | Compaq Deskpro 2000 Model 5233X | Compaq Deskpro 2000 Model 6233X | Compaq Deskpro 2000 Model 6266X |
| --- | --- | --- | --- | --- | --- |
| Multimedia |  |  |  |  |  |
| Business Multimedia 24X Kit | Optional | Optional | Optional | Optional | Optional |
| Enhanced Business Pro Audio6 | Optional | Optional | Standard (CDS Model) | Standard (CDS Model) | Standard (CDS Model) |
| Business Wavetable Sound Card7 | Optional | Optional | Optional | Optional | Optional |
| Multimedia Sound System (speakers) | Optional | Optional | Optional | Optional | Optional |
| Keyboard |  |  |  |  |  |
| Enhanced Keyboard | Standard | Standard | Standard | Standard | Standard |
| Supported System Software |  |  |  |  |  |
| Software Included | Windows 95, Diagnostics for Windows, Optimized graphics/audio drivers, Intelligent Manageability Agent | Windows 95, Diagnostics for Windows, Optimized graphics/audio drivers, Intelligent Manageability Agent | Windows 95, Diagnostics for Windows, Optimized graphics/audio drivers, Intelligent Manageability Agent | Windows 95, Diagnostics for Windows, Optimized graphics/audio drivers, Intelligent Manageability Agent | Windows 95, Diagnostics for Windows, Optimized graphics/audio drivers, Intelligent Manageability Agent |
| Banyan VINES v 4.11 | Optional | Optional | Optional | Optional | Optional |
| Microsoft LAN Manager v 2 | Optional | Optional | Optional | Optional | Optional |
| Novell NetWare v 4.01 | Optional | Optional | Optional | Optional | Optional |
| IBM OS/2 Warp 3.05 | Optional | Optional | Optional | Optional | Optional |
| Microsoft Windows NT 4.0 | Optional | Optional | Optional | Standard | Standard |
| Microsoft Windows 3.1 | Optional | Optional | Optional | Optional | Optional |
| Microsoft Windows 95 | Standard | Standard | Standard | Optional | Optional |
| 1 Not upgradable to the 333 MHz Pentium II Processor. 2 Data transfer rate will vary from 150 KB/s to 2400 KB/s. 3 Data transfer rate will vary from 150 KB/s to 3600 KB/s. 4 Pentium Processor with MMXÔ technology models manufactured prior to February 15, 1998 may be configured with EIDE hard drives. 5 Not all product features are supported in this OS. 6 Includes internal speaker. 7 Supports external speakers only, speakers not included. |  |  |  |  |  |