PC/X - MC/X - MODEM/X

OS/2

92000043C

Software Installation and Operation Manual

for

DigiCHANNEL PC/X

DigiCHANNEL MC/X

DigiCHANNEL MODEM/X

OS/2

DBI 92000043C

DigiBoard, DigiCHANNEL, DigiWARE, DigiCHANNEL PC/X DigiCHANNEL MC/X and

DigiCHANNEL MODEM/X are trademarks of DigiBoard.  All other brand and

product names are the trademarks of their respective holders.

DigiBoard Incorporated 1990, 94

All Rights Reserved

DigiBoard

6400 Flying Cloud Drive

Eden Prairie, MN 55344

(612) 943-9090

Information in this document is subject to change without notice and does not

represent a commitment on the part of DigiBoard.

DigiBoard provides this document as is, without warranty of any kind, either ex-

pressed or implied, including, but not limited to, the particular purpose.  Digi-

Board may make improvements and/or changes in this manual or in the

product(s) and/or the program(s) described in this manual at any time.

This product could include technical inaccuracies or typographical errors.

Changes are periodically made to the information herein; these changes may be

incorporated in new editions of the publication.

NOTES

Introduction

This OS/2 device driver software documentation is for all of the

communication boards in the DigiCHANNEL PC/X, MC/X and

MODEM/X series.  The software is a fully interrupt-driven,

Microsoft API call-compatible device driver, and will support up

to 32 serial communication channels.

Board Differences

The DigiCHANNEL PC/4 and PC/8 are nearly identical boards,

the PC/4 merely having four fewer channels.   The DIP switch

and jumper setting procedures are the same for these boards, as

is the device driver software installation.

The PC/16 has no DIP switches to set, as its addresses are

burned into PAL (Programmable Array Logic) chips.  The

device driver software installation is the same as for the PC/4 and

PC/8, with the exception that the Per-Card parameters are

required.  (The default settings are for the PC/4 or PC/8, and

won't work with the PC/16.)

The MC/X boards are all configured through the POS Setup utilities.

Software Installation

To install the OS/2 device driver, simply copy the file DGX.SYS

from the OS/2 device driver diskette onto your hard disk.  Then

use a text editor to add the line

DEVICE=DGX.SYS [parameters]

to the CONFIG.SYS file.  Note that if DGX.SYS is not in the

root directory, you will have to give its full pathname.  (Example:

DEVICE = C:\BIN\DGX.SYS).

If you are installing a single four or eight-port board with the

default parameters , the parameters portion may be omitted.

DigiCHANNEL PC/16 users must enter the line

above with Per-Card Parameters as follows:

DEVICE=DGX.SYS /P:140 /C:16 /I:IRQ

where IRQ is the Interrupt Request line you have

selected for the board.

Default Device Names

With one board in the system, the default names for the devices

are COM3, COM4, COM5, COM6, COM7, COM8, COM9,

COM10...

Default modes for each of the channels follow the MS-OS/2

standards of 1200 Baud, 7 data bits, 1 stop bit, even parity, and

RTS control-mode.

Command Line Arguments to the Device Driver

There are both global as well as per-card arguments that can be

specified on the command line.  If any arguments are given, the

defaults in the data segment of the driver will be ignored.  Use a

comma ( , ) to separate sets of arguments for each card in your

system.  For the PC/X each card in your system must be fully described by

these arguments.  For MicroChannel systems (MC/X) only the below global

type arguments apply.

Global Arguments

The following Global Arguments set parameters for all channels

on all boards controlled by the device driver.  To change

parameters for individual ports, use the utility DMODE.EXE

provided on the distribution diskette with the driver.

DMODE.EXE is discussed at beginning on of this manual.

/n:device_names

Sets up the naming algorithm to be used to

generate device names.  This consists of an

optional new stem followed by the starting device

number. Without the new stem, the stem name

COM will be used.  Quotes are mandatory to set

a new stem.  Without a number, 3 is assumed.

Examples:

/n:"digi"0  DIGI0, DIGI1, DIGI2 ...

/n:4        COM4, COM5, COM6 ...

/n:"digi"   DIGI3, DIGI4, DIGI5 ...

Per-Card Arguments (PC/X cards only)

Per-Card arguments MUST be in complete sets identifying the

Status Port address and IRQ number.  The preceding global

arguments may be given without interfering with the default Per-

Card settings in the driver file.

Each board in your system must be fully described

by these Per-Card arguments.  Use a comma to

separate the board sets from one another.

Also note that the channel numbers (COM3, COM4, etc.) will be

assigned to the boards in the same order in which the per-card

arguments are given.

/p:Status Port Address

Set the Status Port address.  This argument speci-

fies the top of the range of I/O Port addresses that

will be used by a PC/X or MODEM/X board.  This

address should correspond to the dip-switch setting

for the Status Port.  Individual channel port ad-

dresses should then begin 40 (hex) bytes below this

address, occupying a contiguous block.

Example:

/p:280   (set Status Port address to 280h)

I/O Port for Channel 1 = 240h

I/O Port for Channel 2 = 248h

I/O Port for Channel 3 = 250h

.

.

.

I/O Port for Channel 8 = 278h

DigiCHANNEL PC/16 users must specify a Status

port address of 140h, as follows:

/p:140

/i:IRQ     Sets the IRQ line number to one of the legal

options for the PC/X or MODEM/X.

Example:

/i:5     (set card to IRQ 5 decimal)

/c:channels Sets the number of channels (either 4, 8 or 16).

This is required for four and sixteen-port boards.

If omitted, eight ports is assumed.

Examples:

/c:4     (PC/4 or MODEM/4)

/c:16    (PC/16)

/u:uart address permits the user to specifiy the starting UART addresses for

daisy chained boards (boards joined by daisy chained cables, sharing

the same IRQ level, and set to the same status port address).

All UART addresses on the PC/8 card must be contiguous starting with

the address specified (100 108 110 ...).

Only boards of the same type may be daisy-chained together;

8 port cards can only be daisy-chained with other 8 port

cards, 4 port cards with 4 port cards and so on.

The status port option "/p:" must be specfied before the "/u:"

option.

Multiple Uart addresses ("/u:") (one for each board configured) must

be specified in the same order as their board ID#'s set on the card.

Daisy-chained PC/16's must use DOS pals #1 and #2 for boards 0 and 1

and must be specified with /u:100 and /u:188 along with the /c:16

specifer.

Examples:

4 PC/8 cards configured as follows:

Board ID# 0 : Addresses @ 200h

208h

210h

218h

220h

228h

230h

238h

Board ID# 1 : Addresses @ 100h

108h

110h

118h

120h

128h

130h

138h

Board ID# 2 : Addresses @ 300h

308h

310h

318h

320h

328h

330h

338h

Board ID# 3 : Addresses @ 248h

250h

258h

260h

268h

270h

278h

280h

All status ports set to 140h

device=dgx.sys /p:140 /u:200 /u:100 /u:300 /u:248 /i:5

2 PC/16 cards

Cards must have DOS pals #1 and #2 which configures both

card's status ports @ 140h, IRQ 5 and starting port

addresses @ 100h and 188h.

device=dgx.sys /p:140 /c:16 /u:100 /u:188 /i:5

/b:baud    Sets the default baud rate to be used for all ports.

If this parameter is not specified, all ports will be

set to the OS/2 default of 1200 baud.  Baud rates of

up to 230,000 are supported.  Use the DMODE.EXE

command to set baud rates of individual ports.

Example:

/b:9600         Initializes all ports to 9600 baud.

/d:dtype   Sets the initial data type (parity, character

length and number of stop bits).  If this parameter

is omitted, the OS/2 default data type of even

parity, 7 data bits and 1 stop bit is used.  The

syntax is [pds], where p is E, O or N; d is 8, 7, 6

or 5; and s is 1 or 2.  All three values must be

supplied, in the correct order.

Example:

/d:N81          No parity, 8 data bits, 1 stop bit.

/s:sigs_off     Forces modem control output signals off on port open.

/s:d              DTR remains low or off on port open.

/s:r              RTS remains low or off on port open.

/s:dr             DTR & RTS remain low or off on port open.

/a:a       Enable the alternate pin assignment of Data Carrier Detect (DCD)

for all ports. This swaps the way the driver interprets the DSR

and DCD input pins. When Alt-Pin processing is enabled, transit-

ions on DSR are reported by the driver as DCD transitions. This

is only of benefit on 8-pin RJ-45 and 6-pin RJ-11 connectors where

the physical DCD pin is not available. To use this option, ensure

that your cabling physically routes the DCD signal to the DSR pin.

Troubleshooting

If, after installation, the device driver software comes up with a

problem message, first check that your configuration matches the

DIP switch settings on the DigiCHANNEL card.  Next, try

powering down the system and then bringing it back up.

Something that can cause problems is the issue of contention for device names.

To help diagnose this, a quick and reliable experiment to try is to temporarily

comment out the lines (by placing the word 'rem' at the start of the lines)

DEVICE=\OS2\COM.SYS

DEVICE=\OS2\VCOM.SYS

and reboot your system.  If this improves things, then try using the /n: flag

above to name the DigiBoard devices something else besides COM.  If it still

fails, call DigiBoard Technical Support at 612-943-9020.

In general, a good way to test the basic functionality of the driver

is to hook up a terminal to one of the ports (COM3 for example),

set it up for the correct Baud rate, Handshake, and Data type

characteristics, and then simply try to re-direct output to it from

the command prompt.  For example:

C:\>dir > com3

If the terminal is set up correctly (the above defaults are typical)

you should get the directory listing coming out on it.  If there is

no output, or if the above command hangs, then you may need

some technical support.  It is possible that the board is not

functioning properly.