RESQ232 WIRING DIAGRAM - PIN CONNECTIONS

| RX |  |  |  |
| --- | --- | --- | --- |
| RESQ232 | CONNECTOR | LINK UNDER TEST CONDITIONS |  |
| 9 Pin Female | 25 Pin Female | 9 Pin Male or Female | 25 Pin Male or Female |
| 2 | 3 | 2 | 3 |
| 8 | 5 | 8 | 5 |
| 6 | 6 | 6 | 6 |
| 5 | 7 | 5 | 7 |
| 1 | 8 | 1 | 8 |
| 9 | 21 | 9 | 21 |

| TX |  |  |  |
| --- | --- | --- | --- |
| RESQ232 | CONNECTOR | LINK UNDER TEST CONDITIONS |  |
| 9 Pin Female | 25 Pin Female | 9 Pin Male or Female | 25 Pin Male or Female |
| 2 | 3 | 3 | 2 |
| 8 | 5 | 7 | 4 |
| 6 | 6 | 4 | 20 |
| 5 | 7 | 5 | 7 |
| 7 9 | 4 21 |  |  |
| 1 4 | 8 20 |  |  |

How to Use the Table: You can create a ResQ232 cable that has either DB9 or DB25 connectors. For example, to build a cable that has 9 pin ResQ232 connectors, and 25 pin "Link Under Test" connectors, connect the pins under the "9 Pin Female" column on the ResQ232 side to the pins under the "25 Pin" column under the "Link Under Test" side. While the "Link Under Test" connectors can be any gender, one is usually male, the other female. Remember that you must connect all pins between the two "Link Under Test" connectors, as well as connecting the pins from the ResQ232 connectors.