YTNC is a "backend" for the G8BPQ switch.

It works in a similar manner to, and has the same command line

arguments as the G8BPQ supplied equivalent BPQHTNC2.

YTNC <number of COM ports> <first COM port> <first BPQ host port>

Examples:

YTNC 25 1 1

Create 25 COM ports numbered COM1 through COM25, using the first 25 host ports.

YTNC 3 12 19

Create 3 COM ports numbered COM12 through COM14, using host ports 19 - 21.

YTNC does not support unproto or monitoring, and has a minimal command set:

CONVerse      -  Change stream to Converse mode.

Trans	       -  Change stream to Transparent mode.

^C	       -  Change stream to Command mode. (send break does same)

Connect       -  Return stream connect status.

Connect CALL  -  Connect application to callsign.

Disconne      -  Disconnect application from switch.

K	       -  Connect application to switch.

APPLFlag      -  Set application flags.

APPL	       -  Set application mask.

BText	       -  Set beacon text.

CONOk	       -  ON/OFF - Allow/disallow connects from users.

CPactime      -  ON/OFF - Enable packetize-on-inactivity timer for CONV mode.

CR	       -  ON/OFF - SENDP character included/not included in packet.

Paclen        -  Set packet size.

SEndp	       -  Character that will force tx of packet.

YTNC resets PACLEN to the node default at disconnect.

On the COM port side, YTNC supports the normal DOS functions 1-7,

plus some G8BPQ node specific function 30 subfunctions:

1  - Return node software version.

2  - Returns connect status of stream.

3  - Returns # unacked frames.

4  - Returns user to node.

5  - Disconnects user.