ODI FOR FDDI, VERSION 1.0   JUNE 26, 1992

_________________________________________

About This File  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  2

Supported Platforms  . . . . . . . . . . . . . . . . . . . . . . . . . .  2

IBMFDDIO LAN Driver Installation Procedure   . . . . . . . . . . . . . .  3

DOS Workstation  . . . . . . . . . . . . . . . . . . . . . . . . . . .  3

OS/2 Workstation   . . . . . . . . . . . . . . . . . . . . . . . . . .  4

NetWare Server   . . . . . . . . . . . . . . . . . . . . . . . . . . .  5

Configurable IBMFDDIO LAN Driver Parameters  . . . . . . . . . . . . . .  7

Sample NET.CFG Configuration File for DOS and OS/2 Workstations  . . .   10

Sample IBMFDDIO.CFG Configuration File for NetWare Server  . . . . . .   11

IBMFDDIO LAN Driver Performance Tuning   . . . . . . . . . . . . . . .   12

DOS Workstation  . . . . . . . . . . . . . . . . . . . . . . . . . .   12

OS/2 Workstation   . . . . . . . . . . . . . . . . . . . . . . . . .   12

NetWare Server   . . . . . . . . . . . . . . . . . . . . . . . . . .   12

IBM FDDI Adapter Custom Statistic Counters   . . . . . . . . . . . . .   13

Messages for the IBMFDDIO LAN Driver   . . . . . . . . . . . . . . . .   16

(C) Copyright IBM Corp. 1992                                                1

ABOUT THIS FILE

This file describes the use of the IBMFDDIO LAN drivers with IBM FDDI

Adapters.  These LAN drivers support the IBM FDDI Adapter in the Novell

Netware environment.

This file is an extension of the IBM FDDI FIBER BASE ADAPTER/A INSTALLATION

AND TESTING INSTRUCTIONS or the IBM FDDI COPPER BASE ADAPTER/A INSTALLATION

AND TESTING INSTRUCTIONS that is shipped with the adapter.  This file

describes the installation and configuration of the IBM FDDI Open Datalink

Interface (ODI) (IBMFDDIO) LAN drivers.  It contains information about the

following topics:

o   Supported platforms

o   Software installation procedures

o   Configurable IBMFDDIO LAN driver parameters

o   Performance tuning the IBMFDDIO LAN drivers

o   IBM FDDI Adapter Custom Statistic Counters

o   Messages for the IBMFDDIO LAN driver.

SUPPORTED PLATFORMS

The IBMFDDIO LAN driver is supported in any of the hardware environments

supported by the IBM FDDI Adapter.  See the IBM FDDI FIBER BASE ADAPTER/A

INSTALLATION AND TESTING INSTRUCTIONS or the IBM FDDI COPPER BASE ADAPTER/A

INSTALLATION AND TESTING INSTRUCTIONS for information about supported

hardware.

The following operating system environments are supported:

o   Novell NetWare 3.11

o   IBM DOS 3.3

o   IBM DOS 4.0

o   IBM DOS 5.0

o   IBM OS/2 1.3

o   IBM OS/2 2.0.

ODI for FDDI, Version 1.0   June 26, 1992  2

IBMFDDIO LAN DRIVER INSTALLATION PROCEDURE

This section describes how to install the IBMFDDIO LAN driver on each of the

supported platforms (DOS, OS/2, or NetWare Server).

See the IBM FDDI FIBER BASE ADAPTER/A INSTALLATION AND TESTING INSTRUCTIONS

or the IBM FDDI COPPER BASE ADAPTER/A INSTALLATION AND TESTING INSTRUCTIONS

that is shipped with the adapter for instructions on how to install the IBM

FDDI Adapter.

DOS Workstation

This section describes how to install the IBMFDDIO LAN driver on a DOS

workstation.

To install the IBMFDDIO LAN driver on a DOS workstation, perform the

following steps:

1.  Choose a target directory path on the diskette or the fixed disk (for

example, C:\NETWARE) for installation of the IBMFDDIO LAN driver and its

associated files.  All files (in step 2 and step 3, below) should be

copied to this target directory.

2.  Copy the following files to the target directory:

o   IBMFDDIO.COM

3.  Copy IBMFDDI.BIN from the root directory of the IBM FDDI Adapter Option

Diskette to the target directory.

NOTE:  The IBMFDDI.BIN file MUST be in the same directory as the

IBMFDDIO.COM file.

4.  Add the directory path from step 1 to the PATH= statement in the

AUTOEXEC.BAT file.  For example:

PATH=C:\DOS;C:\NETWARE;

5.  Add the following lines to the AUTOEXEC.BAT file (after the PATH=

statement):

LSL

IBMFDDIO

IPXODI

NETX

REM  Add ROUTE.COM only if Source Routing is needed.

ROUTE

NOTE:  See NOVELL NETWARE ODI SHELL FOR DOS for information about

ROUTE.COM.

6.  If you need to change any IBMFDDIO LAN driver parameter values from

defaults, edit the NET.CFG file to change only those parameters.  See

ODI for FDDI, Version 1.0   June 26, 1992  3

"Sample NET.CFG Configuration File for DOS and OS/2 Workstations" for a

sample NET.CFG file.

You can use any DOS text editor to edit AUTOEXEC.BAT and NET.CFG.  See NOVELL

NETWARE ODI SHELL FOR DOS for more information about AUTOEXEC.BAT and

NET.CFG.

OS/2 Workstation

This section describes how to install the IBMFDDIO LAN driver on an OS/2

workstation.

To install the IBMFDDIO LAN driver on an OS/2 workstation perform the

following steps:

1.  Choose a target directory path on the diskette or the fixed disk (for

example, C:\NETWARE) for installation of the IBMFDDIO LAN driver and its

associated files.  All files (in step 2 and step 3, below) should be

copied to this target directory.

2.  Copy the following files to the target directory:

o   IBMFDDIO.OS2

3.  Copy IBMFDDI.BIN from the root directory of the IBM FDDI Adapter Option

Diskette to the target directory.

NOTE:  The IBMFDDI.BIN file MUST be in the same directory as the

IBMFDDIO.OS2 file.

4.  Add the following lines to the CONFIG.SYS file (if not already present):

DEVICE=C:\NETWARE\LSL.SYS

RUN=C:\NETWARE\DDAEMON.EXE

DEVICE=C:\NETWARE\IBMFDDIO.OS2

REM Add ROUTE.SYS only if Source Routing is needed.

DEVICE=C:\NETWARE\ROUTE.SYS

DEVICE=C:\NETWARE\IPX.SYS

NOTES:

a.  The Novell NetWare OS/2 support files must be installed before

running the OS/2 requester.  See NOVELL NETWARE REQUESTER FOR OS/2

for more information.

b.  See NOVELL NETWARE REQUESTER FOR OS/2 for information about

ROUTE.SYS.

c.  These lines must appear in the order indicated.

ODI for FDDI, Version 1.0   June 26, 1992  4

See NOVELL NETWARE REQUESTER FOR OS/2 for information about how to edit

the CONFIG.SYS file.

5.  If you need to change any IBMFDDIO LAN driver parameter values from their

defaults, edit the NET.CFG file to change only those parameters.  You can

use any text editor to edit the NET.CFG file.  See "Sample NET.CFG

Configuration File for DOS and OS/2 Workstations" for a sample NET.CFG

file.

See NOVELL NETWARE REQUESTER FOR OS/2 for more information about CONFIG.SYS

and NET.CFG.

NetWare Server

This section describes how to install the IBMFDDIO LAN driver on a NetWare

Server.

To install the IBMFDDIO LAN driver on a NetWare Server, perform the following

steps:

1.  Choose a target directory path on the diskette or the fixed disk (for

example, C:\NW) for installation of the IBMFDDIO LAN driver and its

associated files.  All files (in step 2, below) should be copied to this

target directory.

2.  Copy the following files to the target directory:

o   IBMFDDIO.LAN

o   MSM.NLM (MSM311.NLM if running Novell NetWare 3.11)

o   FDDITSM.NLM.

3.  Add the following lines to the STARTUP.NCF file:

Set Minimum Packet Receive Buffers = 30

Set Maximum Physical Receive Packet Size = 4500

NOTE:  If running on NetWare 3.11,

Set Maximum Physical Receive Packet Size = 4202

4.  Add the following line to the AUTOEXEC.NCF file:

LOAD C:\NW\IBMFDDIO.LAN @IBMFDDIO.CFG

NOTES:

a.  @IBMFDDDIO.CFG is an optional command line parameter (see step 5,

below).

b.  IBMFDDIO.LAN automatically loads MSM.NLM (MSM311.NLM if running

Novell NetWare 3.11) and FDDITSM.NLM.

ODI for FDDI, Version 1.0   June 26, 1992  5

See NOVELL NETWARE SYSTEM ADMINISTRATION for more information about the

AUTOEXEC.NCF and STARTUP.NCF files.

5.  If you need to change any IBMFDDIO LAN driver parameter values from their

defaults, these parameters can be specified on the command line when the

IBMFDDIO LAN driver is loaded or they can be specified by creating an

IBMFDDIO.CFG configuration file.  If the optional @IBMFDDIO.CFG command

line parameter is used to specify an IBMFDDIO.CFG configuration file,

then the configuration file must be located in the same directory as the

IBMFDDIO.LAN file.  You can use any text editor to create or edit the

IBMFDDIO.CFG file.  See "Sample IBMFDDIO.CFG Configuration File for

NetWare Server" for a sample IBMFDDIO.CFG file.

6.  If you run the MONITOR.NLM program at your NetWare server, obtain the

latest copy of the MONITOR.NLM program from Novell.  Update your NetWare

servers with the latest copy of the MONITOR.NLM program.

ODI for FDDI, Version 1.0   June 26, 1992  6

CONFIGURABLE IBMFDDIO LAN DRIVER PARAMETERS

The following parameters can be configured at DOS and OS/2 workstations by

setting keywords in the NET.CFG file.  These parameters can be configured at

the NetWare Server by setting keywords on the command line and in the

IBMFDDIO.CFG file.  A sample NET.CFG file is shown in "Sample NET.CFG

Configuration File for DOS and OS/2 Workstations." A sample IBMFDDIO.CFG file

is shown in "Sample IBMFDDIO.CFG Configuration File for NetWare Server."

SLOT

This keyword defines the slot number of the corresponding IBM FDDI

Adapter in the workstation.  The system defaults to the lowest

numbered slot containing a network adapter of this type.  Specify this

keyword when more than one IBM FDDI Adapter is present in the same

workstation.  This keyword must be within the range 1-8.

FRAME

This keyword defines the frame type used with the network adapter.

The default is FDDI_802.2.  The value for this keyword must be one of

the following:

o   FDDI_802.2

o   FDDI_SNAP

NODE ADDRESS (OS/2 or DOS) or NODE (Server)

To set a locally administered address for the adapter, type in the

local address you want to use.  The local address must contain 12

hexadecimal characters and can be specified in either MSB (Most

Significant Bit) or LSB (Least Significant Bit --the default) format.

To specify a MSB node address the letter "M" immediately follows the

last digit of the node address.  To specify a LSB node address the

letter "L" immediately follows the last digit of the node address.

Since LSB is the default format for the node address parameter, the

use of a letter "L" at the end of a node address is for clarity.

This keyword is specified as NODE ADDRESS in NET.CFG for DOS and OS/2

workstations.  See "Sample NET.CFG Configuration File for DOS and OS/2

Workstations" for a sample NET.CFG file.  This keyword is specified on

the command line or in the IBMFDDIO.CFG as NODE for the NetWare

Server.  See "Sample IBMFDDIO.CFG Configuration File for NetWare

Server" for a sample IBMFDDIO.CFG file.

If a node address is specified in MSB format, the characters must be

within the range 400000000000-7FFFFFFFFFFF.  To specify a node address

in MSB format, append an "M" to the end of the address (for example,

400000000001M).  This address specified in LSB format would be

020000000080L.

ODI for FDDI, Version 1.0   June 26, 1992  7

NOTES:

1.  Do not assign the same local address to two or more stations.

Severe station communication or network failures may occur when

two or more stations are assigned the same address.

2.  If you do not assign a locally administered address, the adapter

uses the universally administered address permanently embedded on

the adapter.

ATTACHMENTCLASS

This keyword defines the station as either a single attachment station

or a dual attachment station.

For single attachment, set ATTACHMENTCLASS to 1.

For dual attachment, set ATTACHMENTCLASS to 2.

If ATTACHMENTCLASS is not specified, it is set according to the

adapters (base or base and extender) present in the system.

PMFPASSWORD

This keyword defines the password that all remote PMF (Parameter

Management Frame) requests must provide to change FDDI attributes

within this adapter.  The value for PMFPASSWORD must be 8 ASCII

characters long and be enclosed in double quotes.  The PMFPASSWORD

parameter is not case sensitive since lower case letters will be

converted to upper case.

The double quote character can be specified within the PMFPASSWORD by

preceding the double quote character with a backslash (\) character

(\").  Hexadecimal characters can be specified in the ASCII string

with a "\" followed by one or two hexadecimal character digits.  To

specify an ASCII "\" character, two backslashes (\\) must immediately

follow each other.  Password protection can be disabled by setting

this keyword to all zeros (default), or by not specifying a

PMFPASSWORD.

USERDATA

This keyword defines a character string up to 32 characters long.  The

character string must be enclosed in double quotes.  The USERDATA

string cannot contain embedded double quotes and is not case sensitive

since lower case letters will be converted to upper case.

RXBUFFERS

This keyword defines the number of receive buffers used by the

IBMFDDIO LAN driver.  Values vary by workstation type as follows:

DOS WORKSTATIONS

The value for this keyword must be: 1, 2, or 4; the default

is 2.  (Each receive buffer takes 4500 bytes of memory.)

ODI for FDDI, Version 1.0   June 26, 1992  8

OS/2 WORKSTATIONS

The value for this keyword must be: 1, 2, 4, or 8; the

default is 4.  (Receive buffer size is defined by the LSL

buffer configuration.  See NOVELL NETWARE REQUESTER FOR OS/2

for more information.)

NETWARE SERVER

The value for this keyword must be: 1, 2, 4, 8, or 16; the

default is 16.  (Receive buffer size is defined by the

server SET command.  See NOVELL NETWARE SYSTEM

ADMINISTRATION for more information.)

TXBUFFERS

This keyword defines the number of 4500-byte transmit buffers used by

the IBMFDDIO LAN driver.  This keyword is applicable only to DOS or

OS/2 workstations; it is not available at the NetWare Server.  Values

vary by workstation type as follows:

DOS WORKSTATIONS

The value for this keyword must be: 1, 2, or 4; the default

is 2.

OS/2 WORKSTATIONS

The value for this keyword must be: 1, 2, 4, or 8; the

default is 4.

ODI for FDDI, Version 1.0   June 26, 1992  9

SAMPLE NET.CFG CONFIGURATION FILE FOR DOS AND OS/2 WORKSTATIONS

Link Support

; For OS/2 workstations, specify the following BUFFERS

; parameter to receive maximum size FDDI frames of 4500.

BUFFERS 10 4500

Link Driver IBMFDDIO

; The following are sample ODI keywords for IBMFDDIO:

NODE ADDRESS 400000000001M

SLOT  2

FRAME FDDI_802.2

; The following are ODI FDDI keywords with sample values:

ATTACHMENTCLASS  1

PMFPASSWORD      "12345678"

USERDATA         "IBM FDDI Adapter at DOS WS 12345"

; Default RXBUFFERS is 2 for DOS, and 4 for OS/2.

RXBUFFERS        2

; Default TXBUFFERS is 2 for DOS, and 4 for OS/2.

TXBUFFERS        2

ODI for FDDI, Version 1.0   June 26, 1992  10

SAMPLE IBMFDDIO.CFG CONFIGURATION FILE FOR NETWARE SERVER

; The following are the supported ODI keywords for IBMFDDIO:

NODE=400000000002M

SLOT=2

FRAME=FDDI_802.2

; The following are ODI FDDI keywords with sample values:

ATTACHMENTCLASS=1

PMFPASSWORD="12345678"

USERDATA="IBM FDDI Adapter at Server"

RXBUFFERS=16

ODI for FDDI, Version 1.0   June 26, 1992  11

IBMFDDIO LAN DRIVER PERFORMANCE TUNING

This section describes how to get the best performance from the IBM FDDI

Adapter and IBMFDDIO LAN drivers.  Since the maximum frame size that an IBM

FDDI Adapter can send is 4500 bytes, the receive buffers used by the IBMFDDIO

LAN drivers must be configured to 4500 bytes.  Below are the instructions on

how to configure the IBMFDDIO LAN driver receive buffers to 4500 bytes on

each of the platforms.

DOS Workstation

The size of the receive buffers used by the DOS IBMFDDIO.COM LAN driver is

4500 bytes and is not configurable.  The number of receive buffers used by

the DOS IBMFDDIO.COM LAN driver can be configured by specifying the RXBUFFERS

keyword in the NET.CFG file.  While changing the number of receive buffers

used by the IBMFDDIO.COM LAN driver may not change the performance of the IBM

FDDI Adapter greatly, it will change the amount of DOS memory that is used by

the IBMFDDIO.COM by 4500 per receive buffer.

See "Sample NET.CFG Configuration File for DOS and OS/2 Workstations" for a

sample NET.CFG file.

OS/2 Workstation

Since the IBMFDDIO.OS2 LAN driver uses Link Support Layer (LSL) buffers to

receive frames, the size of receive buffers used by the IBMFDDIO.OS2 LAN

driver is configurable by changing the size of the LSL buffers in the NET.CFG

file.  Create or edit the NET.CFG file, and under the Link Support heading,

change the second value of the BUFFERS parameter to 4500, and then restart

your computer.

See "Sample NET.CFG Configuration File for DOS and OS/2 Workstations" for a

sample NET.CFG file.

NetWare Server

Since the IBMFDDIO.LAN driver uses NetWare receive buffers to receive frames,

the size of receive buffers used by the IBMFDDIO.LAN driver is configurable

by changing the size of the NetWare receive buffers.  Create or edit the

STARTUP.NCF file and add the following line to change the size of the receive

buffers:

Set Maximum Physical Receive Packet Size = 4500

NOTE:  If running on NetWare 3.11,

Set Maximum Physical Receive Packet Size = 4202

ODI for FDDI, Version 1.0   June 26, 1992  12

IBM FDDI ADAPTER CUSTOM STATISTIC COUNTERS

The custom statistic counters give the user information that is specific to

the IBM FDDI Adapter and IBMFDDIO LAN drivers.  Following are descriptions of

the custom statistic counters and what action, if any, should be taken based

on these counters.

Receive Frames Discarded

Description:  This counter indicates the number of frames that were

received by the IBM FDDI Adapter but were discarded by

the adapter because they were not processed within the

allowed time period by the IBMFDDIO LAN driver.

Cause/Action:  If this counter steadily increases, it may indicate

a problem at your computer.  Try increasing the number

of link support buffers for an OS/2 workstation, the

RXBUFFERS parameter in the NET.CFG file for a DOS work-

station, or the number of NetWare Server communications

buffers for a server; then, reload the IBMFDDIO LAN

driver.  If this problem persists, contact you network

administrator.

Transmit Frames Discarded

Description:  This counter indicates the number of frames that were not

transmitted by the IBM FDDI Adapter within the allowed

time period; the frames were discarded by the IBMFDDIO LAN

driver.

Cause/Action:  Possible explanations for this problem are as follows:

1.  The FDDI ring is experiencing problems and,

therefore, the IBM FDDI Adapter could not transmit

frames.  If this problem persists, contact your

network administrator.

2.  The IBM FDDI Adapter is not responding.

Check the adapter by running the diagnostic tests

located on the Option Diskette that was shipped

with the IBM FDDI Adapter.  If there are no

problems with the adapter, reload the IBMFDDIO LAN

driver.

Adapter Check Count

Description:  This counter indicates the number of adapter check errors

reported by the IBM FDDI Adapter.

Cause/Action:  The IBM FDDI Adapter and IBMFDDIO LAN driver

may recover from this error.

If this counter continues to increase, check the adapter

ODI for FDDI, Version 1.0   June 26, 1992  13

by running the adapter diagnostic tests located on the

Option Diskette that was shipped with the IBM FDDI

Adapter.  If there are no problems with the adapter,

reload the IBMFDDIO LAN driver.

Adapter Diagnostic Errors

Description:  This counter indicates the number of diagnostic

test errors reported by the IBM FDDI Adapter.

Cause/Action:  If this counter continues to increase, check the adapter

by running the adapter diagnostic tests located on the

Option Diskette that was shipped with the IBM FDDI

Adapter.  If there are no problems with the adapter,

reload the IBMFDDIO LAN driver.

Disconnect Frames Received

Description:  This counter indicates the number of disconnect

request frames that were received by the IBM

FDDI Adapter from another FDDI adapter.

Cause/Action:  If you are experiencing problems communicating

on the network and this counter is nonzero,

reload the IBMFDDIO LAN driver.

If this counter continues to be nonzero, contact your

network administrator.

Micro Channel Check Count

Description:  This counter indicates that an error occurred with the

Micro Channel bus.

Cause/Action:  The IBM FDDI Adapter detected an error with the

Micro Channel bus while accessing or attempting to

access the bus.

If this counter continues to increase, run the

diagnostics on the reference disk that came with your

computer to test for problems with the Micro Channel

bus.

Microcode CRC Errors

Description:  The cyclic redundancy check (CRC) of the IBM FDDI

Adapter microcode did not match the expected value.

Cause/Action:  When the IBM FDDI Adapter is initialized or reset,

it performs a CRC of its microcode to make sure that the

ODI for FDDI, Version 1.0   June 26, 1992  14

microcode has not become corrupted.  If this counter

is nonzero, perform one of the following actions:

For a Server:

Replace the IBMFDDIO LAN driver with a copy from the

original distribution diskette.

For a DOS or OS/2 workstation:

Replace the IBMFDDIO LAN driver with a copy from the

original distribution diskette.

Replace the IBMFDDI.BIN microcode file with a copy

from the IBM FDDI Adapter Option Diskette that was

shipped with your IBM FDDI Adapter.

If the counter continues to be nonzero, contact your

network administrator or IBM support center for a new

copy of the appropriate file.

Path Test Errors

Description:  This counter indicates that the IBM FDDI Adapter

detected an error while performing a path test

when inserting into the ring.

Cause/Action:  When the IBM FDDI Adapter attempts to insert into

the ring, it performs a path test.  This test checks

the various data paths on the adapter to make sure that

the adapter is capable of sending and receiving data.

If this counter is nonzero, there may be a hardware

problem with the IBM FDDI Adapter. Check the adapter

by running the adapter diagnostic tests located on the

Option Diskette that was shipped with the IBM FDDI

Adapter.  If this problem persists, contact your network

administrator or IBM support center.

Warm Start Count

Description:  This counter indicates how many times the IBM FDDI

Adapter has restarted itself because of detected errors.

Cause/Action:  The IBM FDDI Adapter will restart itself if it detects

an error that can be solved by restarting the adapter

microcode.  The IBM FDDI Adapter and IBMFDDIO LAN

driver may recover from this condition.

If you are experiencing problems with your network and

this counter continues to increase, check the adapter

by running the adapter diagnostic tests located on the

Option Diskette that was shipped with the IBM FDDI

Adapter.  If there are no problems with the adapter,

reload the IBMFDDIO LAN driver.

ODI for FDDI, Version 1.0   June 26, 1992  15

MESSAGES FOR THE IBMFDDIO LAN DRIVER

IBMFDDIO-xx-yyy  Where the -xx- can be DOS, OS/2, or NW (NetWare Server).

IBMFDDIO-xx-001: The LSL is not loaded.

Cause:  The Link Support Layer (LSL) module was not loaded

before loading the IBMFDDIO LAN driver.  The LSL is

required by the IBMFDDIO LAN driver.

The IBMFDDIO LAN driver did not load.

Action: Load the LSL module for the operating system you are

using.  Refer to the appropriate Novell NetWare manual

listed below for information about loading the LSL module

for your operating system.

For DOS:

NOVELL NETWARE ODI SHELL FOR DOS

For OS/2:

NOVELL NETWARE REQUESTER FOR OS/2

IBMFDDIO-xx-002: The LSL has no room for a board using Frame <type>.

Variable Data:  <type>

- FDDI_802.2

- FDDI_SNAP

Cause:  An error occurred while trying to load the IBMFDDIO

LAN driver with the specified frame type.  The

maximum number of LAN drivers may already be loaded.

The IBMFDDIO LAN driver did not load.

Action: Unload LAN drivers that are not being used, and then

load the IBMFDDIO LAN driver.  If this error persists,

load the IBMFDDIO LAN driver before loading other LAN

drivers.  This may require you to edit the CONFIG.SYS

or AUTOEXEC.BAT file before restarting your workstation.

Refer to the Novell NetWare manual listed below for

information about loading LAN drivers for your operating

system.

For DOS:

NOVELL NETWARE ODI SHELL FOR DOS

For OS/2:

NOVELL NETWARE REQUESTER FOR OS/2

IBMFDDIO-xx-003: Could not find IBMFDDIO MLID to unload.

Cause:  The IBMFDDIO LAN driver or Multiple Link Interface

Driver (MLID) could not be unloaded.  The IBMFDDIO

ODI for FDDI, Version 1.0   June 26, 1992  16

LAN driver may not have been loaded or was previously

unloaded.

Action: None.

IBMFDDIO-xx-004: A TSR is loaded above the IBMFDDIO MLID.

Cause:  The IBMFDDIO Multiple Link Interface Driver (MLID)

could not be unloaded because Terminate Stay Resident

(TSR) programs were loaded after the IBMFDDIO LAN driver

was loaded.

Action: Try to unload the TSR programs that were loaded after

IBMFDDIO LAN driver, and then unload the IBMFDDIO LAN

driver.  If some TSR programs cannot be unloaded, then

load the IBMFDDIO LAN driver after loading the TSR

programs that could not be unloaded.  You may need to

edit the CONFIG.SYS or AUTOEXEC.BAT file to change the

order of loading the IBMFDDIO LAN driver.  Refer to the

Novell NetWare manual listed below for information

about loading LAN drivers for your operating system.

For DOS:

NOVELL NETWARE ODI SHELL FOR DOS

For OS/2:

NOVELL NETWARE REQUESTER FOR OS/2

IBMFDDIO-xx-005: IBMFDDIO MLID could not unload; the operation is

aborted.

Cause:  An error occurred while trying to unload the IBMFDDIO

Multiple Link Interface Driver (MLID).

Action: Try to unload the IBMFDDIO LAN driver again.  If this

error persists, check the adapter by running the

adapter diagnostic tests located on the Option Diskette

that was shipped with the IBM FDDI Adapter.  If the

adapter passes the diagnostic tests and this error

continues to occur, contact your network administrator

or IBM support center.

IBMFDDIO-xx-006: The adapter did not initialize.  IBMFDDIO did not load.

Cause:  An error occurred during initialization of the IBM FDDI

Base Adapter/A and Extended Adapter/A.  The reason for

this error is in the message

that is written on the screen above this error message.

The IBMFDDIO LAN driver did not load.

Action: Follow the recommended action in the message that is

ODI for FDDI, Version 1.0   June 26, 1992  17

written on the screen above this error message.

IBMFDDIO-xx-007: You need another MLID Section Heading in the NET.CFG file

in order to load the MLID again.

Cause:  The IBMFDDIO LAN driver could not be loaded again because

there was no configuration information describing how

to load it.  The IBMFDDIO LAN driver can be loaded once

with default values.  If the IBMFDDIO LAN driver is loaded

again, configuration information for loading an additional

copy of the IBMFDDIO LAN driver must be provided in an MLID

Section Heading of the NET.CFG file.

Action: Edit the NET.CFG file and add an MLID Section Heading

for the IBMFDDIO LAN driver.  Refer to the appropriate

Novell NetWare manual listed below for information

about editing a NET.CFG file, and to

"Configurable IBMFDDIO LAN Driver Parameters"

for information about valid NET.CFG file parameters for

the IBMFDDIO LAN driver.

For DOS:

NOVELL NETWARE ODI SHELL FOR DOS

For OS/2:

NOVELL NETWARE REQUESTER FOR OS/2

IBMFDDIO-xx-008: A NET.CFG file is required to load the MLID again.

Cause:  The IBMFDDIO LAN driver could not be loaded again

because there was no NET.CFG file describing how to

load it.  The IBMFDDIO LAN driver can be loaded once

with default values.  If the IBMFDDIO LAN driver is

loaded again, a NET.CFG file containing an MLID Section

Header for the IBMFDDIO LAN driver must be created.

Action: Create a NET.CFG file and add an MLID Section Heading

for the IBMFDDIO LAN driver.  Refer to the appropriate

Novell NetWare manual listed below for information

about editing an NET.CFG file, and to

"Configurable IBMFDDIO LAN Driver Parameters"

for information about valid NET.CFG file parameters for

the IBMFDDIO LAN driver.

For DOS:

NOVELL NETWARE ODI SHELL FOR DOS

For OS/2:

NOVELL NETWARE REQUESTER FOR OS/2

IBMFDDIO-xx-009: The NET.CFG entry has been ignored.

ODI for FDDI, Version 1.0   June 26, 1992  18

Cause:  An entry was specified in the NET.CFG file for the IBM

FDDI Adapter but was ignored. The entry may have been

incorrect or incompatible with the IBM FDDI hardware.

The IBMFDDIO LAN driver was loaded.

Action: Refer to

"Configurable IBMFDDIO LAN Driver Parameters"

for information about valid NET.CFG file parameters for

the IBMFDDIO LAN driver.

IBMFDDIO-xx-010: The IBMFDDIO MLID has been successfully removed.

Cause:  The IBMFDDIO LAN driver or Multiple Link Interface

Driver (MLID) was unloaded successfully.

Action: None.

IBMFDDIO-xx-011: The MLID does not support frame <type>.

The protocol keyword has been ignored.

Variable Data:   <type>

- Supplied from the NET.CFG file, the frame

type field of the PROTOCOL keyword found in the

MLID Section Heading for the IBMFDDIO LAN driver.

Cause:  The frame type field of the PROTOCOL keyword that

was found in the IBMFDDIO MLID Section Heading of the

NET.CFG file is not supported by the IBMFDDIO LAN driver.

Action: Edit the NET.CFG file and specify a valid frame type

parameter for the PROTOCOL keyword in the MLID Section

Heading for the IBMFDDIO LAN driver.  Refer to the

appropriate Novell NetWare manual listed below for

information about editing a NET.CFG file, and to

"Configurable IBMFDDIO LAN Driver Parameters"

for information about valid NET.CFG file parameters for

the IBMFDDIO LAN driver.

For DOS:

NOVELL NETWARE ODI SHELL FOR DOS

For OS/2:

NOVELL NETWARE REQUESTER FOR OS/2

IBMFDDIO-xx-012: The protocol keyword must have a frame type.  Entry ignored.

Cause:  The PROTOCOL keyword that was found in the IBMFDDIO

MLID Section Heading of the NET.CFG file did not

specify a required frame type parameter.

ODI for FDDI, Version 1.0   June 26, 1992  19

Action: Edit the NET.CFG file and add a valid frame type

parameter to the PROTOCOL keyword in the MLID Section

Heading for the IBMFDDIO LAN driver.  Refer to the

appropriate Novell NetWare manual listed below for

information about editing a NET.CFG file, and to

"Configurable IBMFDDIO LAN Driver Parameters"

for information about valid NET.CFG file parameters for

the IBMFDDIO LAN driver.

For DOS:

NOVELL NETWARE ODI SHELL FOR DOS

For OS/2:

NOVELL NETWARE REQUESTER FOR OS/2

IBMFDDIO-xx-013: The MLID could not register Protocol ID <number>

for protocol stack <name> for frame type <frame>.

Variable Data:  <number> -  Hexadecimal protocol ID for

PROTOCOL keyword

<name>   -  Name parameter for PROTOCOL

keyword

<frame>  -  Frame type parameter for

PROTOCOL keyword.

Cause:  One or more of the parameters specified for the PROTOCOL

keyword in the NET.CFG file for the IBMFDDIO LAN driver

was incorrect.

Action: Edit the NET.CFG file and correct the incorrect PROTOCOL

keyword parameters in the MLID Section Heading for the

IBMFDDIO LAN driver.  Refer to the appropriate Novell

NetWare manual listed below for information about editing

a NET.CFG file, and to

"Configurable IBMFDDIO LAN Driver Parameters"

for information about valid NET.CFG file parameters for

the IBMFDDIO LAN driver.

For DOS:

NOVELL NETWARE ODI SHELL FOR DOS

For OS/2:

NOVELL NETWARE REQUESTER FOR OS/2

IBMFDDIO-xx-014: This version of the LSL is not supported.

Cause:  The version level of the LSL module that is loaded is no

longer supported.  The IBMFDDIO LAN driver did not load.

Action: Unload the LSL module and load a later version of the LSL

module.

ODI for FDDI, Version 1.0   June 26, 1992  20

IBMFDDIO-xx-015: The frame type is already active for the frame <type>.

Variable Data:   <type>

- FDDI_802.2

- FDDI_SNAP

Cause:  The IBMFDDIO LAN driver has been loaded previously to

support the specified frame type.  The NET.CFG file

may contain duplicate FRAME keyword entries for the

IBMFDDIO LAN driver.  The IBMFDDIO LAN driver did not

load.

Action: Check the NET.CFG file for 2 FRAME keywords that have

the same frame type for the IBMFDDIO LAN driver as

specified in this message, but that are in different MLID

Section Headings.  If duplicate entries exist, change one

of the FRAME keywords.  Valid frame type parameters for

the IBMFDDIO LAN driver can be found in

"Configurable IBMFDDIO LAN Driver Parameters."

If duplicate entries do not exist in the NET.CFG

file, then the IBMFDDIO LAN driver was previously

loaded from either the command line or the NET.CFG

file with the FRAME keyword frame type specified

in this error message.  No action is necessary.

IBMFDDIO-xx-016: The node address was incorrectly specified in the NET.CFG.

Cause:  The NODE ADDRESS keyword specified in the NET.CFG file

for the IBM FDDI Adapter is an incorrect adapter

node address.  The node address specified has either

the group address bit set or the locally administered

address bit not set.  The node address was corrected

and the IBMFDDIO LAN driver was loaded.

Action: Correct the NODE ADDRESS keyword parameter in the

NET.CFG file to specify a correct locally administered

address according to the IBM TOKEN-RING NETWORK

ARCHITECTURE REFERENCE.

IBMFDDIO-xx-017: An invalid keyword was specified in the NET.CFG on line

<xxx>.

Variable Data:   <xxx> -

Decimal value of the line number in the NET.CFG file.

Cause:  The keyword specified on line xxx in the NET.CFG file

is incorrect.  The IBMFDDIO LAN driver was loaded.

Action: Correct the keyword specified on line xxx of the NET.CFG

file to a valid keyword.  Refer to the appropriate Novell

ODI for FDDI, Version 1.0   June 26, 1992  21

NetWare manual listed below for information about

NET.CFG file keywords.

For DOS:

NOVELL NETWARE ODI SHELL FOR DOS

For OS/2:

NOVELL NETWARE REQUESTER FOR OS/2

IBMFDDIO-xx-018: The frame type specified in the NET.CFG is not supported.

Cause:  The frame type specified for the FRAME keyword in

an IBMFDDIO MLID Section Heading of the NET.CFG file

is not supported.

Action: Edit the NET.CFG file and specify a valid frame type

parameter for the FRAME keyword in the MLID Section

Heading for the IBMFDDIO LAN driver.  Refer to the

appropriate Novell NetWare manual listed below for

information about editing a NET.CFG file, and to

"Configurable IBMFDDIO LAN Driver Parameters"

for information about valid NET.CFG file parameters for

the IBMFDDIO LAN driver.

For DOS:

NOVELL NETWARE ODI SHELL FOR DOS

For OS/2:

NOVELL NETWARE REQUESTER FOR OS/2

IBMFDDIO-xx-019: An invalid FDDI node address is specified in NET.CFG.

The MLID modified the incorrect address bits.

Cause:  The NODE ADDRESS keyword specified in the NET.CFG file

for the IBM FDDI Adapter is an incorrect adapter node

address.  The IBMFDDIO LAN driver did not load.

Action: Correct the NODE ADDRESS keyword parameter in the

NET.CFG file to specify a correct locally administered

address according to the IBM TOKEN-RING NETWORK

ARCHITECTURE REFERENCE.

IBMFDDIO-xx-020: This version of the statistics table is not supported.

Cause:  The version level of the statistics table for the IBMFDDIO

LAN driver is not supported.  The IBMFDDIO LAN driver did

not load.

Action: Try to load the IBMFDDIO LAN driver again.  If this

error persists, reinstall the IBMFDDIO LAN driver.

See "IBMFDDIO LAN Driver Installation Procedure"

ODI for FDDI, Version 1.0   June 26, 1992  22

for installation instructions.  Load the IBMFDDIO LAN

driver again.  If this error still persists, contact your

network administrator or IBM support center.

IBMFDDIO-xx-021: Could not add MLID Protocol ID.

Cause:  The LSL is configured with resources for a set number of

Protocol Identifiers (default is 4).  This number has been

exceeded.

Action: Edit the NET.CFG file and add the MAX STACKS keyword to

reflect the number of boards to be loaded.  Unload the driver

and then reload it.

example of NET.CFG

.

.

.

LINK SUPPORT

MAX STACKS 6

.

.

.

Refer to the, NOVELL NETWARE ODI SHELL FOR DOS or

NOVELL NETWARE REQUESTER FOR OS/2, manual for

information on the NET.CFG file.

IBMFDDIO-xx-022: Could not allocate memory.  Virtual board did not load.

Cause:  Cannot get enough memory from the system to load another

virtual board.

Action: Do one of the following:

1) Install more system memory in the machine.

2) Edit the CONFIG.SYS file and remove drivers that are

not needed.

IBMFDDIO-xx-024: The MSM is unable to parse a required custom keyword.

Cause:  The value entered for a required custom keyword is

incorrect.

Action: Check the values for specified custom keyword when

the IBMFDDIO LAN driver was loaded.  Correct keyword

values that are incorrect, and then reload the

IBMFDDIO LAN driver.

ODI for FDDI, Version 1.0   June 26, 1992  23

IBMFDDIO-xx-050: The board cannot be found.

Cause:  There is no IBM FDDI Adapter installed in this computer.

The IBMFDDIO LAN driver did not load.

Action: Install an IBM FDDI Adapter in this computer if it has

a 80386 or higher microprocessor, and then reload the

IBMFDDIO LAN driver.

IBMFDDIO-xx-056: This interrupt is already used and cannot be shared.

Cause:  The interrupt level configured for the IBM FDDI Adapter

with the reference diskette cannot be set.  The IBMFDDIO

LAN driver for DOS and OS/2 workstations cannot share the

same interrupt level with other hardware adapters or

device drivers.

Action: Run the reference diskette for your computer and check

that the interrupt level specified for the IBM FDDI

Adapter does not conflict with other hardware adapters

installed in your computer.  If it does, change the

configured interrupt level for the IBM FDDI Adapter to

a value that does not conflict with any other hardware

adapters installed in your computer.

IBMFDDIO-xx-071: The matching virtual adapter could not be found.

Cause:  An error occurred while trying to load another copy

of the IBMFDDIO LAN driver to support an additional

frame type (FRAME= parameter) on an IBM FDDI Adapter.

The MSM could not find a previous copy of the IBMFDDIO

LAN driver require to load the specified frame type.

Action: Obtain the most current copy of the MSM.NLM or

MSM311.NLM (if running NetWare version 3.11) and then

reload the IBMFDDIO LAN driver for the desired frame

types.  If this error persists, contact your network

administrator or IBM support center.  Refer to the

NOVELL NETWARE SYSTEM ADMINISTRATION manual for

information about loading and unloading LAN drivers.

IBMFDDIO-xx-072: A resource tag is unavailable.

Cause:  The NetWare Operating system was unable to allocate

memory for a resource tag.  The IBMFDDIO LAN driver

did not load because required internal resources

were not available.  There may be too many NetWare

Loadable Modules (NLMs) running in the NetWare Server.

Action: Unload the NLMs that are loaded but are not being used,

and then load the IBMFDDIO LAN driver.  If this error

persists, take down and restart the server.  Load the

ODI for FDDI, Version 1.0   June 26, 1992  24

IBMFDDIO LAN driver before loading NLMs, this may require

you to edit the AUTOEXEC.NCF file before restarting the

server.  Refer to NOVELL NETWARE SYSTEM ADMINISTRATION for

information about the AUTOEXEC.NCF file.

IBMFDDIO-xx-073: Unable to allocate memory.

Cause:  The NetWare Operating system was unable to allocate

memory for the IBMFDDIO LAN driver.  The IBMFDDIO

LAN driver did not load.

Action: Unload the NetWare Loadable Modules (NLMs) that are

loaded but are not being used, and then load the IBMFDDIO

LAN driver.  If this error persists, take down and

restart the server.  Load the IBMFDDIO LAN driver before

loading NLMs; this may require you to edit the

AUTOEXEC.NCF file before restarting the server.  Refer to

NOVELL NETWARE SYSTEM ADMINISTRATION for information

about the AUTOEXEC.NCF file.

IBMFDDIO-xx-074: The hardware interrupt cannot be set.

Cause:  The NetWare Operating system was unable to set the

hardware interrupt level requested by the IBMFDDIO

LAN driver.  The IBMFDDIO LAN driver did not load.

Action: Try to load the IBMFDDIO LAN driver again.  If the error

persists, contact your network administrator or IBM

support center.

IBMFDDIO-xx-075: The MLID cannot be registered with the LSL.

Cause:  The IBMFDDIO LAN driver could not be registered with the

Link Support Layer (LSL) module.  The maximum number of

LAN drivers are already loaded or there was not enough

memory to load the IBMFDDIO LAN driver.  The IBMFDDIO

LAN driver did not load.

Action: Unload LAN drivers that are not being used and then

load the IBMFDDIO LAN driver.  If this error persists,

take down and restart the server.  Load the IBMFDDIO LAN

driver before loading other LAN drivers; this may require

you to edit the AUTOEXEC.NCF file before restarting the

server.  Refer to NOVELL NETWARE SYSTEM ADMINISTRATION for

information about the AUTOEXEC.NCF file.

IBMFDDIO-xx-076: The polling procedure cannot be added.

Cause:  There are too many polling processes registered with

the NetWare Operating system.  The IBMFDDIO LAN

ODI for FDDI, Version 1.0   June 26, 1992  25

driver did not load.

Action: Unload LAN drivers or NetWare Loadable Modules that

are loaded but are not being used.  Try to load the

IBMFDDIO LAN driver again.  If this error persists,

contact your network administrator or IBM support

center.

IBMFDDIO-xx-077: The event notification routine cannot be registered.

Cause:  The NetWare Operating system was unable to allocate

memory to register the IBMFDDIO LAN driver event

notification routine.  The IBMFDDIO LAN driver did

not load.

Action: Unload LAN drivers or NetWare Loadable Modules that

are loaded but are not being used.  Try to load the

IBMFDDIO LAN driver again.  If this error persists,

contact your network administrator or IBM support

center.

IBMFDDIO-xx-126: The group bit in the node address override was cleared.

Cause:  The address specified for the node address parameter

is an incorrect adapter node address.  The node

address specified had the group address bit set.

The group address bit was cleared by the IBMFDDIO

LAN driver.  The IBMFDDIO LAN driver was loaded.

Action: Correct the node addres parameter to specify a locally

administered address according to the IBM TOKEN-RING

NETWORK ARCHITECTURE REFERENCE.  On a DOS or

OS/2 Workstation change the node address parameter in

the NET.CFG file.  On a NetWare Server change the node

address parameter specified on the command line when the

IBMFDDIO LAN driver was loaded or change the node address

specified in the IBMFDDIO.CFG configuration file.

IBMFDDIO-xx-078: The firmware file cannot be read.

Cause:  The Netware Operating system was unable to load the

IBMFDDIO LAN driver.  The IBMFDDIO LAN driver was not

loaded.

Action: Obtain another copy of the IBMFDDIO LAN driver from

the original distribution diskette.  Reload the IBMFDDIO

LAN driver.  If this error persists, contact you network

network administrator or IBM support center.

IBMFDDIO-xx-084: Unable to allocate memory below 16 Megabyte boundary.

ODI for FDDI, Version 1.0   June 26, 1992  26

Cause:  The Netware Operating system ran out of memory buffers

below 16 megabytes.  The IBMFDDIO LAN driver was not

loaded.

Action: Set the Reserved Buffers Below 16 Meg parameter in the

STARTUP.NCF to a larger number.

IBMFDDIO-xx-086: The driver parameter block is too small.

Cause:  The IBMFDDIO.LAN file is designed to run with an older

version of the MSM.NLM.  The IBMFDDIO LAN driver was

not loaded.

Action: Obtain an older version of the MSM.NLM or a newer

version of the IBMFDDIO.LAN file.

IBMFDDIO-xx-087: The media parameter block is too small.

Cause:  The FDDITSM.NLM NetWare Loadable Module (NLM) is

designed to run with an older version of the MSM.NLM

or MSM311.NLM if running NetWare version 3.11.

Action: Obtain an older version of the MSM.NLM (MSM311.NLM

if running NetWare version 3.11) or a new version of

the FDDITSM NLM.

IBMFDDIO-xx-091: The hardware configuration conflicts.

Cause:  The IBMFDDIO LAN driver requested use of a hardware

resource that was already being used by another

hardware device.  The IBMFDDIO LAN driver did not load.

Action: Change the hardware configuration of the IBM FDDI

Adapter or the hardware configuration of the

conflicting hardware device.

IBMFDDIO-xx-092: Cannot schedule AES without an HSM routine.

Cause:  The IBMFDDIO.LAN file is damaged.

Action: Obtain another copy of the IBMFDDIO LAN driver from

the original distribution diskette.  Reload the IBMFDDIO

LAN driver.  If this error persists, contact you network

network administrator or IBM support center.

IBMFDDIO-xx-093: Cannot schedule interrupt time call back without an

HSM routine.

ODI for FDDI, Version 1.0   June 26, 1992  27

Cause:  The IBMFDDIO.LAN file is damaged.

Action: Obtain another copy of the IBMFDDIO LAN driver from

the original distribution diskette.  Reload the IBMFDDIO

LAN driver.  If this error persists, contact you network

network administrator or IBM support center.

IBMFDDIO-xx-094: Cannot set hardware interrupt witout an HSM routine.

Cause:  The IBMFDDIO.LAN file is damaged.

Action: Obtain another copy of the IBMFDDIO LAN driver from

the original distribution diskette.  Reload the IBMFDDIO

LAN driver.  If this error persists, contact you network

network administrator or IBM support center.

IBMFDDIO-xx-095: Cannot add polling without an HSM routine.

Cause:  The IBMFDDIO.LAN file is damaged.

Action: Obtain another copy of the IBMFDDIO LAN driver from

the original distribution diskette.  Reload the IBMFDDIO

LAN driver.  If this error persists, contact you network

network administrator or IBM support center.

IBMFDDIO-xx-127: The local bit in the node address override was set.

Cause:  The address specified for the node address parameter

is an incorrect adapter node address.  The node address

specified does not have the locally administered address

bit set.  The locally administered address bit was set

by the IBMFDDIO LAN driver.  The IBMFDDIO LAN driver was

loaded.

Action: Correct the node address parameter to specify a locally

administered address according to the IBM TOKEN-RING

ARCHITECTURE REFERENCE.  On a DOS or OS/2

Workstation change the node address parameter in the

NET.CFG file.  On a NetWare Server change the node

address parameter specified on the command line when the

IBMFDDIO LAN driver was loaded or change the node address

specified in the IBMFDDIO.CFG configuration file.

IBMFDDIO-xx-200: Initializing the IBM FDDI Adapter ... please wait.

Cause:  The IBM FDDI Adapter initialization process (resetting

the adapter and downloading the adapter microcode) can take

up to 30 seconds.

ODI for FDDI, Version 1.0   June 26, 1992  28

Action: None.

IBMFDDIO-xx-201: Inserting the IBM FDDI Adapter into the ring.

Cause:  The IBMFDDIO LAN driver is waiting for the IBM FDDI

Adapter to insert into the ring.

Action: None.

IBMFDDIO-xx-202: The IBM FDDI Adapter could not insert into the ring.

Cause:  The IBM FDDI Adapter did not insert into the ring within

the timeout value.  This is normal if there are no other

stations on the ring.  If there are other stations on the

ring, and the adapter did not insert, contact your network

administrator.

Action: This condition should correct itself when a second

station inserts into the ring.  If another station

inserts and this condition does not correct itself,

contact your network administrator or IBM support center.

IBMFDDIO-xx-203: The IBMFDDIO LAN driver and IBM FDDI Adapter require a

computer with at least an 80386 microprocessor.

Cause:  The IBM FDDI Adapter must be installed in a computer

that has a 80386 or higher microprocessor.  The IBMFDDIO

LAN driver did not load.

Action: Install the adapter in a computer with a 80386 or higher

microprocessor, and reload the IBMFDDIO LAN driver.

IBMFDDIO-xx-204: The IBM FDDI Adapter requires a computer with a Micro

Channel bus.

Cause:  The IBM FDDI Adapter will work only in a computer that

supports the IBM Micro Channel Architecture.

Action: Install the IBM FDDI Adapter and IBMFDDIO LAN driver

in a computer that supports the IBM Micro Channel

Architecture.

IBMFDDIO-xx-205: The IBMFDDIO LAN driver was not loaded because of an

incorrect keyword value.

Cause:  The value for one of the keywords specified on the

command line or in the IBMFDDIO.CFG file was not within

the allowable range.  The IBMFDDIO LAN driver did not

load.

ODI for FDDI, Version 1.0   June 26, 1992  29

Action: Correct the value of the keyword displayed in the

message that was displayed above this error message, and

reload the IBMFDDIO LAN driver.

IBMFDDIO-xx-206: The IBMFDDIO LAN driver was not loaded because of an

incorrect keyword value in the NET.CFG file.

Cause:  The value for one of the keywords specified in the

NET.CFG file was not within the allowable range.  The

IBMFDDIO LAN driver did not load.

Action: Correct the value for the keyword displayed in the

message that was displayed above this error message, and

reload the IBMFDDIO LAN driver.

IBMFDDIO-xx-207: The IBM FDDI Adapter could not be reset.

Cause:  The IBMFDDIO LAN driver timed out while waiting for the

IBM FDDI Adapter to complete a hardware reset.  The

IBMFDDIO LAN driver did not load.

Action: Restart your computer and try to load the IBMFDDIO LAN

driver again.  If this error persists, contact your

network administrator or IBM support center.

IBMFDDIO-xx-208: The IBM FDDI Adapter microcode file, IBMFDDI.BIN,

could not be loaded.

Cause:  A checksum error occurred when the IBMFDDIO LAN driver

tried to load the IBM FDDI Adapter microcode file,

IBMFDDI.BIN.

Action: Reinstall the IBMFDDIO LAN driver and the adapter

microcode file, IBMFDDI.BIN, and restart the computer.

If the error persists, contact your IBM support center or

place of purchase.

IBMFDDIO-xx-209: The IBMFDDIO LAN driver failed to initialize the

IBM FDDI Adapter.

Cause:  There was a problem while initializing the IBM FDDI

Adapter.  There may be previous error messages

above this one or the IBM FDDI Adapter may not be

responding.

Action: Correct any problems with error messages displayed

above this error message, and then reload the IBMFDDIO

LAN driver.

ODI for FDDI, Version 1.0   June 26, 1992  30

If this is the only error message that is displayed,

restart your computer and check the adapter by running

the adapter diagnostic tests located on the Option

Diskette that was shipped with the IBM FDDI Adapter.

Then, reload the IBMFDDIO LAN driver.  If this error

persists, contact your network administrator or IBM

support center.

IBMFDDIO-xx-210: Could not find an IBM FDDI Adapter in this computer.

Cause:  There is no IBM FDDI Adapter installed in this computer.

The IBMFDDIO LAN driver did not load.

Action: Install an IBM FDDI Adapter in this computer if it has

a 80386 or higher microprocessor, and then reload the

IBMFDDIO LAN driver.

IBMFDDIO-xx-211: The IBM FDDI Adapter was not found in the slot

specified in the NET.CFG file.

Cause:  There is no IBM FDDI Adapter in the slot specified by the

SLOT keyword in the NET.CFG file.

Action: Correct the SLOT keyword so that it indicates a slot

number in which an IBM FDDI Adapter is located, and then

reload the IBMFDDIO LAN driver.

If you do not include a SLOT keyword, the IBMFDDIO LAN

driver will default to using the first IBM FDDI Adapter

it finds.

If there is an IBM FDDI Adapter in the specified slot,

contact your network administrator or IBM support center.

IBMFDDIO-xx-212: Insufficient memory available for configured transmit and

receive buffers.

Cause:  During initialization of the IBMFDDIO LAN driver, there

was insufficient memory available for the configured

number of transmit and receive buffers.

Action: (Server)

Increase the number of buffers that the server operating

system can allocate by editing the STARTUP.NCF file on

the server.  Increase the Minimum Packet Receive Buffers

server parameter in the STARTUP.NCF file by the total

number of transmit and receive buffers that the IBMFDDIO

LAN driver is using.  Take down, and then restart the

NetWare Server.

If this error persists, unload the LAN drivers and

ODI for FDDI, Version 1.0   June 26, 1992  31

NetWare Loadable Modules (NLMs) that are loaded but are

not being used at the NetWare Server.

(OS/2 Workstation)

Increase the number of buffers that are allocated by

the Link Support Layer (LSL.SYS) when it is loaded by

editing or creating a NET.CFG file.  Under the Link

Support heading in the NET.CFG file, increase the first

value of the BUFFERS parameter by the total number of

transmit and receive buffers that the IBMFDDIO LAN

driver is using.  Restart your computer.

See "Sample NET.CFG Configuration File for DOS and OS/2

Workstations" for a sample NET.CFG file.

(All Platforms)

If this error still persists, decrease the number of

buffers that are being used by the IBMFDDIO LAN driver

by specifying the transmit and receive keywords for the

IBMFDDIO LAN driver.

See "Configurable IBMFDDIO LAN Driver Parameters"

for information about specifying keywords for the

operating system that you are using.

IBMFDDIO-xx-213: A dual attached station was configured as a single

attached station.

Cause:  The IBMFDDIO LAN driver detected both an IBM FDDI Base

Adapter/A and an IBM FDDI Extender Adapter/A, which

enable this computer to function as a dual attachment

station.  However, the ATTACHMENTCLASS parameter is set

to 1 for single attachment.

Action: If you want this computer to remain a single attachment

station, no action is necessary.  Otherwise, remove

the ATTACHMENTCLASS parameter or change the value to

2 for dual attachment.  Reload the IBMFDDIO LAN driver.

IBMFDDIO-xx-214: A single attached station cannot be configured as a dual

attached station.

Cause:  The IBMFDDIO LAN driver detected only an IBM FDDI Base

Adapter/A, so the computer cannot function as a dual

attached station.  However, the ATTACHMENTCLASS parameter

is set to 2 for dual attachment.  The IBMFDDIO LAN driver

did not load.

Action: Either install an IBM FDDI Extender Adapter/A or change

the ATTACHMENTCLASS parameter to 1 for single attachment.

Reload the IBMFDDIO LAN driver.

ODI for FDDI, Version 1.0   June 26, 1992  32

IBMFDDIO-xx-215: The value specified for RXBUFFERS is not valid.  The

value has been decreased to the next supported value.

Cause:  An incorrect value was specified for RXBUFFERS either in

the NET.CFG file (for DOS and OS/2), on the command line,

or in the IBMFDDIO.CFG file (for the server).  The

IBMFDDIO LAN driver used the next lower supported value

from the one specified.

Action: None.  However, to keep this message from being displayed

when the IBMFDDIO LAN driver is loaded, change the value

specified by the RXBUFFERS keyword to one of the supported

values for your operating system.

See "Configurable IBMFDDIO LAN Driver Parameters"

for the supported values.

IBMFDDIO-xx-216: The value specified for TXBUFFERS is not valid.  The

value has been decreased to the next supported value.

Cause:  An incorrect value was specified for TXBUFFERS either in

the NET.CFG file (for DOS and OS/2), on the command

line, or in the IBMFDDIO.CFG file (for the server).  The

IBMFDDIO LAN driver used the next lower supported value

from the one specified.

Action: None.  However, to keep this message from being displayed

when the IBMFDDIO LAN driver is loaded, change the value

specified by the TXBUFFERS keyword to one of the supported

values for your operating system.

See "Configurable IBMFDDIO LAN Driver Parameters"

for the supported values.

IBMFDDIO-xx-217: The value specified for ATTACHMENTCLASS is not valid.

Cause:  The value specified for the ATTACHMENTCLASS parameter is

incorrect.  The IBMFDDIO LAN driver did not load.

Action: Specify either 1 for a single attachment station or 2

for a dual attachment station, and then reload the

IBMFDDIO LAN driver.

IBMFDDIO-xx-218: The value specified for USERDATA is not valid.

Cause:  The value specified for the USERDATA parameter is

incorrect for one of the following reasons:

- Too long

- Not enclosed in double quotes.

The IBMFDDIO LAN driver did not load.

ODI for FDDI, Version 1.0   June 26, 1992  33

Action: Remove or correct the USERDATA parameter as described in

"Configurable IBMFDDIO LAN Driver Parameters."

Reload the IBMFDDIO LAN driver.

IBMFDDIO-xx-219: The value specified for PMFPASSWORD is not valid.

Cause:  The value specified for the PMFPASSWORD parameter is

incorrect for one of the following reasons:

- Too long

- Too short

- Not enclosed in double quotes.

The IBMFDDIO LAN driver did not load.

Action: Remove or correct the PMFPASSWORD parameter as decribed

in "Configurable IBMFDDIO LAN Driver Parameters."

Reload the IBMFDDIO LAN driver.

IBMFDDIO-xx-220: The IBM FDDI Adapter path test failed while inserting.

Cause:  The adapter performs a test of its various data paths

when it is attempting to insert into the ring.  This

message indicates that the data path test has failed and

that the adapter could not insert into the ring.  If you

are running the IBMFDDIO LAN driver on a DOS or OS/2

workstation, then the IBMFDDIO LAN driver did not load.

Action: Restart the computer and try to load the IBMFDDIO LAN

driver again.  If the path test continues to fail,

check the adapter by running the adapter diagnostic tests

located on the Option Diskette that was shipped with the

IBM FDDI Adapter.  If there is a problem with the adapter

hardware, contact your network administrator or IBM

support center.

IBMFDDIO-xx-221: A disconnect frame was received from a remote station.

Cause:  The IBM FDDI Adapter received a disconnect frame request

that was sent by another FDDI adapter at a remote

station.  The IBM FDDI Adapter installed in this

computer disconnected from the ring.

Action: To insert the IBM FDDI Adapter back into the ring,

unload the IBMFDDIO LAN driver (for the NetWare Server

and DOS workstations) and then reload it.  If this error

persists, contact your network administrator or IBM

support center.

IBMFDDIO-xx-222: The IBM FDDI Adapter failed to insert into the ring.

ODI for FDDI, Version 1.0   June 26, 1992  34

Cause:  The IBM FDDI Adapter was unable to insert into the ring

because of an incorrect network configuration.

Action: Check that the FDDI adapter cable is connected correctly

to the IBM FDDI Base Adapter/A.  If you have a dual

attached station, check that the FDDI adapter cable is

connected correctly to the IBM FDDI Extender Adapter/A.

Refer to the hardware installation manual shipped with the

IBM FDDI Adapter for information about connecting your IBM

FDDI Adapter to the network.

Reload the IBMFDDIO LAN driver.  If this error persists,

have the network administrator check the network

configuration.

IBMFDDIO-xx-223: The IBM FDDI Adapter failed to insert into the ring;

another adapter is using the same node address.

Cause:  The IBM FDDI Adapter could not insert into the ring

because another station is using the same value that was

specified for the node address parameter at your station.

Action: Change the node address parameter value specified for

your station.  This parameter is specified as NODE= (at

the NetWare Server) or NODE ADDRESS in NET.CFG (at DOS

and OS/2 workstations).

See "Configurable IBMFDDIO LAN Driver Parameters"

for more information about configuration parameters.

IBMFDDIO-xx-224: Could not open the IBM FDDI Adapter microcode file,

IBMFDDI.BIN.

Cause:  The microcode file, IBMFDDI.BIN, could not be opened by

the IBMFDDIO LAN driver.  It is likely that the microcode

file is not or was never installed in the same directory

as the IBMFDDIO LAN driver.  The IBMFDDIO LAN driver did

not load.

Action: Copy the microcode file, IBMFDDI.BIN, from the adapter

diskette shipped with the IBM FDDI Adapter to the file

directory where the IBMFDDIO LAN driver is located; then,

reload the IBMFDDIO LAN driver.  If this error persists,

reinstall the IBMFDDIO LAN driver, and then reload the

driver.

See "IBMFDDIO LAN Driver Installation Procedure"

for installation instructions.

If this error persists, contact your IBM support center or

place of purchase.

IBMFDDIO-xx-225: Could not close the IBM FDDI Adapter microcode file,

ODI for FDDI, Version 1.0   June 26, 1992  35

IBMFDDI.BIN.

Cause:  An error occurred when the IBMFDDIO LAN driver attempted

to close the microcode file, IBMFDDI.BIN.  This error may

indicate a hardware problem with the computer.  The

IBMFDDIO LAN driver did not load.

Action: Reinstall the IBMFDDIO LAN driver and IBMFDDIO.BIN, and

then reload the IBMFDDIO LAN driver.

See "IBMFDDIO LAN Driver Installation Procedure"

for installation instructions.

If this error persists, contact your IBM support center or

place of purchase.

IBMFDDIO-xx-226: Could not read the IBM FDDI Adapter microcode file,

IBMFDDI.BIN.

Cause:  An error occurred when the IBMFDDIO LAN driver attempted

to read the microcode file, IBMFDDI.BIN.  This error may

indicate a hardware problem with the computer.  The

IBMFDDIO LAN driver did not load.

Action: Reinstall the IBMFDDIO LAN driver and IBMFDDIO.BIN, and

then reload the IBMFDDIO LAN driver.

If this error persists, contact your IBM support center or

place of purchase.

IBMFDDIO-xx-227: Could not set the configured interrupt level for the IBM

FDDI Adapter.

Cause:  The interrupt level configured for the IBM FDDI Adapter

with the reference diskette cannot be set.  The IBMFDDIO

LAN driver for DOS and OS/2 workstations cannot share the

same interrupt level with other hardware adapters or

device drivers.

Action: Run the reference diskette for your computer and check

that the interrupt level specified for the IBM FDDI

Adapter does not conflict with other hardware adapters

installed in your computer.  If it does, change the

configured interrupt level for the IBM FDDI Adapter to

a value that does not conflict with any other hardware

adapters installed in your computer.

IBMFDDIO-xx-228: A Micro Channel bus error occurred.

Cause:  An error was detected with the Micro Channel bus while

data was being transferred between the IBM FDDI Adapter

and the computer.  This condition is reported as an

ODI for FDDI, Version 1.0   June 26, 1992  36

alert.

Action: The adapter will retry the data transfer, and the

IBMFDDIO LAN driver MicroChannelError statistics counter

will be incremented.

If this error persists, check the adapter by running the

adapter diagnostic tests located on the Option Diskette

that was shipped with the IBM FDDI Adapter, and check your

computer by running the hardware diagnostic tests.  If

this error still persists, contact your IBM support center

or place of purchase of the IBM FDDI Adapter.

IBMFDDIO-xx-229: The IBM FDDI Adapter reported an adapter check error.

Cause:  The IBM FDDI Adapter detected an adapter check error.

The IBM FDDI Adapter is reset by the IBMFDDIO LAN

driver when this error is reported.  This condition

is reported as an alert.

Action: The IBM FDDI Adapter and IBMFDDIO LAN driver may

recover from this error.  If the Adapter Check Count

continues to increase, check the adapter by running the

adapter diagnostic tests located on the Option Diskette

that was shipped with the IBM FDDI Adapter.  See

"IBM FDDI Adapter Custom Statistic Counters"

for information about the Adapter Check Count.  If there

are no problems with the adapter, reload the IBMFDDIO LAN

driver.

IBMFDDIO-xx-230: The IBM FDDI Adapter failed a diagnostic test.

Cause:  The IBM FDDI Adapter failed a diagnostic test

when the adapter was reset.  This condition is

reported as an alert.

Action: The IBM FDDI Adapter and IBMFDDIO LAN driver may

recover from this error.  If the Adapter Diagnostic Count

continues to increase, check the adapter by running the

adapter diagnostic tests located on the Option Diskette

that was shipped with the IBM FDDI Adapter.  See

"IBM FDDI Adapter Custom Statistic Counters"

for information about the Adapter Diagnostic Count.  If

there are no problems with the adapter, reload the

IBMFDDIO LAN driver.

IBMFDDIO-xx-231: The IBM FDDI Adapter alert condition was corrected.

Cause:  The IBM FDDI Adapter and IBMFDDIO LAN driver have

recovered from a previously reported alert condition.

ODI for FDDI, Version 1.0   June 26, 1992  37

Action: None.

ODI for FDDI, Version 1.0   June 26, 1992  38