---------------------------------------------------------------------------

Bootloading Z80 using NMI, RESET and CLOCK.

---------------------------------------------------------------------------

Z80 system without ROM. Using NMI, RESET and CLOCK to bootload.

An NMI will push a value on the stack. The value depends on the instruction

address at the time where NMI is given. At beginning is the memory contens

unknown, and it is only possible to know the address immidiately after NMI

is given (0066H). Because instructions is unknown, will it not be possible

to know the address later.

The memory will need to be filled with a known value to control it.

To fill out memory with known values, is a lot of NMI's given until all

memory is filled by pushing 0067H to stack.

The value may not actualy be 0067H because some instruction is more than

one byte. Jump and restarts cause another address too. But in nearly all

cases, will the instruction pushed on the stack not be an instruction that

change the address (jump or restart). And when modifying the instruction at

0066H will it cause 0067H to be stored on stack. Now 0067H is stored.

Number of cycles to fill memory:

Any instruction at 0066H:

The 4-cycle instruction after          5+3+3+23 cycles.

modifying 0066H:                       5+3+3+4 cycles.

Total cycles of fill:                  ramsize * 49 cycles.

The fill is done by relasing the clock and running at 4MHz.

The value 0067H is a 4 cycle load and a 4 cycle nop instruction. Now will

the known number of clocks to next instruction after 0066H be known. An NMI

pushes the instruction address on stack, and set the instruction pointer to

0066H. The clock is now controlled to the address that should be pushed

next.

Now is it possible to load the code by pushing values on the stack by an

NMI at the address with the value that should be pushed into memory.

Everytime NMI is given is instruction pointer set to 0066H by the

processor.

It is only possible to use instructions, with instruction code less than

the address where the code is stored. If the stack is at top of memory,

then most instructions are accessable. If memory is small is less

instructions accessable. To handle this, will it need to be loaded a

bootloader. This bootloader use instructions from 0067H to 0100H+$ only,

where $ is address of code to store, and the bootloader allow to be

runned on a small system with 256 bytes only.

Before loading bootloader, is stack set to the destination address. The LD

SP,n instruction may be placed at any address in memory. It need two

pushes, and it is importent that first push not desturb the timing. The

push, will need to store instructions that is four cycles, even they are

intended for loading the stack. The stack load operation has more cycles,

but it will not desturb timing, since it is last word to store. SP is now

known after the memory has been executed. This is done by running the clock

at 4MHz.

The LD SP,n is somewhere in memory, and the addresses used for pushing

values by NMI, need to be known. A fill by making lot of NMI's will

push 0067H from SP and down. A typical SP is 402H. (0404H + 0231H

stored, where 0404H is two 4 cycles INC B instructions.). NMI is given

until SP is down at 0066H. The 67H 00H instructions is stored on

stack, and will cause the number of clocks to be known. Now is numbers

between 0067H to 0400H able to be pushed on stack.

First a special bootloader is saved into memory. The bootloader only use

instructions from 0067H to 0113H. It is possible to use on a small 256 byte

system (8085 + 8155 only). The bootloader is stored at 0000H to 001BH and a

256 byte system use address 00H..1BH for instructions in range 0100H to

0100H+$. These instruction need to be stored before the address is used.

Address is stored from top, and the instructions that use 0100H to 0113H is

not in 0000H to 0013H address room.

The bootloader uses reset, and clock to store (or read) data. Operation

depends on instruction at reset.

The bootloader is not critical to which value to store, and it is possible

to change, modify, read etc. A15 is used to return with bit's from

bootloader.

As far as I see may the methode be used for a wide range of CPU's.

I have only testet it on Z80A, but more processors may be

able to bootload using the methode.

The code, and the schematic is attached to this document.

Copyright (c) 1998, Jens Dyekjr Madsen.